Inputs for Vehicle selection and preparation for testing

I. Proposal (in Track changes)

2.2.	Vehicle

2.2.1.	Vehicle Selection

The vehicle shall be representative of vehicles to be put on the market as specified by the manufacturer in agreement with the Technical Service to fulfil the requirements of this Regulation.

Measurements shall be made without any trailer, except in the case of non-separable vehicles. At the request of the manufacturer, measurements may be made on vehicles with lift axle(s) in a raised position.

2.2.2.	Vehicle test mass mt and vehicle target mass mtarget

2.2.2.1.	Measurements shall be made on vehicles at the test mass mt specified according to the table 2 below.

When testing indoors, the test mass, mt shall be utilized by the control system of the dyno roller.  Actual mass of the vehicle has no effect on results and it is permitted to load the vehicle as necessary to prevent slip between the tyres and the dyno rolls. To detect excessive slip, it is recommended to control the ratio of engine rotational speed and vehicle speed between the acceleration phase and the constant-speed status. To avoid slip, it is possible to increase the axle load.

2.2.2.2.	Target mass, mtarget, is used to denote the mass that N2 and N3 vehicles should be tested at. The actual test mass of the vehicle can be less due to limitations on vehicle and axle loading.

| Vehicle category | Vehicle test mass |
| --- | --- |
| M1 | The test mass mt of the vehicle shall be between 0.9 mro ≤ mt ≤ 1.2 mro |
| N1 | The test mass mt of the vehicle shall be between 0.9 mro ≤ mt ≤ 1.2 mro |
| N2, N3 | mtarget = 50 [kg/kW] x Pn [kW] Extra loading, mxload, to reach the target mass, mtarget, of the vehicle shall be placed above the rear axle(s). If the test mass mt is equal to the target mass mtarget, the test mass mt shall be 0.95 mtarget ≤ mt ≤ 1.05 mtarget The sum of the extra loading and the rear axle load in an unladen condition, mra load unladen, is limited to 75 per cent of the technically permissible maximum laden mass allowed for the rear axle, mac ra max. If the test mass mt is lower than the target mass mtarget, the test mass mt shall be achieved with a tolerance of ±5 per cent. If the centre of gravity of the extra loading cannot be aligned with the centre of the rear axle, the test mass, mt, of the vehicle shall not exceed the sum of the front axle in an unladen condition, mfa load unladen, and the rear axle load in an unladen condition, mra load unladen plus the extra loading, mxload, and the mass of the driver md. The test mass for vehicles with more than two axles shall be the same as for a two-axle vehicle. If the vehicle mass of a vehicle with more than two axles in an unladen condition, munladen, is greater than the test mass for the two-axle vehicle, then this vehicle shall be tested without extra loading. If the vehicle mass of a vehicle with two axles, munladen, is greater than the target mass, then this vehicle shall be tested without extra loading. |
| M2 (M ≤ 3,500 kg) | The test mass mt of the vehicle shall be between 0.9mro ≤ mt ≤ 1.2mro |
| Complete M2 (M > 3,500 kg), M3 | If the tests are carried out with a complete vehicle having a bodywork, mtarget = 50 [kg/kW] x Pn [kW] is calculated either in compliance with conditions above (see N2, N3 category) or the test mass mt of the vehicle shall be 0.9 mro ≤ mt ≤ 1.1 mro . |
| Incomplete M2 (M > 3,500 kg), M3 | If the tests are carried with an incomplete vehicle not having a bodywork, mtarget = 50 [kg/kW] x Pn [kW] is calculated either in compliance with conditions above (see N2, N3 category), or the test mass mt of the vehicle shall be 0.9 mro ≤ mt ≤ 1.1 mro . where mro = mchassisM2M3 + mxloadM2M3 . |

Table 2: Specification of test mass for the various vehicle categories

2.2.2.3.	Calculation procedure to determine extra loading of N2 and N3 vehicles only

2.2.2.3.1.	Calculation of extra loading

The target mass, mtarget, (per kW rated power) for two-axle vehicles of category N2 and N3 is specified in the Table in paragraph 2.2.1: above.

mtarget = 50 [kg / kW] × Pn [kW]	(1)

To reach the required target mass, mtarget, for a vehicle being tested, the unladen vehicle, including the mass of the driver, md, shall be loaded with an extra mass, mxload, which shall be placed above the rear axle as given in Formula (8):

mtarget = munladen + md + mxload 	(2)

The target mass, mtarget, shall be achieved with a tolerance of ±5 per cent.

The vehicle mass of the test vehicle in the unladen condition, munladen, is calculated by measuring on a scale the unladen front axle load, mfa load unladen, and the unladen rear axle load, mra load unladen, as given in Formula (3):

munladen = mfa load unladen + mra load unladen 	(3)

By using Formulae (2) and (3), the extra loading, mxload, is calculated as given in Formulae (4) and (5):

mxload = mtarget – (md + munladen) 	(4)

mxload = mtarget − (md + mfa load unladen + mra load unladen) 	(5)

The sum of the extra loading, mxload, and the unladen rear axle load, mra load unladen, is limited to 75 per cent of the technically permissible maximum laden mass for the rear axle, mac ra max, as given in Formula (6):

0,75 mac ra max ≥ mxload + mra load unladen 	(6)

The mxload is limited according to Formula (7):

mxload ≤ 0,75 mac ra max − mra load unladen 	(7)

If the calculated extra loading, mxload, in Formula (5) fulfils Formula (7), then the extra loading is equal to Formula (5). The test mass, mt, of the vehicle is as calculated from Formula (8):

mt = mxload + md + mfa load unladen + mra load unladen 	(8)

In this case, the test mass of the vehicle is equal to the target mass

mt = mtarget	(9)

If the calculated extra loading, mxload, in Formula (5) does not fulfil Formula (7), but rather fulfils Formula (10)

mxload > 0,75 mac ra max − mra load unladen 	(10)

then, the extra loading, mxload, shall be as given by Formula (11):

mxload = 0,75 mac ra max − mra load unladen 	(11)

and the test mass, mt, of the vehicle shall be as given by Formula (12):

mt = 0,75 mac ra max + md + mfa load unladen 	(12)

In this case, the test mass of the vehicle is lower than the target mass

mt < mtarget 	(13)

The test mass, mt, shall be achieved with a tolerance of ± 5 per cent.

2.2.2.3.2.	Loading considerations if load cannot be aligned with the centre of rear axle

If the centre of gravity of the extra loading, mxload, cannot be aligned with the centre of the rear axle, the test mass of the vehicle, mt, shall not exceed the sum of the unladen front axle load, mfa load unladen, and the unladen rear axle load, mra load unladen, plus the extra loading, mxload, and the mass of the driver, md.

This means that if the actual front and rear axle loads are measured on a scale when the extra loading, mxload, is placed onto the vehicle and it is aligned with the centre of the rear axle, the test mass of the vehicle minus the mass of the driver is as given by Formula (14):

mt − md = mfa load laden + mra load laden 	(14)

Where:

mfa load laden = mfa load unladen	(15)

If the centre of gravity of the extra loading cannot be aligned with the centre of the rear axle, Formula (14) is still fulfilled, but

mfa load laden > mfa load unladen 	(16)

because the extra loading has partly distributed its mass to the front axle. In that case, it is not allowed to add more mass onto the rear axle to compensate for the mass moved to the front axle.

2.2.2.3.3.	Test mass for vehicles with more than two axles

If a vehicle with more than two axles is tested, then the test mass of this vehicle shall be the same as the test mass for the two-axle vehicle.

If the unladen vehicle mass of a vehicle with more than two axles is greater than the test mass for the two-axle vehicle, then this vehicle shall be tested without extra loading.

2.2.2.3.4. 	Calculation of the test mass of a virtual vehicle with two axles:

When a vehicle family is not represented by a two-axle vehicle because it is physically not available, the vehicle family can be represented by a vehicle with more than two axles (vrf). In that case the test mass of a virtual two-axle vehicle (mt (2 axles virtual)) can be calculated in the following way:

For the calculation of the unladen vehicle mass of the virtual two-axle vehicle (munladen (2 axles virtual)), take from the vehicle with more than two axles (vrf) the measured unladen front axle load (mfa (vrf) load unladen) and the measured unladen rear axle load of that driven rear axle (mra (vrf) load unladen) which has the highest unladen load.

If the vehicle (vrf) has more than one front axle, take the one with the highest unladen front axle load.

munladen (2 axles virtual) = mfa (vrf) load unladen + mra (vrf) load unladen

mxload (2 axles virtual) = mtarget − (md + munladen (2 axles virtual))

Due to the requirement that the sum of the extra loading (mxload (2 axles virtual)) and the unladen rear axle load, mra (vrf) load unladen, is limited to 75 per cent of the technically permissible maximum laden mass allowed for the rear axle, mac ra max (2 axles virtual),  this value, mac ra max (2 axles virtual), has to be chosen in such a way that it represents the rear axle of the forecasted highest production-volume in the manufacturer's variation with a technically permissible maximum laden mass allowed for the rear axle (mac ra max (chosen) ) for the vehicle family as declared by the manufacturer.

mac ra max (4x2 virtual) = mac ra max (chosen)

If mxload (2 axles virtual) ≤ 0,75 mac ra max (chosen) − mra (vrf) load unladen

then

mt (2 axles virtual) = mxload (2 axles virtual) + md + mfa (vrf) load unladen + mra (vrf) load unladen

and

mt (2 axles virtual) = mtarget

If mxload (2 axles virtual) > 0,75 mac ra max (chosen) − mra (vrf) load unladen

then

mt (2 axles virtual) = 0,75 mac ra max (chosen) + md + mfa (vrf) load unladen

and

mt (2 axles virtual) < mtarget

The test mass of the vehicle with more than two axles representing the vehicle family is defined as followed:

mt (vrf) = mt (2 axles virtual)

and the extra loading is calculated as

mxload (vrf) = mt (2 axles virtual) – md  – munladen (vrf)

2.2.2.4	At the applicant's request the vehicle of a category M2, M3, N2 or N3 is deemed representative of its completed type if the tests are carried out to an incomplete vehicle not having a bodywork. In the test of an incomplete vehicle all relevant soundproofing materials, panels and noise reduction components and systems shall be fitted on the vehicle as designed by the manufacturer except a part of bodywork which is built in a later stage.

No new test shall be required due to fitting of a supplement fuel tank or re-location of the original fuel tank on condition that other parts or structures of the vehicle apparently affecting sound emissions have not been altered.

The tyres to be used for the test shall be representative for the vehicle and shall be selected by the vehicle manufacturer and recorded in Addendum to the Communication form (Annex 1, Appendix 1). They shall correspond to one of the tyre sizes designated for the vehicle as original equipment. The tyre is or will be commercially available on the market at the same time as the vehicle. The tyres shall be inflated to the pressure recommended by the vehicle manufacturer for the test mass of the vehicle. The tyres shall have at least 1.6 mm tread depth.

When performing indoor testing, tyre/road sound is evaluated independently on the test track with the tyres to be used, according to this paragraph. Propulsion sound is independently evaluated on the dynamometer using tyres and other sound control measures to produce tyre/road sound which does not influence the measurement result.

2.2.3.	Preparation of the vehicle before testing

2.2.3.1.	General

The vehicle shall be equipped as specified by the vehicle manufacturer. Before the measurements are started, the vehicle shall be brought to its normal operating conditions, which means that essential components for the operation of the vehicle are at their nominal temperatures as specified by the manufacturer. This applies especially, but is not limited to

- the cooling water (if applicable);

- oil temperature (if applicable).

2.2.3.2.	Battery state of charge

If so equipped, propulsion batteries shall have a state-of-charge sufficiently high to enable all key functionalities according to the specifications of the vehicle manufacturer. Propulsion batteries shall be within their component temperature window to enable all key functionalities. Any other type of rechargeable energy storage system shall be ready to operate during the test.

2.2.3.3.	Active Sound Systems

Any active sound devices, either for noise control, or sound enhancement, shall operate as foreseen by the vehicle manufacturer and not be interfered with during the measurements.

2.2.3.4.	Tyres

2.2.3.4.1.	Tyre Selection

The tyres and rims to be used for the test shall be representative for the vehicle and shall be selected by the vehicle manufacturer and recorded in Addendum to the Communication form (Annex 1, Appendix 1). They shall correspond to one of the tyre sizes designated for the vehicle as original equipment. The tyre is or will be commercially available on the market at the same time as the vehicle. The tyres shall be inflated to the pressure recommended by the vehicle manufacturer for the test mass of the vehicle. The tyres shall have at least 1.6 mm tread depth.

When performing indoor testing, tyre/road sound is evaluated independently on the test track with the tyres to be used, according to this paragraph. Propulsion sound is independently evaluated on the dynamometer using tyres and other sound control measures to produce tyre/road sound which does not influence the measurement result.

2.2.3.4.2.	Tyre conditioning

Tyres with special fitment requirements, such as asymmetric or directional design, shall also be mounted in accordance with these requirements.

Before testing, tyres shall be conditioned (broken-in). Tyre break-in shall be representative to about 100 km of normal on-road operation. Tyres with special fitment requirements shall be broken-in in accordance with these requirements. The tyres fitted to the test vehicle shall rotate in the same direction as when they were broken-in.

Test tyres shall be warmed-up immediately prior to testing for at least 10min in the range of the test speed, with moderate lateral & longitudinal acceleration. The lateral acceleration shall be selected in a way to avoid excessive tire tread wear effects.

2.2.3.5.	If the vehicle is fitted with more than two-wheel drive, it shall be tested in the drive which is intended for normal road use.

2.2.3.6.	If the vehicle is fitted with fan(s) having an automatic actuating mechanism, this system shall not be interfered with during the measurements.

2.2.3.7.	If the vehicle is equipped with an exhaust system containing fibrous materials, it might be necessary to carry out a conditioning test prior to testing. The provisions of Annex 4, paragraph 1. in conjunction with the flowchart (Figure 2) of the appendix to Annex 4 shall be followed.

2.2.4.1	Suspension Trim Level

If fitted, the trim level of a height adjustable suspension shall be set to its normal level for on-road operation as specified by the vehicle manufacturer.

2.2.7.	Calculation procedure to determine extra loading of N2 and N3 vehicles only

2.2.7.1.	Calculation of extra loading

The target mass, mtarget, (per kW rated power) for two-axle vehicles of category N2 and N3 is specified in the Table in paragraph 2.2.1: above.

mtarget = 50 [kg / kW] × Pn [kW]	(1)

To reach the required target mass, mtarget, for a vehicle being tested, the unladen vehicle, including the mass of the driver, md, shall be loaded with an extra mass, mxload, which shall be placed above the rear axle as given in Formula (8):

mtarget = munladen + md + mxload 	(2)

The target mass, mtarget, shall be achieved with a tolerance of ±5 per cent.

The vehicle mass of the test vehicle in the unladen condition, munladen, is calculated by measuring on a scale the unladen front axle load, mfa load unladen, and the unladen rear axle load, mra load unladen, as given in Formula (3):

munladen = mfa load unladen + mra load unladen 	(3)

By using Formulae (2) and (3), the extra loading, mxload, is calculated as given in Formulae (4) and (5):

mxload = mtarget – (md + munladen) 	(4)

mxload = mtarget − (md + mfa load unladen + mra load unladen) 	(5)

The sum of the extra loading, mxload, and the unladen rear axle load, mra load unladen, is limited to 75 per cent of the technically permissible maximum laden mass for the rear axle, mac ra max, as given in Formula (6):

0,75 mac ra max ≥ mxload + mra load unladen 	(6)

The mxload is limited according to Formula (7):

mxload ≤ 0,75 mac ra max − mra load unladen 	(7)

If the calculated extra loading, mxload, in Formula (5) fulfils Formula (7), then the extra loading is equal to Formula (5). The test mass, mt, of the vehicle is as calculated from Formula (8):

mt = mxload + md + mfa load unladen + mra load unladen 	(8)

In this case, the test mass of the vehicle is equal to the target mass

mt = mtarget	(9)

If the calculated extra loading, mxload, in Formula (5) does not fulfil Formula (7), but rather fulfils Formula (10)

mxload > 0,75 mac ra max − mra load unladen 	(10)

then, the extra loading, mxload, shall be as given by Formula (11):

mxload = 0,75 mac ra max − mra load unladen 	(11)

and the test mass, mt, of the vehicle shall be as given by Formula (12):

mt = 0,75 mac ra max + md + mfa load unladen 	(12)

In this case, the test mass of the vehicle is lower than the target mass

mt < mtarget 	(13)

The test mass, mt, shall be achieved with a tolerance of ± 5 per cent.

2.2.7.2.	Loading considerations if load cannot be aligned with the centre of rear axle

If the centre of gravity of the extra loading, mxload, cannot be aligned with the centre of the rear axle, the test mass of the vehicle, mt, shall not exceed the sum of the unladen front axle load, mfa load unladen, and the unladen rear axle load, mra load unladen, plus the extra loading, mxload, and the mass of the driver, md.

This means that if the actual front and rear axle loads are measured on a scale when the extra loading, mxload, is placed onto the vehicle and it is aligned with the centre of the rear axle, the test mass of the vehicle minus the mass of the driver is as given by Formula (14):

mt − md = mfa load laden + mra load laden 	(14)

Where:

mfa load laden = mfa load unladen	(15)

If the centre of gravity of the extra loading cannot be aligned with the centre of the rear axle, Formula (14) is still fulfilled, but

mfa load laden > mfa load unladen 	(16)

because the extra loading has partly distributed its mass to the front axle. In that case, it is not allowed to add more mass onto the rear axle to compensate for the mass moved to the front axle.

2.2.7.3.	Test mass for vehicles with more than two axles

If a vehicle with more than two axles is tested, then the test mass of this vehicle shall be the same as the test mass for the two-axle vehicle.

If the unladen vehicle mass of a vehicle with more than two axles is greater than the test mass for the two-axle vehicle, then this vehicle shall be tested without extra loading.

2.2.7.4. 	Calculation of the test mass of a virtual vehicle with two axles:

When a vehicle family is not represented by a two-axle vehicle because it is physically not available, the vehicle family can be represented by a vehicle with more than two axles (vrf). In that case the test mass of a virtual two-axle vehicle (mt (2 axles virtual)) can be calculated in the following way:

For the calculation of the unladen vehicle mass of the virtual two-axle vehicle (munladen (2 axles virtual)), take from the vehicle with more than two axles (vrf) the measured unladen front axle load (mfa (vrf) load unladen) and the measured unladen rear axle load of that driven rear axle (mra (vrf) load unladen) which has the highest unladen load.

If the vehicle (vrf) has more than one front axle, take the one with the highest unladen front axle load.

munladen (2 axles virtual) = mfa (vrf) load unladen + mra (vrf) load unladen

mxload (2 axles virtual) = mtarget − (md + munladen (2 axles virtual))

Due to the requirement that the sum of the extra loading (mxload (2 axles virtual)) and the unladen rear axle load, mra (vrf) load unladen, is limited to 75 per cent of the technically permissible maximum laden mass allowed for the rear axle, mac ra max (2 axles virtual),  this value, mac ra max (2 axles virtual), has to be chosen in such a way that it represents the rear axle of the forecasted highest production-volume in the manufacturer's variation with a technically permissible maximum laden mass allowed for the rear axle (mac ra max (chosen) ) for the vehicle family as declared by the manufacturer.

mac ra max (4x2 virtual) = mac ra max (chosen)

If mxload (2 axles virtual) ≤ 0,75 mac ra max (chosen) − mra (vrf) load unladen

then

mt (2 axles virtual) = mxload (2 axles virtual) + md + mfa (vrf) load unladen + mra (vrf) load unladen

and

mt (2 axles virtual) = mtarget

If mxload (2 axles virtual) > 0,75 mac ra max (chosen) − mra (vrf) load unladen

then

mt (2 axles virtual) = 0,75 mac ra max (chosen) + md + mfa (vrf) load unladen

and

mt (2 axles virtual) < mtarget

The test mass of the vehicle with more than two axles representing the vehicle family is defined as followed:

mt (vrf) = mt (2 axles virtual)

and the extra loading is calculated as

mxload (vrf) = mt (2 axles virtual) – md  – munladen (vrf)

II. Justification:

General:	The chapter for vehicle selection and preparation has been completely restructured for more clarity.

2.2.2.	Addition of new paragraph heading to clarify intent and organization of subsections.

2.2.2.1	Clarification of where to find the specifications on mass.

2.2,2,2	Relocation of specific section for target mass to clarify use and importance of this term for N2 and N3 vehicles for better structure.

2.2.2.3.	Addition of new paragraph heading to clarify intent and organization of extra loading and test mass sections.  The restructuring of paragraph numbering is designed to assist users of the regulation to come to a correct load and test mass result.

2.2.2.3.1	Addition of new paragraph heading to clarify intent and organization of extra loading sections.  This subsection contains all of the calculations necessary for the general loading calculation and is no longer mixed with other loading or mass provisions.

2.2.2.3.2	Addition of new paragraph heading to clarify intent and organization of extra loading sections.  This subsection contains all of the calculations necessary for the loading calculation for the case, where load cannot be aligned with the rear axle.  These calculations are no longer mixed with other loading or mass provisions.

2.2.2.3.3	Addition of new paragraph heading to clarify intent and organization of test mass for vehicles with more than two axles.  These specifications are no longer mixed with other loading or mass provisions.

2.2.2.3.4	Addition of new paragraph heading to clarify intent and organization of test mass for “virtual” vehicles with two axles.  These specifications are no longer mixed with other loading or mass provisions.

2.2.2.4	Addition of new paragraph heading to clarify intent and use of an incomplete vehicle.  These specifications are no longer mixed with other loading or mass provisions.

2.2.3	Addition of new paragraph heading to clarify intent and procedures for vehicle preparation.  Specific sub-paragraphs are added for each vehicle preparation action.

2.2.3.1	Addition of new paragraph and wording to clarify intent and procedures for general vehicle preparation.  Explanation of “normal operating conditions” provided to assist the user of this regulation correctly “warm up” the vehicle power train.

2.2.3.2	Addition of new paragraph to clarify intent and procedures for battery state of charge.  Battery state of charge is a key vehicle parameter for electric or hybrid electric vehicles to enable correct vehicle operation. These provisions are the same language as in UN R138, ISO 362-1, and ISO 16254.

2.2.3.3	Addition of new paragraph to include any sort of exterior sound generation system as part of the test conditions for this regulation. These provisions prevent the condition where the test result according to this regulation may not be representative of on road and in-use vehicle operation.

2.2.3.4	Addition of new paragraph heading to clarify and organize all provisions relating to tyres.

2.2.3.4.1	Addition of new paragraph to clarify provisions relating to tyre selection.  Rims (wheels) are added to indicate both parts of a tyre/wheel assembly work together to provide the correct result.

2.2.3.4.2	Addition of new paragraph to clarify provisions relating to tyre conditioning.  Proper tyre conditioning is necessary to achieve representative, repeatable and reproducible test results.

2.2.3.5	Addition of new paragraph to clarify provisions relating to drive mode when a vehicle has more than two-wheel drive.  Drive mode provisions are no longer mixed with other vehicle preparation provisions

2.2.3.6	Addition of new paragraph to clarify provisions relating to automatic cooling fans.  Cooling fan provisions are no longer mixed with other vehicle preparation provisions.

2.2.3.7	Addition of new paragraph to clarify provisions relating to fibrous materiel in exhaust systems.  Fibrous materiel provisions are no longer mixed with other vehicle preparation provisions

Given that the tyre contribution for overall sound emission is significant, regard shall be given for existing regulatory provisions concerning tyre/road sound emissions. Traction tyres, snow tyres and special-use tyres as defined in paragraph 2. of Regulation No. 117 shall be excluded during type-approval and conformity of production measurements at the request of the manufacturer in accordance with Regulation No. 117.