|  | E/ECE/324/Rev.1/Add.50/Rev.4−E/ECE/TRANS/505/Rev.1/Add.50/Rev.4 |  |
| --- | --- | --- |
|  |  | 25 May 2021 |

- yellow, still to be decided including new subjects

Agreed from discussions in Drafting group and highlighted in

- Green on April 12, 2021

- Blue on June 07, 2021.

Agreement

Concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be  Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions*

(Revision 3, including the amendments which entered into force on 20 January 2016)

_________

Addendum XX – Regulation No. 51

Revision 5

Covering all series including the 04 series of amendments:

04 series to the original version of the Regulation – Date of entry into force: xxxxx

Uniform provisions concerning the approval of motor vehicles having at least four wheels with regard to their sound emissions

_________

UNITED NATIONS

UN Regulation No. 51

Uniform provisions concerning the approval of motor vehicles having at least four wheels with regard to their sound emissions

Contents

Page

Regulation

1.		Scope		  4

2.		Definitions		  4

3.		Application for approval		14

4.		Markings		14

5.		Approval		15

6.		Specifications		16

7.		Modification and extension of approval of a vehicle type		19

8.		Conformity of production		19

9.		Penalties for non-conformity of production		19

10.	Production definitively discontinued		20

11.	Transitional provisions		20

12.	Names and addresses of Technical Services   responsible for conducting approval tests and of
Type Approval Authorities		22

Annexes

1		Communication		23

Appendix 1 - Addendum to the communication form No …		25

Appendix 2 - Technical Information Document		26

2		Arrangements of the approval mark		30

3		Methods and instruments for measuring the sound made by motor vehicles		31

Appendix		50

4		Exhaust silencing systems containing acoustically absorbing fibrous materials		63

Appendix		65

5		Compressed air noise		66

Appendix		67

6		Checks on conformity of production		68

7		Measuring method to evaluate compliance with the Additional Sound Emission Provisions		84

Appendix 1  Sound expectation model …………………………………………………….	..             92

Appendix 2 - Statement of Compliance with the Additional Sound Emission Provisions 	 100

Appendix 3 - Test report sheet …………………………………………………………….     101

Appendix 4 - Flowcharts 										103

8	Indoor testing	 											109

1.		Scope

This Regulation contains provisions on the sound emitted by motor vehicles and applies to vehicles of categories M and N.

The specifications in this Regulation are intended to reproduce the sound levels which are generated by vehicles during normal driving in urban traffic.

This regulation provides, as well, Real Driving Additional Sound Emission Provisions (RD-ASEP) for vehicles of categories M1 and N1 covering driving conditions outside of normal driving with higher accelerations and vehicle speeds up to 100 km/h.

.

2.		Definitions

For the purpose of this Regulation,

2.1.	"Approval of a vehicle" means the approval of a vehicle type with regard to exterior sound;

2.2.	"Vehicle type" means a category of motor vehicles which do not differ in such essential respects as:

2.2.1.	For vehicles tested according to Annex 3, paragraph 3.1.2.1.:

2.2.1.1.	The shape or materials of the engine compartment and its soundproofing;

2.2.1.2.	The type of engine (positive or compression ignition, two- or four-stroke, reciprocating or rotary piston), number and capacity of cylinders, number and type of carburettors or injection system, arrangement of valves, or the number and the type of electric motor(s);

2.2.1.3.	Rated maximum net power and corresponding rated engine speed(s); however if the rated maximum net power and the corresponding rated engine speed differs only due to different engine mappings, these vehicles may be regarded as from the same type;

2.2.1.4.	The silencing system.

2.2.2.	For vehicles tested according to Annex 3, paragraph 3.1.2.2.:

2.2.2.1.	The shape or materials of the engine compartment and its soundproofing;

2.2.2.2.	The type of engine (positive or compression ignition, two- or four-stroke, reciprocating or rotary piston), number and capacity of cylinders, type of injection system, arrangement of valves, rated engine speed (S), or the type of electric motor;

2.2.2.3.	Vehicles having the same type of engine and/or different overall gear ratios may be regarded as vehicles of the same type;

2.3.	However, if the differences in paragraph 2.2.2. provide for different target conditions, as described in paragraph 3.1.2.2. of Annex 3, these differences are considered as a change of type;

2.4.	"Mass of a vehicle in running order (mRO)"  means

(a) 	In the case of a motor vehicle:

The mass of the unladen vehicle, with its fuel tank(s) filled to at least 90 per cent of its or their capacity/ies, including the mass of the driver, of 100% liquids, fitted with the standard equipment in accordance with the manufacturer's specifications and, when they are fitted, the mass of the bodywork, the cabin, the coupling and the spare wheel(s) as well as the tools;

(b) 	In the case of a towing vehicle designed to be coupled to a trailer:

The mass of the vehicle including the fuel and liquids, fitted with the standard equipment in accordance with the manufacturer's specifications, and, when they are fitted, the mass of the bodywork, additional coupling(s), the spare wheel(s) and the tools.

2.5.	"Technically permissible maximum laden mass (M)"  means the maximum mass allocated to a vehicle on the basis of its construction features and its design performances; the technically permissible laden mass of a trailer or of a semi-trailer includes the static vertical mass transferred to the towing vehicle when coupled;

2.6.	"Vehicle length" means a dimension which is measured according to paragraphs 6.1.1., 6.1.2, and 6.1.3. of ISO standard 612-1978. In addition to the provisions of that standard, when measuring the vehicle structural length the following devices shall not be taken into account:

(a)	Wiper and washer devices;

(b)	Front or rear marker-plates;

(c)	Customs sealing devices and their protection;

(d)	Devices for securing the tarpaulin and their protection;

(e)	Lighting equipment;

(f)	Device for indirect vision;

(g)	Rear space watching aids;

(h)	Air-intake pipes;

(i)	Length stops for demountable bodies;

(j)	Access steps;

(k)	Ram rubbers;

(l)	Lifting platforms, access ramps and similar equipment in running order, not exceeding 300 mm, provided that the loading capacity of the vehicle is not increased;

(m)	Coupling devices for motor vehicles.

2.7	"Vehicle width" means a dimension which is measured according to ISO standard 612-1978, term No. 6.2. In addition to the provisions of that standard, when measuring the vehicle structural width the following devices shall not be taken into account:

(a)	Customs sealing devices and their protection;

(b)	Devices for securing the tarpaulin and their protection;

(c)	Tyre failure tell-tale devices;

(d)	Protruding flexible parts of a spray-suppression system;

(e)	Lighting equipment.

2.8.

"Maximum net power, Pn" means the engine power available for propulsion expressed in kW and measured dependent on the drive train concepts.

Applicable power sources are those, which provide drive power for forward motion to the vehicle.

2.8.1.	For vehicles with combustion engine(s) only

The maximum engine power is the net power Pn of the combustion engine(s) measured at full engine load pursuant to UN Regulation No. 85 paragraph 5.2.

2.8.2.	For pure electric vehicles that have only one propulsion energy converter

The net power Pn of the electric motor of the electric drive train is determined pursuant to UN Regulation No. 85 paragraph 5.3.

2.8.3.	For hybrid electric vehicles [where at least one electric machine contributes to propulsion], or pure electric vehicles that have more than one propulsion energy converter

The maximum engine power is the "vehicle system power rating" according to GTR 21, paragraph 6.9.1 (b) “sustained vehicle system power”.

2.8.4.	For fuel cell electric vehicles

The maximum engine power is the net power Pn of the combustion engine(s) measured at full engine load pursuant to UN Regulation No. 85 paragraph 5.2.

2.9.	"Rated engine speed, S" means the declared engine speed in min-1 (rpm) at which the engine develops its rated maximum net power pursuant to UN Regulation No. 85 or, where the rated maximum net power is reached at several engine speeds, the highest one of those speeds.

2.10.	"Power to Mass Ratio index (PMR)" means a numerical quantity (see Annex 3, paragraph 3.1.2.1.1.) with no dimension used for the calculation of acceleration.

2.11.	"Reference point" means one of the following points:

2.11.1.	In the case of vehicles of categories M1, N1 and M2 < 3,500 kg technically permissible maximum laden mass:

(a)	For front engine vehicles: the front end of the vehicle;

(b)	For mid-engine vehicles: the centre of the vehicle;

(c)	For rear engine vehicles: the rear end of the vehicle.

For vehicles having multiple propulsion sources, the reference point is determined by the position of the propulsion source having the highest power. If there are multiple propulsion sources of equivalent power, then the position of the most forward propulsion source shall prevail.

2.11.2.	In the case of vehicles of categories M2 > 3,500kg technical permissible maximum laden mass, M3, N2, N3:

(a)	For front-engine vehicles, the front end of the vehicle;

(b)	For all other vehicles, the border of the engine closest to the front of the vehicle;

2.12.	"Engine" means the power source without detachable accessories.

Power source includes in this context all sources of motive power; for example, electric or hydraulic power sources used alone or in combination with other power sources.

2.13.	"Target acceleration" means acceleration at a partial throttle condition in urban traffic and is derived from statistical investigations.

2.14.	"Reference acceleration" means the required acceleration during the acceleration test on the test track.

2.15.	"Gear ratio weighting factor k" means a dimensionless numerical quantity used to combine the test results of two gear ratios for the acceleration test and the constant speed test.

2.16.	"Partial power factor kP" means a numerical quantity with no dimension used for the weighted combination of the test results of the acceleration test and the constant speed test for vehicles.

2.17.	"Pre-acceleration" means application of acceleration control device prior to AA' for the purpose of achieving stable acceleration between AA' and BB' as referred to in Figure 1 of Appendix 1 to Annex 3.

2.18.	Gear

2.18.1.		"Gear ratios"

2.18.1.1.	“Internal gearbox ratio” means the ratios of engine to gearbox output shaft revolutions.

2.18.1.2.	“Final drive ratio” means the ratio(s) of gearbox output shaft to driven wheel revolutions.

2.18.1.3.	“Total gear ratio” means the ratios between vehicle speed and engine speed during the passage of the vehicle through the test track.

2.18.1.4.	“Gear ratio” used in context with vehicles tested according to 3.1.2.1. of Annex 3 and Annex 7 is the total gear ratio as defined in 2.18.1.3. above (especially in junction with CVTs where any gear ratio may be available).

2.18.2.	"Locked gear ratio" means the control of the transmission such that the gear will not change during a test.

2.18.3.	"Gear" means in the context of this Regulation a discrete gear ratio either selectable by the driver or by an external device.

2.18.4.	For vehicles tested according to paragraph 3.1.2.1. of Annex 3 and Annex 7, "geari" and "geari+1" are defined as two gears in sequence, where geari either provides an acceleration within the 5 per cent tolerance according to paragraph 3.1.2.1.4.1. (a) of Annex 3 or an acceleration greater than the reference acceleration, and geari+1 an acceleration lower than the reference acceleration according to paragraph 3.1.2.1.4.1. (b) or (c) of Annex 3.

2.19.	"Silencing system" means a complete set of components necessary for limiting the sound produced by an engine, its intake and its exhaust (the exhaust manifold(s), the catalyst(s) and emission after-treatment device(s) are not considered part of the silencing system; these parts belong to the engine).

2.20.	"Design family of exhaust silencing system or exhaust silencing system components" means a group of silencing systems or components thereof in which all of the following characteristics are the same:

(a)	The presence of net gas flow of the exhaust gases through the absorbing fibrous material when in contact with that material;

(b)	The type of the fibres;

(c)	Where applicable, binder material specifications;

(d)	Average fibre dimensions;

(e)	Minimum bulk material packing density in kg/m³;

(f)	Maximum contact surface between the gas flow and the absorbing material.

2.21.	"Exhaust silencing system of different types" means silencing systems which significantly differ in respect of at least one of the following:

(a)	Trade names or trademarks of their components;

(b)	The characteristics of the materials constituting their components, except for the coating of those components;

(c)	The shape or size of their components;

(d)	The operating principles of at least one of their components;

(e)	The assembly of their components;

(f)	The number of exhaust silencing systems or components.

2.22.	"Replacement silencing system" means any part of the silencing system or its components intended for use on a vehicle, other than a part of the type fitted to this vehicle when submitted for type-approval pursuant to this Regulation.

2.23.	"R-point" means R-point as defined in paragraph 2.4. of Annex 1 to the Consolidated Resolution on the Construction of Vehicles (R.E.3.).

2.24.	Table of symbols

2.24.1.	Table of symbols for Annex 3

Symbols listed under the paragraphs 2.2.1 up to 3.1.1 are general symbols used for both test methods described under paragraphs 3.1.2.1 and 3.1.2.2.

Symbols listed under the paragraphs 3.1.2.1 are used for the test method of vehicles of category M1, N1 and M2 ≤ 3.500 kg technically permissible maximum laden mass.

Symbols listed under the paragraphs 3.1.2.2 are used for the test method of vehicles of category M2 > 3,500 kg technically permissible maximum laden mass, M3, N2, N3.

| Symbol | Unit | Paragraph | Explanation |
| --- | --- | --- | --- |
| mRO | kg | 2.2.1. | mass in running order; value to be reported and used for calculations to a precision of 10 kg |
| mT | kg | 2.2.1. | test mass of the vehicle; value to be reported and used for calculations to a precision of 10 kg |
| mTARGET | kg | 2.2.1. | target mass of the vehicle |
| mXLOAD | kg | 2.2.1. | extra loading |
| mFA LOAD UNLADEN | kg | 2.2.1. | front axle load in unladen condition |
| mRA LOAD UNLADEN | kg | 2.2.1. | rear axle load in unladen condition |
| mUNLADEN | kg | 2.2.1. | unladen vehicle mass |
| mAC RA MAX | kg | 2.2.1. | Technically permissible maximum laden mass allowed for the rear axle as declared by the manufacturer |
| mD | kg | 2.2.1. | mass of driver |
| mCHASSIS M2M3 | kg | 2.2.1. | mass of the incomplete vehicle (M2 or M3) |
| mXLOAD M2M3 | kg | 2.2.1. | extra load to be added to the incomplete vehicle (M2 or M3) to reach the mass of the vehicle in running order as chosen by the manufacturer |
| mFA LOAD LADEN | kg | 2.2.7.2. | front axle load in laden condition |
| mRA LOAD LADEN | kg | 2.2.7.2. | rear axle load in laden condition |
| mT (2 axles virtual) | kg | 2.2.7.4. | test mass of a virtual vehicle with two axles (4x2 or 4x4) |
| vRF | — | 2.2.7.4. | vehicle with more than two axles representing the vehicle family |
| mUNLADEN (2 axles virtual) | kg | 2.2.7.4. | unladen vehicle mass of the virtual vehicle with two axles |
| mXLOAD (2 axles virtual) | kg | 2.2.7.4. | extra loading for the virtual vehicle with two axles |
| mAC RA MAX (chosen) | kg | 2.2.7.4. | Technically permissible maximum laden mass allowed for the chosen rear axle as defined in paragraph 2.2.7.4. in Annex 3 |
| AA' | — | 3.1.1. | line perpendicular to vehicle travel which indicates beginning of zone in which to record sound pressure level during test |
| BB' | — | 3.1.1. | line perpendicular to vehicle travel which is 10.00 m behind line PP' |
| CC' | — | 3.1.1. | line of vehicle travel through test surface defined in ISO 10844 |
| PP' | — | 3.1.1. | line perpendicular to vehicle travel which indicates location of microphones |
| vTEST | km/h | 3.1.2.1. | vehicle test speed |
| PMR | — | 3.1.2.1.1. | power-to-mass ratio index to be used for calculations; value to be reported and used for calculations to the first decimal place |
| PN | kW | 3.1.2.1.1. | rated total engine net power |
| l | m | 3.1.2.1.2. | reference length; value to be reported and used for calculations to a precision of 0.01 m (1 cm) |
| lVEH | m | 3.1.2.1.2. | length of vehicle; value to be reported and used for calculations to a precision of 0.01 m (1 cm) |
| vAA' | km/h | 3.1.2.1.2. | vehicle velocity when the reference point passes line AA' (see paragraph 2.11. for definition of reference point); value to be reported and used for calculations to the first decimal place |
| vBB' | km/h | 3.1.2.1.2. | vehicle velocity when the reference point or rear of vehicle passes line BB' (see paragraph 2.11. for definition of reference point); value to be reported and used for calculations to the first decimal place |
| vPP' | km/h | 3.1.2.1.2. | vehicle velocity when the reference point passes line PP' (see paragraph 2.11. for definition of reference point); value to be reported and used for calculations to the first decimal place |
| aACC TEST | m/s2 | 3.1.2.1.2.1. | acceleration from AA' to BB'; value to be reported and used for calculations to the second decimal place |
| aACC TEST i | m/s2 | 3.1.2.1.2.1. | acceleration achieved in a particular gear i; value to be reported and used for calculations to the second decimal place |
| lPA | m | 3.1.2.1.2.1. | point of depressing the accelerator before line AA'; value to be reported in full meter |
| aACC TEST PP’-BB’ | m/s2 | 3.1.2.1.2.2. | acceleration from PP' to BB'; value to be reported and used for calculations to the second decimal place |
| aURBAN | m/s2 | 3.1.2.1.2.3. | target acceleration representing urban traffic acceleration; value to be reported and used for calculations to the second decimal place |
| aACC REF | m/s2 | 3.1.2.1.2.4. | reference acceleration for the acceleration test; value to be reported and used for calculations to the second decimal place |
| kP | — | 3.1.2.1.3. | partial power factor; value to be reported and used for calculations to the second decimal place |
| aACC i | m/s2 | 3.1.2.1.4.1. | acceleration in gear ratio i; value to be reported and used for calculations to the second decimal place |
| aACC (i + 1) | m/s2 | 3.1.2.1.4.1. | acceleration in gear ratio (i + 1); value to be reported and used for calculations to the second decimal place |
| gear ratio i | — | 3.1.2.1.4.1. | gear ratio which provides an acceleration within the 5 per cent tolerance of the reference acceleration aACC REF or greater than the reference acceleration aACC REF |
| gear ratio i+1 | — | 3.1.2.1.4.1. | second of two gear ratios, with an acceleration lower than gear ratio i |
| gear ratio i+2, i+3, … | — | 3.1.2.1.4.1. | gear ratios selectable for the pass-by test of Annex 3, if gear ratio i and gear ratio i+1 exceed an acceleration of 2.0 m/s² |
| k | — | 3.1.2.1.4.1. | gear ratio weighting factor; value to be reported and used for calculations to the second decimal place |
| nMAX | 1/min | 3.1.2.1.4.1 | Maximum engine rotational speed permitted for M1, N1, and M2 less than 3500 kg; value to be reported and used for calculations to a precision of 10 min-1 (xxx0) |
| nBB' | 1/min | 3.1.2.2. | engine rotational speed of the vehicle, when the reference point passes BB'; value to be reported and used for calculations to a precision of 10 min-1 |
| S | 1/min | 3.1.2.2. | rated engine rotational speed in revs per minute, synonymous with the engine rotational speed at maximum power |
| nTARGET BB' | 1/min | 3.1.2.2.1.1.(a) | target engine rotational speed of the vehicle when the reference point has to pass line BB' (see 2.11.2. for definition of reference point) |
| vTARGET BB' | km/h | 3.1.2.2.1.1.(a) | target vehicle velocity when it is necessary that the reference point has to pass line BB' (see 2.11.2. for definition of reference point) |
| vBB' GEAR i | km/h | 3.1.2.2.1.1.(b) | target vehicle velocity when certain conditions are met |
| vBB' GEAR i, i= 1,2 | km/h | 3.1.2.2.1.1. (c) | target vehicle velocity when certain conditions are met |
| gearX | - | 3.1.2.2.1.1. (d) | first of two gear ratios used for testing of M2 having a maximum authorized mass of more than 3 500 kg, M3, N2, and N3 where certain criteria on test conditions are met |
| gearY | - | 3.1.2.2.1.1. (d) | second of two gear ratios used for testing of M2 having a maximum authorized mass of more than 3,500 kg, M3, N2, and N3 where certain criteria on test conditions are met |
| vBB'X | km/h | 3.1.2.2.1.1. (d) | target vehicle velocity when certain conditions are met |
| vBB'Y | km/h | 3.1.2.2.1.1. (d) | target vehicle velocity when certain conditions are met |
| vBB'1 | km/h | 3.1.2.2.1.2. (b) | target vehicle velocity when certain conditions are met |
| vBB'2 | km/h | 3.1.2.2.1.2. (b) | target vehicle velocity when certain conditions are met |
| nBB'i, i=1,2 | 1/min | 3.1.2.2.1.2. (d) | engine rotational speed when the reference point passes BB' when certain conditions are met |
| LCRS i | dB(A) | 3.1.3.1. | vehicle sound pressure level at constant speed test for gear i; value to be reported and used for calculations to the first decimal place |
| LCRS (i + 1) | dB(A) | 3.1.3.1. | vehicle sound pressure level at constant speed test for gear (i + 1); value to be reported and used for calculations to the first decimal place |
| LCRS REP | dB(A) | 3.1.3.1. | reported vehicle sound pressure level at constant speed test; value to be reported and used for calculations to the first decimal place |
| LACC (i) | dB(A) | 3.1.3.1. | vehicle sound pressure level for the acceleration test for gear i; value to be reported and used for calculations to the first decimal place |
| LACC (i + 1) | dB(A) | 3.1.3.1. | vehicle sound pressure level for the acceleration test for gear (i + 1); value to be reported and used for calculations to the first decimal place |
| LACC REP | dB(A) | 3.1.3.1. | reported vehicle sound pressure level for the acceleration; value to be reported and used for calculations to the first decimal place |
| LURBAN | dB(A) | 3.1.3.1. | reported vehicle sound pressure level representing urban operation; value to be reported mathematically rounded to the nearest integer |

2.24.2.		Table of symbols for Annex 7

| aACC ASEP | m/s2 | 2.3. | maximum required acceleration |
| --- | --- | --- | --- |
| κ | — | 2.3. | gears to be tested under "Additional Sound Emission Provisions" (ASEP) |
| nBB’ ASEP | 1/min | 2.3. | maximum test engine speed; value to be reported and used for calculations to a precision of 10 min-1 |
| vAA’ ASEP | km/h | 2.3. | target vehicle velocity for test point P1 of the assessment method according paragraph 2.4 |
| vBB’ ASEP | km/h | 2.3. | target vehicle velocity for test point P4 of the assessment method according paragraph 2.4 |
| Pj | — | 2.4. | test point(s) under ASEP |
| j | — | 2.4. | index for the test points under ASEP |
| vBB’ j | km/h | 2.4. | vehicle test speed at BB' for a particular ASEP test point |
| aACC TEST κj | m/s2 | 2.5. | acceleration achieved in gear κ and at test point j |
| LACC κj | dB(A) | 2.5. | sound pressure level measured for a gear κ and at a test point j; value to be reported and used for calculations to the first decimal place |
| nBB’ κj | 1/min | 2.5. | vehicle test engine speed at BB' for a gear κ and at test point j |
| vAA’ κj | km/h | 2.5. | vehicle test speed at AA' for a gear κ and at test point j; value to be reported and used for calculations to the first decimal place |
| vBB’ κj | km/h | 2.5. | vehicle test speed at BB' for a gear κ and at test point j; value to be reported and used for calculations to the first decimal place |
| vPP’ κj | km/h | 2.5. | vehicle test speed at PP' for a gear κ and at test point j; value to be reported and used for calculations to the first decimal place |
| LANCHOR | dB(A) | 3.1. | reported vehicle sound pressure level for gear ratio i from Annex 3; value to be reported and used for calculations to the first decimal place |
| nANCHOR κ | 1/min | 3.1. | reported vehicle engine speed for gear ratio i from Annex 3 |
| vANCHOR κ | km/h | 3.1. | reported vehicle test speed for gear ratio i at BB’ from Annex 3; value to be reported and used for calculations to the first decimal place |
| Lκj | dB(A) | 3.5. | sound pressure level measured for a gear κ and at a test point j; value to be reported and used for calculations to the first decimal place |
| kP ASEP | — | 4.2.1. | partial power factor determined for the LURBAN principle of ASEP |
| LACC ASEP | dB(A) | 4.2.1. | vehicle sound pressure level measured for the LURBAN principle of ASEP; value to be reported and used for calculations to the first decimal place |
| LURBAN ASEP | dB(A) | 4.2.1. | Estimated urban sound pressure level determined for the LURBAN principle of ASEP; value to be reported and used for calculations to the first decimal place |
| LURBAN MEASURED ASEP | dB(A) | 4.2.1. | interim result for calculation of ΔLURBAN ASEP; value to be reported and used for calculations to the first decimal place |
| LURBAN NORMALIZED | dB(A) | 4.2.1. | interim result for calculation of ΔLURBAN ASEP; value to be reported and used for calculations to the first decimal place |
| ΔLURBAN ASEP | dB(A) | 4.2.1. | estimated deviation from urban sound pressure level; value to be reported to the first decimal place |
| α | __ | 5.2 | gear to be determined for the reference sound assessment according to the type of transmission |
| LREF | dB(A) | 5.3. | reference sound pressure level for reference sound assessment; value to be reported and used for calculations to the first decimal place |
| nBB’ REF | 1/min | 5.3. | Reference vehicle test engine speed for reference sound assessment |
| vBB’ REF | km/h | 5.3. | reference vehicle test speed for reference sound assessment |

| Symbol | Unit | Paragraph | Explanation |
| --- | --- | --- | --- |
| aMAX REF | m/s2 | 3.4. | maximum reference acceleration as determined in a low gear under full load condition. |
| LTEST | dB(A) | 3.5.3. | sound pressure level measured for any target operation condition; value to be reported and used for calculations to the first decimal place |
| vAA’ TEST | km/h | 3.5.3. | vehicle speed measured for target operation condition when the reference point passes line AA'; value to be reported and used for calculations to the first decimal place |
| vPP’ TEST | km/h | 3.5.3. | vehicle speed measured for target operation condition when the reference point passes line PP; value to be reported and used for calculations to the first decimal place |
| vBB’ TEST | km/h | 3.5.3. | vehicle speed measured for target operation condition when the rear end of the vehicle passes line BB'; value to be reported and used for calculations to the first decimal place |
| nAA’ TEST | 1/min | 3.5.3. | engine speed measured for target operation condition when the reference point of the vehicle passes line AA'; value to be reported and used for calculations to a precision of 10 min-1 |
| nBB’ TEST | 1/min | 3.5.3. | engine speed measured for target operation condition when the rear end of the vehicle passes line BB'; value to be reported and used for calculations to a precision of 10 min-1 |
| aTEST | m/s2 | 3.5.4.1. | acceleration from PP' to BB'; value to be reported and used for calculations to the second decimal place |
| va | m²/s³ | 3.5.4.2 | performance calculated from the reported vehicle speed at line BB’ in meters per second and the acceleration result from paragraph 3.5.4.1. and rounded to the first digit after the decimal place. |
| LEXP | dB(A) | 3.5.4.3. | the expected sound pressure level |
| LTEST EXP | dB(A) | 4.1. | calculated value of the expected sound pressure level; value to be reported and used for calculations to the first decimal place |
| Annex 7-Appendix 1 |  |  |  |
| LACC ANCHOR | dB(A) | 2.2.1.2. | vehicle sound pressure level for the acceleration test to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to the first decimal place |
| vBB’ ACC ANCHOR | km/h | 2.2.1.2. | vehicle speed value when the rear of the vehicle passes line BB' for the acceleration test to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to the first decimal place |
| nBB’ACC ANCHOR | 1/min | 2.2.1.2. | engine speed value when the rear of the vehicle passes line BB' for the acceleration test to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to a precision of 10 min-1 |
| LCRS ANCHOR | dB(A) | 2.2.1.2. | vehicle sound pressure level at constant speed test; value to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to the first decimal place |
| vBB’CRS ANCHOR | km/h | 2.2.1.2. | vehicle speed value when the rear of the vehicle passes line BB' for the constant speed test to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to the first decimal place |
| nBB’ CRS ANCHOR | 1/min | 2.2.1.2. | engine speed value when the rear of the vehicle passes line BB' for the constant speed test to be reported from Annex3 with the tested gear in single-gear test or the lower tested gear in two-gear test and used for calculations to a precision of 10 min-1 |
| x | - | 2.3.1. | energy contribution ratio of tyre/rolling sound level at LCRS ANCHOR |
| LREF TR | dB(A) | 2.3.1. | calculated Reference Tyre Rolling Sound Level |
| LREF PT | dB(A) | 2.5. | calculated Reference Power Train Mechanics Sound Level |
| LREF DYN | dB(A) | 2.6. | calculated Reference Dynamic Sound Level |
| ∆LDYN | dB(A) | 2.7. | Vehicle Dynamic Delta Sound Level |
| LREF TR ADJ | dB(A) | 2.7. | Tyre Rolling Sound Level with adjusted vehicle speed for ∆LDYN calculation |
| LREF PT ADJ | dB(A) | 2.7. | Power Train Mechanical Sound Level with adjusted engine speed for ∆LDYN calculation |
| TR LO | dB(A) | 2.7. | sound slope for tyre rolling sound when vehicle speed is not greater than the reference speed |
| PT LO | dB(A) | 2.7. | sound slope for Power Train Sound when vehicle engine speed is not greater than nBB’ CRS’ ANCHOR |
| nSHIFT PT | 1/min | 2.7. | constant (shifting factor) for the regression curve of the mechanic sound model |
| κiTEST | Km/h/(1000 1/min) | 3.2.1. | vehicle speed to engine speed ratio under "Real Driving Additional Sound Emission Provisions" (RD-ASEP) as expressed in km/h per 1000 min-1 during a test run, rounded to the second decimal |
| κiREF | km/h/(1000 1/min) | 3.2.1. | Reference vehicle speed to engine speed ratio t defined in paragraph 3.6.1. |
| LCRS ANCHOR’ | dB(A) | 3.2.3.2.2. | corrected cruise test result for HEV |
| LACC ANCHOR’ | dB(A) | 3.2.3.3.2. | corrected acceleration test result for HEV |
| Limit | dB(A) | 3.2.3.3.2. | sound level limits in the table of paragraph 6.2.2. of main body to this Regulation for the valid phase |
| LTR EXP | dB(A) | 3.3. | expected tyre rolling sound |
| TR HI | dB(A) | 3.3. | sound slope for tyre rolling sound when vehicle speed is greater than the reference speed |
| LPT EXP | dB(A) | 3.4. | expected power train mechanics sound |
| PT HI | dB(A) | 3.4. | sound slope for Power Train Mechanics Sound when vehicle engine speed is exceeding nBB’ CRS’ ANCHOR |
| LDYN EXP | dB(A) | 3.5. | expected base dynamic sound |
| DYN LO | dB(A) | 3.5. | sound slope for Dynamic Sound when vehicle engine speed is not greater than nBB’ ACC’ ANCHOR |
| DYN HI | dB(A) | 3.5. | sound slope for Dynamic Sound when vehicle engine speed is exceeding nBB’ ACC’ ANCHOR |
| ∆LDYN EXP | dB(A) | 3.6. | expected dynamic delta sound |
| LOADTEST |  | 3.6.2. | load achieved during the test run |
| aTEST | m/s2 | 3.6.2. | acceleration achieved during the test run |
| aMAX κ | m/s2 | 3.6.2. | Maximum acceleration in gear κ |
| vaTEST | m²/s³ | 3.6.3.1. | dynamic performance achieved during the test |
| ∆LDYN va |  | 3.6.3.2. | dynamic performance component calculated based on the achieved performance vaTEST relative to a reference performance. |
| β |  | 3.6.3.2. | coefficient for calculating dynamic performance component |
| vaREF | m²/s³ | 3.6.3.2. | reference performance |
| α |  | 3.6.3.3. | coefficient for calculating dynamic sound components |
| nMAX TORQUE | 1/min | Appendix 4 | engine speed when the engine is at the maximum torque |

2.25.	Modes

2.25.1.	"Mode" means a distinct driver-selectable condition which affects the sound emission of the vehicle.

2.26.	Stable acceleration

For the purpose of this Regulation, three conditions are provided for stable 	acceleration.

2.26.1.	"Stable acceleration" applicable to all vehicles subject to this Regulation for low engine speed conditions, which eliminates power train reactions such as bumping and jerking.

2.26.2.	"Stable acceleration" applicable to vehicles of category M1, N1 and M2 < 3,500 kg technically permissible maximum laden mass avoids acceleration delays in acceleration due to engine control application, at the moment when the acceleration unit is depressed. This is typically ensured by using pre-acceleration.

2.26.3.	"Stable acceleration" for the purpose of Annex 7 is based on the assumption of a constant acceleration over the complete measurement distance between AA' and BB' plus the vehicle length.

2.27.	"Kickdown" means a driver initiated automated gear shift to a test condition outside the specific target conditions for the vehicle as defined in Annex 3.

2.28.		"Prevention of downshift" means a measure by the vehicle manufacturer to ensure that the vehicle is tested within its specific target conditions as defined in Annex 3 and Annex 7.

2.29.	"Exhaust silencing system with variable geometry" means a silencing system, not including pressure charging, containing one or more active, passive, or self-actuated moving parts or devices.

These parts or devices will cause a change of the gas flow through the exhaust silencing system and result in a change of the sound reduction performance, by opening or closing one or more valves in the exhaust gas flow as a function of varying driving or engine conditions (rpm engine speed, load, vehicle speed, etc.).

Active devices mean actuators, controlled by any means.

Passive or self-actuated devices mean units controlled by exhaust flow.

2.30.	“Exterior sound enhancement system” means a system that is installed to a vehicle for producing exterior sound, such as but not limited to sound actuators, either integrated into an exhaust silencing system or mounted as an individual unit.

2.31. 	“Deceleration” means the vehicle deceleration stipulated by the release of the acceleration control unit only, without any driver applied braking (service brake, retarder, parking brake, etc…).

2.32. 	“Performance” means the product of acceleration and vehicle speed as quantity of the achieved vehicle performance.

2.33. 	"Electronic Control System" means a combination of units, designed to co-operate in the production of the stated vehicle control function by electronic data processing. Such systems, often controlled by software, are built from discrete functional components such as sensors, electronic control units and actuators and connected by transmission links. They may include mechanical, electro-pneumatic or electro-hydraulic elements. "The System", referred to herein, is the one for which type approval is being sought.

2.34.	"Software" means the part of an Electronic Control System that consists of digital data and instructions."

2.35.	"Rx Software Identification Number (RXSWIN)" means a dedicated identifier, defined by the vehicle manufacturer, representing information about the type approval relevant software of the Electronic Control System contributing to the UN Regulation No. 51.04 type approval relevant characteristics of the vehicle.

2.36	 “Power trains” means a propulsion system as combination of the energy storage system, the energy supply system and the powertrain according to the UN Mutual Resolution No.2 (for example PEV, HEV, FCHEV)

3.		Application for approval

3.1.	The application for approval of a vehicle type with regard to sound shall be submitted by its manufacturer or by his duly accredited representative.

3.2.	It shall be accompanied by the undermentioned documents and the following particulars in triplicate:

3.2.1.	A description of the vehicle type with regard to the items mentioned in paragraph 2.2. above. The numbers and/or symbols identifying the engine type and the vehicle type shall be specified;

3.2.2.	A list of the components, duly identified, constituting the sound reduction system;

3.2.3.	A drawing of the assembled sound reduction system and an indication of its position on the vehicle;

3.2.4.	Detailed drawings of each component to enable it to be easily located and identified, and a specification of the materials used.

3.2.5.	A technical information document including the information as outlined in Annex 1, Appendix 2.

3.3.	In the case of paragraph 2.2.2. the single vehicle, representative of the type in question, will be selected by the Technical Service conducting approval tests, in accordance with the vehicle manufacturer, following the specification laid down in paragraph 3.1.2.2. in Annex 3.

3.4.	At the request of the Technical Service conducting approval tests, the vehicle manufacturer shall, in addition, submit a sample of the sound reduction system and an engine of at least the same cylinder capacity and rated maximum net power as that fitted to the vehicle in respect of which type-approval is sought.

3.5.	The Type Approval Authority shall verify the existence of satisfactory arrangements for ensuring effective control of the conformity of production before type approval is granted.

4.		Markings

4.1.	The following components relevant for exterior sound emissions, excluding fixing hardware and piping, shall bear a marking consisting the trade name or mark of the manufacturer of the components, and the manufacturer’s trade description:

- exhaust silencer,

- sound enhancement system.

4.2.	These markings shall be clearly legible and be indelible even after fitting.

4.3.	A component may carry several approval numbers if it has been approved as component of several replacement silencing systems.

5.		Approval

5.1.	Type approval shall only be granted if the vehicle type meets the requirements of paragraphs 6. and 7. below.

5.2.	An approval number shall be assigned to each type approved. Its first two digits (at present 04 corresponding to the 04 series of amendments) shall indicate the series of amendments incorporating the most recent major technical amendments made to the Regulation at the time of issue of the approval. The same Contracting Party shall not assign the same number to another vehicle type.

5.3.	Notice of approval or of extension or of refusal or withdrawal of approval or of production definitively discontinued of a vehicle type pursuant to this Regulation shall be communicated to the Parties to the Agreement applying this Regulation, by means of a form conforming to the model in Annex 1 to this Regulation.

5.4.	There shall be affixed, conspicuously and in a readily accessible place specified on the approval form, to every vehicle conforming to a vehicle type approved under this Regulation an international approval mark consisting of:

5.4.1.	A circle surrounding the letter "E" followed by the distinguishing number of the country which has granted approval;

5.4.2.	The number of this Regulation, followed by the letter "R", a dash and the approval number to the right of the circle prescribed in paragraph 5.4.1.

5.5.	If the vehicle conforms to a vehicle type approved under one or more other Regulations annexed to the Agreement, in the country which has granted approval under this Regulation, the symbol prescribed in paragraph 5.4.1. need not be repeated; in such a case the regulation and approval numbers and the additional symbols of all the Regulations under which approval has been granted in the country which has granted approval under this Regulation shall be placed in vertical columns to the right of the symbol prescribed in paragraph 5.4.1.

5.6.	The approval mark shall be clearly legible and be indelible.

5.7.	The approval mark shall be placed close to or on the vehicle data plate affixed by the manufacturer.

5.8.	Annex 2 to this Regulation gives examples of arrangements of the approval mark.

6.		Specifications

6.1.	General specifications for durability and against manipulation

6.1.1.	The vehicle, its engine and its sound reduction system shall be so designed, constructed and assembled as to enable the vehicle, in normal use, despite the vibration to which it may be subjected, to comply with the provisions of this Regulation.

6.1.2.	The sound reduction system shall be so designed, constructed and assembled as to be able to reasonably resist the corrosive phenomena to which it is exposed having regard to the conditions of use of the vehicle, including regional climate differences, and against manipulation.

6.2.	Specifications regarding sound levels

6.2.1.	Methods of measurement

6.2.1.1.	The sound made by the vehicle type submitted for approval shall be measured either indoors or outdoors by the methods described in Annex 3 and Annex 7 to this Regulation. The specific conditions for indoor testing are provided in Annex 8 to this Regulation. The results of the outdoor and indoor tests are deemed equivalent.

For each specific test condition for vehicles, the manufacturer can select to test the vehicle either indoors or outdoors. The Type Approval Authority shall always have the option to mandate an outdoor test for verification.  The option of the type approval authority to mandate an outdoor test shall apply to any test specified in this Regulation, including conformity of production testing.

In addition, the sound shall be measured on the stationary vehicle in the case of a vehicle where an internal combustion engine cannot operate when the vehicle is stationary, the emitted sound shall only be measured in motion. In the case of a hybrid electrical vehicle of category M1 where an internal combustion engine cannot operate when the vehicle is stationary, the emitted sound shall be measured according to Annex 3, paragraph 4.

Vehicles having a technically maximum permissible laden mass exceeding 2,800 kg shall be subjected to an additional measurement of the compressed air noise with the vehicle stationary in accordance with the specifications of Annex 5, if the corresponding brake equipment is part of the vehicle.

6.2.1.2.	The values measured in accordance with the provisions of paragraph 6.2.1.1. above shall be entered in the test report and a certificate corresponding to the model shown in Annex 1.

6.2.2.	Sound level limits

The sound level measured in accordance with the provisions of paragraph 3.1. of Annex 3 to this Regulation, mathematically rounded to the nearest integer value, shall not exceed the following limits:

| Vehicle category | Vehicles used for the carriage of passengers | Limit Values (dB(A)) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Phase 1 | Phase 2 | Phase 3 |
| M1 | PMR ≤ 120 | 72 | 70 | 68 |
|  | 120 < PMR ≤ 160 | 73 | 71 | 69 |
|  | PMR > 160 | 75 | 73 | 71 |
|  | PMR > 200, no. of seats ≤ 4, R-point height < 450mm from the ground | 75 | 74 | 72 |
| M2 | M ≤ 2.5 t | 72 | 70 | 69 |
|  | 2.5 t < M ≤ 3.5 t | 74 | 72 | 71 |
|  | M > 3.5 t; PN ≤ 135 kW | 75 | 73 | 72 |
|  | M > 3.5 t; PN > 135 kW | 75 | 74 | 72 |
| M3 | PN ≤ 150 kW | 76 | 74 | 73 |
|  | 150 kW < PN ≤ 250 kW | 78 | 77 | 76 |
|  | PN > 250 kW | 80 | 78 | 77 |
| Vehicle category | Vehicles used for the carriage of goods | Phase 1 | Phase 2 | Phase 3 |
| N1 | M ≤ 2.5 t | 72 | 71 | 69 |
|  | M > 2.5 t | 74 | 73 | 71 |
| N2 | PN ≤ 135kW | 77 | 75 | 74 |
|  | PN > 135 kW | 78 | 76 | 75 |
| N3 | PN ≤ 150 kW | 79 | 77 | 76 |
|  | 150 kW < PN ≤ 250 kW | 81 | 79 | 77 |
|  | PN > 250 kW | 82 | 81 | 79 |

6.2.2.1.	For vehicle types of category M1 derived from N1 vehicle types having a technically permissible maximum laden mass above 2.5 tons and a R-point height greater than 850 mm from the ground, the limits of vehicles types of category N1 having a technically permissible maximum laden mass above 2.5 tons apply.

6.2.2.2.	For vehicle types designed for off-road use, the limit values shall be increased by 2 dB(A) for M3 and N3 vehicles category and 1 dB(A) for any other vehicle category.

For vehicle types of category M1 the increased limit values for off-road vehicles are only valid if the technically permissible maximum laden mass > 2 tons.

6.2.2.3.	Limit values shall be increased by 2 dB(A) for wheelchair accessible vehicles of category M1 constructed or converted specifically so that they accommodate one or more persons seated in their wheelchairs when travelling on the road, and armoured vehicles, as defined in paragraph 2.5.2. of R.E.3.

6.2.2.4.		For vehicle types of category M3 having a gasoline only engine, the applicable limit value is increased by 2 dB(A).

6.2.2.5.	For vehicle types of category N1 having a technically permissible maximum laden mass of less than or equal to 2.5 tons, the engine capacity not exceeding 660 cc and the power-to-mass ratio (PMR) calculated by using the technically permissible maximum laden mass not exceeding 35 and a horizontal distance "d" between the front axle and the driver's seat R point of less than 1,100 mm, the limits of the vehicle types of category N1 having a technically permissible maximum laden mass above 2.5 tons apply.

6.2.3.	Real Driving Additional sound emission provisions

The Real Driving Additional Sound Emission Provisions (RD-ASEP) apply only to vehicles of categories M1 and N1 equipped with:

- an internal combustion engine (ICE) for propulsion of the vehicle, or

any other propulsion technology fitted with an exterior sound enhancement system.

6.2.3.1. 	Exemptions

Notwithstanding the requirements above, vehicles equipped with an ICE for propulsion, inclusive HEV are exempted from RD-ASEP, if their exhaust silencing systems have no variable geometries,

- and the vehicle has no sound enhancement systems, or

- a sound enhancement system is fitted to the vehicle solely for the purpose of fulfilling the provisions of UN Regulation No. 138, and the sound emitting device does not emit a sound pressure level of more than 75 dB(A) under any operation conditions exceeding the specification range of UN Regulation No. R138.

Manufacturer shall demonstrate compliance to the 75dB(A) by using the test set up according to UN Regulation No. R138 Annex 3 paragraph 3.3.2.2. by using the control range of RD-ASEP described in Annex 7 paragraph 3.3

Notwithstanding the requirements above, vehicles which have no ICE for propulsion are exempted from RD-ASEP, if a sound enhancement system is fitted to the vehicle solely for the purpose of fulfilling the provisions of UN Regulation No. 138, and the sound emitting device (AVAS) does not emit a sound pressure level of more than 75 dB(A) under any operation conditions exceeding the specification range of UN Regulation No. R138.

Manufacturer shall demonstrate compliance to the 75dB(A) by using the test set up according to UN Regulation No. R138 Annex 3 paragraph 3.3.2.2. by using the control range of RD-ASEP described in Annex 7 paragraph 3.3.

6.2.3.2. 	Specifications

6.2.3.2.1.	The vehicle shall meet the requirements of Annex 7 to this Regulation.

6.2.3.2.2.	The vehicle manufacturer shall not intentionally alter, adjust, or introduce any mechanical, electrical, thermal, or other device or procedure solely for the purpose of fulfilling the sound emission requirements as specified under this Regulation which is not operational during typical on-road operation.

6.2.3.2.3. 	Regardless of the specifications above, the sound emissions of the vehicle under other driving conditions not subject to the measuring conditions of the Annex 3 and Annex 7, shall not deviate from the evaluation principles outlined by Annex 7 in a significant manner.

6.2.3.3. 	Provisions against manipulation

6.2.3.3.1.	Software manipulation

6.2.3.3.1.1.	Requirements for software identification

For the purpose of ensuring the software of the System can be identified, an RXSWIN may be implemented by the vehicle manufacturer.

6.2.3.3.1.2.	If the manufacturer implements an RXSWIN the following shall apply:

6.2.3.3.1.2.1.	The vehicle manufacturer shall have a valid approval according to UN Regulation No. 156.

6.2.3.3.1.2.2.	The vehicle manufacturer shall provide the following information in the communication form of this Regulation:

- 	the RXSWIN

- 	how to read the RXSWIN or software version(s) in case the RXSWIN is not held on the vehicle

6.2.3.3.1.2.3.	The vehicle manufacturer may provide in the communication form of this Regulation a list of the relevant parameters that will allow the identification of those vehicles that can be updated with the software represented by the RXSWIN. The information provided shall be declared by the vehicle manufacturer and may not be verified by an Approval Authority.

6.2.3.3.1.3.	The vehicle manufacturer may obtain a new vehicle approval for the purpose of differentiating software versions intended to be used on vehicles already registered in the market from the software versions that are used on new vehicles. This may cover the situations where type approval regulations are updated, or hardware changes are made to vehicles in series production. In agreement with the testing agency duplication of tests shall be avoided where possible.

6.2.3.3.2.     	Hardware manipulation

All exhaust silencing systems shall be designed, constructed and assembled in a way that does not easily permit removal of baffles, exit-cones and other parts whose primary function is as part of the silencing/expansion chambers. It also applies to control units for active systems and to components like exhaust or intake flaps, or to vacuum tubes controlling the flaps, whose removal for purposes of maintenance, repair, or replacement is only possible by special tools and expertise.

6.3.	Specifications regarding exhaust systems containing fibrous materials

6.3.1.	Requirements of Annex 4 shall be applied.

7.		Modification and extension of approval of a vehicle type

7.1.		Every modification of the vehicle type shall be notified to the Type Approval Authority which approved the vehicle type. The Type Approval Authority may then either:

7.1.1.	Consider that the modifications made are unlikely to have an appreciable adverse effect and that in any case the vehicle still complies with the requirements, or

7.1.2.	Require a further test report from the Technical Service responsible for conducting the tests.

7.2.	Confirmation or refusal of approval, specifying the alterations shall be communicated by the procedure specified in paragraph 5.3. above to the Parties to the Agreement applying this Regulation.

7.3.		The Type Approval Authority issuing the extension of approval shall assign a series number for such an extension and inform thereof the other Parties to the 1958 Agreement applying this Regulation by means of a communication form conforming to the model in Annex 1 to this Regulation.

8.		Conformity of production

8.1	The conformity of production procedures shall comply with those set out in the Agreement, Schedule 1 (E/ECE/TRANS/505/Rev.3).

8.2.	Control procedures carried out by the manufacturer for conformity of production according to paragraph 2.3 of Schedule 1 of the Agreement (see 8.1 above) shall at least comply with the requirements of Annex 6 to this Regulation.

8.3	Whenever measurements on Conformity of Production according to paragraph 3. of Schedule 1 of the Agreement (see 8.1 above) or measurements other than type approval or conformity of production initiated by an authority to evaluate the conformity of the vehicle with regard to this Regulation are carried out, the setup of the vehicle and the assessment methods as established during type approval shall be taken as reference for the conformity assessment

Notwithstanding this provision, the requirements of 6.2.1.1 remain valid.

9.		Penalties for non-conformity of production

9.1.		The approval granted in respect of a vehicle type pursuant to this Regulation may be withdrawn if the requirements set forth above are not met.

9.2.	If a Contracting Party to the Agreement applying this Regulation withdraws an approval it has previously granted, it shall forthwith so notify the other Contracting Parties applying this Regulation, by means of a communication form conforming to the model in Annex 1 to this Regulation.

10.		Production definitively discontinued

10.1.	If the holder of the approval completely ceases to manufacture a vehicle type approved in accordance with this Regulation, he shall so inform the authority which granted the approval. Upon receiving the relevant communication that authority shall inform thereof the other Parties to the 1958 Agreement applying this Regulation by means of a communication form conforming to the model in Annex 1 to this Regulation.

11.		Transitional provisions

  - As from the official date of entry into force of the 04 series of amendments to this Regulation, no Contracting Party applying this Regulation shall refuse to grant or refuse to accept type-approvals under this Regulation as amended by the 04 series of amendments.

[Further transitional provisions to be completed]

12.		Names and addresses of Technical Services responsible for conducting approval tests and of Type Approval Authorities

The Contracting Parties to the 1958 Agreement applying this Regulation shall communicate to the United Nations Secretariat the names and addresses of the Technical Services responsible for conducting approval tests and of the Type Approval Authorities which grant approval and to which forms certifying approval or extension or refusal or withdrawal of approval, issued in other countries, are to be sent.

Annex 1

Communication

(maximum format: A4 (210 x 297 mm))

issued by:		Name of administration:

......................................

......................................

......................................

1

concerning:	Approval granted

Approval extended

Approval refused

Approval withdrawn

Production definitively discontinued

of a vehicle type with regard to its sound emission pursuant to UN Regulation No. 51

Approval No. ……………………………..	Extension No. ……………………………

### Section I

0.1.	Make (trade name of manufacturer):

0.2.	Type:

0.3.	Means of identification of type if marked on the vehicle:

0.3.1.	Location of that marking:

0.4.	Category of vehicle:

0.4.1.	Subcategory according to paragraph 6.2.2., the 2nd column of the table and the paragraphs 6.2.2.1. to 6.2.2.5.

0.5.	Company name and address of manufacturer:

0.6.	Names and Address(es) of assembly plant(s):

0.7.	Name and address of the manufacturer's representative (if any):

### Section II

1.	Additional information (where applicable): See Addendum (Appendix 1)

2.	Technical Service responsible for carrying out the tests:

3.	Date of test report:

4.	Number of test report:

5.	Remarks (if any): See Addendum (Appendix 1)

6.	Place:

7.	Date:

8.	Signature:

9.	Reasons for extensions:

Attachments:

Information package

Test report(s)

Annex 1 – Appendix 1

Addendum to the communication form No …

1.	Additional information

1.1.	Power plant

1.1.1.	Manufacturer of the engine:

1.1.2.	Manufacturer's engine code:

1.1.3	Rated maximum net power: ........... kW at ........... min-1
or maximum continuous rated power (electric motor) ........... kW

1.1.4.	Pressure charger(s), Make and Type:

1.1.5.	Air filter, Make and Type:

1.1.6.	Intake silencer(s), Make and Type:

1.1.7.	Exhaust Silencer(s), Make and Type:

1.1.8.	Catalyst(s), Make and Type:

1.1.9.	Particulate Trap(s), Make and Type:

1.2.	Transmission

1.2.1.	Type (mechanical, hydraulic, electric, etc.):

1.3.	Non-engine devices designed to reduce noise:

2.	Test results

2.1.	Sound level of moving vehicle (Annex 3): 	 dB(A)

Selected mode for tests of the vehicle in motion: ................................

2.2.	Sound level of stationary vehicle: .......... dB(A) at …….. min-1 in mode ………………

Sound level of stationary vehicle: .......... dB(A) at ........ min-1 in mode ………………1

Sound level of stationary vehicle: .......... dB(A) at ........ min-1 in mode ………………1

Sound level of stationary vehicle: .......... dB(A) at ........ min-1 in mode ………………1

Sound level of stationary vehicle: .......... dB(A) at ........ min-1 in mode ………………1

2.2.1.	Sound level of compressed air, service brake: 	 dB(A)

2.2.2.	Sound level of compressed air, parking brake: 	 dB(A)

2.2.3.	Sound level of compressed air, during the pressure regulator actuation: 
….. dB(A)

2.3.	Data to facilitate in-use compliance test of hybrid vehicles, where an internal combustion engine cannot operate when the vehicle is stationary

2.3.1.	Gear (i) or position of the gear selector chosen for the test:

2.3.2.	Position of the operating switch during measurement LACC (i), (if switch is fitted)

2.3.3.	If applicable, pre-acceleration length lPA (Point of the accelerator depression in meter before line AA'). If the pre-acceleration length differs per gear, reporting per gear is required."

2.3.4.	Sound pressure level LACC (i) 	 dB(A)

2.4. 		Testing method information

2.4.1. 	Test method selected: Outdoor/Indoor

3.	Remarks:

Annex 1 – Appendix 2

Technical Information Document

0.	General

0.1.	Make (trade name of manufacturer):

0.2. 	Type: ……………………………………

0.3.	Means of identification of type, if marked on the vehicle:

0.3.1.	Location of that marking:

0.4.	Category of vehicle:

0.5.	Company name and address of manufacturer:

0.6.	Name and address of the manufacturer's representative (if any):

0.7.	Name(s) and address(es) of assembly plant(s):

1.	General construction characteristics of the vehicle

1.1.	Photographs and/or drawings of a representative vehicle:

1.2.	Number of axles and wheels:

1.2.1.	Powered axles (number, position, interconnection):

1.3.	Position and arrangement of the engine:

2.	Masses and dimensions (in kg and mm) (Refer to drawing where applicable):

2.1.	Range of vehicle dimensions (overall):

2.1.1.	For chassis without bodywork:

2.1.1.1.	Length:

2.1.1.2.	Width:

2.1.2.	For chassis with bodywork

2.1.2.1.	Length:

2.1.2.2.	Width:

2.2.	Mass in running order

(a) 	Minimum and maximum for each variant:

(b) 	Mass of each version (a matrix shall be provided):

2.3.	Technically permissible maximum laden mass stated by the manufacturer:,

3.	Power plant

3.1.	Manufacturer of the engine:

3.1.1.	Manufacturer's engine code (as marked on the engine, or other means of identification):

3.2.		Internal combustion engine

3.2.1.	Specific engine information

3.2.1.1.	Working principle: positive ignition/compression ignition, cycle four stroke/two stroke/rotary

3.2.1.2.	Number and arrangement of cylinders:

3.2.1.2.1.	Firing order:

3.2.1.3.		Engine capacity: …. cm3

3.2.1.4.		Rated maximum net power: …. kW at ….. min–1 (manufacturer's declared value)

3.2.2.		Fuel feed

3.2.2.1.		By fuel injection (compression ignition only): yes/no9

3.2.2.1.1.	Working principle: Direct injection/pre-chamber/swirl chamber9

3.2.2.1.2.	Governor

3.2.2.1.2.1.	Type:

3.2.2.1.2.2.	Speed at which Cut-off starts under load: …. min–1

3.2.2.2.		By fuel injection (positive ignition only): yes/no9

3.2.2.2.1.	Working principle: Intake manifold (single-/multi-point2)/direct injection/other (specify)9

3.2.3.		Intake system

3.2.3.1.	Air filter, drawings, or

3.2.3.1.1.	Make(s):

3.2.3.1.2.	Type(s):

3.2.3.2.	Intake silencer, drawings,

3.2.3.2.1.	Make(s):

3.2.3.2.2.	Type(s):

3.2.4.	Exhaust system

3.2.4.1.	Description and/or drawing of the exhaust system:

3.2.4.2.	Exhaust silencer(s):

Type, marking of exhaust silencer(s):

Where relevant for exterior noise, reducing measures in the engine compartment and on the engine:

3.2.4.3.	Location of the exhaust outlet:

3.2.4.4.	Exhaust silencer containing fibrous materials:

3.2.5.	Catalytic convertor: yes/no9

3.2.5.1.	Number of catalytic convertors and elements (provide the information below for each separate unit):

3.2.6. 	Pressure charger(s)

3.2.6.1. 	Make(s): ……………………………………

3.2.6.2. 	Type(s): …………………………………….

3.3.	Electric motor

3.3.1.	Type (winding, excitation):

3.3.1.1.	Maximum hourly output: …. kW

3.3.1.2.	Operating voltage: …. V

3.4.	Engine or motor combination:

3.4.1.	Hybrid electric vehicle: yes/no9

3.4.2.	Category of hybrid electric vehicle: off-vehicle charging/not off-vehicle charging:9

3.4.3.	Operating mode switch: with/without9

3.4.3.1.	Selectable modes

3.4.3.1.1.	Pure electric: yes/no9

3.4.3.1.2.	Pure fuel consuming: yes/no9

3.4.3.1.3.	Hybrid modes: yes/no9 (if yes, short description):

3.4.4.	Electric motor (describe each type of electric motor separately)

3.4.4.1.	Make:

3.4.4.2.	Type:

3.4.4.3.	Rated maximum net power: …… kW

4.	Transmission

4.1.	Type (mechanical, hydraulic, electric, etc.):

4.2.	Gear ratios

| Gear | Internal gearbox ratios (ratios of engine to gearbox output shaft revolutions) | Final drive ratio(s) (ratio of gearbox output shaft to driven wheel revolutions) | Total gear ratios |
| --- | --- | --- | --- |
| Maximum for CVT |  |  |  |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| … |  |  |  |
| Minimum for CVT |  |  |  |
| Reverse |  |  |  |

4.3.	Maximum vehicle design speed (in km/h):

5.	Suspension

5.1.	Tyres and wheels

5.1.1.	Tyre/wheel combination(s)

(a) 	For tyres indicate size designation, load-capacity index and speed category symbol;

(b) 	For wheels indicate rim size(s) and off-set(s).

5.1.2.	Upper and lower limits of rolling radii

5.1.2.1.	Axle 1:

5.1.2.2.	Axle 2:

5.1.2.3.	Axle 3:

5.1.2.4.	Axle 4:

etc.

6.	Bodywork

6.1.	Type of bodywork:

6.2.	Materials used and methods of construction:

7.	Miscellaneous

7.1.	Details of any non-engine devices designed to reduce noise (if not covered by other items):

Signed:

Position in company:

Date:

Annex 2

Arrangements of the approval mark

Model A

(See paragraph 5.4. of this Regulation)

a = 8 mm min.

The above approval mark affixed to a vehicle shows that the vehicle type concerned has, with regard to its noise emission, been approved in the Netherlands (E 4) pursuant to UN Regulation No. 51 under approval No. 032439.

The first two digits of the approval number indicate that UN Regulation No. 51 already included the 04 series of amendments when the approval was granted.

Model B

(See paragraph 5.5. of this Regulation)

a = 8 mm min.

The above approval mark affixed to a vehicle shows that the vehicle type concerned has been approved in the Netherlands (E 4) pursuant to UN Regulations Nos. 51 and 33. The approval numbers indicate that, at the dates when the respective approvals were granted, UN Regulation No. 51 included the 04 series of amendments while UN Regulation No. 33 was in its original form.

Annex 3

Methods and instruments for measuring the sound made by motor vehicles

1.	Measuring instruments

1.1.	Acoustic measurements

The apparatus used for measuring the sound level shall be a precision sound-level meter or equivalent measurement system meeting the requirements of Class 1 instruments (inclusive of the recommended windscreen, if used). These requirements are described in "IEC 61672-1:2002 or later - Precision sound level meters", second edition, of the International Electrotechnical Commission (IEC).

For indoor testing, when no general statement or conclusion can be made about conformance of the sound level meter by each channel of the array conformance (e.g. when pass-by sound level simulation algorithms do not compute the overall level but spectrum or temporal to recompose it), a simulated pass-by run shall be performed at a constant speed of 50 km/h while a constant tone signal is supplied to all channels of the arrays. The simulated A-weighted sound level is processed and the deviation from a reference tone signal shall be checked in accordance to IEC 61672-3:2006 or later.

Measurements shall be carried out using the "fast" response of the acoustic measurement instrument and the "A" weighting curve also described in "IEC 61672-1:2002 or later". When using a system that includes a periodic monitoring of the A-weighted sound pressure level, a reading should be made at a time interval not greater than 30 ms.

The instruments shall be maintained and calibrated in accordance to the instructions of the instrument manufacturer.

1.2.	Calibration of the entire Acoustic Measurement System for a Measurement Session

At the beginning and at the end of every measurement session the entire measurement system shall be checked by means of a sound calibrator that fulfils the requirements for sound calibrators of at least precision Class 1 according to IEC 60942:2003 or later. Without any further adjustment the difference between the readings of two consecutive checks shall be less than or equal to 0.5 dB(A).

If this value is exceeded, the results of the measurements obtained after the previous satisfactory check shall be discarded.

For indoor testing, the entire measurement system shall be checked at the beginning and at the end of a series of sessions.

A qualified calibration method (i.e. electrical calibration) is recommended to be provided by the hardware supplier and, in that case, shall be implemented in the measurement software used. Simulation algorithms using sound source localization detection should deactivate that feature for these tests.

1.3.	Compliance with requirements

Compliance of the acoustic measurement instrumentation shall be verified by the existence of a valid certificate of compliance. These certificates shall be deemed to be valid if certification of compliance with the standards was conducted within the previous 12 months period for the sound calibration device and within the previous 24 months period for the instrumentation system. All compliance testing shall be conducted by a laboratory, which is authorized to perform calibrations traceable to the appropriate standards.

1.4.	Instrumentation for speed measurements

The engine speed shall be measured with instrumentation having an accuracy of ±2 per cent or better at the engine speeds required for the measurements being performed.

The road speed of the vehicle shall be measured with instrumentation having an accuracy of at least ±0.5 km/h, when using continuous measurement devices.

If testing uses independent measurements of speed, this instrumentation shall meet specification limits of at least ±0.2 km/h.

1.5.	Meteorological instrumentation

The meteorological instrumentation used to monitor the environmental conditions during the test shall include the following devices, which meet at least the given accuracy:

(a)	Temperature measuring device, ±1 °C;

(b)	Wind speed-measuring device, ±1.0 m/s;

(c)	Barometric pressure measuring device, ±5 hPa;

(d)	A relative humidity measuring device, ±5 per cent.

2.	Conditions of measurement

2.1.	Test Site and ambient conditions

The specifications for the test site provide the necessary acoustic environment to carry out the vehicle tests documented in this Regulation. Outdoor and indoor test environments that meet the specifications of this Regulation provide equivalent acoustic environments and produce results that are equally valid.

2.1.1. 		Test Site Outdoor

The surface of the test track and the dimensions of the test site shall be in accordance with ISO 10844:2014 or later.

2.1.2. 		Test Site Indoor

Test Site Indoor requirements shall be as specified below.

- The test room dimensions are described in paragraph 7.2. of ISO 362-3:2016 or later. All room dimensions may be adjusted to meet the specific application for the products being tested according to Annex 8, paragraph 4.

- The test facility shall meet the requirements of ISO 26101:2012 or later with the qualification criteria and measurement requirements appropriate to this test method as described in ISO 362-3:2016 or later, paragraph 7.3.

- Condition of the floor is described in ISO 362-3:2016 or later, paragraph 7.4.

- Cooling, ventilation, and exhaust gas management are described in ISO 362-3:2016 or later, paragraph 7.5.

- Dynamometer requirements are described in ISO 362-3:2016 or later, paragraph 8.

- Vehicle fixing system is described in ISO 362-3:2016 or later, paragraph 9.3.

2.1.3. 		Ambient conditions

The surface of the site shall be free of powdery snow, tall grass, loose soil or cinders. There shall be no obstacle which could affect the sound field within the vicinity of the microphone and the sound source. The observer carrying out the measurements shall so position themself as not to affect the readings of the measuring instrument.

Measurements shall not be made under adverse weather conditions. It shall be ensured that the results are not affected by gusts of wind.

The meteorological instrumentation should be positioned adjacent to the test area at a height of 1.20 m ± 0.02 m. The measurements shall be made when the ambient air temperature is within the range from 5 °C to 40 °C.

Tests carried out on request of the manufacturer at temperatures below 5° C shall be accepted as well.

The tests shall not be carried out if the wind speed, including gusts, at microphone height exceeds 5 m/s, during the sound measurement interval.

A value representative of temperature, wind speed and direction, relative humidity, and barometric pressure shall be recorded during the sound measurement interval.

Any sound peak which appears to be unrelated to the characteristics of the general sound level of the vehicle shall be ignored in taking the readings.

The background noise shall be measured for duration of 10 seconds immediately before and after a series of vehicle tests. The measurements shall be made with the same microphones and microphone locations used during the test. The A-weighted maximum sound pressure level shall be reported.

The background noise (including any wind noise) shall be at least 10 dB(A) below the A-weighted sound pressure level produced by the vehicle under test. If the difference between the ambient noise and the measured sound is between 10 and 15 dB(A), in order to calculate the test results the appropriate correction shall be subtracted from the readings on the sound-level meter, as in the following table:

| Difference between ambient noise and sound to be measured dB(A) | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- |
| Correction dB(A) | 0.5 | 0.4 | 0.3 | 0.2 | 0.1 | 0.0 |

For indoor testing, background noise shall take into account noise emissions produced by the dynamometer rollers, ventilation systems, and facility exhaust gas systems.

2.2.	Vehicle

2.2.1.		The vehicle shall be representative of vehicles to be put on the market as specified by the manufacturer in agreement with the Technical Service to fulfil the requirements of this Regulation.

Measurements shall be made without any trailer, except in the case of non-separable vehicles. At the request of the manufacturer, measurements may be made on vehicles with lift axle(s) in a raised position.

Measurements shall be made on vehicles at the test mass mT specified according to the following table.

When testing indoors, the test mass, mT shall be utilized by the control system of the dyno roller.  Actual mass of the vehicle has no effect on results and it is permitted to load the vehicle as necessary to prevent slip between the tyres and the dyno rolls. To detect excessive slip, it is recommended to control the ratio of engine rotational speed and vehicle speed between the acceleration phase and the constant-speed status. To avoid slip, it is possible to increase the axle load.

Target mass, mTARGET, is used to denote the mass that N2 and N3 vehicles should be tested at. The actual test mass of the vehicle can be less due to limitations on vehicle and axle loading.

| Vehicle category | Vehicle test mass |
| --- | --- |
| M1 | The test mass mT of the vehicle shall be between 0.9 mRO ≤ mT ≤ 1.2 mRO |
| N1 | The test mass mT of the vehicle shall be between 0.9 mRO ≤ mT ≤ 1.2 mRO |
| N2, N3 | mTARGET = 50 [kg/kW] x PN [kW] Extra loading, mXLOAD, to reach the target mass, mTARGET, of the vehicle shall be placed above the rear axle(s). If the test mass mT is equal to the target mass mTARGET, the test mass mT shall be 0.95 mTARGET ≤ mT ≤ 1.05 mTARGET The sum of the extra loading and the rear axle load in an unladen condition, mRA LOAD UNLADEN, is limited to 75 per cent of the technically permissible maximum laden mass allowed for the rear axle, mAC RA MAX. If the test mass mT is lower than the target mass mTARGET, the test mass mT shall be achieved with a tolerance of ±5 per cent. If the centre of gravity of the extra loading cannot be aligned with the centre of the rear axle, the test mass, mT, of the vehicle shall not exceed the sum of the front axle in an unladen condition, mFA LOAD UNLADEN, and the rear axle load in an unladen condition, mRA LOAD UNLADEN plus the extra loading, mXLOAD, and the mass of the driver mD. The test mass for vehicles with more than two axles shall be the same as for a two-axle vehicle. If the vehicle mass of a vehicle with more than two axles in an unladen condition, mUNLADEN, is greater than the test mass for the two-axle vehicle, then this vehicle shall be tested without extra loading. If the vehicle mass of a vehicle with two axles, mUNLADEN, is greater than the target mass, then this vehicle shall be tested without extra loading. |
| M2 (M ≤ 3,500 kg) | The test mass mT of the vehicle shall be between 0.9 mRO ≤ mT ≤ 1.2 mRO |
| Complete M2 (M > 3,500 kg), M3 | If the tests are carried out with a complete vehicle having a bodywork, mTARGET = 50 [kg/kW] x PN [kW] is calculated either in compliance with conditions above (see N2, N3 category) or the test mass mT of the vehicle shall be 0.9 mRO ≤ mT ≤ 1.1 mRO . |
| Incomplete M2 (M > 3,500 kg), M3 | If the tests are carried with an incomplete vehicle not having a bodywork, mTARGET = 50 [kg/kW] x PN [kW] is calculated either in compliance with conditions above (see N2, N3 category), or the test mass mT of the vehicle shall be 0.9 mRO ≤ mT ≤ 1.1 mRO . where mRO = mCHASSIS M2M3 + mXLOAD M2M3 |

2.2.2.		At the applicant's request the vehicle of a category M2, M3, N2 or N3 is deemed representative of its completed type if the tests are carried out to an incomplete vehicle not having a bodywork. In the test of an incomplete vehicle all relevant soundproofing materials, panels and noise reduction components and systems shall be fitted on the vehicle as designed by the manufacturer except a part of bodywork which is built in a later stage.

No new test shall be required due to fitting of a supplement fuel tank or re-location of the original fuel tank on condition that other parts or structures of the vehicle apparently affecting sound emissions have not been altered.

The tyres to be used for the test shall be representative for the vehicle and shall be selected by the vehicle manufacturer and recorded in Addendum to the Communication form (Annex 1, Appendix 1). They shall correspond to one of the tyre sizes designated for the vehicle as original equipment. The tyre is or will be commercially available on the market at the same time as the vehicle. The tyres shall be inflated to the pressure recommended by the vehicle manufacturer for the test mass of the vehicle. The tyres shall have at least 1.6 mm tread depth.

When performing indoor testing, tyre/road sound is evaluated independently on the test track with the tyres to be used, according to this paragraph. Propulsion sound is independently evaluated on the dynamometer using tyres and other sound control measures to produce tyre/road sound which does not influence the measurement result.

2.2.3.	Before the measurements are started, the engine shall be brought to its normal operating conditions.

2.2.4.	If the vehicle is fitted with more than two-wheel drive, it shall be tested in the drive which is intended for normal road use.

2.2.5.	If the vehicle is fitted with fan(s) having an automatic actuating mechanism, this system shall not be interfered with during the measurements.

2.2.6.	If the vehicle is equipped with an exhaust system containing fibrous materials, it might be necessary to carry out a conditioning test prior to testing. The provisions of Annex 4, paragraph 1. in conjunction with the flowchart (Figure 2) of the appendix to Annex 4 shall be followed.

2.2.7.	Calculation procedure to determine extra loading of N2 and N3 vehicles only

2.2.7.1.	Calculation of extra loading

The target mass, mTARGET, (per kW rated power) for two-axle vehicles of category N2 and N3 is specified in the Table in paragraph 2.2.1: above.

mTARGET = 50 [kg / kW] × PN [kW]	(1)

To reach the required target mass, mTARGET, for a vehicle being tested, the unladen vehicle, including the mass of the driver, mD, shall be loaded with an extra mass, mXLOAD, which shall be placed above the rear axle as given in Formula (8):

mTARGET = mUNLADEN + mD + mXLOAD 	(2)

The target mass, mTARGET, shall be achieved with a tolerance of ±5 per cent.

The vehicle mass of the test vehicle in the unladen condition, mUNLADEN, is calculated by measuring on a scale the unladen front axle load, mFA LOAD UNLADEN, and the unladen rear axle load, mRA LOAD UNLADEN, as given in Formula (3):

mUNLADEN = mFA LOAD UNLADEN + mRA LOAD UNLADEN 	(3)

By using Formulae (2) and (3), the extra loading, mXLOAD, is calculated as given in Formulae (4) and (5):

mXLOAD = mTARGET – (mD + mUNLADEN) 	(4)

mXLOAD = mTARGET − (mD + mFA LOAD UNLADEN + mRA LOAD UNLADEN) 	(5)

The sum of the extra loading, mXLOAD, and the unladen rear axle load, mRA LOAD UNLADEN, is limited to 75 per cent of the technically permissible maximum laden mass for the rear axle, mAC RA MAX, as given in Formula (6):

0,75 mAC RA MAX ≥ mXLOAD + mRA LOAD UNLADEN 	(6)

The mXLOAD is limited according to Formula (7):

mXLOAD ≤ 0,75 mAC RAMAX – mRA LOAD UNLADEN 	(7)

If the calculated extra loading, mXLOAD, in Formula (5) fulfils Formula (7), then the extra loading is equal to Formula (5). The test mass, mT, of the vehicle is as calculated from Formula (8):

mT = mXLOAD + mD + mFA LOAD UNLADEN + mRA LOAD UNLADEN 	(8)

In this case, the test mass of the vehicle is equal to the target mass

mT = mTARGET	(9)

If the calculated extra loading, mXLOAD, in Formula (5) does not fulfil Formula (7), but rather fulfils Formula (10)

mXLOAD > 0,75 mAC RA MAX – mRA LOAD UNLADEN 	(10)

then, the extra loading, mXLOAD, shall be as given by Formula (11):

mXLOAD = 0,75 mAC RA MAX – mRA LOAD UNLADEN 	(11)

and the test mass, mT, of the vehicle shall be as given by Formula (12):

mT = 0,75 mAC RA MAX + mD + mFA LOAD UNLADEN 	(12)

In this case, the test mass of the vehicle is lower than the target mass

mT < mTARGET 	(13)

The test mass, mT, shall be achieved with a tolerance of ± 5 per cent.

2.2.7.2.	Loading considerations if load cannot be aligned with the centre of rear axle

If the centre of gravity of the extra loading, mXLOAD, cannot be aligned with the centre of the rear axle, the test mass of the vehicle, mT, shall not exceed the sum of the unladen front axle load, mFA LOAD UNLADEN, and the unladen rear axle load, mRA LOAD UNLADEN, plus the extra loading, mXLOAD, and the mass of the driver, mD.

This means that if the actual front and rear axle loads are measured on a scale when the extra loading, mXLOAD, is placed onto the vehicle and it is aligned with the centre of the rear axle, the test mass of the vehicle minus the mass of the driver is as given by Formula (14):

mT − mD = mFA LOAD LADEN + mRA LOAD LADEN 	(14)

Where:

mFA LOAD LADEN = mFA LOAD UNLADEN	(15)

If the centre of gravity of the extra loading cannot be aligned with the centre of the rear axle, Formula (14) is still fulfilled, but

mFA LOAD LADEN > mFA LOAD UNLADEN 	(16)

because the extra loading has partly distributed its mass to the front axle. In that case, it is not allowed to add more mass onto the rear axle to compensate for the mass moved to the front axle.

2.2.7.3.	Test mass for vehicles with more than two axles

If a vehicle with more than two axles is tested, then the test mass of this vehicle shall be the same as the test mass for the two-axle vehicle.

If the unladen vehicle mass of a vehicle with more than two axles is greater than the test mass for the two-axle vehicle, then this vehicle shall be tested without extra loading.

2.2.7.4. 	Calculation of the test mass of a virtual vehicle with two axles:

When a vehicle family is not represented by a two-axle vehicle because it is physically not available, the vehicle family can be represented by a vehicle with more than two axles (vRF). In that case the test mass of a virtual two-axle vehicle (mT (2 axles virtual)) can be calculated in the following way:

For the calculation of the unladen vehicle mass of the virtual two-axle vehicle (mUNLADEN (2 axles virtual)), take from the vehicle with more than two axles (vRF) the measured unladen front axle load (mFA (vRF) LOAD UNLADEN) and the measured unladen rear axle load of that driven rear axle (mRA (vRF) LOAD UNLADEN) which has the highest unladen load.

If the vehicle (vRF) has more than one front axle, take the one with the highest unladen front axle load.

mUNLADEN (2 axles virtual) = mFA (vRF) LOAD UNLADEN + mRA (vRF) LOAD UNLADEN

mXLOAD (2 axles virtual) = mTARGET − (mD + mUNLADEN (2 axles virtual))

Due to the requirement that the sum of the extra loading (mXLOAD (2 axles virtual)) and the unladen rear axle load, mRA (vRF) LOAD UNLADEN, is limited to 75 per cent of the technically permissible maximum laden mass allowed for the rear axle, mAC RA MAX (2 axles virtual),  this value, mAC RA MAX (2 axles virtual), has to be chosen in such a way that it represents the rear axle of the forecasted highest production-volume in the manufacturer's variation with a technically permissible maximum laden mass allowed for the rear axle (mAC RA MAX (chosen) ) for the vehicle family as declared by the manufacturer.

mAC RA MAX (4x2 virtual) = mAC RA MAX (chosen)

If mXLOAD (2 axles virtual) ≤ 0,75 mAC RA MAX (chosen) – mRA (vRF) LOAD UNLADEN

then

mT (2 axles virtual) = mXLOAD (2 axles virtual) + mD + mFA (vRF) LOAD UNLADEN + mRA (vRF) LOAD UNLADEN

and

mT (2 axles virtual) = mTARGET

If mXLOAD (2 axles virtual) > 0,75 mAC RA MAX (chosen) – mRA (vRF) LOAD UNLADEN

then

mT (2 axles virtual) = 0,75 mAC RA MAX (chosen) + mD + mFA (vRF) LOAD UNLADEN

and

mT (2 axles virtual) < mTARGET

The test mass of the vehicle with more than two axles representing the vehicle family is defined as followed:

mT (vRF) = mT (2 axles virtual)

and the extra loading is calculated as

mXLOAD (vRF) = mT (2 axles virtual) – mD  – mUNLADEN (vRF)

3.	Methods of testing

Outdoor tests shall be performed according to paragraph 3.1.

Indoor tests shall be performed according to paragraph 3.1. using the specifications of ISO 362-3:2016 or later, variant A. For indoor application, the manufacturer shall provide to the technical service, documentation according to Annex 8, paragraph 1.  Variant A is a combination of indoor testing (power train sound) and outdoor testing (tyre/road sound).

3.1.	Measurement of sound of vehicles in motion

3.1.1.	General conditions of test

For outdoor testing, two lines, AA' and BB', parallel to line PP' and situated respectively 10 m ± 0.05m forward and 10 m ± 0.05m rearward of line PP’ shall be marked out on the test runway.

For indoor testing, the virtual line AA' indicates the beginning of the test track, PP' indicates the virtual position of the two pass-by microphones, and BB' indicates the end of the test track. The simulated vehicle speed at AA', vAA', or vehicle speed at PP', vPP', is defined by the roller speed when the reference point of the vehicle passes the virtual line AA' or PP', respectively. The simulated vehicle speed at BB', vBB', is defined when the rear of the vehicle passes the virtual line BB'.

At least four measurements shall be made on each side of the vehicle and for each gear. Preliminary measurements may be made for adjustment purposes, but shall be disregarded.

The microphones shall be located on both sides of the pathway at a distance of 7.5 m ± 0.05 m from the reference line CC' of the track and 1.2 m ± 0.02 m above the ground.

The reference axis for free field conditions (see IEC 61672-1:2002 or later) shall be horizontal and directed perpendicularly towards the path of the vehicle line CC'.

3.1.2.	Specific test conditions for vehicles

3.1.2.1.	Vehicles of category M1, N1 and M2 ≤ 3,500 kg technically permissible maximum laden mass:

The path of the centreline of the vehicle shall follow line CC' as closely as possible throughout the entire test, from the approach to line AA' until the rear of the vehicle passes line BB' +20 m.

If the vehicle is fitted with an auxiliary manual transmission or a multi-gear axle, the position used for normal urban driving shall be used. In all cases, the gear ratios for slow movements, parking or braking shall be excluded.

The test mass of the vehicle shall be according to the table of paragraph 2.2.1.

The test speed vTEST is 50 km/h  1 km/h. The test speed shall be reached, when the reference point is at line PP'.

If the test speed is modified according to paragraph 3.1.2.1.4.1.(d) of Annex 3 to this Regulation, the modified test speed shall be used for both the acceleration and constant speed test.

3.1.2.1.1.	Power to mass ratio index (PMR)

PMR is defined as follows:

PMR = (PN / mRO) * 1000 kg/kW, where PN is measured in kW and mRO is measured in kg and defined according to paragraph 2.4. of the main body.

If two or more sources of propulsive power operate at the conditions of test specified in paragraph 3.1.2.1 of Annex 3 to this Regulation, the total engine net power, PN, shall be the arithmetic sum of parallel propulsive engines on the vehicle. Applicable parallel propulsive engines are those power sources which provide forward motion to the vehicle in combination at the conditions of test specified in paragraph 3.1.2.1. of Annex 3 to this Regulation. Specified power for non-combustion engines shall be the power stated by the manufacturer.

The PMR with no dimension is used for the calculation of acceleration.

3.1.2.1.2.	Calculation of acceleration

Acceleration calculations are applicable to M1, N1 and M2 ≤ 3,500 kg technically permissible maximum laden mass categories only.

All accelerations are calculated using different speeds of the vehicle on the test track.  The formulae given are used for the calculation of aACC i, aACC i+1 and aACC TEST. The speed either at AA' or PP' is defined as the vehicle speed when the reference point passes AA' (vAA') or PP' (vPP'). The speed at BB' is defined when the rear of the vehicle passes BB' (vBB'). The method used for determination of the acceleration shall be indicated in the test report.

Due to the definition of the reference point for the vehicle the length of the vehicle (lVEH) is considered differently in the formula below. If the reference point is in the front of the vehicle, then l = lVEH, mid: l = ½ lVEH and rear: l = 0.

At the choice of the vehicle manufacturer, front engine vehicles may use 
l = 5 m, and mid-engine vehicles may use l = 2.5 m.

3.1.2.1.2.1.	Calculation procedure for vehicles with manual transmission, automatic transmission, adaptive transmissions and transmissions with continuous variable transmissions (CVT's) tested with locked gear ratios:

aACC TEST = ((vBB'/3.6)² - (vAA'/3.6)²) / (2*(20+l))

aACC TEST used in the determination of gear selection shall be the average of the four aACC TEST i during each valid measurement run.

Pre-acceleration may be used. The point of depressing the accelerator before line AA' shall be reported in the Addendum to the Communication form (Annex 1, Appendix 1).

3.1.2.1.2.2.	Calculation procedure for vehicles with automatic transmissions, adaptive transmissions and CVT's tested with non-locked gear ratios:

aACC TEST used in the determination of gear selection shall be the average of the four aACC TEST i during each valid measurement run.

If devices or measures described in paragraph 3.1.2.1.4.2. can be used to control transmission operation for the purpose of achieving test requirements, calculate aACC TEST using the equation:

aACC TEST = ((vBB'/3.6)² - (vAA'/3.6)²) / (2*(20+l))

Pre-acceleration may be used.

If devices or measures described in paragraph 3.1.2.1.4.2. are not used, calculate aACC TEST using the equation:

aACC TEST PP’-BB’ = ((vBB'/3.6)² - (vPP'/3.6)²) / (2*(10+l))

Pre-acceleration shall not be used.

The location of depressing the accelerator shall be where the reference point of the vehicle passes line AA'.

3.1.2.1.2.3.	Target acceleration

The target acceleration aURBAN defines the typical acceleration in urban traffic and is derived from statistical investigations. This function depends on the PMR of a vehicle.

The target acceleration aURBAN is defined by:

aURBAN = 0.63 * log10 (PMR) - 0.09

3.1.2.1.2.4.	Reference acceleration

The reference acceleration aACC REF defines the required acceleration during the acceleration test on the test track. It is a function depending on the power-to-mass ratio of a vehicle. That function is different for specific vehicle categories.

The reference acceleration aACC REF is defined by:

aACC REF = 1.59 * log10 (PMR) -1.41	for PMR ≥ 25

aACC REF = aURBAN = 0.63 * log10 (PMR) - 0.09	for PMR < 25

3.1.2.1.3.	Partial power factor kP

The partial power factor kP (see paragraph 3.1.3.1.) is used for the weighted combination of the test results of the acceleration test and the constant speed test for vehicles of category M1 and N1 and M2 < 3,500 kg technically permissible maximum laden mass

In cases other than a single gear test, aACC REF shall be used instead of aACC TEST (see paragraph 3.1.3.1.).

3.1.2.1.4.	Gear ratio selection

The selection of gear ratios for the test depends on their specific acceleration potential aACC, according to the reference acceleration aACC REF required for the acceleration test.

If the vehicle allows different transmission setups like automatic or manual gear selection and/or has different software programs or modes (e.g. sporty, winter, adaptive) leading to valid accelerations, the vehicle manufacturer shall prove to the satisfaction of the Technical Service, that the vehicle is tested in the mode which achieves an acceleration being closest to aACC REF.

The vehicle transmission, gear, or gear ratio may be controlled by electronic or mechanical measures for prevention of kick-down.

Appendix 1, Figure 4a to Figure 4e, give gear selection criteria and test run criteria for categories M1 and M2 having a technically permissible maximum laden mass not exceeding 3.500 kg and for category N1, in a flowchart form as an aid to test operation.

3.1.2.1.4.1.	Vehicles with manual transmission, automatic transmissions, adaptive transmissions or CVTs tested with locked gear ratios

The following conditions for selection of gear ratios are possible:

(a)	If one specific gear ratio gives an acceleration in a tolerance band of 5 per cent of the reference acceleration aACC REF, not exceeding 2.0 m/s2, test with that gear ratio.

(b)	If none of the gear ratios give the required acceleration, then choose a gear ratio i, with an acceleration higher and a gear ratio i+1, with an	acceleration lower than the reference acceleration. If the acceleration value in gear ratio i does not exceed 2.0 m/s2, use both gear ratios for the test. The weighting ratio in relation to the reference acceleration aACC REF is calculated by:

k = (aACC REF – aACC (i+1))/(aACC (i) – aACC (i+1))

(c)	If the acceleration value of gear ratio i exceeds 2.0 m/s2, the first gear ratio shall be used that gives an acceleration below 2.0 m/s2 unless gear ratio i+1 (or i+2, or i+3 or …) provides acceleration less than aURBAN. In this case, two gears, i and i+1 (or i+2, or i+3 or …) shall be used, including the gear i with acceleration exceeding 2.0 m/s2. In other cases, no other gear shall be used. The achieved acceleration aACC TEST during the test shall be used for the calculation of the part power factor kP instead of aACC REF.

(d)	If maximum engine speed nMAX is exceeded in a gear i before the vehicle passes BB' the next higher gear i+1 shall be used. If the next higher gear i+1 results in an acceleration below aURBAN, the vehicle test speed, vTEST, in the gear ratio i shall be reduced by 2.5 km/h and the gear ratio selection shall proceed as specified by the options given in this paragraph. In no case shall the vehicle test speed be reduced below 40 km/h.

If the maximum engine speed nMAX is exceeded in gear ratio i before the vehicle passes BB' and the vehicle test speed is equal to 40 km/h, the higher gear ratio i+1 is allowed even if aACC TEST does not exceed aURBAN.

The vehicle test speed in the higher gear ratio i+1 shall be 50 km/h.

The maximum engine speed nMAX is given by the formula below:

nMAX=1.56 ×PMR-0.227×S, but not more than 80% of S.

(e) 	If no gear ratio is available with an acceleration below 2.0 m/s², the manufacturer shall, if possible take measures to avoid an acceleration value aACC TEST greater than 2.0 m/s².

Table 1 in Appendix to Annex 3 provides examples for valid measures to control the downshift of gears or to avoid accelerations beyond 2.0 m/s². Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report.

3.1.2.1.4.2.	Vehicles with automatic transmission, adaptive transmissions and CVTs tested with non-locked gear ratios:

Manufacturers may take measures to lock discrete gear ratios by electronic or mechanical measures and follow the gear selection previsions of paragraph 3.1.2.1.4.1. above. If so selected, this shall be stated in the test report.

Otherwise, the gear selector position for full automatic operation shall be used.

The acceleration value aACC TEST shall be calculated as defined in paragraph 3.1.2.1.2.2.

The test may then include a gear change to a lower range and a higher acceleration or a higher engine speed. A gear change to a higher range and a lower acceleration is not allowed. A gear shifting to a gear ratio which is not representative for urban traffic shall be avoided.

Therefore, it is permitted to establish and use electronic or mechanical devices, including alternate gear selector positions, to avoid:

-	accelerations beyond 2.0 m/s². Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report. The achieved acceleration aACC TEST shall be greater or equal to aURBAN.

-	a test engine speed exceeding nMAX.

o	Therefore, the vehicle test speed vTEST may be reduced in steps by 2.5 km/h. In no case the vehicle test speed shall be reduced to a vehicle speed below 40 km/h, or

o	The engine load is reduced to avoid a downshift to a gear ratio where nMAX is exceeded.

If possible, the manufacturer shall take measures to avoid an acceleration value aACC TEST greater than 2.0 m/s2.

If possible, the manufacturer shall take measures to avoid an engine speed higher than nMAX.

Table 1 in Appendix to Annex 3 provides examples for valid measures to enable a test condition within the above specified boundaries. Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report.

The achieved acceleration aACC TEST is then used for the calculation of the partial power factor kP (see paragraph 3.1.2.1.3.) instead aACC REF.

3.1.2.1.4.3.	Vehicles with only one gear ratio, like but not limited to Battery Electric Vehicles (BEV) and Fuel Cell Vehicles (FCV)

The gear selector position for forward driving shall be used. The acceleration value aACC TEST shall be calculated as defined in paragraph 3.1.2.1.2.1.

The achieved acceleration aACC TEST shall be greater or equal to aURBAN.

If possible, the manufacturer shall take measures to avoid an acceleration value aACC TEST greater than 2.0 m/s².

Table 1 in Appendix to Annex 3 provides examples for valid measures to avoid accelerations beyond 2.0 m/s². Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report.

The achieved acceleration aACC_TEST is then used for the calculation of the partial power factor kP (see paragraph 3.1.2.1.3.) instead aACC REF.

3.1.2.1.5.	Acceleration test

The manufacturer shall define the position of the reference point in front of line AA' of fully depressing the accelerator. The accelerator shall be fully depressed (as rapidly as is practicable) when the reference point of the vehicle reaches the defined point. The accelerator shall be kept in this depressed condition until the rear of the vehicle reaches line BB'. The accelerator shall then be released as rapidly as possible. The measurement reading shall not end before the rear of the vehicle is 20 m behind the BB' line. The point of fully depressing the accelerator shall be reported in Addendum to the Communication form (Annex 1, Appendix 1). The Technical Service shall have the possibility of pretesting.

If the vehicle length was set according to the provisions of 3.1.2.1.2. the accelerator shall be kept in the depressed condition until the reference point reaches BB’ + 5 m for front engine vehicles, and BB’ + 2.5 m for mid-engine vehicles.

In the case of articulated vehicles consisting of two non-separable units regarded as a single vehicle, the semi-trailer shall be disregarded in determining when line BB' is crossed.

3.1.2.1.6.	Constant speed test

The constant speed test shall be carried out with the same gear(s) specified for the acceleration test and a constant speed of 50 km/h with a tolerance of ±1 km/h between AA' and BB', or if applicable at the speed determined for the acceleration test according 3.1.2.1.4.1. (d) or 3.1.2.1.4.2. with a tolerance of ±1 km/h between AA' and BB'.

During the constant speed test the acceleration control shall be positioned to maintain a constant speed between AA' and BB' as specified. If the gear is locked for the acceleration test, the same gear shall be locked for the constant speed test.

The constant speed test is not required for vehicles with a PMR < 25.

3.1.2.2.	Vehicles of categories M2 > 3,500 kg technically permissible maximum laden mass, M3, N2, N3:

The path of the centreline of the vehicle shall follow line CC' as closely as possible throughout the entire test, from the approach to line AA' until the rear of the vehicle passes line BB'. The test shall be conducted without a trailer or semi-trailer. If a trailer is not readily separable from the towing vehicle the trailer shall be ignored when considering the crossing of line BB'. If the vehicle incorporates equipment such as a concrete mixer, a compressor, etc., this equipment shall not be in operation during the test. The test mass of the vehicle shall be according to the table of paragraph 2.2.1. of Annex 3 to this Regulation.

The value of nBB' and vBB' used in the determination of gear and vehicle speed selection shall be the average of the four nBB', j and vBB', j values during each valid measurement run.

The value of nBB' shall be reported to a precision of 10 revolutions per minute. The reported nBB' shall be used in all subsequent calculation.

The value of vBB' shall be reported to the first digit after the decimal (xx,x).  The reported vBB' shall be used in all subsequent calculation.

Target conditions of category M2 > 3,500 kg technically permissible maximum laden mass, N2:

When the reference point passes line BB', the engine speed nBB' shall be between 70 and 74 per cent of speed S, at which the engine develops its rated maximum net power, and the vehicle speed shall be 35 km/h ± 5 km/h. Between line AA' and line BB' a stable acceleration condition according to definition 2.26.1. shall be ensured.

Target conditions of category M3, N3:

When the reference point passes line BB', the engine speed nBB' shall be between 85 and 89 per cent of speed S, at which the engine develops its rated maximum net power, and the vehicle speed shall be 35 km/h ± 5 km/h. Between line AA' and line BB' a stable acceleration condition according to definition 2.26.1. shall be ensured.

3.1.2.2.1.	Gear ratio selection

It is the responsibility of the manufacturer to determine the correct manner of testing to achieve the required conditions.

The vehicle transmission, gear, or gear ratio, shall be chosen to be able to fulfil the target conditions according to paragraphs 3.1.2.2.1.1. or 3.1.2.2.1.2. of Annex 3 to this Regulation. The vehicle transmission, gear, or gear ratio may be controlled by electronic or mechanical measures including exclusion of kick-down function.

Appendix 3, Figure 5a to Figure 5d, give gear selection criteria and test run criteria for categories M2 having a technically permissible maximum laden mass exceeding 3,500 kg, and for category N2, M3 and N3, in a flowchart as an aid to test operation.

3.1.2.2.1.1.	Manual transmission, automatic transmissions, adaptive transmissions or transmissions with continuously variable gear ratios (CVTs) tested with locked gear ratios

Stable acceleration condition according to definition 2.26.1. shall be ensured. The gear choice is determined by the target conditions.

The following conditions for fulfilling the target conditions in paragraph 3.1.2.2. of Annex 3 to this Regulation are possible:

(a) 	If one gear choice fulfils both target conditions for the rotational engine speed nTARGET BB’ and for the vehicle speed vTARGET BB’, test with that gear.

(b)	If more than one gear choice fulfils both target conditions for the rotational engine speed nTARGET BB’ and for the vehicle speed vTARGET BB’, test in gear i that gives velocity vBB’ GEAR i closest to 35 km/h.

(c)	If two gear choices fulfil both target conditions for the rotational engine speed nTARGET BB’ and for the vehicle speed vTARGET BB’, and fulfil the following condition.

(vTARGET BB’ – vBB’ GEAR i) = (vBB’ GEAR i+1 – vTARGET BB’)

then both gears are taken for further calculation of LURBAN.

(d)	If one gear choice fulfils the target condition for the rotational engine speed nTARGET BB’ but not the target condition for the vehicle speed 
vTARGET BB’, use two gears, gearX and gearY. The target conditions for the vehicle speed for these two gears are as follows:

gearX

25 km/h ≤ vBB’X ≤ 30 km/h

and

gearY

40 km/h ≤ vBB’Y ≤ 45 km/h

Both gears, gearX and gearY shall fulfil the target rotational engine speed nTARGET BB’. Both gears shall be used for further calculation of LURBAN.

If only one of the gears fulfils the target rotational engine speed, 
nTARGET BB’, test with that gear. This gear shall be used for further calculation of LURBAN.

(e)	If none of the two gears fulfils the target rotational engine speed 
nTARGET BB’ under condition d) then condition f) shall be chosen.

(f)	If no gear choice fulfils the target rotational engine speed choose the gear that fulfils the target vehicle velocity vTARGET BB’ and is closest to the target rotational engine speed nTARGET BB’, but not higher than nTARGET BB’.

vBB’ GEAR i  =  vTARGET BB’

nBB’ GEAR i  ≤  nTARGET BB’

A stable acceleration condition according to definition 2.26.1. shall be ensured. If such a stable acceleration cannot be ensured in a gear, this gear shall be disregarded. In all conditions, the rated engine speed shall not be exceeded while the reference point of the vehicle is in the measurement zone. If the rated engine speed is exceeded within the measurement zone, this gear shall be disregarded.

3.1.2.2.1.2.	Automatic transmission, adaptive transmissions, and transmissions with variable gear ratio tested with non-locked gear ratios

The gear selector position for full automatic operation shall be used.

The test may then include a gear change to a lower range and a higher acceleration. A gear change to a higher range and a lower acceleration is not allowed. In any case a gear change to a gear ratio that is typically not used at the specified condition as defined by the manufacturer in urban traffic shall be avoided.

Therefore, it is permitted to establish and use electronic or mechanical devices, including alternative gear selector positions, to prevent a downshift to a gear ratio that is typically not used at the specified test condition as defined by the manufacturer in urban traffic.

Table 1 in Appendix to Annex 3 provides examples for valid measures to control the downshift of gears. Any measure used by manufacturer for the above-mentioned purposes shall be documented in the test report.

The following conditions for fulfilling the target conditions in paragraph 3.1.2.2. of Annex 3 to this Regulation are possible:

(a) 	If the choice of the gear selector position fulfils both target conditions for the rotational engine speed nTARGET BB’ and for the vehicle speed 
vTARGET BB’, test with the gear selector in that position.

(b) 	If the choice of the gear selector position fulfils the target condition for the rotational engine speed nTARGET BB’ but not the target condition for the vehicle speed vTARGET BB’, change the target condition for the vehicle speed to two vehicle target speeds as follows:

Define vBB’1 as

25 km/h ≤ vBB’1 ≤ 35 km/h

and

Define vBB’2 as

35 km/h ≤ vBB’2 ≤ 45 km/h.

Conduct two tests, one with vBB’1 and one with vBB’2.

Both test conditions are used for further calculation of LURBAN.

(c)	If under condition b) the target rotational engine speed nTARGET BB’ cannot be fulfilled, condition d) shall be chosen.

(d)	If the choice of the gear selector position cannot fulfil the target condition for the rotational engine speed nTARGET BB’ but the target condition for the vehicle speed vTARGET BB’, change the target condition for the vehicle speed to two vehicle target speeds as follows:

Define vBB’1 as

25 km/h ≤ vBB’1 ≤ 30 km/h

and

Define vBB’2 as

40 km/h ≤ vBB’2 ≤ 45 km/h.

Conduct two tests, one with vBB’1 and one with vBB’2.

Use the test where nBB’ is closest to the target rotational engine speed nTARGET BB’ but not higher than nTARGET BB’.

nBB’ i ≤ nTARGET BB’ for i = 1, 2

If the vehicle cannot fulfil the condition:

nBB’ i ≤ nTARGET BB’ for i = 1, 2

condition (e) shall be used.

(e)	If the choice of the gear selector position cannot fulfil the target conditions for the rotational engine speed nTARGET BB’ and the target condition for the vehicle speed vTARGET BB’, change the target condition for the vehicle speed to the following:

vBB’ = vTARGET BB’ + 5 km/h

Conduct the test with that vehicle speed vBB’ where nBB’ is closest to the target rotational engine speed nTARGET BB’. A gear change to a higher range and a lower acceleration is allowed after the vehicle passes line PP’.

(f)	If the vehicle includes a transmission design that provides only a single gear selection (D) that limits engine speed during the test, the vehicle shall be tested using only the target vehicle speed vTARGET BB’.

3.1.2.2.1.3.	Powertrain with no combustion engine rotational engine speed available

Vehicles with a powertrain where no combustion engine rotational engine speed is available shall fulfil only the target condition for the vehicle speed vTARGET BB’

The following conditions for fulfilling the target condition vTARGET BB’ in paragraph 3.1.2.2. of Annex 3 to this Regulation are possible.

(a)	If no rotational engine speed is available, it is necessary to fulfil only the target vehicle speed vTARGET BB’.

(b)	If no rotational engine speed is available and the target vehicle speed vTARGET BB’ cannot be fulfilled, two test conditions shall be conducted as follows:

vBB’1 for the first test condition is defined as

25 km/h ≤ vBB’1 ≤ 35 km/h

and

vBB’2 for the second test condition is defined as

35 km/h ≤ vBB’2 ≤ 45 km/h

Both test conditions are used for further calculation of LURBAN.

(c)	If no rotational engine speed is available and the target vehicle speed vTARGET BB’ and vBB’1 defined as

25 km/h ≤ vBB’1 ≤ 35 km/h

cannot be fulfilled, it is necessary to conduct, only one test with vBB’2 where vBB’2 is defined as

35 km/h ≤ vBB’2 ≤ 45 km/h

The test condition for vBB’2 is taken for further calculation of LURBAN.

3.1.2.2.2.	Acceleration test

When the reference point of the vehicle reaches the line AA' the accelerator control shall be fully depressed (without operating the automatic downshift to a lower range than normally used in urban driving) and held fully engaged until the reference point reaches BB’ + 5 m. The acceleration control unit can then be released on request of the manufacturer.

In the case of articulated vehicles consisting of two non-separable units regarded as a single vehicle, the semi-trailer shall be disregarded in determining when line BB' is crossed.

3.1.3.	Interpretation of results

For vehicles of categories M1 and M2 having a maximum authorized mass not exceeding 3,500 kg, and category N1 the maximum A-weighted sound pressure level indicated during each passage of the vehicle according to paragraphs 3.1.2.1.5. and 3.1.2.1.6.  shall be rounded to the first significant digit after the decimal place (e.g. XX,X).

For vehicles of category M2 having a maximum authorized mass exceeding 3,500 kg and categories M3, N2, and N3 the maximum A-weighted sound pressure level indicated during each passage of the reference point of the vehicle between line AA’ and line BB’ + 5 m shall be rounded, to the first significant digit after the decimal place (e.g. XX,X).

For indoor testing, pass-by sound is simulated by measurement of power train sound on the dynamometer and energetical addition of the tyre/road sound (measured separately on an outdoor test track) according to Annex 8, paragraph 2 of this Regulation.

If a sound peak obviously out of character with the general sound pressure level is observed, the measurement shall be discarded. At least four measurements for each test condition shall be made on each side of the vehicle and for each gear ratio. Left and right sides may be measured simultaneously or sequentially. The first four valid consecutive measurement results, within 2 dB(A), allowing for the deletion of non-valid results (see paragraph 2.1.), shall be used for the calculation of the final result for the given side of the vehicle. The results of each side shall be averaged separately and rounded to the first decimal place.

All further calculations to derive LURBAN shall be done separately for the left and right vehicle side. The final value to be reported as the test result mathematically rounded to the nearest integer shall be the higher value of the two sides.

The speed measurements at AA', BB', and PP' shall be noted and used in calculations to the first significant digit after the decimal place.

The calculated acceleration aACC TEST shall be noted to the second digit after the decimal place.

3.1.3.1.	Vehicles of categories M1, N1 and M2 ≤ 3,500 kg technically permissible maximum laden mass

The calculated values for the acceleration test and the constant speed test are given by:

LACC REP = LACC (i+1) + k * (LACC(i) – LACC (i+1))

LCRS REP = LCRS (i+1) + k * (LCRS (i) – LCRS (i+1))

Where k = (aACC REF – aACC (i+1))/(aACC (i) – aACC (i+1))

In the case of a single gear ratio test the values are the test result of each test.

The final result is calculated by combining LACC REP and LCRS REP. The equation is:

LURBAN = LACC REP – kP * (LACC REP – LCRS REP)

The weighting factor kP gives the part power factor for urban driving. In cases other than a single gear test, kP is calculated by:

kP = 1 – (aURBAN / aACC REF)

If only one gear was specified for the test, kP is given by:

kP = 1 – (aURBAN / aACC TEST)

In cases where kP aACC TEST is less than aURBAN:

kP =  0

In case of a vehicle with a PMR<25, the final result LURBAN is equal to LACC REP.

3.1.3.2.	Vehicles of categories M2 > 3,500 kg technically permissible maximum laden mass, M3, N2, N3

When the result of one test condition is used the final result LURBAN is equal to the intermediate result.

When the results of two test conditions are used the arithmetic mean of the intermediate results of the two averages for each side of the two conditions shall be calculated. The final result LURBAN is the higher value of the two calculated averages.

3.2.	Measurement of sound emitted by stationary vehicles

3.2.1.	Sound level in the vicinity of vehicles

The measurement results shall be entered into the Addendum to the Communication form (Annex 1, Appendix 1).

3.2.2.	Acoustic measurements

A precision sound level meter as defined in paragraph 1.1 of this annex shall be used for the measurements.

3.2.3.	Test site - local conditions (see appendix of Annex 3, Figure 2)

3.2.3.1.	In the vicinity of the microphone, there shall be no obstacle that could influence the acoustical field and no person shall remain between the microphone and the sound source. The meter observer shall be positioned so as not to influence the meter reading.

3.2.4.	Disturbance noise and wind interference

Readings on the measuring instruments produced by ambient noise and wind shall be at least 10 dB(A) below the sound level to be measured. A suitable windscreen may be fitted to the microphone provided that account is taken of its effect on the sensitivity of the microphone (see paragraph 1.1. of this annex).

3.2.5.	Measuring method

3.2.5.1.	Nature and number of measurements

The maximum sound level expressed in A-weighted decibels (dB(A)) shall be measured during the operating period referred to in paragraph 3.2.5.3.2.1.

At least three measurements shall be taken at each measuring point.

3.2.5.2.	Positioning and preparation of the vehicle

The vehicle shall be located in the centre part of the test area with the gear selector in neutral position and the clutch engaged. If the design of the vehicle does not allow this, the vehicle shall be tested in conformity with the manufacturer's prescriptions for stationary engine testing. Before each series of measurements, the engine shall be brought to its normal operating condition, as specified by the manufacturer.

If the vehicle is fitted with fan(s) having an automatic actuating mechanism, this system shall not be interfered with during the sound level measurements.

The engine hood or compartment cover, if so fitted, shall be closed.

3.2.5.3.	Measuring of noise in proximity to the exhaust (see appendix of Annex 3, Figure 3a)

3.2.5.3.1.	Positions of the microphone

3.2.5.3.1.1.	The microphone shall be located at a distance of 0.5 m ± 0.01 m from the reference point of the exhaust pipe defined in Figure 2 and at an angle of 45º (±5º) to the vertical plane containing the flow axis of the pipe termination. The microphone shall be at the height of the reference point, but not less than 0.2 m from the ground surface. The reference axis of the microphone shall lie in a plane parallel to the ground surface and shall be directed towards the reference point on the exhaust outlet.

If two microphone positions are possible, the location farthest laterally from the vehicle longitudinal centreline shall be used.

If the flow axis of the exhaust outlet pipe is at 90º to the vehicle longitudinal centreline, the microphone shall be located at the point, which is furthest from the engine.

3.2.5.3.1.2.	For vehicles having an exhaust provided with outlets spaced more than 0.3 m apart or more than one silencer, one set of measurement is made for each outlet.

3.2.5.3.1.3.	If a vehicle has two or more exhaust outlets spaced less than or equal to 0.3 m apart and connected to a single silencer, only one set of measurement shall be made. The microphone shall be located relative to the outlet furthest away from the vehicle longitudinal centreline, or when such outlet does not exist, to the outlet, which is highest above the ground.

3.2.5.3.1.4.	For vehicles with a vertical exhaust (e.g. commercial vehicles) the microphone shall be placed at the height of the exhaust outlet. Its axis shall be vertical and oriented upwards. It shall be placed at a distance of 0.5 m ± 0.01 m from the exhaust pipe reference point, but never less than 0.2 m from the side of the vehicle nearest to the exhaust.

3.2.5.3.1.5.	For vehicles, where the reference point of the exhaust pipe is not accessible, or located under the vehicle body, as shown in Figure 3b and 3c in Annex 3, because of the presence of obstacles which form part of the vehicle (e.g. spare wheel, fuel tank, battery compartment), the microphone shall be located at least 0.2 m from the nearest obstacle, including the vehicle body, and shall not be located under the vehicle. Its axis of maximum sensitivity shall face the exhaust outlet from the position least concealed by the above-mentioned obstacles. In case the distance from the exhaust outlet to the other side of the vehicle is larger than 0.2 m (Figures 3c and 3d in Annex 3), the following distances of d1 and d2 shall be chosen:

Case 1:

d1 shall be equal to 0.5 m and the distance from the side (outer border of the vehicle) shall be at least 0.2 m.

d2 shall be equal to 0.5 m and the distance from the side (outer border of the vehicle) shall be at least 0.2 m.

Case 2 (if Case 1 is not fulfilled):

d1 shall be at least 0.5 m and the distance from the side (outer border of the vehicle) shall be equal to 0.2 m.

d2 shall be at least 0.5 m and the distance from the side (outer border of the vehicle) shall be equal to 0.2 m.

When several positions are possible, as shown in Figure 3c, the microphone position giving the lowest value of d1 or d2 shall be used.

3.2.5.3.1.6.	Examples of the position of the microphone, depending on the location of the exhaust pipe, are given in Figures 3a-3d in Appendix 1 to Annex 3.

3.2.5.3.2.	Operating conditions of the engine

3.2.5.3.2.1.	Target engine speed

The target engine speed is defined as:

(a)	75 per cent of the rated engine speed S for vehicles with a rated engine 
	speed ≤ 5,000 min-1;

(b)	3.750 min-1 for vehicles with a rated engine speed above 5.000 min-1 
	and below 7,500 min-1;

(c)	50 per cent of the rated engine speed S for vehicles with a rated engine 
	speed ≥ 7,500 min-1.

If the vehicle cannot reach the engine speed as stated above, the target engine speed shall be 5 per cent below the maximum possible engine speed for that stationary test.

For vehicles for which the engine speed is a fixed value, (for example, but not limited to, series hybrids) either above or below the target engine speed and cannot be adjusted by the accelerator, the test shall be carried out at the fixed engine speed.

In case the engine speed deviates from the applicable target engine speed, the engine speed used for the test and the reason for the deviation shall be documented in the test report and at paragraph 2.2. of Appendix 1 to Annex 1.

3.2.5.3.2.2.	Test procedure

The engine speed shall be gradually increased from idle to the target engine speed, not exceeding the tolerance band of ±3 per cent of the target engine speed, and held constant. Then the throttle control shall be rapidly released and the engine speed shall be returned to idle. The sound pressure level shall be measured during a period of operation consisting of a maintaining constant engine speed of 1 second and throughout the entire deceleration period. The maximum sound level meter reading during this period of operation, mathematically rounded to the first decimal place, is taken as the test value.

3.2.5.3.2.3.	Test validation

The measurement shall be regarded as valid if the test engine speed does not deviate from the target engine speed by more than ±3 per cent for at least 1 second.

3.2.6.	Results for sound emitted by stationary vehicles

3.2.6.1.	Single test position (outlet)

For the vehicles equipped with one exhaust outlet or two or more exhaust outlets as specified in paragraph 3.2.5.3.1.3., the stationary sound of the vehicle shall be determined for one test position.

At least three measurements for a test position (outlet) shall be made.

The maximum A-weighted sound pressure level indicated during each of the three measurements shall be recorded to the first decimal place.

The first three valid consecutive measurement results, within 2 dB(A), allowing for the deletion of non-valid results (see paragraph 2.1. except the specifications of the test site), shall be used for the determination of the final result for the given measurement position.

The result for a test position (outlet) is the arithmetic average of the three valid measurements, mathematically rounded to the nearest integer value (e.g. 72.5 shall be noted as to 73 while 72.4 shall be noted as to 72).

3.2.6.2.	Multiple test positions (outlets)

For vehicles equipped with multiple exhaust outlets as specified in paragraph 3.2.5.3.1.2., the stationary sound of the vehicle shall be determined for each test position, following the measurement and calculation principles above.

The reported sound pressure level shall be for the test position having the highest average sound pressure level.

3.2.6.3.	Modes

If the vehicle has different modes according to the definition 2.25.1. the stationary sound of the vehicle shall be determined for each mode, following the measurement and calculation principles above.

The sound pressure level for each mode shall be reported according to 3.2.6.1. in case the vehicle has only a single test position (outlet) and according to 3.2.6.2. in case of multiple test positions (outlets).

3.2.7.	Stationary sound pressure level representative for the vehicle type

If the vehicle has only one mode and a single test position (outlet), the representative sound pressure level for the vehicle type is determined by the measurement result according to paragraph 3.2.6.1.

If the vehicle has only one mode but multiple test positions (outlets), the representative sound pressure level for the vehicle type is determined by the measurement result according to paragraph 3.2.6.2.

If the vehicle has multiple modes and one or more test positions (outlets), the representative sound pressure level for the vehicle type is determined by the measurement result according to paragraph 3.2.6.3. In the test report and paragraph 2.2. of Appendix 1 to Annex 1 the representative test result determined by the principles above and the name of the mode shall be documented for every mode. The representative sound pressure level for the vehicle type and its registration papers is the highest reported sound pressure level of all modes as documented in paragraph 2.2. of Appendix 1 to Annex 1.

4.	Sound from the hybrid vehicle of categories M1 in motion, where an internal combustion engine cannot operate when the vehicle is stationary (data reported to facilitate testing of the vehicle in use).

4.1.	In order to facilitate in-use compliance test of hybrid vehicles – where an internal combustion engine cannot operate when the vehicle is stationary –, the following information relating to the sound-pressure level measurements carried out in accordance with paragraph 3.1. of Annex 3 for the motor vehicles in motion is referred to as in-use compliance reference data:

(a) 	Gear (i) or, for vehicles tested with non-locked gear ratios, the position of the gear selector chosen for the test;

(b) 	Position of the operating switch during measurement of the sound pressure level LACC (i) (if switch is fitted);

(c) 	The pre-acceleration length lPA in m;

(d)	The average vehicle speed in km/h at the beginning of the acceleration for tests in gear (i); and

(e) 	The sound pressure level LACC (i) in dB(A) of the acceleration tests in gear (i), defined as the maximum of the two values resulting from averaging the individual measurement results at each microphone position separately.

4.2.	The in-use compliance reference data shall be entered in the type approval certificate as specified in paragraph 2.3. of the Addendum to the Communication form (Annex 1, Appendix 1).

Annex 3 - Appendix

# Figure 1

Measuring positions for vehicles in motion

# Figure 2

Reference point for measurement of sound emitted by stationary vehicles

# Figure 3a

# Figure 3b

# Figure 3c

# Figure 3d

# Figure 4a

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – LURBAN computation

Determine PMR for test vehicle 3.1.2.1.1.

Determine target acceleration aURBAN (3.1.2.1.2.3.) and reference acceleration aACC REF (3.1.2.1.2.4.)

Select test method (3.1.2.1.4.)

locked gears (3.1.2.1.4.1.)  See Figures 4b, 4c and 4d

non-locked gears (3.1.2.1.4.2.) See Figure 4e

Acceleration test (3.1.2.1.5.)

Constant speed test (3.1.2.1.6.)

Calculation of gear weighting factor k if 2-gear test (3.1.2.1.4.1.)

Calculate LACC REP and L CRS REP (3.1.3.1.)

Calculate kP (3.1.3.1.)

Calculate LURBAN (3.1.3.1.)

# Figure 4b

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation - 
Gear selection using locked gear PART 1

Select Gear

Is acceleration stable according to 2.26.2.?

Calculate test acceleration according to 3.1.2.1.2.1.

Select Pre-acceleration and entry speed

Yess

No

Is acceleration within aACC REF tolerance band?

Yess

No

Is acceleration less than or equal 2,0 m/s2? and engine speed less than nMAX prior to BB'?

Use gear and compute kP according to 3.1.3.1.

Compute LACC REP using results of valid runs

No

Select gears to obtain gear i with stable acceleration above

aACC REF and gear i+1 with stable acceleration below aACC REF

See Case 2 in Figure 4c

See Case 1 in Figure 4c

Yess

Testing locked gears according to 3.1.2.1.4.1.

# Figure 4c

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – 
Gear selection using locked gear PART 2

Yess

No

Is acceleration of gear i less than or equal 2,0 m/s2? and engine speed less than nMAX prior to BB’?

Use both gears i and i+1, (i+2, i+3, or,… )   and compute kP  according to 3.1.3.1. and k by 3.1.2.1.4.1.

Compute LACC REP using results of valid runs

# Case 1:

# Two gears, gear i with stable acceleration above aACC REF and gear i+1 with stable acceleration below aACC REF

Case 2:

One gear with stable acceleration above 2,0 m/s2 or engine speed greater than nMAX prior to BB'

Determine first gear i + n (n=1, 2,…) with stable acceleration less than or equal to 2,0 m/s2 and engine speed less than nMAX prior to BB'

Is acceleration of gear i +n more than aURBAN?

Yess

Use gear and compute kP according to 3.1.3.1.

No

Use both gears i with acceleration higher than 2,0 m/s2 and i+1, (i+2, i+3, or,…)  with acceleration less than a URBAN

Is engine speed of gear i more than nMAX prior to BB’ ?

See Case 3 in Figure 4d

No

Yess

# Figure 4d

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – 
Gear selection using locked gear PART 3

Yess

No

Yes

Determine first gear i + n (n=1, 2, …) with stable acceleration less than or equal to 2,0m/s² and engine speed less than S prior to BB’. Test with this gear at a speed, vTEST, of 50km/h

Reduce test speed vTEST by 2,5 km/h with gear i

Test locked gears according to 3.1.2.1.4.1 with new test speed

Is engine speed of gear i less than nMAX prior to BB’

No

# Case 3:

# No gear with acceleration more than aURBAN and engine speed less than nMAX prior to BB’

Is test speed vTEST 40km/h

# Figure 4e

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – 
Gear Selection using non-locked gears

engine speed greater than nMAX prior to BB'

Yes

Yess

No

Compute LACC REP using results of valid runs

Calculate test acceleration according to 3.1.2.1.2.2.  Pre-acceleration is not allowed

Compute kP according to 3.1.3.1.

Can measures be taken to control downshifts?

Is acceleration stable?  i.e. there is no delay.

Calculate test acceleration according to 3.1.2.1.2.2.

Select Pre-acceleration and entry speed

No

Yess

If possible, control downshift to obtain acceleration less than or equal to 2,0 m/s2 or aACC REF, whichever is lower.  If not possible, run higher than 2,0 m/s2 is valid.

Testing unlocked gears according to 3.1.2.1.4.2.

Select entry speed

No

See flowchart 4f.

# Figure 4f

# Flowchart for vehicles tested according to paragraph 3.1.2.1. of Annex 3 to this Regulation – 
Gear Selection using non-locked gears

Yess

No

Yes

Not valid test condition.

Reduce test speed vTEST by 2,5 km/h

Test according to 3.1.2.1.4.2 with new test speed

Is engine speed less than nMAX prior to BB’

No

# Engine speed exceed nMAX prior to BB’

Is test speed vTEST 40km/h

# Figure 5a*

# Flowchart for vehicles tested according to paragraph 3.1.2.2. of Annex 3 to this Regulation – 
Test in locked gears

Select test method (3.1.2.2.1.)

Determine vehicle load (2.2.1.)

Locked gear ratios (3.1.2.2.1.1.)

Fulfilling target conditions (3.1.2.2.)?

Establish target conditions (3.1.2.2.)

One test

condition

Final result (3.1.3.2.)

Test criteria

(3.1.2.2.1.1. d)

nBB’ = nTARGET BB’

25 km/h ≤  vBB’X ≤ 30 km/h

40 km/h ≤  vBB’Y ≤ 45 km/h

Test criteria

(3.1.2.2.1.1. a)

nBB’ = nTARGET BB’

vBB’ = vTARGET BB’

Test criteria

(3.1.2.2.1.1. b)

nBB’ = nTARGET BB’

Gear i with vBB’ GEAR i

Closest to  vTARGET BB’

Test criteria

(3.1.2.2.1.1. f)

vBB’ GEAR i = vTARGET BB’

and

nBB’ GEAR i  ≤ nTARGET BB’

vTARGET BB’ = Yes

Yes, on one gear

nTARGET BB’ = Yes

LURBAN X for vBB’X

LURBAN Y for vBB’Y

(3.1.3.)

LURBAN

(3.1.3.)

LURBAN for gear i

with vBB’ GEAR i

closest to  vTARGET BB’

(3.1.3.)

LURBAN for nBB’ GEAR i

closest to but

≤ nTARGET BB’

(3.1.3.)

If (3.1.2.2.1.1. c)

(vTARGET BB’ - vBB’ GEAR i )

is equal to

(vBB’ GEAR i+1 - vTARGET BB’)

At least two test

conditions

Two test

conditions

At least two test

conditions

Yes, on more than one gear

vTARGET BB’ = Yes

nTARGET BB’ = No

vTARGET BB’ = No

If (3.1.2.2.1.1 e)

For both gears

For one gear only

Calculate average (3.1.3.2.)

If nBB’ = nTARGET BB’

# Figure 5b*

# Flowchart for vehicles tested according to paragraph 3.1.2.2. of Annex 3 to this Regulation – 
Test in non-locked gears PART 1

Select test method (3.1.2.2.1.)

Determine vehicle load (2.2.1.)

Non-locked gear ratios (3.1.2.2.1.2.)

Fulfilling target conditions (3.1.2.2.)?

Establish target conditions (3.1.2.2.)

One test

condition

Final result (3.1.3.2.)

Test criteria

(3.1.2.2.1.2. d)

25 km/h ≤ vBB’1 ≤ 30 km/h

40 km/h ≤ vBB’2 ≤ 45 km/h

nBB’ i ≤ nTARGET BB’

Test criteria

(3.1.2.2.1.2. a)

nBB’ = nTARGET BB’

vBB’ = vTARGET BB’

Test criteria

(3.1.2.2.1.2. b)

nBB’ = nTARGET BB’

25 km/h ≤ vBB’1 ≤ 35 km/h

35 km/h ≤ vBB’2 ≤ 45 km/h

Test criteria

(3.1.2.2.1.2. e)

vBB’ = vTARGET BB’ + 5km/h

nBB’ closest to

nTARGET BB’

vTARGET BB’ = Yes

nTARGET BB’ = Yes

LURBAN for nBB’ i

closest to but

≤ nTARGET BB’ for i=1,2

(3.1.3.)

LURBAN

(3.1.3.)

LURBAN 1 for vBB’1

LURBAN 2 for vBB’2

(3.1.3.)

LURBAN

for nBB’   closest to

nTARGET BB’

(3.1.3.)

One test

conditions

Two test

conditions

Two test

conditions

vTARGET BB’ = No

vTARGET BB’ = Yes

If (3.1.2.2.1.2. c)

Calculate average (3.1.3.2.)

vTARGET BB’ = No

nTARGET BB’ = No

f

If nBB’ i > nTARGET BB’

# Figure 5c*

# Flowchart for vehicles tested according to paragraph 3.1.2.2. of Annex 3 to this Regulation – 
Test in non-locked gears PART 2

One test

condition

Final result (3.1.3.2.)

Test criteria

(3.1.2.2.1.2. f)

vBB’ = vTARGET BB’

vTARGET BB’ = Limited

by

gear selection D

LURBAN

(3.1.3.)

f

# Figure 5d*

# Flowchart for vehicles tested according to paragraph 3.1.2.2. of Annex 3 to this Regulation – 
Test for no- combustion engine speed available

Select test method (3.1.2.2.1.)

Determine vehicle load (2.2.1.)

No rotational engine speed available (3.1.2.2.1.3.)

Fulfilling target conditions (3.1.2.2.)?

Establish target conditions (3.1.2.2.)

One test

condition

Final result (3.1.3.2.)

Test criteria

(3.1.2.2.1.3. a)

vBB’ = vTARGET BB’

Test criteria

(3.1.2.2.1.3. b)

25 km/h ≤ vBB’1 ≤ 35 km/h

35 km/h ≤ vBB’2 ≤ 45 km/h

Test criteria

(3.1.2.2.1.3. c)

35 km/h ≤ vBB’2 ≤ 45 km/h

vTARGET BB’ = Yes

LURBAN

(3.1.3.)

LURBAN 1 for vBB’1

LURBAN 2 for vBB’2

(3.1.3.)

LURBAN 2 for vBB’2

(3.1.3.)

One test

conditions

Two test conditions

vBB’1 = No

Calculate average (3.1.3.2.)

vTARGET BB’ = No

If

Note:

*	Broaden the window for the target vehicle speed vTARGET BB’ for vehicles of category M2>3,500 kg technically permissible maximum laden mass and for vehicles of category N2, M3 and N3.

The target vehicle velocity vTARGET BB’ is defined as vTARGET BB’ = 35 km/h ± 5 km/h which results in a window for the velocity vBB’, when the reference point passes line BB’, from 30 km/h to 40 km/h. If the target vehicle velocity vTARGET BB’ is changed into two target vehicle velocities, a lower and a higher one, the following is meant: The lower target vehicle velocity is defined as the target vehicle velocity vTARGET BB’ reduced by 5 km/h (v TARGET BB’ - 5 km/h) which results in a window for the velocity vBB’1, when the reference point passes line BB’, from 25 km/h to 35 km/h.

25 km/h ≤ vBB’1 ≤ 35 km/h.

The higher target vehicle velocity is defined as the target vehicle velocity vTARGET BB’ increased by 5 km/h (vTARGET BB’ + 5 km/h) which results in a window for the velocity vBB’2, when the reference point passes line BB’, from 35 km/h to 45 km/h.

35 km/h ≤ vBB’2 ≤ 45 km/h.

Figure 6

Flowchart for measurement and data processing of stationary sound according to paragraph 3.2.

Stationary sound Measurements according to paragraph 3.2.

Setup of the vicinity and vehicle according paragraphs 3.2.1. to 3.2.4.

Stationary sound measurement at a test position (outlet) according para. 3.2.5.

Multiple

test positions

available?

NO

All test positions

are measured?

NO

NO

NO

Driver

selectable modes

available?

YES

Outlet(s)

are measured in all

modes?

Select next mode, repeat all measurements at any test position (outlet)

YES

Report sound pressure level according to paragraph 3.2.6.1.

YES

Select next test position (outlet) for measurement.

YES

Report for each mode the representative sound pressure level

Report highest sound pressure level of all modes and outlets as representative sound pressure level for the vehicle type

Report sound pressure level for according to paragraph 3.2.6.2.

Table 1

# Examples for Devices and Measures to Enable a Vehicle Tested within the Acceleration Boundaries

| No. | Impact | Sub No. | Measure | Additional Requirements |
| --- | --- | --- | --- | --- |
| 1 | Lock of a discrete gear ratio | 1* | A discrete gear ratio can be locked by the driver | none |
|  |  | 2 | A discrete gear ratio is available onboard, but is not available to the driver. Locking can be activated by the manufacturer with an onboard (hidden) function or with an external device | none |
| 2 | Controlled gear shift management: Applicable to transmissions which cannot be locked, or where no locked gear provides a valid test result | 1* | Kickdown is deactivated | none |
|  |  | 2 | Gear shift change(s) can happen during the test, gear shift is controlled by activation of an internal function or external device | Acceleration** shall be between aURBAN and aACC REF, not exceeding 2.0 m/s². |
| 3 | Defined partial load driving**** | 1 | Acceleration is limited by a mechanical device | Defined acceleration** shall be between aURBAN and aACC,REF, not exceeding 2.0 m/s². For RD-ASEP**, the reference point parameter is calculated by: LACC ANCHOR = (LACC - kP*LCRS) / (1-kP) where kP = 1-(aURBAN/aACC TEST) nBB’ ACC ANCHOR = nBB' TEST*3.6/vBB' TEST *(aACC REF *(20+2*l)+192,9)0,5 |
|  |  | 2 | External Programming for partial load acceleration*** |  |
| 4 | Mixed Solution (Mode): This measure will be a mix of the above solutions combined in a specific mode | 1* | Mode is available onboard and can be selected by the driver | none |
|  |  | 2 | Mode is available onboard and can only be activated by the manufacturer with a hidden function or an external device | none |
|  |  | 3 | Mode is not available onboard, an external software overrides the internal software | Acceleration** shall be between aURBAN and aACC,REF, not exceeding 2.0 m/s². |

*	Comment: This is a standard situation, already covered by the Regulation text.

** 	Applicable to vehicles of category M1, N1 and M2 ≤ 3,500 kg.

*** 	Partial load shall be achieved by simulation of the travel restriction of the accelerator. It is not allowed to interfere with the engine control management.

****	Applicable to vehicles of categories M1, N1 and M2 ≤ 3,500 kg, for the further calculation of LURBAN in Annex 3 the sound level measured under partial load shall replace the sound level under acceleration test.

The achieved acceleration under partial load during the test shall be used for the calculation of the part power factor kP instead of aACC REF.

The test procedure and the data processing follow the same principle. Although it is tested in partial load, symbols xACC (e.g. LACC, aACC, …) shall be used.

Annex 4

Exhaust silencing systems containing acoustically absorbing fibrous materials

1.	General

Sound absorbing fibrous materials may be used in silencing systems or components thereof only if

(a)	The exhaust gas is not in contact with the fibrous materials; or if

(b)	The silencing system or components thereof are of the same design family as systems or components for which it has been proven, in the course of type approval process in accordance with the requirements of this Regulation for another vehicle-type, that they are not subject to deterioration.

Unless one of these conditions is fulfilled, the complete silencing system or components thereof shall be submitted to a conventional conditioning using one of three installations and procedures described below, or - on request of the manufacturer - by removing the fibrous materials from the silencer.

1.1.	Continuous road operation for 10,000 km

1.1.1.	50 ± 20 per cent of this operation shall consist of urban driving and the remaining operation shall be long-distance runs at high speed; continuous road operation may be replaced by a corresponding test-track programme.

1.1.2.	The two speed regimes shall be alternated at least twice.

1.1.3.	The complete test programme shall include a minimum of 10 breaks of at least three hours duration in order to reproduce the effects of cooling and any condensation which may occur.

1.2.	Conditioning on a test bench

1.2.1.	Using standard parts and observing the vehicle manufacturer's instructions, the silencing system or components thereof shall be fitted to the vehicle referred to in paragraph 3.3. of this Regulation or the engine referred to in paragraph 3.4. of this Regulation. In the former case the vehicle shall be mounted on a roller dynamometer. In the second case, the engine shall be coupled to a dynamometer.

1.2.2.	The test shall be conducted in six six-hour periods with a break of at least 12 hours between each period in order to reproduce the effects of cooling any condensation which may occur.

1.2.3.	During each six-hour period, the engine shall be run, under the following conditions:

(a)	Five minutes at idling speed;

(b)	One-hour sequence under 1/4 load at 3/4 of rated maximum speed (S);

(c)	One-hour sequence under 1/2 load at 3/4 of rated maximum speed (S);

(d)	10-minute sequence under full load at 3/4 of rated maximum speed (S);

(e)	15-minute sequence under 1/2 load at rated maximum speed (S);

(f)	30-minute sequence under 1/4 load at rated maximum speed (S).

Each period shall comprise two sequenced sets of the six above-mentioned conditions in consecutive order from (a) to (f).

1.2.4.	During the test, the silencing system or components thereof shall not be cooled by a forced draught simulating normal airflow around the vehicle. Nevertheless, at the request of the manufacturer, the silencing system or components thereof may be cooled in order not to exceed the temperature recorded at its inlet when the vehicle is running at maximum speed.

1.3.	Conditioning by pulsation

1.3.1.	The silencing system or components thereof shall be fitted to the vehicle referred to in paragraph 3.3. of this Regulation or the engine referred to in paragraph 3.4. of this Regulation. In the former case the vehicle shall be mounted on a roller dynamometer.

In the second case, the engine shall be mounted on a dynamometer. The test apparatus, a detailed diagram of which is shown in Figure 1 of the appendix to this annex shall be fitted at the outlet of the silencing system. Any other apparatus providing equivalent results is acceptable.

1.3.2.	The test apparatus shall be adjusted in such a way that the exhaust-gas flow is alternatively interrupted and re-established by the quick-action valve for 2,500 cycles.

1.3.3.	The valve shall open when the exhaust-gas back pressure, measured at least 100 mm downstream of the intake flange, reaches a value of between 35 and 40 kPa. It shall close when this pressure does not differ by more than 10 per cent from its stabilized value with the valve open.

1.3.4.	The time-delay switch shall be set for the duration of gas exhaust resulting from the provisions laid down in paragraph 1.3.3. above.

1.3.5.	Engine speed shall be 75 per cent of the rated engine speed (S) at which the engine develops rated maximum net power.

1.3.6.	The power indicated by the dynamometer shall be 50 per cent of the full-throttle power measured at 75 per cent of rated engine speed (S).

1.3.7.	Any drain holes shall be closed off during the test.

1.3.8.	The entire test shall be completed within 48 hours.

If necessary, one cooling period will be observed after each hour.

Annex 4 - Appendix

# Figure 1

Test apparatus for conditioning by pulsation

1.	Inlet flange or sleeve for connection to the rear of the test exhaust system.

2.	Hand-operated regulating valve.

3.	Compensating reservoir with a maximum capacity of 40 l and a filling time of not less than one second.

4.	Pressure switch with an operating range of 0.05 to 2.5 bar.

5.	Time delay switch.

6.	Pulse counter.

7.	Quick-acting valve, such as exhaust brake valve 60 mm in diameter, operated by a pneumatic cylinder with an output of 120 N at 4 bar. The response time, both when opening and closing, shall not exceed 0.5 second.

8.	Exhaust gas evacuation.

9.	Flexible pipe.

10.	Pressure gauge.

Figure 2: Flowchart for the check on the necessity for a silencer conditioning test

Check on Necessity for a Silencer Conditioning

Does the silencer
contain a fibrous

material?

Provide documentation that the silencer is belonging to a family of silencers for which it was proven that the fibrous material will not deteriorate.

Conditioning test IS necessary

Conditioning test

IS NOT necessary

NO

YES

Is the fibrous material in contact with exhaust gas?

NO

YES

Is the silencer member of a design family for which it has been proven that the fibrous material will not deteriorate?

Provide information about the family representative silencer and the way of proof for the non-deterioration

Proceed to the 
sound emission tests

YES

NO

Perform either a conditioning test or – on request of the manufacturer – remove the fibrous material

Annex 5

Compressed air noise

1.	Method of measurement

The measurement is performed at microphone positions 2 and 6 according to Figure 1, with the vehicle stationary. The highest A-weighted sound level is registered during venting the pressure regulator and during ventilating after the use of both the service and parking brakes.

The noise during venting the pressure regulator is measured with the engine at idling speed. The ventilating noise is registered while operating the service and parking brakes; before each measurement, the air-compressor unit has to be brought up to the highest permissible operating pressure, and then the engine switched off.

2.	Evaluation of the results

For all microphone positions two measurements are taken. In order to compensate for inaccuracies of the measuring equipment, the meter reading is reduced by 1 dB(A), and the reduced value is taken as the result of measurement. The results are taken as valid if the difference between the measurements at one microphone position does not exceed 2 dB(A). The highest value measured is taken as the result. If this value exceeds the sound limit by 1 dB(A), two additional measurements are to be taken at the corresponding microphone position.

In this case, three out of the four results of measurement obtained at this position have to comply with the sound limit.

3.	Limiting value

The sound level shall not exceed the limit of 72 dB(A).

Annex 5 - Appendix

# Figure 1

# Microphone positions for measurement of compressed air noise

The measurement is performed at the stationary vehicle according to Figure 1, using two microphone positions at a distance of 7 m from the contour of the vehicles, and at 1.2 m above ground.

Annex 6

Checks on conformity of production

1.	General

These requirements are consistent with the test for checking Conformity of Production (COP) according to paragraph 8. of this Regulation.

2.	Testing procedure

The test site and measuring instruments shall be those as described in Annex 3.

2.1.	The vehicle(s) under test shall be subjected to the test for measurement of sound of vehicle in motion as described in paragraph 3.1. of Annex 3.

For vehicles of category M1, N1 and M2 ≤ 3,500 kg technically permissible maximum laden mass,

- the same mode, gear(s)/gear ratio(s), gear weighting factor k and partial power factor kP as determined during the type approval process shall be used, provided this information are available from the type approval test report for the applicable vehicle variant of the family. If not, this information shall be determined anew. The test report shall document which way of data processing was selected;

- the test mass mT of the vehicle shall be between 0.9mRO ≤ mT ≤ 1.2mRO

2.2.	Compressed air noise

Vehicles having maximum mass exceeding 2,800 kg and equipped with compressed air systems shall be subjected to an additional test for measurement of the compressed air noise as described in paragraph 1. of Annex 5.

2.3.	Real Driving Additional Sound Emission Provisions (RD-ASEP)

The vehicle manufacturer shall assess the compliance with RD-ASEP by an appropriate evaluation. This can be for example, but not limited to, part checks, test as described in Annex 7 either outdoors or indoors as described in Annex 8, or by virtual assessment.

If test according to Annex 7 are carried out, they shall be randomised over the whole control range of RD-ASEP and shall be at least 3 runs and not more than 5 runs.

3.	Sampling and evaluation of the results

One vehicle shall be chosen and subjected to the tests set out in point 2. If the sound level of the vehicle tested does not exceed by more than 1 dB(A) the limit value specified in paragraph 6.2.2. and 6.2.3.  of this Regulation, and, where appropriate, paragraph 3. of Annex 5, the vehicle type shall be considered to conform to the requirements of this Regulation.

If one of the test results does not fulfil the COP requirements of this annex and of paragraph 8. of the main body of this Regulation two more vehicles of the same type shall be tested pursuant to paragraph 2. above.

If the test results for the second and the third vehicle fulfil the COP requirements of this annex and of paragraph 8. of the main body of this Regulation, the vehicle is considered in compliance with regard to the COP.

If one of the test results of the second or third vehicle does not fulfil the COP requirements of this annex and of paragraph 8. of the main body of this Regulation the vehicle type shall be considered not to conform to the requirements of this Regulation and the manufacturer shall take the necessary measures to re-establish the conformity.

Annex 7

Measurement method to evaluate compliance with the Real Driving Additional Sound Emission Provisions (RD-ASEP)

This annex is applicable to vehicles as specified in paragraph 6.2.3. of this Regulation.

- General

Any vehicle operating conditions within the control range, as specified in paragraph 2.3. of Annex 7 are typical on road driving conditions and are covered by RD-ASEP.

Exterior sound enhancement system(s) shall operate as designed during the type-approval test and not be interfered with.

- Facilities

Due to limitations of test facilities not every test condition may be performed on every test facility.

Notwithstanding such restrictions, the type approval shall be granted on these test facilities, however the vehicle has to comply to all provisions of this Annex 7. In these cases, the vehicle manufacturer shall explain to the satisfaction of the authority present at type approval that the vehicle fulfils the requirements which could not be tested due to the restriction of the test facility.

Tests for Annex 7 may be carried out on different test facilities in case of facility limitation(s). However, it is recommended to carry out all tests on one test facility and under similar environmental conditions to reduce measurement uncertainties.

3.	Measurement method

3.1.	Measurement instruments and condition of measurements

Unless otherwise specified, the measurement instruments, the conditions of the measurements and the condition of the vehicle are equivalent to those specified in Annex 3, paragraphs 1. and 2.

3.2.	Method of testing

Unless otherwise specified, the conditions and procedures of Annex 3, paragraphs 3.1. to 3.1.2.1.2.2. shall be used. For the purpose of this annex, single test runs are performed and evaluated.

3.3.	Control range

A measurement for RD-ASEP is valid, if all parameters are within the specifications of the table below during the test run between lines AA' and BB'.

| Parameter | Minimum | Maximum |
| --- | --- | --- |
| Vehicle Speed | > 0 km/h at line AA' | 100 km/h at line BB' |
| Acceleration | 0 m/s² | 4 m/s² |
| Performance | 0 m²/s³ | 35 m²/s³ |
| Gear | ANY for forward driving |  |
| Mode | ANY |  |

In any operation condition, the engine speed of a vehicle, which can be propelled with an ICE operating, is limited to 80% of S.

3.4.	Target operation conditions

The target operation condition for a single test run is randomly selected by the authority present during the tests carried out for type approval.

The operation condition during measurements per run is defined by

- the gear selector position,

- the vehicle mode,

- the vehicle entry speed at line AA', and

- the percentage of accelerator depression, either for constant speed or in steps of 25% acceleration depression.

The manufacturer may establish a mechanical or electronic device to enable the requested percentage of accelerator depression.

The requested accelerator depression shall be achieved during the test with a tolerance of ±10% of full range.

The requested vehicle speed at AA' shall be achieved during the test with a tolerance of ±3 km/h.

If under the chosen operation condition a stable acceleration according to 2.26.1. cannot be enabled, the authority present during the test shall decide how to modify the operation condition (e.g. different gear selector position, speed, acceleration, driving mode).

The chosen operation condition shall result in a run within the control range. Each operation condition shall differ substantially from the test conditions of Annex 3 and all other operation conditions already chosen for this type approval already carried out under this Annex. For vehicles with a combustion engine for propulsion, the operating conditions shall be chosen to differ substantially in engine speed.

The number of operation conditions per vehicle in total is dependent on the vehicles technology and provided by the table below.

|  | D-Range | M (locked) |
| --- | --- | --- |
| Automatic Transmissions (lockable) | [5] | [10] |
| Automatic Transmissions (non-lockable) | [15] | n.a. |
| Vehicles with only one gear | [15] | n.a. |
| Manual Transmissions | n.a. | [15] |

(to be revised after refinement of the sound model)

The operation conditions and test results shall be entered into the test report sheet according to the table of the Appendix 3 to this Annex.

The first operation condition shall be selected according to the principles outlined in paragraph 3.6.1. of the Appendix 1 to determine the maximum reference acceleration aMAX REF.

3.5.	Test of the vehicle

3.5.1. 	The path of the centerline of the vehicle shall follow line CC' as closely as possible throughout the entire test, starting from the approach of the reference point according to definition in paragraph 2.11. of the main body to line AA' until the rear of the vehicle passes line BB' + 20 m.

The accelerator shall be positioned such to achieve the requested operation condition for this run latest when the reference point of the vehicle reaches line AA'. The accelerator shall be kept in its position until the rear of the vehicle passes line BB'. The accelerator shall then be fully released between BB' and BB'+ 5 m and kept in this released position until the rear of the vehicle passes the line BB'+20 m.

3.5.2. 	Non-locked transmissions

In case of non-locked transmission conditions, the test may include a gear change to a lower range and a higher acceleration under accelerated conditions.

A gear change to a higher range could occur under cruise and low load acceleration conditions. Such upshifts shall be avoided. The authority present during the test shall modify operation conditions so that these upshifts are avoided between AA' and BB'.

In the approach to line AA', the vehicle shall be driven in a way to allow the transmission to stabilize the gear.

3.5.3. 	Measurement readings

Per operation condition, one test run is carried out.

If a measurement within the control range is invalid due to background noise disturbances, wind gusts or other reasons, the measurement shall be discarded and repeated.

For every test run, the following parameters shall be determined and noted:

- The maximum A-weighted sound pressure level of both sides of the vehicle, indicated during each passage of the vehicle between the two lines AA' and BB' + 20 m, shall be measured and shall be mathematically rounded to the first decimal place (LTEST ACC κj). (symbol to be checked)

If a sound peak obviously out of character with the general sound pressure level is observed, the measurement shall be discarded.

For further processing, the higher sound pressure level of both sides shall be used.

- The vehicle speed readings at lines AA' and PP’, when the vehicle reference point passes these lines, shall be rounded and reported with the first significant digit after the decimal place (vAA' TEST κj; vPP' TEST κj). (symbols to be checked)

- The vehicle speed reading at lines BB', when the rear end of the vehicle passes this line, shall be rounded and reported with the first significant digit after the decimal place (vBB’ TEST κj). (symbols to be checked)

- If applicable, the engine speed readings at line AA' and BB' shall be rounded to 10 min-1 and reported (nAA' TEST κj nBB' TEST κj). (symbol to be checked)

All measured values shall be entered into the test report sheet according to the table of Appendix 3 of this Annex.

3.5.4 . 	Calculated values

All calculated values shall be entered into the test report sheet according to the table of Appendix 3 of this Annex.

3.5.4.1. 	Acceleration a

The accelerations shall be calculated between lines PP' to BB', in accordance to the formula provided in paragraph 3.1.2.1.2.2. of Annex 3 and be reported to the second digit after the decimal place (aTEST) as results. (symbol to be checked)

3.5.4.2. 	Performance va

The performance shall be calculated from the reported vehicle speed at line BB' and the acceleration result from paragraph 3.5.4.1. and rounded to the first digit after the decimal place.

3.5.4.3. 	Expected sound pressure level LEXP

For the calculation of the expected sound pressure level, the measured values according to paragraph 3.5.3. and calculated values according to paragraphs 3.5.4.1. and 3.5.4.2. shall be used. All calculations are done according to Appendix 1 to this Annex.

4.	Compliance assessment

4.1.	Case 1

The compliance of the vehicle is acceptable if the measured sound pressure levels of all valid test runs are lower than or equal to the expected sound pressure levels of paragraph 3.5.4.3. (rounded to integer needed or not  to be decided later)

LTEST  ≤ LEXP

4.2.	Case 2

If not more than [two] valid runs of the specified runs exceed the expected sound pressure level of paragraph 3.5.4.3. by not more than [2] dB(A) the compliance of the vehicle is acceptable.

4.3.	Case 3

If more than [two] valid runs of the specified runs exceed the expected sound pressure level of paragraph 3.5.4.3. then the vehicle is non-compliant with RD-ASEP.

4.4.	Case 4

No valid run shall exceed the expected sound pressure level of paragraph 3.5.4.3. by more than [2 dB(A)]. If this is the case, the vehicle is non-compliant with RD-ASEP.

Annex 7 – Appendix 1 – Sound Expectation Model

- General

xxxxx

- Extraction of parameters from measurements according to Annex 3

  - The procedure set out in this Annex requires the performance of tests in accordance with Annex 3.

For a better understanding on the procedure, guidance is provided by the flowchart 2 in the Appendix 4 to Annex 7.

  - Determination of reference data from Annex 3:

    - The necessary reference data to establish the sound expectation model are taken from the pass-by and from the cruise-by test of one gear of the Annex 3 tests.

      - In the case the test has been carried out with two gear ratios, the reported parameters for gear i shall be selected. In the case the test has been carried out in a single gear, the reported parameters for this single gear shall be selected.

      - The Parameters taken from Annex 3 tests are in all cases the arithmetic average of the four valid runs as determined by the measurements in Annex 3. All values shall be overtaken without any temperature or test track correction. Engine speed is not requested for Annex 3. However, for the purpose of Annex 7 it needs to be measured.

The parameters to be reported from the acceleration test are:

- The sound pressure level LACC ANCHOR  which is the higher value of the measured sound pressure levels of the left and right side of the vehicle, rounded to the first decimal.

- [Use of measures specified in paragraph 3.1. or 3.2. and described in Table 1 of Annex 3 (Yes/No)

- If the acceleration test has been carried out under partial load according to the measures 3.1 or 3.2 described in table 1 to the Appendix of Annex 3. ]

- The vehicle speed vBB' ACC ANCHOR when the rear of the vehicle passes line BB', rounded to the first decimal.

- The engine speed nBB' ACC REP when the rear of the vehicle passes line BB', rounded to 10 min-1.

- The sound level shall be corrected …

- The acceleration …

The parameters to be reported from the constant speed test are:

- The sound pressure level LCRS ANCHOR which is the higher value of the measured sound pressure levels of the left and right side of the vehicle, rounded to the first decimal.

- The reference vehicle speed vREF is 50 km/h, unless the vehicle was tested in Annex 3 at a different vehicle speed. In this case use the reported vehicle speed vBB' CRS, rounded to the first decimal.

- The engine speed nBB' CRS ANCHOR when the rear of the vehicle passes line BB', rounded to 10 min-1.

  - Selection of parameter coefficients 	
The necessary coefficients are dependent on the vehicle design and listed in the table in the Appendix 2 to this Annex.

    - Discrete Determination of the factor x

On request of the manufacturer the x – factor may be determined by discrete coast-down measurement to determine LREF TR directly, according to Appendix 2 of Annex 3 to this Regulation for the reference vehicle speed. Rounding and temperature correction shall not apply.

]

  - Calculate the Reference Tyre/Rolling Sound Level LREF TR

LREF TR=10 ×log(x*100.1LCRS ANCHOR )

  - Calculate the Reference Power Train Mechanics Sound Level LREF PT

LREF PT=10 ×log((1-x)*100.1 LCRS ANCHOR)

  - Calculate the Reference Dynamic Sound Level LREF DYN

LREF DYN=LREF PT-15

  - Determine the Vehicle Dynamic Delta Sound Level ∆LDYN

If the arithmetic sound level difference between the reported acceleration sound level LACC ANCHOR and the reported constant speed sound level LCRS ANCHOR is at least 1.1 dB(A) or higher, the vehicle dynamic delta sound level ∆LDYN is  calculated by

∆LDYN=10 ×log100.1 LACCANCHOR-100.1 LREF TR ADJ - 100.1LREF PT ADJ-LREF DYN

with

LREF TR ADJ=TR LO * log(vBB'ACC ANCHOR50vREF)+ LREF TR

LREF PT ADJ=PT LO * log(nBB'ACC ANCHOR+nSHIFT PT)(nBB'CRS ANCHOR + nSHIFT PT)+ LREF PT

If the arithmetic sound level difference between the reported acceleration sound level LACC ANCHOR and the reported constant speed sound level LCRS ANCHOR is less than 1,1 dB(A), the vehicle dynamic delta sound level ∆LDYN is set to 10 dB(A).

∆LDYN=10 dBA

  - Having established the sound expectation model for a given vehicle based on its particular pass-by test results according to Annex 3 of this Regulation; proceed to the single point evaluation for each test run performed according to paragraphs 3.4 and 3.5 of Annex 7.

- Calculation of the expected sound level LTEST EXP

  - For each single test run performed, for the purpose of Annex 7, an expected sound level LTEST EXP shall be calculated.

The flowchart 3 in the Appendix 4 to Annex 7 provides guidance through the necessary calculation steps provided by the following paragraphs.

  - Necessary input data for the sound model are taken from the pass-by measurement according to paragraph 3.5.1. of Annex 7.

    - For the calculation of the expected sound level the parameters listed in paragraphs 3.5.3. and 3.5.4.1 and 3.5.4.2 of Annex 7 are needed.

In addition, the vehicle speed to engine speed ratio κTEST, expressed in km/h per 1000 min-1 and calculated by the formula below, rounded to the second decimal

κiTEST=vBB' TESTnBB' TEST×1000

The vehicle speed to engine speed ratio κTEST  of the first operation condition according to paragraph 3.6.1 of Annex 7 is defined as reference vehicle speed to engine speed ratio κREF.

    - Virtual engine speed for vehicles without internal combustion engine

When testing vehicles without an internal combustion engine for direct forward propulsion, an engine speed will not be available. In such cases the engine speed is simulated on the basis of the measured vehicle speed vBB' TEST by using a virtual uniform gear ratio of [30] km/h per 1000 min-1.

nBB' TEST=vBB' TEST30×1000

    - Virtual engine speed for hybrid electric vehicles

In case that an internal combustion engine is mechanically coupled with drive axel whenever internal combustion engine is operating, this paragraph shall be applied.

In case of the other HEV systems, paragraph 3.2.4. shall be used.

Hybrid electric vehicle may have been tested in Annex 3 partly or fully in electric condition. For evaluation according RD-ASEP, engine speeds and, if applicable corrected sound pressure levels, will have to be assigned to the cruise and the acceleration test.

      - Case 1 – Internal combustion engine is operational during acceleration test and constant speed test:

        - Assignment of engine speed	
An engine speed information is available for both test results.

        - Correction of sound pressure level	
No correction is applied.

      - Case 2 – Internal combustion engine is operational during acceleration test but not during the constant speed test:

        - Assignment of engine speed	
An engine speed information is available for the acceleration test result. For the constant speed test, the engine speed is calculated with the lowest transmission gear ratio that can be engaged at the target speed of the vehicle vTEST as selected for the constant speed test in Annex 3.

        - Correction of sound pressure level	
No correction is applied to the acceleration test result. The corrected cruise test result LCRS ANCHOR' is determined by

LCRS ANCHOR'=LCRS ANCHOR+0.5 dB

      - Case 3 – Internal combustion engine is operational during constant speed test but not during the acceleration test

        - Assignment of engine speed	
An engine speed information is available for the constant speed test. 	

The engine speed for the acceleration test is calculated with the lowest transmission gear ratio that provides an acceleration when the internal combustion engine and the electric propulsion power are operational greater than the reference acceleration aACC REF or an acceleration not exceeding 2.0 m/s², whichever is lower.

        - Correction of sound pressure level to remove tire sound component	
No correction is applied to the constant speed test result.

The sound pressure level for the acceleration test is determined by

LACC ANCHOR'=Limit-kP×LCRS ANCHOR/(1-kP)

where Limit is the applicable limit value for this vehicle type and kP is the determined kP-factor from the Annex 3 test.

Limit: see the limit value in the table of paragraph 6.2.2. of main body to this Regulation for the valid phase.

      - Case 4 – Internal combustion engine does not operate in both acceleration test and constant speed test

        - Assignment of engine speed	
The engine speed for the constant speed test is calculated with the lowest transmission gear ratio that can be engaged at the target speed of the vehicle vTEST as selected for the constant speed test in Annex 3.	
[The engine speed for the acceleration test is calculated with the lowest transmission gear ratio that provides an acceleration when the internal combustion engine and the electric propulsion power is operational greater than the reference acceleration aACC REF or an acceleration not exceeding 2.0 m/s², whichever is lower.]

        - Correction of sound pressure level	
The sound pressure level for the constant speed test is determined by

LCRS ANCHOR'=LCRS ANCHOR+0.5 dB

The sound pressure level for the acceleration test is determined by

LACC ANCHOR'=Limit-kP×LCRS ANCHOR'/(1-kP)

where Limit is the applicable limit for this vehicle type and kP is the determined kP-factor from the Annex 3 test.

3.2.4               Virtual engine speed for hybrid electric vehicle the other system than paragraph 3.2.3.)

    - Virtual engine speed for hybrid electric vehicles (series HEV)

Hybrid electric vehicle may have been tested in Annex 3 partly or fully in electric condition. For evaluation according RD-ASEP, virtual engine speeds shall be defined due to independent from vehicle speed. And, if applicable corrected sound pressure levels, will have to be assigned to the cruise and the acceleration test.

      - Case 1 – Internal combustion engine is operational during acceleration test and constant speed test

        - Assignment of engine speed	
An engine speed information is available for both test results.

        - Correction of sound pressure level	
No correction is applied

      - Case 2 – Internal combustion engine is operational during acceleration test but not during the constant speed test

        - Assignment of engine speed	
The engine speed for the constant speed test is calculated with a virtual uniform gear ratio of [30] km/h per 1000 min-1 at the target speed of the vehicle vTEST as selected for the constant speed test in Annex 3.  For the acceleration test, an engine speed information is available in the test result.

        - Correction of sound pressure level	
No correction is applied to the acceleration test result. The corrected cruise test result LCRS,ANCHOR' is determined by

LCRS ANCHOR'=LCRS ANCHOR+0.5 dB

      - Case 3 – Internal combustion engine is operational during constant speed test but not during the acceleration test

        - Assignment of engine speed	
An engine speed information is available for the constant speed test. 	
The engine speed for the acceleration test is calculated with a virtual uniform gear ratio of [20] km/h per 1000 min-1.

        - Correction of sound pressure level	
No correction is applied to the constant speed test result.

The sound pressure level for the acceleration test is determined by

LACC ANCHOR'=Limit-kP×LCRS ANCHOR/(1-kP)

where Limit is the applicable limit for this vehicle type and kP is the determined kP-factor from the Annex 3 test.

      - Case 4 – Internal combustion engine is neither operational during the acceleration test nor during the constant speed test

        - Assignment of engine speed	
The engine speed for the constant speed test is calculated with a virtual uniform gear ratio of [30] km/h per 1000 min-1 at the target speed of the vehicle vTEST as selected for the constant speed test in Annex 3.	
The engine speed for the acceleration test is calculated with a virtual uniform gear ratio of [20] km/h per 1000 min-1.

        - Correction of sound pressure level	
The sound pressure level for the constant speed test is determined by

LCRS ANCHOR'=LCRS ANCHOR+0.5 dB

The sound pressure level for the acceleration test is determined by

LACC ANCHOR'=Limit-kP×LCRS ANCHOR'/(1-kP)

where Limit is the applicable limit for this vehicle type and kP is the determined kP-factor from the Annex 3 test.

  - Calculation of expected tyre rolling sound component LTR EXP

The expected tyre rolling sound component LTR EXP is calculated dependent on the achieved vehicle speed vBB' TEST during the test.

For vehicles speeds up to and inclusive vREF, LTR EXP is calculated by

LTR EXP=θTR LO×logvBB'TESTvBB'CRS+LREF TR with θTR LO=[20]

For vehicle speeds vBB' TEST exceeding vREF, LTR EXP is calculated by

LTR EXP=θTR HI×logvBB'TESTvBB'CRS+LREF TR with θTR HI=[40]

  - Calculation of expected power train mechanical sound component LPT EXP

The expected power train base mechanical sound component LPT EXP is calculated dependent on the achieved engine speed nBB' TEST during the test.

For engine speeds up to and inclusive nBB' CRS ANCHOR, LPT EXP is calculated by	LPT EXP=θPT LO×lognBB' TEST+nSHIFTnBB'CRS ANCHOR+nSHIFT+LREF PT 
with	θPT LO=[60]

nSHIFT PT=[5000]

For engine vehicle speeds exceeding nBB' CRS ANCHOR vBB' TEST exceeding 50 km/h, LPT EXP is calculated by

LPT EXP=θPT HI×lognBB' TEST+nSHIFTnBB'CRS ANCHOR+nSHIFT+LREF PT
with	θPT HI=[115]

nSHIFT PT=[5000]

  - Calculation of expected base dynamic sound component LDYN EXP

The expected base dynamic sound component LDYN EXP is calculated dependent on the achieved engine speed nBB' TEST during the test.

For engine speeds up to and inclusive nBB' ACC ANCHOR, LDYN EXP is calculated by

LDYN EXP=θDYN LO×lognBB' TEST+nSHIFTnBB'ACC ANCHOR+nSHIFT+LREF DYN

With	θDYN LO=50

nSHIFT PT=5000

For engine vehicle speeds exceeding nBB' ACC ANCHOR vBB' TEST exceeding 50 km/h, LDYN EXP is calculated by

LDYN EXP=θDYN HI×lognBB'TEST+nSHIFTnBB'CRS ANCHOR+nSHIFT+LREF DYN

with	θDYN HI=[105]

nSHIFT PT=[5000]

  - Calculation of expected dynamic delta sound component ∆LDYN EXP

    - Determination of the maximum reference acceleration aMAX REF

      - The maximum reference acceleration aMAX REF is the maximum acceleration performance determined in a low gear under full load condition.

A test run shall be performed in a gear ratio and at a vehicle entry speed such, that the vehicle engine speed nBB' TEST is between the 50% of S and 80% of S.

This operation condition is determined by the vehicle manufacturer in agreement with the authority present during the test.

The acceleration aMAX REF is calculated according to the provisions of paragraph 3.5.4.1. of Annex 7.

    - Calculation of the partial load LOADTEST achieved during the test run

The partial load normalized with the maximum load is calculated based on the achieved acceleration aTEST, relative to the reference acceleration aMAX i with the formula below

LOADTEST=aTESTaMAX i where aMAX i=κiREF GEARκiGEARTEST REF×aMAX REF

    - Performance related calculations

      - Calculation of the performance vaTEST

The performance achieved during the test is calculated from the achieved acceleration expressed in m/s² and the vehicle speed expressed in km/h by	

vaTEST=vBB' TEST3,6×aTEST   [m²/s³]

      - Calculation of the dynamic performance component ∆LDYN va

The dynamic performance component of the vehicle dynamic sound is calculated based on the achieved performance vaTEST relative to a reference performance.

If the achieved performance does not exceed the reference performance vaREF according to the parameter coefficient table 1, the dynamic performance component ∆LDYN va is equal zero.

If the achieved performance exceeds the reference performance, the dynamic performance component ∆LDYN va is calculated by

∆LDYN va=β× logvaTESTvaREF

with

β = [8]

The maximum dynamic performance component ∆LDYN va is limited to 8 dB(A)

      - Aggregation of dynamic sound components

The final dynamic delta sound component ∆LDYN,EXP dynamic is calculated by

∆LDYN EXP=∆LDYN+∆LDYN va×1-αLOADTEST+α1-α

With α=[0,111]

  - Calculation of the expected sound level LTEST EXP

The calculation results of the paragraphs 3.3. to 3.6. are used to calculate the expected  sound level for an individual run to be compared with the measured maximum sound pressure level, by the following formula
LTEST EXP=10×log100,1×LTR EXP+100,1×LPT EXP+100,1×LDYN EXP+∆LDYN EXP+2dB(A)

  - Proceed with the compliance assessment according to paragraph 4 of Annex 7

Annex 7 – Appendix 2 – Statement of Compliance with the Additional Sound Emission Provisions

[to be completed]

|  |  |  |  | M1/N1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model Part | Parameter | Symbol | Unit | ICE | BEV FCEV PEV | HEV PMR < 25 |
| TYRE | Reference Vehicle Speed | vREF | km/h | 50 min.40 | 50 min.40 | 50 min.40 40 |
|  | Tyre Rolling Sound Energy Fraction of Annex 3 Cruise Test LCRS ANCHOR | x | % | 90 or measure | 98 | 90 or 50measure |
|  | T/R Sound Slope < 50 km/h | qTR LO | dB/log(v/vREF) | 20 | 20 | 20 |
|  | T/R Sound Slope > 50 km/h | qTR,HI | dB/log(v/vrEF) | 40 | 40 | 40 |
| MECHANIC NO LOAD | P/T Sound Slope < nBB’ CRS’ ANCHOR | qPT LO | dB/Log(n/nREF) | 60 | 60 | 60 |
|  | P/T Sound Slope > nBB’ CRS’ ANCHOR | qPT HI | dB/Log(n/nREF) | 115 | 115 | 115 |
|  | Form Factor for the logarithm function of the mechanic sound model | nSHIFT PT | 1/min rpm | 5000 | 5000 | 5000 |
| DYNAMIC LOAD | Dynamic Sound Slope < nBB’ ACC ANCHOR | qDYN LO | dB/Log(n/nREF) | 50 | 50 | 50 |
|  | Dynamic Sound Slope > nBB’ ACC ANCHOR | qDYN HI | dB/Log(n/nREF) | 105 | 105 | 105 |
|  | Form Factor for the logarithm function of the dynamic sound model | nSHIFT DYN | 1/min rpm | 5000 | 5000 | 5000 |
| DYNAMIC VA | Reference Performance | vaREF | m²/s³ | 28 | 28 | 28 |
|  | Dynamic va Factor |  | dB(A) | 8 | 8 | 8 |
|  | Partial Load Form Factor a | a | --- | 0,111 | 0,111 | 0,111 |
| GENERAL | Base Margin | m | dB(A) | 2 | 2 | 2 |

Annex 7 – Appendix 3 – Test report sheet (example)[to be completed]

|  | Target Condition |  |  |  | Measured Values |  |  |  |  |  |  |  |  |  | Calculated Values |  |  |  | Conformity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Run | Gear Selector Position | Mode | Entry Speed | Accelerator Position | Starting Point of Acceleration | Entry Speed | Microphone Speed | Exit Speed | Max Engine Speed | Exit Engine Speed | Maximum Sound Pressure Level Left Side | Maximum Sound Pressure Level Right Side | Run Valid | Run comment | Acceleration AA'-BB' | Acceleration PP'-BB' | Vehicle Performance | Expected Sound Pressure Level | Vehicle Pass x | Vehicle Pass x + y | Vehicle Fail |
| Nr | Gear/Nr. |  |  |  | l | vAA' | vPP' | vBB' | nMAX | nBB' | LLEFT | LRIGHT | Yes/No |  | aAA',BB' | aPP',BB' | va | LEXP | Cross X if applicable | Cross X if applicable | Cross X if applicable |
|  |  |  | km/h | % | m | km/h | km/h | km/h | 1/min | 1/min | dB(A) | dB(A) |  |  | m/s² | m/s² | m²/s³ | dB(A) |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Add Runs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Annex 7 – Appendix 4 – Flowcharts

Evaluation

ASEP Annex 7

General Process

Perform Type Approval Test according to Annex 3; Report LACCi, nBB ACCi, vBB’ ACCi, LCRSi, nBB CRSi, vBB’ CRSi,

Test Run FAILS

NO

Subroutine

Sound Model Reference

Vehicle Testing

Test Run PASSES

YES

LTEST < LTEST EXP

Subroutine

Sound Level Expectation

Establish Sound Model Reference

Establish test conditions according to 
paragraphs 2.1 and 2.2

Report for each ASEP Test LASEP, nASEP, vASEP, aASEP

Perform an ASEP Test 
within the Control Range according to 
paragraphs 2.3 to 2.5

Calculate expected sound level LTEST EXP for each test run

NO

Max number Tests reached?

YES

Testing and Test run evaluation is finished

Proceed with

final vehicle compliance

evaluation

Sound model

reference established

Subroutine End

Select Parameter Set Applicable for the Vehicle

Calculate the Reference Power Train Mechanics Sound Level

LREF PT

Calculate the Reference Tyre/Rolling Sound Level

LREF TR

Calculate the Reference Dynamic Sound Level

LREF DYN

LACC REF – LCRS  REF  < 1,1?

NO

∆LDYN = 10 dB(A)

YES

Determine the Vehicle Dynamic Delta Sound Level

∆LDYN

Establish Sound Model Reference

LREF TR=10 log(x*100.1 LCRS ANCHOR)

LREF PT=10 log((1-x)*100.1 LCRS ANCHOR)

LREF DYN=LREF PT-15

∆LDYN=10 log100.1 LACC REF-100.1 (LREF TR ADJ) - 100.1(LREF PT ADJ)-LREF PT

LREF TR ADJ=TR LO *logvBB ACC REFvREF50+ LREF TR

LREF PT ADJ=PT LO *lognBB'ACC REF+nSHIFT PTnBB'CRS REF + nSHIFT PT+ LREF PT

with

Calculate expected tyre rolling sound LTR EXP

Calculate expected 
sound level LTEST EXP for a single test run

vTEST 
> 50 km/h

Calculate expected power train mechanics sound level LPT EXP

nACC TEST 
> nACC REF

Calculate expected base dynamic sound level

LDYN EXP

nACC TEST 
> nACC REF

1

NO

LTR EXP=TR LO *logvBB' TESTvREF50+ LREF TR

YES

LTR EXP=TR HI *logvBB' TESTvREF50+ LREF TR

NO

LPT EXP=PT LO *lognBB'ACC TEST+nSHIFT PTnBB'CRS REF+ nSHIFT PT+ LREF PT

YES

LPT,EXP=PT,HI * lognBB'ACC TEST+nSHIFT PTnBB'CRS REF+ nSHIFT PT+ LREF PT

NO

LDYN EXP=DYN LO *lognBB'ACC TEST+nSHIFT DYNnBB'CRS REF + nSHIFT DYN+ LREF DYN

YES

LDYN EXP=DYN HI *lognBB'ACC TEST+nSHIFT DYNnBB'CRS REF+ nSHIFT DYN+ LREF DYN

Calculate expected dynamic delta sound level (load and performance)

∆LDYN EXP

1

aMAX REF determined?

Perform a measurement in a medium gear ratio with an engine speed between

nMAX TORQUE and S

NO

Calculate

reference acceleration aMAX REF

Calculate

reference gear ratio REF

Calculate performance vaTEST

vaTEST =vBB'TEST3.6 *aTEST

va > vaREF

Set performance component

NO

∆LDYN  va =0

YES

Calculate performance component

YES

∆LDYN va =β*logvaTESTvaREF2

Calculate expected dynamic delta sound level (load and performance)

∆LDYN EXP

2

∆LDYN,EXP=LDYN+LDYN va*1-LOADTEST+ / (1-

Calculate reference acceleration per gear

aMAX i =iGEARTESTiREF *aMAX REF

Calculate the load portion LOADTEST = aTEST / aMAX i

Calculate expectation sound level L TEST EXP for an individual test run

2

LTEST EXP=10 log100.1 LEXP TR-100.1 (LEXP PT) - 100.1(LEXP DYN+LDYN EXP)

LTEST EXP determined
 Subroutine End

Compliance

Evaluation

Compliance Evaluation

Subroutine End

Compliance

Evaluation

LTEST≦L TEST EXP

Compliance Evaluation

Subroutine End

More [2] runs

Compliance Evaluation

Subroutine End

Non-Compliance Evaluation

Subroutine End

Non-Compliance Evaluation

Subroutine End

LTEST EXP≦[2]

No

Valid

Invalid

Yes

No

Yes

| CASE | WOT | CRS | Virtual Engine Speed |
| --- | --- | --- | --- |
| A | ICE ON | ICE ON | Use measured data |
| B | ICE ON | EV ONLY | nBB,CRS = vREF / 30 x 1000 |
| C | EV ONLY | ICE ON | nBB,WOT = vBB,WOT / 20 x 1000 |
| D | EV ONLY | EV ONLY | nBB,WOT = vBB,WOT / 20 x 1000 nBB,CRS = vREF / 30 x 1000 |

Vehicle is xEV and test carried out in EV mode *)

Annex 3 provides all necessary information for acceleration and cruise test conditions

Annex 3 acceleration test done in locked gear

RD-ASEP Annex 7

Subroutine for low PMR vehicles (PMR<25)

Calculate virtual cruise test result

LCRS REP=LACC REP- [1,1] dB(A)

nBB'CRS REP=vTESTvBB'ACC REP ×nBB'ACC REP

Missing information on cruise test condition have been generated.

PMR < 25

NO

YES

YES

nBB'CRS REP=vTEST30×1000

Continue to establish 
the Sound Expectation Model of RD-ASEP

NO

YES

NO

*) 	See cases for hybrid vehicles in paragraphs 3.2.3. and 3.2.4.

Annex 8

Indoor testing

1.		Documentation for indoor application

Documentation shall include:

(a)	Validation of facility, e.g. free field propagation, dyno and air handling background noise level, dyno dynamic performance, software.

(b)	Procedures to be applied for indoor testing, e.g. dyno and software set-up, loading and tie-down, air-handling and vehicle’s temperature management.

(c)	Coast down and tyre sound level data used for calculation of dynamometer load coefficients and tyre sound data used for determination of final reported results.

(d)	Test results on a representative selection of the manufacturer’s production to demonstrate that indoor testing delivers comparable results as outdoor testing within acceptable accuracy.

2.		Vehicle tested indoor using Variant A for Annex 3.

Indoor pass-by test is simulated by measurement of power train sound on the dynamometer and energetical addition of the tyre/road sound (measured separately on an outdoor test track).

2.1.	 	General

This method is a combination of indoor testing (power train sound) and outdoor testing (tyre/road sound). It is not necessary to repeat the measurement of the tyre/road sound every time a vehicle is tested. The data of several tyres can be stored in a database and a matching data set from the database can then be used for the test.

2.2.	 	Power train sound

It shall be ensured that there is no remaining tyre/road sound affecting the measurements. In any case it shall be ensured that the remaining tyre/road sound shall be at least 10 dB below the maximum A-weighted sound pressure level produced by the vehicle under test. If this condition cannot be fulfilled, a correction shall be carried out. This correction procedure is described in ISO 362-3:2016 Annex B, paragraph B.6.

The vehicle shall be measured according to the operating condition specified in paragraphs 3.1.2.1. or 3.1.2.2. of Annex 3 of this Regulation.

2.3.  		Tyre/road sound

The measurements of the tyre/road sound shall be performed on a test track as described paragraph 2.1.1. of Annex 3 of this Regulation. The evaluation of tyre/road sound consists of two procedures, namely:

(a) 	Evaluation of free rolling sound;

(b) 	Evaluation of tyre/road sound including torque influence which can be derived from a) by a simplified method.

All conditions for evaluation of tyre/road sound shall be done according to paragraph 3. of this Annex.

2.4.  		Calculation of the total vehicle sound

The total vehicle sound is the energetical sum of tyre/road sound and power train sound. This calculation shall be carried out for each single run as describe in ISO 362-3.2016, paragraph 10.2.4.

2.5.	Procedure for measurement, evaluation, and calculation of tyre/road sound when using variant A

All conditions for evaluation of tyre/road sound, free rolling sound, and torque influence are described in ISO 362-3:2016, Annex B.

3.		Vehicle tested indoor for Annex7

3.1.		General

The use of indoor testing for Annex 7 may not require tire/road data gathered on an outdoor track.

3.2	Measurement method

The vehicle shall be measured according to paragraph 3. of Annex 7 of this Regulation.

3.3 	Compliance assessment

The compliance of the vehicle shall be followed in paragraph 4. of Annex 7 of this Regulation.

3.4 		Determination of reference data

The necessary reference data to establish the sound estimation model are taken from the pass-by and from the cruise-by test of one gear of the Annex 3 tests.

In the case the test has been carried out with two gear ratios, the reported parameters for gear i shall be selected. In the case the test has been carried out in a single gear, the reported parameters for this single gear shall be selected.

These reference data shall be measured in the same indoor facility as measurement of paragraph 3.2 in this Annex.

4. 		Adjustment of room dimensions

To cater for the smaller size test rooms, the maximum levels shall be evaluated with caution though to avoid missing them according to ISO 362-3:2016, Annex E.

*	Former title of the Agreement: Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), ECE/TRANS/WP.29/78/Rev.6

Note: The common understanding of a "low gear" or a "high gear" shall not apply to gear ratios. For example, the lowest gear for forward driving, the first gear, has the highest gear ratio of all forward driving gears. While manual transmission has discrete gears, many non-manual transmissions can have more gear ratios engaged by the control unit of the transmission.

- A CVT might have predefined by soft- or hardware discrete gears, but could provide as well more gear ratios by external control measures.

As defined in the Mutual Resolution No.2 (M.R.2) ECE/TRANS/WP.29/1121

The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.6

A test is made on a stationary vehicle in order to provide a reference value for administrations which use this method to check vehicles in use.

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.6

See footnote 3 in paragraph 6.2.7 of UN Regulation No. 138 “The maximum overall sound pressure level of 75 dB(A) measured at a distance of 2 m is corresponding to the overall sound pressure level of 
66 dB(A) measured at a distance of 7,5 m.”

Distinguishing number of the country which has granted/extended/refused/withdrawn approval (see approval provisions in the Regulation).

Delete what does not apply.

If the means of identification of type contains characters not relevant to describe the vehicle types covered by the type-approval certificate such characters shall be represented in the documentation by the symbol: '?' (e.g. ABC??123??).

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.6

If applicable

Delete what does not apply.

If the means of identification of type contains characters not relevant to describe the vehicle types covered by the type-approval certificate such characters shall be represented in the documentation by the symbol: '?' (e.g. ABC??123??).

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.6

Only for the purpose of defining "off-road vehicles".

Standard ISO 612: 1978 — Road vehicles — Dimensions of motor vehicles and towed vehicles - terms and definitions.
(a) 	Where there is one version with a normal cab and another with a sleeper cab, both sets of masses and dimensions are to be stated. 
(b) 	Optional equipment that affects the dimensions of the vehicle shall be specified.

The mass of the driver is assessed at 75 kg. The liquid containing systems (except those for used water that shall remain empty) are filled to 90 per cent of the capacity specified by the manufacturer. The information referred to in points 2.2. (b) do not need to be provided for vehicle categories N2, N3, M2 and M3.

For vehicles coupled with a trailer or a semi-trailer, which exert a significant vertical load on the coupling device or the fifth wheel, this load, divided by standard acceleration of gravity, is included in the maximum technically permissible mass. Please fill in here the upper and lower values for each variant.

Please fill in here the upper and lower values for each variant.

In the case of a vehicle that can run either on petrol, diesel, etc., or also in combination with another fuel, items shall be repeated. In the case of non-conventional engines and systems, particulars equivalent to those referred here shall be supplied by the manufacturer.

Delete what does not apply.

This value shall be calculated (π = 3.1416) and rounded off to the nearest cm³.

The specified particulars are to be given for any proposed variants.

Continuous Variable Transmission (CVT): transmission with variable gear ratios.

With respect to trailers, maximum speed permitted by the manufacturer.

The latter number is given as an example only.

Given that the tyre contribution for overall sound emission is significant, regard shall be given for existing regulatory provisions concerning tyre/road sound emissions. Traction tyres, snow tyres and special-use tyres as defined in paragraph 2. of Regulation No. 117 shall be excluded during type-approval and conformity of production measurements at the request of the manufacturer in accordance with Regulation No. 117.

2	See Annex 3, Appendix, Figure 1.

See Figure 6 of Annex 3, Appendix

Facilities may provide restriction for safety reasons, such as for vehicle speed.

Tests for Annex 3 and Annex 7 may be carried out on different test facilities if documentation exists that demonstrates that the differences in sound performance are neglectable.