Proposal for a new UN GTR on Laboratory Measurement of Brake Emissions for Light-Duty Vehicles

Submitted to the Informal Working Group on non-exhaust for Particulate Measurement Programme on 17.06.2022

Contents

I	Statement of technical rationale and justification	     4

A.	Introduction	4

B.	Procedural background and future development	5

C.	Background on the technical work of the PMP group	7

II	Text of the GTR		     1

1.	Purpose	1

2.	Scope and application	1

3.	Definitions	1

3.1.	Vehicle and Brake Dynamometer settings	1

3.2.	Test setup	3

3.3.	Brake hardware	4

3.4.	WLTP-Brake cycle	5

3.5.	PM and PN Measurement	6

3.6.	Test system	7

3.7.	Reserved [Pure electric, pure ICE, hybrid electric]	7

4.	Abbreviations and Symbols	8

5.	General Requirements	12

5.1.	Compliance Requirements	12

5.2.	Definition of Brake Family	12

5.3.	Rounding Requirements	13

6.	General Overview	13

7.	Test System Requirements	14

7.1.	Overall Test System Layout	14

7.2.	Climatic Conditioning Unit and Cooling Air	16

7.3.	Brake Inertia Dynamometer and Automation Systems	22

7.4.	Brake Enclosure Design	24

7.5.	Design of the Sampling Tunnel	27

7.6.	Sampling Plane	28

8.	Test Preparation Requirements	29

8.1.	Input Parameters	29

8.2.	Test Setup Preparation	32

8.3.	Brake Temperature Measurement	33

8.4.	Brake Positioning	34

9.	WLTP-Brake Cycle	36

9.1.	General Information	36

9.2.	WLTP-Brake Cycle Application	37

9.3.	WLTP-Brake Cycle Interruptions	38

9.4.	WLTP-Brake Cycle Quality Checks	39

10.	Cooling Airflow Adjustment	41

10.1.	Method Description	42

11.	Bedding Section	45

11.1.	Front Brakes	45

11.2.	Rear Brakes	46

12.	Particle Emissions Measurements Section	47

12.1.	Measurement of Particle Mass (PM) Concentration	47

12.2.	Measurement of Particle Number (PN) Concentration	55

12.3.	Mass Loss Measurement	63

13.	Test Report	64

13.1.	Event-Based File	65

13.2.	Time-Based File	66

13.3.	PM-Wear Measurement File	69

13.4.	Test Report File	72

14.	Calibration Requirements and Ongoing Quality Controls	86

14.1.	General Calibration Requirements	86

14.2.	Brake Inertia Dynamometer	87

14.3.	Cooling Airflow Measurement Device	87

14.4.	Microgram Balance	88

14.5.	Sample Treatment and Conditioning Devices	88

14.6.	Particle Number Counter	90

Statement of technical rationale and justification

Introduction

1. Over recent years there has been a sharp increase in the international interest to characterise non-exhaust emissions of particles from road transport. Until recently, exhaust sources dominated road transport emissions, and all regulatory efforts have been aiming at their reduction. As exhaust emissions were reduced due to increasingly stringent regulations, the relative contribution of non-exhaust emissions to overall ambient concentrations of particulate matter increased.

2. Most manufacturers produce vehicles for a global clientele, or at least for several regions. Since manufacturers tend to cater to the preferences, needs, and lifestyles of specific geographic regions, vehicle designs will vary worldwide. As compliance with different emission standards in each region can create burdens from an administrative and vehicle design point of view, vehicle manufacturers tend to have a strong interest in harmonising brake emission test procedures and performance requirements on a global scale. Global harmonisation is also of interest to regulators as it offers more efficient development and adaptation to technical progress, potential collaborations with market surveillance, and facilitates the exchange of information between regulatory authorities.

3. In this context, stakeholders launched the work for this United Nations Global Technical Regulation (UN GTR) on Worldwide harmonized Light vehicle Test Procedures (WLTP) for particle emissions from brake wear. This UN GTR aims at harmonising test procedures for emissions from Light-Duty vehicles to the extent possible. Laboratory test procedures need to represent real driving conditions as much as possible and to enable a direct comparison between the performance of vehicles during certification procedures and in real life. However, this aspect puts some limitations on the level of harmonisation to be achieved. Furthermore, different countries will show varying levels of development, population densities, and costs associated with braking system technology. Consequently, the regulatory stringency of legislation is expected to vary from region to region for the foreseeable future. Therefore, the definition of emission limit values is not part of this UN GTR.

4. UN GTRs are intended to be implemented into regional legislation by as many Contracting Parties as possible. The selection of vehicle categories to be covered by the scope of regional legislations represents a challenge as it depends on regional conditions that cannot be anticipated. However, according to the provisions of the 1998 UN ECE agreement, a UN GTR being implemented by a Contracting Party must apply to all vehicles, conditions, and equipment falling under its formal scope. Therefore, care must be taken in developing the scope of the UN GTR, as an unduly large formal scope may prevent or hamper its implementation into regional legislation. For this reason, the formal scope of this UN GTR is limited to Light-Duty vehicles up to 3.5 tons. This limitation does not, however, indicate that the scope of this UN GTR should not be applied to a larger group of vehicle categories when implemented into regional legislation. Indeed, Contracting Parties are encouraged to do so if this is feasible and appropriate from a technical, economical, and administrative point of view.

5. A harmonised approach for measuring brake particle emissions would allow manufacturers to better understand the behaviour of different brake systems, reduce inconsistencies in results and; therefore, compare them more efficiently, and develop strategies to decrease brake emissions.

6. This version of the UN GTR does not contain test requirements specific to other types of vehicles e.g. off-road or special purpose vehicles. Thus, these vehicles are not included in the scope of the UN GTR. However, Contracting Parties may apply the provisions within this UN GTR to such vehicles to the extent possible from a technical point of view, and complement them with additional provisions in regional legislation e.g. brake emission testing with different types of friction materials or mating parts.

Procedural background and future development

7. In 2013, following the submission of informal documents by the Russian Federation, UNECE WP.29 agreed with the GRPE decision to task the Informal Working Group on Particle Measurement Programme (IWG on PMP) to investigate the issues concerning the emissions of non-exhaust emissions of particles from road transport. The main objective of the IWG on PMP was to investigate whether there is a need to extend particle measurement procedures to additional sources such as brake, tyre, and road wear.

8. The IWG on PMP identified brake and tyre wear as the most relevant sources of non-exhaust emissions of particles from road transport and selected them as the main topics for future investigation (Informal document GRPE-69-23). With regards to brake wear emissions, the primary task of the IWG on PMP has been to define a set of “normal” or “typical” driving conditions and develop a novel test cycle reproducing the driving and braking activity of Light-Duty vehicles in real-world conditions. Other main tasks included the development of guidelines and best practices for sampling and measuring brake wear particles and establishing the minimum requirements to report test results.

9. Following the approval by AC.3 of the first mandate of the IWG on PMP regarding non-exhaust emissions in June 2013, the IWG on PMP aimed to accomplish the following objectives, which were completed by June 2016:

- Conduct a literature survey summarising the current knowledge on the physical/chemical nature, mass, number, and size distribution of non-exhaust particle emissions;

- Identify and report the main knowledge gaps and the needs for future research. These findings were included in a report submitted to the 69th GRPE session (Informal Document GRPE-69-23);

- Establish a group of experts in the field of non-exhaust emissions, sharing information and ongoing research on topics related to non-exhaust emissions;

- Analyse the WLTP database to define normal and extreme driving conditions, and gather information on existing methodologies for sampling and measuring non-exhaust emissions;

- Consider the most suitable testing approach for brake emissions, and define the pros and cons of different available options (brake test rig, full vehicle chassis dyno, full vehicle on-road, etc.).

10. The IWG on PMP extensively discussed several options for a standardised method to sample and characterise brake wear particles, and eventually selected an approach based on a fully-enclosed brake inertia dynamometer. This method allows the sampling of particles from brake wear without interferences from other sources and minimises particle losses over the entire sampling and measurement line. Furthermore, brake inertia dynamometers offer a flexible platform to test different friction couples under various driving conditions and vehicle loads. The laboratory setup needs to enable repeatable and reproducible measurements at least for a defined set of core parameters. An appropriate installation of the laboratory setup will then allow the end-user to select additional values worth measuring, within the capabilities of the system.

11. A second mandate for the IWG on PMP with specific reference to non-exhaust emissions was approved in June 2016 by AC.3. The IWG on PMP was mandated to develop a commonly accepted test procedure for sampling and assessing brake wear particles both in terms of mass and number. The methodology would aim to provide the necessary tool to support future studies on brake emissions that can be easily compared. During the reporting period of the mandate (2016-2019) the following items were addressed:

- Development and validation of a novel test cycle appropriate for the investigation of brake wear particles;

- Investigation and selection of the appropriate methodologies for particle generation and sampling;

- Investigation and selection of the appropriate instrumentation for the measurement and characterisation of brake wear particles.

12. After completing a thorough analysis regarding the suitability of existing brake cycles, the IWG on PMP decided to proceed with the development of a novel test cycle appropriate for the investigation of brake wear particles. Therefore, the IWG on PMP decided to create a dedicated Task Force (TF1) to accelerate the development of a test cycle in October 2016. In September 2017, the IWG on PMP decided to create a dedicated Task Force (TF2) to investigate and select the appropriate methodologies and instrumentation for the measurement of brake wear particles. TF2 initiated its activities in October 2017.

13. During the reporting period (2016-2019), the IWG on PMP aimed to accomplish the following objectives:

- Selection of the brake test rig methodology for the generation and sampling of brake wear particles;

- Agreement on the method’s target measurement parameters. TF2 agreed unanimously that both PM (PM10 and PM2.5) and PN (≥10 nm) emissions shall be addressed;

- Development and publication of the WLTP-Brake cycle. The cycle is based on real-world data extracted from the WLTP database and is considered representative of real-world applications;

- Validation of the WLTP-Brake cycle through an interlaboratory accuracy study exercise which was completed in 8 different laboratories in Europe and the United States;

- A thorough analysis of the existing methods and setups for the sampling and measurement of brake particle emissions. Agreement on the need of defining a set of minimum specifications and requirements for sampling and measurement of brake particle emissions.

14. The mandate for the IWG on PMP regarding non-exhaust emissions was further extended in June 2019 by AC.3. The revised mandate included an additional item compared to 2016, which foresaw the validation of the proposed methodology for the measurement and characterisation of brake wear particles. During the reporting period (2019-2020), the IWG on PMP aimed to accomplish the following objectives:

- Update the GRPE of the work of the IWG on PMP (TF1) on the development of the novel WLTP-Brake cycle and its application in the measurement and characterisation of brake emissions at brake dynamometer level;

- A first discussion on how to address future technologies took place at the IWG on PMP level following the request of several GRPE stakeholders.

15. The mandate for the IWG on PMP regarding non-exhaust emissions was further extended in June 2020 by AC.3. Following the discussion at the IWG on PMP level, the revised mandate included the extension of the proposed methodology to future technologies. In June 2020, several GRPE Contracting Parties urged the IWG on PMP to start considering a possible use of the proposed method as a regulatory tool. Therefore, the IWG on PMP was requested to start looking at the necessary changes and adaptations to extend the method to all existing technologies and other vehicle categories.

16. Stakeholders and Contracting Parties discussed the possible approaches to regulate brake particle emissions in a workshop in January 2021. The main topics discussed during the workshop include:

- The ideal scheme for regulating brake emissions from conventional ICE Light-Duty vehicles;

- How to handle non-conventional Light-Duty vehicles (i.e. HEVs, EVs) in a possible future regulatory approach;

- Heavy-Duty vehicle brake emissions and possible approaches.

17. As a follow-up of the workshop, the interested Contracting Parties and the IWG on PMP recommended that a UN GTR on brake PM and PN emissions from all types of LDV’s brake systems is developed under a new mandate. Therefore, the representatives of the European Union, the UK and Japan sought and obtained authorisation from AC.3 to develop a new UN GTR on brake PM and PN emissions from all types of LDVs brake systems.

Background on the technical work of the PMP group

18. REFERENCE TO THE TF2 ACTIVITIES – TO BE ELABORATED

19. REFERENCE TO THE TF3 ACTIVITIES – TO BE ELABORATED.

20. REFERENCE TO THE TF4 ACTIVITIES – TO BE ELABORATED

21. This UN GTR provides all the information necessary to carry out brake particle emissions testing in the laboratory. The main elements of the protocol contained in this GTR include:

- References, definitions, and terminology applying to the methodology for sampling and measuring particles from brake wear emissions;

- General requirements and capabilities of the required test setup. Main test conditions for the different elements of the setup overall setup;

- A detailed description of the WLTP-Brake Cycle;

- A detailed description of the different sections of a brake emissions test, including cooling airflow adjustment, bedding procedure, and emissions measurement;

- Minimum requirements to report results from the dynamometer test including actual emissions measurements and metrics.

23. Future work to further expand the protocol contained in this UN GTR might address:

- Definition of a real-world cycle/s for use in the laboratory;

- Adaptation of the proposed methodology to include future technologies;

- Adaptation of the proposed methodology to address brake emissions from heavy-duty vehicles.

Text of the GTR

Purpose

This Global Technical Regulation (GTR) provides a worldwide harmonised methodology for the measurement of brake wear particle emissions from Light-Duty vehicles.

This GTR defines the test cycle, minimum system requirements, test conditions, and equipment preparation to execute the WLTP-Brake Cycle using brake inertia dynamometers. This GTR also provides requirements for the design and set up of test systems to measure brake emissions.

Scope and application

This GTR applies to vehicles using some type of mechanical braking, using a combination of dry friction materials and a mating brake disc or brake drum.

This GTR applies to category 1-1 and category 2 vehicles with a fully laden mass below 3500 kg.

The Contracting Parties shall make a decision about the applicability of this GTR to Small Volume Manufacturers for their jurisdiction.

Definitions

Vehicle and Brake Dynamometer settings

"Category 1 vehicle" means a power-driven vehicle with four or more wheels designed and constructed primarily for the carriage of one or more persons.

"Category 1‒1 vehicle" means a category 1 vehicle comprising not more than eight seating positions in addition to the driver's seating position. A category 1‒1 vehicle may not have standing passengers.

"Category 2 vehicle" means a power-driven vehicle with four or more wheels designed and constructed primarily for the carriage of goods. This category shall also include (a) Tractive units and (b) Chassis explicitly designed to be equipped with special equipment.

"Mass in running order" is the mass of the vehicle, with its fuel tank(s) filled to at least 90 per cent of its capacity, including the mass of the driver, fuel, and liquids, fitted with the standard equipment in accordance with the manufacturer's specifications and, when they are fitted, the mass of the bodywork, the cabin, the coupling and the spare wheel(s) as well as the tools.

"Mass of the driver" means a mass rated at 75 kg located at the driver's seating reference point. In the context of the current regulation, the term "mass of additional 0.5 passengers" means a mass rated at 37.5 kg.

"Maximum vehicle load" means the technically permissible maximum laden mass minus the MRO, 25 kg, and the mass of the optional equipment.

"Optional equipment" means all the features not included in the standard equipment fitted to a vehicle under the manufacturer's responsibility and that the customer can order.

"Standard equipment" means the basic configuration of a vehicle equipped with all the features required under the regulatory acts of the Contracting Party, including all features that are fitted without giving rise to any further specifications on configuration or equipment level.

"Vehicle test mass" means the mass in running order plus the mass of the optional fitted equipment to an individual vehicle (kg) on which the tested brake is mounted plus:

- 37.5 kg that corresponds to an additional mass of 0.5 passengers for category 1‒1 vehicle (or M1 vehicle category);

- 25 kg plus 28 per cent of the Maximum Vehicle Load (MVL) for category 2 vehicles with a fully laden mass below 3.5t (or N1 vehicle category).

"Road Loads" means the total force or power required to move the vehicle on a level and smooth surface at a specified speed and mass. Road loads take account of transmission friction, rolling friction, and air resistance. In this GTR, a reduction of the brake nominal inertia by a fixed percentage of 13 per cent is considered to account for road loads in full-friction brakes emissions testing.

"Tyre dynamic rolling radius" means the tyre radius that equates to the revolutions per mile (or revolutions per kilometre) published by the tyre manufacturer for the specific tyre size (mm).

"Brake force distribution" means the ratio between the braking force of each axle and the total braking force expressed as a percentage for each axle.

"Nominal Wheel load" means the (equivalent) rotating mass as a function of the total vehicle test mass, the axle under test (front or rear), and the brake work distribution among the two axles. It represents the load at the brake corner under testing before accounting for vehicle road loads.

"Test wheel load" means the (equivalent) rotating mass as a function of the total vehicle test mass, the axle under test (front or rear), and the brake work distribution among the two axles. It represents the load at the brake corner under testing after accounting for vehicle road loads. Also referred to as "Applied wheel load".

"Brake nominal inertia" means the inertia of the wheel load at the radius of gyration equal to the tyre dynamic rolling radius, which imposes the same kinetic energy on the brake as in the actual vehicle before subtracting the total road loads for the vehicle.

"Brake test inertia" means the inertia of the wheel load at the radius of gyration equal to the tyre dynamic rolling radius, which imposes the same kinetic energy on the brake as in the actual vehicle after subtracting the total road loads for the vehicle. Also referred to as "Brake applied inertia".

"Brake torque" means the product of the frictional forces resulting from the tangential actuating forces in a brake assembly and the distance between the points of generation of these frictional forces and the axis of rotation. The brake torque is a function of the hydraulic piston area, apparent friction coefficient, and the effective brake radius of the brake corner.

"Hydraulic pressure" means the net pressure supplied by the brake to generate clamping force between the brake and friction material. The hydraulic pressure, combined with the brake's friction coefficient and the effective brake radius, induces the actual brake torque output.

"Threshold pressure" means the minimum hydraulic pressure to overcome the internal friction and seal forces, move the brake calliper's piston or drum wheel cylinder, and onset brake torque output.

"Piston diameter" means the diameter of the hydraulic piston(s) in the calliper or drum wheel cylinder and is used to calculate the total piston(s) area. Also referred to as "Hydraulic piston diameter".

"Piston area" means the active area of all hydraulic pistons acting on one side of the brake calliper or drum brake cylinder.

"Effective brake radius" means for a disc brake, the distance between the centre of rotation and the centerline of the calliper piston(s) when assembled on the fixture. For drum brakes, the effective radius is half of the drum's inner diameter.

"Friction coefficient " means the ratio between the tangential force and the normal force acting between the brake pad and the disc or the brake shoe and the drum. For a disc brake, the apparent friction coefficient value from the brake under testing is a function of braking torque, effective brake radius, and the piston area. The apparent coefficient of friction is a calculated (mathematical) value and is not directly measurable. Also referred to as "Brake effectiveness".

"Brake fluid displacement" means the transient (volumetric) use of hydraulic fluid by the brake calliper or the brake wheel cylinder during a brake deceleration event.

"Average by time" means the averaging method for a given measurand where the sampling frequency is a unit of time (between data points) through a braking event. The resultant value yields the same result as the integration between two points in time.

"Average by distance" means the averaging method for a given measurand where the sampling frequency is a unit of vehicle distance (between data points) travelled (or driven) during a braking event. The resultant value yields the same result as the integration between two distance points.

"Fast sampling rate" means the sampling rate for the data collection system greater than or equal to 250 Hz. The "fast sampling rate" applies to the dynamometer channels.

"Slow sampling rate" means the sampling rate for the data collection system that is less than or equal to 10 Hz. The "fast" sampling rate applies to the dynamometer channels.

Test setup

"Brake inertia dynamometer" means a technical system that imposes, controls, and records the mechanical and electrical work from the brake under testing while operating with a pre-programmed test procedure.

"Torque measurement sensor" means the electromechanical device that converts the torsional strain on the brake assembly into the equivalent voltage output. The equivalent torque derives from the angular deceleration rate and the effective brake inertia.

"Servo controller" means a system that modulates the braking torque or hydraulic pressure to the intended (setpoint) value. The servo controller also provides the algorithm to control the release of braking torque or pressure at the end of the brake deceleration events.

"Climatic conditioning unit" means the air handling system which provides clean, conditioned, and controlled cooling air into the transport duct and the brake enclosure.

"Cooling air" means the clean, conditioned, and controlled air provided to the brake assembly by the environmental conditioning system during the test through fixed and smooth ducts and the sampling tunnel.

"Cooling air temperature" means the temperature of the cooling air stream measured near the entry to the brake enclosure.

"Cooling air relative humidity" means the amount of water vapour present in the cooling air stream expressed as a percentage of the amount needed for saturation at the same temperature. It is measured near the entry to the brake enclosure.

"Cooling airspeed" means the average speed of the cooling airstream measured in real-time in a length of straight duct with constant shape and cross-sectional area. The airspeed can be reported as measured or corrected for standard conditions.

"Cooling airflow" means the average flow of the cooling airstream provided to the brake assembly. The airflow shall be reported as measured or corrected for standard conditions.

"Brake enclosure" means a sealed chamber that prevents untreated air from entering the air flowing around the brake assembly. The brake enclosure shrouds the brake assembly.

"Sampling Tunnel" means a rigid duct connecting the brake enclosure to the sampling plane and continuous to the airflow measurement device. It represents the part of the tunnel where the brake particles emitted inside the brake enclosure travel towards the sampling and measurement devices.

Brake hardware

"Brake under testing" means the friction brake assembly and its associated vehicle parameters used by the test facility to measure brake particle emissions according to this UN GTR. For vehicles with regenerative braking, the vehicle parameters include those from the electric machine.

"Brake assembly" in the case of disc brakes means the set of matching brake discs, brake pad assemblies, brake calliper, and associated hardware for a given vehicle and axle application. In the case of drum brake systems, the hardware set comprises the brake drum, brake shoes, drum assembly parts, and brake mounting plate for a given vehicle and rear axle application. The brake assembly mounts on a brake fixture to adapt and connect to the brake inertia dynamometer.

"Service brake" means the (friction or regenerative) braking system allowing the driver to control, directly or indirectly and in a graduated manner, the speed of a vehicle during normal driving or to bring the vehicle to a halt (standstill).

"Full-friction brake" means the service brake mounted on a vehicle with an internal combustion engine.

"Brake fixture" means a mechanical device or jig to mount the brake assembly by connecting the tailstock (or non-rotating surface) to the brake inertia dynamometer shaft (rotating). The tailstock side (or non-rotating surface) absorbs the braking torque and associated tangential forces. The rotating shaft transmits the kinetic energy from the brake test inertia to the brake assembly.

"Universal style fixture" means a brake fixture cylindrical and symmetrical without additional extensions or protrusions different from those needed to mount the brake assembly. A wheel hub is not included in the assembly.

"Post style fixture" means a dynamometer fixture that uses a round and stiff tubing and adaptors, instead of the vehicle knuckle, to mount the brake assembly. A wheel hub is attached to complete the assembly.

"Brake calliper" means a mechanical device that converts driver brake pedal input into a clamping force on the brake pads to generate braking torque.

"Brake disc" means a rotating, wearable device against which the brake calliper clamps the brake pads in a disc brake system. This device acts as the primary heat absorption and dissipation device as the brake corner transforms vehicle kinetic energy into heat.

"Brake pad" means a wearable device that mounts onto the brake calliper consisting of a structural (metal) pressure plate and a friction material element. The brake pads clamp against the brake disc, generating a retarding friction force and thus the brake torque.

"Brake drum" means a rotating, wearable mechanism against which the brake wheel cylinder clamps the brake shoes in a drum brake system. This device acts as the primary heat absorption and dissipation device as the brake corner translates vehicle kinetic energy into heat.

"Brake shoe" means a wearable device consisting of an arced structural metal shoe and a (bonded or riveted) friction material. The brake shoe is clamped against the drum to generate friction and thus brake torque.

"Friction material part number" means the unique edge code to identify the specific friction material supplier, formulation, and environmental marking.

"Disc or drum part number" means the unique code labelled by the manufacturer to identify the specific disc or drum.

"Lateral runout" means the change in the axial distance (from a datum plane) to the braking surface of the brake disc during one complete revolution at a given radial position.

"Running clearance" means the axial distance between the braking surface of the disc and the brake pad during one complete revolution with the brake released. For drum brakes, it is the radial distance between the inner diameter of the drum and the brake shoe.

WLTP-Brake cycle

"Driving cycle" means a series of data points representing the speed of a vehicle versus time. The driving cycle consists of individual trips and each trip of a series of separate and consecutive events. These include brake dwell, acceleration, cruising, and deceleration.

"WLTP-Brake Cycle" means the driving cycle derived from the vehicle activity of the Worldwide Light vehicle Test Procedure database with a total duration of 15 826 seconds and the cooling sections in-between trips. The cycle comprises ten trips and 303 brake deceleration events.

"Brake emissions test" means a sequence of three consecutive sections (cooling air adjustment, brake bedding, and brake emissions measurement) to characterise the particle emissions of the brake under testing.

"Cooling air adjustment" means the section that follows a procedure with the brake under testing to define the appropriate incoming cooling airflow for the bedding and emissions measurement sections.

"Brake bedding" means the section with a sequence of braking events to develop a friction couple with a stable transfer layer, brake effectiveness, and brake emissions behaviour before conducting the brake emissions measurement section. Also referred to as the "Bedding procedure".

"Brake emissions measurement" means the section of the brake emissions test where PM and PN emissions are sampled and measured.

"Brake acceleration event" means a measurable period during which the linear speed increases to a predetermined set value at a known rate. This event always precedes a brake cruising or a brake deceleration event.

"Brake cruising event" means a measurable period during which the (non-zero) linear speed is constant.

"Brake dwell event" means a measurable and predictable brake pause at zero speed during the cycle.

"Brake deceleration event" means a measurable period during which the linear speed decreases at a known rate to a predetermined release speed during the cycle.

"Deceleration rate" means the total rate of reduction in the linear speed of the vehicle induced by the application of the service brake, the road loads, and the regenerative torque from the electric machine.

"Brake stop" is the generic term denoting a brake deceleration event that brings the vehicle to a halt or standstill.

"Brake snub" means the generic term used to denote a brake deceleration event that reduces the vehicle speed to a non-zero level.

"Cooling section" means the section in-between trips when the brake is rotating at low speed (approximately five or fewer revolutions per minute) waiting for the brake to cool down and the initial brake temperature to reach the predefined level for commencing the next cycle trip.

"Initial speed" means the speed at the start of a brake deceleration event.

"Release speed" means the speed at the end of a brake deceleration event.

"Nominal linear speed" means the target (or set) speed at the time i per the WLTP-Brake cycle.

"Actual linear speed" means the linear speed at the time i during the test cycle execution. Also referred to as "Measured speed"

"Speed violation" means any instance when the actual dynamometer speed trace exceeds the speed trace tolerances prescribed in this GTR during the WLTP-Brake Cycle.

"Initial brake temperature" means the bulk temperature of the brake disc or brake drum at the start of a given brake event during the WLTP-Brake cycle.

"Final brake temperature" means the bulk temperature of the brake disc or brake drum at the end of a given brake event during the WLTP-Brake Cycle.

"Average brake temperature" means the unfiltered average of the time-resolved brake disc or brake drum temperature at 1 Hz during a predetermined period.

"Peak brake temperature" means the highest unfiltered temperature measured using the time-resolved temperature at 1 Hz during a predetermined period.

PM and PN Measurement

The term "particle" is conventionally used for the matter being characterised (measured) in the airborne phase (suspended matter), and the term "particulate matter" for the deposited matter.

"Particle number emissions" (PN) means the number of particles emitted from the brake under testing and quantified according to the dilution, sampling, and measurement methods specified in this UN GTR.

"Total Particle number emissions" (TPN10) means the number of total particles (i.e. solids and volatiles) at a nominal particle size of approximately 10 nm electrical mobility diameter and larger emitted from the brake under testing and quantified according to the dilution, sampling, and measurement methods specified in this UN GTR.

"Solid Particle number emissions" (SPN10) means the number of solid particles at a nominal particle size of approximately 10 nm electrical mobility diameter and larger emitted from the brake under testing and quantified according to the dilution, sampling, and measurement methods specified in this UN GTR.

"Particle mass emissions" (PM) means the mass of any particle from the brake under testing quantified according to the dilution, sampling, and measurement methods specified in this UN GTR.

"PM2.5 emissions" means the mass of PM with an aerodynamic diameter (µm) of approximately ≤2.5 µm.

"PM10 emissions" means the mass of PM with an aerodynamic diameter (µm) of approximately ≤10 µm.

"Sampling plane" means the fixed plane (perpendicular to the sampling tunnel axis) at the entry of the sampling nozzle(s).

"Sampling probe" means a thin-walled stainless steel tube designed to extract and transfer a representative portion from the tunnel to the sampling system.

"Sampling nozzle" means a thin-walled stainless steel cylinder with a knife-edge nozzle tip that mounts at the inlet of a sampling probe. The sampling nozzle aims to extract isokinetically particle-laden air from the sampling tunnel to the emissions measurement instrument(s).

"Sampling nozzle tip" means the outer edge at the entry of the sampling nozzle.

"Sampling line" means the rigid or flexible tubing connecting devices in the sampling train downstream of the sampling probe.

"Particle transfer tube" means the flexible tubing connecting the probe’s outlet and the pre-classifier in the PN sampling train downstream of the sampling probe.

"Standard conditions" means pressure equal to 101.325 kPa and temperature corresponding to 293 K.

"Background emissions" means the measurement of particles using the same instrumentation as for emissions when the environmental conditioning system and the dynamometer cooling air are running under the test conditions, without any brake applications or brake rotation to influence the result.

Test system

"Calibration" means the process of setting a measurement system's response so that its output agrees with a range of reference signals.

"Major maintenance" means the adjustment, repair, or replacement of a component or module that could affect the accuracy of a measurement.

"Reference value" means a value traceable to a national standard.

"Setpoint" means the target value a control system aims to reach.

"Verification" means evaluating whether a measurement system's outputs agree with applied reference signals within one or more predetermined thresholds for acceptance.

"Response time" means the difference in time between the change of the component to be measured at the reference point and a system response of 90 per cent of the final reading (t90) with the sampling probe being defined as the reference point, whereby the change of the measured component is at least 60 per cent full scale (FS) and takes place in less than 0.1 seconds. The system response time consists of the delay time to the system and of the rise time of the system.

Reserved [Pure electric, pure ICE, hybrid electric]

"Regenerative braking" means the decelerating of the vehicle when the battery management system diverts part of the kinetic energy at the wheel to charge the battery pack or drive batteries.

"Electric machine" means an energy converter transforming between electrical and mechanical energy.

"Category of propulsion energy converter" means (i) an internal combustion engine, or (ii) an electric machine.

"Hybrid electric vehicle" means a hybrid vehicle where one of the propulsion energy converters is an electric machine.

"Hybrid vehicle" means a vehicle equipped with a powertrain containing at least two different categories of propulsion energy converters and at least two different categories of propulsion energy storage systems.

"Not off-vehicle charging hybrid electric vehicle" (NOVC-HEV) means a hybrid electric vehicle that cannot be charged from an external source.

"Off-vehicle charging hybrid electric vehicle" (OVC-HEV) means a hybrid electric vehicle that can be charged from an external source.

"Pure electric vehicle" (PEV) means a vehicle equipped with a powertrain containing exclusively electric machines as propulsion energy converters and exclusively rechargeable electric energy storage systems as propulsion energy storage systems.

"Pure ICE vehicle" means a vehicle where all of the propulsion energy converters are internal combustion engines.

Abbreviations and Symbols

| Symbol/Abbreviation | Definition | Unit | Paragraph |
| --- | --- | --- | --- |
| a | Transition angle of the brake enclosure | ° | 7.3 |
| a | Deceleration rate | m/s² | 13.2 |
| A1…3 | Metrics for vehicle or target temperatures | ℃ | 10.1 |
| B1…3 | Metrics for dynamometer temperatures | ℃ | 10.1 |
| C1…3 | Metrics for the temperature difference between vehicle/target and dynamometer | ℃ | 10.1 |
| CFD | Computational fluid dynamics | - | 7.3 |
| CSV | Comma-separated values | - | 13.1 |
| d | Total distance travelled (driven) during WLTP-Brake cycle | km | 12.1 |
| di | Reference duct diameter where there is a requirement in the minimum number of duct diameters for length or radii. Same as sampling tunnel diameter | mm | 7.1 |
| dn | The inner diameter of the sampling nozzle (applies to both PN and PM) | mm | 12.1 |
| dn-PM2.5 | The inner diameter of the isokinetic nozzle for sampling PM2.5 | mm | 12.1 |
| dn-PM10 | The inner diameter of the isokinetic nozzle for sampling PM10 | mm | 12.1 |
| dn-SPN10 | The inner diameter of the isokinetic nozzle for sampling SPN10 | mm | 12.2 |
| dn-TPN10 | The inner diameter of the isokinetic nozzle for sampling TPN10 | mm | 12.2 |
| dpiston | Calliper piston mean or hydraulic diameter | mm | 8.1 |
| dp | The inner diameter of the sampling probe (applies to both PN and PM) | mm | 12.1 |
| ds | The inner diameter of the PM sampling line | mm | 12.1 |
| dtl | The inner diameter of the PN transfer line | mm | 12.2 |
| dtt | The inner diameter of the PN transfer tube | mm | 12.2 |
| dx | Electrical mobility diameter | µm | 14.5 |
| DF | Dilution factor | - | 14.5 |
| DM | Disc mass before testing | kg | 8.1 |
| DOP | Dioctyl phthalate | - | 12.1 |
| η | Brake caliper or drum efficiency | % | 8.1 |
| ECE | Economic Commission for Europe | - | 8.1 |
| EF | Emission factor | - | 12.1 |
| EN | "European Norm" - European technical standard | - | 7.2 |
| f | Brake rotational speed | rev/min | 9.4 |
| fr (dx) | PCRF for each particle of electrical mobility diameter dx | - | 14.5 |
| fr-SPN10 | Arithmetic averaged PCRF for SPN10 measuring device | - | 12.2 |
| fr-TPN10 | Arithmetic averaged PCRF for TPN10 measuring device | - | 12.2 |
| FA | Vehicle front axle | - | 8.1 |
| FBT | Final brake temperature at the end of braking | - | 10.1 |
| GTR | Global technical regulation | - | 8.1 |
| hB | Length of plane B (enclosure) | mm | 7.3 |
| hD | Length of plane D (enclosure) | mm | 7.3 |
| H13 | High-efficiency air filter with a filtering efficiency of at least 99.95% | - | 7.2 |
| He | Point that defines the end of the mandatory horizontal part in the layout | - | 7.1 |
| Hs | Point that defines the start of the mandatory horizontal part in the layout | - | 7.1 |
| HEPA | High efficiency particulate filter | - | 12.1 |
| In | Brake nominal inertia | kg·m² | 8.1 |
| It | Brake test inertia | kg·m² | 8.1 |
| IBT | Initial brake temperature at start of braking | ℃ | 10.1 |
| IR | Isokinetic ratio | - | 12.1 |
| lA1 | Length of plane A1 (enclosure) | mm | 7.3 |
| li | Length of inlet or oulet transition of brake enclosure | mm | 7.3 |
| L1 | Minimum length of the straight duct at the inlet of the brake enclosure | mm | 7.1 |
| L2 | Minimum length from the last disturbance and upstream of the sampling plane | mm | 7.1 |
| L3 | Minimum length to the next disturbance and downstream of the sampling plane | mm | 7.1 |
| L4 | Minimum length from the last disturbance and upstream of the airflow measurement element | mm | 7.1 |
| L5 | Minimum length to the next disturbance and downstream of the airflow measurement element | mm | 7.1 |
| L0-P | Post style brake fixture with wheel hub connection | - | 8.1 |
| L0-U | Univeral style brake fixture without wheel hub connection | - | 8.1 |
| LHC | Left-hand corner of the vehicle | - | 8.1 |
| LRO | Lateral runout | µm | 8.1 |
| μ | Average by distance friction value for disc brakes (Friction Coefficient) | (unitless) | 13.1 |
| M1 | Vehicles used for the carriage of passengers, comprising not more than eight seats in addition to the driver's | - | 7.1 |
| MMix | Molar mass of air in the balance room | g/mol | 12.1 |
| Mveh | Vehicle test mass to simulate on the dynamometer | kg | 8.1 |
| MRO | Mass in running order | - | 8.1 |
| MS | Microsoft | - | 13.1 |
| MVL | Maximum vehicle load | - | 8.1 |
| ν | kinematic viscocity of air | m2/s | 7.3 |
| N1 | Vehicles used for the carriage of goods and having a maximum mass not exceeding 3.5 tons | - | 7.1 |
| Nin(dx) | Upstream PN concentration for particles of electrical mobility dx | #/cm3 | 14.5 |
| Nout(dx) | Downstream PN concentration for particles of electrical mobility dx | #/cm3 | 14.5 |
| NQ | Average normalized cooling airflow | Nm³/h | 7.2 |
| NQPM2.5 | Average normalized PM2.5 sampling airflow | Nl/min | 12.1 |
| NQPM10 | Average normalized PM10 sampling airflow | Nl/min | 12.1 |
| NQTPN10 | Average normalized TPN10 sampling airflow | Nl/min | 12.2 |
| NQSPN10 | Average normalized SPN10 sampling airflow | Nl/min | 12.2 |
| NQs | Average normalized airflow in the sampling nozzle | Nm³/h | 12.1 |
| OD | Disc/drum outer diameter | mm | 8.1 |
| Pb | Atmospheric pressure in the balance room | kPa | 12.1 |
| Pbrake | Brake pressure | kPa | 13.2 |
| Pr | Particle penetration | % | 14.5 |
| pthreshold | Threshold pressure or minimum pressure required to develop braking torque | kPa | 8.1 |
| PM2.5 | Particle mass for aerosols with aerodynamic diameter below 2.5 µm | mg, mg/m3, mg/km | 7.1 |
| PM10 | Particle mass for aerosols with aerodynamic diameter below 2.5 µm | mg, mg/m3, mg/km | 7.1 |
| PMMass | Buoyancy-corrected filter mass | µg, mg | 12.1 |
| PMLoaded | Filter mass without buoyancy correction | µg, mg | 12.1 |
| Plane A | Vertical plane aligned with the enclosure’s inlet | - | 7.3 |
| Plane A1 | Horizontal level aligned with the centerline of the brake assembly and the duct axis | - | 7.3 |
| Plane B | Vertical plane at the end of the transition from the inlet duct to the central section of the enclosure | - | 7.3 |
| Plane C | Vertical plane tangential to the largest brake for M1, N1 vehicle category | - | 7.3 |
| Plane D | Vertical plane aligned with the centerline of the brake assembly | - | 7.3 |
| PND1 | Primary particle diluter | - | 12.2 |
| PND2 | Secondary particle diluter | - | 12.2 |
| PAO | poly-alpha-olefin | - | 12.1 |
| PBT | Peak brake temperature | ℃ | 13.1 |
| PCRF | Particle concentration reduction factor | - | 12.2 |
| PM | Particle mass | mg, mg/m3, mg/km | 6.0 |
| PN | Particle number | #/m³, #/km | 6.0 |
| PNC | Particle number counter | - | 12.2 |
| PTFE | Poly tetra fluoroethylene | - | 12.1 |
| PTT | Particle transfer tube | - | 12.2 |
| Q | Average measured (actual) cooling airflow | m3/h | 7.2 |
| Qset | Nominal (or set) cooling airflow | m3/h | 7.2 |
| QPM2.5 | PM2.5 sampling airflow (actual) | l/min | 12.1 |
| QPM2.5-set | Nominal (or set) PM2.5 sampling airflow | l/min | 13.2 |
| QPM10 | PM10 sampling airflow (actual) | l/min | 12.1 |
| QPM10-set | Nominal (or set) PM10 sampling airflow | l/min | 13.2 |
| rb | Bending radius of the cooling air duct | mm | 7.4 |
| reff | Brake effective radius | mm | 8.1 |
| rP | Bending radius of the sampling probe or sampling line | mm | 12.1 |
| ρa | Density of air | kg/m3 | 12.1 |
| ρf | Density of PM filter material | kg/m3 | 12.1 |
| ρw | Density of PM microbalance calibration object | kg/m3 | 12.1 |
| R | Molar mass constant | J/kg/mol | 12.1 |
| R1 | The radius of the bend downstream of the sampling plane (or upstream of the sampling plane in a different layout) | mm | 7.1 |
| RA | Vehicle rear axle | - | 8.1 |
| RH | Cooling air relative humidity | % | 7.2 |
| RHC | Right-hand corner of the vehicle | - | 8.1 |
| RR | Tyre dynamic rolling radius | mm | 8.1 |
| SPN10 | Solid particle concentration | #/cm3 | 7.1 |
| SPN10# | Average normalized and PCRF-corrected SPN10 concentration | #/Ncm3 | 12.2 |
| SPNback | Average normalized SPN10 concentration during background check | #/Ncm3 | 7.2 |
| SPNback(dist) | Average SPN10 concentration per unit distance driven during the background check | #/km | 7.2 |
| SQ | Output signal for cooling airflow | V | 7.1 |
| SRH | Output signal for cooling air humidity | V | 7.1 |
| St | Output signal for cooling air temperature | V | 7.1 |
| SAE | Society of Automotive Engineers | - | 12.3 |
| SEE | Standard error of estimate | - | 14.3 |
| tbrake | Total duration of deceleration event (stop duration) | s | 13.1 |
| t90 | Response time of particle number counter | s | 12.2 |
| τbrake | Brake torque | N·m | 9.4 |
| τbrake-avg | Time-averaged brake torque | N·m | 13.1 |
| T | Cooling air temperature | ℃ | 7.2 |
| Ta | Air temperature in the balance room | ℃ | 12.1 |
| Tavg | Average brake temperature | ℃ | 10.1 |
| Tbrake | Brake (disc/drum) temperature | ℃ | 8.3 |
| TPN10 | Total particle concentration | #/cm3 | 7.1 |
| TPN10# | Average normalized and PCRF-corrected TPN10 concentration | #/Ncm3 | 12.2 |
| TPNback | Average normalized TPN10 concentration during background check | #/Ncm3 | 7.2 |
| TPNback(dist) | Average TPN10 concentration per unit distance driven during the background check | #/km | 7.2 |
| U | Average cooling airspeed | km/h, m/s | 7.2 |
| Us | Average airspeed of air entering the sampling nozzle | km/h, m/s | 12.1 |
| ULPA | Ultra-low particulate air | - | 12.1 |
| V | Average actual linear speed of the WLTP-Brake cycle | km/h | 12.2 |
| Vset | Average nominal linear speed of the WLTP-Brake cycle (=43.7 km/h) | km/h | 7.2 |
| VPR | Volatile particle remover | - | 12.2 |
| Wf | Friction work | J/kg | 9.4 |
| WLn | Nominal wheel load without accounting for vehicle road loads or any other type of losses | kg | 8.1 |
| WLt | Test wheel load after accounting for vehicle road loads or any other type of losses | kg | 8.1 |
| WLTP | World-wide harmonized light vehicle test procedure | - | 6.0 |

General Requirements

Compliance Requirements

The compliance of a brake system with this GTR shall be evaluated against the regional emission limits as defined by each Contracting Party. The compliance shall be demonstrated by testing the worst-performing representative of a brake family according to paragraphs 6-14 of this UN GTR.

Definition of Brake Family

A brake family shall be composed of brake systems that may be used in the same vehicle category and that are the same in terms of the following emission characteristics and technical criteria (Criteria based on UNR 13 to be further elaborated and finalized before the submission of the final working document):

	……

	……

	……

The manufacturer shall identify the worst performing brake system in terms of emissions and submit it to the authority as a candidate for testing. The authority shall approve the selection if appropriate and may also select any member of the family for testing. The maximum number for testing during type approval is two brake systems per family.

Rounding Requirements

Rounding of data in the data exchange file is not permitted. In the pre-processing file, the data may be rounded to the same order of magnitude of the accuracy of the measurement of a respective parameter and based on the number of decimals defined for the parameter in Paragraph 13 of this UN GTR.

General Overview

A brake emissions test includes three test sections. Each section contains one or more trips with a series of events. The main events which induce brake work and generate brake emissions are the deceleration events. Figure 6.1 provides a schematic overview of a brake emissions test. The three sections of the brake emissions test are:

- Brake cooling adjustment. This section uses Trip #10 of the WLTP-Brake cycle. The cooling adjustment section is described in detail in Paragraph 10;

- Brake bedding. This section uses five executions of the WLTP-Brake cycle. The bedding section is described in detail in Paragraph 11;

- Brake emissions measurement. This section includes one performance of the WLTP-Brake cycle. The emissions measurement section is described in detail in Paragraph 12.

Figure 6.1
Structure of the brake emissions tests for vehicles with full-friction braking

The correct execution of a brake emissions test requires the test facility to conduct and document the following steps:

- Ensure the test system meets the requirements defined in Paragraph 7 regarding the system layout, cooling airflow, temperature, and humidity control, brake inertia dynamometer capabilities, brake enclosure design, sampling tunnel design, and sampling plane design;

- Meet all requirements defined in Paragraph 8 for test preparation involving input parameters, test setup, measurement of brake temperature, and brake positioning;

- Be capable of executing the WLTP-Brake cycle per Paragraph 9 and demonstrate compliance with the quality checks;

- Perform the brake cooling adjustment section as defined in Paragraph 10;

- Perform the brake bedding section for all types of brakes as defined in Paragraph 11;

- Execute all items from Paragraph 12 for the brake emissions measurement, including particle mass, particle number, and mass loss of the wearable brake hardware;

- Report the outcomes of the test following Paragraph 13;

- Comply with Paragraph 14 for minimum calibration requirements and periodic evaluations.

Test System Requirements

Overall Test System Layout

This GTR defines a standard dynamometer test method aiming for repeatable and reproducible measurements of particle emissions from brakes equipped in category 1-1 vehicles and category 2 vehicles with a fully laden mass below 3.5 tons (also mentioned as M1 and N1 vehicle categories). The technical system to perform brake emissions tests requires a system approach. The execution of a valid brake emissions test requires a robust integration of several subsystems to ensure the drive cycle, cooling air, dynamometer control, brake enclosure, sampling tunnel, aerosol sampling train, and the data collection, altogether meet the requirements specified in this GTR.

Figure 7.1
Indicative layout for performing brake emissions test in the laboratory
The layout has the sampling tunnel (7) connected directly to the brake enclosure (a layout with a bend in the sampling tunnel i.e. downstream of the enclosure and upstream of the sampling plane is also feasible) and assumes three sampling probes (a four sampling probes layout is also feasible). The brake dynamometer is not depicted but only denoted (gray area) – a graphical representation of the brake dynamometer is given in Figure 7.2

Figure 7.1 provides a layout that includes the minimum required subsystems to carry out a brake emissions test using a brake inertia dynamometer. The illustrated layout features a climatic conditioning unit with variable flow fan(s) that supplies the setup with conditioned air. The conditioned air enters a brake enclosure designed to fit the entire assembly of the brake under testing. The brake inertia dynamometer enables and controls the testing of the brake. The enclosure is directly connected to the sampling tunnel near the end of which three (or four) sampling probes are mounted. The sampling probes are used to extract the particles from the tunnel towards the PM and PN measurement setup. A flow measurement device is installed in the tunnel downstream of the sampling plane. The positioning and dimensions of the different elements are indicative and are provided for illustration purposes; therefore, exact conformance with Figure 7.1 is not required.

There are several accepted configurations to lay out the air handling and control subsystems. All designs can use the same (not depicted) brake inertia dynamometer, control software, data acquisition, and brake fixture. However, the testing facility shall ensure that all configurations include at the least the subsystems and characteristics laid down in Table 7.1. Details regarding the different elements of the setup are given in the corresponding paragraphs of this GTR as indicated in Table 7.1.

Table 7.1
Subsystems and characteristics required for the brake emissions testing setup

| Element in Figure 7.1 | Subsystem |
| --- | --- |
| 1 | Climatic conditioning unit with variable flow fan(s), air temperature, and air humidity control per paragraph 7.2.1 |
| 2 | Cooling air filtering medium per paragraph 7.2.2.1 |
| 3 | Cooling air temperature and humidity sensors placed upstream of the brake enclosure per paragraphs 7.2.1.1 and 7.2.1.2 |
| 4 | Brake enclosure per paragraph 7.4 |
| 5 | Brake assembly connected to the brake dynamometer per paragraph 8.4.1 |
| 6 | Brake dynamometer (not depicted but only denoted in gray) per paragraph 7.3 |
| 7 | Sampling tunnel per paragraph 7.5 |
| 8 | Particles sampling plane with the corresponding PM and PN sampling probes per paragraph 7.6 |
| 9 | Instrument cluster to measure PM mass and PN concentrations per paragraphs 12.1 and 12.2, respectively |
| TPN10, SPN10 | Systems to control, measure, and output readings of total and solid particle count per paragraph 12.2 |
| PM2.5, PM10 | Systems to control sampling airflow, sample particle mass on filters, and output signals for PM2.5 and PM10 per paragraph 12.1 |
| 10 | Airflow measurement element placed downstream of the sampling plane per paragraph 7.2.3 |
| Symbol in Figure 7.1 | Characteristic |
| L1 | Minimum length of the straight duct at the inlet of the brake enclosure per paragraph 7.4.2 |
| L2 | Minimum length from the last disturbance and upstream of the sampling plane per paragraph 7.6 |
| L3 | Minimum length to the next disturbance and downstream of the sampling plane per paragraph 7.6 |
| L4 | Minimum length from the last disturbance and upstream of the airflow measurement element per paragraph 7.2.3 |
| L5 | Minimum length to the next disturbance and downstream of the airflow measurement element per paragraph 7.2.3 |
| R1 | The radius of the bend downstream of the sampling plane (or upstream of the sampling plane in a different layout) per paragraph 7.5 |
| SQ, St, SRH | Output electronic signals for cooling airflow, temperature, and humidity per paragraphs 7.2.1 and 7.2.3 |
| di | Reference duct diameter where there is a requirement in the minimum number of duct diameters for length or radii. This is the same as the sampling tunnel diameter |
| Hs, He | Points that define the beginning (Hs) and the end (He) of the mandatory horizontal part in the layout (in the direction of the flow) per paragraph 7.4.2 |

Climatic Conditioning Unit and Cooling Air

The conditioned cooling air a) provides clean and continuous cooling to the brake assembly and b) transports the aerosol particles from the enclosure into the sampling tunnel and the sampling probe. The cooling air needs to be under stable conditions for temperature and humidity following the specifications described in paragraph 7.2.1, clean with low background concentration values as defined in paragraph 7.2.2, and at a constant flow to ensure repeatable and reproducible test conditions following the specifications described in paragraph 7.2.3.

The conditioned cooling air is supplied to the testing setup by the climatic conditioning unit. A typical system configuration includes cooling devices to cool and dehumidify the air, heating devices to increase the air temperature, and steam or water mist generators to increase the amount of water. Integral to the unit are the closed-loop proportional integral derivative controls, alarms, and sensors to monitor the condition of all devices and interfaces. The system shall consist of a variable flow fan able to supply the layout with conditioned cooling air over a wide range of airflows. The system can combine two variable flow fans (one to push and one to pull) to provide a slight negative pressure inside the sampling tunnel. The climatic conditioning unit control shall provide the interfaces to the operator and the dynamometer.

Cooling Air Conditioning

The testing facility shall continuously monitor the temperature and humidity of the conditioned cooling air. For that reason, the testing facility shall install temperature and humidity sensors in the last segment of the duct before entering the brake enclosure. Positioning the sensors upstream of the brake enclosure avoids influencing the feedback signals with the thermal load from the braking events. Figure 7.1 provides an indicative position for the temperature and air humidity sensors (element 3).

The temperature sensor shall have an accuracy of ±1 °C. The relative humidity sensor shall have an accuracy of ±5 per cent of the nominal value. The testing facility shall use the signals from these sensors to assess the stability of the cooling air’s temperature and humidity.

Cooling Air Temperature

Cooling air temperature at the measurement point shall be constant throughout the entire brake emissions test. The testing facility shall carry out the following steps:

- Set the cooling air temperature at 20 °C. The average cooling air temperature shall not deviate more than ±2 °C of the set (nominal) value. Testing facilities shall stay as close as possible to the nominal value of 20 °C;

- The average temperature requirements for the cooling air defined in point (a) of this paragraph apply to all sections of the brake emissions test including cooling air adjustment, bedding procedure, and emissions measurement;

- Calculate the average cooling air temperature using the data of the given parameter (Column N: “Cooling Air Temperature”) in the Time-Based file. Report the average cooling air temperature as defined in paragraph 13.4;

- The instantaneous cooling air temperature may deviate up to ±5 °C of the nominal value (15 °C ≤ T ≤ 25 °C), for no longer than the 10 per cent duration of the test (cooling sections not included), provided that the average temperature meets the requirements defined in point (a) of this paragraph:

- The total time during the ±5 °C deviations allowed over the cooling air adjustment section shall be less than 527 s;

- The total time during the ±5 °C deviations allowed over each WLTP-Brake cycle of the bedding and emission measurement sections shall be less than 1583 s (cooling sections not included).

- If the average or the instantaneous cooling air temperature falls out of the limits specified in this paragraph, the test is invalid.

Cooling Air Humidity

Cooling air relative humidity shall be constant throughout the entire brake emissions test. The testing facility shall carry out the following steps:

- Set the relative humidity of the cooling air to a nominal value of 50 per cent. The average cooling air humidity shall not deviate more than ±5 per cent of the nominal value. Testing facilities shall stay as close as possible to the target value of 50 per cent;

- The average relative humidity requirements for the cooling air defined in point (a) of this paragraph apply to all sections of the brake emissions test including cooling air adjustment, bedding procedure, and emissions measurement;

- Calculate the average relative humidity of the cooling air using the data of the given parameter (Column O: “Cooling Air Relative Humidity”) in the Time-Based file. Report the average relative humidity of the cooling air as defined in paragraph 13.4;

- The instantaneous cooling air relative humidity can deviate up to ±30 per cent of the nominal value (20 % ≤ RH ≤ 80 %) for no longer than the 10 per cent of the duration of the test (cooling sections not included), provided that the average relative humidity meets the requirements defined in point (a) of this paragraph:

- The total time during the ±30 per cent deviations allowed over the cooling air adjustment section shall be less than 527 s;

- The total time during the ±30 per cent deviations allowed over each WLTP-Brake cycle of the bedding and emission measurement sections shall be less than 1583 s (cooling sections not included).

- If the average or the instantaneous relative humidity falls out of the predefined limits specified in this paragraph, the test is invalid.

Cooling Air Cleaning

Cooling Air Filtering

The cooling air entering the test system shall pass through a medium capable of reducing particles of the most penetrating particle size in the filter material by at least 99.95 per cent or through a filter of at least class H13 as specified in EN 1822. Any other type of filter applied to remove volatile organic species (charcoal, activated carbon, or equivalent) shall be installed upstream of the H13 (or equivalent) filter. The testing facility shall follow the manufacturer’s specifications and ensure the installation of the filter does not affect the flow rate and the airflow distribution. Figure 7.1 provides an indicative position for the air filtering device (element 2).

Particle Background Verification

The particle background in the layout shall be defined at a PN concentration basis. The testing facility shall measure the particle background using the same instrumentation used for the PN emission measurements. Details regarding the PN measurement system are provided in paragraph 12.2. The testing facility shall measure and report both TPN10 and SPN10 background concentrations at two levels: System-level and brake emissions test level.

7.2.2.2.1.	Particle Background Verification at the System Level

The first level concerns the system background check upon the installation of the setup, after major maintenance, or when there are indications of a system malfunction. The testing facility shall apply the following steps for a complete background verification at the system level:

- Perform the background verification with neither the brake fixture nor any brake components installed inside the brake enclosure;

- Perform the background verification with the PN measurement system operating at the minimum certified dilution ratio;

- Commence the background verification at least five minutes after the cooling airflow is stabilized to the average values per paragraph 7.2.3 (for the nominal value at each level) for cooling airflow stability and to the average values per paragraph 7.2.1 for cooling air temperature and relative humidity;

- Perform the background verification at three different tunnel flow settings representing the operating range of the test facility. The minimum, 50 per cent, and maximum operational airflow of the system shall be tested. The test facility may use a single nozzle size for sampling PN during the system background verification with the different airflow settings;

- The background verification shall run for approximately 30 minutes to allow the background concentration to stabilize. The background concentration is stable when its average value is below the maximum permissible level per paragraph 7.2.2.2.3 for at least five minutes or when its value does not vary by more than ±2 #/Ncm³ for at least one minute.

- 7.2.2.2.2.	Particle Background Verification at the Test Level

The second level concerns the background verification before and after the execution of a brake emissions test. The testing facility shall carry out the following steps for the pre-test verification:

- Perform the regular background pre-test before the bedding section with the brake assembly mounted; however, with the disc/drum not rotating and the pads/shoes being fully retracted. Do not apply braking during the background concentration check procedure (zero brake pressure);

- Perform the pre-test verification with the PN measurement system operating at the dilution ratio selected for the corresponding brake emissions test;

- Commence the background pre-test check at least five minutes after the cooling airflow is stabilized to the average values per paragraph 7.2.3 (for the nominal value at each level) for cooling airflow stability, and to the average values per paragraph 7.2.1 for cooling air temperature and relative humidity;

- Perform the background pre-test check for at least 5 min, or as long as it takes for the background concentration to stabilize at the airflow setting of the emissions test. The background concentration shall be considered stabilized when its average value is below the maximum limit defined in paragraph 7.2.2.2.3 for at least five minutes or when its value does not vary by more than ±2 #/Ncm³ for at least one minute.

The testing facility shall carry out the following steps for the post-test verification:

- Perform the regular background post-test before purging and with the brake assembly mounted. The disc/drum shall not rotate and the pads/shoes shall not be disturbed. Do not apply braking during the background concentration check procedure (zero brake pressure);

- Perform the post-test verification with the PN measurement system operating at the dilution ratio selected for the brake emissions test;

- Commence the background post-test check right after the emissions test and with the cooling airflow stabilized to the average values per paragraph 7.2.3 (for the nominal value at each level) for cooling airflow stability, and to the average values per paragraph 7.2.1 for cooling air temperature and relative humidity;

- Perform the background post-test for at least 5 min, or as long as it takes for the background concentration to stabilize at the airflow setting of the emissions test. The background concentration shall be considered stabilized when its average value is below the maximum limit defined in paragraph 7.2.2.2.3 for at least five minutes or when its value does not vary by more than ±2 #/Ncm³ for at least one minute.

- 7.2.2.2.3.	Calculation and Reporting of the Particle Background

The background shall be measured and reported at a TPN10 and SPN10 concentration basis at standard conditions (273.15 K and 101.325 kPa). The testing facility shall apply the following procedure:

- Measure the average value of both TPN10 (TPNback) and SPN10 (SPNback) background concentrations at system and test levels following the instructions in paragraphs 7.2.2.2.1 and 7.2.2.2.2. Report the background values in particle number concentration (#/Ncm3) as defined in paragraph 13.4 for all brake emissions tests;

- Perform a zero verification of the PN measurement device. Apply a filter of appropriate performance at the PN measurement device inlet and record the PN concentration. The reading shall not exceed 0.2 #/cm³. Upon removal of the filter, the PN measurement device shall show an increase in measured concentration and a return to ≤ 0.2 #/cm³ on the replacement of the filter. The PN measurement device shall not report any errors;

- The average background concentration in the tunnel shall not exceed the maximum limit of 20 #/Ncm³ for each TPN10 and SPN10. The limit of 20 #/Ncm3 applies to the background concentration at both system and test levels as described in paragraphs 7.2.2.2.1 and 7.2.2.2.2;

- Failure to comply with the particle background limits defined in point (c) of this paragraph results in an invalid test;

- The test facility shall not subtract the background concentration values when reporting the TPN10 and SPN10 concentration values of the brake emissions measurement section per paragraph 12.2.3.3.

The testing facility shall also report the background concentrations expressed as the number of particles per distance driven to reflect the changes in the cooling air settings when testing different brakes. The calculation of the background concentration as the number of particles per unit distance driven is determined by equations 7.1 and 7.2:

| TPNback(dist)=106×TPNback ×NQ÷VSet | (Eq. 7.1) |
| --- | --- |
| SPNback(dist)=106×SPNback ×NQ÷VSet | (Eq. 7.2) |

Where:

TPNback(dist)		is the TPN10 background concentration in #/km;

SPNback(dist)		is the SPN10 background concentration in #/km;

TPNback		is the TPN10 background concentration in #/Ncm³;

SPNback		is the SPN10 background concentration in #/Ncm³;

NQ			is the average normalized airflow in the tunnel in Nm³/h;

VSet		is the average nominal linear speed of the WLTP-Brake cycle in km/h.

- The PN background concentrations (TPNback and SPNback) correspond to the average PN values calculated throughout the background check from the given parameters in the Time-Based file (Column Y: “TPN10 Concentration Normalized - PCRF Corrected” and Column AB: “SPN10 Concentration Normalized - PCRF Corrected”);

- Calculate the normalized average cooling airflow (NQ) during the background verification procedure from the given parameter in the Time-Based file (Column L: “Cooling Airflow Actual Normalized”);

- The average nominal linear speed of the WLTP-Brake cycle equals to 43.7 km/h (Vset = 43.7 km/h);

- Calculate and report the background particle concentration values per unit distance driven only at the test level (and not at a system level) in all emissions tests.

Cooling Airflow

The testing facility shall measure and report the cooling airflow throughout the entire brake emissions testing procedure. The measurement of the cooling airflow shall meet the following requirements:

- The method of measuring cooling airflow shall be such that measurement is accurate to ±2 per cent under all operating conditions;

- Measure the cooling airflow downstream of the enclosure and the sampling plane. Figure 7.1 provides an indicative position for the flow measurement device (element 10);

- For a single-point measurement, locate the flow measurement element at the centre of the duct, at least five diameters downstream and two diameters upstream of any flow disturbance;

- For a multi-point measurement, install the flow measurement element perpendicular to the flow direction, at least five diameters downstream and two diameters upstream of any flow disturbance;

- Use a flow measurement device calibrated to report airflow at both operating and standard conditions (273.15 K and 101.325 kPa). The temperature sensor shall have an accuracy of ±1 °C. The pressure measurements shall have precision and accuracy of ±0.4 kPa;

- When the airflow measurement device is not calibrated to report values at standard conditions, ensure it includes a temperature sensor installed immediately before the measuring device. The temperature sensor shall fulfil the accuracy requirements described in point (e) of this paragraph. Use this measurement for normalizing the airflow values;

- When the airflow measurement device is not calibrated to report values at standard conditions, ensure it includes the measurement of the pressure difference from atmospheric pressure taken upstream from the measuring device. The pressure measurements shall fulfil the precision and accuracy requirements described in point (e) of this paragraph. Use this measurement for normalizing the airflow values;

- When using air filters to protect the airflow measurement device from contamination, install the filter at least five diameters upstream of the flow measurement device. Follow the recommendations regarding the type and specifications of the filter from the manufacturer of the flow measurement device.

The testing facility shall ensure that the cooling airflow is constant throughout the entire brake emissions testing procedure as follows:

- The set (nominal) value for the cooling airflow (Qset) shall be the same and constant during a given brake emissions test. The same set value shall apply to brake cooling adjustment, bedding, and emissions measurement (including cooling sections);

- During the cooling adjustment section, the average measured cooling airflow shall be within ±5 per cent of the set value defined at the beginning of the test;

- During the bedding section, the average measured cooling airflow shall be within ±5 per cent of the nominal value as defined during the cooling adjustment section for the given brake;

- During the emissions measurement, the average measured cooling airflow shall be within ±5 per cent of the nominal value as defined during the cooling adjustment section for the given brake;

- In case the average nominal or measured cooling airflow does not meet any of the requirements defined in this paragraph, the test is invalid;

- For the calculation of the average measured cooling airflow use the data of the given parameter (Column K: “Cooling Airflow Actual”) in the Time-Based file;

- The instantaneous cooling airflow can deviate up to ±10 per cent of the nominal value for no longer than 5 per cent of the duration of the cycle, provided that the average airflow meets the requirements (b) and (d) from this paragraph. The instantaneous cooling airflow requirement applies to the cooling adjustment and the emissions measurement section:

- For the cooling adjustment section, the instantaneous cooling airflow can deviate up to ±10 per cent of the set value for no longer than 264 s;

- For the emissions measurement section, the instantaneous cooling airflow can deviate up to ±10 per cent of the set value for no longer than 792 s (cooling sections not included).

- In addition to the compliance to the average and instantaneous limits defined in this paragraph, the cooling airflow in combination with the airflow in the PM and PN sampling lines shall meet the isokinetic sampling requirements per paragraphs 12.1.2.3 and 12.2.3.2, respectively;

- A system leak check covering the ductwork and the enclosure shall take place before testing. Set the cooling airflow at 50 per cent of the test setup’s maximum capacity. If the average measured flow in the sampling tunnel is within ±5 per cent of the set value proceed with the testing. If the flow fluctuates beyond ±5 per cent of the set value cease testing activities, verify the flow measurement device, identify possible sources of the leak(s), take corrective action to resolve the issue, and resume testing by first performing a successful leak check.

The testing facility shall report the cooling airflow in the Time-Based file of the brake emissions test as follows:

- When the cooling airflow is measured, report the values in Column K: “Cooling Airflow Actual”;

- Normalize and report the measured cooling airflow at standard conditions in Column L: “Cooling Airflow Actual Normalized”;

- If the measurement device reports flow at standard conditions, introduce the values directly in Column L: “Cooling Airflow Actual Normalized”. In this case, report also the non-normalized measured airflow as described in point (a) of this paragraph;

- Calculate the corresponding cooling airspeed using the measured airflow and the duct diameter based on equation 7.3:

| U=(3.6×4×Q)(3600×π×di2) | (Eq. 7.3) |
| --- | --- |

Where:

U	is the average cooling airspeed in km/h per Table 13.2;

Q 	is the average measured cooling airflow in m³/h per Table 13.2;

di	is the sampling tunnel diameter in m per Table 7.1.

- Report the calculated cooling airspeed in Column M: “Cooling Airspeed Actual”;

- When the cooling airspeed is measured, report the values in Column M: “Cooling Airspeed Actual” in km/h;

- Calculate the corresponding cooling airflow using the measured cooling airspeed and the duct diameter based on equation 7.4:

| Q=(U×1000×π×di2)4 | (Eq. 7.4) |
| --- | --- |

- Report the calculated cooling airflow in Column K: “Cooling Airflow Actual”;

- Normalize and report the calculated cooling airflow at standard conditions in Column L “Cooling Airflow Actual Normalized”.

Brake Inertia Dynamometer and Automation Systems

The brake inertia dynamometer is a technical system that provides the controlled kinetic energy to the brake under test. It primarily transforms rotational kinetic energy into thermal energy (Figure 7.2‒S1). Other energy transformations (not part of this GTR) include vibration or noise. The brake inertia dynamometer shall consist of at least the following elements:

- A variable speed electric motor to accelerate or keep the rotational speed constant. It also modulates the test inertia in real driving conditions and simulates regenerative braking;

- A servo controller (hydraulic or electric) to actuate the brake under testing;

- A mechanical assembly to mount the brake under testing, allow free rotation of the disc or drum, and absorb the reaction forces from braking;

- A rigid structure to mount all the mandatory subsystems. The structure shall be capable of absorbing the forces and moments generated by the brake under testing;

- Sensors and devices to collect data and monitor the operation of the test system.

Figure 7.2 provides a layout of the test system with the brake inertia dynamometer and shows the interactions with the minimum subsystems required to execute a brake emissions test following this UN GTR. Integral to the test system is the automation, controls, and data acquisition system (Figure 7.2‒S2) for continuously controlling the rotational speed of the motor (Figure 7.2‒1) as well as the operation and the interactions between the different systems (Figure 7.2‒S3, S4, S5). Subsystems S3, S4, and S5 are described in detail in paragraphs 7.2, 7.4-7.5, and 9.1-9.2, respectively. The different elements and subsystems in Figure 7.2 are indicative; therefore, exact conformance with the figure is not mandatory.

Figure 7.2

Brake inertia dynamometer and automation systems in the overall test layout

S1: Brake inertia dynamometer, S2: Automation, control, and data acquisition system, S3: Air handling system, S4: Brake enclosure and sampling plane, S5: Emissions measurement system, C1 and C2: Testing facility energy controls and monitoring system. The grey arrow represents the particle emissions sample from the brake under testing

The automation, control, and data acquisition system performs all the functions that enable the brake emissions test. It accelerates the brake during acceleration events, maintains constant speed during cruise events, and modulates the frictional torque during deceleration events to reduce the kinetic energy of the rotating masses (Figure 7.2‒2). Additionally, the automation, controls, and data acquisition system provides an interface to the operator, stores the data from the test, and handles the interfaces with other systems in the testing facility. The automation system shall be capable of using active torque control on the electric motor to increase or decrease the total effective test inertia during deceleration events. The electric motor shall also be capable of absorbing part of the kinetic energy equivalent to the road loads and the regenerative braking from the vehicle’s powertrain. The software that operates the test system shall be capable of performing at least the following functions:

- Execute the driving cycle automatically by operating all the closed-loop processes (mainly for brake controls, cooling air handling, and emissions measurements instruments);

- Continuously record data from all relevant sensors to generate the outputs defined in Paragraph 13 of this UN GTR;

- Monitor signals, messages, alarms, or emergency stops from the operator and the different systems connected to the test system.

Brake Enclosure Design

The brake enclosure is defined as the test chamber where the brake assembly is installed during brake emissions testing. It is a sealed chamber that prevents untreated air from entering and contaminating the air flowing around the brake assembly. The brake enclosure directs uniform conditioned air to cool down the brake and sweep the aerosol particles into the sampling tunnel. Design requirements for the enclosure aim at providing general guidelines to ensure systems’ comparability related to brake cooling and particle transport efficiency. Figure 7.1 provides an indicative position for the brake enclosure (element 4).

General Elements

An indicative shape of the enclosure is illustrated in Figure 7.3. The enclosure is defined by one horizontal and four vertical planes. Plane A1 represents the horizontal level aligned with the centerline of the brake assembly and the axis of the inlet and outlet ducts. A represents the vertical plane aligned with the enclosure’s inlet. B represents the vertical plane at the end of the transition from the inlet duct to the central section of the enclosure. Plane C shall be defined by the largest brake system applied on M1 and N1 vehicles or any brake with similar dimensions. D represents the vertical plane aligned with the centerline of the brake assembly (rotation axis).

The inlet transition volume (Figure 7.3 – 1) is defined as the section of the enclosure between planes A and B and is illustrated with a grey colour. The enclosure may have flow straighteners or diffusion plates placed between planes A and B. The transition angle “a” (Figure 7.3 – 2) defines how smoothly the transition area develops in the enclosure. In Figure 7.3, the cooling air flows from right to left.

Figure 7.3
Indicative schematic representation of the brake enclosure

Design Specifications

The following general specifications shall be met for the design of the brake enclosure and the verification of proper mixing and flow uniformity therein:

- The brake enclosure shall have two conical or trapezoidal sections with an intersecting cylinder in the middle concentric to the brake assembly;

- The transition from plane A to plane B shall be smooth and continuous with no abrupt changes both in the vertical plane along the duct axis and in the horizontal plane A1 to the sides of the enclosure’s centre section (intersecting cylinder);

- Design the inlet and outlet cross-sections, ensuring smooth transition angles (15° ≤ a ≤ 30°) to avoid sudden changes in cross-section shape or size;

- The transition points between the segments shall not have any imperfections or features that may collect brake particles that could become airborne later during the test;

- If fasteners are applied along the transition points, they shall not protrude into the enclosure area;

- The cooling air shall enter and exit the enclosure only in the horizontal direction (i.e. the major axis of the enclosure – plane A1 – shall align with the airflow direction). The tunnel ducting shall be horizontal for at least two duct diameters (2∙di) upstream of the enclosure’s inlet. The tunnel ducting after the enclosure shall also be horizontal at least until two duct diameters (2∙di) downstream of the sampling plane;

- The surfaces of the brake enclosure that come into contact with the aerosol shall have a seamless construction. Use stainless steel with an electropolished finish (or equivalent) to attain an ultra-clean and ultra-fine surface and to enhance corrosion resistance;

- Select all materials (including seals) to ensure sufficient protection against the media used (e.g. brake fluid) during setup. All enclosure gaps shall be air-tight sealed using gasket linings or equivalent;

- The airflow at the entrance of the enclosure shall remain turbulent with a Reynolds number of at least 4000 for all airflow testing settings to ensure sufficient mixing. Calculate the Reynolds number Re for each brake emissions test using equation 7.5;

| Re=U×diυ | (Eq. 7.5) |
| --- | --- |

Where:

Re		is the Reynolds number for the brake emissions test (unitless);

U		is the average cooling airspeed in m/s per Table 13.2;

di		is the sampling tunnel diameter in m per Table 7.1;

υ		is the kinematic viscosity of air (use default value of 1.48×10−5 m²/s).

- Design the cross-section area at the inlet so that the airspeed at plane C remains below the maximum permissible tolerance for speed uniformity defined in point (k) of this paragraph. If necessary, use flow straighteners or diffusion plates at the inlet’s side to ensure the highest possible level of uniform flow;

- Apply Computational Fluid Dynamics (CFD) to calculate the airspeed values at nine positions of plane C as defined in Figure 7.4. Use the entire cross-section area of plane C to determine the calculation positions. Define plane C considering a disc diameter of 450 mm. Carry out the computation at three different cooling flows representing the minimum, 50 per cent, and the maximum of the operational range of the test system. Simulate for a period sufficient to detect any instability in the airspeed pattern that may affect the airspeed values. Conduct the simulation without a brake assembly or a brake fixture installed. Airspeed at each position shall not vary by more than ±20 per cent of the arithmetic mean of all measurements for a given flow;

- It is strongly recommended that the testing facilities conduct measurements instead of the simulations to verify the airspeed uniformity defined in point (k) of this paragraph;

- Cleaning and maintenance of the brake enclosure shall follow the specifications provided by the manufacturer regarding the frequency and means.

Figure 7.4
Reference positions for airspeed verification

Left-hand side – Verification of proper mixing and flow uniformity using plane C for a disc with 450 mm outside diameter. Right-hand side – Distribution of calculation or measurement positions evenly distributed on plane C

Dimensions

The testing facility shall exercise due diligence to design the brake enclosure to fit the largest brake system applied on M1 and N1 vehicles. This includes possible additional parts designed to reduce particle emissions (e.g. brake filtering devices) provided their dimensions fit the corresponding wheel dimensions on which the brake is mounted. In addition, the testing facility shall verify the selection is within the capabilities for speed, brake test inertia, and brake torque expected during the test. Oversized brake enclosures may lead to low-pressure regions, low airspeeds to achieve the target brake temperatures, longer particle transport times, and increased possibility of leaks or defects. An indicative layout with the principal dimensions of the enclosure is illustrated in Figure 7.5.

Figure 7.5
Indicative schematic representation of the brake enclosure and main dimensions

- The brake enclosure shall be designed following the minimum specifications described below. In addition to the dimension specifications described in this paragraph, the testing facility shall ensure the selected dimensions provide a design that meets all requirements defined in paragraph 7.4.2.

- Design the brake enclosure symmetrically to plane A1. The length of plane A1 (IA1) represents the most extended length of the enclosure along the flow direction. The overall length of plane A1 (IA1) shall be between 1200 mm and 1400 mm (1200 mm ≤ IA1 ≤ 1400 mm);

- Design the brake enclosure symmetrically to plane D. The length of plane D (hD) represents the longest distance (height) of the enclosure perpendicular to the flow direction. The overall height of plane D shall be between 600 mm and 750 mm (600 mm ≤ hD ≤ 750 mm);

- The distance from plane C to plane D is approximately as large as the radius of the largest market available brake on an M1 and N1 vehicle (i.e. between 200-250 mm). Plane C’s position in Figure 7.5 is given only for illustration purposes and does not correspond to any actual dimension specification;

- Design the cross-section’s height at plane B (hB) such that the hB/hD ratio is always greater than 60 percent (hB/hD > 60 %). Design the cross-section’s transition depth at plane B to equal the axial depth of the enclosure as defined in (g) of this paragraph;

- Design the outlet’s transition length (li) and height (hB) such that they equal to the inlet’s transition length (li) and height (hB);

- The inlet and outlet diameters (di) shall equal to the diameter of the duct in the sampling tunnel as specified in paragraph 7.5;

- The maximum axial depth of the brake enclosure at plane D (parallel to the brake rotation axis) shall be between 400 mm and 500 mm.

Design of the Sampling Tunnel

The sampling tunnel is defined as the part between the outlet of the brake enclosure and the inlet of the probes in the sampling plane. Figure 7.1 provides an indicative position for the sampling tunnel in the overall layout (element 7). The testing facility shall ensure the design of the sampling tunnel meets the following requirements:

- The cooling air shall flow through round ducts with no or minimal variations in the cross-section between the enclosure exit and the sampling plane;

- Use stainless steel with an electropolished finish (or equivalent) for the surfaces of the tunnel that come into contact with the aerosol;

- Any transition between adjacent sectors shall not have imperfections or features that may accumulate brake particles. Whenever this is not feasible, ensure the transitions are engineered to minimize the accumulation of brake particles in the size range that is relevant for brake emissions testing;

- Ducts shall have a constant inner diameter di of at least 175 mm and a maximum of 225 mm (175 mm ≤ di ≤ 225 mm). The inner duct diameter di is defined as shown in Figure 7.6;

- A maximum of one bend of 90° or less may be applied downstream of the brake enclosure and upstream of the sampling plane provided that the specifications for the design and the minimum distancing described in (f) and (g) are met;

- If a bend is applied downstream of the brake enclosure and upstream of the sampling plane, the bending radius rb shall be at least two times the duct diameter (2∙di). The bending radius is defined as shown in Figure 7.6;

Figure 7.6
Definition of duct diameter (di) and bending radius (rb)

- If a bend is applied downstream of the brake enclosure and upstream of the sampling plane, a straight duct with a length of at least six times the duct diameter (6∙di) shall follow the bend before the sample plane is located. Additionally, a straight duct with a length of at least two times the duct diameter (2∙di) shall follow the sampling plane before any flow disturbance (e.g. a second bend in the setup);

- If there is no bend at the sampling plane, a straight duct with a length of at least six times the duct diameter (6∙di) shall follow the exit of the enclosure before the sample plane is located. Additionally, a straight duct with a length of at least two times the duct diameter (2∙di) shall follow the sampling plane before any flow disturbance;

- The provisions for the ducts described in points (a), (b), (c), and (d) of this paragraph shall apply to the overall test layout (including the ducts connecting to and from the climatic conditioning unit, fans, flow restrictors, and filter boxes).

Sampling Plane

The sampling plane shall consist of a minimum of three and a maximum of four sampling probes with corresponding sampling nozzles. Figure 7.1 provides an indicative position for the sampling plane in the overall layout (element 8). The following specifications apply to the sampling plane:

- PM and PN sampling shall take place in the same cross-section area in the sampling tunnel. Reference paragraphs 12.1.1.1 and 12.2.1.1 for PM and PN sampling, respectively;

- Select a 3-probes or 4-probes configuration depending on the duct diameter as specified in points (d) and (e) of this paragraph. Figure 7.7 illustrates the proper positioning of the PM and PN sampling probes for both the three and four sampling probes layout;

- Place the sampling probes/nozzles equally spaced around the central longitudinal axis of the sampling tunnel with a minimum distance between them of 50±2.5 mm (a1 ≥ 47.5 mm). Measure the distance using the outer diameter of the nozzle tips. A longer distance between the probes/nozzles can be applied provided that the nozzle-to-duct distance meets the specifications described in point (d) of this paragraph;

- The 3-probes setup requires a minimum duct inner diameter of 175 mm. Use the 3-probes setup when the duct diameter is smaller than 190 mm (di <190 mm);

- The 4-probes setup requires a minimum duct inner diameter of 190 mm. Use the 4-probes setup when the duct diameter is bigger or equal to 190 mm (190 mm ≤ di ≤ 225 mm).

Figure 7.7
Graphic representation of the spacing of the probes in the tunnel 
Red dots represent the PM sampling probes (PM2.5/PM10) as viewed directly facing the sampling plane. Blue dots represent the PN sampling probe(s) (TPN/SPN). Figure provided for illustration purposes

Test Preparation Requirements

Input Parameters

Full-friction brakes

The following parameters related to the brake – and the vehicle on which the brake under testing is mounted – shall be available to the testing facility to carry out brake emissions testing with full-friction brakes following this GTR.

Table 8.1
Required test parameters for full-friction brakes

| Serial No. | Parameters and Inputs | Short description | Symbol | Unit |
| --- | --- | --- | --- | --- |
| 1 | Vehicle make and model | The vehicle make and model where the brake under testing is mounted |  | [-] |
| 2 | Vehicle axle | The axle on the vehicle, front or rear, where the brake under testing is mounted | FA or RA | [-] |
| 3 | Vehicle test mass | The vehicle mass to be simulated on the brake dynamometer as defined in point (a) in this paragraph | Mveh | [kg] |
| 4 | Brake mounting position in the vehicle | The location of the brake under testing on the vehicle, right-hand corner or left-hand corner | RHC or LHC | [-] |
| 5 | Brake force distribution | The ratio between the braking force of each axle and the total braking force on the vehicle as described in point (b) in this paragraph | FA/RA | [%] |
| 6 | Fixture style | The support fixture of the brake assembly according to the requirements defined in paragraph 8.4.1. | L0-U or L0-P | [-] |
| 7 | Part number for the disc or drum | The code labelled by the brake manufacturer on the disc/drum |  | [-] |
| 8 | Part number for the friction material | The code labelled by the friction manufacturer on the pads/shoes |  | [-] |
| 9 | Nominal Wheel Load | The load at the brake corner under testing before accounting for vehicle road loads or any other type of losses as defined in point (c) in this paragraph | WLn | [kg] |
| 10 | Test (or applied) Wheel Load | Load at the brake corner under testing after accounting for vehicle road loads or any other type of losses as defined in point (d) in this paragraph | WLt | [kg] |
| 11 | Tyre dynamic rolling radius | Tyre radius that equates to the revolutions per distance driven as published by the tyre manufacturer for the specific tyre size | RR | [mm] |
| 12 | Brake Effective radius | The distance from the center of the brake (disc or drum) to the theoretical center of the friction material as defined in point (e) in this paragraph | reff | [mm] |
| 13 | Brake nominal inertia | Wheel load with a gyration radius that equals the tyre dynamic rolling radius which imposes the same kinetic energy on the service brake as in the actual vehicle. It is defined in point (f) in this paragraph | In | [kg·m2] |
| 14 | Brake test (or applied) inertia | Nominal brake inertia after subtracting the decelerating forces induced by vehicle road loads or any other type of losses as defined in point (g) in this paragraph | It | [kg·m2] |
| 15 | Disc/Drum maximum outer diameter | The largest diameter of the disc or drum under testing | OD | [mm] |
| 16 | Disc Mass | Mass of the disc before testing – It is used for the allocation of the brake under testing to a nominal wheel load to disc mass group as described in Paragraph 10 | DM | [kg] |
| 17 | Number of pistons per side | Number of pistons (or “pots”) on one side of the brake calliper |  | [#] |
| 18 | Piston Mean (or hydraulic) Diameter | The diameter of the piston of the brake under testing as defined in point (h) in this paragraph |  | [mm] |
| 19 | Disc calliper bolt tightening torque (if applicable) | Bolt tightening suggested torque if specified by the brake manufacturer |  | [N·m] |
| 20 | Brake calliper or brake drum efficiency (if applicable) | Efficiency to account for friction losses, piston travel, etc. if specified by the brake manufacturer. If not specified, use 100 per cent | η | [%] |
| 21 | Threshold pressure | Minimum pressure to overcome internal resistance before onset of brake torque | pthreshold | kPa |
| 22 | Lateral run out limit | The maximum lateral movement allowed for the disc/drum along its rotating axis when installed on the brake fixture | LRO | [µm] |

The following considerations shall be taken into account when calculating some of the required testing parameters provided in Table 8.1:

- Vehicle Test Mass (Mveh): It is the mass in running order (MRO) of the vehicle (kg) on which the tested brake is mounted plus:

- 37.5 kg that corresponds to an additional mass of 0.5 passengers, for M1 vehicle category;

- 25 kg plus 28 percent of the Maximum Vehicle Load (MVL), for N1 vehicle category.

- Brake Force Distribution (FA/RA) represents the ratio between the braking force of each axle and the total braking force on the vehicle. This ratio is unique for the front and rear axles. The brake force distribution is expressed as a percentage. The summation of braking force ratios for the front and rear axle does not necessarily equal 100 per cent. The brake force distribution for each vehicle is provided by the vehicle manufacturer.

- The brake force distribution per the default method on the UN Regulation No 90 for decelerations below 0.65 g shall be applied only whenever the vehicle manufacturer’s specific value is not available. This corresponds to:

- 77 per cent for the front axle and 32 per cent for the rear axle, for M1 vehicle category;

- 66 per cent for the front axle and 39 per cent for the rear axle, for N1 vehicle category.

- Nominal Wheel Load (WLn) represents the load at the brake corner under testing before accounting for vehicle road loads or any other type of losses. It is a function of the vehicle test mass and the brake force distribution and is calculated from equation 8.1. The nominal wheel load is used only to classify the tested brake into a nominal wheel load to disc mass group according to its (WLn/DM) ratio when adjusting the cooling settings as specified in Paragraph 10.

| WLn=0.5×Mveh×FARA | (Eq. 8.1) |
| --- | --- |

- Where:

- WLn	is the nominal wheel load in kg per Table 8.1;

- Mveh 	is the vehicle test mass in kg per Table 8.1;

FARA	is the brake force distribution per Table 8.1.

- Test (or applied) Wheel Load (WLt) represents the load at the brake corner under testing after accounting for vehicle road loads or any other type of losses. It is a function of the vehicle test mass and the brake force distribution and is calculated from equation 8.2. For full-friction brakes, the WLt is reduced by 13 per cent compared to the WLn to account for the road loads of the vehicle during real-world operation. The WLt is applied during the entire brake emissions test including cooling adjustment, bedding, and emissions measurement sections.

| WLt=0.87×WLn | (Eq. 8.2) |
| --- | --- |

- Brake Effective Radius (reff) is defined as the distance from the center of the brake (disc or drum) to the theoretical center of the friction material. A circle drawn with this radius would be placed at the centre of the disc-pads or drum-shoes contact surface.

For disc brakes, the brake effective radius shall be provided by the brake manufacturer. For a drum brake, the effective radius is half of the drum's inner diameter.

- Brake Nominal Inertia (In) represents the wheel load with a radius of gyration equal to the tyre dynamic rolling radius which imposes the same kinetic energy on the service brake as in the actual vehicle. It is a function of the nominal wheel load and the tyre dynamic rolling radius and is calculated from equation 8.3:

| In=WLn×RR2 | (Eq. 8.3) |
| --- | --- |

- Where:

- In 		is the brake nominal inertia in kg·m2 per Table 8.1;

- WLn 	is the nominal wheel load in kg per Table 8.1;

RR		is the tyre dynamic rolling radius in m per Table 8.1.

- Brake Test (or applied) Inertia (It) represents the brake nominal inertia after subtracting the decelerating forces induced by vehicle road loads or any other type of losses. The brake test inertia is the primary source of kinetic energy during braking. It is a function of the nominal wheel load and the tyre dynamic rolling radius and is calculated following equation 8.4. For full-friction brakes, the brake test inertia is reduced by 13 per cent compared to the brake nominal inertia to account for the road load losses of the vehicle during real-world operation. The brake test inertia applies to the entire brake emissions test including cooling adjustment, bedding, and emissions measurement sections.

| It=0.87×In | (Eq. 8.4) |
| --- | --- |

- Piston Mean (or hydraulic) Diameter (dpiston) for disc brakes represents the equivalent piston diameter of the brake under testing. If the calliper contains several (n) pistons, the testing facility shall determine the piston hydraulic diameter using the equivalent individual piston diameters with equation 8.5:

| dpiston=d12+d22+…+dn2 | (Eq. 8.5) |
| --- | --- |

Regenerative Brakes

[placeholder – reserved]

Test Setup Preparation

Full-Friction Brakes

The testing facility shall perform the following tasks before commencing a brake emissions test:

- Verify the availability of all the test documentation, brake information, control program, dynamometer capabilities, and test conditions;

- Update or upload the corresponding control program, test parameters and conditions, and brake information onto the brake inertia dynamometer control system;

- Install the brake assembly onto the test fixture and the dynamometer tailstock following the specifications described in paragraph 8.4.1. Connect with the adaptors to the main dynamometer shaft;

- Install the brake pads or brake shoes and perform a thorough brake bleed to remove air bubbles from the brake lines spanning from the master cylinder up to the brake;

- Measure the lateral run out (LRO) using a dial indicator while manually rotating the brake installed on the dynamometer. Verify that the LRO is less than 50 μm. Check for running clearance between pads/shoes and disc/drum;

- Perform a visual inspection of the brake under testing, brake fixture, thermocouple wires, and hydraulic brake lines to ensure proper routing and connections;

- Ensure all the instruments and filter media are available per the standard operating procedure;

- Perform brake static applies at brake pressures in the range of 3-30 bar to verify fluid displacement curve for bleed check and fluid leak check inside the enclosure;

- Close the brake enclosure, turn on the environmental conditioning system, and verify the operation of the cooling air system following the specifications in paragraphs 7.2.1 and 7.2.3;

- Perform acceleration events to reach different linear speeds (5 km/h, 50 km/h, and 135 km/h) and record residual torque during the acceleration to the set speed and after cruising at the target speeds for 10 seconds (at zero brake pressure). Verify that this spinning torque remains less than 20 N·m. If the spinning torque exceeds this value check again the LRO, running clearance (including thermocouples wiring), and brake bleed, in that order of diagnosis;

- Perform not more than three stops using mild test conditions for speed and deceleration to verify data collection, test parameters, brake test inertia, and overall system operation;

- When the cooling airflow for the axle and brake type under test is not known adjust to a known value used for similar brakes as described in paragraph 10.2.4. Verify that the selected cooling airflow meets the specifications defined in Paragraph 10. If not, adjust its value following the instructions in paragraph 10.2.4. until the nominal value is set;

- Verify the pre-test background emissions levels are within the acceptable limits as defined in paragraph 7.2.2.2.2 using the nominal cooling airflow;

- Verify all instruments and devices for brake emissions measurements are enabled and operational;

- If no issues arise, continue with the bedding and emissions measurement sections following the standard operating procedures;

- When the testing facility carries out the cooling adjustment section with a different friction couple than the one used for testing all steps in this paragraph except for point (l) shall apply.

Regenerative Brakes

[placeholder – reserved]

Brake Temperature Measurement

The testing facility shall use embedded thermocouples for the measurement of the brake disc or drum temperature. The following specifications apply:

- Use commercially available temperature sensors containing Nickel-chromium (Chromel) and Nickel-aluminum (Alumel) conductors (Type K thermocouples);

- Use embedded thermocouples with a measurement temperature range of (0 to 1260) °C and a maximum permissible error (tolerance) of ±2.2 °C or ±0.75 per cent;

- Use embedded thermocouples with a solid tip readily installed to embed them onto the brake components.

Additionally, the following specifications for placing the embedded thermocouples onto the brake components apply:

- Disc brakes: Locate the embedded thermocouple in the outboard plate rubbing surface – radially positioned 10 mm outwards of the centre of the friction path – and recessed (0.5 ± 0.1) mm deep into the face of the disc. On vented discs, centre the thermocouple between two fins of the disc plate. Figure 8.1 illustrates the proper installation of embedded thermocouples on brake discs;

- Drum brakes: Locate the embedded thermocouple at the centre of the friction path (0.5±0.1) mm deep in the inside surface of the brake drum. Figure 8.2 illustrates the proper installation of embedded thermocouples on brake drums (to be provided in the draft GTR);

- The installation of embedded or other types of thermocouples for measuring brake pad or shoe temperature over brake particle emissions tests in the context of this GTR is strongly discouraged.

Brake temperature shall be registered in the Time-Based file and reported as described in paragraph 13.4. The testing facility shall use the 1Hz temperature readings of the embedded thermocouples in the Time-Based file (Column I: “Brake Temperature” - Tbrake) to check the correct application of the initial temperature at the individual trips of the WLTP-Brake cycle following the specifications described in paragraph 9.2.

Figure 8.1
Schematic installation of embedded thermocouples for brake discs

Brake Positioning

Brake Assembly

The installation position of the brake assembly shall be at the centre of the enclosure at the point where planes A1 and D intersect. The proper installation position is illustrated in Figure 8.3.

Figure 8.3
Installation position of the brake assembly and the calliper

- The testing facility shall use a suitable brake fixture to mount the brake assembly by connecting the tailstock (non-rotating side) to the brake inertia dynamometer shaft (rotating side). The minimum subsystems of the dynamometer brake fixture shall include:

- Mounting components to attach the brake test fixture to the (non-rotating) tailstock;

- Structural components to transfer the braking torque and forces to the tailstock;

- Mounting components to take the brake calliper or the backing plate assembly for drum brakes;

- Rotating parts to mount the brake disc or brake drum onto;

- Rotating components to connect the shaft of the brake inertia dynamometer to the brake disc or brake drum.

The support fixture of the brake assembly shall allow the brake to freely rotate by 360° with low friction and without exhibiting vibration or oscillations during testing. The testing facility shall mount the brake system on the dynamometer using a universal style (L0-U) or post style (L0-P) brake fixture. The L0-U allows for directly attaching the brake assembly onto the dynamometer driveshaft without a wheel hub. The L0-P allows for the installation of the specific vehicle’s bearing. Figures 8.4 and 8.5 illustrate some examples of the fixture style schematics for disc and drum brakes, respectively.

Figure 8.4
Example of allowed fixture styles schematics for disc brakes

Figure 8.5
Example of allowed fixture styles schematics for drum brakes

Any variant of these fixtures (one side bearing right or left or both sides bearing) may be applied provided they use an L0 style fixture as a reference (i.e. cylindrical and symmetrical base without additional extensions or protrusions different from those needed to mount the calliper assembly). For example, Figure 8.4 illustrates three different versions of an L0-U fixture: With two side bearings, one side bearing, and a cantilevered spindle. Unique brake mounting systems for braking technologies that the L0-U or the L0-P cannot accommodate may deviate from this requirement. In such a case, the testing facility shall submit the proper documentation demonstrating the need for their use.

The testing facility shall install the brake configuration (brake disc and calliper or drum assembly) such that it always rotates in the evacuation direction when driving forward as shown in Figure 8.6. When the cooling air flows in a direction from right to left (Figure 8.6 left-hand side), the disc shall rotate in a counter-clockwise direction (CCW). When the cooling air flows in a direction from left to right (Figure 8.6 right-hand side), the disc shall rotate in a clockwise direction (CW). Alternative rotation directions are not allowed and will invalidate the test.

Figure 8.6
Schematic representation of disc rotation viewed from the wheel side (road side)

Calliper Orientation

The testing facility shall position the calliper to minimize potential interference with the incoming cooling air. Install the calliper above the disc in a 12-o’clock position as illustrated in Figure 8.6 irrespective of the mounting position of the vehicle. Other calliper orientations (i.e. vehicle’s mounting position) are not allowed and will invalidate the test.

WLTP-Brake Cycle

General Information

The testing cycle for all types of brake systems shall be the time-based WLTP-Brake cycle. The WLTP-Brake cycle demands the continuous control of the equivalent linear speed on the brake dynamometer. Figure 9.1 illustrates the time-resolved speed trace of the WLTP-Brake cycle. In summary, the WLTP-Brake cycle includes:

‒	Ten (10) individual trips (Trips #1 - #10) that represent different driving and braking conditions. The trips are separated by cooling sections. The trips’ numbers are indicated on the right-hand side Y-axis in Figure 9.1;

‒	15 826 seconds of active speed control, without including the cooling sections between the individual trips of the cycle;

‒	303 brake deceleration events. The main properties of the brake deceleration events are described in the “Brake Events Information” tab in the “WLTP-Brake Cycle“ file;

‒	192 km of total distance driven with an average speed of 43.7 km/h and a maximum speed of 132.5 km/h;

‒	192 An average brake deceleration rate of 0.97 m/s². A maximum brake deceleration rate of 2.18 m/s²;

‒	An average brake duration of 5.7 s. A maximum brake duration of 15 s.

Figure 9.1
Time-resolved vehicle speed for the WLTP-Brake Cycle

WLTP-Brake Cycle Application

Cooling Adjustment Section

The cooling air adjustment for testing different brakes shall be carried out using Trip #10 of the WLTP-Brake Cycle as described in Paragraph 10 of this GTR. Specific provisions related to the brake temperature at the beginning of Trip #10 apply to the cooling adjustment section. The testing facility shall perform the following steps:

- Set the cooling airflow to the value determined in Paragraph 10;

- Commence Trip #10 of the WLTP-Brake cycle at a brake temperature of (40±1) °C;

- Warm the brake to (40±1) °C following a sequence of brake stops 1 to 7 of Trip #10 with a subsequent cooling phase down to (40±1) °C;

- Alternatively, select one of the brake events 1 to 7 of Trip #10 and repeat it several times until the brake temperature reaches (40±1) °C;

- Run Trip #10 of the WLTP-Brake cycle without any interruption. Paragraph 9.3.1 describes the necessary actions in case of interruptions.

Failure to comply with the described brake temperature provisions shall result in an invalid cooling adjustment. In this case, the testing facility shall repeat the cooling adjustment section. The same brake parts are allowed for repeating the cooling adjustment.

Bedding Section

The correct execution of the WLTP-Brake cycle involves the execution of all ten (10) trips in succession. The bedding procedure consists of five consecutive runs of the WLTP-Brake cycle as described in Paragraph 11 of this GTR. Specific provisions related to the brake temperature at the beginning of the WLTP-Brake cycle apply to the bedding procedure. The testing facility shall carry out the following steps:

- Set the cooling airflow to the value determined in Paragraph 10;

- Commence the first run of the WLTP-Brake cycle at a brake temperature of (20 ± 5) °C;

- Do not apply cooling sections between the individual trips of a WLTP-Brake cycle during the bedding procedure;

- Commence the subsequent four WLTP-Brake cycle runs at an initial brake temperature of 40 °C. If the brake temperature at the end of the previous WLTP-Brake cycle is between 30 °C to 40 °C, commence the subsequent cycle immediately without any intervention to warm the brake;

- If the brake temperature at the end of the previous WLTP-Brake cycle is below 30 °C, discontinue the bedding section and identify discrepancies in the test execution or repeat the cooling adjustment. After fixing the issue, repeat the bedding section from the beginning;

- Run the five individual WLTP-Brake cycles consecutively without any interruption. Paragraph 9.3.2 describes the necessary actions in case of interruptions.

The minimum threshold temperature of 30 °C specified in this paragraph applies to all tested brake systems. Failure to comply with the described brake temperature provisions shall result in an invalid bedding test and the testing facility shall repeat the bedding section. A new set of brake parts shall be used in case of repeating the bedding procedure.

Emissions Measurement Section

The correct execution of the WLTP-Brake cycle involves the performance of all ten (10) trips in succession. Cooling sections are mandatory between the individual trips of the WLTP-Brake cycle during the execution of the emissions measurement. Specific provisions related to the brake temperature at the beginning of the WLTP-Brake cycle apply to the emissions measurement. The testing facility shall carry out the following steps:

- Set the cooling airflow to the value determined in Paragraph 10;

- Commence Trip #1 of the WLTP-Brake Cycle at a brake temperature of (20 ± 5) °C, without conducting any warm-up stops or snubs;

- For Trips #2-10, wait until the brake temperature cools down to 40 °C;

- For Trips #2-10, if the brake temperature at the end of the previous trip is between 30 °C to 40 °C, commence the subsequent trip immediately without any intervention to warm the brake disc;

- For Trips #2-10, if the brake temperature at the end of the previous trip is below 30 °C, discontinue the emissions test and identify discrepancies in the test execution or repeat the cooling adjustment. After fixing the issue, repeat from the beginning of the bedding section using a new set of brake parts;

- Run the WLTP-Brake cycle without any interruption. Paragraph 9.3.3 describes the necessary actions in case of interruptions.

The minimum threshold temperature of 30 °C specified in this paragraph applies to full-friction brake systems equipped on conventional M1 and N1 vehicles under the scope of this GTR. A minimum threshold temperature of 20 °C applies to brakes equipped on M1 and N1 vehicles featuring regenerative braking systems.

WLTP-Brake Cycle Interruptions

Cooling Adjustment Section

- If the test is interrupted (or the dynamometer faults) during the cooling adjustment section, the testing facility shall discontinue the test and restart the cooling adjustment procedure from the beginning. In that case, after performing a data review and a visual inspection without disturbing the brake assembly, the testing facility shall use the same brake assembly to continue the cooling adjustment. If upon inspection there are reasons to compromise the test (loose components, brake fluid leakage, incorrect mounting, excessive vibration, etc.), the testing facility shall mount a new brake assembly and repeat the procedure following the specifications described in 8.2.1.

Bedding Section

- If the test is interrupted (or the dynamometer faults) during the bedding section, the testing facility shall continue bedding from the point of interruption considering the last recorded timestamp in the Time-Based file with non-zero values for the braking parameters. The testing facility shall not conduct any warm-up stops or snubs to reach 30 °C if the actual brake temperature is lower and shall not disassemble the parts. If the brake parts are disassembled after the beginning of the bedding section, they are no longer suitable for completing bedding and the subsequent emissions measurement. In that case, the testing facility shall replace with new brake parts and repeat the bedding procedure from the beginning.

Emissions Measurement Section

- If the test is interrupted (or the dynamometer faults) during one or more cooling sections between two consecutive trips, the testing facility shall continue the test without disassembling the parts or conducting any warm-up stops or snubs provided that the interruption does not exceed 1h. In that case, the testing facility shall deactivate the particle sampling pumps and the cooling air supply at the time of the interruption (auto-controls are strongly recommended). The testing facility shall resume their function once the test is commenced again and after the cooling flow is stabilized following the specifications described in paragraph 7.2.3. If the test is interrupted during Trips #1 through #10, the testing facility shall discontinue the emissions measurement section. The testing facility shall replace used PM2.5 and PM10 filters with new and restart the emissions measurement from Trip #1 at an initial brake temperature of (20 ± 5) °C without disassembling the parts.

WLTP-Brake Cycle Quality Checks

The following quality checks shall be carried out to verify the correct execution of the WLTP-Brake cycle. A valid emissions test shall comply with all the criteria described below.

Speed Violations

- The speed violations quality check is necessary to ensure that the brake dynamometer has correctly executed the WLTP-Brake cycle speed trace. A speed violation during any trip occurs whenever the actual speed of the dynamometer exceeds the speed trace tolerances, compared to the prescribed (nominal) speed. Figure 9.2 depicts the upper and lower speed tolerance limits.

Figure 9.2
Tolerance limits for speed violations during WLTP-Brake Cycle [UN GTR No 15]

- ‒	Upper-speed tolerance: 2.0 km/h higher than the nominal linear speed trace within ±1.0 second of the given point in time;

- ‒	Lower-speed tolerance: 2.0 km/h lower than the nominal linear speed trace within ±1.0 second of the given point in time.

- During the cooling adjustment section, the number of speed violations shall not exceed 158 for each complete Trip #10 of the WLTP-Brake cycle. This corresponds to 3 per cent of the Trip #10 duration;

- During the bedding section, the number of speed violations shall not exceed 475 for each complete WLTP-Brake cycle. This corresponds to 3 per cent of the WLTP-Brake cycle duration and applies to all five repetitions of the WLTP-Brake cycle;

- During the emissions measurement section, the number of speed violations shall not exceed 475 for each complete WLTP-Brake cycle. This corresponds to 3 per cent of the WLTP-Brake cycle duration. Cooling sections shall not be included in the calculation;

- The testing facility shall use data from the Time-Based file of the brake emissions test to compute the number of speed violations. The computation of speed violations shall include all types of events (dwell, acceleration, cruising, and deceleration). The parameters “Linear Speed Actual” (Column C) and “Linear Speed Nominal” (Column B) shall be compared at 1Hz for the calculation of the speed violations number at each section;

- Failure to run Trip #10 of the WLTP-Brake cycle (cooling adjustment section) or the entire WLTP-Brake cycle (bedding and emissions measurement section) within the speed tolerances defined in this paragraph shall result in an invalid brake emissions test.

Number of Deceleration Events

- This quality check relates to the number of the executed brake events during the WLTP-Brake cycle. It is necessary to ensure that all 303 brake events of the WLTP-Brake cycle were applied during the emissions measurement section. A violation of this criterion occurs whenever the actual number of applied events is not equal to the nominal value.

- The testing facility shall verify the number of applied brake events using the submitted Event-Based file of the brake emissions test. More specifically, the parameters “Stop Duration” (Column D) and “Deceleration Rate - Distance Averaged” (Column M) shall be cross-checked and verified that both include 303 numerical and non-zero values that correspond to the respective 303 brake events of the WLTP-Brake cycle.

- This quality check applies only to the emissions measurement section. Failure to perform the 303 braking events of the WLTP-Brake cycle during the emissions measurement section shall result in an invalid test.

Kinetic Energy Dissipation

This quality check is necessary to ensure the application of the correct amount of friction work (Wf) during the execution of the WLTP-Brake cycle. It is also an additional quality check that other input parameters (e.g. brake test inertia) have been calculated and applied correctly.

- A friction work violation occurs whenever the sum of the calculated friction work of all brake events throughout Trip #10 of the WLTP-Brake cycle (cooling adjustment section) and the entire WLTP-Brake cycle (bedding and emissions measurement section) exceeds the defined tolerances, compared to the prescribed (nominal) friction work values:

- ‒	Trip #10 of the WLTP-Brake cycle upper-friction work tolerance: 278 J/kg higher than the nominal friction work value of 5555 J/kg. Thus, the upper-friction work tolerance is 5833 J/kg;

- ‒	Trip #10 of the WLTP-Brake cycle lower-friction work tolerance: 278 J/kg lower than the nominal friction work value of 5555 J/kg. Thus, the lower-friction work tolerance is 5277 J/kg;

- ‒	WLTP-Brake cycle upper-friction work tolerance: 799 J/kg higher than the nominal friction work value of 15983 J/kg. Thus, the upper-friction work tolerance is 16782 J/kg;

- ‒	WLTP-Brake cycle lower-friction work tolerance: 799 J/kg lower than the nominal friction work value of 15983 J/kg. Thus, the lower-friction work tolerance is 15184 J/kg.

- During the cooling adjustment section, the calculated friction work over Trip #10 shall be between 5277 J/kg and 5833 J/kg. This corresponds to ±5 per cent of the nominal value;

- During the bedding section, the calculated friction work over the WLTP-Brake cycle shall be between 15184 J/kg and 16782 J/kg. This corresponds to ±5 per cent of the nominal value and applies to all five repetitions of the WLTP-Brake cycle;

- During the emissions measurement section, the calculated friction work over the WLTP-Brake cycle shall be between 15184 J/kg and 16782 J/kg. This corresponds to ±5 per cent of the nominal value. Cooling sections shall not be included in the calculation;

- The testing facility shall calculate the friction work by applying the integrals for torque and angular speed over time for each braking event using the submitted Event-Based file of the brake emissions test. The parameters “Stop Duration” (Column D), “Rotational Speed – Time Averaged” (Column K), and “Brake Torque – Time Averaged” (Column O) shall be used to calculate the actual friction work of each braking event. The calculation requires also the use of the test (or applied) wheel load (WLt) and follows the equation 9.1:

| Wf=(2×π60)×f×τbrake×tbrakeWLt | (Eq. 9.1) |
| --- | --- |

Where:

Wf			is the friction work in J/kg;

f			is the rotational speed per Table 13.1;

τbrake		is the brake torque per Table 13.1;

tbrake		is the stop duration per Table 13.1;

WLt		is the test (or applied) wheel load per Table 8.1.

- Equation 9.1 provides the friction work for each one of the 114 and 303 brake events of Trip #10 and the WLTP-Brake cycle, respectively. The testing facility shall calculate the total friction work by summing the calculated individual brake events’ friction work. The sum friction work shall be compared to the prescribed (nominal) friction work value as described;

- Failure to perform Trip #10 of the WLTP-Brake cycle (cooling adjustment section) or the entire WLTP-Brake cycle (bedding and emissions measurement section) with a total friction work within the tolerances defined in this paragraph shall result in an invalid brake emissions test. The kinetic energy dissipation quality check applies only to full friction brakes equipped on conventional M1 and N1 vehicles and not to brakes equipped on vehicles with regenerative braking capabilities.

Cooling Airflow Adjustment

Different test systems can embody different combinations of brake enclosure design and size, airflow or airspeed levels, and duct system layout and geometry. This paragraph establishes the proper methodology to adjust the airstream speed to provide comparable brake thermal regimes across the testing facilities.

Method Description

Definition of Brake Groups and Verification Parameters

To determine the appropriate cooling airflow for a given brake, the testing facility shall first classify the tested brake into a nominal wheel load (WLn) to disc or drum mass (DM) group according to its (WLn/DM) ratio.

The WLn/DM ratio is calculated by dividing the WLn (kg) with the pre-test disc or drum mass (kg). The testing facility shall determine the WLn following paragraph 8.1.1 (c). Four different WLn/DM groups are defined based on the WLn/DM ratio:

‒	Group 1: WLn/DM ≤ 45;

‒	Group 2: 45 < WLn/DM ≤ 65;

‒	Group 3: 65 < WLn/DM ≤ 85;

‒	Group 4: WLn/DM > 85.

For the different brake emissions test sections (cooling adjustment, bedding, and emissions measurements) the testing facility shall use the test wheel load (WLt) which is calculated following paragraph 8.1.1 (d).

For each WLn/DM group, the target values and allowed tolerances for three check parameters have been defined. The testing facility shall use the following parameters as a reference against which the cooling adjustment test results shall be compared:

- Average brake temperature over Trip #10 of the WLTP-Brake Cycle (Tavg);

- Average initial brake temperature of six selected brake events from Trip #10 of the WLTP-Brake Cycle (IBT);

- Average final brake temperature of six selected brake events from Trip #10 of the WLTP-Brake Cycle (FBT);

- The brake events referred to (b) and (c) of this paragraph are #46, #101, #102, #103, #104, and #106 and their details are specified in Table 10.1. When the entire WLTP-Brake cycle is considered, the corresponding events’ sequence numbers are #235, #290, #291, #292, #293, and #295.

Table 10.1
Specific brake events from Trip #10 of the WLTP-Brake cycle

| Parameter | Unit | Deceleration event |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | #46 | #101 | #102 | #103 | #104 | #106 |
| Start time | s | 2088 | 4438 | 4459 | 4494 | 4522 | 4903 |
| End time | s | 2092 | 4447 | 4467 | 4503 | 4529 | 4918 |
| Brake duration | s | 4.0 | 9.0 | 8.0 | 9.0 | 7.0 | 15.0 |
| Initial speed | km/h | 97.4 | 112.0 | 68.2 | 80.9 | 73.4 | 132.5 |
| Final speed | km/h | 82.7 | 56.1 | 12.0 | 35.3 | 39.3 | 34.0 |

Verification of Parameters and Tolerances for Brake Temperature

The target values and the corresponding tolerances for the three check parameters are given in Table 10.2.

Table 10.2
Default temperature metrics and tolerances for brakes during Trip #10 of the WLTP-Brake Cycle

| Group | Tavg [A1] | IBT [A2] ± Tolerance | FBT [A3] ± Tolerance |
| --- | --- | --- | --- |
| WLn/DM ≤ 45 | ≥ 50 °C | 60 ± 20 °C | 90 ± 30 °C |
| 45 WLn/DM ≤ 65 | ≥ 55 °C | 70 ± 20 °C | 110 ± 30 °C |
| 65 WLn/DM ≤ 85 | ≥ 60 °C | 80 ± 20 °C | 125 ± 30 °C |
| WLn/DM > 85 | ≥ 65 °C | 90 ± 20 °C | 145 ± 30 °C |

- The target values and the corresponding tolerances for the three check parameters apply to all types of front brake systems mounted in M1 and N1 vehicle categories;

- For rear brake systems of M1 and N1 vehicle categories, the nominal (or set) cooling airflow defined for the corresponding front brake application (i.e. same vehicle data) shall be applied;

- For rear drum brakes, the cooling airflow defined for the corresponding front brake application (i.e. same vehicle data) shall be applied.

Computation of Verification Parameters and Acceptance Criteria

- Once the brake is classified to its WLn/DM Group per paragraph 10.1.1, the testing facility shall run Trip #10 of the WLTP-Brake cycle with new brake parts to obtain the values of the check parameters to populate the cells in Table 10.3. The test shall be carried out according to paragraph 10.1.4. The testing facility shall apply the WLt as defined in paragraph 8.1.1 (d) for the allocation of the brake to a WLn/DM group. The measured values for the check parameters shall be calculated using the produced test report files as follows:

- Average brake temperature over Trip #10 of the WLTP-Brake Cycle (Tavg):

- The target value (A1) depends on the WLn/DM Group and is defined in Table 10.2;

- The measured value (B1) is calculated from the parameter “Brake Temperature” recorded in the Time-Based File of the test (Column I);

- B1 equals the average of all brake temperature entries corresponding to the entire duration of Trip #10 (5272 s).

- Average initial brake temperature of selected brake events from Trip #10 of the WLTP-Brake Cycle (IBT):

- The target value (A2) and tolerances depend on the WLn/DM Group and are defined in Table 10.2;

- The measured value (B2) is calculated from the Event-Based File of the test. The testing facility shall use the parameter “Initial Brake Temperature” (Column R) for the defined target brake events;

- B2 equals the average temperature value of the individual IBT values recorded for each of the six selected brake events described in Table 10.1. The testing facility shall calculate B2 as shown in Table 10.3 (B2 = AVERAGE (Y1:Y6)).

- Average final brake temperature of selected brake events from Trip #10 of the WLTP-Brake Cycle (FBT):

- The target value (A3) and tolerances depend on the WLn/DM Group and are defined in Table 10.2;

- The measured value (B3) is calculated from the Event-Based File of the test. The testing facility shall use the parameter “Final Brake Temperature” (Column S) for the defined target brake events;

- B3 equals the average temperature value of the individual FBT values recorded for each of the six selected brake events described in Table 10.1. The testing facility shall calculate B3 as shown in Table 10.3 (B3 = AVERAGE (Z1:Z6)).

- After the execution of the cooling adjustment test with the selected air flow, the testing facility shall compare the recorded temperature values of the check parameters to the corresponding target values defined in Table 10.2. The difference between target and test results for the check temperature parameters shall be calculated as follows:

- Difference (C1) in average brake temperatures B1 and A1 as defined above (B1-A1) and shown in Table 10.3;

- Absolute difference (C2) in average IBT of the selected events B2 and A2 as defined above (|A2-B2|) and shown in Table 10.3;

- Absolute difference (C3) in average FBT of the selected events B3 and A3 as defined above (|A3-B3|) and shown in Table 10.3.

The testing facility shall compare the obtained results with the acceptance criteria as defined in Table 10.3.

Table 10.3
Calculation of brake temperature metrics [°C] and acceptance criteria during Trip #10

| Trip #10 Event | Metric | Target Temperature | Cooling Adjustment Test Temperature | Difference | Acceptance Criteria |
| --- | --- | --- | --- | --- | --- |
| ‒ | TAVG | A1 | B1 | C1=B1 ‒ A1 | C1 ≥ 0 °C |
| ‒ | Average 5 % IBT | A2 | B2 = Average (Y1:Y6) | C2=¦A2 ‒ B2¦ | C2 ≤ 20 °C |
| #46 |  |  | Y1 | N/A | N/A |
| #101 |  |  | Y2 |  |  |
| #102 |  |  | Y3 |  |  |
| #103 |  |  | Y4 |  |  |
| #104 |  |  | Y5 |  |  |
| #106 |  |  | Y6 |  |  |
| ‒ | Average 5 % FBT | A3 | B3 = Average (Z1:Z6) | C3=¦A3-B3¦ | C3 ≤ 30 °C |
| #46 |  |  | Z1 | N/A | N/A |
| #101 |  |  | Z2 |  |  |
| #102 |  |  | Z3 |  |  |
| #103 |  |  | Z4 |  |  |
| #104 |  |  | Z5 |  |  |
| #106 |  |  | Z6 |  |  |

- All three criteria shall be fulfilled for successful completion of the cooling airflow adjustment section;

- In case the cooling adjustment test does not meet all metrics from Table 10.2, the testing facility shall repeat the procedure adjusting the cooling airflow accordingly;

- If there is no suitable cooling airflow meeting all metrics from Table 10.2, the testing facility shall select the cooling airflow within the acceptable criteria for at least two parameters, always including the average Trip #10 temperature;

- In case of (c), the testing facility shall submit the reporting files for the non-successful cooling adjustment tests. In the case of brakes with calculated temperatures lower than the lower threshold values of the target temperatures, the testing facility shall demonstrate that the minimum operational airflow of the setup was applied and full compliance with the target parameters was not possible. In the case of brakes with calculated temperatures higher than the higher threshold values of the target temperatures, the testing facility shall demonstrate that the maximum operational airflow of the setup was applied and full compliance with the target parameters was not possible.

Brake Dynamometer Testing to Adjust the Cooling Airflow

The testing facility shall carry out the following steps to adjust the cooling airflow when testing a brake for the first time on a given dynamometer.

- Follow the test setup preparation specifications described in paragraph 8.2.1;

- Adjust the cooling airflow to a known value used for similar brakes. In the absence of a useful reference, use an airflow of 50 per cent of the maximum test setup capacity to start the test;

- Conduct one run of Trip #10 of the WLTP-Brake Cycle starting at a brake temperature of 40 °C. Warm up the brake to 40 °C following the specifications described in paragraph 9.2.1;

- Alternatively, program a series of tests at different cooling airflow levels starting each test at an initial temperature of 40 °C;

- Perform the calculations using paragraph 10.1.3 and assess the results and deviations for the target parameters;

- If the test run meets all the metrics from Table 10.2, finish the process and prepare the test report following the specifications described in Paragraph 13;

- In the case of front brakes, proceed with the subsequent sections of the brake emissions test ensuring the application of the same dynamometer settings as in the cooling adjustment procedure. The same set of brakes shall be used for brake emissions testing;

- In the case of rear brakes, proceed with the subsequent sections of the brake emissions test ensuring the application of the appropriate dynamometer settings and the same cooling settings as in the cooling adjustment procedure of the corresponding front brake;

- If the test run does not meet all the metrics from Table 10.2, use engineering judgement to determine a new cooling airflow level and repeat the process from step (a). The same set of brakes can be used for repeating the cooling airflow adjustment section.

Bedding Section

The bedding procedure is necessary to appropriately precondition the brake couple and stabilize its emission behavior before performing emissions measurement. The bedding procedure shall be carried out with the same brake parts used during the cooling adjustment section or with completely new brake parts.

Front Brakes

The testing facility shall perform the bedding procedure of all types of brake materials and couples equipped at the front axle of all M1 and N1 vehicles following the specifications described below:

- Set the cooling airflow according to the adjustment of the cooling settings for the brake under testing as specified in paragraph 10.1.4;

- Define all relevant testing parameters and dynamometer settings (testing wheel load, brake test inertia, etc.) same as in the cooling adjustment and emissions measurement sections;

- Apply five repetitions of the WLTP-Brake cycle for complete bedding of the front brake under testing;

- The five WLTP-Brake cycles shall run consecutively without any interruption. If the test is interrupted during the bedding section, the testing facility shall follow the instructions defined in paragraph 9.3.2;

- Run each repetition of the WLTP-Brake cycle without the application of cooling sections between the individual trips of the WLTP-Brake cycle. Cooling sections shall apply only between the five repetitions of the WLTP-Brake cycle (i.e. between Trip #10 of a given WLTP-Brake cycle and Trip #1 of the following WLTP-Brake cycle);

Commence the first WLTP-Brake cycle of the bedding section at a brake temperature of (20 ± 5) °C. Commence the subsequent four repetitions of the WLTP-Brake cycle at a brake temperature of 40 °C. The temperature provisions described in paragraph 9.2.2 apply to the bedding procedure of front brakes;

- Perform the bedding section on the same dynamometer as for the emissions measurement section. Do not disassemble the brake parts between the two sections of the test to avoid modifying the contact points. If the brake parts are disassembled after the beginning of the bedding procedure, they are no longer suitable for completing bedding and emissions measurements. In that case, the testing facility shall replace with new brake parts and repeat the bedding procedure from the beginning.

Failure to comply with any of the provisions described in this paragraph shall result in an invalid bedding procedure and; therefore, no subsequent emissions measurement can be performed. In this case, the testing facility shall perform the bedding procedure from the beginning using new brake parts.

Rear Brakes

The testing facility shall perform the bedding procedure of all types of brake materials and couples equipped at the rear axle of all M1 and N1 vehicles following the specifications described below:

- Use the cooling airflow according to the adjustment of the cooling settings for the corresponding front brake as specified in paragraph 10.1.2;

- Define all relevant testing parameters and dynamometer settings (testing wheel load, brake test inertia, etc.) same as in the emissions measurement section;

- Apply five repetitions of the WLTP-Brake cycle for complete bedding of the rear brake under testing;

- The five WLTP-Brake cycles shall run consecutively without any interruption. If the test is interrupted during the bedding section, the testing facility shall follow the instructions defined in paragraph 9.3.2;

- Run each repetition of the WLTP-Brake cycle without the application of cooling sections between the individual trips of the WLTP-Brake cycle. Cooling sections shall apply only between the five repetitions of the WLTP-Brake cycle (i.e. between Trip #10 of a given WLTP-Brake cycle and Trip #1 of the following WLTP-Brake cycle);

- Commence the first WLTP-Brake cycle of the bedding section at a brake temperature of (20 ± 5) °C. Commence the subsequent four repetitions of the WLTP-Brake cycle at a brake temperature of 40 °C. The temperature provisions described in paragraph 9.2.2 apply to the bedding procedure of rear brakes;

- Perform the bedding section on the same dynamometer as for the emissions measurement section. Do not disassemble the brake parts between the two sections of the test to avoid modifying the contact points. If the brake parts are disassembled after the beginning of the bedding procedure, they are no longer suitable for completing bedding and emissions measurements. In that case, the testing facility shall replace with new brake parts and repeat the bedding procedure from the beginning.

Failure to comply with any of the provisions described in this paragraph shall result in an invalid bedding procedure and, therefore, no subsequent emissions measurement can be performed. In this case, the testing facility shall perform the bedding procedure from the beginning using new brake parts.

Particle Emissions Measurements Section

Measurement of Particle Mass (PM) Concentration

This paragraph describes the specifications for the PM emissions measurement during brake testing. The PM sampling system enables the quantification of the particle mass generated by the friction couple during the test. The PM emissions and the parameters from the test provide the emissions factors for the brake under test in mg/km. The test system shall measure brake PM10 and PM2.5 emissions gravimetrically using separate sampling systems for each cut-off diameter (2.5 μm and 10 μm). Each PM sampling system shall consist of the following elements:

- Two PM sampling probes are located in the tunnel. The specifications for the design of the PM probes are described in paragraph 12.1.1.2;

- Appropriate sampling nozzles are installed at the tip of the PM sampling probes. The specifications for the design of the nozzles are described in paragraph 12.1.1.3;

- A cyclone is applied as a PM separation device. The specifications for the cyclone are described in paragraph 12.1.2.1;

- A particle sampling line transfers the particles from the PM separation device to the filter holder. The specifications for the design of the sampling line are described in paragraph 12.1.2.2;

- A filter holder collects the particles. The specifications for the filter holder are described in paragraph 12.1.3.1;

- One or more pumps with means to control the flow rate in real-time and the corresponding sensors. The specifications for the sampling flow are described in paragraph 12.1.2.3.

PM10 and PM2.5 shall be measured with separate units. Figure 12.1 illustrates an indicative setup of the PM sampling unit. The positioning and dimensions of the different elements are indicative and are provided for illustration purposes; therefore, exact conformance with the figure is not required.

Figure 12.1
Indicative setup of the PM sampling unit

Particle Extraction

Sampling Plane

The design of the sampling plane shall follow the specifications described in paragraph 7.6. The following additional specifications apply to the sampling plane for the installation of the PM sampling probes:

- Apply two sampling probes with the corresponding sampling nozzles for the PM measurements, one for PM2.5 and one for PM10. Red dots in Figure 7.7 indicate the PM sampling probes for both the three and four sampling probes layout;

- When using the 3-probes setup place the two PM sampling probes (PM2.5 and PM10) at the same horizontal plane to the lower part of the tunnel as shown in Figure 7.7 (left-hand side);

- When using the 4-probes setup place the two PM sampling probes (PM2.5 and PM10) at the same horizontal plane to the lower part of the tunnel as shown in Figure 7.7 (right-hand side);

- Do not use flow splitters for PM measurements anywhere in the sampling and measurement system.

PM Sampling Probes

Appropriate sampling probes shall be used to transport the particles from the tunnel to the separation device. The testing facility shall select sampling probes meeting the following design requirements:

- Use probes appropriately designed to minimize particle losses from the nozzle tip to the separation device;

- Use probes made of stainless steel with an electropolished finish (or equivalent) to attain an ultra-clean and ultra-fine surface;

- Select probes with a constant inner diameter (dp) of at least 10 mm and a maximum of 18 mm ensuring a laminar flow (10 mm ≤ dp ≤ 18 mm);

- Design the sampling probe aiming for the shortest possible length to minimize losses and possible tubing pollution. The overall length of the probe from the sampling nozzle tip to the inlet of the PM separation device shall not exceed 1 m;

- A maximum of one bend of 90° may be applied to the probe provided that the specifications for the design of the bend described in point (f) of this paragraph are met;

- If a bend is applied to the probe, the bending radius rp shall be at least four times the probe's inner diameter (4∙dp);

- Inspect and clean the inner walls of the sampling probes frequently following the specifications of their manufacturer regarding the cleaning frequency and means. If no such specifications are provided clean the probes at least once every six months.

PM Sampling Nozzles

Appropriate nozzles to ensure isokinetic sampling for PM10 and PM2.5 as defined in paragraph 12.1.2.4 shall be used. The testing facility shall select sampling nozzles meeting the following requirements:

- Use nozzles compatible with all sample probes;

- Use nozzles made of stainless steel with an electropolished finish (or equivalent) to attain an ultra-clean and ultra-fine surface;

- Use the appropriate nozzles to achieve an isokinetic ratio (IR) as close as possible to 1.0 following the specifications described in paragraph 12.1.2.4. The average isokinetic ratio in a brake emissions test shall be between 0.90-1.15 (0.90 ≤ IR ≤ 1.15);

- Select the nozzle size depending on the applied sampling flow. Do not use nozzles with an inner diameter (dn) lower than 4 mm;

- The nozzle shall have a constant internal diameter along a length equal to at least one internal diameter or at least 10 mm from the nozzle tip, whichever is greater;

- Use nozzles with a thin wall at the tip to minimize distortion of flow. These shall have an outer to inner diameter ratio lower than 1.1 at the nozzle tip;

- Any variation in bore diameter of the nozzles shall be tapered with a conical angle less than 30°;

- Restrict the aspiration angle to ±15°;

- Clean the nozzles before every brake emissions test following the specifications defined by their manufacturer regarding the cleaning means.

PM sampling

PM Separation Device

Single cyclonic separators followed by gravimetrical filter holders shall be used for the collection of the PM10 and PM2.5 samples. The testing facility shall select cyclonic separators meeting the following specifications:

- Use commercially available cyclonic separators with cut-off sizes of 10 μm and 2.5 μm for the collection of the PM10 and PM2.5 samples, respectively;

- The cyclones shall fulfill the specifications for the separation efficiency described in Tables 12.1 and 12.2;

- Place the cyclonic separators at the end of the sampling probe. Connect the cyclonic separator directly at the end of the sampling probe using appropriate fittings made of conductive stainless steel. Do not use any kind of sampling tubes between the probe and the cyclonic separator;

- Inspect and clean the inner walls of the cyclones frequently following the specifications of the instrument manufacturer regarding the cleaning frequency and means.

Table 12.1 
Separation efficiency specifications of PM10 cyclones

| PM10 | 4 μm | 8 μm | 12.5 μm | 20 μm |
| --- | --- | --- | --- | --- |
| Separation Efficiency | < 20 % | < 50 % | > 60 % | > 90 % |

Table 12.2 
Separation efficiency specifications of PM2.5 cyclones

| PM10 | 1.5 μm | 2 μm | 3 μm | 4 μm |
| --- | --- | --- | --- | --- |
| Separation Efficiency | < 20 % | < 50 % | > 60 % | > 90 % |

PM Sampling Line

The testing facility shall ensure that the design of the sampling line that transfers the particles from the cyclonic separator to the filter holder meets the specifications described below:

- Use sampling lines appropriately designed to minimize particle transport losses between the cyclonic separator and the filter holder;

- Use sampling lines made of conductive stainless steel applying the appropriate fittings. Alternatively, flexible antistatic PTFE sampling lines may be used;

- Select sampling lines with a constant inner diameter (ds) of at least 10 mm and a maximum of 20 mm (10 mm ≤ ds ≤ 20 mm). A sampling line with a constant inner diameter close to 15 mm is highly recommended;

- The overall length of the sampling line from the cyclonic separator to the tip of the filter holder shall be as short as possible. In any case, do not use sampling lines longer than the maximum allowed length of 1 m;

- Design the sampling train outside the tunnel (the part of the sampling train that includes the cyclonic separator and the PM sampling line) in a way that no condensation of water can occur. The temperature inside the sample train shall always remain above 15 °C;

- A bend may be applied to the sampling line provided that the specifications for the design of the bend described in point (g) of this paragraph are met;

- If a bend is applied to the sampling line, the bending radius rp shall be at least twenty-five times the sampling line diameter (25∙ds).

PM Sampling Flow

The testing facility shall apply the following provisions for the regulation and measurement of the sampling volumetric flow:

- The method of measuring the volumetric flow of the sampling system (QPM2.5 and QPM10) shall have a maximum permissible error of ±2.5 per cent of the reading or ±1.5 per cent of the full-scale, whichever is the smallest, under all operating conditions;

- Use a volumetric flow measurement device calibrated to report flow at both operating and standard conditions (273.15 K and 101.325 kPa). The temperature sensor shall have an accuracy of ±1.0 °C. The pressure measurements shall have precision and accuracy of ±1.0 kPa;

- The set (nominal) value for the sampling volumetric flow (QPM2.5-set and QPM10-set) shall be the same and constant for each parameter during the emissions measurement section of a specific brake system;

- The average sampling volumetric flow shall be within ±2 per cent of the set value for the given brake emissions test. Use a device with a flow control feature (e.g. critical orifice, pressure regulator, feedback controller, or other) to ensure a stable flow through the filter medium;

- Calculate the deviation of the average actual sampling volumetric flow from the set value using the data of the given parameters in the Time-Based file (PM2.5: Column Q “PM2.5 Sampling Airflow Set” and Column R “PM2.5 Sampling Airflow Actual” and PM10: Column T “PM10 Sampling Airflow Set” and Column U “PM10 Sampling Airflow Actual”). Report the calculated values as specified in Paragraph 13;

- Set the sampling volumetric flow such that the isokinetic ratio is as close as possible to 1.0. The average isokinetic ratio during the emissions measurement section of a specific brake system shall be between 0.90-1.15 (paragraph 12.1.2.4). Calculate and report the average isokinetic ratio for both PM2.5 and PM10 following the procedure described in paragraph 12.1.2.4;

- Check for possible leaks by sealing the nozzle and starting the suction device. The flow rate shall be at most 2 per cent of the normal flow rate at the maximum vacuum reached during sampling. Perform the leak check upon the system installation and after every maintenance or upgrade.  The leak check shall be carried out at least once every year;

- In case the sampling volumetric flow and/or the isokinetic requirements set out in this paragraph are not met, the test is invalid;

- The PM sampling device shall operate continuously during the brake emissions measurement section. This includes also the cooling sections between the individual trips of the WLTP-Brake cycle where the PM sampling flow shall not be paused or bypass the main sampling line.

Isokinetic Ratio

Sampling is defined as isokinetic when the airspeed in the tunnel and the sampling nozzle are equal. The airspeeds are calculated from the airflow values in the tunnel and the nozzle taking into account their inner diameters (di and dn, respectively). Equations 12.1 and 12.2 apply for the calculation of the airspeeds in the tunnel and the sampling nozzle:

| U=4×Q1000×π×di2 | (Eq. 12.1) |
| --- | --- |
| US=4×QS1000×π×dn2 | (Eq. 12.2) |

Where:

U	is the average airspeed in the tunnel in km/h per Table 13.2;

US	is the average speed of the sampling air entering the nozzle in km/h;

Q	is the average airflow in the tunnel in m³/h per Table 13.2;

QS	is the average airflow in the sampling nozzle in m³/h;

dn	is the inner diameter at the nozzle tip in m;

di	is the sampling tunnel inner diameter in m per Table 7.1.

The isokinetic ratio is defined as the ratio of the airspeed in the sampling nozzle to the airspeed in the tunnel. Equation 12.3 provides the means to calculate the isokinetic ratio by combining equations 12.1 and 12.2. The airflow values in the sampling tunnel and the nozzle shall refer to the same temperature and pressure conditions; therefore, normalized values shall be used:

| IR=NQSdn2NQdi2 | (Eq. 12.3) |
| --- | --- |

Where:

IR	is the isokinetic ratio;

NQS	is the average normalized airflow in the sampling nozzle in Nm³/h;

NQ	is the average normalized airflow in the sampling tunnel in Nm³/h per Table 13.2;

dn	is the inner diameter at the nozzle tip in m;

di	is the sampling tunnel inner diameter in m per Table 7.1.

Converting the sampling flow units from [Nm³/h] to [Nl/min] and the inner diameter units from [m] to [mm] to reflect conventional units, equation 12.3 becomes equation 12.4.

| IR=0.06×NQSdn2NQdi2 | (Eq. 12.4) |
| --- | --- |

Where:

NQS	is the average normalized airflow in the sampling nozzle in Nl/min;

NQ	is the average normalized airflow in the sampling tunnel in Nm³/h per Table 13.2;

dn	is the inner diameter at the nozzle tip in mm;

di	is the sampling tunnel inner diameter in mm per Table 7.1.

- Use equation 12.4 to calculate the average isokinetic ratio at the emissions measurement section of a test for both PM2.5 and PM10;

- Use the corresponding values for the isokinetic nozzle inner diameters for PM2.5 (dn-PM2.5) and PM10 (dn-PM2.5)  sampling;

- Use the average normalized tunnel flow (Column L “Cooling Airflow Actual Normalized”) and the average actual normalized sample flows (Column S “PM2.5 Sampling Airflow Actual Normalized” and Column V “PM10 Sampling Airflow Actual Normalized”) in the Time-Based file;

- Report the calculated values as specified in paragraph 13.4.

Sampling Media

Filter Holder

The particle sample shall be collected on a 47 mm single filter per test mounted within a dedicated holder. The filter holder shall be located as close as possible to the cyclonic separator’s exit. The testing facility shall follow the specifications described below for the filter holder assembly:

- Select a filter holder made of inert, non-corroding material such as stainless steel or anodized aluminium;

- Use a filter holder suitable for insertion of circular filters, such that the diameter of the exposed area through which the sampled air passes is between 34 mm and 44 mm;

- Use a filter holder that provides an even flow distribution across the filter stain area;

- Design the filter holder arrangement in a way that no condensation of water can occur. The temperature at the filter shall follow the specification for the entire sample path and shall always remain above 15 °C during brake emissions testing.

Sampling Filters

Fluorocarbon-coated glass fibre filters or fluorocarbon membrane filters shall be used for the brake PM10 and PM2.5 measurement. All filter types shall have a 0.3 μm DOP (Dioctyl phthalate) or PAO (poly-alpha-olefin) CS 68649-12-7 or CS 68037-01-4 collection efficiency of at least 99 per cent at a gas filter face velocity of 5.33 cm/s measured according to one of the following standards:

- U.S.A. Department of Defense Test Method Standard, MIL-STD-282 method 102.8: DOP-Smoke Penetration of Aerosol-Filter Element;

- U.S.A. Department of Defense Test Method Standard, MIL-STD-282 method 502.1.1: DOP-Smoke Penetration of Gas-Mask Canisters;

- Institute of Environmental Sciences and Technology, IEST-RP-CC021: Testing HEPA and ULPA Filter Media.

The efficiency requirements for the sampling media described in this paragraph shall be certified by the filter supplier.

Weighing Procedure

Only the filter shall be weighed and not any other part of the measurement equipment. The sampling facility shall ensure that the different steps of the weighing procedure are carried out according to the following requirements:

- Pre-sampling conditioning and weighing – Condition/stabilize the filters at (22 ± 2) °C and (45 ± 8) per cent RH for a minimum of 2 hours before sampling. Weigh the filter at the end of the stabilization period and register its weight in all relevant test sheets. No deviation from the conditions specified in this paragraph is permitted during the weighing operation. Store the filter in a closed petri dish or sealed filter holder until testing. Use the filter within 12 hours of its removal from the weighing chamber (or room);

- Post-sampling conditioning and weighing – Take the filters to the conditioning room within 8 hours after testing is completed. Condition/stabilize the filters at (22 ± 2) °C and (45 ± 8) per cent RH for a minimum of 2 hours. Weigh the filter at the end of the stabilization period and register its weight in all relevant test sheets. No deviation from the conditions specified in this paragraph is permitted during the weighing operation. Store the filter in a closed petri dish or sealed filter holder;

- Weighing room – The room environment shall be free of any ambient contaminants (such as dust, aerosol, or semi-volatile material) that could contaminate the particle filters. Regulate the weighing room environmental conditions at (22 ± 2) °C and (45 ± 8) per cent RH. Make sure that the air flow for the air exchange does not influence the balance stability;

- Static electricity effects – Nullify the effects of static electricity by grounding the balance through placement upon an antistatic mat and neutralizing the particle sampling filters before weighing using a polonium neutralizer or a device of similar effect. Alternatively, nullify static effects through equalization of static charge;

- Buoyancy correction – Correct the sample and reference filter weights for their buoyancy in air. The buoyancy correction is a function of sampling filter density, air density, and the density of the balance calibration weight, and does not account for the buoyancy of the particulate matter itself.

- Use the following values for the density of the filter material (pf)  when it is not known: (a) Fluorocarbon coated glass fibre filter: 2300 kg/m³; (b) Fluorocarbon membrane filter: 2144 kg/m³.

- Use a density (pw) of 8000 kg/m³ for stainless steel calibration weights or the known density for different calibration weight materials. Follow the International Recommendation OIML R 111-1 Edition 2004(E) (or equivalent) from the International Organization of Legal Metrology on calibration weights.

- Use the uncorrected particle sample mass to calculate the buoyance-corrected particle sample mass (for PM2.5 and PM10, individually) following equation 12.5:

| PMMass= PMloaded×1-papw1-papf | (Eq. 12.5) |
| --- | --- |

- Where:

- PMMass 	is the corrected mass for each filter in mg;

- PMloaded	is the uncorrected mass for each filter in mg per Table 12.4;

- pa 		is the density of air in the balance room per equation 12.6 in kg/m³;

- pw		is the density of the calibration balance weight per paragraph (e);

- pf		is the density of the (unsued) sampling filter per paragraph (e).

- Use the conditions in the balance room at the time of weighing to calculate the density of air, following equation 12.6.

| pa=pb×MmixR×Ta | (Eq. 12.6) |
| --- | --- |

- Where:

- pb		is the atmospheric pressure in the balance room in kPa;

- Mmix	is the molar mass of air in the balance room, 28.836 g mol–1;

- R		is the molar mass constant, 8.3144 J mol–1 K–1;

Ta		is the air temperature in the balance room in K.

- Weighing balance – Use the same microbalance for both pre-sampling and post-sampling weighing in a given emission test. Isolate the balance from vibrations, electrostatic forces, and air streams. Place the balance in a controlled environment – the weighing chamber or room – following the specifications described in point (c) of this paragraph. The balance resolution shall be of at least 1 μg. Use certified calibration weights to verify the stability and the proper function of the microbalance regularly. The microbalance shall fulfill the calibration requirements described in paragraph 14.4;

- Storage and transfer conditions – Keep weighed filters in appositely made filter boxes designed to host the specific filter size. Use stainless steel forceps or tongs for filter handling. Minimize filter movement within the Petri dishes/bags and transport as much as possible. Carefully install the particle sample filter into the filter holder. Rough or abrasive filter handling will result in erroneous weight determination;

- Reference Filters – Use reference filters to validate PM weighing. Select at least two Fluorocarbon coated glass fibre or fluorocarbon membrane reference filters that match each sampled filter media. Weigh the reference filters at the beginning and the end of a weighing session. The average difference between the initial and final measurement for the reference filter shall remain within ±10 µg. Furthermore, the average difference between the reference filters and their moving average (min 1 day – max 15 days) shall be within ±10 µg. Replace the reference filters at a maximum every 30 days and in such a manner that no sample filter is weighted without comparison to a reference filter that has been present in the weighing chamber (room) for at least one day;

- Sample weighing – Weigh each filter sample twice. If the difference between the first and second measurements is lower than 30 µg use the average to report PMloaded and calculate PMMass. When the difference between the first and second measurements is higher than 30 µg measure the sampled filter for the third time. If the difference between the second and third measurements is lower than 30 µg use the average of the two measurements to report PMloaded and calculate PMMass. If the difference between the second and third measurements is higher than 30 µg consider the measurement invalid and the filter void.

PM Emission Factor Calculation

The testing facility shall report the PM emissions in particle mass per distance driven. Calculate the PM2.5 and PM10 emission factors following equations 12.7 and 12.8, respectively.

| PM2.5EF=PM2.5 Mass×60 000×NQNQPM2.5d | (Eq. 12.7) |
| --- | --- |
| PM10EF=PM10 Mass×60 000×NQNQPM10d | (Eq. 12.8) |

Where:

PM2.5 EF	is the PM2.5 emissions in particle mass per distance driven in mg/km;

PM2.5 EF	is the PM10 emissions in particle mass per distance driven in mg/km;

PM2.5 Mass	is the PM2.5 filter mass per equation 12.5 in mg;

PM10 Mass	is the PM10 filter mass per equation 12.5 in mg;

NQ	is the average normalized airflow in the sampling tunnel in Nm³/h per Table 13.2;

NQPM2.5	is the average normalized airflow in the PM2.5 sampling nozzle in Nl/min per Table 13.2;

NQPM10	is the average normalized airflow in the PM10 sampling nozzle in Nl/min per Table 13.2;

d	is the total distance driven during the WLTP-Brake cycle in km per Table 13.2.

- Calculate the PM masses (PM10Mass and PM2.5Mass) as specified in paragraph 12.1.4 (i) after correcting the values for buoyancy as specified in paragraph 12.1.4 (e);

- Calculate the average normalized tunnel flow throughout the emissions measurement (NQ) from the given parameter in the Time-Based file (Column L “Cooling Airflow Actual Normalized”);

- Calculate the average normalized sampling flows throughout the emissions measurement (NQPM2.5 and NQPM10) from the given parameters in the Time-Based file (Column S “PM2.5 Sampling Airflow Actual Normalized” and Column V “PM10 Sampling Airflow Actual Normalized”);

- Calculate the total distance of the WLTP-Brake cycle (d) from the given parameter in the Time-Based file (Column D “Driven Distance”).

Measurement of Particle Number (PN) Concentration

This paragraph describes the specifications for the PN emissions measurement during brake emissions testing. The PN sampling and measurement systems enable the quantification of the number of particles generated by the friction couple during the test. The measured PN emissions along with the parameters from the test provide the emissions factors for the brake under test in a number of particles emitted per distance driven. The test system shall be capable of measuring Total-PN (TPN10) and Solid-PN (SPN10) at a nominal particle size of approximately 10 nm electrical mobility diameter and larger using separate measurement systems.

Figure 12.2 illustrates the PN sampling and measurement setup. The positioning and dimensions of the different elements are indicative and are provided for illustration purposes; therefore, exact conformance with the figure is not required.

Figure 12.2
Indicative setup of the PN sampling and measurement unit

The TPN10 and SPN10 sampling and measurement systems shall consist of the following elements:

Total-PN

- A PN sampling probe extracts a sample from homogeneously mixed flow in the sampling tunnel. The specifications for the design of the PN sampling probe are described in paragraph 12.2.1.2;

- An appropriate PN sampling nozzle is installed at the tip of the PN sampling probe. The specifications for the design of the nozzle are described in paragraph 12.2.1.3;

- A suitable tube (Particle Transfer Tube – PTT) transfers particles from the sampling probe to the pre-classifier. The specifications for the design of the PTT are described in paragraph 12.2.1.4;

- A pre-classifier is applied to remove bigger particles. The specifications for the pre-classifier are described in paragraph 12.2.2.1;

- A dilution system that incorporates one or more dilution stages. The specifications for the design of the dilution system are described in paragraph 12.2.2.2;

- A particle line transfers the particles from the dilution system to the particle number counting device. The specifications for the design of the outlet line are described in paragraph 12.2.2.3;

- A Particle Number Counter (PNC) measures the TPN10 concentration. The specifications for the PNC are described in paragraph 12.2.3.1.

Solid-PN

- A PN sampling probe extracts a sample from homogeneously mixed flow in the sampling tunnel. The specifications for the design of the PN sampling probe are described in paragraph 12.2.1.2;

- An appropriate PN sampling nozzle is installed at the tip of the PN sampling probe. The specifications for the design of the nozzle are described in paragraph 12.2.1.3;

- A suitable PTT transfers particles to the pre-classifier. The specifications for the design of the PTT are described in paragraph 12.2.1.4;

- A pre-classifier is applied to remove bigger particles. The specifications for the pre-classifier are described in paragraph 12.2.2.1;

- A Volatile Particle Remover (VPR) dilutes the sample and removes volatile particles before the measurement device. The specifications for the design of the VPR are described in paragraph 12.2.2.2;

- A particle line transfers the particles from the VPR to the PNC. The specifications for the design of the outlet line are described in paragraph 12.2.2.3;

- A PNC measures the SPN10 concentration. The specifications for the PNC are described in paragraph 12.2.3.1.

- TPN10 and SPN10 sampling shall use different probes as specified in 12.2.1.1 (a). The same sampling probe can be used provided that the applied flow splitter fulfils the requirements specified in paragraph 12.2.1.1 (b-e).

Sample Extraction

Sampling Plane

The design of the sampling plane shall follow the specifications described in paragraph 7.6. The following additional specifications apply to the sampling plane for the installation of the PN sampling probes:

- Apply two sampling probes for the PN emission measurements, one for TPN10 and one for SPN10. Blue dots in Figure 7.7 indicate the PN sampling probes for both the three and four sampling probes layout;

- Alternatively, use a single sampling probe for both TPN10 and SPN10 applying an appropriate flow splitting device as specified in points (c-e) of this paragraph;

- When applying a flow splitting device it shall be made from stainless steel and an electropolished interior to minimize the contamination of the sample;

- When applying a flow splitting device, keep the change in the flow angle within 20⁰ (≤ 20⁰) for each outlet to ensure similar flow velocities in all branches. Verify that the flow velocities in the different branches do not differ from each other by more than ±5 per cent;

- When applying a flow splitting device, demonstrate that the penetration with and without the splitter remains within ±5 per cent at all operating conditions. Perform the comparison by measuring the particle penetration at 15 nm and 1.5 μm with and without the flow splitter using at least one brake system.

PN Sampling Probes

Appropriate PN sampling probe(s) shall be used to extract the sample from the tunnel to the inlet of the particle transfer tube or the pre-classifier. The testing facility shall select PN sampling probes meeting the following design requirements::

- Use probes appropriately designed to minimize particle losses from the nozzle tip to the inlet of the particle transfer tube;

- Use probes made of electrically conductive materials that do not react with brake particles. Use probes made of stainless steel with an electropolished finish (or equivalent) to attain an ultra-clean and ultra-fine surface;

- Select probes with a constant inner diameter (dp) of at least 10 mm and a maximum of 18 mm ensuring a laminar flow (10 mm ≤ dp ≤ 18 mm) under all operating conditions;

- The overall length of the probes from the sampling nozzle tip to the inlet of the particle transfer tube shall not exceed 1 m;

- The residence time from the inlet of the nozzle tip to the inlet of the particle transfer tube shall be below 3s;

- A maximum of one bend of 90° may be applied to the probes provided that the bending radius rp is at least four times the probe diameter (4∙dp).

PN Sampling Nozzles

Appropriate nozzles to ensure isokinetic sampling based on the total extracted sampling flow and the average cooling airflow shall be used. The testing facility shall select sampling nozzles meeting the following requirements for both TPN10 and SPN10 sampling:

- Use nozzles made of stainless steel with an electropolished finish (or equivalent) to attain an ultra-clean and ultra-fine surface;

- Use the appropriate nozzles to achieve an isokinetic ratio in the range of 0.6 to 1.5;

- Select the nozzle size depending on the applied flow to keep the isokinetic ratio (paragraph 12.1.2.4) within the specifications defined in point (b) of this paragraph. Do not use nozzles with an inner diameter lower than 4 mm;

- The nozzle shall have a constant internal diameter along a length equal to at least one internal diameter or at least 10 mm from the nozzle tip, whichever is greater;

- Use nozzles with a thin wall at the tip to minimize distortion of flow. These shall have an outer to inner diameter ratio lower than 1.1 at the nozzle tip;

- Any variation in bore diameter of the nozzles shall be tapered with a conical angle less than 30°;

- Place the nozzle with its axis parallel to that of the dilution tunnel making sure that the aspiration angle remains lower or equal to 15°;

- Clean the nozzles before every brake emissions test following the specifications defined by their manufacturer regarding the cleaning means.

Particle Transfer Tube

When the pre-classifier is not directly connected to the probe’s outlet, a suitable transfer tube (PTT) shall be used to transfer particles from the probe’s outlet to the pre-classifier. The testing facility shall ensure that the design of the PTT meets the following requirements for both TPN10 and SPN10 sampling:

- Use transfer tubes appropriately designed to minimize particle transport losses between the probe’s outlet and pre-classifier’s inlet;

- Use transfer tubes with gradual diameter changes, if needed, to transfer particles from the probe’s outlet to the pre-classifier’s inlet;

- Use transfer tubes made of electrically conductive materials that do not react with brake aerosol components;

- Select transfer tubes with a constant inner diameter (dtt) of at least 4 mm ensuring a laminar flow under all operating conditions;

- The length of the transfer tubes to sample flow ratio shall be below 60000 s/m²;

- The particles' residence time inside the transfer tubes shall be below 1 s;

- A bend may be applied to the transfer tubes provided that the bending radius rp shall be at least twenty-five times the tube diameter (25∙dtt).

Sample Treatment and Conditioning

Pre-classifier

The testing facility shall use a single cyclonic separator to protect the dilution system and the VPR from possible contamination. The following specifications apply to the pre-classifier for both TPN10 and SPN10 sampling and measurement:

- Place the cyclonic separator anywhere between the outlet of the sampling probe and the inlet of the sample conditioning system;

- Use commercially available cyclonic separators with a 50 per cent cut point particle diameter between 2.5 µm and 10 µm at the volumetric sample flow rate that passes through the cyclonic separator;

- The cyclone shall achieve a minimum penetration efficiency of 80 per cent for a particle diameter of 1.5 µm;

- Inspect and clean the inner walls of the cyclones frequently following the specifications of the instrument manufacturer regarding the cleaning frequency and means.

Sample Conditioning

The particles entering the PN system shall undergo conditioning before entering the PNC. The testing facility shall ensure the sample conditioning system meets the following requirements depending on the measured parameter:

TPN10

A dilution system is mandatory and shall comprise at least one particle number diluter. A dilution system like the VPR for SPN10 described in the next paragraph may be used. In this case, any active heating of the evaporation tube and the diluters shall be deactivated. The following specifications apply to the dilution system for conditioning particles when measuring TPN10:

- All parts of the dilution system that come in contact with the sample shall be made of electrically conductive materials, shall be electrically grounded to prevent electrostatic effects, and shall be designed to minimize deposition of the particles;

- It shall be capable of diluting the sample in one or more stages to achieve a PN concentration below the upper threshold of the single-particle count mode of the PNC. The dilution system shall be capable of providing a dilution factor of at least 10:1;

- It shall be capable of keeping the dilution factor constant throughout the entire emissions test;

- It shall be capable of maintaining the diluted gas temperature at the inlet to the PNC below 38 °C;

- It shall be supplied with dilution air filtered through a HEPA filter of at least class H13 (EN 1822:2008), or equivalent performance;

- It shall achieve a particle concentration reduction factor (PCRF) for particles of 15 nm, 30 nm, and 50 nm electrical mobility diameters not higher than 100 per cent, 30 per cent, and 20 per cent, respectively, compared to particles of 100 nm electrical mobility diameter for the system as a whole. Additionally, it shall achieve a PCRF for particles of 15 nm, 30 nm, and 50 nm not lower than 5 per cent than that for particles of 100 nm for the system as a whole. The calculation of the PCRF at different sizes shall follow the method described in paragraph 14.5.1;

- It shall monitor the arithmetic average PCRF (fr-TPN10) in real-time and report it at a frequency of at least 1 Hz. The calculation of the arithmetic average PCRF shall follow the method described in paragraph 14.5.1;

- It shall achieve a total particle penetration efficiency of at least 70 per cent for particles of 100 nm electrical mobility diameter;

- It shall be capable of operating at sample pressures in the (850 to 1050) mbar range and relative pressure differences from ambient in the ±50 mbar range.

SPN10

The volatile particle remover (VPR) shall comprise at least one particle number diluter (PND1) and an evaporation tube. A second diluter (PND2) may be optionally installed in series with the PND1 and the evaporation tube. The following specifications apply to the VPR for conditioning particles when measuring SPN10:

- All parts of the VPR that come in contact with the sample shall be made of electrically conductive materials, shall be electrically grounded to prevent electrostatic effects, and shall be designed to minimize deposition of the particles;

- It shall be capable of diluting the sample in one or more stages to achieve a PN concentration below the upper threshold of the single-particle count mode of the PNC. The overall system shall be capable of providing a dilution factor of at least 10:1;

- It shall be capable of maintaining the gas temperature at the inlet to the PNC below a maximum allowed inlet temperature specified by the PNC manufacturer;

- It may include an initial heated dilution stage which outputs the sample at a wall temperature between 150 °C and 350 °C. Alternatively, the use of cold dilution is allowed provided that all requirements set out in this paragraph are met. The wall temperature set point shall not exceed the wall temperature of the evaporation tube. The diluter shall be supplied with air filtered through a HEPA filter of at least class H13 (EN 1822:2008), or equivalent performance;

- It shall include a catalytically active evaporation tube which is controlled to a wall temperature greater than or equal to that of the PND1. The wall temperature of the evaporation tube shall remain at a fixed nominal operating temperature of 350 °C;

- It shall control heated stages to constant nominal operating temperatures to a tolerance of ±10 °C. Additionally, the VPR system shall indicate whether heated stages are at their correct operating temperatures;

- It shall achieve a PCRF for particles of 15 nm, 30 nm, and 50 nm electrical mobility diameters not higher than 100 per cent, 30 per cent, and 20 per cent, respectively, compared to particles of 100 nm electrical mobility diameter for the VPR as a whole. Additionally, it shall achieve a PCRF for particles of 15 nm, 30 nm, and 50 nm not lower than 5 per cent than that for particles of 100 nm for the VPR as a whole. The calculation of the PCRF at different sizes shall follow the method described in paragraph 14.5.1;

- It shall monitor the arithmetic average PCRF (fr-SPN10) in real-time and report it at a frequency of at least 1 Hz. The calculation of the arithmetic average PCRF shall follow the method described in paragraph 14.5.1;

- It shall achieve more than 99.9 per cent vaporization of tetracontane (CH3(CH2)38CH3) particles with a count median diameter larger than 50 nm and mass above 1 mg/m³, by means of heating and reduction of partial pressures of the tetracontane;

- It shall achieve a solid particle penetration efficiency of at least 70 per cent for particles of 100 nm electrical mobility diameter;

- It shall operate at sample pressures in the (850 to 1050) mbar range and relative pressure differences from ambient in the ±50 mbar range.

PN Internal Transfer Line

The testing facility shall select lines that transfer the particles from the dilution system (TPN10) and the VPR (SPN10) to the inlet of the PNC meeting the specifications described below:

- Use outlet transfer lines appropriately designed to minimize particle transport losses between the dilution system (TPN10) or the VPR (SPN10) and the inlet of the PNC;

- Use outlet transfer lines made of electrically conductive materials that do not react with brake aerosol components;

- Select outlet transfer lines with a constant inner diameter (dtl) of at least 4 mm ensuring a laminar flow under all operating conditions;

- The overall length of the outlet lines from the exit of the dilution system (TPN10) or the VPR (SPN10) to the inlet of the PNC shall not exceed 1 m;

- The particles’ residence time inside the outlet lines shall be below 1 s;

- A bend may be applied to the outlet lines provided that the bending radius rp shall be at least twenty-five times the outlet line diameter (25∙dtl).

Particle Measurement

Particle Number Counter

Particle Number Counters (PNC) shall be applied for the measurement of the TPN10 and SPN10 concentrations. The testing facility shall ensure that the PNC meets the following requirements for both TPN10 and SPN10:

- Operate under full flow operating conditions;

- Have a counting accuracy of ±10 per cent across the range from 1 #/cm³ to the upper threshold of the single-particle count mode of the PNC against a traceable standard;

- Have readability of at least 0.1 #/cm³ at concentrations below 100 #/cm³;

- Have a linear response to particle concentrations over the full measurement range in single-particle count mode;

- Report PCRF-corrected PN concentrations at standard conditions at a reporting frequency equal to or greater than 0.5 Hz;

- Have a t90 response time over the measured concentration range of less than 5 s;

- Incorporate an internal calibration factor from the linearity calibration against a traceable reference which shall be applied to determine the PNC counting efficiency. The counting efficiency shall be reported including the calibration factor according to the specifications provided in paragraph 14.6;

- The PNC calibration material shall be 4 cSt polyalphaolefin (Emery oil) or soot-like particles (e.g. flame generated soot or graphite particles);

- Have counting efficiencies at nominal particle sizes of 10 nm and 15 nm electrical mobility diameter of (65 ± 15) per cent and above 90 per cent, respectively. These counting efficiencies may be achieved by internal (e.g. control of instrument design) or external (e.g. size pre-classification) means;

- If the PNC makes use of a working liquid, it shall be replaced at the frequency specified by the instrument manufacturer.

PN Sampling Flow

The PN measurement system shall meet the following provisions for the regulation and measurement of the sampling volumetric flow (i.e. flow at the PN sampling probe). These apply to both TPN10 and SPN10 sampling:

- The method of measuring the volumetric flow of the sampling and measurement system shall have a maximum permissible error of ±5 per cent of the reading under all operating conditions;

- Use a volumetric flow measurement device calibrated to report flow at both operating and standard conditions (273.15 K and 101.325 kPa). The temperature sensor shall have an accuracy of ±1.0 °C. The pressure measurements shall have precision and accuracy of ±1.0 kPa;

- The actual normalized sampling volumetric flow (NQTPN10 and NQSPN10) shall not deviate more than ±10 per cent of the average value for the given test. Use a device with a flow control feature (e.g. critical orifice, pressure regulator, feedback controller, or other) to ensure a stable flow;

- Register the actual normalized sampling volumetric flow and report it at a frequency of 1Hz in the Time-Based file (TPN10: Column W “TPN10 Sampling Airflow Actual Normalized” – SPN10: Column Z “SPN10 Sampling Airflow Actual Normalized”). Report the average actual normalized sampling volumetric flows as specified in paragraph 13.4;

- Ensure the average isokinetic ratio during the emissions measurement section of a specific brake system is between 0.60 and 1.50;

- Use equation 12.4 to calculate the average isokinetic ratio at the emissions measurement section of a test for both TPN10 and SPN10. Use the corresponding values for the isokinetic nozzle inner diameters for TPN10 (dn-TPN10) and SPN10 (dn-SPN10) sampling. Use the average normalized tunnel flow (Column L “Cooling Airflow Actual Normalized”) and the average normalized sample flows (Column W “TPN10 Sampling Airflow Actual Normalized” and Column Z “SPN10 Sampling Airflow Actual Normalized”) in the Time-Based file. Report the calculated values as specified in paragraph 13.4;

- If the sampling flow or the isokinetic requirements set out in this paragraph are not met, the test is invalid;

- The PN sampling devices shall operate continuously during the brake emissions measurement section. This includes also the cooling sections between the individual trips of the WLTP-Brake cycle where the PN sampling flow shall not be paused or bypass the main sampling line.

PN Emissions Calculation

The testing facility shall report PN emissions in a number of particles per distance driven. Calculating the PN emission factors for TPN10 and SPN10 follows equations 12.9 and 12.10, respectively.

| TPN10 EF=106×TPN10 #×NQV | (Eq. 12.9) |
| --- | --- |
| SPN10 EF=106×SPN10 #×NQV | (Eq. 12.10) |

Where:

TPN10 EF	is the number of TPN10 per distance driven in #/km;

SPN10 EF	is the number of SPN10 per distance driven in #/km;

TPN10 #	is the average normalized and PCRF-corrected TPN10 emissions in #/Ncm³ per Table 13.2;

SPN10 #	is the average normalized and PCRF-corrected SPN10 emissions in #/Ncm³ per Table 13.2;

NQ	is the average normalized airflow in the sampling tunnel in Nm³/h per Table 13.2;

V 		is the average actual velocity of the WLTP-Brake cycle in km/h per Table 13.2.

- Calculate the average normalized and PCRF-corrected TPN10 and SPN10 emissions from the given parameters in the Time-Based file (Column Y: “TPN10 Concentration Normalized - PCRF Corrected” and Column AB: “SPN10 Concentration Normalized - PCRF Corrected”);

- Calculate the average normalized tunnel flow throughout the emissions measurement (NQ) from the given parameter in the Time-Based file (Column L “Cooling Airflow Actual Normalized”);

- Calculate the average velocity of the WLTP-Brake cycle (V) from the given parameter in the Time-Based file (Column C “Linear Speed Actual”);

- In case measured TPN10 or SPN10 emissions are out of the specified measurement range of the PNC device, the test shall be considered invalid.

PN System Verification Procedures

The testing facility shall apply the following PN system check procedures to verify the whole system is fully operational:

- The flow into the particle counter PNC shall have a measured value within 5 per cent of the particle counter PNC nominal flow rate when checked with a calibrated flow meter. Here the term ‘nominal flow rate’ refers to the flow rate stated in the last calibration for the PNC. The testing facility shall perform this check every month;

- A zero check on the PNC, using a filter of appropriate performance at the PNC inlet, shall report a concentration of ≤0.2 #/cm³. Upon removal of the filter, the PNC shall show an increase in measured concentration and a return to 0.2 #/cm³ or less on replacement of the filter. The PNC shall not report any errors. The testing facility shall perform this check every day;

- The particle counter shall report a measured concentration of less than 0.5 #/cm³ when a HEPA filter of at least class H13 (EN 1822:2008), or equivalent performance, is attached to the inlet of the entire particle sampling system. The testing facility shall perform this check before each brake emissions test;

- Before the start of each test, the testing facility shall confirm that the measurement system indicates that the VPR has reached its correct operating temperatures.

Mass Loss Measurement

The mass loss of the friction couple under test provides helpful information regarding the emissions test's robustness and correctness. It helps identify possible issues during the execution of the test campaign. Mass loss can provide additional information when correlating to PM measurements for multiple tests. The testing facility shall measure the initial and final mass of the brake couple under test ensuring the following:

- Inspect all brake parts for burs, cracks, voids, or detachments and record accordingly. If there are no problems with the brake under testing, measure the brake disc or drum and the brake pads or shoes mass before commencing the brake emissions test;

- Measure the mass of each part separately with the thermocouple installed and the thermocouple connector removed (in case of discs and drums);

- Measure the brake friction material including the anti-noise shims, pad-shim springs, and other elements when part of the product assembly;

- Vacuum clean the parts before conducting the measurements to remove any possible contamination;

- Use a weighing scale of a resolution of at least 0.01 g or better for parts below 30 kg of total weight. Install the weighing scale in a room with controlled air and humidity to standard laboratory conditions of (22 ± 2) °C and (45 ± 8) per cent RH;

- Measure the brake disc or drum and the brake pads or shoes mass after the completion of the brake emissions test. Make sure not to disrupt the brake assembly during the brake emissions test;

- Ensure the brake parts are cool down to a temperature of 30 °C or below before measuring their final mass;

- Clean the parts to remove any grease or contamination before performing the final mass measurements;

- Calculate the mass loss of the disc or drum and the brake pads or shoes by subtracting the final from the initial total mass. Report the mass loss of each part of the brake couple to the PM-Wear Measurement file following the instructions defined in Table 13.5;

- Calculate the overall mass loss of the brake couple by summing the values for the individual parts calculated in (i) of this paragraph. Report the overall mass loss following the instructions defined in Table 13.5;

- Calculate the averaged weight loss emission factor by dividing the total mass loss calculated in (j) of this paragraph by the total distance driven during the brake emissions test considering all testing sections. Report the averaged weight loss emission factor following the instructions defined in Table 13.5.

The test facility can also measure the thickness of brake discs or brake drums. The thickness loss can sometimes be below the detection limit or below the resolution of the measurement instrument. The same measuring position ensures the repeatability and producibility of the thickness loss data. Other elements that can influence the accuracy of the measurement technique include anti-noise shims, pad-piston springs, and other parts installed on brake pads.

Wear measurement specifications and methodology have been adopted from the SAE J2986:2019 Recommended Practice. It is not required that the test facility reports the wear measurement results.

- Inspect all brake parts for burs, cracks, voids, or detachments and record accordingly. Use only instruments in proper working conditions;

- Use a micrometer or equivalent device with an accuracy of 0.01 mm or better, flat surface(s) on the side(s) contacting the friction material or anti-noise shims, and a ratchet mechanism;

- Use a digital vernier calliper or equivalent device with an accuracy of 0.01 mm or better for measuring the internal brake drum diameter;

- For brake pads, measure at eight locations equally spaced and located approximately 5.0 mm from any edge of the friction material (pad contour, chamfers, or slots). Smaller pads (like those on rear brakes for small vehicles) may accommodate as few as four locations;

- For brake shoes, measure at eight positions equally spaced. Locate the measurement position midpoint between the long edge and the nearest edge of the shoe web. Keep at least 10 mm or 20 per cent of the total lining width from the edge of the friction material and chamfers at the ends;

- For brake discs, measure at eight positions separated into four groups of two and distributed every 90°. At each clock position, measure at 5.0 mm from the outer diameter and 5.0 mm from the inner diameter of the braking surface. Exclude chamfers or round edges to mark the measurement position;

- For brake drums, measure at six locations separated into three groups of two and distributed every 120°. At each clock position, measure at 10 mm from the outer (open) edge and 10 mm from the inner (mounting) edge of the braking surface. Exclude chamfers or round edges to mark the measurement position.

Test Report

This section describes the four main outputs of a brake emissions test. These are summarized in the following:

- Event-Based file. A detailed description of the file and the required parameters is provided in paragraph 13.1;

- Time-Based file. A detailed description of the file and the required parameters is provided in paragraph 13.2;

- PM-Wear measurement file. A detailed description of the file and the required parameters is provided in paragraph 13.3;

- Reporting file. A detailed description of the file and the required parameters is provided in paragraph 13.4.

Event-Based File

The testing facility shall generate a CSV or MS Excel™ Event-Based file for the brake emissions test. The file shall include the necessary data for each brake deceleration event throughout the entire brake emissions test. This file format is agnostic to the control technology and software and allows control authorities to have direct access to the test outputs. Each section of the brake emissions test shall be reported in a separate tab as follows:

- Tab 1 shall include all raw data registered by the brake dynamometer throughout the entire test;

- Tab 2 shall include the registered data and the reporting parameters specified in this paragraph for the cooling adjustment section;

- Tabs 3-7 shall include the registered data and the reporting parameters specified in this paragraph for the bedding section. Each tab shall correspond to one repetition of the WLTP-Brake cycle. Tabs 3-7 shall not include data from the cooling sections applied between the repetitions of the WLTP-Brake cycle;

- Tab 8 shall include the registered data and the reporting parameters specified in this paragraph for the brake emissions measurement. Tab 8 shall not include data from the cooling sections applied between the individual trips of the WLTP-Brake cycle.

The testing facility shall continuously and automatically register and/or calculate the parameters listed in Table 13.1. Details regarding the applied units, number of decimals, and the registration frequency of each parameter are given in Table 13.1. Additionally, a short description of each parameter and the symbol used throughout the text (where applicable) are provided.

Table 13.1. 
Necessary parameters for registration and reporting at the Event-Based file of a brake emissions test

| Measurand | Symbol | Unit | Decimals | Description | Registration Frequency | Column in the File |
| --- | --- | --- | --- | --- | --- | --- |
| Test Section | - | # | [N/A] | A three digits “ABC” identification code for each deceleration event. “A” represents the cycle’s serial number in a given brake emissions test (A=1 for cooling adjustment, A=2-6 for bedding, A=7 for emissions measurement). BC represents the trip’s serial number (B=01-10) | Calculated 1Hz | A |
| Trip Stop Number | - | # | [N/A] | The deceleration event’s serial number within the individual trip (Can take values between 1 and 114) | Registered 1Hz | B |
| Cycle Stop Number | - | # | [N/A] | The deceleration event’s serial number within the WLTP-Brake cycle (Can take values between 1 and 303) | Registered 1Hz | C |
| Stop Duration | tbrake | s | [1] | The total duration of the deceleration event | Registered 250Hz | D |
| Time of Stop | - | hh:mm:ss | [N/A] | Time at the beginning of the deceleration event registered by the brake dynamometer | Registered 1Hz | E |
| Date of Stop | - | yy:mm:dd | [N/A] | The date at the beginning of the deceleration event registered by the brake dynamometer | Registered 1Hz | F |
| Brake Speed Setpoint | - | km/h | [1] | The nominal linear speed at the beginning of the deceleration event as defined in the WLTP-Brake cycle | Registered 1Hz | G |
| Brake Speed | - | km/h | [1] | The actual linear speed at the beginning of the deceleration event registered by the brake dynamometer | Registered 250Hz | H |
| Release Speed Setpoint | - | km/h | [1] | The nominal linear speed at the end (release) of the deceleration event as defined in the WLTP-Brake cycle | Registered 1Hz | I |
| Release Speed | - | km/h | [1] | The actual linear speed at the end (release) of the deceleration event registered by the brake dynamometer | Registered 250Hz | J |
| Rotational Speed – Time Averaged | f | rpm | [1] | Time-averaged rotational brake speed registered by the brake dynamometer | Registered 250Hz | K |
| Deceleration Setpoint | - | m/s2 | [2] | Nominal deceleration rate of the event as defined in the WLTP-Brake cycle | Registered 1Hz | L |
| Deceleration rate – Distance Averaged | - | m/s2 | [2] | Deceleration rate of the given brake event | Registered 1Hz | M |
| Brake Torque – Distance Averaged | - | Nm | [1] | Distance averaged brake torque registered by the brake dynamometer | Registered 250Hz | N |
| Brake Torque – Time Averaged | τbrake-avg | Nm | [1] | Time-averaged brake torque registered by the brake dynamometer | Registered 250Hz | O |
| Brake Pressure – Distance Averaged | - | bar | [2] | Distance averaged brake pressure registered by the brake dynamometer | Registered 250Hz | P |
| Friction Coefficient – Distance Averaged | μ | - | [3] | A function of braking torque, effective brake radius, and the piston area | Registered 1Hz | Q |
| Initial Brake Temperature | IBT | °C | [1] | Brake temperature at the beginning of the deceleration event measured by the embedded thermocouple | Registered 250Hz | R |
| Final Brake Temperature | FBT | °C | [1] | Brake temperature at the end of the deceleration event measured by the embedded thermocouple | Registered 250Hz | S |
| Peak Brake Temperature | PBT | °C | [1] | Peak brake temperature of the deceleration event measured by the embedded thermocouple | Registered 250Hz | T |
| Friction Work | Wf | J/kg | [1] | The actual friction work applied to the brake in the given deceleration event | Calculated 1Hz | U |

Time-Based File

The testing facility shall generate a CSV or MS Excel™ Time-Based file for the brake emissions test. The file shall include information about certain testing parameters registered throughout the entire brake emissions test. Each section of the brake emissions test shall be reported in a separate tab as follows:

- Tab 1 shall include all raw data registered by the brake dynamometer, the sampling instruments, and the measurement devices as specified in this paragraph;

- Tab 2 shall include the data for the parameters specified in this paragraph collected during the cooling adjustment section;

- Tabs 3-7 shall include the data for the parameters specified in this paragraph collected during the bedding section. Each tab shall correspond to one repetition of the WLTP-Brake cycle. Tabs 3-7 shall not include data from the cooling sections applied between the repetitions of the WLTP-Brake cycle;

- Tab 8 shall include the data for the parameters specified in this paragraph collected during the brake emissions measurement. Tab 8 shall not include data from the cooling sections applied between the individual trips of the WLTP-Brake cycle.

The testing facility shall continuously and automatically register and/or calculate the parameters listed in Table 13.2. Details regarding the applied units, number of decimals, and the registration frequency of each parameter are given in Table 13.2. Additionally, a short description of each parameter and the symbol used throughout the text is provided.

Table 13.2. 
Necessary parameters for registration and reporting at the Time-Based file of a brake emissions test

| Measurand | Symbol | Unit | Decimals | Description | Registration Frequency | Column in the File |
| --- | --- | --- | --- | --- | --- | --- |
| Timestamp | - | sec | [0] | Timestamp from the beginning to the end of the test | Registered 1Hz | A |
| Linear Speed Nominal | Vset | km/h | [1] | The nominal linear speed at the given point in time as defined in the WLTP-Brake cycle | Registered 1Hz | B |
| Linear Speed Actual | V | km/h | [1] | Actual linear speed registered by the brake dynamometer at the given point in time | Registered 10Hz | C |
| Driven Distance | d | km | [1] | Distance driven registered by the brake dynamometer at the given point in time | Registered 10Hz | D |
| Deceleration Rate | α | m/s2 | [2] | Deceleration rate registered by the brake dynamometer at the given point in time | Registered 10Hz | E |
| Brake Torque | τbrake | N·m | [1] | Brake torque registered by the brake dynamometer at the given point in time | Registered 10Hz | F |
| Brake Pressure | Pbrake | bar | [2] | Brake pressure registered by the brake dynamometer at the given point in time | Registered 10Hz | G |
| Friction Coefficient | μ | - | [3] | Brake Effectiveness calculated at the given point in time | Registered 10Hz | H |
| Brake Temperature | Tbrake | °C | [1] | Registered brake temperature at the given point in time | Registered 10Hz | I |
| Cooling Airflow Set | Qset | m3/h | [1] | Set (nominal) cooling airflow for the given brake emissions test | Registered 1Hz | J |
| Cooling Airflow Actual | Q | m3/h | [1] | Measured cooling airflow at the given point in time | Registered 10Hz | K |
| Cooling Airflow Actual Normalized | NQ | Nm3/h | [1] | Normalized cooling airflow at standard conditions (273.15 K and 101.325 kPa) at the given point in time | Registered 10Hz | L |
| Cooling Airspeed Actual | U | km/h | [1] | Cooling airspeed at the given point in time (measured or calculated) | Registered 10Hz | M |
| Cooling Air Temperature | T | °C | [1] | The temperature of the cooling air at the given point in time | Registered 10Hz | N |
| Cooling Air Relative Humidity | RH | % | [1] | Relative humidity of the cooling air at the given point in time | Registered 10Hz | O |
| Cooling Air Pressure | P | kPa | [1] | The pressure of the cooling air at the given point in time | Registered 10Hz | P |
| PM2.5 Sampling Airflow Set | QPM2.5-set | l/min | [2] | Set (nominal) PM2.5 sampling airflow for the given brake emissions test | Registered 1Hz | Q |
| PM2.5 Sampling Airflow Actual | QPM2.5 | l/min | [2] | PM2.5 sampling airflow measured at the given point in time | Registered 10Hz | R |
| PM2.5 Sampling Airflow Actual Normalized | NQPM2.5 | Nl/min | [2] | Normalized PM2.5 sampling airflow at standard conditions (273.15 K and 101.325 kPa) at the given point in time | Registered 10Hz | S |
| PM10 Sampling Airflow Set | QPM10-set | l/min | [2] | Set (nominal) PM10 sampling airflow for the given brake emissions test | Registered 1Hz | T |
| PM10 Sampling Airflow Actual | QPM10 | l/min | [2] | PM10 sampling airflow measured at the given point in time | Registered 10Hz | U |
| PM10 Sampling Airflow Actual Normalized | NQPM10 | Nl/min | [2] | Normalized PM10 sampling airflow at standard conditions (273.15 K and 101.325 kPa) at the given point in time | Registered 10Hz | V |
| TPN10 Sampling Airflow Actual Normalized | NQTPN10 | Nl/min | [2] | TPN10-related sampling airflow measured at the given point in time and reported at standard conditions (273.15 K and 101.325 kPa) | Registered 1Hz | W |
| TPN10 - Average PCRF | fr-TPN10 | - | [1] | Arithmetic average particle concentration reduction factor for the TPN10 measurement | Registered 10Hz | X |
| TPN10 Concentration Normalized - PCRF Corrected | TPN10# | #/Ncm3 | [1] | TPN10 normalized concentration at standard conditions (273.15 K and 101.325 kPa) measured by the PNC and corrected for the PCRF at the given point in time | Registered 10Hz | Y |
| SPN10 Sampling Airflow Actual Normalized | NQSPN10 | Nl/min | [2] | SPN10-related sampling airflow measured at the given point in time and reported at standard conditions (273.15 K and 101.325 kPa) | Registered 1Hz | Z |
| SPN10 - Average PCRF | fr-SPN10 | - | [1] | Arithmetic average particle concentration reduction factor for the SPN10 measurement | Registered 10Hz | AA |
| SPN10 Concentration Normalized - PCRF Corrected | SPN10# | #/Ncm3 | [1] | Total SPN10 normalized concentration at standard conditions (273.15 K and 101.325 kPa) measured by the PNC and corrected for the PCRF at the given point in time | Registered 10Hz | AB |

PM-Wear Measurement File

The testing facility shall generate a CSV or MS Excel™ PM-Wear Measurement file for the entire test. The file shall include information about weighing the filters as specified in paragraph 12.1.5 as well as for weighing the brake parts as specified in paragraph 12.3. PM mass weighing data shall be reported in one tab as specified in Table 13.3. Information about the reference filters shall be reported in a different tab as specified in Table 13.4. Finally, information regarding wear shall be reported in a separate tab as specified in Table 13.5.

PM Mass Weighing Data

The testing facility shall register and calculate the parameters related to the PM mass measurement listed in Table 13.3. Details regarding the applied units and the number of decimals of each parameter are provided in Table 13.3. Additionally, a short description of each parameter is given. PM mass weighing data shall be reported in a tab titled “PM Mass Weighing Data Entry” of the PM-Wear Measurement file.

Table 13.3. 
Necessary parameters related to the PM mass measurement procedure for registration and reporting at the PM-Wear Measurement file of a brake emissions test

| Measurand | Unit | Decimals | Description | Column in the File |
| --- | --- | --- | --- | --- |
| Test ID | # | [N/A] | A unique code that allows the testing facility to identify the tested brake | A |
| Filter Material | # | [N/A] | Specifies the type of filter used for PM sampling as per paragraph 12.1 | B |
| PM2.5 | # | [N/A] | Specifies whether the input data refer to PM2.5 sampling and measurement | C |
| PM10 | # | [N/A] | Specifies whether the input data refer to PM10 sampling and measurement | D |
| Weighing Date | yyyy-mm-dd | [N/A] | The date on which weighing of the unloaded filter takes place | E |
| Weighing Time | hh:mm | [N/A] | Time at which weighing of the unloaded filter takes place | F |
| Stabilization time before weighing | hh:mm | [N/A] | Reports the stabilization time of the unloaded filter before being weighed and used for sampling as per paragraph 12.1 | G |
| Elapsed time from weighing to test start | hh:mm | [N/A] | Reports the elapsed time from weighing the unloaded filter to the beginning of the emissions test as per paragraph 12.1 | H |
| Measurement 1 | mg | [3] | Weight of the unloaded filter measured at the first weighing as per paragraph 12.1 | I |
| Measurement 2 | mg | [3] | Weight of the unloaded filter measured at the second weighing as per paragraph 12.1 | J |
| Measurement 3 | mg | [3] | Weight of the unloaded filter measured at the third weighing as per paragraph 12.1 – This measurement is necessary only if the deviation between the first two measurements is higher than 30 μg | K |
| Mean Value | mg | [3] | The average weight of the unloaded filter as specified in paragraph 12.1 | L |
| Mean Value – Corrected | mg | [3] | The corrected average weight of the unloaded filter after applying the buoyancy correction per paragraph 12.1 | M |
| Ambient Air Temperature | °C | [1] | Weighing room temperature – Report the average temperature of the room during the last hour before the weighing procedure | N |
| Ambient Air Relative Humidity | % | [1] | Weighing room relative humidity – Report the average relative humidity of the room during the last hour before the weighing procedure | O |
| Weighing Date | yyyy-mm-dd | [N/A] | The date on which weighing of the loaded filter takes place | P |
| Weighing Time | hh:mm | [N/A] | Time at which weighing of the loaded filter takes place | Q |
| Stabilization time before weighing | hh:mm | [N/A] | Reports the stabilization time of the loaded filter after sampling and before being weighed as per paragraph 12.1 | R |
| Elapsed time from weighing to test start | hh:mm | [N/A] | Reports the elapsed time from the end of the emissions tests to weighing the loaded filter as per paragraph 12.1 | S |
| Measurement 1 | mg | [3] | Weight of the loaded filter measured at the first weighing as per paragraph 12.1 | T |
| Measurement 2 | mg | [3] | Weight of the loaded filter measured at the second weighing as per paragraph 12.1 | U |
| Measurement 3 | mg | [3] | Weight of the loaded filter measured at the third weighing as per paragraph 12.1 – This measurement is necessary only if the deviation between the first two measurements is higher than 30 μg | V |
| Mean Value | mg | [3] | The average weight of the loaded filter as specified in paragraph 12.1 | W |
| Mean Value – Corrected | mg | [3] | The corrected average weight of the loaded filter after applying the buoyancy correction per paragraph 12.1 | X |
| Ambient Air Temperature | °C | [1] | Weighing room temperature – Report the average temperature of the room during the last hour before the weighing procedure | Y |
| Ambient Air Relative Humidity | % | [1] | Weighing room relative humidity – Report the average temperature of the room during the last hour before the weighing procedure | Z |
| Loaded Mass | mg | [3] | PM10loaded or PM2.5loaded: The difference between the mean corrected value of the loaded and unloaded filter – Subtract the value in column M by the value in column X | AA |

Reference Filters Data

The testing facility shall register the parameters related to the reference filters used for the PM mass measurement of a given brake. Details regarding the parameters, the applied units, and the number of decimals of each parameter are provided in Table 13.4. The reference filter data shall be reported in a tab titled “Reference Filter Measurement” of the PM-Wear Measurement file.

Table 13.4. 
Necessary parameters related to the reference filters used at the PM mass measurement procedure for registration and reporting at the PM-Wear Measurement file of a brake emissions test

| Measurand | Unit | Decimals | Description | Column in the File |
| --- | --- | --- | --- | --- |
| Test ID | # | [N/A] | A unique code that allows the testing facility to identify the tested brake – Shall equal to “Test ID” in Table 13.3 | A |
| Filter Material | # | [N/A] | Type of filter used as reference per paragraph 12.1 – Shall be the same as the filter used in the emissions test | B |
| Weighing Date Beginning | yyyy-mm-dd | [N/A] | The date on which the initial weighing of the reference filter takes place | C |
| Weighing Time Beginning | hh:mm | [N/A] | Time at which initial weighing of the reference filter takes place | D |
| Measurement Beginning | mg | [3] | Weight of the reference filter measured at the beginning as defined in paragraph 12.1 | E |
| Ambient Air Temperature | °C | [1] | Weighing room temperature – Average temperature of the room during the last hour before the weighing procedure | F |
| Ambient Air Relative Humidity | % | [1] | Weighing room relative humidity – Average RH of the room during the last hour before the weighing procedure | G |
| Weighing Date End | yyyy-mm-dd | [N/A] | The date on which the final weighing of the reference filter takes place | H |
| Weighing Time End | hh:mm | [N/A] | Time at which final weighing of the reference filter takes place | I |
| Measurement End | mg | [3] | Weight of the reference filter measured at the end as defined in paragraph 12.1 | J |
| Ambient Air Temperature | °C | [1] | Weighing room temperature – Average temperature of the room during the last hour before the weighing procedure | K |
| Ambient Air Relative Humidity | % | [1] | Weighing room relative humidity – Average RH of the room during the last hour before the weighing procedure | L |
| Difference | mg | [3] | Difference between the weighed value of the reference filter at the beginning and the end of the testing campaign – Subtract the value in column E by the value in column L | M |

Mass Loss Measurement Data

The testing facility shall register the parameters related to the total mass loss of the tested brake in a separate tab as specified in paragraph 12.3. Details regarding the parameters, the applied units, and the number of decimals of each parameter are provided in Table 13.5. The mass loss measurement data shall be reported in a tab titled “Wear measurement Data Entry” of the PM-Wear Measurement file.

Table 13.5. 
Necessary parameters related to the total mass loss of the brake for registration and reporting at the PM-Wear Measurement file of a brake emissions test

| Measurand | Unit | Decimals | Description | Column in the File |
| --- | --- | --- | --- | --- |
| Test ID | # | [N/A] | A unique code that allows the testing facility to identify the tested brake – Shall equal to “Test ID” in Table 13.3 | A |
| Disc Brake | # | [N/A] | Specifies whether the testing brake couple consists of a disc and a pair of pads | B |
| Drum Brake | # | [N/A] | Specifies whether the testing brake couple consists of a drum and a pair of shoes | C |
| Initial Weightings Inner pad / Leading shoe | mg | [3] | Weight of the inner pad or the leading shoe before the beginning of the overall testing procedure - Leading shoe is the first shoe after the wheel cylinder in the direction of the wheel rotation | D |
| Initial Weightings Outer pad / Trailing shoe | mg | [3] | Weight of the outer pad or the trailing shoe before the beginning of the overall testing procedure - Trailing shoe is the shoe behind the wheel cylinder in the direction of the wheel rotation | E |
| Initial Weightings Disc / Drum | mg | [3] | Weight of the disc or drum before the beginning of the overall testing procedure | F |
| Final Weightings Inner pad / Leading shoe | mg | [3] | Weight of the inner pad or the leading shoe after the end of the overall testing procedure | G |
| Final Weightings Outer pad / Trailing shoe | mg | [3] | Weight of the outer pad or the trailing shoe after the end of the overall testing procedure | H |
| Final Weightings Disc / Drum | mg | [3] | Weight of the disc or drum after the end of the overall testing procedure | I |
| Mass Loss Inner pad / Leading shoe | mg | [3] | Difference between the weighed value of the inner pad or the leading shoe at the beginning and the end of the overall testing procedure – Subtract the value in column D from the value in column G | J |
| Mass Loss Outer pad / Trailing shoe | mg | [3] | Difference between the weighed value of the outer pad or the trailing shoe at the beginning and the end of the overall testing procedure – Subtract the value in column E from the value in column H | K |
| Mass Loss Disc / Drum | mg | [3] | Difference between the weighed value of the disc or the drum at the beginning and the end of the overall testing procedure – Subtract the value in column F from the value in column I | L |
| Mass Loss Total | mg | [3] | Total mass loss of the brake system during the overall testing procedure – Add the values in columns J, K, and L | M |
| Total Distance | km | [1] | Total distance covered during the entire brake emissions test including all sections | N |
| Mass Loss Rate Averaged | mg/km | [1] | Averaged mass loss rate of the brake system during the overall testing procedure – Divide the values in columns M/N | O |

Test Report File

The testing facility shall create a unique, complete, correctly timed, and traceable dataset as an input file for the generation of the test report. The test facility shall submit the report in a *.pdf or equivalent format. Use Table 13.6 for the all the information and values to include in the report. [TO BE FURTHER ELABORATED AND MAYBE AMENDED BASED ON THE GROUP’S FEEDBACK]

Table 13.6. 
Overall testing parameters to report after a brake particle emissions test

| Serial No. | Parameters and Inputs | Short description | Paragraph | Unit |
| --- | --- | --- | --- | --- |
| 1 | Vehicle make and model | Report vehicle make and model where the brake under testing is mounted | 8.1.1 | - |
| 2 | Axle (front or rear) | Report the axle position on the vehicle for the brake under testing | 8.1.1 | - |
| 3 | Vehicle test mass | Report vehicle mass to be simulated on the inertia brake dynamometer during the brake emissions test | 8.1.1 | kg |
| 4 | Brake orientation (mounting position in the vehicle) | Report the location of the brake under testing on the vehicle, right-hand corner or left-hand corner | 8.1.1 | - |
| 5 | Brake force distribution | Report the distribution of brake force between front and rear axle | 8.1.1 | % |
| 6 | Fixture style | Report the style of the support fixture of the brake assembly (L0-U or L0-P) | 8.4.1 | - |
| 7 | Part number for the disc or drum | Report the code labelled by the brake manufacturer on the disc/drum | 8.4.1 | # |
| 8 | Part number for the friction material | Report the code labelled by the friction manufacturer on the pads/shoes | 8.4.1 | # |
| 9 | Nominal wheel load | Calculate and report the nominal wheel load of the tested brake taking into account the vehicle parameters | 8.1.1 | kg |
| 10 | Test (or applied) Wheel Load | Calculate and report the test wheel load applied on the brake dynamometer – the value derives from the nominal wheel load | 8.1.1 | kg |
| 11 | Tyre dynamic rolling radius | Report the tyre dynamic rolling radius related to the brake under testing | 8.1.1 | mm |
| 12 | Brake effective radius | Report the effective radius of the brake under testing | 8.1.1 | mm |
| 13 | Brake nominal inertia | Calculate and report the nominal moment of inertia for the brake under testing, taking into account the vehicle characteristics | 8.1.1 | kg·m2 |
| 14 | Brake Test (or applied) inertia | Calculate and report the applied inertia during testing, including the road loads | 8.1.1 | kg·m2 |
| 15 | Disc/Drum outer diameter | Report the outer diameter of the brake under testing. Verify its suitability to fit in the brake enclosure | 7.4.3 | mm |
| 16 | Disc mass | Report the actual mass of the unused disc to allocate the brake to a nominal wheel load-to-disc mass group | 10.1.1, 10.1.2 | kg |
| 17 | Number of pistons per side | Report the number of pistons on one side of the brake calliper | 8.1.1 | # |
| 18 | Piston Mean (or hydraulic) Diameter | Report the diameter of the piston of the brake under testing | 8.1.1 | mm |
| 19 | Disc calliper bolt tightening torque (if applicable) | Report the bolt tightening suggested torque if specified by the brake manufacturer | 8.1.1 | N·m |
| 20 | Brake calliper or brake drum efficiency (if applicable) | Report the efficiency to account for friction losses, piston travel, etc. if specified by the brake manufacturer | 8.1.1 | % |
| 21 | Threshold pressure | Report the minimum pressure to overcome internal resistance before onset of brake torque | 8.1.1 | kPa |
| 22 | Lateral run out limit | Report the the maximum lateral movement allowed for the brake under testing along its rotating axis when installed on the brake fixture | 8.1.1 | µm |
| 23 | Climatic conditioning unit | Verify the compliance of the climatic conditioning unit with the mandatory specifications | 7.2 | Y/N |
| 24 | Measured average cooling air temperature – cooling adjustment section | Calculate and report the average cooling air temperature | 7.2.1.1 | °C |
| 25 | Measured average cooling air temperature – bedding section | Calculate and report the average cooling air temperature. Report the average cooling air temperature for all five WLTP-Brake cycles | 7.2.1.1 | °C |
| 26 | Measured average cooling air temperature – emissions measurement section | Calculate and report the average cooling air temperature | 7.2.1.1 | °C |
| 27 | Measured instantaneous cooling air temperature – cooling adjustment section | Calculate and report the number of the instantaneous cooling air temperature readings (1Hz) with a difference between 2-5 °C compared to the nominal value during the cooling adjustment procedure | 7.2.1.1 | # |
| 28 | Measured instantaneous cooling air temperature – bedding section | Calculate and report the number of the instantaneous cooling air temperature readings (1Hz) with a difference between 2-5 °C compared to the nominal value during the bedding procedure. Report the number of readings for all five repetitions of the WLTP-Brake cycle during bedding | 7.2.1.1 | # |
| 29 | Measured instantaneous cooling air temperature – emissions measurement section | Calculate and report the number of the instantaneous cooling air temperature readings (1Hz) with a difference between 2-5 °C compared to the nominal value during emissions testing | 7.2.1.1 | # |
| 30 | Cooling air temperature | Verify that all parts of the test are compliant for the average cooling air temperature | 7.2.1.1 | Y/N |
| 31 | Measured average cooling air relative humidity – cooling adjustment section | Calculate and report the average cooling air relative humidity | 7.2.1.2 | % |
| 32 | Measured average cooling air relative humidity – bedding section | Calculate and report the average cooling air relative humidity. Report the average cooling air relative humidity for all WLTP-Brake cycles | 7.2.1.2 | % |
| 33 | Measured average cooling air relative humidity – emissions measurement section | Calculate and report the average cooling air relative humidity | 7.2.1.2 | % |
| 34 | Measured instantaneous cooling air relative humidity – cooling adjustment section | Calculate and report the number of the instantaneous cooling air relative humidity readings (1 Hz) with a difference between 5 per cent and 30 per cent compared to the nominal value | 7.2.1.2 | # |
| 35 | Measured instantaneous cooling air relative humidity – bedding section | Calculate and report the number of the instantaneous cooling air relative humidity readings (1 Hz) with a difference between 5 per cent and 30 per cent compared to the nominal value. Report the number of readings for all five WLTP-Brake cycles | 7.2.1.2 | # |
| 36 | Measured instantaneous cooling air relative humidity – emissions measurement section | Calculate and report the number of the instantaneous cooling air relative humidity readings (1 Hz) with a difference between 5 per cent and 30 per cent compared to the nominal value | 7.2.1.2 | # |
| 37 | Cooling air relative humidity | Verify that all parts of the test are compliant for the average cooling air relative humidity | 7.2.1.2 | Y/N |
| 38 | Cooling air cleaning | Verify that the cooling air entering the system complies with the cleaning specifications defined in this GTR | 7.2.2.1 | Y/N |
| 39 | System background check | Calculate and report the TPN10 and SPN10 background concentration measured at the minimum operational airflow of the system used for brake emissions testing | 7.2.2.2.1 | #/Ncm3 |
| 40 | System background check | Calculate and report the TPN10 and SPN10 background concentration measured at the 50 per cent of the maximum operational airflow of the system used for brake emissions testing | 7.2.2.2.1 | #/Ncm3 |
| 41 | System background check | Calculate and report the TPN10 and SPN10 background concentration measured at the maximum operational airflow of the system used for brake emissions testing | 7.2.2.2.1 | #/Ncm3 |
| 42 | System background check | Verify that the background concentration at the system level is compliant with the maximum limit defined in this GTR | 7.2.2.2.3 | Y/N |
| 43 | Pre-test background check | Calculate and report the TPN10 and SPN10 background concentration measured during the pre-test check | 7.2.2.2.2 | #/Ncm3 |
| 44 | Post-test background check | Calculate and report the TPN10 and SPN10 background concentration measured during the post-test check | 7.2.2.2.2 | #/Ncm3 |
| 45 | Test background check | Verify that the background concentration for the given brake emissions test is compliant with the maximum limit defined in this GTR | 7.2.2.2.3 | Y/N |
| 46 | Test background check | Report the certified value of the dilution ratio applied during the test background check for both TPN10 and SPN10 | 7.2.2.2.2 | 1:X |
| 47 | Pre-test background check | Calculate and report the TPN10 and SPN10 background concentration measured during the pre-test check in # per km | 7.2.2.2.3 | #/km |
| 48 | Post-test background check | Calculate and report the TPN10 and SPN10 background concentration measured during the post-test check in # per km | 7.2.2.2.3 | #/km |
| 49 | Airflow measurement device | Verify the compliance of the airflow measurement element with the measurement accuracy defined in this GTR | 7.2.3 | Y/N |
| 50 | Airflow measurement device | Report the distance in duct diameters from the last flow disturbance to the airflow measurement element | 7.2.3 | X·di |
| 51 | Airflow measurement device | Report the distance in duct diameters from the airflow measurement element to the next flow disturbance | 7.2.3 | X·di |
| 52 | Airflow measurement device | Verify that the placement of the airflow measurement element in the ducy meets the requirements defined in this GTR | 7.2.3 | Y/N |
| 53 | Airflow measurement device | Verify the compliance of the temperature and pressure sensors of the airflow measurement element with the requirements defined in this GTR | 7.2.3 | Y/N |
| 54 | Definition of the nominal cooling airflow | Verify that the same nominal cooling airflow has been applied during all brake emissions test sections | 7.2.3 | Y/N |
| 55 | Measured average cooling airflow – cooling adjustment section | Calculate and report the average measured cooling airflow | 7.2.3 | m3/h |
| 56 | Measured average cooling airflow – cooling adjustment section | Calculate and report the per cent difference with the nominal cooling airflow | 7.2.3 | % |
| 57 | Measured average cooling airflow – bedding section | Calculate and report the average cooling airflow of each WLTP-Brake cycle. Report the difference for all five WLTP-Brake cycles | 7.2.3 | m3/h |
| 58 | Measured average cooling airflow – bedding section | Calculate and report the per cent difference with the nominal cooling airflow. Report the difference for all five WLTP-Brake cycles | 7.2.3 | % |
| 59 | Measured average cooling airflow – emissions measurement section | Calculate and report the average cooling airflow during the emissions measurement section. | 7.2.3 | m3/h |
| 60 | Measured average cooling airflow – emissions measurement section | Calculate and report the per cent difference with the nominal cooling airflow | 7.2.3 | % |
| 61 | Measured average cooling airflow | Verify that all trips and cycles of the test comply with the requirements set out in this GTR regarding the difference between the average nominal cooling airflow and the measured cooling airflow | 7.2.3 | Y/N |
| 62 | Deviations of the measured instantaneous cooling airflow – cooling adjustment section | Calculate and report the number of the cooling airflow readings (1 Hz) with a difference between 5 per cent and 10 per cent compared to the nominal value during the cooling adjustment section | 7.2.3 | # |
| 63 | Deviations of the measured instantaneous cooling airflow – bedding section | Calculate and report the number of the cooling airflow readings (1 Hz) with a difference between 5 per cent and 10 per cent compared to the nominal value during the bedding section. Report the number of readings for all five WLTP-Brake cycles | 7.2.3 | # |
| 64 | Deviations of the measured instantaneous cooling airflow – emissions measurement section | Calculate and report the number of the instantaneous cooling air flow readings (1 Hz) with a difference between 5-10per cent compared to the nominal value during the emissions measurement section | 7.2.3 | # |
| 65 | Instantaneous cooling airflow violations | Verify that all trips and cycles of the test comply with the maximum allowed number of the instantaneous cooling air flow readings (1 Hz) with a difference between 5 per cent and 10 per cent compared to the nominal value defined in this GTR | 7.2.3 | Y/N |
| 66 | Instantaneous cooling airflow violations | Verify that instantaneous cooling airflow readings (1 Hz) do not exceed a 10 per cent difference compared to the nominal value during all the test sections | 7.2.3 | Y/N |
| 67 | System leak check | Calculate and report the average measured airflow during the leak check | 7.2.3 | m3/h |
| 68 | System leak check | Verify that the average measured airflow during the leak check meets the requirements set out in this GTR | 7.2.3 | Y/N |
| 69 | Measured average normalized cooling airflow – emissions measurement section | Calculate and report the average normalized cooling airflow for the brake emissions measurement section | 7.2.3 | Nm3/h |
| 70 | Measured average cooling airspeed – emissions measurement section | Calculate and report the average cooling airspeed for the brake emissions measurement section | 7.2.3 | km/h |
| 71 | Brake inertia dynamometer and automation system | Verify that the mandatory specifications for the brake inertia dynamometer set out in this GTR are met | 7.3 | Y/N |
| 72 | Brake inertia dynamometer and automation system | Verify that the mandatory specifications for the automation, control, and data acquisition system set out in this GTR are met | 7.3 | Y/N |
| 73 | Brake enclosure design | Verify that the cooling air enters and exits the enclosure in the horizontal direction and that the tunnel ducting is horizontal for at least two duct diameters (2∙di) upstream of the enclosure’s inlet and downstream of the sampling plane | 7.4.2 | Y/N |
| 74 | Brake enclosure design | Report the transition angle of the inlet and outlet cross-sections | 7.4.2 | ° |
| 75 | Brake enclosure design | Verify that the transition angles of the inlet and outlet cross-sections meet the requirements of this GTR | 7.4.2 | Y/N |
| 76 | Brake enclosure design | Report the Reynolds number of the airflow at the entrance of the enclosure for the given brake emissions test | 7.4.2 | - |
| 77 | Brake enclosure design | Verify that the airflow at the cross-section area is turbulent at the testing conditions for the given brake emissions test | 7.4.2 | Y/N |
| 78 | Brake enclosure design | Verify that the airspeed at each position of the plane C used for the speed uniformity verification does not vary by more than ±20 per cent of the arithmetic mean of all measurements for the minimum operational airflow of the system used for brake emissions testing | 7.4.2 | Y/N |
| 79 | Brake enclosure design | Verify that the airspeed at each position of the plane C used for the speed uniformity verification does not vary by more than ±20 per cent of the arithmetic mean of all measurements at the 50 per cent of the maximum operational airflow of the system used for brake emissions testing | 7.4.2 | Y/N |
| 80 | Brake enclosure design | Verify that the airspeed at each position of the plane C used for the speed uniformity verification does not vary by more than ±20 per cent of the arithmetic mean of all measurements for the maximum operational airflow of the system used for brake emissions testing | 7.4.2 | Y/N |
| 81 | Brake enclosure design | Report the type of application used at the inlet’s side to ensure homogenous flow (flow straighteners, diffusion plates, etc.) – if applicable | 7.4.2 | - |
| 82 | Brake enclosure design | Specify if the speed uniformity verification was performed via CFD or experimental method | 7.4.2 | - |
| 83 | Brake enclosure dimensions | Verify that the brake enclosure has been designed symmetrically to the horizontal plane A1 | 7.4.3 | Y/N |
| 84 | Brake enclosure dimensions | Report the length of plane A1 | 7.4.3 | mm |
| 85 | Brake enclosure dimensions | Verify that the brake enclosure has been designed symmetrically to the vertical plane D | 7.4.3 | Y/N |
| 86 | Brake enclosure dimensions | Report the length of plane D (enclosure’s height) as defined in paragraph 7.2 | 7.4.3 | mm |
| 87 | Brake enclosure dimensions | Report the maximum axial depth of the enclosure at plane D | 7.4.3 | mm |
| 88 | Brake enclosure dimensions | Verify that the length, height, and depth of the enclosure meets the requirements defined in this GTR | 7.4.3 | Y/N |
| 89 | Brake enclosure dimensions | Report the inlet and outlet diameter (di) of the enclosure | 7.4.3 | mm |
| 90 | Brake enclosure dimensions | Verify that the inlet and outlet diameters (di) equal to the diameter of the duct in the sampling tunnel | 7.4.3 | Y/N |
| 91 | Brake enclosure dimensions | Report the inlet and outlet transition length (li) | 7.4.3 | mm |
| 92 | Brake enclosure dimensions | Report the inlet and outlet transition height (hB) | 7.4.3 | mm |
| 93 | Brake enclosure dimensions | Report the inlet’s height (hB) to enclosure’s height (hD) ratio | 7.4.3 | % |
| 94 | Brake enclosure dimensions | Verify that the hB/hD ratio meets the specifications defined in this GTR | 7.4.3 | Y/N |
| 95 | Design of the sampling tunnel | Report the inner diameter (di) of the duct in the sampling tunnel | 7.5 | mm |
| 96 | Design of the sampling tunnel | Report the distance of the sampling plane from the exit of the enclosure in duct diameters when a bend is not applied in the sampling tunnel | 7.5 | X·di |
| 97 | Design of the sampling tunnel | Report the distance of the sampling plane from the next flow disturbance in duct diameters when a bend is not applied in the sampling tunnel | 7.5 | X·di |
| 98 | Design of the sampling tunnel | Report if a bend is applied in the sampling tunnel downstream of the brake enclosure and upstream of the sampling plane | 7.5 | Y/N |
| 99 | Design of the sampling tunnel | Report the bending radius when a bend is applied in the sampling tunnel downstream of the brake enclosure and upstream of the sampling plane | 7.5 | X·di |
| 100 | Design of the sampling tunnel | Report the distance of the bend from the exit of the enclosure in duct diameters when a bend is applied in the sampling tunnel downstream of the brake enclosure and upstream of the sampling plane | 7.5 | X·di |
| 101 | Design of the sampling tunnel | Report the distance of the bend from the sampling plane in duct diameters when a bend is applied in the sampling tunnel downstream of the brake enclosure and upstream of the sampling plane | 7.5 | X·di |
| 102 | Design of the sampling tunnel | Verify that all specifications set out in this GTR for the design of the sampling tunnel are met | 7.5 | Y/N |
| 103 | Design of the sampling plane | Verify that PM and PN probes are installed in the same cross-section area in the sampling tunnel | 7.6 | Y/N |
| 104 | Design of the sampling plane | Report the number of probes used for the brake emissions test | 7.6 | # |
| 105 | Design of the sampling plane | Report the distance between the probes as specified in Figure 7.7 | 7.6 | mm |
| 106 | Design of the sampling plane | Verify that the specifications regarding placing and distance of the probes set out in this GTR are met | 7.6 | Y/N |
| 107 | Brake temperature measurement | Verify the compliance of the embedded thermocouple with the measurement range and the maximum permissible error specifications defined in paragraph 4.1 | 8.3 | Y/N |
| 108 | Brake temperature measurement | Verify the placement of the embedded thermocouple meets the specifications defined in this GTR | 8.3 | Y/N |
| 109 | Brake temperature measurement | Report whether brake pads or shoes temperature was also measured | 8.3 | Y/N |
| 110 | Brake installation | Verify that the brake was installed at the centre of the enclosure as specified in this GTR | 8.4.1 | Y/N |
| 111 | Fixture type | Verify that the brake fixture used to mount the brake under testing includes the minimum subsystems specified in this GTR | 8.4.1 | Y/N |
| 112 | Fixture type | Specify the type of support fixture used for the brake assembly | 8.4.1 | - |
| 113 | Fixture type | Verify that the type of support fixture used for the brake assembly meets the requirements specified in this GTR | 8.4.1 | Y/N |
| 114 | Brake rotation | Report the rotation direction of the brake disc or drum (CW or CCW) | 8.4.1 | - |
| 115 | Brake rotation | Verify that the tested brake disc or drum rotates in the direction of the evacuation | 8.4.1 | Y/N |
| 116 | Caliper orientation | Report the caliper orientation of the brake under testing | 8.4.2 | o’clock |
| 117 | Caliper orientation | Verify that the caliper orientation of the brake under testing meets the requirements specified in this GTR | 8.4.2 | Y/N |
| 118 | Initial temperature – cooling adjustment section | Report the initial brake temperature of the successful cooling adjustment iteration | 9.2.1 | °C |
| 119 | Initial temperature – bedding section | Report the initial brake temperature in all five WLTP-Brake cycle repetitions during the bedding section | 9.2.2 | °C |
| 120 | Initial temperature – emissions measurement section | Report the initial temperature of the brake during the WLTP-Brake cycle ten trips as defined in paragraph 1.2.3 | 9.2.3 | °C |
| 121 | Initial temperature | Verify that all trips from all sections comply with the criteria for the initial temperature of the brake defined in this GTR | 9.2.1, 9.2.2, 9.2.3 | Y/N |
| 122 | WLTP-Brake cycle interruptions | Verify that all sections of the brake emissions test were completed without any interruption | 9.3.1, 9.3.2, 9.3.3 | Y/N |
| 123 | WLTP-Brake cycle interruptions | Verify that all interruptions (if any) occurred during the brake emissions test were handled according to the specifications defined in this GTR | 9.3.1, 9.3.2, 9.3.3 | Y/N |
| 124 | WLTP-Brake cycle interruptions | Verify that the brake parts under testing were not disassembled at any point of the brake emissions test | 9.3.1, 9.3.2, 9.3.3 | Y/N |
| 125 | Speed violations – cooling adjustment section | Calculate and report the speed violations as specified in this GTR | 9.4.1 | % |
| 126 | Speed violations – bedding section | Calculate and report the speed violations as specified in this GTR | 9.4.1 | % |
| 127 | Speed violations – emissions measurement section | Calculate and report the speed violations as specified in this GTR | 9.4.1 | % |
| 128 | Speed violations | Verify that all trips and cycles of the test comply with the speed violations criteria | 9.4.1 | Y/N |
| 129 | Number of brake events count – emissions measurement section | Calculate and report the number of brake events | 9.4.2 | # |
| 130 | Number of brake events limit – emissions measurement section | Verify that the number of brake events complies with the criteria for the total count | 9.4.2 | Y/N |
| 131 | Kinetic energy dissipation – cooling adjustment section | Calculate and report the kinetic energy dissipation during the cooling adjustment section | 9.4.3 | J/kg |
| 132 | Kinetic energy dissipation – cooling adjustment section | Calculate and report the deviation from the nominal value during the cooling adjustment section | 9.4.3 | % |
| 133 | Kinetic energy dissipation – bedding section | Calculate and report the kinetic energy dissipation during the bedding section. Report the kinetic energy dissipation for all five WLTP-Brake cycles | 9.4.3 | J/kg |
| 134 | Kinetic energy dissipation – bedding section | Calculate and report the deviation from the nominal value during the bedding section. Report the deviation from the nominal value for all five WLTP-Brake cycles | 9.4.3 | % |
| 135 | Kinetic energy dissipation – emissions measurement section | Calculate and report the kinetic energy dissipation during the emissions measurement section | 9.4.3 | J/kg |
| 136 | Kinetic energy dissipation – emissions measurement section | Calculate and report the deviation from the nominal value during the emissions measurement section | 9.4.3 | % |
| 137 | Kinetic energy dissipation | Verify that all trips and cycles of the test comply with the kinetic energy dissipation criteria specified in this GTR | 9.4.3 | Y/N |
| 138 | Nominal wheel load/disc or drum mass ratio | Calculate and report the Nominal Wheel Load / Disc or Drum Mass ratio for the brake under testing | 10.1.1 | - |
| 139 | Brake group for Nominal Wheel Load / Disc or Drum Mass ratio | Report the group number where the brake under testing belongs according to its Nominal Wheel Load / Disc Mass ratio | 10.1.1 | - |
| 140 | Nominal (or set) cooling airflow obtained during the cooling adjustment section | Define and report the nominal (or set) cooling airflow for the given brake after completing the section | 10.1.2, 10.1.3 | m3/h |
| 141 | Average brake temperature over Trip #10 of the WLTP-Brake Cycle | Calculate and report the average brake temperature during the cooling adjustment section for the brake under testing | 10.1.3 | °C |
| 142 | Average brake temperature over Trip #10 of the WLTP-Brake Cycle | Calculate and report the difference between the average brake temperature to the target average brake temperature for the brake under testing | 10.1.3 | °C |
| 143 | Average IBT of selected brake events from Trip #10 of the WLTP-Brake Cycle | Calculate and report the average IBT of the selected brake events during the cooling adjustment section of the brake under testing | 10.1.1, 10.1.3 | °C |
| 144 | Average IBT of selected brake events from Trip #10 of the WLTP-Brake Cycle | Calculate and report the difference between the average IBT of the selected brake events to the target average brake temperature for the brake under testing | 10.1.1, 10.1.3 | °C |
| 145 | Average FBT of selected brake events from Trip #10 of the WLTP-Brake Cycle | Calculate and report the average FBT of the selected brake events during the cooling adjustment section of the brake under testing | 10.1.1, 10.1.3 | °C |
| 146 | Average FBT of selected brake events from Trip #10 of the WLTP-Brake Cycle | Calculate and report the difference between the average FBT of the selected brake events to the target average brake temperature for the brake under testing | 10.1.1, 10.1.3 | °C |
| 147 | Definition of the nominal (set) cooling airflow for the specific brake | Verify that all temperatures of the target parameters measured during the cooling adjustment procedure for the brake under testing are compliant with the target values defined in this GTR | 10.1.3 | Y/N |
| 148 | Bedding section | Verify that cooling airflow, testing wheel load, and applied inertia have been set as during the cooling air adjustment and emissions measurement section | 11.1, 11.2 | Y/N |
| 149 | Bedding section | Report the number of completed WLTP-Brake cycles carried out during the bedding section | 11.1, 11.2 | # |
| 150 | Bedding section | Verify that cooling sections have not been applied between Trips #1-#10 of each WLTP-Brake cycle during the bedding section | 11.1, 11.2 | Y/N |
| 151 | Bedding section | Verify that the brake parts have not been disassembled during the bedding section | 11.1, 11.2 | Y/N |
| 152 | Bedding section | Verify that the brake parts have not been disassembled after the bedding section and until emissions measurement was completed | 11.1, 11.2 | Y/N |
| 153 | Bedding section | In case of a failed bedding procedure as specified in paragraphs 11.1 and 11.2, verify that new brake parts have been used to start over the bedding section | 11.1, 11.2 | Y/N |
| 154 | PM sampling plane | Verify that the PM2.5 and PM10 sampling probes used for the brake under testing are different | 12.1.1.1 | Y/N |
| 155 | PM sampling plane | Verify that the PM2.5 and PM10 sampling probes used for the brake under testing are placed at the same horizontal plane to the lower part of the tunnel as specified in this GTR | 12.1.1.1 | Y/N |
| 156 | PM sampling plane | Verify that each sampling line for PM2.5 and PM10 used for the brake under testing does not apply flow splitting at any part from the probe’s inlet to the filters | 12.1.1.1 | Y/N |
| 157 | PM sampling probes | Report the PM2.5 and PM10 sampling probes inner diameters (dp) used for the brake under testing | 12.1.1.2 | mm |
| 158 | PM sampling probes | Report the PM2.5 and PM10 sampling probes overall length from the sampling nozzle tip to the inlet of the PM separation device used for the brake under testing | 12.1.1.2 | mm |
| 159 | PM sampling probes | Report if a bend is applied to the PM2.5 and/or PM10 sampling probes used for the brake under testing | 12.1.1.2 | Y/N |
| 160 | PM sampling probes | When a bend is applied to the PM2.5 and/or PM10 sampling probe report its bending radius in probe diameters | 12.1.1.2 | X·dp |
| 161 | PM sampling probes | Verify that the PM2.5 and PM10 sampling probes used for the brake under testing meet all the design requirements specified in this GTR | 12.1.1.2 | Y/N |
| 162 | PM sampling nozzles | Report the PM2.5 and PM10 sampling nozzles inner diameter (dn) used for the brake under testing | 12.1.1.3 | mm |
| 163 | PM sampling nozzles | Report the PM2.5 and PM10 sampling nozzles outer to inner diameter ratio at the nozzle tip used for the brake under testing | 12.1.1.3 | - |
| 164 | PM sampling nozzles | Report the PM2.5 and PM10 sampling nozzles aspiration angle applied for the brake under testing | 12.1.1.3 | ° |
| 165 | PM sampling nozzles | Verify that the PM2.5 and PM10 sampling nozzles used for the brake under testing meet all the design requirements specified in this GTR | 12.1.1.3 | Y/N |
| 166 | PM separation device | Report the PM2.5 cyclonic separator cut-off size used for the brake under testing | 12.1.2.1 | μm |
| 167 | PM separation device | Report the PM10 cyclonic separator cut-off size used for the brake under testing | 12.1.2.1 | μm |
| 168 | PM separation device | Verify that the PM2.5 and PM10 cyclonic separators used for the brake under testing fulfil all the specifications for the separation efficiency defined in this GTR | 12.1.2.1 | Y/N |
| 169 | PM separation device | Verify that the PM2.5 and PM10 cyclonic separators used for the brake under testing are mounted directly at the end of the corresponding PM2.5 and PM10 sampling probes | 12.1.2.1 | Y/N |
| 170 | PM separation device | Verify that the PM2.5 and PM10 cyclonic separators used for the brake under testing meet all the requirements specified in this GTR | 12.1.2.1 | Y/N |
| 171 | PM sampling line | Report the PM2.5 and PM10 sampling lines inner diameters (ds) used for the brake under testing | 12.1.2.2 | mm |
| 172 | PM sampling line | Report the PM2.5 and PM10 sampling lines overall length from the cyclone to the tip of the filter holder used for the brake under testing | 12.1.2.2 | mm |
| 173 | PM sampling line | Report if a bend is applied to the PM2.5 and/or PM10 sampling lines used for the brake under testing | 12.1.2.2 | Y/N |
| 174 | PM sampling line | When a bend is applied to the PM2.5 and/or PM10 sampling lines report its bending radius in sampling line diameters | 12.1.2.2 | X·ds |
| 175 | PM sampling line | Verify that the PM2.5 and PM10 sampling lines used for the brake under testing meet all the design requirements specified in this GTR | 12.1.2.2 | Y/N |
| 176 | PM sampling flow | Verify the compliance of the PM sampling flow measurement elements with the measurement accuracy defined in this GTR | 12.1.2.3 | Y/N |
| 177 | PM sampling flow | Verify the compliance of the temperature and pressure sensors of the PM sampling flow measurement elements with the requirements defined in this GTR | 12.1.2.3 | Y/N |
| 178 | PM sampling flow | Report the set (nominal) flow value for PM2.5 sampling for the brake under testing | 12.1.2.3 | l/min |
| 179 | PM sampling flow | Report the set (nominal) flow value for PM10 sampling for the brake under testing | 12.1.2.3 | l/min |
| 180 | PM sampling flow | Verify that the set (nominal) flow values for PM2.5 and PM10 sampling are the same and constant throughout the brake emissions test | 12.1.2.3 | Y/N |
| 181 | PM sampling flow | Report the average measured flow value for PM2.5 sampling for the brake under testing | 12.1.2.3 | l/min |
| 182 | PM sampling flow | Report the average measured flow value for PM10 sampling for the brake under testing | 12.1.2.3 | l/min |
| 183 | PM sampling flow | Verify that the average measured flow value for PM2.5 sampling for the brake under testing is within ±2 per cent of the nominal flow value | 12.1.2.3 | Y/N |
| 184 | PM sampling flow | Verify that the average measured flow value for PM10 sampling for the brake under testing is within ±2 per cent of the nominal flow value | 12.1.2.3 | Y/N |
| 185 | PM sampling flow | Report the average normalized measured flow value for PM2.5 sampling for the brake under testing | 12.1.2.3 | Nl/min |
| 186 | PM sampling flow | Report the average normalized measured flow value for PM10 sampling for the brake under testing | 12.1.2.3 | Nl/min |
| 187 | PM sampling flow | Report the average isokinetic ratio for PM2.5 sampling for the brake under testing | 12.1.2.3 | - |
| 188 | PM sampling flow | Report the average isokinetic ratio for PM10 sampling for the brake under testing | 12.1.2.3 | - |
| 189 | PM sampling flow | Verify that the average isokinetic ratio for PM2.5 sampling for the brake under testing meets the requirements defined in this GTR | 12.1.2.3 | Y/N |
| 190 | PM sampling flow | Verify that the average isokinetic ratio for PM10 sampling for the brake under testing meets the requirements defined in this GTR | 12.1.2.3 | Y/N |
| 191 | PM sampling flow | Verify that a leak check been carried out according to the requirements defined in this GTR | 12.1.2.3 | Y/N |
| 192 | PM filter holder | Verify that the PM2.5 filter holder meets all the requirements set out in this GTR | 12.1.3.1 | Y/N |
| 193 | PM filter holder | Verify that the PM2.5 filter holder meets all the requirements set out in this GTR | 12.1.3.1 | Y/N |
| 194 | PM sampling filters | Verify that the filter used for PM2.5 sampling for the brake under testing meets all the requirements set out in this GTR | 12.1.3.2 | Y/N |
| 195 | PM sampling filters | Verify that the filter used for PM10 sampling for the brake under testing meets all the requirements set out in this GTR | 12.1.3.2 | Y/N |
| 196 | Weighing procedure | Verify that pre-sampling conditioning and weighing of the PM2.5 and PM10 filters used for the brake under testing was carried out according to the specifications of this GTR | 12.1.4 | Y/N |
| 197 | Weighing procedure | Verify that post-sampling conditioning and weighing of the PM2.5 and PM10 filters used for the brake under testing was carried out according to the specifications of this GTR | 12.1.4 | Y/N |
| 198 | Weighing procedure | Report the weighing room’s average temperature during the measurement of the PM10 and PM2.5 filter weights | 12.1.4 | °C |
| 199 | Weighing procedure | Report the weighing room’s average relative humidity during the measurement of the PM10 and PM2.5 filter weights | 12.1.4 | % |
| 200 | Weighing procedure | Report the resolution of the weighing balance used for weighing the PM10 and PM2.5 filters for the brake under testing | 12.1.4 | μg |
| 201 | Weighing procedure | Verify that appropriate reference filters were used to validate PM weighings for the brake under testing | 12.1.4 | Y/N |
| 202 | Weighing procedure | Report the average of the two valid measurements for the PM2.5 filter for the brake under testing as specified in point (i) of the 12.1.4 | 12.1.4 | μg |
| 203 | Weighing procedure | Report the average of the two valid measurements for the PM10 filter for the brake under testing as specified in point (i) of the 12.1.4 | 12.1.4 | μg |
| 204 | Weighing procedure | Verify that all requirements set out in this GTR for the filters’ weighing for the brake under testing have been followed | 12.1.4 | Y/N |
| 205 | PM emission factor calculation | Report the PM2.5 emission factor in particle mass per distance driven for the brake under testing as specified in 12.1.5 of this GTR | 12.1.5 | mg/km |
| 206 | PM emission factor calculation | Report the PM10 emission factor in particle mass per distance driven for the brake under testing as specified in 12.1.5 of this GTR | 12.1.5 | mg/km |
| 207 | PN sampling plane | Verify that the TPN10 and SPN10 sampling probes used for the brake under testing are different | 12.2.1.1 | Y/N |
| 208 | PN sampling plane | When a single sampling probe is used for both TPN10 and SPN10 verify that the applied flow splitter meets all the requirements defined in 12.2.1.1 of this GTR | 12.2.1.1 | Y/N |
| 209 | PN sampling probes | Report the TPN10 and SPN10 sampling probe(s) inner diameters (dp) used for the brake under testing | 12.2.1.2 | mm |
| 210 | PN sampling probes | Report the TPN10 and SPN10 sampling probe(s) overall length from the sampling nozzle tip to the inlet of the particle transfer tube used for the brake under testing | 12.2.1.2 | mm |
| 211 | PN sampling probes | Report the residence time of the particles in the TPN10 and SPN10 sampling probe(s) for the brake under testing | 12.2.1.2 | s |
| 212 | PN sampling probes | Report if a bend is applied to TPN10 and SPN10 sampling probe(s) used for the brake under testing | 12.2.1.2 | Y/N |
| 213 | PN sampling probes | When a bend is applied to the TPN10 and SPN10 sampling probe(s) report its bending radius in probe diameters | 12.2.1.2 | X·dp |
| 214 | PN sampling probes | Verify that the TPN10 and SPN10 sampling probe(s) used for the brake under testing meet all the design requirements specified in this GTR | 12.2.1.2 | Y/N |
| 215 | PN sampling nozzles | Report the TPN10 and SPN10 sampling nozzles inner diameter (dn) used for the brake under testing | 12.2.1.3 | mm |
| 216 | PN sampling nozzles | Report the TPN10 and SPN10 sampling nozzles outer to inner diameter ratio at the nozzle tip used for the brake under testing | 12.2.1.3 | - |
| 217 | PN sampling nozzles | Report the TPN10 and SPN10 sampling nozzles aspiration angle applied for the brake under testing | 12.2.1.3 | ° |
| 218 | PN sampling nozzles | Verify that the TPN10 and SPN10 sampling nozzles used for the brake under testing meet all the design requirements specified in this GTR | 12.2.1.3 | Y/N |
| 219 | PN transfer tube | Report the TPN10 and SPN10 transfer tubes inner diameters (dtt) used for the brake under testing | 12.2.1.4 | mm |
| 220 | PN transfer tube | Report the TPN10 and SPN10 transfer tubes length to sample flow ratio for the brake under testing | 12.2.1.4 | s/m2 |
| 221 | PN transfer tube | Report if a bend is applied to the TPN10 and SPN10 transfer tubes used for the brake under testing | 12.2.1.4 | Y/N |
| 222 | PN transfer tube | When a bend is applied to the TPN10 and SPN10 transfer tubes report its bending radius in sampling transfer tube diameters | 12.2.1.4 | X·dtt |
| 223 | PN transfer tube | Report the residence time of the particles in the TPN10 and SPN10 transfer tubes for the brake under testing | 12.2.1.4 | s |
| 224 | PN transfer tube | Verify that the TPN10 and SPN10 transfer tubes used for the brake under testing meet all the design requirements specified in this GTR | 12.2.1.4 | Y/N |
| 225 | PN separation device | Report the PN cyclonic separator cut-off size used for the brake under testing | 12.2.2.1 | μm |
| 226 | PN separation device | Verify that the PN cyclonic separator used for the brake under testing fulfil all the specifications for the separation efficiency defined in this GTR | 12.2.2.1 | Y/N |
| 227 | PN separation device | Verify that the PN cyclonic separator used for the brake under testing meet all the requirements specified in this GTR | 12.2.2.1 | Y/N |
| 228 | PN sample conditioning | Report the average PCRF applied for the TPN10 sampling and measurement for the brake under testing | 12.2.2.2 | - |
| 229 | PN sample conditioning | Report the average PCRF applied for the SPN10 sampling and measurement for the brake under testing | 12.2.2.2 | - |
| 230 | PN sample conditioning | Verify that the dilution system applied for the TPN10 sampling and measurement for the brake under testing meets all the requirements defined in 12.2.2.2 of this GTR | 12.2.2.2 | Y/N |
| 231 | PN sample conditioning | Verify that the volatile particle removal system applied for the SPN10 sampling and measurement for the brake under testing meets all the requirements defined in 12.2.2.2 of this GTR | 12.2.2.2 | Y/N |
| 232 | PN internal transfer line | Report the TPN10 and SPN10 internal transfer line inner diameters (dtl) used for the brake under testing | 12.2.2.3 | mm |
| 233 | PN internal transfer line | Report the TPN10 and SPN10 internal transfer line length from the exit of the dilution system or the VPR to the inlet of the PNC for the brake under testing | 12.2.2.3 | mm |
| 234 | PN internal transfer line | Report if a bend is applied to the TPN10 and SPN10 internal transfer line used for the brake under testing | 12.2.2.3 | Y/N |
| 235 | PN internal transfer line | When a bend is applied to the TPN10 and SPN10 internal transfer line report its bending radius in transfer line diameters | 12.2.2.3 | X·dtl |
| 236 | PN internal transfer line | Report the residence time of the particles in the TPN10 and SPN10 internal transfer lines for the brake under testing | 12.2.2.3 | s |
| 237 | PN internal transfer line | Verify that the TPN10 and SPN10 internal transfer lines used for the brake under testing meet all the design requirements specified in this GTR | 12.2.2.3 | Y/N |
| 238 | Particle number counter | Verify that the particle number counter used for the measurement of TPN10 and SPN10 for the brake under testing meets all the requirements specified in 12.2.3.1 in this GTR | 12.2.3.1 | Y/N |
| 239 | PN sampling flow | Verify the compliance of the PN sampling flow measurement elements with the measurement accuracy defined in this GTR | 12.2.3.2 | Y/N |
| 240 | PN sampling flow | Verify the compliance of the temperature and pressure sensors of the PN sampling flow measurement elements with the requirements defined in this GTR | 12.2.3.2 | Y/N |
| 241 | PN sampling flow | Report the average normalized PN sampling flow value for TPN10 for the brake under testing | 12.2.3.2 | Nl/min |
| 242 | PN sampling flow | Report the average normalized PN sampling flow value for SPN10 for the brake under testing | 12.2.3.2 | Nl/min |
| 243 | PN sampling flow | Verify that the actual normalized PN sampling flow value for TPN10 for the brake under testing is within ±10 per cent of the average normalized PN sampling flow | 12.2.3.2 | Y/N |
| 244 | PN sampling flow | Verify that the actual normalized PN sampling flow value for SPN10 for the brake under testing is within ±10 per cent of the average normalized PN sampling flow | 12.2.3.2 | Y/N |
| 245 | PN sampling flow | Report the average isokinetic ratio for TPN10 sampling for the brake under testing | 12.2.3.2 | - |
| 246 | PN sampling flow | Report the average isokinetic ratio for SPN10 sampling for the brake under testing | 12.2.3.2 | - |
| 247 | PN sampling flow | Verify that the average isokinetic ratio for TPN10 sampling for the brake under testing meets the requirements defined in this GTR | 12.2.3.2 | Y/N |
| 248 | PN sampling flow | Verify that the average isokinetic ratio for SPN10 sampling for the brake under testing meets the requirements defined in this GTR | 12.2.3.2 | Y/N |
| 249 | PN emission factor calculation | Report the TPN10 emission factor in a number of particle per distance driven for the brake under testing as specified in 12.2.3.3 of this GTR | 12.2.4 | #/km |
| 250 | PN emission factor calculation | Report the SPN10 emission factor in a number of particles per distance driven for the brake under testing as specified in 12.2.3.3 of this GTR | 12.2.4 | #/km |
| 251 | PN system verification procedures | Verify that the PN system verification procedures defined in paragraph 12.2.5 of this GTR have been applied for the brake under testing | 12.2.5 | Y/N |
| 252 | Mass loss measurement | Report the overall mass loss of the brake under testing following the procedure defined in point (j) in paragraph 12.3 of this GTR | 12.3 | mg |
| 253 | Mass loss measurement | Report the averaged weight loss emission factor of the brake under testing following the procedure defined in point (k) in paragraph 12.3 of this GTR | 12.3 | mg/km |
| 254 | Mass loss measurement | Verify that the mass loss measurement of the brake under testing has been conducted following all the specifications described in this protocol | 12.3 | Y/N |
| 255 | Calibration requirements | Verify that the calibration requirements defined in this protocol for the brake inertia dynamometer are met | 14.2 | Y/N |
| 256 | Calibration requirements | Verify that the calibration requirements defined in this protocol for the cooling airflow measurement device are met | 14.3 | Y/N |
| 257 | Calibration requirements | Verify that the calibration requirements defined in this protocol for the PM and PN cyclonic separators are met | 14.1 | Y/N |
| 258 | Calibration requirements | Verify that the calibration requirements defined in this protocol for the microgram balance are met | 14.4 | Y/N |
| 259 | Calibration requirements | Verify that the calibration requirements defined in this protocol for the PM sampling flow measurement device are met | 14.1 | Y/N |
| 260 | Calibration requirements | Verify that the calibration requirements defined in this protocol for the PM sampling flow measurement device are met | 14.1 | Y/N |
| 261 | Calibration requirements | Verify that the calibration requirements defined in this protocol for the dilution system and the volatile particle remover are met | 14.5 | Y/N |
| 262 | Calibration requirements | Verify that the calibration requirements defined in this protocol for the particle number counter are met | 14.6 | Y/N |

Calibration Requirements and Ongoing Quality Controls

General Calibration Requirements

This paragraph summarizes the minimum calibration requirements for the equipment used for brake emissions testing. Table 14.1 summarizes the calibration criteria and the intervals for the main equipment defined in this GTR.

Table 14.1
Calibration requirements for main equipment for emissions measurements

| Instrument | Interval | Criterion | Paragraph |
| --- | --- | --- | --- |
| Brake inertia dynamometer | Upon initial installation, yearly, and at major setup maintenance | Table 14.3 | Paragraph 14.2 |
| Torque measurement device | Upon initial installation, yearly, and at major setup maintenance | Table 14.4 | Paragraph 14.2 |
| Cooling airflow measurement device | Upon initial installation, yearly, and at major setup maintenance | Table 14.5 | Paragraph 14.3 |
| Cooling airflow temperature sensor | Yearly | ±1°C | Paragraph 14.3 |
| Cooling airflow atmospheric pressure sensor | Yearly | ±0.4 kPa | Paragraph 14.3 |
| Cooling air temperature sensor | Yearly | ±1°C | Paragraph 7.2.1 |
| Cooling air relative humidity sensor | Yearly | ±5 per cent of nominal | Paragraph 7.2.1 |
| PM10 Cyclonic separator | Upon initial installation | Table 12.1 | Paragraph 12.1 |
| PM2.5 Cyclonic separator | Upon initial installation | Table 12.2 | Paragraph 12.1 |
| Microgram balance | Upon initial installation, yearly, and at major setup maintenance | Table 14.6 | Paragraph 14.4 |
| PM sampling flow measurement device | Upon initial installation, yearly, and at major setup maintenance | ±2.5 per cent of reading or ±1.5 per cent of full scale (whichever is the least) | Paragraph 12.1 |
| PM sampling flow temperature sensor | Yearly | ±1°C | Paragraph 12.1 |
| PM sampling flow pressure sensor | Yearly | ±1 kPa | Paragraph 12.1 |
| PN Cyclonic separator | Upon initial installation | Penetration efficiency of ≥80 per cent for a particle electrical mobility diameter of 1.5 µm | Paragraph 12.2 |
| PN sampling flow measurement device | 13 months | ±5 per cent of reading under all operating conditions | Paragraph 12.2 |
| PN sampling flow temperature sensor | Yearly | ±1°C | Paragraph 12.2 |
| PN sampling flow pressure sensor | Yearly | ±1 kPa | Paragraph 12.2 |
| Dilution system for TPN10 | 6 months or 13 months depending on the setup | As per paragraph 14.5.1 | Paragraph 14.5 |
| Volatile Particle Remover for SPN10 | 6 months or 13 months depending on the setup | As per paragraph 14.5.2 | Paragraph 14.5 |
| Particle Number Counter | 13 months and at major maintenance | As per paragraph 14.6 | Paragraph 14.6 |

Any other sensor or auxiliary equipment used to determine e.g. temperature, atmospheric pressure, and ambient humidity of the facilities room or the balance room shall fulfil the requirements prescribed in Table 14.2.

Table 14.2
Calibration requirements for auxiliary equipment

| Instrument | Interval | Criterion |
| --- | --- | --- |
| Temperature sensor | Yearly | ±1°C |
| Atmospheric pressure sensor | Yearly | ±1 kPa |
| Relative humidity sensor | Yearly | ±5 per cent of nominal |
| Absolute humidity sensor | Yearly | ±10 per cent of reading or 1 gH2O/kg dry air (whichever is larger) |

Brake Inertia Dynamometer

Table 14.3 summarizes the calibration criteria and the intervals for the brake dynamometer defined in this GTR. The rotational speed, brake torque, and brake pressure measurement devices shall comply with the linearity requirements of Table 14.4.

Table 14.3
Calibration requirements for the brake dynamometer

| Instrument | Interval | Criterion |
| --- | --- | --- |
| Rotational speed device | Upon initial installation, yearly, and at major setup maintenance | Table 14.4 |
| Brake torque sensor | Upon initial installation, yearly, and at major setup maintenance | Table 14.4 |
| Brake pressure sensor | Upon initial installation, yearly, and at major setup maintenance | Table 14.4 |
| Brake fluid displacement (if applicable) | Upon initial installation, yearly, and at major setup maintenance | ±0.5 per cent maximum |
| Temperature data acquisition | Upon initial installation, yearly, and at major setup maintenance | ±0.25 per cent maximum |

Table 14.4
Linearity requirements of rotational speed, brake torque, and brake pressure measurement devices

| Measurement system | Intercept a0 | Slope a1 | Standard error of estimate (SEE) | Coefficient of determination r² |
| --- | --- | --- | --- | --- |
| Brake Rotational Speed | ≤ 0.05 per cent maximum | 0.98 – 1.02 | ≤ 0.25 per cent maximum | ≥ 0.990 |
| Brake Torque | ≤ 0.05 per cent maximum | 0.98 – 1.02 | ≤ 0.5 per cent maximum | ≥ 0.990 |
| Brake pressure | ≤ 0.05 per cent maximum | 0.98 – 1.02 | ≤ 0.5 per cent maximum | ≥ 0.990 |

Apart from the calibrations of the systems listed in Tables 14.3 and 14.4, the testing facility shall verify the torque zero level and pressure zero level every time before commencing a brake emissions test.

Cooling Airflow Measurement Device

The calibration of the flow measurement device used for the determination of the cooling airflow shall be traceable to national or international standards. The flow measurement device shall comply with the linearity requirements of Table 14.5 with at least four equally spaced reference flows applying a linear regression between the minimum and maximum applicable flowrate of the setup. In addition, each flow measurement point shall be within ±2 per cent of the measured reference flow. The testing facility shall perform the calibration of the airflow measurement device upon the initial installation, annually, and at every major maintenance of the setup.

Table 14.5
Linearity requirements of the flow measurement device

| Measurement system | Intercept a0 | Slope a1 | Standard error of estimate (SEE) | Coefficient of determination r² |
| --- | --- | --- | --- | --- |
| Flow meter | ≤ 1 per cent maximum | 0.98 – 1.02 | ≤ 2 per cent maximum | ≥ 0.990 |

The testing facility shall use a flow measurement device calibrated to report airflow at both operating and standard conditions (273.15 K and 101.325 kPa). The temperature sensor shall have an accuracy of ±1 °C. The pressure measurements shall have precision and accuracy of ±0.4 kPa. The testing facility shall carry out the calibration of both sensors annually.

Microgram Balance

The calibration of the microgram balance used for particle filter weighing according to paragraph 12.1.4 shall be traceable to national or international standards. The balance shall comply with the linearity requirements of Table 14.6 with at least four equally spaced reference weights applying linear regression. This implies a precision of at least ±2 μg and a resolution of at least 1 μg (1 digit = 1 μg). The testing facility shall use certified calibration weights to verify the stability and the proper function of the microbalance regularly. The testing facility shall perform the calibration of the microgram balance upon the initial installation, annually, and at every major maintenance of the setup.

Table 14.6
Verification criteria for microgram balance

| Measurement system | Intercept a0 | Slope a1 | Standard error of estimate (SEE) | Coefficient of determination r² |
| --- | --- | --- | --- | --- |
| PM balance | ≤ 1 μg | 0.99 – 1.01 | ≤ 1 per cent maximum | ≥ 0.998 |

Sample Treatment and Conditioning Devices

Dilution System for TPN10 Measurement

The calibration of the dilution system’s PCRF across its full range of dilution settings shall be carried out at the instrument’s fixed nominal operating temperatures when the unit is new and following any major maintenance. The periodic validation requirement for the dilution system’s PCRF shall be limited to a check at a single setting typical of that used for emissions testing of any typical brake available in the market. The Technical Service shall ensure the existence of a calibration or validation certificate within a 6-month period prior to the emissions test. A 13-month validation interval shall be permissible when the dilution system incorporates temperature monitoring alarms.

The dilution system shall be characterized for PCRF with thermally stable particles at the operating conditions of the system of 15 nm, 30 nm, 50 nm, and 100 nm electrical mobility diameters. PCRFs (fr (dx)) for particles of 15 nm, 30 nm, and 50 nm electrical mobility diameters shall be no more than 100 per cent, 30 per cent, and 20 per cent higher respectively, and no more than 5 per cent lower than that for particles of 100 nm electrical mobility diameter. For validation, the mean PCRF shall be within ±10 per cent of the arithmetic mean particle concentration reduction factor (fr) determined during the primary calibration of the dilution system.

The test aerosol for these measurements shall be thermally stable particles at the operating conditions of the system of 15, 30, 50, and 100 nm electrical mobility diameters. The minimum concentration at the dilution system inlet shall be 3000 #/cm-³ for particles of 15 nm electrical mobility diameter and 5000 #/cm-³ for particles of 30, 50, and 100 nm electrical mobility diameters. Particle concentrations shall be measured upstream and downstream of the components. The PCRF at each particle size (fr (dx)) shall be calculated following equation 14.1. Nin (dx) and Nout (dx) shall refer to the same conditions (i.e. standard conditions):

| fr(dx)=Nin(dx)Nout(dx) | (Eq. 14.1) |
| --- | --- |

Where:

Nin(dx)		is the upstream PN concentration for particles of electrical mobility diameter dx;

Nout(dx)	is the downstream PN concentration for particles of electrical mobility diameter 			dx.

The arithmetic average particle concentration reduction (fr) at a given dilution setting shall be calculated using equation 14.2:

| fr=fr30 nm+fr50 nm+fr(100 nm)3 | (Eq. 14.2) |
| --- | --- |

Where:

fr(30 nm)	is the PCRF for particles of 30 nm electrical mobility diameter;

fr(50 nm)	is the PCRF for particles of 50 nm electrical mobility diameter;

fr(100 nm)	is the PCRF for particles of 100 nm electrical mobility diameter.

The instrument manufacturer shall prove the particle penetration Pr(dx) by testing one unit for each system model. A system model here covers all systems with the same hardware, i.e. same geometry, conduit materials, flows, and temperature profiles in the aerosol path. The particle penetration Pr(dx) at a particle size, dx, shall be calculated using equation 14.3. DF is the dilution factor between measurement positions of Nin (dx) and Nout (dx) determined either with trace gases or flow measurements.

| Pr(dx)=DF×Nout(dx)Nin(dx) | (Eq. 14.3) |
| --- | --- |

Volatile Particle Removal for SPN10 Measurement

The calibration of the VPR’s PCRF across its full range of dilution settings shall be carried out at the instrument’s fixed nominal operating temperatures when the unit is new and following any major maintenance. The periodic validation requirement for the VPR’s PCRF shall be limited to a check at a single setting typical of that used for emissions testing of any typical brake available in the market. The Technical Service shall ensure the existence of a calibration or validation certificate within a 6-month period prior to the emissions test. A 13-month validation interval shall be permissible when the VPR incorporates temperature monitoring alarms.

The VPR shall be characterized for PCRF with solid particles of 15 nm, 30 nm, 50 nm, and 100 nm electrical mobility diameters. PCRFs for particles of 15 nm, 30 nm, and 50 nm electrical mobility diameters shall be no more than 100 per cent, 30 per cent, and 20 per cent higher respectively, and no more than 5 per cent lower than that for particles of 100 nm electrical mobility diameter. For validation, the mean PCRF shall be within ±10 per cent of the arithmetic mean particle concentration reduction factor (fr) determined during the last calibration of the VPR.

The test aerosol for these measurements shall be solid particles of 15 nm, 30 nm, 50 nm, and 100 nm electrical mobility diameters. The minimum concentration at the dilution system inlet shall be 3000 #/cm-³ for particles of 15 nm electrical mobility diameter and 5000 #/cm-³ for particles of 30 nm, 50 nm, and 100 nm electrical mobility diameters. Particle concentrations shall be measured upstream and downstream of the components. The PCRF at each particle size (fr (dx)) shall be calculated following equation 14.1. The arithmetic average particle concentration reduction (fr) at a given dilution setting shall be calculated using equation 14.2

It is recommended that the VPR is calibrated and validated as a complete unit. The volatile particle removal efficiency of a VPR needs to be proven only once for the instrument family measuring SPN10. The instrument manufacturer must provide the maintenance or replacement interval that ensures that the removal efficiency of the VPR does not drop below the technical requirements. If such information is not provided, the volatile removal efficiency has to be checked yearly for each instrument.

The VPR used for SPN10 measurements shall demonstrate greater than 99.9 per cent removal efficiency of Tetracontane (CH3(CH2)38CH3) particles with count median electrical mobility diameter > 50 nm and mass > 1 mg/m3 when operated at its minimum dilution setting and the manufacturers recommended operating temperature.

The instrument manufacturer shall prove the particle penetration Pr(dx) by testing one unit for each system model. A system model here covers all systems with the same hardware, i.e. same geometry, conduit materials, flows, and temperature profiles in the aerosol path. The particle penetration Pr(dx) at a particle size, dx, shall be calculated using equation 14.3.

Particle Number Counter

The responsible authority shall ensure the existence of a calibration certificate for the PNC demonstrating compliance with a traceable standard within a 13-month period prior to the emissions test. Between calibrations, either the counting efficiency of the PNC shall be monitored for deterioration, or the PNC wick shall be routinely changed every 6 months if recommended by the instrument manufacturer. The PNC shall also be recalibrated and a new calibration certificate issued following any major maintenance.

Calibration shall be traceable to a standard calibration method. The testing facility shall use one of the two following methods for the calibration of the PNC:

- Comparison of the response of the PNC under calibration with that of a calibrated aerosol electrometer when simultaneously sampling electrostatically classified calibration particles;

Comparison of the response of the PNC under calibration with that of a second PNC that has been directly calibrated by the above method.

The calibration shall be undertaken using at least six standard concentrations across the PNC’s measurement range. Five of these standard concentrations shall be as uniformly spaced as possible between the standard concentration of 3000 #/cm-³ or below and the maximum of the PNC’s range in single-particle count mode. The six standard concentration points shall include a nominal zero concentration point produced by attaching HEPA filters of at least Class H13 of EN 1822:2008 (or equivalent performance) to the inlet of each instrument. The gradient from a linear least-squares regression of the two data sets shall be calculated and recorded. A calibration factor equal to the reciprocal of the gradient shall be applied to the PNC under calibration. Linearity of response is calculated as the square of the Pearson product moment correlation coefficient (r) of the two data sets and shall be equal to or greater than 0.97. In calculating both the gradient and R2, the linear regression shall be forced through the origin (zero concentration on both instruments). The calibration factor shall be between 0.9 and 1.1. Each concentration measured with the PNC under calibration shall be within ±5 per cent of the measured reference concentration multiplied with the gradient, with the exception of the zero point.

The calibration shall also include a check against the requirements on the PNC’s detection efficiency with particles of 10 nm electrical mobility diameter. A check of the counting efficiency with 15 nm particles is not required during periodical calibration.

A separate excel file with i. The 1Hz speed trace for the entire cycle; ii. The 1Hz speed trace for Trip #10; iii. Information about the brake events.