|  |  | ECE/TRANS/180/Add.22 |
| --- | --- | --- |

|  |  | xx xx xxxx |
| --- | --- | --- |

Global Registry

Created on 18 November 2004, pursuant to Article 6 of the Agreement concerning the establishing of global technical regulations for wheeled vehicles, equipment and parts which can be fitted and/or be used on wheeled vehicles (ECE/TRANS/132 and Corr.1) done at Geneva on 25 June 1998

Addendum 22: United Nations Global Technical Regulation No. 22

United Nations Global Technical Regulation on In-vehicle Battery Durability for Electrified Heavy Duty  Vehicles

Established in the Global Registry on xxxxx

UNITED NATIONS

UN Global Technical Regulation on In-vehicle Battery Durability for Electrified Vehicles

Contents

Page

I.	Statement of technical rationale and justification		3

II.	Text of the GTR		13

1.	Purpose			13

2.	Scope and application		13

3.	Definitions		13

4.	Abbreviations		15

5.	Requirements		15

5.1.	State-of-Certified Range and State-of Certified Energy (SOCR and SOCE) monitors		15

5.2.	Battery Performance Requirements		15

6.	In-Use Verification		17

6.1.	Definitions of Families		17

6.2.	Information gathering		18

6.3.	Part A: Verification of SOCR/SOCE monitors		18

6.4.	Part B: Verification of Battery Durability		20

6.5.	Process flow charts for Part A and Part B		21

7.	Rounding		22

Annexes

1		Vehicle survey		23

2 		Values to be read from vehicles….……………………………….		27

3		Determination of Performance Parameter during Part A Test Procedure….…………………………….	28

I.	Statement of technical rationale and justification

A.	Introduction

1.	Owing to the pressing need to reduce emissions of greenhouse gases (GHG) and other air pollutants, the market share of electrified vehicles is expected to grow in the future. A key component of these vehicles is the traction battery that is used to store and deliver energy to power the movement of the vehicle and the systems within it. Improvements in the performance of batteries to deliver increased driving range, reduced charging times and greater affordability are a significant focus for manufacturers and technological developments in this area are expected to accelerate the uptake of electrified vehicles by consumers.

2.	Despite the expected improvements in the performance of new electrified vehicles, the continued in-use performance of the battery over time is not currently regulated. The primary motivation for this GTR therefore stems from the recognition that the environmental performance of electrified vehicles may be affected by excess degradation of the battery system over time.

3.	Loss of electric range and loss of vehicle energy efficiency are both primary concerns. Loss of electric range could lead to a loss of utility, meaning electric vehicles are driven less and therefore displace less distance travelled that might otherwise be driven in conventional vehicles. A loss in utility could also dampen consumer sentiment and curb the market growth necessary for electric vehicle sales to deliver on fleet emissions reductions. Loss of vehicle efficiency could impact the upstream emissions by increasing the amount of electricity needed per vehicle distance travelled. Both can affect not only the utility of the vehicle to the consumer, but also the environmental performance of the vehicle. Loss of environmental performance is important, in particular because governmental regulatory compliance programs often credit electrified vehicles with a certain level of expected environmental benefit, which might not be realized over the life of the vehicle if excess battery degradation occurs.

4.	In addition to changes in range and energy consumption, hybrid electric vehicles are often equipped with both a conventional and electric powertrain, and for these vehicles the criteria pollutant emissions from the conventional powertrain could be impacted by the degradation of the battery over time.

5.	This GTR therefore aims to provide a harmonised methodology to address these concerns by introducing a method by which the health of the battery can be monitored over time and by setting minimum performance requirements for the durability of the battery.

B.	Procedural background

6.	The Informal Working Group (IWG) on Electric Vehicle and the Environment (EVE) was set up in June 2012 following the approval by WP.29/AC.3 of ECE/TRANS/WP.29/AC.3/32. This document established two distinct IWGs to examine environmental and safety issues related to Electric Vehicles (EVs): the IWG on EVE, reporting to the Working Party on Pollution and Energy (GRPE), and the IWG on Electric Vehicle Safety (EVS), reporting to the Working Party on Passive Safety (GRSP). The proposal was supported by the European Commission, the United States of America, China, and Japan.

7.	A second mandate for the IWG on EVE, divided into Parts A and B was approved in November 2014 by AC.3 to conduct additional research to address several recommendations that grew out of the first mandate, and develop UN GTR(s), if appropriate. The second mandate was separate from the IWG on EVS.

8.	Part A of the second mandate of the IWG on EVE (ECE/TRANS/WP.29/AC.3/40) included “battery performance and durability” as one of the topics authorized for study and potential GTR development. Specifically, Part A authorized activity “to further develop the recommendations for future work outlined in the Electric Vehicle Regulatory Reference Guide by: (i) conducting additional research to support the recommendations; (ii) identifying which recommendations are suitable for the development of (a) global technical regulation(s) (GTR(s)) by the World Forum for Harmonization of Vehicle Regulations (WP.29); and (iii) developing a work plan. The work of the IWG on EVE on battery performance and durability under Part A of the EVE mandate was reported to WP.29 in a status report as informal document WP.29-170-31 at the 170th meeting of WP.29, 15-18 November 2016.

9.	At the close of Part A the IWG on EVE recommended that GRPE and WP.29 endorse the option of extending the mandate of the IWG on EVE to continue active research into the topic of battery performance and durability without committing to the development of a GTR at that time. This was endorsed and work continued on this topic within Part B of the mandate.

10.	The IWG on EVE presented a draft status report to GRPE in May 2019 on the research on in-vehicle battery durability and performance. The status report indicated that there was sufficient information to allow a UN GTR for in-vehicle battery durability to be started. The IWG on EVE recommended at the 79th GRPE in May 2019 that the UN GTR on in-vehicle battery durability be developed under a new mandate.

11.	AC.3 subsequently approved document ECE/TRANS/WP.29/AC.3/57 authorizing the IWG on EVE to develop a new UN GTR on in-vehicle battery durability which will be developed in 2 phases:

Phase 1:

(a)	Deliver a first version of a UN GTR on in-vehicle battery durability to AC.3 by November 2021 with;

(i)	Definition of and requirements for electrified vehicle battery performance criteria

(ii) 	Requirements for reading and/or displaying battery health information and usage data from the vehicle; and

(iii)	 A provisional in-service conformity test which will include generic usage criteria and a statistical method.

(iv)	A certain certification reference procedure for HDV, to be able to evaluate compliance against

Phase 2:

(b)	Develop a second version of the UN GTR on in-vehicle battery durability with the following:

(i)	The development of a methodology to define Normal Usage Indices (NUI) based on data read from vehicles

(ii)	Refined performance criteria requirements for in-vehicle battery durability through assessment of further modelling and data collected from real vehicles and the use of NUIs

C.	Technical background

Battery degradation in electrified vehicles

12.	The effect of battery degradation on environmental performance is likely to differ significantly among the various electrified vehicle architectures (PEV, OVC-HEV and NOVC-HEV). The primary forms of battery degradation are capacity degradation and power degradation. Capacity degradation is the loss of energy capacity, resulting in a loss of electric driving range (for PEVs and OVC-HEVs) and possibly increased use of the engine during hybrid operation (for NOVC-HEVs). Power degradation is the loss of battery power, which can also lead to increased use of the engine for OVC-HEV and NOVC-HEVs and possibly reduced performance of the vehicle overall.

13.	There are at least six major vehicle operating parameters that affect in-vehicle battery durability. Each differs in importance depending on electrified vehicle architecture:

(a)	Discharge rates, as determined by vehicle duty cycle and operator use including, but not limited to, vehicle speed, auxiliary loads, towing, payload and ambient conditions;

(b)	Charge rates, as determined by type (normal, fast, super-fast) and frequency of charging;

(c)	State of charge (SOC) window used in system operation of the battery and the amount typically used between charge events (depth of discharge);

(d)	Battery temperature during operation (operation includes all temperature exposures from vehicle purchase through retirement, both while being operated and during periods of charging and inactivity);

(e)	Time (calendar life);

(f)	Other uses not reflected in calendar life or distance travelled, such as Vehicle to Grid (V2G), or any PTO/auxiliary operations during stand-still.

14.	The extent and nature of battery degradation that will occur is a result of complex mechanisms and heavily dependent on the battery cell chemistry and operating conditions. A variety of physical and electrochemical processes influence the durability of battery cells and these have been documented comprehensively within a literature review commissioned by the IWG on EVE. For typical lithium-ion batteries the primary mechanisms leading to capacity degradation were summarised as:

(a)	Loss or deposition of cyclable lithium or a loss of balance between electrodes;

(b)	Loss of electrode area; and

(c)	Loss of electrode material or conductivity.

15.	These aging processes are often further complicated by the fact that many of the mechanisms are associated with a rise in cell impedance, leading to a reduction in maximum cell power.

Management of battery degradation

16.	Whilst manufacturers have found it possible to establish the durability of specific battery implementations sufficiently to bring the products to market with some degree of confidence that normal provisions for customer satisfaction and warranty terms are being met, not every manufacturer is establishing durability in the same way. Manufacturers employ a wide variety of testing regimes often tailored to specific product configurations, applications, customer groups, and geographic considerations.

17.	To reduce the effect of capacity degradation on range, manufacturers may choose to slightly oversize a PEV or OVC-HEV battery to allow the range to be maintained by widening of the state-of-charge (SOC) window to make more capacity available as capacity degrades. Others may choose to design for a beginning-of-life range, and account for degradation by warranting the battery to a specified degree of capacity retention over a specified period of time or distance travelled. In the latter case, the consumer is expected to understand that a potential reduction in electric range is to be expected during the life of the vehicle.

18.	Despite the expected loss of electric range and battery capacity retention over time, regulatory practice does not uniformly account for it. For example, US EPA range labelling rules for PEVs and OVC-HEVs effectively treat driving range as a beginning-of-life criterion, by measuring range at beginning of-life and omitting any adjustment for future capacity degradation, while still accounting for some cold temperature and high speed operation. For OVC-HEVs, however, manufacturers are indirectly compelled to account for degradation in range, in that it directly affects the calculated in-use emissions later in life. OVC-HEV greenhouse gas (GHG) emissions are calculated using the SAE J1711 procedure, which accounts for utility factor, a function of all-electric range. If range degrades during useful life, the utility factor correction would change and thus, the calculated GHG emissions would increase. Because vehicles are considered noncompliant if their emissions exceed the certified emission level by more than 10 percent during the useful life, manufacturers that do not factor capacity degradation into their OVC-HEV designs risk exceeding the GHG standards in-use. Accordingly, for OVC-HEVs, manufacturers typically use a combination of battery oversizing and an energy management strategy that provides for a consistent range throughout the useful life.

19.	A number of further measures are employed by manufacturers to limit battery degradation. These typically include, but are not limited to, the use of properly optimised battery management systems (BMS) and battery thermal management. BMS can reduce stress on the battery and prolong battery life by controlling some operations to protect the battery cells, maintain cell charge balancing and moderate the battery temperature. For example, BMS might control enhanced cooling systems, limit fast charging events through modulation of the charging current, control the available state of charge window, keep the cell voltages balanced, or reduce the maximum available torque as necessary to protect battery health. The inputs to BMS can include anything from ambient conditions and driver behaviour to individual cell metrics. Each manufacturer, vehicle, battery and cell could have unique and highly optimised BMS, that are updated and improved with every iteration. BMS are very complex and generally considered highly proprietary and should not be tampered with out of environmental and safety concerns. Another important factor is the battery thermal management capability. While some batteries are only passively cooled by ambient air, others are actively cooled and heated by use of forced air, liquid coolant, or refrigerant, which leads to greater BMS control over battery operating temperature and hence longer life.

Prediction and/or estimation of battery degradation

20.	Accelerated aging is a familiar technique used by many manufacturers as a component of their battery durability testing methods. This technique assumes that a regime of rapid aging cycles can be translated to a projected useful life in service. However, it is uncertain whether the translation from accelerated aging to an in-use life projection is equally applicable to all forms of lithium-ion chemistries either currently in use or in the future.

21.	One of the major mechanisms by which capacity and power degradation occurs in these chemistries is the microscopic fragmentation that accompanies swelling and contraction of anode and cathode materials during cycling. Specific chemistries differ significantly in this respect, suggesting that the relation between rapid cycling and long-term cycling may also differ significantly. An accelerated test that accurately projects useful life for one chemistry may therefore predict poor life for another chemistry, even though both chemistries may achieve an equal life in actual use.

22.	Furthermore, accelerated ageing cannot take into account the real use of batteries inside vehicles and therefore can only partially estimate the real degradation.

23.	To monitor degradation in-use, most manufacturers employ some form of in situ, on-board capacity estimation through the BMS. This estimation can vary in accuracy and precision depending on a number of factors including the sensors and estimation algorithm used, the charge/discharge behaviour of the user, and the cell type and cell model parameters. Proprietary algorithms are used to handle inaccuracies and output an estimate that can be utilised by other systems within the vehicle.

24.	There are currently no requirements on the accuracy of on-board monitors and the estimates generated are not typically easily accessible to the vehicle user. The IWG on EVE has therefore made a decision to set the performance requirement in this field.

D.	Technical rationale and justification

25.	The mandate of the first phase of this GTR on in-vehicle battery durability includes the development of:

(a)	Requirements for reading and/or displaying battery health information and usage data from the vehicle;

(b)	Requirements for electrified vehicle battery durability performance criteria; and

(c)	A provisional in-service conformity test which will include generic usage criteria and a statistical method.

(d)	A reference certification procedure to check compliance against

26.	This section sets out the key considerations of the IWG on EVE in developing the respective elements of Phase 1 of the GTR as set out above.

State-of-Certified Range and State-of Certified Energy (SOCE) monitors

27.	Whilst the term State of Health (SOH) is commonly applied to refer to the health of a battery at a given point in its life, this term is not commonly defined and is determined through a variety of different methodologies. It was therefore chosen to define two a new related metrics for use within the GTR: the State of Certified Energy (SOCE) and the State of Certified Range (. This metric represent a percentage of the certified battery energy remaining at a given point in time. In the case of SOCE, it was decided to base the metric on the Usable Battery Energy (UBE).

28.	Metrics based upon electric range and UBE were both chosen as the values can be determined through the respective certification test methodologies already applied within the Contracting Parties and are based on key performance parameters relating to battery health. These metrics are intended to provide both the basis of information made available to consumers and also values for assessment against Minimum Performance Requirements (MPR) relating to battery durability by manufacturers and authorities. The IWG decided to exclude NOVC-HEVs from Phase 1 of the GTR development. While batteries of NOVC-HEVs can experience degradation, NOVC-HEVs have no electric range, and their battery UBE is not typically determined at certification. Moreover, battery degradation in an NOVC-HEV is likely to result in degradation of fuel economy, which can be detected by existing in-service conformity practices. The IWG will continue to consider the need for extending this GTR to NOVC-HEVs in the future.

29.	From discussions within the IWG it was concluded that it would not be appropriate to define the process or algorithm by which the SOCE monitors determine their estimated values. It would be highly complex to define an algorithm that could accurately account for the battery management strategies in use within the market. Instead, it was determined to allow manufacturers to determine their own means to estimate these metrics, whilst ensuring the accuracy through an in-use verification procedure.

Battery performance requirements

30.	The key battery durability requirements set out within this GTR are defined in terms of MPR. MPR are expressed as a minimum allowable value of SOCE after a given length of time, distance travelled or energy utilized. This follows a similar format applied by manufacturers when providing warranty for electrified vehicles.

31.	In determining appropriate MPR values for this GTR the IWG on EVE considered a range of publicly available data as well as input from stakeholders within the IWG, which is summarised paragraphs 32. to 37. below.

32.	Warranty analysis was conducted by the US EPA to understand the current warranty offering from manufacturers for electric vehicle batteries. The review primarily focussed on the US market, but values were also consistent with typical offerings within the European market. The review showed that batteries are covered for failure for between 7 to 10 years and typically up to 160,000 km. Warranties that define failure in terms of a specific capacity retention specified between 60 and 75 per cent retention, most commonly 70 per cent. Warranty offerings of 8 years or 160,000 kilometres were found to be the most common. It has been highlighted by manufacturers that warranty offerings are not based solely on the technical performance of the battery and include further considerations from a commercial and customer satisfaction perspective. Nevertheless, the review provides an insight into the degree of confidence in products currently on the market.

33.	The European Commission’s Joint Research Centre (JRC) has developed a dedicated in-vehicle battery durability assessment module within its ‘Transport tEchnology and Mobility Assessment’ (TEMA) platform. This is based on performance-based models as this class of models is the most suitable to be used with large-scale real-world driving data. TEMA is a modular big data platform designed to reproduce mobility behaviours of vehicles from datasets of navigation system data of conventional fuel vehicles and quantify possible impacts of new vehicle technologies on real-world mobility while supporting transport policy assessment.

34.	TEMA combines recent performance-based capacity and power fade models for Lithium-ion batteries from literature with information on battery and vehicle architectures, together with real world vehicle driving data from different geographical areas of Europe, to develop a scenario-based analysis for predicting in-vehicle performance degradation of automotive traction batteries. The analysis includes the calendar and cycle capacity fade of three Lithium-ion variants (LiFePO4, NCM with spinel Mn and NCM-LMO) in different vehicle architectures (OVC-HEV and PEV of different driving range segments), combined with different recharging strategies to explore the effect of different driving duty cycles related to different mobility patterns and environmental temperatures. Preliminary analyses on vehicle battery power fade have been also carried out.

35.	The TEMA model was used to estimate the capacity retention of traction batteries after a range of distances and time periods to allow consideration of appropriate choices for MPR. Example TEMA modelling results in Figure I/1 for two different mid-sized BEVs configurations either charged with slow or fast charging show greater than 70 per cent capacity retention after 8 years. This result is generally consistent with prevailing warranty practice observed in the warranty survey. Additionally, good agreement was previously found between TEMA modelling results and electric vehicle lifetime performance testing data provided by Environment and Climate Change Canada and Transport Canada during work within the previous mandate of the IWG on EVE.

Figure I/1

Example of a capacity retention curve generated from JRC TEMA modelling for two different BEV configurations

36.	Geotab is a U.S. company that collects in-use data from vehicle fleets, and provides a publicly available set of estimated battery degradation histories computed from data collected from different makes and model years of BEVs and PHEVs. Raw data is sourced from telematics from 6,300 fleet and consumer vehicles, covering 21 vehicle models and representing 64 distinct combinations of makes, models and years. The data is then converted to SOH estimates by means of a proprietary algorithm applied to the collected data, such as current and voltage measurements during charge and discharge. The data (see Figure I/2) also provides some insight into extreme vehicle use, extreme climates and charging methods. SOH estimates from Geotab were examined to understand the typical battery durability of existing vehicles in the fleet and also to compare with ageing predictions from the JRC’s TEMA model, resulting in good correlation.

Figure I/2

Example comparison between estimated results from the TEMA model with in-use data from Geotab

37.	Further analysis of the publicly available Geotab data was conducted by Japan and the Alliance for Automotive Innovation, where available SOH data was extrapolated to time points of 5 and 8 years to understand the expected SOH. Japan’s analysis indicated that 90 percent of the vehicle models within the sample were able to achieve approximately 80 percent SOH after 5 years and 70 percent after 8 years. The Alliance for Automotive Innovation conducted similar analysis, but also included probability estimates which indicated that between 85 and 90 percent of the current fleet covered by the Geotab data would be able to meet an 80 percent SOH target after 5 years.

38.	Following consideration of the available evidence and views of stakeholders within the IWG on EVE, two sets of MPR values were introduced based upon two different time and distance combinations. This approach allows coverage of the wide range of different distance-based requirements needed across Contracting Parties and provides the option for a Contracting Party to optionally apply only one of the MPR if appropriate for their market.

39.	The MPR values selected were deemed to be sufficiently achievable based on the available evidence presented within the IWG, whilst also being sufficiently stringent to achieve the goal of preventing substandard products from entering the market. Following discussion within the IWG, the same MPR were set for OVC-HEVs and PEVs.

40.	It was highlighted by manufacturers that the understanding and estimation of SOCR after a given duration of use or distance travelled currently presents an increased challenge compared to SOCE. There are many factors other than those originating from the battery leading to greater uncertainty of SOCR, including the measurement, test to test variability and precision of range retention calculations. As the majority of the evidence assessed by the IWG was also based primarily on remaining capacity or battery energy, it was decided to only monitor but not subject SOCR to an MPR requirement within Phase 1. No electric range based MPR have been selected at this point in time due to the above concerns with SOCR estimation. Inclusion of MPR for SOCR in future has, however, been highlighted as an area of importance for a number of Contracting Parties. Therefore, a placeholder for these values has been included to allow inclusion within a future amendment to the GTR.

41.	Electrified vehicles of category 2 are at an earlier stage of adoption within the fleets of many Contracting Parties and subsequently in-use data relating to battery durability is scarce for these vehicles. For this reason, it is difficult presently to determine a suitable and achievable MPR for category 2 vehicles. It has been raised by manufacturers that in future the batteries of these vehicles may also be used for supplementary purposes, in addition to propulsion, which could have further impacts on battery durability that are not currently well understood. MPR values for category 2 vehicles therefore remain reserved for inclusion in a future amendment to this GTR.

42.	Battery longevity is a key consideration for consumers and regulating agencies, therefore there is an incentive for manufacturers to accredit batteries that perform beyond the minimum requirements of this GTR. To allow manufacturers to market or quantify the benefits of any improved battery technology they may have installed within their vehicles the GTR also allows for a Declared Performance Requirement (DPR) to be proposed by manufacturers, based upon a higher value of SOCE

. To ensure the accuracy of any declared values, the DPR would be considered to act in place of the MPR for the purposes of in-use verification. The DPR is not intended to indicate a regulatory warrantied value. Manufacturer warranties are beyond the scope of this GTR. A manufacturer may wish to warranty batteries beyond the MPR without attaching additional warranty benefits to a DPR.

43.	The new MPR setting concept as depicted with the matrix below, Figure I/3, was proposed by Japan in order to let each Contracting Party decide its own MPR as option. The matrix exemplified the area to be defined as substandard category of MPR and the area that the MPR can be determined.

The IWG considered this proposal and decided to not adopt it during the first phase.

Figure I/3

Sample of MPR Matrix

In-use verification

44.	To ensure the accuracy of the SOCE monitors and also ensure that MPR are being met it was necessary to introduce a two part in-use verification process, with Part A verifying the accuracy of the monitors and Part B verifying the battery durability against MPR.

45.	Part A verification involves measurement of the UBE/ under the applicable test procedure and determination of a measured SOCE by dividing by the respective values from certification. These measured values can then be compared to the on-board values from the SOCE monitors to ensure the accuracy is within a given tolerance. For this purpose, the resolution of the on-board values from SOCE/ was set to 1 part in 100, and the required accuracy defined by a statistical process as described below.

46.	A pass or fail decision on a sample of vehicles will be reached through a statistical process, which evaluates the average of the ratios of measured/on-board-indicated SOCE from a series of vehicles tested. After testing a minimum of 3 vehicles a decision on either a pass or fail decision or on testing a further vehicle will be reached, on the basis of a statistical formula considering the quantitative deviation of the latter average from a value A = (1 + tolerance) and the variance of these ratios over the whole test series with a tolerance = 5% granted for a single test due to technical reasons. The method proposed is particularly suitable for cases where a quantity (such as SOCE) is likely to differ from the ‘true’ measured quantity continuously and reflects the method used for evaluating the conformity of production (CoP) in UN Regulation No. 154.

47.	As Part A verification is expected to involve a relatively small number of sample vehicles to limit the testing burden, it is important to ensure that the sample result is not unduly impacted by the abnormal use or poor maintenance of a vehicle within the sample. A vehicle survey has therefore been introduced within Annex 1 containing information designed to ensure that the vehicle has been properly used and maintained according to the specifications of the manufacturer. Any vehicles not meeting the required criteria may be removed from the test sample.

48.	Due to the accuracy of the SOCE/SOCR monitors being assured through verification in Part A, it is possible to verify the battery durability of a sample of vehicles within Part B through remote collection of the on-board SOCE/SOCR values, together with information on the age of the vehicles, distance travelled and energy utilized. Where a vehicle has been equipped with V2X capabilities, an equivalent virtual distance will be calculated using the V2X discharge energy and the certified energy consumption. This will be summed with the distance travelled to calculate the total distance. This approach avoids the need for further testing of vehicles within Part B and enables a simple route to the assessment of a large sample size of vehicles, thereby minimising the impact that outliers (e.g. vehicles that have been used abnormally) may have on the sample result.

49.	It is recognised that SOCE values read from a sample of vehicles are likely to be in the form of a distribution, with values for individual vehicles dependent on the vehicle usage and any inherent variation in the performance of the vehicle or traction battery. Where a vehicle has been used abnormally (e.g. with prolonged periods of storage or being regularly used in extremes of temperature) this may also give rise to more significant degradation of battery health. To reduce the impact of vehicles that may have been used abnormally, it was decided to make the overall pass decision dependent on more than or equal to 90 per cent of monitor values read from the vehicle sample being above the MPR. This approach thereby ensures that the MPR is being met by the significant majority of the vehicle sample, whilst accounting for abnormal usage.

50.	To further minimise influence of abnormally used vehicles, a maximum of 5 per cent of the values taken from smaller durability families that consist of less than 500 vehicles may be excluded from the verification sample in Part B with appropriate reasoning.

51.	To support this two part in-use verification process, whilst minimising the burden of increased testing for manufacturers within Part A, two family concepts were developed within the GTR. This includes the concept of a monitor family for use in Part A and a battery durability family for Part B. This is likely to reduce the need for additional testing where multiple battery durability families may have the same characteristics with respect to verification of the SOCE monitors. In addition, the Part A in-use verification for the vehicles in the same monitor family may be combined between different regions with the agreement of all Contracting Parties involved. This contributes not only to minimise the testing burden but also to increase the verification robustness. However, this concept shall not be applied for Part B since battery deterioration might be different in different regions due to different usage patterns and ambient conditions.

E.	Technical feasibility, anticipated costs and benefits

52.	The use of SOH monitoring within some electrified vehicles is already common practice and the improvement of monitor accuracy is an area under active development by manufacturers. Updating these monitors to ensure the specific requirements within this GTR are met and to allow information to be provided to the consumer may, however, entail some further development costs for manufacturers.

53.	The Part A verification of SOCE accuracy will involve additional testing by manufacturers and, optionally, by authorities who choose to further verify monitor accuracy. The additional costs associated with testing have been mitigated to an extent by making use of the relevant test procedure already applied for range (or UBE) determination in the respective Contracting Party. This should allow manufacturers to incorporate verification into any existing verification programme planned for those vehicles.

54.	Any costs associated with verification within Part B are likely to be borne by the authorities of the Contracting Parties applying this regulation and will depend on the extent and means by which they choose to collect data for verification. Costs may be borne by the manufacturer in the case of a fail for a battery durability family relating to the costs of measures agreed with the relevant authorities to bring those vehicles back into compliance.

55.	This GTR is likely to give rise to benefits for manufacturers and authorities through the prevention of inferior products which could undermine the market. Introduction of inferior products within the market could disadvantage those manufacturers who have invested in the development of technology aimed at ensuring battery longevity and could also undermine the environmental benefits that can arise from electrified vehicles, as well as public acceptance.

56.	The longevity of battery life is a key concern for prospective consumers of electrified vehicles. The health of a vehicle’s battery upon entry into the second-hand market is also a significant point of consideration for the consumer and is not easily understood in the absence of access to accurate battery health monitors. The availability of both accurate battery health information and assurances on battery longevity for consumers that is provided by this GTR are therefore likely to have a positive impact on the cost retention of electric vehicles and consumer confidence in buying an electrified vehicle.

F.	Future development of the GTR

57.	The mandate for development of this GTR included the future development of improvements to the GTR within Phase 2 that includes, but not limited to:

(a) 	The development of a methodology to define Normal Usage Indices (NUIs) based on data read from vehicles; and

(b) 	Refined performance criteria requirements for in-vehicle battery durability through assessment of further modelling and data collected from real vehicles and the use of NUIs.

58.	The concept of an NUI is a data field stored on the vehicle that represents a history or assessment of lifetime usage patterns of the vehicle that are influential to battery degradation. For example, one NUI might characterise temperature exposures during the life of the vehicle, while another might characterise charging rates or number of fast-charging events. The definition of NUIs was highlighted within the discussions of the IWG on EVE as a technically challenging task that will require further data collection and validation to achieve. It could, however, provide an alternative and more robust means of handling SOCE values recorded from vehicles that have been used abnormally in future. The possibility of incorporating NUIs within this GTR should therefore be explored.

59.	The implementation of this GTR by Contracting Parties will enable the collection of further data on SOCE and SOCR to better inform our understanding of battery health degradation. This information will, in turn, allow further refinement of the GTR, including MPR values, based upon the latest available battery technologies employed within the market. This will be important given the rapid development of technology in the field of battery technology for electrified vehicles that is already underway.

60.	The monitoring of SOCR values following the implementation of this GTR will provide a sound basis for the consideration of appropriate range-based MPR in a future revision to this GTR. Equally, the monitoring of both SOCE and SOCR for category 2 vehicles should allow the inclusion of MPR for this category of vehicle in future.

II.	Text of the GTR

### 1.	Purpose

This Global Technical Regulation provides a worldwide harmonized method to set and verify minimum performance requirements on in-vehicle battery durability of Heavy Duty Electric Vehicles (HDEVs) and Off-vehicle Charging Hybrid Heavy Duty Vehicles (OVC-HDVs).

### 2.	Scope and application

[This GTR applies to vehicles of categories 1–2 and 2, that have a technically permissible maximum laden mass exceeding 3,855 kg, and to vehicles of category 1–1, that (a) are HDEV or OVC-HDV vehicles, and (b) have an originally installed battery as defined in this GTR].

At the option of the Contracting Party, the scope be more than [3,500 kg] for the relevant categories.

The authorities shall make a decision about the applicability of this GTR to Small Volume Manufacturers for their jurisdiction.

Special purpose, off-road and all-wheel-drive vehicles shall be out of scope due to their relatively small contribution of overall energy consumption over lifetime compared to on-road long-haul, regional delivery and urban delivery heavy duty operations. Hydraulically driven axles are not considered qualifying a vehicle being all-wheel drive. Furthermore, contracting parties shall decide on even more exclusions such as special axle formulas or additional axle numbers given especially in their markets (8x2, 8x6, 10x, …).

Vehicle classes comparable to European class “O” (semitrailers & trailers) which may be electrified too in future. (Maybe to be considered for Phase 2).

### 3.	Definitions

The following definitions shall apply in this Global Technical Regulation.

3.1.	"Battery" means, in the context of this GTR, a rechargeable electrical energy storage system (REESS) installed in an electrified vehicle and used mainly for traction purposes.

3.2.	"Originally installed battery" means the battery that is installed in the vehicle at the time of manufacture, or if the vehicle is manufactured without an installed battery, the battery that is installed in the vehicle when it is first operated on the road.

3.3.	"Usable Battery energy (UBE)" means the energy supplied by the battery from the beginning of the test procedure used for certification until the applicable break-off criterion of the test procedure used for certification is reached.

3.4.	"Certified usable battery energy" (UBEcertified) refers to the UBE that was determined during the certification of the vehicle, according to Annex 3 of this GTR.

3.5.	"Measured usable battery energy" (UBEmeasured) means the UBE determined at the present point in the lifetime of the vehicle by the test procedure used for certification, according to Annex 3 of this GTR.

3.11.	"Minimum Performance Requirement" (MPR) means the minimum durability performance, in terms of SOCE at a specific point in the life of the vehicle, that constitutes compliance with the durability provisions of this GTR.

3.12.	"Declared Performance Requirement" (DPR) means an SOCE value declared by the manufacturer that is greater than that of the corresponding MPR and which then becomes the minimum durability performance that constitutes compliance of that manufacturer with the durability provisions of this GTR.

3.12.	"Rechargeable electrical energy storage system" (REESS) means a propulsion energy storage system that stores electrical energy and which is rechargeable. A battery whose primary use is to supply power for starting the engine and/or lighting and/or other vehicle auxiliaries systems is not considered as a REESS. The REESS may include the necessary ancillary systems for physical support, thermal management, electronic controls and casing.

3.12.	"State of charge" (SOC) means the available electrical charge in a REESS expressed as a percentage of its rated capacity.

3.13.

3.14.	"SOCE monitor" means an apparatus installed in the vehicle that maintains an estimate of the state of certified energy by means of an algorithm operating on data collected from the vehicle systems.

3.16.	"On-board SOCE" (SOCEread) means an estimate of state of certified energy produced by an SOCE monitor.

3.18.	"Measured SOCE" means the state of certified energy as determined by the measured usable battery energy divided by the certified usable battery energy.

3.19.	"V2X" means the use of the traction batteries to cover external power and energy demand, such as V2G (Vehicle-to-Grid) for grid stabilization by utilising traction batteries, V2H (Vehicle-to-Home) for utilizing traction batteries as residential storage for local optimisation or emergency power sources in times of power failure, and V2L (Vehicle-to-Load, only connected loads are supplied) for use in times of power failure and/or outdoor activity in normal times.

3.20.	"Total discharge energy during V2X" means the total amount of discharged energy during V2X which needs to be provided according to Annex 2.

3.21.	"Off-vehicle charging hybrid electric vehicle" (OVC-HEV) means an OVC-HEV vehicle as defined in UN GTR No. 15.

3.22.	"Pure electric vehicle" (PEV) means a PEV vehicle as defined in UN GTR No.15.

3.23.	"Maximum charging power" means the highest available charging power for the considered Part B family.

3.24.	“Non off-vehicle charging hybrid electric vehicle” (NOVC-HEV) means an OVC-HEV vehicle as defined in […] .

3.25.	“PTO-operation” means any energy throughput during stand still of the vehicle for operational purposes.

### 4.		Abbreviations

DPR	Declared Performance Requirement

MPR	Minimum Performance Requirement

OTA	Over the Air

REESS	Rechargeable Electrical Energy Storage System

SOC	State of Charge

SOCE	State of Certified Energy

UBE	Usable Battery Energy

V2G	Vehicle to Grid

V2H	Vehicle to Home

V2L	Vehicle to Load

### 5.		Requirements

## 5.1.	State-of Certified Energy (SOCE) monitor

The manufacturer shall install  a SOCE monitor that operates during the life of the vehicle. The SOCE monitor shall maintain an estimate of the state of certified energy (on-board SOCE).

The manufacturer shall determine the algorithms by which on-board SOCE are determined for the vehicles they produce. The manufacturer shall update the on-board SOCE with sufficient frequency as to maintain the necessary degree of accuracy during all normal vehicle operation.

The on-board SOCE shall have a resolution of 1 part in 100 and be reported as the nearest whole number from 0 to 100.

The manufacturer shall make available the most recently determined values of the on-board SOCE via the OBD port and optionally over-the-air (OTA).

For the purposes of consumer information, the manufacturer shall make easily available to the owner of the vehicle the most recently determined value of the SOCE monitor via at least one appropriate method. The resolution for the customer values shall be determined in agreement with the authorities. For example:

(a)	dashboard indicator;

(b)	infotainment system;

(c)	remote access (such as via mobile-phone applications).

### 5.2.	Battery Performance Requirements

The battery durability requirements of this GTR are defined in terms of Minimum Performance Requirements (MPRi), which represent minimum allowable values for SOCE at specific points in the lifetime of the vehicle. Vehicles falling under the categories of OVC-HDVs and HDEVs shall meet both of the Minimum Performance Requirements in Tables 1 and 2 below. [The MPRs may differ depending on the category of the vehicle and type of propulsion].

In order to address regional considerations, a Contracting Party may optionally elect to enforce only one of the two Minimum Performance Requirements (MPRi) in each of the tables below (i.e. either the one ending at 5 years or 100,000 km, or the one ending at 8 years or 160,000 km).

Table 1

Battery Energy based (SOCE) MPR

| Vehicle age/km for [categories 1-1 and 1-2] in the scope of this GTR | OVC-HDV | HDEV |
| --- | --- | --- |
| From start of life to [] years or [km or kWh], whichever comes first | [80] per cent | [80] per cent |
| Vehicles more than [] years or [km], and up to whichever comes first of [ years or km] | [70] per cent | [70] per cent |

| Vehicle age/km for [category 2] in the scope of this GTR | OVC-HEV | PEV |
| --- | --- | --- |
| From start of life to [ years or km], whichever comes first | (Reserved) | (Reserved) |
| Vehicles more than [ years or km], and up to whichever comes first of [ years or km] | (Reserved) | (Reserved) |

[SOCE monitors of vehicles of category 2 shall be installed and their values monitored in view of setting the values in the tables for part B as well as accuracy requirements in paragraph 6.3 of Part A in a future amendment of this GTR.]

A manufacturer may elect to declare a Declared Performance Requirement (DPRi) having an SOCE value that is higher than that of the corresponding MPR. The DPRi shall then replace the MPRi for the purposes of determining compliance by that manufacturer.

The manufacturer shall ensure that batteries installed in vehicles comply with the rules specified in paragraph 6.4.2. for the MPRi (or DPRi if applicable).

At the request of the manufacturer and for vehicles designed with V2X, the equivalent virtual distance calculated following the equation below will be reported by each vehicle.

Virtual distance (km)=total discharge energy during V2X [Wh]worst case certified energy consumption of PART B family  [Wh/km]

Where:

"worst case certified energy consumption of Part B family" means the worst case certified energy consumption of a Part B family which needs to be provided according to Annex 2.

At the option of the manufacturer, instead of using the worst case certified energy consumption value of the Part B family, the manufacturer may be allowed to use any higher energy consumption value.

The total distance used for confirming the compliance with the minimum performance requirements will consist of the sum of the distance driven and the virtual distance. The total percentage of the virtual distance shall be recorded and monitored.

### 6.		In-Use Verification

6.1.	Definitions of Families

Vehicles having the same characteristics with respect to their evaluation under Part A or Part B below shall be grouped into vehicle families for the purpose of compliance verification. Families under Part A shall have the same characteristics with respect to verification of the SOCE monitors. Families under Part B shall have the same characteristics with respect to verification of battery durability.

Families with the same characteristics with respect to compliance verification shall be defined as follows:

6.1.1.	For Part A: Verification of Monitors

Only vehicles that are substantially similar with respect to the following elements may be part of the same monitor family:

(a)	Algorithm for estimating on-board and on-board SOCE;

(b)	Sensor configuration (for sensors used in determination of SOCE estimates);

(c)	Characteristics of battery cell which have a non-negligible influence on accuracy of monitor;

(d)	Type of vehicle (PEVs or OVC-HEVs)

At the request of the manufacturer, with the approval of the responsible authority and with appropriate technical justification, the manufacturer may deviate from the above criteria for families.

6.1.2.	For Part B: Verification of Battery Durability

Only vehicles that are substantially similar with respect to the following elements may be part of the same battery durability family:

      - Type of battery chemistry;

      - Battery Control Unit BCU (with relevant functions for battery monitoring, estimations, and controls)

        - Or BMS

      - Operational strategy influencing the battery durability

      - At the request of the OEM, additional family criteria may be aligned with type approval authority

At the request of the manufacturer, with the approval of the responsible authority and with appropriate technical justification, the manufacturer may deviate from the above criteria for families.

### 6.2.	Information gathering

The following information shall be made available to the authorities by the manufacturer in a format to be agreed between the authorities and the manufacturer: annual report on relevant warranty claims; and annual statistics on repairs for both batteries and other systems that might influence the electric energy consumption of the vehicle. Such information shall be made available once a year for each battery durability family for the duration of the period defined in paragraph 5.2. after the last vehicle of this family is sold.

### 6.3.	Part A: Verification of /SOCE monitor

### 6.3.1.	Frequency of verifications

The manufacturer shall complete the procedure for in-use verification for Part A with a frequency agreed with the authorities, [ until 5 or 8 years] as defined in paragraph 5.2. after the last vehicle of each monitor family is sold and report the results of the verification to the authorities. The authorities may decide to proceed with their own verification of Part A, at a frequency and magnitude based on risk assessment, or request more information from the manufacturers. With the agreement of all Contracting Parties involved, the verification of Part A for vehicles in the same monitor family may be combined between different Contracting Parties. In such cases the relevant Contracting Parties shall be considered as a single authority for the purposes of this verification.

At the option of the Contracting Party, the verification of the monitors shall not be mandatory [if the annual sales of the monitor family are less than 5,000 vehicles in the market for the previous year.] Such families may still be selected to be tested for Part A, at the request of the responsible authorities.

### 6.3.2.	Verification procedure

In order to verify the SOCE monitor, the value for the usable battery energy shall be measured at the time of the verification and the related value from the monitor shall be collected before the verification test procedure. To support future improvement of the GTR, indicator values shall be collected again after the verification test procedure. Those indicators read after the verification test procedure shall not be considered in the Part A verification.

The measured SOCE values shall be determined by dividing the measured values for the usable battery energy by the certified values for  the usable battery energy, respectively, expressed in per cent.

SOCEmeasured=UBEmeasured UBEcertified *100

SOCRmeasured=Rangemeasured Rangecertified*100

### In cases where UBEmeasured is higher than the UBEcertified, the SOCEmeasured shall be set to 100 per cent.

#### 6.3.3.	Statistical Method for Pass/Fail decision for a sample of vehicles

An adequate number of vehicles [(at least 3 and not more than 16)] shall be selected from the same monitor family for testing following a vehicle survey (see Annex 1) which contains information designed to ensure that the vehicle has been properly used and maintained according to the specifications of the manufacturer. The following statistics shall be used to take a decision on the accuracy of the monitor.

For evaluating the SOCE monitors normalised values shall be calculated:

xi=SOCEread, i-SOCEmeasured,  i

Where

SOCEread, i	is the on-board SOCE read from the vehicle i ; and

SOCEmeasured, i	is the measured SOCE of the vehicle i.

For the total number of N tests and the normalised values of the tested vehicles, x1, x2, … xN, the average Xtests and the standard deviation s shall be determined:

Xtests= x1+x2+x3+…+xNN

and

s=(x1-Xtests)2+(x2-Xtests)2+...+(xN-Xtests)2N-1

For each N tests 3 ≤ N ≤ 16, one of the three following decisions can be reached, where the factor A shall be set at 5:

(a)	Pass the family if Xtests≤A-tP1,N+tP2,N∙s

(b)	Fail the family if Xtests>A+tF1,N-tF2∙s

(c)	Take another measurement if:

A-tP1,N+tP2,N∙s<Xtests≤A+tF1,N-tF2∙s

where the parameters tP1,N, tP2,N, tF1,N, and tF2 are taken from Table 3.

# Table 3

Pass/fail decision criteria for the sample size

|  | PASS |  | FAIL |  |
| --- | --- | --- | --- | --- |
| Tests (N) | tP1,N | tP2,N | tF1,N | tF2 |
| 3 | 1.686 | 0.438 | 1.686 | 0.438 |
| 4 | 1.125 | 0.425 | 1.177 | 0.438 |
| 5 | 0.850 | 0.401 | 0.953 | 0.438 |
| 6 | 0.673 | 0.370 | 0.823 | 0.438 |
| 7 | 0.544 | 0.335 | 0.734 | 0.438 |
| 8 | 0.443 | 0.299 | 0.670 | 0.438 |
| 9 | 0.361 | 0.263 | 0.620 | 0.438 |
| 10 | 0.292 | 0.226 | 0.580 | 0.438 |
| 11 | 0.232 | 0.190 | 0.546 | 0.438 |
| 12 | 0.178 | 0.153 | 0.518 | 0.438 |
| 13 | 0.129 | 0.116 | 0.494 | 0.438 |
| 14 | 0.083 | 0.078 | 0.473 | 0.438 |
| 15 | 0.040 | 0.038 | 0.455 | 0.438 |
| 16 | 0.000 | 0.000 | 0.438 | 0.438 |

#### 6.3.4.	Corrective measures for the SOCE monitor

A fail decision for the sample means that the monitors fail to report accurately the durability of the system and appropriate action shall be taken by the manufacturer with the agreement of the responsible authority. This may lead to the requirement that the manufacturer repairs or replaces the faulty monitor including the relevant sensors or by applying software measures in all affected vehicles in the monitor family.

A pass decision or correction of the non-compliance is required for proceeding with Part B.

6.4.	Part B: Verification of Battery Durability

6.4.1.	Frequency of verifications

Data shall be collected yearly by the authorities from a statistically adequate sample of vehicles within the same battery durability family. The decision on the number of the vehicles in the sample may be taken by the responsible authority based on risk assessment methodology, [but in principle should not be less than 500].

If the number of vehicles in the sample is less than 500, then on the request of the manufacturer and with the agreement of the responsible authority, a maximum of 5 per cent of the values may be excluded from the sample. In such a case, the manufacturer needs to provide adequate information on the reason behind the exclusion for each vehicle to the authority.

If the number of vehicles in the sample is equal to or more than 500, then all vehicles shall be included in the sample. The data read shall be those of the SOCE monitor (and other relevant data, such as those defined in Annex 2).

6.4.2.	Pass/Fail Criteria for the battery durability family

A battery durability family shall pass if equal to or [more than 90] per cent of monitor values read from the vehicle sample are above the MPRi or DPRi.

A battery durability family shall fail if less than 90 per cent of monitor values read from the vehicle sample are above the MPRi or DPRi.

For the pruposes of this GTR, the first and second sub-paragraph shall be applied X years after its implementation in contracting parties laws.

- Or -

For the classes of this GTR, a monitoring phase of X years shall be given to collect in-vehicle data from first electric heavy duty vehicles in the market.

6.4.3.	Corrective Measures for the Battery Durability Family

In case of a fail for a battery durability family, corrective measures shall be taken with the agreement of the responsible authority in order to bring the family or part of the family affected by the issue into compliance.

6.5.	Process flow charts for Part A and Part B

The flow charts below illustrate the various steps in the verification process of Part A (Figure 1) and Part B (Figure 2).

Figure 1

Flow chart for Part A: Verification of Monitors

YES

None

YES

NO

Part A: Verification of Monitors

Vehicle Procurement

Vehicle Survey

according to Annex 1

Obtain on-board SOCE

(used for Part A verification)

Pass/Fail Decision

according to paragraph 6.3.3.

Obtain on-board SOCE before and after update.

Use the one before update for monitoring

Use the one after update for Part A verification

	Perform Verification Test

	Obtain measured SOCE

	Obtain on-board SOCE (not used for Part A verification)

Check the Exclusion

Criteria

Need more testing according to paragraph

6.3.3.

Exclude from sampling

Or update the monitor by procedure in vehicle survey

Figure 2

Flow chart for Part B : Verification of Battery Durability

YES

Part B : Verification of Battery Durability

Data Acquisition according to paragraph 5.1.

Pass/Fail Decision

according to paragraph 6.4.2.

Annual Sample Size is equal or more than [500]

NO

Exclude maximum 5% with responsible authority agreement

### 7.	Rounding

7.1.	When the digit immediately to the right of the last place to be retained is less than 5, that last digit retained shall remain unchanged.

Example:

If a result is 1.2344 kWh but only three places of decimal are to be retained, the final result shall be 1.234 kWh.

7.2.		When the digit immediately to the right of the last place to be retained is greater

than or equal to 5, that last digit retained shall be increased by 1.

Example:

If a result is 1.2346 kWh but only three places of decimal are to be retained, and because 6 is greater than 5, the final result shall be 1.235 kWh.

Annex 1

Vehicle Survey

The vehicle survey shall be used for all vehicles selected for testing in Part A of the verification. Vehicles that fall under one of the exclusion criteria below shall be eliminated from testing, or otherwise updated according to the procedures described below.

|  | x = Exclusion Criteria | x = Checked and reported | Confidential |  |
| --- | --- | --- | --- | --- |
| Date: |  |  | x |  |
| Name of investigator: |  |  | x |  |
| Location of test: |  |  | x |  |
| Country of registration: |  | x |  |  |
|  | x = Exclusion Criteria | x = Checked and reported |  |  |
| Vehicle Characteristics |  |  | Confidential |  |
| Registration plate number: |  | x | x |  |
| The vehicle must have age and distance travelled (defined as the time elapsed after first registration) below the one required in paragraph 5.2. for the MPR verification | x |  |  |  |
| Is the vehicle either HDEV or OVC-HDV? If no: the vehicle cannot be selected | x |  |  |  |
| Date of first registration: |  | x |  |  |
| VIN: |  | x |  |  |
| Emission class and character or Model Year |  | x |  |  |
| Country of registration: The vehicle must be registered in a Contracting Party | x | x |  |  |
| Model: |  | x |  |  |
| Engine code: |  | x |  |  |
| Engine volume (l): |  | x |  |  |
| Engine power (kW): |  | x |  |  |
| Electric Engine code: |  | x |  |  |
| Electric Engine power (kW): |  | x |  |  |
| Electric powertrain type |  | x |  |  |
| Energy capacity and type of battery |  | x |  |  |
| Gearbox type (auto/manual): |  | x |  |  |
| Drive axle (FWD/AWD/RWD): |  | x |  |  |
| Tyre size (front and rear if different): |  | x |  |  |
| Average fuel consumption for PHEVs |  | x |  |  |
| Is the vehicle involved in a recall or service campaign? If yes: Which one? Have the campaign repairs already been done? The repairs must have been done before selecting the vehicle. | x | x |  |  |
| Vehicle Owner Interview (the owner will only be asked the main questions and shall have no knowledge of the implications of the replies) |  |  |  |  |
| Name of the owner (only available to the accredited inspection body or laboratory/technical service) |  |  | x |  |
| Contact (address / telephone) (only available to the accredited inspection body or laboratory/technical service) |  |  | x |  |
| How many owners did the vehicle have? |  | x |  |  |
| Did the odometer work? If no, the vehicle cannot be selected. | x |  |  |  |
| Was the vehicle used for one of the following? |  |  |  |  |
| As car used in show-rooms? |  | x |  |  |
| As a taxi? |  | x |  |  |
| As a delivery vehicle? |  | x |  |  |
| For racing / motor sports? | x |  |  |  |
| As a rental car? |  | x |  |  |
| Has the vehicle carried heavy loads over the specifications of the manufacturer? If yes, the vehicle cannot be selected. | x |  |  |  |
| Have there been major engine, electric motor or vehicle repairs? |  | x |  |  |
| Have there been unauthorised major engine or vehicle repairs? If yes, the vehicle cannot be selected. | x |  |  |  |
| Was the propulsion battery changed or repaired? If yes, the vehicle cannot be selected for testing, but information should be collected | x | x |  |  |
| Has there been an unauthorised power increase/tuning? If yes, the vehicle cannot be selected. | x |  |  |  |
| Was any part of the emissions after-treatment system modified? If yes, the vehicle cannot be selected | x |  |  |  |
| Where do you use your vehicle more often? |  |  |  |  |
| % motorway |  | x |  |  |
| % rural |  | x |  |  |
| % urban |  | x |  |  |
| Has the vehicle been maintained and used in accordance with the manufacturer's instructions? If not, the vehicle cannot be selected. | x |  |  |  |
| Full service and repair history including any re-works If the full documentation cannot be provided, the vehicle cannot be selected. | x |  |  |  |
| Battery related checks: |  |  |  |  |
| How often did you charge the vehicle when: %with battery almost at 0 charge %with battery half charged %with battery almost fully charged | - - - | x x x |  |  |
| On average how often were fast or superfast chargers used in a month? |  | x |  |  |
| What is your estimation of the percentage of time that the vehicle was used in the following ambient temperature ranges: Below -7C: Between -7C and 35C: More than 35C: |  | x x x |  |  |
| Vehicle Examination and Maintenance by the Testing Centre (please use the relevant entries according to the type of vehicle) | x= Exclusion Criteria | x=checked and reported |  | Relevant for BEV |
| Was the vehicle not charged adequately* for the last month? If the vehicle was not charged adequately for the last month (as evidenced by values read from the vehicle under point 9, Annex 2) and the tester wishes to use it for testing, then it has to be conditioned by driving the vehicle no less than 50 km and in a manner that results in discharge of at least 50 per cent of the usable capacity of the battery, followed by a full recharge. Note: * Adequately in this sense means that the vehicle was not charged in a manner that would lead to an accurate SOCE | x |  |  |  |
| Fuel tank level (full / empty) Is the fuel reserve light ON? If yes, refuel before test. |  | x |  |  |
| Are there any warning lights on the instrument panel activated indicating a vehicle or exhaust after-treatment system malfunctioning that cannot be resolved by normal maintenance? (Malfunction Indication Light, Engine Service Light, etc?) If yes, the vehicle cannot be selected | x |  |  |  |
| Is the SCR light on after engine-on? If yes, the reagent should be filled, or the repair executed before the vehicle is used for testing. | x |  |  |  |
| Visual inspection exhaust system Check leaks between exhaust manifold and end of tailpipe. Check and document (with photos) If there is damage or leaks, the vehicle cannot be tested | x |  |  |  |
| Exhaust gas relevant components Check and document (with photos) all emissions relevant components for damage. If there is damage, the vehicle cannot be tested | x |  |  |  |
| Air filter and oil filter Check for contamination and damage. Change if damaged or heavily contaminated or less than 800 km before the next recommended change. |  | x |  |  |
| Wheels (front & rear) Check whether the wheels are freely moveable or blocked or impeded by the brake. If not freely moveable, the vehicle cannot be selected. | x |  |  | Y |
| Drive belts & cooler cover In case of damage, the vehicle cannot be tested. | x |  |  |  |
| Check fluid levels Check the max. and min. levels (engine oil, cooling liquid) / top up if below minimum |  | x |  |  |
| Vacuum hoses and electrical wiring Check all for integrity. In case of damage, the vehicle cannot be tested. | x |  |  | Y |
| Injection valves / cabling Check all cables and fuel lines. In case of damage, the vehicle cannot be tested. | x |  |  | Y |
| Ignition cable (gasoline) Check spark plugs, cables, etc. In case of damage, replace them. |  | x |  |  |
| EGR & Catalyst, Particle Filter Check all cables, wires and sensors. In case of tampering or damage, the vehicle cannot be selected. | x |  |  |  |
| Safety condition Check tyres, vehicle’s body, electrical and braking system status are in safe conditions for the test and respect road traffic rules. If not, the vehicle cannot be selected. | x |  |  | Y |
| Semi-trailer Are there electric cables for semi-trailer connection, where required? |  | x |  | Y |
| Check if less than 800 km away from next scheduled service, if yes, then perform the service. |  | x |  | Y |
| Powertrain Control Module calibration part number and checksum |  | x |  | Y |
| OBD diagnosis (before or after the range test) Read Diagnostic Trouble Codes & Print error log |  | x |  |  |
| OBD Service Mode 09 Query (before or after the range test) Read Service Mode 09. Record the information. |  | x |  |  |
| OBD mode 7 (before or after the range test) Read Service Mode 07. Record the information |  |  |  |  |

Remarks for: Repair / replacement of components / part numbers

Annex 2

Values to be read from vehicles:

1.	On board SOCE value

2.

3.	Odometer (in km)

4.	Date of manufacture of the vehicle

5.	Total distance (sum of the distance driven and the virtual distance) [km], if applicable

6.	Percentage of virtual distance [in per cent], if applicable

7.	Worst case certified energy consumption of PART B family [Wh/km], if applicable

8.	Total discharge energy in V2X [Wh], if applicable

9.	Elapsed time since last charged by more than 50 per cent SOC swing [Days]

10.	Maximum, minimum, average battery pack temperature* the battery experienced   during its lifetime

11.	Energy throughput

12. 	Capacity throughput

13. 	Total time of use of the battery

Note: * temperature to be read as daily averages

## Annex 3

Test procedure and determination of performance parameters

### 1.	General

For the calculation of SOCEread and SOCEmeasured and  according to paragraph 6.3.2. of this GTR, the measured and certified values of usable battery energy (UBE)  for HDEVs and OVC-HDVs are required:

	UBEmeasured and UBEcertified



This annex describes the determination of these parameters, in [paragraph 2.] for HDEVs and in paragraph 3. for OVC-HDVs and gives guidance on which measurements need to be performed and which certified values need to be applied for a vehicle selected in the Part A verification procedure.

For the purposes of this annex, the term ’battery‘ includes not only REESS used mainly for traction purposes, but also all other REESSs.

### 2.                   Test conditions

2.1.	Test instrumentation

2.1.1.	Bidirectional charger/bidirectional power supplier/…

[…]

2.1.2.	Test room

The test cell shall have a temperature set point of [25] °C. The tolerance of the actual value shall be within ±10 °C.

2.1.3.	Cooling fan

[… ]

2.1.4.	Soak area

The temperature of the soak area shall be maintained at [25 °C ±10 °C].

2.2.	Measurement

2.2.1.	Measurement items and accuracy

Measurement devices shall be of certified accuracy as shown in Table 1 traceable to an approved regional or international standard.

2.2.2.	Measurement frequency

All the items in Table 1 of ‎2.2.1 of this Annex, unless specified otherwise in the table, shall be measured and recorded at a frequency equal to or greater than [10 Hz].

The items atmospheric pressure and room temperature shall be at least recorded as single measurement activity at start of the vehicle battery testing (see [3.8.1 of this Annex]) and after end of the vehicle battery testing (see [‎3.8.5 of this Annex]).

Table 1

Measurement items and required accuracy

| Item | Units | Accuracy | Remarks |
| --- | --- | --- | --- |
| Electrical voltage | V | ±0.3 % FSD or ±1 % of reading. | Whichever is greater. Resolution 0.1 V. |
| Electrical current | A | ±0.3 % FSD or ±1 % of reading | Whichever is greater. Current integration frequency [20 Hz] or more for external measurement. Resolution 0.1 A. |
| Room temperature | K | ±1 °C, with a measurement frequency of at least 0.1 Hz |  |
| Time | s | ± 10 ms; min. precision and resolution: 10 ms |  |
| Electrical power | W | [ …] | Charging discharging electrical power of the bidirectional charger/ bidirectional power supplier, [i.e. accuracy, calibration] |
| Bidirectional charger/ bidirectional power supplier/… |  | […] | AC, DC … |

### 3.		Test procedure for determine the battery energy capacity fade

3.1.	General

The following test procedure determine the energy capacity fade of the batteries of the HDEV or OVC-HDEV.

A Round Trip Efficiency (RTE) test (full charge/discharge cycle) at certification and at given years or [mileage kWh/equivalent full cycles] should be applied via the charging port. This can be done with a [bidirectional charging unit as per paragraph xx] after pre-conditioning.

[The manufactures shall guarantee that all the traction batteries installed on the vehicle are engaged during the RTE test to determine the Usable Battery Energy (UBE) certified and measured.]

3.1.1.	Required information

The manufacturer shall provide the following information required to conduct the test procedure.

[Boundary conditions that qualify vehicle for testing

- Battery cell temperature normally distributed with average temperature at Y C and variance <Z

- Average SoC normally distributed with average value Y*% and variance <Z*

- Depth of discharge (DoD): share of cycles with DoD >Y**% must be below Z**%]

The manufacturer shall specify if a RTE operation mode shall be set at vehicle level for performing the test.

3.1.2.	Required measurements

[The test vehicle shall be instrumented with measurement devices for measuring the necessary input values for the UBE calculation (voltage and electrical current). The discharge and charge energy shall be measured at the battery to avoid combined battery-inverter efficiency and energy losses.

As an alternative to use of measurement devices, use of on-board measurement data is permissible if the accuracy and frequency of these data is demonstrated to the responsible authority to meet the minimum requirements for accuracy and frequency described in [‎2.2. of this Annex]. ]

3.1.3.	Other information

The manufacturer shall specify the normal operating range for each operational metric listed in [3.8.1. of this Annex].

[Regarding any RTE operation mode (see 3.7), the manufacture shall provide a list of the deactivated devices and justification for the deactivation.]

3.2.	Preparation of bidirectional charger/bidirectional power supplier

3.2.1.	Bidirectional charger/ bidirectional power supplier warm-up

The bidirectional charger shall be [conditioned] or [warmed up in accordance with the manufacturer’s recommendations], as appropriated, so that the internal electrical systems may be stabilized.

3.3.	Preparation of vehicle

The vehicle shall be presented in good technical condition and shall be run-in in accordance with the manufacturer’s recommendations.

[HDEVs shall have been run-in at least [300XX km] or one full charge distance, whichever is longer.]

[OVC-HDVs shall have been run-in and driven between XX  and XX km before the test.

3.4.	Preparation of measurement devices

The measurement devices shall be installed at suitable position(s) within the vehicle.

3.5.	Initial charge of REESS

[For HDEVs and OVC-HDVs, prior to or during vehicle soak (3.6. of this Annex), the REESS shall be charged to an initial SOC of [xx]. At the request of the manufacturer, with the approval of the responsible authority and with appropriate technical justification, the manufacturer may specify a different initial SOC of the REESS].

The initial charge of the REESS shall be conducted at an ambient temperature [of xx ± 10 °C].

The REESS shall be charged to the initial SOC in accordance with the procedure specified by the manufacturer for normal operation until the charging process is normally terminated.

[The SOC shall be confirmed by a method provided by the manufacturer.]

3.6.	Vehicle soak

The vehicle shall be soaked in the soak area for a [minimum of 9 hours]. The manufacturer may recommend a specific soak time or range of soak times within the range of [9 to 36 hours] if necessary to ensure temperature stabilization of the high voltage battery. The soak area conditions during soak shall be as specified in [‎2.1.4].

The REESS shall not be charged during the soak period.

3.7.	Vehicle and bidirectional charger installation

The vehicle and the bidirectional charger shall be installed for the preconditioning.

Auxiliary devices of the vehicle shall be switched off or deactivated during bidirectional charge.

If necessary to operate properly the bidirectional charger, the vehicle’s bidirectional charge operation mode shall be activated by using the manufacturer's instruction.

The manufacturer shall provide the responsible authority a list of the deactivated devices and justification for the deactivation. The bidirectional charge operation mode shall be approved by the responsible authority and the use of a bidirectional charge operation mode shall be recorded.

The vehicle’s bidirectional charge operation mode shall not activate, modulate, delay or deactivate the operation of any part that affects the energy throughput under the test conditions.

Measurement devices installed within the vehicle shall be warmed up as appropriate.

3.8.	Test sequence

3.8.1.	General

The test shall be carried out in accordance with ‎[3.8.2. to ‎3.8.5. of this Annex], (see Figure x). The test shall be stopped immediately if warning indicator(s) with regard to the batteries turns on.

[The following operational metrics, if present, shall be monitored and recorded throughout the test: (a) battery temperature (as indicated by temperature of battery cells, modules, or pack, as available), (b) battery SOC, (c) battery cooling on/off, as available.  The manufacturer shall specify the normal operating range for each operational metric.]

3.8.2.	Vehicle conditioning

The vehicle transfer from the soak area, if needed, shall be done without any unjustified delay and in any case within [xx] minutes.

The measurement devices shall start collecting data.

The object of conditioning is to fully charge the REESS of the vehicle at [xx]C and leave the REESS stabilized for [1h].

The full charge of the battery is reached when [..],  according to operating limits.

During the preconditioning the vehicle shall not receive unjustified exposure to other temperatures but if that is unavoidable this time should in any case be limited to a maximum of [10] minutes.

[At the end of initial vehicle conditioning, the operational metrics (see ‎3.8.1. of this Annex) shall be recorded.]

[During the test, monitor the operating metrics and perform additional conditioning as necessary to maintain the operating metrics within the normal operating temperature ranges].

3.8.3.	RTE test

The battery shall be fully discharged at defined [C-rate/power/discharge current], AC or DC]. End of discharge is reached when [….]. The UBEdischarge is the total discharged energy calculated as described in [Annex x, paragraph x].

The REESS shall be left stabilizing for [1h].

The REESS shall be fully charged  with a defined [C-rate/power/charging current, AC and DC] according to operating limits. Full charge is reached when […]. The UBEcharge is the total charged energy calculated as described in [Annex x, paragraph x].

The full cycle efficiency is calculated by dividing the UBEdischarge by the UBEcharge.

These two steps (discharge and charge) are performed on [ new vehicles within a family] for determine the  UBEcertified.

These two steps (discharge and charge) are performed on [aged vehicles within a family] for determine the  UBEmeasured.

The SOCEmeasured is derived according to paragraph [3.3.2.]

[For OVC-HDV the internal combustion engine shall not operate during the duration of the test.]

3.8.4.	Repetition of RTE test

[The RTE test of ‎3.8.3. of this Annex shall be repeated for a total of xx repetitions as shown in Figure x].

[The temperature-related operational metrics listed in ‎3.8.1. of this Annex shall be monitored during all repetitions and seen to remain within the normal operating range specified by the manufacturer during each repetition. Re-condition the vehicle and REESS according to ‎3.8.2. of this Annex between repetitions if necessary].

3.8.5.	End of RTE test

At the end of the RTE test, the measured values and the operational metrics (see ‎3.8.1. of this Annex) shall be recorded.

After the measurements are complete, the vehicle and measurement devices shall be stopped.

Figure x

Test sequence

### 4.		Performance parameters

4.1.	UBE for HDEVs and OVC-HDVs

4.1.1.	Measured and certified UBE values

| Parameters | Explanation |
| --- | --- |
| UBEcertified | UBE certified is the usable battery energy (UBE) of the vehicle measured at the point of certification. |
| UBEmeasured | UBE measured determined at Part A verification by the test procedure |
|  | The required input parameter UBE is calculated as follows: UBE= i=1n∆EREESS,i where: ∆EREESS,i is the measured electric energy change of battery i, Wh; i is the index number of the considered battery; n is the total number of batteries; and: ∆EREESS,i= 13600×t0tendU(t)REESS,i×ItREESS,i dt where: U(t)REESS,i is the voltage of battery i, V; I(t)REESS,i is the electric current of battery i, A; t0 is the time at the beginning of the charge or discharge test, s; tend is the time at the end of the charge or discharge test, s; 13600 is the conversion factor from Ws to Wh. |
| No rounding shall be applied on UBE. |  |

[Bidirectional charger values/ bidirectional power supplier …]

[As an alternative to use of measurement devices, use of on-board measurement data is permissible if the accuracy and frequency of these data is demonstrated to the responsible authority to meet the minimum requirements for accuracy and frequency described in [‎2.2. of this Annex]. ]

|  | UBEmeasured=UBEmeasured,nc-∆EREESS,CCave ∆EREESS,CCave |
| --- | --- |
|  | UBEmeasured,nc= i=1n∆EREESS,i ∆EREESS,i ∆EREESS,i= 13600×t0tendU(t)REESS,i×ItREESS,i dt U(t)REESS,i I(t)REESS,i 13600 |
|  | ∆EREESS,CC,ave ∆EREESS.CC,ave=i=1nΔEREESS,avg,i,CC ∆EREESS,avg,i,CC i ΔEREESS,avg,i,CC= 13600×1tend,CC-tstart,CC×tstart,CCtend,CC tstart,CCtend,CCUREESS,it×IREESS,itdt dt UREESS,it IREESS,it tstart,CC tend,CC 13600 |

3.1.2.	Certified UBE values for OVC-HEVs

| Parameters | Explanation |
| --- | --- |
| UBEcertified | UBEcertified is the adjusted measured usable battery energy (UBE) of the vehicle at certification: UBEcertified=UBEmeasured*AFOVC-HEV Where: UBEmeasured is the measured usable battery energy according to paragraph 3.1.1. of this annex, Wh; AFOVC-HEV is the adjustment factor determined as described below. At the option of the Contracting Party, one out of the following two adjustment factors shall be selected: - Adjustment factor 1: AFOVC-HEV=ECmeasuredECcertified where: ECcertified is the electric energy consumption ECAC.CD according to GTR15 Annex 8, Table A8/8, Step no. 14 at certification, Wh/km; ECmeasured is the measured electric energy consumption ECAC,CD according to GTR15 Annex 8, Table A8/8, Step no. 13 at certification. Wh/km. - Adjustment factor 2: AFOVC.HEV=ECmeasuredECcertified where: ECcertified is EC according to GTR15 Annex 8, Table A8/9, Step no. 8 at certification, Wh/km; ECmeasured is measured EC according to GTR15 Annex 8, Table A8/9, Step no. 7 at certification. Wh/km. |
|  | UBEcertfied shall be rounded according to paragraph 7 of this GTR: - To the nearest whole number in case unit is Wh - To three significant numbers in case unit is kWh |
|  | In the case the interpolation method is applied, UBEcertified shall be determined by selecting: - The maximum UBEmeasured amongst vehicle H and vehicle L and (if applicable) vehicle M; - The AF which is closest to 1. |

|  | EAERmeasured=MCO2,CS - MCO2,CD,avgMCO2,CS×Rcdc |
| --- | --- |

[Annex battery charging procedure

…..

…]

Only one vehicle manufacturer provides warranty up to 1,000,000 km