New structure to ATP 2023 – Chapter 4

Checks and test to be performed on equipment

| ADR 2022 | Restructured | Comments – future text |
| --- | --- | --- |
| General | A1-A1 para 1 Checks for conformity with the standards prescribed in this annex shall be made: (a) before equipment enters into service; (b) periodically, at least once every six years; and (c) whenever required by the competent authority. Except in the cases provided for in appendix 2, sections 5 and 6, to this annex, the checks shall be made at a testing station designated or approved by the competent authority of the country in which the equipment is registered or recorded, unless, in the case of the check referred to in (a) above, a check has already been made on the equipment itself or on its prototype in a testing station designated or approved by the competent authority of the country in which the equipment was manufactured. | It need to be checked what is best here and in the “approval” chapter |
| Determination of the average surface area of a body. | 1.2 The mean surface area S of the body is the geometric mean of the inside surface area Si and the outside surface area Se of the body: In determining the two surface areas Si and Se, structural peculiarities and surface irregularities of the body, such as chamfers, wheel-arches and similar features, shall be taken into account and shall be noted under the appropriate heading in test reports; however, if the body is covered with corrugated sheet metal the area considered shall be that of the plane surface occupied, not that of the developed corrugated surface. For calculating the mean surface area of the body of a panel van, the test station appointed by the competent authority shall select from one of the following three methods. Method A. The manufacturer shall provide drawings and calculations of the inside and outside surfaces. The surface areas Se and Si are determined taking into consideration the projected surface areas of specific design features of the irregularities of its surface such as curves, corrugations, wheel boxes, etc. Method B. The manufacturer shall provide drawings and the test station appointed by the competent authority shall use the calculations according to the schemes and formulae below. Si = (((WI × LI) + (HI × LI) + (HI × WI)) × 2) Se = (((WE × LE) + (HE × LE) + (HE × WE)) × 2) Where: WI is the Y axis of the internal surface area LI is the X axis of the internal surface area HI is the Z axis of the internal surface area WE is the Y axis of the external surface area LE is the X axis of the external surface area HE is the Z axis of the external surface area Using the most appropriate formula for the Y axis of the internal surface area WI = (WIa x a + WIb x (b + c/2) + WIc x c/2) / (a + b + c) WI = (WIa × a/2 + WIb (a/2 + b/2) + WIc (b/2)) / (a + b) WI = (WIa × a + WIb × b + (WIb + WIc)/2 × c) / (a + b + c) Where: WIa is the internal width at the floor or between the wheel arches WIb is the internal width at the height of the vertical edge from the floor or above the wheel arches. WIc is the internal width along the roof a is the height of the vertical edge from the floor b is either the height between the bottom of the vertical edge and the roof or between the top of the wheel arch and the top of the vertical edge from the floor. c is the height between the roof and point b Along with the two formulae for the X and Z axes of the internal surface: LI = ((LIa x a) + (LIb + LIc) / 2 x b + (LIc x c)) / (a + b + c) Where: LIa is the internal length along the floor LIb is the internal length above the wheel arches LIc is the internal length along the roof a is the height between LIa and LIb b is the height between LIb and LIc c is the height between LIc and the roof WI = (WI back + WI front) / 2 Where: WI back is the width at the bulkhead WI front is the width at the door end The external surface area is calculated using the formulae below: WE = WI + declared mean thickness × 2 LE = LI + declared mean thickness × 2 HE= HI + declared mean thickness × 2 Method C. If neither of the above is acceptable to the experts, the internal surface shall be measured according to the figures and formulae in method B. The K value shall then be calculated based on the internal surface area, taking the insulation thickness as nil. From this K value, the average insulation thickness is calculated from the assumption that λ for the insulation has a value of 0.025 W/m.°C. d = Si x ΔT x λ / W Once the thickness of the insulation has been estimated, the external surface area is calculated and the mean surface area is determined. The final K value is derived from successive iteration. |  |
| Temperature measuring points |  |  |
|  | A1-A2 1.3 to 1.6 In the case of parallelepipedic bodies, the mean inside temperature of the body (Ti) is the arithmetic mean of the temperatures measured 10 cm from the walls at the following 12 points: (a) The eight inside corners of the body; and (b) The centres of the four inside faces having the largest area. If the body is not parallelepipedic, the 12 points of measurements shall be distributed as satisfactorily as possible having regard to the shape of the body. In the case of parallelepipedic bodies, the mean outside temperature of the body (Te) is the arithmetic mean of the temperatures measured 10 cm from the walls at the following 12 points: (a) The eight outside corners of the body; and (b) The centres of the four outside faces having the largest area. If the body is not parallelepipedic, the 12 points of measurement shall be distributed as satisfactorily as possible having regard to the shape of the body. 1.5 The mean temperature of the walls of the body is the arithmetic mean of the mean outside temperature of the body and the mean inside temperature of the body: 1.6 Temperature measuring instruments protected against radiation shall be placed inside and outside the body at the points specified in paragraphs 1.3 and 1.4 of this appendix. |  |
| Steady state period and duration of test |  |  |
|  | A1-A2 para 1.7 The mean outside temperatures and the mean inside temperatures of the body, taken over a steady period of not less than 12 hours, shall not vary by more than ± 0.3 °C, and these temperatures shall not vary by more than ± 1.0 °C during the preceding 6 hours. The difference between the heating power or cooling capacity measured over two periods of not less than 3 hours at the start and at the end of the steady state period, and separated by at least 6 hours, shall be less than 3 %. The mean values of the temperatures and heating or cooling capacity over at least the last 6 hours of the steady state period will be used in K coefficient calculation. The mean inside and outside temperatures at the beginning and the end of the calculation period of at least 6 hours shall not differ by more than 0.2 °C. |  |
|  |  | Test protocols, test methods and documentation see (New) Annex 1, appendix 1: Part 1: insulated bodies other than tanks Part 2: Liquid foodstuffs tanks. Part 3 : Refrigerated appliances eutectic plates (fixed) Part 4: Refrigerated appliances liquefied gas (in mono temp and multi temp configurations) Part 5: Refrigerated appliances others than eutectic plates (fixed) and liquefied gas. Part 6: Mechanically refrigerated appliances (in mono temp and multi temp configurations, in refrigerated-heated configurations) Part 7: Heating appliances (and mechanically refrigerated and heated configurations). Part 10: periodic check on the insulated body. Part 11: periodic check on the thermal appliance Mech refrig Part 12: periodic check on thermal appliance liquid gas Part 13: periodic check on thermal appliance eutectic system Part 20: Replacement of refrigerant. Part 21: Dimensioning capacities of thermal appliances Part 22: Dimensioning capacities in multi temperature applications . |

The relevant figures can be found in the ATP Handbook at the following link: https://unece.org/atp-handbook