New structure to ATP 2023 – Chapter 6

Marking of equipment

| ADR 2022 | Restructured | Comments – future text |
| --- | --- | --- |
| Marking of equipment |  |  |
|  | A1-A1 para 4 Distinguishing marks and particulars shall be affixed to the equipment in conformity with the provisions of appendix 4 to this annex. They shall be removed as soon as the equipment ceases to conform to the standards laid down in this annex. |  |
|  | A1-A4 para 1 - start The distinguishing marks prescribed in appendix 1, paragraph 4 to this annex shall consist of capital Latin letters in dark blue on a white ground. The height of the letters shall be at least 100 mm for the classification marks and at least 50 mm for the expiry dates. For special equipment, such as a laden vehicle with maximum mass not exceeding 3.5 t, the height of the classification marks could likewise be 50 mm and at least 25 mm for the expiry dates. The classification and expiry marks shall at least be affixed externally on both sides in the upper corners near the front. |  |
|  | A1-A4 end The date (month, year) entered under section A, item 8 in appendix 3 of this annex as the date of expiry of the certificate issued in respect of the equipment shall be quoted under the distinguishing mark or marks aforesaid. Model: FRC 02 - 2020 02 = month (February) ) of expiry of the 2020= year ) certificate |  |
|  | A1-A4 In the case of multi-compartment road equipment divided in two compartments the equipment mark shall consist in the distinguishing marks of each compartment (example: FRC-FRA) starting with the compartment located at the front or on the left side of the equipment. In the case of other multi-compartment equipment, the distinguishing mark shall be selected only for the highest ATP class, i.e. the class that permits the highest difference between inside and outside temperatures, and supplemented by the letter M (example: FRC-M). |  |
|  | A1-A4 penultimate If the equipment is fitted with a removable or dependent thermal appliance and if special conditions exist for the use of the thermal appliance, the distinguishing mark or marks shall be supplemented by the letter X in the following cases: 1. FOR REFRIGERATED EQUIPMENT: Where the eutectic plates have to be placed in another chamber for freezing; 2. FOR MECHANICALLY REFRIGERATED EQUIPMENT AND MECHANICALLY REFRIGERATED AND HEATED EQUIPMENT: 2.1 Where the compressor is powered by the vehicle engine; 2.2 Where the refrigeration or refrigeration-heating unit itself or a part is removable, which would prevent its functioning. |  |
|  | The marks shall be as follows….. To be entered here or it to be left as Appendix 4 |  |