Looming problem

The paragraph 6.2.2 of annex 1 appendix 2 of the ATP Agreement specifies that, from next January 2024 non-independent refrigerated vehicles registered after 6 January 2018, in addition to the pull down test (as per paragraph 6.2) will have to prove that they maintain the class temperature by operating the vehicle engine at idle speed for 90 minutes.

There are two points to clarify:

1) what is meant by "maintain class temperature"

2) It is correct not to take into account the ambient temperature in establishing the time of 90 min

The text as it is can be interpreted very strictly (+/-0.5°C) and make impossible the positive result of the requested test. (excluding vehicles with alternator instead of compressor direct-drive) It would be appropriate to specify with what tolerance we can define that "the class temperature" has been maintained? +/- 1.5°C or +/- 3°C.

From the informal tests carried out so far, no vehicle, even those equipped with an very powerful reefer, does not comply with the +/-0.5°C with respect to the "Class Temperature".

Another consideration is needed; according to the ambient temperature (which for the validity of the test can be between +15°C and +30°C) the maintenance of the class temperature may or may not be satisfied with the same vehicle. (Example: same vehicle - positive winter test - negative summer test). Therefore we propose to include in the text of paragraph 6.2.2 a table similar to that of paragraph 6.2.1 (ambient temp/pull-down time limit) which relates the ambient temperature to the time required to satisfy the maintain test of the class temperature.

The table below is a subdivision hypothesis that could be integrated

| Outside Temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +30°C | +29°C | +28°C | +27C | +26C | +25C | +24C | +23C | +22C | +21C | +20C | +19C | +18C | +17C | +16C | +15C | ATP setpoint |
| 60 | 62 | 64 | 66 | 68 | 70 | 72 | 74 | 76 | 78 | 80 | 82 | 84 | 86 | 88 | 90 | 0°C |
| 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | 66 | 68 | 70 | 72 | 74 | 76 | 78 | 80 | -10°C |
| 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | 66 | 68 | 70 | -20°C |