I.	Proposal

Paragraph 2.3.8., amend to read:

"2.3.8.	"Movable components" of the vehicle mean those body panels or other vehicle parts the position(s) of which can be changed by tilting, rotating or sliding without the use of tools. They do not include tiltable driver cabs of trucks."

Add new paragraphs 2.3.12. to 2.3.12.3., to read:

"2.3.12.	Terms and definitions related to automated driving

2.3.12.1.	“Dynamic Driving Task (DDT)” means the real-time operational and tactical functions required to operate the vehicle.

2.3.12.2.	“Automated Driving System (ADS)” means the hardware and software that are collectively capable of performing the entire DDT on a sustained basis.

2.3.12.3.	“Driver” means a human user who performs in real time part or all of the DDT and/or DDT fallback for a particular vehicle.”

Paragraph 2.5.3., amend to read:

"2.5.3.	"Direction-indicator lamp" means the lamp used to indicate to other road-users that the driver vehicle driver or ADS intends to change direction to the right or to the left. A direction-indicator lamp or lamps may also be used according to the provisions of UN Regulations Nos. 97, 116, 162 or 163."

Paragraph 2.7.4.7., amend to read:

"2.7.4.7.	"Adaptive main-beam" means a main-beam of the AFS that adapts its beam pattern to the presence of oncoming and preceding vehicles in order to improve the long-range illumination visibility ahead of the vehicle for the driver without causing discomfort, distraction or glare to other road users."

Add a new paragraph 3.2.8. to read:

“3.2.8.	Vehicles If a driver’s seat is fitted but it is not the front seat nearest to the opposing traffic, due to the design or the operational conditions of the vehicle, the applicant shall specify this seating position.”

Paragraph 5.14.4., amend to read:

"5.14.4.	It shall not be possible deliberately, from the inside of the vehicle from the driver's seat, to stop the movement of switched ON lamps before they reach the position of use. If there is a danger of dazzling other road users by the movement of the lamps, they may light up only when they have reached their position of use."

Paragraph 5.26.4., amend to read:

"5.26.4.	No sharp variation of intensity shall be observed during transition.

It may be possible for the driver to set the functions above to static luminous intensities."

Add new paragraphs 5.36. and 5.37., to read:

“5.36.	Any automatic lighting or driving functionality, which requires the driver to permanently monitor the environment and the vehicle/system performance, is not an ADS.

5.37.	Whilst the ADS is active, any faults, failures and tell-tale information specified in paragraph 6 of this UN Regulation shall be transmitted to the ADS.”

Add a new paragraph 6.1.7.2.1., to read:

“6.1.7.2.1.	In the case that the vehicle is controlled by an ADS For vehicles equipped with an ADS, either:

Authority that the automatic main-beam operation is controlled by the ADS to avoid causing discomfort, distraction or glare to other road users; or

- the main-beam headlamps shall be deactivated while the ADS is active.”

Paragraph 6.1.7.3., amend to read:

“6.1.7.3.	With the exception of an active ADS, it shall always be possible for the driver to switch the main-beam headlamps ON and OFF manually and to manually switch OFF the automatic control of the main-beam headlamps.

Moreover, the switching OFF, of the main-beam headlamps and of their automatic control, shall be by means of a simple and immediate manual operation; the use of sub-menus is not allowed.”

Paragraph 6.2.6.1.1., amend to read:

6.2.6.1.1.	Initial downward inclination

The initial downward inclination of the cut-off of the passing-beam shall be:

-	set in the unladen vehicle state with one person in the driver’s seat front seat, nearest to the opposing traffic, and

-	specified within an accuracy of 0.1 per cent by the manufacturer and

-	within the range defined in paragraph 6.2.6.1.2.

The value of this specified initial downward inclination shall be indicated in a clearly legible and indelible manner on each vehicle close to either headlamp or the manufacturer's plate by the symbol shown in Annex 7.

Different values of the initial downward inclination for different variants/versions of the same vehicle type may be specified, within the range defined in paragraph 6.2.6.1.2., provided that only the pertinent value is indicated on each variant/version.

Paragraph 6.2.7.7., amend to read:

"6.2.7.7.	The driver or the ADS shall at all times be able to engage the automatic operation."

Paragraph 6.3.6.1.2.1.1., amend to read:

"6.3.6.1.2.1.1.	The vertical inclination of the cut-off to be set in the unladen vehicle state with one person in the driver’s seat front seat, nearest to the opposing traffic, shall be – 1.0 per cent or lower. "

Paragraph 6.3.6.1.2.2.2., amend to read:

"6.3.6.1.2.2.2.	The initial downward inclination of the cut-off to be set in the unladen vehicle state with one person in the driver’s seat front seat, nearest to the opposing traffic, shall be specified within an accuracy of one decimal place by the manufacturer and indicated in a clearly legible and indelible manner on each vehicle close to either the front fog lamp or the manufacturer's plate or in combination with the indication referred to in paragraph 6.2.6.1.1. by the symbol shown in Annex 7 to this Regulation. The value of this indicated downward inclination shall be defined in accordance with paragraph 6.3.6.1.2.2.1."

Paragraph 6.4.7.2., amend to read:

"6.4.7.2.	Moreover, the electrical connections of the two optional devices mentioned in paragraph 6.4.2.2. shall be such that these devices cannot be switched ON unless the lamps referred to in paragraph 5.11. are switched ON.

The devices fitted on the side of the vehicle may be switched ON for slow manoeuvres in forward motion of the vehicle up to a maximum speed of 15 km/h, provided that the following conditions are fulfilled:

- The devices shall be switched ON and OFF manually by a separate control; they may also be switched ON and OFF automatically by an ADS;

(b)	If so switched ON, they may remain ON after reverse gear is disengaged;

(c)	They shall be automatically switched OFF if the forward speed of the vehicle exceeds 15 km/h, regardless of the position of the separate control; in this case they shall remain switched OFF until deliberately being switched ON again."

Paragraph 6.5.7., amend to read:

"6.5.7.	Electrical connections

Direction-indicator lamps shall switch ON independently of the other lamps. All direction-indicator lamps on one side of a vehicle shall be switched ON and OFF by means of one control and shall flash in phase.  In addition, in the case of an activated ADS, the direction-indicator lamps shall be operated automatically.

On M1 and N1 vehicles less than 6 m in length, with an arrangement complying with paragraph 6.5.5.2. above, the amber side-marker lamps, when mounted, shall also flash at the same frequency (in phase) with the direction-indicator lamps.

A direction indicator capable of being activated in different modes (static or sequential), shall not switch between both modes once activated.

If two optional lamps (category 2a or 2b) are installed on vehicles in categories M2, M3, N2, N3, they shall be operated in the same mode as the other mandatory rear direction indicator lamps (category 2a or 2b); i.e. static or sequential."

Paragraph 6.6.7.1. amend to read:

"6.6.7.1.	The signal shall be operated by means of a separate manual control, enabling all the direction-indicator lamps to flash in phase. In addition, in the case of an activated ADS, the direction-indicator lamps shall be operated automatically."

Paragraph 6.20.7.2., amend to read:

"6.20.7.2.	When the reversing lamp is switched ON, both cornering lamps may be switched ON simultaneously, independently from the steering angle wheel position or direction-indicator activation position.

If so switched ON, both cornering lamps shall be switched OFF either:

(a)	When the reversing lamp is switched OFF; or

(b)	When the forward speed of the vehicle exceeds 15 km/h."

Paragraph 6.22.6.1.1., amend to read:

« 6.22.6.1.1.	The initial downward inclination of the cut-off of the basic passing-beam to be set in the unladen vehicle state with one person in the driver’s seat front seat, nearest to the opposing traffic,shall be specified within an accuracy of 0.1 per cent by the manufacturer and indicated in a clearly legible and indelible manner on each vehicle, close to either the front lighting system or the manufacturer's plate, by the symbol shown in Annex 7.

Where differing initial downward inclinations are specified by the manufacturer for different lighting units that provide or contribute to the cut-off of the basic passing-beam, these values of downward inclination shall be specified within an accuracy of 0.1 per cent by the manufacturer and indicated in a clearly legible and indelible manner on each vehicle, close to either the relevant lighting units or on the manufacturers plate, by the symbol shown in Annex 7 in such a way that all the lighting units concerned can be unambiguously identified.

The value(s) of this (these) indicated initial downward inclination(s) shall be specified by the vehicle manufacturer within the range defined in paragraph 6.2.6.1.2. in relation to the mounting height of the lighting units that provide or contribute to the cut-off of the basic passing-beam.

Different values of the initial downward inclination for different variants/versions of the same vehicle type may be specified, within the range defined in paragraph 6.2.6.1.2., provided that only the pertinent value is indicated on each variant/version."

Add a new paragraph 6.22.7.1.2.1., to read:

“6.22.7.1.2.1.	In the case that the vehicle is controlled by an ADS For vehicles equipped with an ADS, either:

- 	the applicant shall prove to the satisfaction of the Type-Approval Authority that the adaptive main-beam operation is controlled by the ADS to avoid causing discomfort, distraction or glare to other road users; or

- 	the main-beam headlamps shall be deactivated while the ADS is active.”

Paragraph 6.22.7.1.3. amend to read:

“6.22.7.1.3.	With the exception of an active ADS, it shall always be possible to switch the main-beam headlamps, adaptive or non-adaptive, ON and OFF manually and to manually switch OFF the automatic control.

Moreover, the switching OFF, of the main-beam headlamps and of their automatic control, shall be by means of a simple and immediate manual operation; the use of sub-menus is not allowed.”

Paragraph 6.22.7.5., amend to read:

"6.22.7.5.	In case the ADS is not active, the driver shall always be able to set the AFS to the neutral state and to return it to its automatic operation. An ADS shall always be able to set the AFS to neutral state."

Paragraph 6.22.8.4., amend to read:

"6.22.8.4.	A tell-tale to indicate that the driver or the ADS has set the system into a state according to paragraph 5.8. of UN Regulation No. 123 or paragraph 4.12. of UN Regulation No. 149 is optional."

Annex 1,

Add a new item 9.31., to read:

"9.31.	Automated Driving System (ADS): yes/no2

Add a new item 10.8., to read:

“10.8.	The A driver’s seat is fitted: yes/no2

The driver’s seat is the front seat nearest to the opposing traffic: yes/no2

If not, to be specified.

Annex 5,

Paragraph 2., amend to read:

2.	Loading conditions for different types of vehicles:

For this paragraph the front seat nearest to the opposing traffic (formerly known as the “driver’s seat”) is referred to as “seating position 1”.

If the seating position 1 is not located on the front seat nearest to the opposing traffic, due to the design or the operational conditions of the vehicle, the manufacturer shall specify the seating position 1.

- For vehicles designed to travel without occupants of Category Y, the presence of any persons shall be disregarded.

- For vehicles designed to travel with occupants the following loading conditions shall apply:”

Paragraphs 2.1.1.1. to 2.4.2.2., amend to read:

"2.1.1.1.	One person in the driver’s seat seating position 1;

2.1.1.2.	The driver One person in seating position 1, plus one passenger person in the front seat farthest from the driver the person in seating position 1;

2.1.1.3.	The driver One person in seating position 1, one passenger person in the front seat farthest from the driver the person in seating position 1, all the seats farthest to the rear occupied;

2.1.1.4.	All the seats occupied;

2.1.1.5.	All the seats occupied, plus an evenly distributed load in the luggage boot, in order to obtain the permissible load on the rear axle or on the front axle if the boot is at the front.  If the vehicle has a front and a rear boot, the additional load shall be appropriately distributed in order to obtain the permissible axle loads. However, if the maximum permissible laden mass is obtained before the permissible load on one of the axles, the loading of the boot(s) shall be limited to the figure which enables that mass to be reached;

2.1.1.6.	Driver One person in seating position 1, plus an evenly distributed load in the boot, in order to obtain the permissible load on the corresponding axle.

However, if the maximum permissible laden mass is obtained before the permissible load on the axle, the loading of the boot(s) shall be limited to the figure which enables that mass to be reached.

2.1.2.	In determining the above loading conditions, account shall be taken of any loading restrictions laid down by the manufacturer.

2.2.	Vehicles in categories M2 and M31;

The angle of the light beam from the dipped-beam headlamps shall be determined under the following loading conditions:

2.2.1.	Vehicle unladen and one person in the driver’s seat seating position 1;

2.2.2.	Vehicles laden such that each axle carries its maximum technically permissible load or until the maximum permissible mass of the vehicle is attained by loading the front and rear axles proportionally to their maximum technically permissible loads, whichever occurs first.

2.3.	Vehicles in category N with load surfaces:

2.3.1.	The angle of the light beam from the dipped-beam headlamps shall be determined under the following loading conditions;

2.3.1.1.	Vehicle unladen and one person in the driver’s seat seating position 1;

2.3.1.2.	Driver One person in seating positon 1, plus a load so distributed as to give the maximum technically permissible load on the rear axle or axles, or the maximum permissible mass of the vehicle, whichever occurs first, without exceeding a front axle load calculated as the sum of the front axle load of the unladen vehicle plus 25 per cent of the maximum permissible payload on the front axle.  Conversely, the front axle is so considered when the load platform is at the front.

2.4.	Vehicles in category N without a load surface:

2.4.1.	Drawing vehicles for semi-trailers:

2.4.1.1.	Unladen vehicle without a load on the coupling attachment and one person in the driver’s seat seating position 1;

2.4.1.2.	One person in the driver’s seat seating position 1: technically permissible load on the coupling attachment in the position of the attachment corresponding to the highest load on the rear axle.

2.4.2.	Drawing vehicles for trailers:

2.4.2.1.	Vehicle unladen and one person in the driver’s seat seating position 1;

2.4.2.2.	One person in the driver’s seat seating position 1, all the other places in the driving cabin being occupied."

OTHER PARAGRAPHS

2.3.3.3. Of sensors and devices for indirect vision;

2.3.4.2. (a) Of sensors and devices for indirect vision;

4.2. …(at present 09, corresponding to the 09 series of amendments …)

5.26.4. It may be possible for the driver to set the functions above to static luminous intensities.

5.35. General provisions relating to Driver Assistance Projection

6.1.4.3., 6.2.4.3., etc. In length: at the front of the vehicle. This requirement shall be deemed to be satisfied if the light emitted does not cause discomfort to the driver either directly or indirectly through the devices for indirect vision and/or other reflecting surfaces of the vehicle.

6.2.7.6. Irrespective of the requirements of paragraph 6.2.7.5., it shall always be possible to switch the dipped beam

headlamps ON manually.

6.22.7.13. … Moreover, the switching OFF, of the main-beam headlamps and the deactivation of their automatic control shall be by means of a simple and immediate manual operation; the use of sub-menus is not allowed

Testing provisions, COP and PTI: AVSR should consider how to ensure that all requirements of R48 can be tested, especially for vehicles of Category X and Y that may have no manual controls.