7.4.	Functional Unit and reference flow

[The primary function of a light-duty vehicle is to transport passengers and/or goods from one location to another. The functional unit (FU) for light-duty vehicles is defined as the passengers and/or goods transported over vehicle lifetime in kilometres. For vehicles carrying passengers, the default functional unit for vehicles is defined as “passenger-km” (p-km). A default functional unit for vehicles carrying goods is defined as “tonne-km” (t-km), based on assumed carriage of goods.

In all regional certification conditions, a predefined occupancy and appropriate loading conditions are already considered for light-duty vehicles, meaning these variables are already factored in in the certification values for the carbon footprint calculation. Thus, the loading condition shall follow to official-national regulation and/or legislation according to each jurisdiction.

However, for vehicles carrying goods, which are designed primarily for goods transport but often serve mixed functions such as carrying tools, equipment, or passengers, the choice of an appropriate functional unit is challenging.

Given these considerations, a more conservative approach should be considered and the occupancy rate and/or loading conditions mandated by the regional certification protocol per vehicle should be assumed, with the presumption that the light-duty vehicle occupancy rate will not change due to different vehicle technologies, energy sources or number of seats. This will also ensure comparability between different vehicles.

Considering the above assumption, the functional unit (FU) for a light-duty vehicle is defined as the transportation of χ passenger per km of distance travelled over the vehicle lifetime. Where χ is the number of passengers and/or loading conditions dictated by the jurisdiction under consideration.

However, since lifetime distance is different in each region, each regional authority, upon providing a thorough explanation and documentation, is entitled to adopt a distinct functional unit (e.g. per vehicle service life). This unit shall be based on well-founded assumptions that are reflective of the respective jurisdiction under consideration]

The reference flow for a vehicle is defined as the measurable quantity of inputs and outputs necessary to meet the defined functional unit of the product over its lifecycle. The reference flow in automotive LCAs translates this functional unit into specific, quantifiable measures of resources consumed and emissions produced, covering aspects such as:

(a)	Vehicle components and materials (e.g., steel, aluminium, plastics required to manufacture the car),

(b)	Fuel and/or energy consumption over the vehicle’s lifetime,

(c)	Maintenance and replacement parts (such as tyres, fluids, SLI batteries, etc.),

(d)	End-of-life treatment for disposal or recycling.

The standardised reference flow enables consistent and comparable assessments across different automotive products, focusing on the impacts associated with achieving the defined service.

Given the current scope of these guidelines, certain definitions, methodological approaches, and calculation rules—including those pertaining to the functional unit, vehicle service life, certification protocols, discrepancy and deterioration factors, leakages, and maintenance parts and activities—are presently limited to the passenger car framework. The processes listed above serve as illustrative examples and are not intended to be exhaustive.]

Batteries for starting-lighting-ignition (SLI) and auxiliary applications only - does not apply to primary battery packs used for electric-drive powertrains