| BMS RATING | BMS FEATURES | TYPICAL APPLICATION EXAMPLE |
| --- | --- | --- |
| Minimum | Comparator-based lithium-ion balancing integrated circuit (heavily dependent on compliant charger) | Toys |
| Fair | BMS Features under Minimum plus the following: - Cells of a type with internal protection being fuse, PTC or e-fuse if suitable - Monitor the voltage and temperature of each cell and balance in a reasonable time to suit the applications charge/discharge cycle - Cells can be monitored in groups - Prevent power entering the pack when fully charged (over voltage) or over temperature - Prevent power leaving the pack when cells are over temperature or exhausted (under voltage) - A pack DC-rated fuse with appropriate separation - Designed and built to SELV (Safety Extra Low Voltage) | General consumer products, e-cigarettes, torches |
| Good | BMS Features under Fair plus the following: In addition to the capabilities above, for a system that has a need for greater quality and reliability due to the pack being used in either a higher powered, more hostile environments or extended life applications, additional features should be included in the BMS: - Quality cells with matched capability, quality control manufacture and characterised in a standard datasheet - Semiconductors rated to twice the pack input or input protection to accommodate accidental and over-voltage inputs including ESD such as lightning strikes and human generated electrostatic - Current sensing of the overall pack - Humidity sensing - Cells monitored and logged over time to gauge performance through SoC and SoH history algorithms - Sensing of cell characteristics and monitoring for cell expansion (gassing) and impact/vibration (accelerometer/MEMS) - Communications scheme to inform the application or host of the state of the cells in the pack - Inclusion of a Piezo-electric alarm and red LED indicator allowing audible and visual indication of a compromised battery | Laptops, power tools, e-scooters |
| Excellent | BMS Features under Good plus the following: In addition to the sets of capabilities above, for applications requiring high performance, safety and the ability to manage the pack condition and expectations, the BMS should include these additional features: - Designed and built to IPC2221, IPC610 and assembled to IPC6012 - Fully isolated design with very low cell leakage - Per cell monitoring including current sense - Both active and passive cell balancing - Bypass diode applied across each cell - For military, aviation or mining, MIL-SPEC, DO-series and Ex Intrinsically Safe specification as required - Additional sensors for gas (hydrogens/carbons) and ARC/Flashover light sensors (500- 900nm wavelength) - Pack switch management, including potential across DC switch and switch temperature - Positive and negative differential earth leakage detection (balanced) - Quality build with minimal cabling, high temperature rated materials and compliant protective enclosure - Authenticated wireless communications to allow both normal BMS operation and exceptions to be communicated to all parties (including emergency services) - Includes thermal management and control system - BMS can interrupt host with critical conditions rather than wait to be polled (protocol) - Acoustic sensing for high pitch gassing/hissing/crackling/popping (PDM – Pulse Density Modulation microphone) and associated FFT algorithm - Pressure sensor to sense enclosure breach or puncture | EV, military, mining, aviation |