Energy Modeling Section

second draft – 8Oct2024

Activity Data (MJ) × Emission Factor (g CO₂eq/MJ) = GHG Emission

Activity Data

Activity data refers to measured, estimated, or assumed data for each stage of the automotive life cycle.

Emission Factor

- The emission factor should be sourced from secondary datasets (potential sources include IEA, Ecoinvent, IDEA, GREET, or other recognized datasets). The datasets used must be reported to the relevant authority in the final document. Practitioners must review these datasets to ensure that the emission factors account for all processes up to actual on-site use. These processes include:

  - Feedstock/raw material extraction

  - Processing

  - Transportation

  - Leakage for fuels

  - Energy losses during fuel/electricity production/transportation

  - Infrastructure emissions, including the construction and maintenance of fuel and electricity production infrastructure, such as solar panels.

- If sub-processes are not fully captured by the emission factors provided in the secondary datasets, practitioners should supplement this data with additional information from other inventories or sources to address these gaps.

- Practitioners may use specific emission factors. In such cases, certification of these emission factors must be attached to the final document. Certification can come from an electricity company, or measured data from a specific site (e.g., factory, yard, etc.).
In this context, market-based or local electricity grid mix/production data can also be used.

  - Electricity Data Requirements (Based on ISO 14067:2018)

### Grid Mix and On-Site Electricity Production:

### For the average consumption grid mix, data should be sourced from the grid and read on the TSO (Transmission System Operator) or DSO (Distribution System Operator) counter.

### On-site electricity production can only be accounted for under the following conditions:

      - The production asset is owned by the same entity as the factory, or the asset has a direct connection to the factory and is not connected to the grid.

      - Only the fraction of electricity that is used on-site and not sourced from the grid should be included.

      - No credits can be claimed for electricity produced in excess and sent to the grid.

- Practitioners may adjust the emission factors to reflect actual activity conditions. Any adjustments, along with detailed explanations of the calculation methods or models used to revise emission factors, must be reported in the final document.

Future Changes in Energy Mix: Practitioners should also account for any potential changes in the fuel or electricity production pathways during the lifetime of the vehicle.

- The final report shall be verified by the relevant authority.

Verification/certification process should be discussed within the other SGs