Energy Modeling Section

Sixth  draft – 21Oct2024

Activity Data (MJ) × Emission Factor (g CO₂eq/MJ) = GHG Emission

Activity Data

Activity data refers to measured, estimated, or assumed data for each stage of the automotive life cycle.

Emission Factor

- The emission factor should be sourced from secondary datasets (potential sources include IEA, Ecoinvent, IDEA, GREET, or other recognized datasets). The datasets used must be reported to the relevant authority in the final document. Practitioners must review these datasets to ensure that the emission factors account for all processes up to actual on-site use. These processes include:

  - Feedstock/raw material extraction

  - Processing

  - Transportation

  - Leakage of fuels

  - Energy losses during fuel/electricity production/transportation

  - ILUC

  - H2 characterization factor

  - Infrastructure emissions, including those from the construction and maintenance of fuel and electricity production infrastructure (e.g., solar panels), shall be excluded from the system boundaries unless specific evidence suggests otherwise. However, certain exceptions shall be considered if there is a consensus.

- If sub-processes are not fully captured by the emission factors provided in the secondary datasets, practitioners should supplement this data with additional information from other inventories or sources to address these gaps.

- Practitioners may use specific emission factors. In such cases, certification of these emission factors must be attached to the final document. Certification can come from an electricity company, or measured data from a specific site (e.g., factory, yard, etc.).
In this context, market-based or local electricity grid mix/production data can also be used.

  - Electricity data shall be based on ISO 14067:2018) and shall include:

- Upstream processes of the electricity supply system (e.g. mining of fuels, transport of fuels to power plants, growing and processing of biomass fuel feedstock, production of equipment generating renewable energy, such as photovoltaic installations)

- Processes related to electricity generation, including losses during transmission and distribution

- Downstream emissions (e.g. the treatment of waste arising from the operation of nuclear electricity generators or treatment of ashes from coal fired electricity plants).

In addition to data based on the average electricity mix of a country, region, etc., specific electricity data can be used if any of the conditions (1), (2), or (3) are met.

(1) Internally generated electricity

Specific electricity data may be used in the calculation for electricity generated on the premises where the target product is produced (on-site electricity generation) and actually consumed in the production, etc. of the target product. To avoid double counting the environmental value, only electricity for which the environmental value of the electricity is not sold externally as a certificate, etc. may be used.

(2) Electricity from a directly connected supplier

Specific electricity data may be used in the calculation for electricity supplied directly from electricity generators via dedicated transmission lines to facilities manufacturing qualifying products and actually consumed in the manufacture of such products, etc. To avoid double counting the environmental value, only products whose environmental value is not sold externally as certificates, etc., may be used.

(3) Electricity from the grid

Specific electricity data from electricity certificates may be used in the calculation for electricity supplied from the grid if it can be ensured that all of the following conditions are met.

- Information on the unit of electricity is provided together with the characteristics of the electricity generator.

- It is guaranteed to be a unique claim.

- Tracking, redemption, depreciation, cancellation, etc., is carried out by the reporting company, etc.

- It is as close as possible to the period covered by the certificate and consists of a corresponding period.

- It is produced domestically or within the market where consumption occurs.

ISO 14067:2018 lists European Guarantee of Origin (GOs) and US Renewable Energy Certificates (RECs) as electricity certificates that meet the above conditions. Electricity certificates that do not guarantee compliance with the above conditions shall not be used.

The electricity certificate should be accompanied by the following information:

- Name and location of the generating facility

- Method of generation

- Amount of electricity generated and amount of electricity certificates issued

- Tracking number

### Grid Mix and On-Site Electricity Production:

### For the average consumption grid mix, data should be sourced from the grid and read on the TSO (Transmission System Operator) or DSO (Distribution System Operator) counter.

### On-site electricity production can only be accounted for under the following conditions:

      - The production asset is owned by the same entity as the factory, or the asset has a direct connection to the factory and is not connected to the grid.

      - Only the fraction of electricity that is used on-site and not sourced from the grid should be included.

      - No credits can be claimed for electricity produced in excess and sent to the grid.

- Practitioners may adjust the emission factors to reflect actual activity conditions. Any adjustments, along with detailed explanations of the calculation methods or models used to revise emission factors, must be reported in the final document.

Future Changes in Energy Mix: Practitioners should also account for any potential changes in the fuel or electricity production pathways during the lifetime of the vehicle.

(detailed methodology shall be considered)

- The final report shall be verified by the relevant authority with either peer review or self-confirmation being requested.

Verification/certification process should be discussed within the other SGs