ODD-based Behavioural competences and Scenario Identification approach

- Introduction

This annex provides an overview on an approach that may be used to derive verifiable performance criteria for the certification or, as relevant, for self-certification of ADS, based on the manufacturer/ ADS developer’s description of the Operational Design Domain (ODD) of the ADS. Such criteria would be developed by identifying behavioural competencies that embody and correspond to specific ADS safety requirements and relevant scenarios that may be used to validate the ADS’s competencies.

The suggested approach includes a description of how such competencies can be classified into nominal, critical and failure categories and mapped to the relevant scenarios, selected either from existing databases or identified through the application of knowledge and data-based approaches.

Different approaches may exist to perform such an activity; therefore, the approach herein presented should be considered as a guideline for both manufacturers and authorities.

  - Operational Design Domain

Operational design domain (ODD) refers to:

Operating conditions under which a given driving automation system or feature thereof is specifically designed to function, including, but not limited to, environmental, geographical, and time-of-day restrictions, and/or the requisite presence or absence of certain traffic or roadway characteristics. (SAE J3016)

Given a specific ODD, it is crucial for the ADS to ensure that:

- it can operate safely within its ODD under conditions reasonably expected in the ODD

- it will be used only within its ODD

- it can monitor whether it is inside/outside its ODD and respond appropriately.

The conditions constituting the ODD in which the ADS was designed to operate will help determine which ADS competencies are required. For example, if an ADS has an ODD which comprises of roads with non-signalised junctions, one of the required behaviour competencies for the ADS in that ODD could potentially be “unprotected left or right turn”. However, the same behaviour competency may not be required if the ODD of an ADS is limited to motorways or highways with signalised junctions.

  - Behavioural competencies

The concept of “behavioural competencies” is useful in determining the safety of the performance of the Dynamic Driving Task (DDT) by an Automated Driving System (ADS):

- Behaviour: Specific goal-oriented actions directed by an engaged ADS in the process of completing the DDT or DDT fallback within the ODD (if applicable) at a variety of timescales.

- Behavioural Competency: Expected and verifiable capability of an ADS to operate a vehicle within the ODD of its feature(s).

Behavioural competencies can be described with different abstraction levels, similarly to functional, logical, and concrete scenarios. Refinement of the competencies from a functional to a more concrete level is possible by following the approach proposed in these guidelines.

Such competencies track the three broad categories of driving situations that may be encountered in performance of the DDT: nominal, critical, and failure.

Nominal driving situations are those in which behaviour of other road users and the operating conditions of the given ODD are reasonably foreseeable (e.g., other traffic participants operating in line with traffic regulations) and no failures occur that are relevant to the ADS’s performance of the DDT.

Critical driving situations are those in which the behaviour of one or more road users (e.g., violating traffic regulations) and/or a sudden and not reasonably foreseeable change of the operating conditions of the given ODD (e.g., sudden storm, damaged road infrastructure) creates a situation that requires a prompt action of the ADS to avoid or mitigate a collision. In this case, as it is recognised that in some cases the ADS may not be able to avoid a collision, the ADS performance are compared with safety model performance to set the threshold between where avoidance is required and where it is not feasible, but mitigation may be possible.

Failure situations involve those in which the ADS or another vehicle system experiences a fault or failure that removes or reduces the ADS’s ability to perform the DDT, such as sensor or computer failure or a failed propulsion system.

- Approach Description

The ODD-based behavioural competences and scenario identification approach is based on the interaction of the following elements:

- Behavioural Competencies and Scenario Identification

- Competencies and Scenarios Mapping

- Assumptions

- Performance Evaluation

Figure 1. Approaches to derive verifiable performance criteria

  - Behavioural Competencies Identification

The approach suggests a series of analytical frameworks that could help to derive measurable criteria appropriate for the specific application. These frameworks are divided into:

- ODD Analysis

- Driving Situation Analysis

- OEDR Analysis.

    - ODD analysis

This analysis represents the first step with the aim to identify the characteristics of the ODD. An ODD may consist of stationary physical elements (e.g., physical infrastructure), environmental conditions, dynamic elements (e.g., reasonably expected traffic level and composition, vulnerable road users) and operational constraints to the specific ADS application. Various sources provide useful guidance for precisely determining the elements of a particular ODD and their format definition.,, ,

As part of this activity, the level of detail of the ODD definition using the ODD attributes will also need to be established.

    - Driving situation analysis

In the driving situation analysis, the behaviours of other road users that are reasonably expected and the presence of roadway characteristics in the ODD are explored in more detail by mapping actors with appropriate properties and defining interactions between the objects.

An example of this analysis is given in Table 1, where static and dynamic behaviours of other objects (including other road users) that the ADS is reasonably expected to encounter within the ODD are described.  In the case of vehicles, this includes behaviours such as “acceleration”, “deceleration”, “cut-in”; for pedestrians, examples of dynamic behaviours include “crossing road”, “walking on sidewalk”, etc. Some of these behaviours may involve nominal situations  while others may involve critical scenarios(e.g., sudden cut-ins or unpredictable pedestrian or cyclist behaviour, including behaviours that may violate local traffic laws such as crossing a road outside a designated cross walk).

The behaviour of other road users and the condition of physical objects within the ODD may fall at any point along a continuum of likelihood. For example, deceleration by other vehicles may range from what is expected and reasonable in the traffic circumstances, to unreasonable but somewhat likely rapid deceleration, to extremely unlikely (e.g., a sudden cut-in combined with full braking on a clear high-speed road). The analysis of the ODD and reasonably expected driving situations within the ODD should make distinctions that include an estimate of the likelihood of situations to ensure that the ADS’s performance is evaluated based on response to reasonably likely occurrences involving nominal, critical and failure situations but not on the expectation that the ADS will avoid or mitigate the most extremely unlikely occurrences.

Table 1. Static / Dynamic elements and their properties

| Objects | Events/Interactions |
| --- | --- |
| Vehicles (e.g. cars, light trucks, heavy trucks, buses, motorcycles) | Lead vehicle decelerating, Lead vehicle stopped, Lead vehicle accelerating, Changing lanes, Cutting in, Turning, Encroaching opposite vehicle, Encroaching adjacent vehicle, Entering roadway, Cutting out, … |
| Pedestrians | Crossing road -inside crosswalk, Crossing Road – outside crosswalk, Walking on sidewalk / shoulder |
| Cyclists | Riding in lane, Riding in adjacent lane, Riding in dedicated lane, Riding on sidewalk/shoulder, Crossing road – inside/outside crosswalk, … |
| Animals | Static in lane, Moving into/out of lane, Static/Moving in adjacent lane, Static/Moving on shoulder, … |
| Debris | Statis in lane |
| Other dynamic objects (e.g. shopping carts) | Static in lane, Moving into/out of lane, … |
| Traffic signs | Stop, Yield, Speed limit, Crosswalk, Railroad crossing School zone, … |
| Vehicle signals | Turn signals |

    - Object and Event Detection and Response (OEDR) Analysis: Behavioural competency identification

Once the objects and their reasonably expected behaviours have been identified, it is possible to map the appropriate ADS response, which can be expressed as a behavioural competency. The detailed response is derived from more general and applicable functional requirements [ref to DDT section. The acceptable ADS response will vary depending on whether the driving situation involves nominal, critical, or failure characteristics.

The outcome of the analysis is a set of behaviour competencies that can be applied to the events characterizing the ODD. Table 2 provides a qualitative example of a matching event – response.

| Event | Response |
| --- | --- |
| Lead vehicle decelerating | Follow vehicle, decelerate, stop |
| Lead vehicle stopped | Decelerate, stop |
| Lead vehicle accelerating | Accelerate, follow vehicle |
| Lead vehicle turning | Decelerate, stop |
| Vehicle changing lanes | Yield, decelerate, follow vehicle |
| Vehicle cutting in | Yield, decelerate, stop, follow vehicle |
| Opposite vehicle encroaching | Decelerate, stop, shift within lane, shift outside lane |
| Adjacent vehicle encroaching | Yield, decelerate, stop |
| Lead vehicle cutting out | Accelerate, decelerate, stop |
| Pedestrian crossing road | Yield, decelerate, stop |
| Cyclist riding in lane | Yield, follow |
| Cyclist crossing road | Yield, decelerate, stop |

Table 2. Example of elementary behavioural competencies for given events.

The combination of objects, events, and their potential interaction, as a function of the ODD, constitute the set of nominal or critical scenarios pertinent to the ADS under analysis.

  - Scenario Identification

To ensure that the behavioural competences identified in the previous paragraphs are ready to be assessed through the application of simulations or physical testing, ODD-relevant scenarios must be developed. Scenario creation involves use of assumptions concerning the actions of road users that incorporate realistic parameters.

This approach suggests two complementary methodologies to derive reasonably expectable situations which might occur for a given ODD:

- Knowledge-based (e.g. goal-based)

- Data-based.

A knowledge-driven scenario generation approach utilizes domain specific (or expert) knowledge to identify hazardous events systematically and create scenarios. A data driven approach utilizes the available data (e.g. accident databases, insurance records) to identify and classify occurring scenarios. Figure 1 illustrates various data-based and knowledge-based scenario generation methods.

What are the near-miss events?

What are the potential causes of failures?

What are the known unsafe situations by regulations?

Accident databases

What are the known safe boundaries for the ADSs?

Real 
world data

Telematics Insurance claims

Analytical Hazard Based Approach

(STPA)

Formal Verification (Highway Code)

Operational Design Domain (ODD)

Real-world deployment and trials

Standards regulations guidelines

Ontology

What are the scenarios within a set of constraints?

What are the existing scenarios set out?

What unsafe situations do we know during trials?

What are the causes of known accidents?

Scenario library: Scenario Database

Scenario description language

Parameter identification & randomisation

Figure  SEQ Figure \* ARABIC 2. Data-based and knowledge-based scenario generation methods.

Accident datasets and field data can be analysed to identify accident hotspots and scenario parameters which contribute to causation of accidents carrying high levels of severity.

Knowledge based methods, or other formal techniques can be used to analyse the characteristics of the ADS architecture and identify system failures and hazardous situations [see SAE J3187]. The analysis is then converted into a set of abstract/logical scenarios together with their corresponding pass/fail criteria.

Other knowledge-based methods include the formal analysis approach with the highway code rules for scenario generation. Each of the highway code rules describes a hypothetical driving scenario with the corresponding behaviour and ODD elements. The ODD is a specification set out by the manufacturer of an ADS and it defines the operating conditions within which the ADS can operate safely. Formal models are generated via a model template to create the mathematical representations of those scenarios, collecting the combinations of ODD and behaviour parameters. The analysis reports the manoeuvre parameters that are close of violating the pass criteria and produce scenarios that represent these set of violations. Other knowledge-based methods use formal representation of the ODD and behaviour competencies of the ADS for scenario generation.

Furthermore, the existing scenarios already defined in the standards, regulations or guidelines (Option 6 - KB) can also be utilized for the testing of ADSs, for example the scenarios set out in ISO22737 and NCAP. ISO22737 has been developed for low-speed automated driving systems (LSAD) and the NCAP provides a set of testing scenarios for the safety assurance of vehicles. Option 7 (DB) includes the scenarios that occur during real world trials and deployments. Such scenarios might have not been considered pre-deployment but are key learnings.

  - Behavioural competencies and scenarios mapping

Once relevant scenarios and behavioural competencies have been identified, it is necessary to link them. The classification in the three broad categories of driving situations an ADS might encounter such as nominal, critical and failure, serves the purpose.

    - Nominal Situations Competencies

In these situations, ADS competencies can often be derived by applying traffic laws of the country where the ADS is intended to operate, as well as by applying general safe driving principles for situations not addressed adequately by current traffic laws for human drivers. Examples of such competencies may include adherence to legal requirements to maintain a safe distance from vehicles ahead, provide pedestrians the right of way, obey traffic signs and signals, etc. Of course, some nominal competencies (e.g., safe merging, safely proceeding around road hazards) may not be explicitly articulated or mandated by traffic laws. In some instances, traffic laws may provide wide discretion for the driver to determine the safest response to a particular situation (for example, how to respond to adverse weather conditions). As such not all traffic laws are stated with sufficient specificity to provide a clear basis for defining a competency.

Therefore, an approach to codify rules of the road to provide additional specificity was developed (see Appendix 1). Additionally, application of models involving safe driving behaviour may be needed in addition to reference to codified rules of the road in developing behavioural competencies for nominal driving situations.

Table 3. Example of competencies and scenario mapping in nominal situation

| ODD Element | Driving Behaviour | Traffic Rule | DDT Requirement | Behaviour Competency | Test Scenario |
| --- | --- | --- | --- | --- | --- |
| Bicycle | Riding in lane |  | The ADS shall adapt its driving behaviour to reduce safety risks | The ADS ensures relative velocity during passing manoeuvre does not exceed [30] km/h | The ADS travels between [30–50]km/h on the centre line of its lane A cyclist travels in the same direction as the ADS between [10–20] km/h, [0.2–1] m away from the lane edge |
|  |  | Drivers will need to use a minimum passing distance for bicycles of 1.5m in urban areas, and 2m out of town | The ADS shall comply with traffic rules in accordance with application of relevant law within the area of operation. | The ADS shifts in lane to pass by cyclist with 1.5.m lateral distance |  |
|  |  |  | The ADS shall avoid unreasonable disruption to the flow of traffic in line with safety risks. | The ADS crosses the centre lane marking to ensure the safe passing distance is not violated |  |
|  |  |  | The ADS shall interact safely with other road users | The ADS activates the turn signal if the centre lane marking is crossed |  |

    - Critical Situations Competencies

The development of these competencies requires analysis of (1) what constitutes such unreasonable behaviour by ORUs and/or a sudden change of the operating conditions that are not reasonably foreseeable and (2) what constitutes an appropriate ADS response to avoid or mitigate the imminent crash. Additionally, it is also important to identify the occurrence of unplanned emergent behaviour in critical situations.

Analysis of the first type may be based on a variety of methodologies, including e.g. IEEE 2846-2022 (which offers guidance on what behaviours by other road users are reasonably foreseeable) and other models of reasonable driving behaviour. Analysis of the second factor may be based on various models of acceptable human driving behaviour in crash imminent situations.

Hazard identification methods (e.g. STPA as mentioned in SAE J3187) which analyse the system design for functional and operational insufficiencies can help identify the occurrence of emergent behaviour which may lead to critical situations.

Development of behavioural competencies for critical driving situations faces several challenges. No general consensus exists on the appropriate models for the behaviour of ORUs or appropriate responses by the ADS to unreasonable ORU behaviours that make a crash imminent.

Table 4. Example of competencies and scenario mapping in critical situation

| Losses | Hazards | Unsafe Control Action | Loss scenario | Causal factors | Test behaviour | Test Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| Collision with object outside the vehicle | ADS does not maintain a safe distance from the lead motor vehicle | Braking demand is not provided | Object in vehicle trajectory is not detected | Undetected/misclassified object; Obscured object; Incorrect sensor fusion result | The ADS is following behind a lead vehicle, with the headway set by the ADS. The lead vehicle decelerates at the max assumed rate depending on the weather conditions | Lead vehicle decelerated to turn [right/left] or travel straight on a [mini / large] roundabout |
|  |  |  | Object is not considered to be in the vehicle trajectory | Localisation issues leading to incorrect positioning of ego vehicle or object |  | Lead vehicle decelerated whilst shifting lane to avoid a [static object/other road user] |

    - Failure Situations Competencies

The ADS safety requirements include management of various failure modes. As noted above, failure situations scenarios involve those in which the ADS or another vehicle system experiences a fault or failure that removes or reduces the ADS’s ability to perform the DDT, such as sensor or computer failure or a failed propulsion system.

In developing the behavioural competencies appropriate for failure situations, the objective is to describe the ability of the ADS to detect and respond safely to specific types of faults and failures. Depending upon the nature and extent of the fault or failure, the responses can include identifying a minor fault for immediate repair after trip completion, responding to a significant fault with restrictions (such as limp-home mode) for the remainder of the trip, or responding to major failures by achieving a minimal risk condition. Communication of the fault or failure condition to vehicle users may also be a desirable ADS behavioural competency.

Table 5. Example of competencies and scenario mapping in failure situation

| Failure Type | Failure Mode | Potential Cause | Response | DDT Requirement | Test Scenario | Pass / Fail Criteria |
| --- | --- | --- | --- | --- | --- | --- |
| Perception | Fail to identify ODD boundary | Failure to detect ODD attribute e.g. heavy rain/fog | Safely stop in lane of travel | The ADS shall recognise the conditions and boundaries of the ODD of its feature(s) | The ADS operates beyond the predicted ODD | The ADS detects the ODD conditions are not met and issues a minimal risk manoeuvre |
|  |  |  |  | In response to a fault, the ADS shall execute a fallback response and prohibit activation of the impacted feature(s) if the fault prevents the ADS from performing the DDT in accordance with the requirements of 5.2., |  | The minimum risk manoeuvre should not cause the vehicle to decelerate greater than [4]m/s2 |

  - Assumptions: Logical to concrete behavioural competencies

Concrete performance requirements depend on the specific situations the ADS encounters, on a reference behaviour that is deemed appropriate for a human driver or a technical system, and on assumptions (e.g. cut-in speed values, reaction times, …) about the behaviour of the vehicle and other road users. Assumptions concerning the actions of other road users may need to account for cultural differences in driving styles in different geolocations, making it impracticable to harmonise these assumptions across different domains. Therefore, evidence should be provided to support the assumptions made. Existing standards e.g. IEEE 2846-2022 provide a set of assumptions to be considered by ADS safety-related models for an initial set of driving situations. Additionally, several other tools including data collection campaigns performed during the development phase, real-world accident analysis and realistic driving behaviour evaluations, constraint randomisation, Bayesian optimisation besides others can be used to inform values for such assumptions.

  - Performance Evaluation - Acceptance Criteria

As previously highlighted, nominal situations are considered reasonably foreseeable and preventable for a given ODD and therefore it is expected that the ADS would be capable of handling them without any resulting collision.

On the other hand, failure situations are performed to assess the ADS ability to recognise faults/failures in the system.

For the purpose of defining performance criteria in critical situations, those where others are at fault, behaving unforeseeably, and the collision might potentially not be prevented have to be analysed further. In these situations, it is proposed that safety models are used to explore and compare the ADS performance with mathematical formulations to derive what is deemed as preventable or where mitigation strategy is needed.

    - Application of Rules of Road

An approach to define an acceptance criterion related to nominal driving situations is to evaluate the ADS performance against the rules of the road. Furthermore, ADS safety requirements state that “The ADS shall comply with traffic rules in accordance with application of relevant law within the area of operation.” It is challenging to test against this requirement in the absence of codified rules of the road.

Appendix 1 of this annex provides a framework for codifying the rules of the road that govern the behaviour of ADS. The approach may be used to define “good behaviour” to inform validation and verification processes (including for scenario-based testing) for nominal scenarios.

Using rules of the road as pass criteria

Figure 3 illustrates the use of codified rules of the road as a pass criterion for scenario-based testing activities. Every test scenario definition will have ODD and behaviour competency attributes defined. Every rule of the road will also have ODD and behaviour competency attributes as part of its definition. Therefore, it is possible to map every scenario to a corresponding rule(s) of the road using ODD and behaviour tags or labels in a scenario catalogue.

Scenario

Behavioural Competency

ODD Attributes

Test case

Metrics

Completeness

ODD-based Rules of the Road

Scenario Database

Scenario Database

Figure  SEQ Figure \* ARABIC 3. Rules of the road as pass/fail criteria.

This approach would allow the test engineer to map each scenario to a corresponding rule (or set of rules). These rules can then serve as the pass criteria during the scenario-based testing approach. This approach can thus enable engineers and authorities to show/assess compliance to traffic rules by making the rules of the road verifiable.

    - Use of Safety Models

Since it is virtually impossible to write a regulation that sets out verifiable criteria for every combination of these variables, this document aims at providing a set of different reference behaviours or safety models together with an overview of the characteristics and required assumptions that can be useful in deriving verifiable performance criteria in some situations. The aim is then to assist those who develop concrete regulations with the selection and parameterization of functions or selection of scalars as pass/fail criteria.

For this, the following is needed:

- An overview of reasonable expectations (which might occur in different ODDs),

- An overview of reference behaviours / safety models that define the boundary between avoidable accidents and mitigation (note that these reference behaviours will not be used for anything else than providing this boundary as a performance criterion).

- A matrix combining suggested reference behaviours / safety models with driving situations.

Even though behavioural competencies will help the automated vehicle to not cause accidents or drive defensively to stay away from conflicts, there are situations where collisions might not be prevented, for example where other traffic participants are at fault or behaving unforeseeably.

It is the task of the automated driving system – like it is the task for human drivers – to perform evasive actions, whether it is possible and reasonable in order to minimize risk.

For this, simple logic models, the so-called safety models, are introduced. They provide assumptions how traffic rule violations and misbehaviour by other traffic participants could be dealt with and use physical properties and fundamental driving dynamics to further detail conditions for accident avoidance.

The set of safety models described in this document should be regarded as a set of tools, whereas selecting the right tool (the right safety model) depends on the boundary conditions. Hence in this document, there exists no preference for any of the safety models being introduced.

Two important points to consider: safety models are a methodology to support considerations on collisions that have to be avoided and those where only mitigation is required. The aim is NOT to prescribe a specific behaviour of the ADS in any given critical situation.

safetymodelsafetymodel

Choosing appropriate model(s) depends, amongst others, on:

- the balance between risk to the ADS itself vs. risk towards the accident partner (e.g. for pedestrians, it would very likely be acceptable to have a slightly increased risk for the typically belted ADS occupants when the risk for the pedestrian would be significantly reduced, e.g. by earlier or stronger brake intervention; for unmanned ADS similar risk balance considerations have to be done),

- the assumed anticipation level (e.g. is it feasible to anticipate actions of other traffic parameters and start countermeasures earlier, or will it be a simple reaction to faults),

- the environmental condition parameters. (e.g. what level of friction is typically available where the ADS are travelling),

- the balance between efficiency and acceptable remaining risk (e.g. passing a pedestrian with no acceptable risk would be possible only with very low speeds, which would render the current sidewalk close to streets infrastructure useless for automation).

The safety models can be grouped into models for the performance in accident avoidance and behaviour models for conflict avoidance, see Table 3. The difference between those two is that the accident avoidance models can be used to understand to what extent accident situations – caused by other traffic - are unavoidable, while conflict avoidance models formalize strategies for the behaviour of an ADS to not come into conflict. Conflict avoidance models are better suited being integrated into the document on the dynamic driving task.

Table 3. Overview of Safety Models*

| Model | Explanation |
| --- | --- |
| Performance Requirements for Accident Avoidance |  |
| Last Point to Steer | Estimate avoidance and mitigation in longitudinal traffic, typically used for driver assistance & active safety |
| Safety Zone | Estimate avoidance and mitigation in cross-traffic accidents with VRU |
| Careful and Competent Human Driver | Estimate avoidance and mitigation in longitudinal traffic cut-in situations, using reaction characteristics of good human driver |
| Fuzzy Surrogate Safety Model | Estimate avoidance and mitigation in longitudinal traffic cut-in situations, taking anticipation of other vehicle behaviour into account |
| *Models discussed during guidelines development and not intended as an exhaustive list. |  |

Annex I - Use-case for Nominal, Critical and Failure Situations Mapping

Include final tables

[Annex 1—Appendix 1

Codification methodology for rules of the road

Current rules of the road (for human drivers) have three components:

| Rule of road (for human drivers) | = Operating condition + Behaviour competency + Assumptions (implicit) |
| --- | --- |

Operating conditions include both ODD aspects and vehicle states (e.g., system failures, hardware failures etc.). Every set of traffic laws or behaviour rules (for human drivers) defined in any country are based on an understanding of the expected behaviours of human drivers. As a result, they do not explicitly define all aspects of the expected driving behaviour but can be argued to include “implicit assumptions” based on this understanding.

| Codified Rule of road | = Operating condition + Behaviour competency + Driving decisions |
| --- | --- |

Following the process (illustrated in section 8.1), a “codified” rule of the road for an automated driving system, will also have three components:

The process of codification helps identify where “implicit assumptions” about driving behaviour are present in the rules for human drivers. The codified rules of the road help to turn “undefined” attributes in the rules of the road (for human drivers) to “defined” attributes in the codified “rules of the road”.

Taking an example of the UK road rules where behaviour (for human drivers) is governed by the Highway Code (HC), the methodology is further explained. UK’s Highway Code Rule 195 states (Zebra crossing):

Rule 195: “As you approach a zebra crossing: look out for pedestrians waiting to cross and be ready to slow down or stop to let them cross; you MUST give way when a pedestrian has moved onto a crossing.”

Figure 3: Example of zebra crossing from UK's Highway Code:

Source: https://www.gov.uk/guidance/the-highway-code/rules-for-pedestrians-1-to-35#rule19

From this rule, one can extract the “operating condition or ODD” variables, as well as the behaviour competencies. “Zebra crossing” and “pedestrian” define the operating condition; and “slow down or stop” defines the behaviour competency. However, the rule doesn’t mention for how long the vehicle should be stopped, or when it is considered safe to proceed again. There is an “implicit assumption” made based on typical human (the driver behaviour), and it is not considered necessary for the rule to define this. However, for an ADS, such assumptions how long the vehicle is stopped for, and when it moves off again will be determined by the automated driving system and its analysis of the relevant parameters specific to that situation and will need to be specified.  For every concrete scenario being tested, the driving decisions exhibited by ADS will need to be explainable.

Figure 4 illustrates this process. After following the codification process of defining the “rules of the road”, there will be no underlying “assumptions” (see Codification methodology below).

Codified

Rule of the Road

=  f(Operating condition, Behaviour competency, driving decision)

=  f(Operating condition, behaviour competency, driving characteristics)

Applying the proposed process

Current Rules of Road

(for human drivers)

Table  SEQ Table \* ARABIC 4. Converting current rules of the road (for human drivers) to codified rules for ADS.

Furthermore, for all areas or jurisdiction or country, there will be a minimum set of behaviour code rules which will have consistent “driving characteristics” – the base or common set of rules of the road (for ADS).

Codification methodology

The codification methodology is a four-step process:

- Step 1: Identify terms and construct a vocabulary: The natural language text of the rule is analysed and words that are associated with the ODD or behaviour of actors in the rule are identified. These terms taken together are used to identify the component of the rule that can be codified.

- Step 2: Identify unspecified terms: Some terms are unclear because they are not unequivocal or absolute and therefore require clarification. In some cases, these terms are codified as is, when a meaning can be inferred, while in others, comments are provided to highlight why the terms are not defined, and how they may be elaborated.

- Step 3: Query / Update/ Add ODD and Behaviour terms: Terms defining predicates (representing facts whose truth may be evaluated) and functions (representing non-Boolean properties – such as ADS attributes, action labels) are identified. The codified rule will consist of these predicates and functions. The outcome of Step 3 is an intermediate rule that is in its minimal form.

- Step 4: Express rule in first order logic: For each rule of the road, a single codified rule, or a set of rules are written. The predicates and functions identified in Step 3, together with the structure of constraints from Step 1 are used to construct the rule(s). The output of Step 2 provides insights concerning the rule and gaps that exist in its codification. Step 4 uses the vocabulary to identify which sub-rules are to be converted to First Order Logic and then perform the conversion.

Vienna Convention codification example

The Vienna convention rule is stated below (Chapter 2 – Rules of the Road – Article 11 (Overtaking – 11)).

Vienna Convention Rule Text:

A vehicle shall not overtake another vehicle which is approaching a pedestrian crossing marked on the carriageway or signposted as such, or which is stopped immediately before the crossing, otherwise than at a speed low enough to enable it to stop immediately if a pedestrian is on the crossing.

The following sections take this rule through each step, explaining how each component of the codification process works.

Step 1: Identify Terms and Construct a Vocabulary

The rule is re-stated below highlighting important terms:

A vehicle shall not overtake another vehicle which is approaching a pedestrian crossing marked on the carriageway or signposted as such, or which is stopped immediately before the crossing, otherwise than at a speed low enough to enable it to stop immediately if a pedestrian is on the crossing.

Terms that are ODD and behaviour related are in bold and underline, while other terms that are relevant to giving the rule meaning are in bold.

Step 2: Identify Unspecified Terms

From the example above, the terms that remain underspecified are as follows:

| Term | Specification Required |
| --- | --- |
| Immediately | How is immediately defined? A distance may be used to define this. |
| Low enough | What speed is considered low enough? This could be a function of distance to the pedestrian, or an absolute threshold. |
| *Overtaking is an action that is applicable to vehicles that are ahead of the ego* | This is an assumption that is understood by a human reader. |

Step 3: Identify Predicates and Functions

The non-highlighted terms are removed and only terms that are important to the meaning of the rule are kept.

Shall not overtake another vehicle

- approaching pedestrian crossing on carriageway or signposted,

- or stopped immediately before crossing,

otherwise speed low enough enable stop immediately if pedestrian on crossing.

| Predicate | Description |
| --- | --- |
| isEgo(x) | x is the Ego |
| canOvertake(x,y) | x can overtake y |
| isApproaching(x,y) | x is approaching y |
| isPedestrianCrossing(x) | x is a pedestrian crossing |
| isCarriageway(x) | x is a carriageway |
| isSignposted(x) | x is signposted |
| isStopped(x) | x is stopped |
| isAhead(x,y) | x is ahead of y |
| hasSpeed(x,y) | x has speed y |
| isLowEnoughSpeed(x,y) | x is a low enough speed for action y |

The terms identified are converted into predicates. For the VC Rule, we construct the following predicates:

Step 4: Express Rule in First Order Logic

The rule determines overtaking behaviour for a vehicle that is close to a pedestrian crossing. The rule contains conditions that would prevent a vehicle from overtaking another, but simultaneously provides an exception, that of being slow enough to stop. Further, the ability of the vehicle to stop is independent of whether there is an actor (such as a pedestrian) on the crossing. The rule makes references to the vehicle having a slow enough speed to stop immediately, which has been identified as an ambiguous phrase and represented as a predicate in Step 3. To represent the action of stopping immediately, we use the constant “STOP_IMM”.

For ease of understanding, the rule may be broken down into four logical statements, that are logically related, with the relationship being stated as the last rule. The predicates that were produced as an outcome of Step 1 are used to construct the logic specification for the rule.

The parameters for the rules: the ego vehicle (x), the other actor (y), the pedestrian crossing (w), the carriageway (c), the speed of the ego (s).

The rules are as follows:

| Rule (a): | isEgo(x) ⋀ isOtherRoadUser(y) | x is the ego and y is the other vehicle |
| --- | --- | --- |
| Rule (b): | isPedestrianCrossing(w) ⋀ (isCarriageway(c) V isSignposted(w)) | w is a pedestrian crossing and (c is a carriageway or w is signposted) |
| Rule (c): | isApproaching(y,w) V isAhead(w,y) | y is approaching w, or w is ahead of y |
| Rule (d): | hasSpeed(x,s) ⋀ ¬isLowEnoughSpeed(s,STOP_IMM) | x has speed s, and s is not a low enough speed to stop immediately. |
| The Rule | (a) ⋀ (b) ⋀ (c) ⋀ (d) → ¬canOvertake(x,z) |  |

The symbol “¬” when used as a prefix to a predicate indicates the negation of the predicate. In this context, in English, the rule may be read as: If “a” is true, and “b” is true, and “c” is true, and “d” is true, then x cannot overtake z. Note that the exception condition, that of being slow, is used in its negative form to assert that the vehicle cannot overtake, since this is explicit in the rule. It is left to interpretation if a positive rule, specifically allowing the vehicle to overtake is necessary. If so, a new rule that allows a vehicle to overtake must be written. This would depend on the interpretation of the rule. ]

; E.g., AVSC Best Practice for Describing an Operational Design Domain:  Conceptual Framework and Lexicon; and A Framework for Automated Driving System Testable Cases and Scenarios (NHTSA).

E.g. BSI PAS 1883:2020 Operational Design Domain (ODD) taxonomy for an automated driving system (ADS) - Specification

ASAM OpenODD

Road Vehicles — Test scenarios for automated driving systems — Taxonomy for operational design domain