|  |  | GRBP-80-xx |  |
| --- | --- | --- | --- |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Noise

Eightieth session

Geneva, September 2024

Item 2 of the provisional agenda

UN Regulation No. 41 (Noise Emissions of motorcycles)

Proposal for a supplement to the 05 series of amendments to UN Regulation No. 41 (Noise Emissions of motorcycles)

Submitted by the experts from the IWG RD-ASEP

The text reproduced below was prepared by the experts from the IWG RD-ASEP in order to clarify the application of the Annex 7 ASEP testing conditions of motorcycles. It is based upon the 05 Series of Amendments to UN Regulation No. 41, up to Supplement 3. The modifications UN Regulation No. 41 are marked in bold for new or strikethrough for deleted characters.

- Proposal

Annex 3, insert a new paragraph 1.3.3.1.3. to read:

        - Manufacturer documentation is provided to the technical service responsible for conducting approval tests. Based on this documentation, the technical service selects the software programs or modes that shall be used. The use of the highest Lwot(i) of a mode which does not restrict the vehicle power [performance] is recommended.

Annex 7, amend paragraph 2.5.  to read:

  - ASEP control range

The requirements of this annex apply to any vehicle operation with the following restrictions:

- vAA' shall be at least 10 km/h

- vBB' shall not exceed 80 km/h for vehicles with PMR≤150

vBB’ shall not exceed 100 km/h for vehicles with PMR>150

- nAA' shall be at least 0,1 * (S – nidle) + nidle

- nBB' shall not exceed 0,8 × S

The values for the RD-ASEP control range shall be seen as absolute values and shall not be increased or lowered by addition or subtraction of the tolerance for vtest as indicated in paragraph 3.3.12.8. of this Annex.

Annex 7, amend paragraph 2.6.  to read:

  - RD-ASEP limits

The maximum noise level recorded during the passage of the motorcycle through the test track shall not exceed:

Lwot,(i) + (1 * (nPP' – nwot,(i)) / 1,000) + 3  	for nPP' < nwot,(i) and

Lwot,(i) + (5 * (nPP' – nwot,(i)) / 1,000) + 3	 for nPP' ≥ nwot,(i)

Where Lwot,(i) and nPP' have the same meaning as in Paragraph 1. of Annex 3 and nwot,(i) refers to the corresponding engine speed when the front of the vehicle passes the Line PP'.

If the tests according to Annex 3 of this UN Regulation and the RD-ASEP tests are performed with the same vehicle in immediate sequence, the values for Lwot(i) and nwot(i) from the Annex 3 test may be used, if agreed by the type approval authority. Otherwise, when compliance with these limits is checked, values for Lwot(i) and nwot(i) shall be newly determined by measurements as defined in Paragraph 1. of Annex 3, however using the same gear (i), the same user selectable drive mode and the same pre-acceleration distance as during type approval.

Annex 7, insert a new paragraph 2.8. to read:

  - Tolerances

For tests according to paragraphs 3.2.2. (b) and 3.3. of this Annex, a tolerance of +/- 5km/h shall apply with a minimum speed of 10 km/h and a maximum one of 100 km/h.

Annex 7, amend paragraph 3.1. to read:

  - General

The Type Approval Authority as well as the technical service shall request tests to check the compliance of the motorcycle with the requirements of paragraph 2 above. To avoid undue workload, testing is restricted to the reference points defined in paragraph 3.2 below and three the additional operating conditions as defined in paragraph 3.3 of this Annex per gear.

For vehicles with manual transmissions, The the total number of operating conditions to be tested according to paragraph 3.3 of this Annex is defined by the number of gears of the vehicle multiplied by 3, and shall be reduced by the number of operating conditions which were applied for tests according to paragraph 3.2 of this Annex and for the determination of Lurb according to Annex 3.

For vehicles with variable gear ratios or automatic transmission with non-lockable gear ratios testing shall be limited to 6 operating conditions as defined in paragraph 3.3 of this Annex, and different from the operating conditions which were applied for the determination of Lurb according to Annex 3. the total number of operating conditions to be tested according to paragraph 3.3 of this Annex is defined as 6, and reduced by the number of operating conditions which were applied for tests according to paragraph 3.2 of this Annex and for the determination of Lurb according to Annex 3.

Notwithstanding the above, a minimum of [3] operaring conditions shall be tested according to paragraph 3.3 of this Annex.

Annex 7, amend paragraph 3.3.1. to read:

    - Test procedure

The vehicle shall approach the line AA' at constant speed or in acceleration or deceleration, according to the throttle operation which may be requested by the technical service responsible for conducting approval tests in agreement with the type approval authorities.

The approach velocity shall be chosen as such that the vehicle reaches a prescribed test speed vtest +/– 5 km/h when its front passes the line AA'.

Examples: (for applying the tolerance as specified in 2.8. of this Annex)

requested vtest=10 km/h → valid vAA’=10-15 km/h

requested vtest=15 km/h → valid vAA’=10-20 km/h

requested vtest=75 km/h → valid vAA’=70-80 km/h

requested vtest=95 km/h → valid vAA’=90-100 km/h

requested vtest=100 km/h → valid vAA’=95-100 km/h

When the front of the vehicle passes the line AA' the throttle control shall be adjusted as rapidly as possible to a position (partial throttle, wide open throttle or maintain present throttle control position) defined by the technical service responsible for conducting approval tests and shall be kept in this position until the rear of the vehicle passes line BB’.

When the rear of the vehicle passes line BB' the throttle control shall be shifted to the idle position as rapidly as possible.

The throttle position between lines AA’ and BB’ shall be such that the speed of the vehicle remains constant with a tolerance of +/- 2.5 km/h between lines AA’ and BB’.

Annex 7, amend paragraph 3.3.2. to read:

    - Test speed, gear and mode selection and throttle operation

The conditions of this paragraph may shall be defined by the technical service responsible for conducting the approval tests.

To assist defining these conditions, manufacturer documentation is provided to the technical service responsible for conducting approval tests. Pre-testing of the vehicle by the manufacturer can be requested by the technical service.

The test speed vtest may be any speed within the RD-ASEP control range as defined in paragraph 2.5 of this Annex. The vehicle may be tested in any of the available gears, including 1st gear.

The Within the total number of additional operating conditions defined in paragraph 3.1, the vehicle may be tested in any of the available user selectable software programs or modes.

The throttle operation shall be in accordance with paragraph 3.3.1 of this Annex.

The throttle operation before line AA’ and between lines AA’ and BB’ shall be defined and described in a way that it can be performed by a skilled rider who has made himself familiar with the riding characteristics of the test vehicle and that the correct execution can be assessed by observation without the necessity of technical equipment on the vehicle or at the test site other than the equipment which is required for the tests according to Annex 3.

If the requested operating conditions lead to an unusual vehicle behaviour (i.e. front wheel lift up, apparent wheel spin, chain slap, engine lugging) or any other riding condition which may not be expected to occur when the vehicle is operated in real traffic, that test run shall be discarded.

- Justification

1. During the application of the Annex 7 ASEP tests of the 05 Series of Amendments to UNECE Regulation No 41, IMMA noticed that Technical Services and Type Approval Authorities had varying interpretations on how to test different user selectable riding modes, on how to determine the number of additional operating conditions and on which speed tolerances to apply for determining deceleration and for the reference test condition of paragraph 3.2.2. (b). With this amendment proposal the IWG RD-ASEP would like to clarify the original intent of the 05 Series of Amendments as well as ensuring uniform application of the ASEP tests.

2. With user selectable riding modes becoming more available on motorcycles, a new Paragraph 1.3.3.1.3 is added in Annex 3, to clarify that the Annex 3 tests should be performed in a mode that does not restrict the vehicle’s power, and that has the highest Lwot(i) . This is relevant to specify as the Lwot(i) value will appear on the manufacrurer’s statutory plate and will be used to determine the ‘anchor point’ for the ASEP limit line in Annex 7.

In Annex 7 paragraph 2.6 it is specifiied that when the Lwot(i) and nwot(i) need to be newly determined, the same user selectable driving mode should be used as described in the paragrah above.

3. A new paragraph 2.8 is included to specify the speed tolerances allowed during the Annex 7 tests. Such speed tolerances were already specified for the vAA’in paragraph 3.3.1, but were lacking for the reference test condition of paragraph 3.2.2.(b).

For the determination of deceleration in the context of RD-ASEP testing (paragraph 3.3.1) a tolerance of -5 km/h is specified.

4. Paragraph 3.1. is rephrased to better describe how to determine the total number of additional operating conditions that should be performed for the RD-ASEP tests, now clearer separating vehicles with manual transmissions from vehicles with variable gear ratios or automatic transmissions with non-lockable gear ratios. To ensure a sufficient number of additional operating conditions, a minimum number of tests is specified.

In paragraph 3.3.2. it is clarified that the additional operating conditions can be performed in any of the available user selectable driving modes, but these operating conditions must fit within the total number of additional operating conditions.

5. Also in paragraph 3.3.2, a paragraph is added for assisting the Technical Service in defining the RD-ASEP operating conditions with either manufacturer documentation or pre testing of the vehicle without this counting as a reduction in the total number of test runs.