|  | United Nations | ECE/TRANS/WP.29/GRPE/2024/xx |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General xx October 2024 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Ninety 1st session

Geneva, xx October, 2024

Light vehicles: UN Regulations No. 154 (Worldwide harmonized Light vehicles

Test Procedures (WLTP))

Proposal for Supplement 2 to the 02 series of amendments of UN Regulation No. 154 (Worldwide harmonized Light vehicles Test Procedures (WLTP))

Submitted by the experts from Japan

The text below is a proposal for Supplement 2 to the 02 series of UN Regulation No. 154 on uniform provisions concerning the approval of light duty passenger and commercial vehicles with regards to the test requirements and methodologies for PEVs, PHEVs and FCHVs  ( adding one for FCHVs as well as PEVs ), mainly revised in level 1B. The modifications to the current text of the Regulation are marked in bold.

I.	 Proposal

Paragraph 2, add new abbreviation:

2. Abbreviations

2.1. General abbreviations

| PLL | Lower limit pressure |
| --- | --- |

Paragraph 6.3, add new sub-paragraph:

6.3.12.	Lower limit pressure family for NOVC-FCHVs

Only NOVC-FCHVs that are identical with respect to the following specifications may be part of the same family of :

- Operation strategy of all components determining the lower limit pressure of on-board hydrogen tank (pressure value set to initiate shutdown of fuel supply system etc.).

- If no specific operation strategy determining the lower limit pressure of on-board hydrogen tank, all items of interpolation family listed in paragraph 6.3.2.4.

Paragraph 3.5.7.6.4., Annex A1, add new sub-paragraph:

3.5.7.6.4. Fuel efficiency and driving range for NOVC-FCHV

3.5.7.6.4.1. Vehicle high: ... km/kg

3.5.7.6.4.2. Vehicle low (if applicable): ... km/kg

3.5.7.6.4.3. Vehicle M (if applicable): ... km/kg

3.5.7.6.4.4. Driving range (if applicable):…. km

Paragraph 2.1.1.4., Annex A1 – Appendix 1, add new sub-paragraph:

2.1.1.4.3.	Ranges for NOVC-FCHVs - driving range (if applicable)

Test 1

| PLL, Lower limit pressure (MPa) | － |
| --- | --- |
| Measured value PLL |  |
| Declared value PLL |  |

Test 2 (if applicable)

Record test results in accordance with the table of Test 1

Test 3 (if applicable)

Record test results in accordance with the table of Test 1

Conclusion

| PLL (MPa) | － |
| --- | --- |
| Final value PLL |  |

| Driving range (km) | Combined |
| --- | --- |
| Calculated value |  |

Paragraph 2.5.4., Annex A2, amend to read:

2.5.4.	Not Off Vehicle Charging Fuel Cell Hybrid Vehicles (NOVC-FCHV)

| Fuel Consumption (kg/100 km) or Fuel Efficiency (km/kg)(1) and lower limit pressure of hydrogen (if applicable) (MPa) | Combined |
| --- | --- |
| Final values FCc or FEc |  |
| Final value PLL |  |

Repeat 2.5.4. in case of base vehicle.

Paragraph 1.2.3.2., Annex B6, amend to read:

1.2.3.2.	Depending on the vehicle type, the manufacturer shall declare as applicable the total cycle values of the CO2 emission, the electric energy consumption, fuel consumption, fuel efficiency as well as PER, AER and PLL according to Table A6/1.

Table A6/1, Annex B6, amend to read:

Table A6/1

Applicable rules for a manufacturer’s declared values (total cycle values)(a) (as applicable)

| Powertrain |  | Level 1A MCO2 (b) (g/km) | Level 1A: FC (kg/100 km) | Level 1B; FE (km/l or km/kg) | Electric energy consumption(c) (Wh/km) | All electric range / Equivalent all-electric range/ Pure Electric Range (c) (km) | Level ;1B Lower limit pressure (MPa) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicles tested according to Annex B6 (pure ICE) |  | MCO2 Paragraph 3. of Annex B7. | - | FE Paragraph 1.4. of Annex B7. | - | - |  |
| NOVC-FCHV |  | - | FCCS Paragraph 4.2.1.2.1. of Annex B8. . | FECS Paragraph 4.2.1.2.1. of Annex B8. | - | - | PLL Paragraph of 4.8.1 of Annex B8 |
| OVC-FCHV | CD | - | FC,CD | N/A | ECAC,CD | AER |  |
|  | CS | - | FCCS | N/A | - | - |  |
|  | CD/CS weighted | - | - | - | - | EAER Paragraph 4.4.6.1. of Annex B8 |  |
| NOVC-HEV |  | MCO2,CS Paragraph 4.1.1. of Annex B8. | - | FECS Paragraph 4.1.1.1. of Annex B8. | - | - |  |
| OVC-HEV | CD | MCO2,CD Paragraph 4.1.2. of Annex B8. | - | FECD Paragraph 4.6.1. of Annex B8. | For Level 1A: ECAC,CD Paragraph 4.3.1. of Annex B8. | AER Paragraph 4.4.1.1. of Annex B8. |  |
|  | CS | MCO2,CS Paragraph 4.1.1. of Annex B8. | - | FECS Paragraph 4.1.1.1. of Annex B8. | - | - |  |
|  | CD/CS weighted | - | - | - | For Level 1B: EC Paragraph 4.6.2. of Annex B8 | EAER Paragraph 4.4.4.1. of Annex B8 |  |
| PEV |  | - | - | - | ECWLTC Paragraph 4.3.4.2. of Annex B8. | PERWLTC Paragraph 4.4.2. of Annex B8. |  |

(a) The declared value shall be the value to which the necessary corrections, as applicable, are applied

(b) Rounding to 2 places of decimal according to paragraph 6.1.8. of this Regulation

(c) Rounding to one place of decimal according to paragraph 6.1.8. of this Regulation

Table A6/2, Annex B6, amend to read:

Table A6/2

Criteria for number of tests

For NOVC-FCHVs and OVC-FCHVs in CS condition (as applicable)

|  | Test | Judgement parameter | For Level 1A: FCCS | For Level 1B: FECS | For Level 1B: PLL |
| --- | --- | --- | --- | --- | --- |
| Row 1 | First test | First test results | ≤ Declared value × 1.0 | ≥ Declared value × 1.0 | ≤ Declared value × 1.0 |
| Row 2 | Second test | Arithmetic average of the first and second test results | ≤ Declared value × 1.0 | ≥ Declared value × 1.0 | ≤ Declared value × 1.0 |
| Row 3 | Third test | Arithmetic average of three test results | ≤ Declared value × 1.0 | ≥ Declared value × 1.0 | ≤ Declared value × 1.0 |

Paragraph 3.5.3., Annex B8, amend to read:

3.5.3.	Type 1 test procedure

3.5.3.1.	Vehicles shall be tested according to the Type 1 test procedure described in Annex B6 and fuel consumption calculated according to paragraph 1 and paragraph 2 of Appendix 7 to this annex.

3.5.3.2.	If required, fuel consumption shall be corrected according to Appendix 2 to this annex.

3.5.3.3.	Driving range of hydrogen (DRH) for NOVC-FCHV

3.5.3.3.1.	The lower limit pressure of hydrogen tank shall be measured for NOVC -FCHVs according to paragragh 3 of appendix 7 to this annex.

3.5.3.3.2.	Usable amount of hydrogen (UAH) shall be calculated according to paragraph 3 of appendix 7 to this annex.

3.5.3.3.3.	Driving range of hydrogen shall be calculated with fuel efficiency and usable amount of hydrogen.

Paragraph 4., Annex B8, add new paragraph:

4.8	Calculation of driving range of hydrogen (DRH) for NOVC-FCHVs

This paragraph is only applicable for Level 1B;

4.8.1	Stepwise procedure for calculating driving range of hydrogen for NOVC-FCHVs.

The results shall be calculated in the order described in Table A8/12. All applicable results in the column "Output" shall be recorded. The column "Process" describes the paragraphs to be used for calculation or contains additional calculations.

Table A8/12

Calculation of the driving range of hydrogen fuel for NOVC-FCHVs (for Level 1B only)

| Step No. | Source | Input | Process | Output |
| --- | --- | --- | --- | --- |
| 1 Result of single test | Paragraph 3.2 of appendix 7 to this annex. | The lower limit pressure of hydrogen tank | The lower limit pressure PLL according to paragraph 3.2 of appendix 7 to this annex. | PLL,1, MPa |
| 2 | Output step 1 | For every test: PLL,1, MPa | Averaging of tests and declared value according to paragraphs 1.2. to 1.2.3. inclusive of Annex B6. | PLL,2, MPa |
| 3 Result of PLL | Output step 2 | PLL,2, MPa PLL, declared, MPa | PLL,2 = PLL, declared PLL values shall be rounded according to paragraph 6.1.8. of this Regulation. PLL shall be rounded to the first place of decimal. | PLL,3, MPa |
| 4 Usable amount of hydrogen | Output step 3 Paragraph 3.3 of appendix 7 to this annex. | PLL,3, MPa | Usable amount of hydrogen according to paragraph 3.3 of appendix 7 to this annex. | UAH, kg |
| 5 Result of driving range of hydrogen | Output step 4 Output step 5 Table A8/7 | UAH, kg FECS,c,5, km/kg | Calculation of driving range of hydrogen according to paragraph 4.8.2. of this annex. DRH shall be rounded down to the nearest 20km. | DRH, km |

4.8.2.	The driving range for NOVC-FCHVs shall be calculated as the following equations:

DRH=FEcs,c,5×UAH

For individual vehicles,

DRH-ind=FEcs,c,ind×UAH

where

DRH	is the driving range of the vehicle, km;

DRH-ind	is the driving range for an individual vehicle, km;

FECS,c,5	is the charge-sustaining fuel efficiency determined according to step 5 of Table A8/7, km/kg

FEcs,c,ind  	is the charge-sustaining fuel efficiency for an individual vehicle determined according to step 6 of Table A8/7, km/kg

UAH	is the usable amount of hydrogen, kg

Annex B8 – Appendix 1, amend to read and add new clause 4:

REESS state of charge profile and hydrogen state of charge profile

4.	Test sequence NOVC-FCHVs the lower limit pressure test (Figure A8.App1/8)

Figure A8.App1/8

NOVC-FCHVs, the lower limit pressure test

Annex B8 – Appendix 7, amend to read and add new clause 3:

Annex B8 - Appendix 7

Fuel consumption and usable amount of hydrogen measurement of compressed hydrogen fuel cell hybrid vehicles

- General requirements

Fuel consumption shall be measured using the gravimetric method in accordance with paragraph 2. of this appendix.

At the request of the manufacturer and with approval of the responsible authority, fuel consumption may be measured using either the pressure method or the flow method. In this case, the manufacturer shall provide technical evidence that the method yields equivalent results. The pressure and flow methods are described in ISO 23828.

Usable amount of hydrogen shall be measured in accordance with paragraph 3. of this appendix.

- Gravimetric method

- Usable amount of hydrogen

This paragraph is only applicable for Level 1B;

This test may be carried out after the fuel consumption test.

3.1	Principles

Usable amount of hydrogen is defined as shown in the figure A8/x. The lower limit pressure of hydrogen tank is the pressure when the vehicle stops running because of interruption of hydrogen supply.

Figure A8.App7/2

Schematic diagram of “usable amount of hydrogen

3.2	Measurement of lower limit pressure of hydrogen tank(s)

The lower limit pressure of hydrogen tank(s) shall be measured.

Figure A8.App7/3

Example of measuring pressure of hydrogen tank(s)

1: is the hydrogen tank(s)

2: is the pressure regulator

3: is the control valve to supply hydrogen to fuel cell system

4: is the fuel cell system

5: is the pressure sensor upstream of a pressure regulator

6: is the pressure sensor downstream of a pressure regulator

In the case that a vehicle was tested for measurement of lower limit pressure of hydrogen tank(s), the lower limit pressure may be applied to vehicles that fulfil the same family of Lower limit pressure defined in paragraph 6.3.12. of this regulation.

Vehicle H shall be tested when a vehicle was selected from an interpolation family as defined in paragraph 6.3.12.(b) of this regulation.

3.2.1.	Units, accuracy and resolution

Units, accuracy and resolution of measurements shall be as shown in Table A8.App7/3.

At the request of the manufacturer and with approval of the responsible authority, the on-board pressure sensor may be used.

Table A8.App7/3 Parameters, units, accuracy and resolution of measurements

| Parameter | Units | Accuracy | Resolution |
| --- | --- | --- | --- |
| Pressure of the hydrogen tank | MPa | ±1 MPa | 0.1 MPa |

3.2.2.	Equipment and setting

The pressure of the on-board hydrogen tank(s) shall be adjusted to the manufacturer’s recommended value to meet the requirement of the minimum driving time in the paragraph 3.5.

3.2.3.	The test cell temperature at the start of the test shall be within ± 5 °C of the set point of 23 °C.  The test vehicle shall be pushed onto a dynamometer.

3.2.4.	Vehicle preparation

3.2.4.1.	Soaking

The test vehicle shall be soaked for a minimum of 6 hours before the test.  However soaking may be omitted in following cases.

- Temperature sensors of on-board tank(s) are applicable, and the temperatures are already within ±5 ℃ of 23 ℃.

- The test is carried out to follow Type 1 in the test cell where the temperature has been kept within ±5 ℃ of 23 ℃.

3.2.4.2.	When a test vehicle of this test is not used for Type 1 test of fuel consumption described in paragraph 1 and paragraph 2 of this appendix, at the request of manufacturer, run-in requirement of paragraph 2.2 of this annex and preconditioning described in paragraph 2.6.4.3. of annex B6 may be omitted.

3.2.5.	Constant speed

The minimum speed of the constant speed segments shall be set to 80 km/h. At the request of manufacturer and with approval of the responsible authority, a higher constant speed in the constant speed segments may be selected.

The acceleration to the constant speed level shall be smooth and accomplished within 1 minute after initiating the powertrain start procedure.

The vehicle shall be driven more than 10 minutes in the constant speed segments.

The pressure of the hydrogen tank(s) shall be measured at a sampling rate of a least 5 Hz.

3.2.6.	Break-off criterion

The break-off criterion is as follows;

- when the vehicle exceeds the prescribed speed trace tolerance as specified in paragraph 2.6.8.3.1.2. of Annex B6 for 4 consecutive seconds or more;

- or when manufacturer declares the end of the constant speed segment.

The accelerator control shall be deactivated. The vehicle shall be braked to standstill within 60 seconds.

3.3.	Calculation of the Usable amount of hydrogen (UAH)

3.3.1.	Equation of UAH

The pressure and if applicable the temperature of the hydrogen tank(s) at the end of the constant speed segment shall be employed for calculation of the usable amount of hydrogen.

The usable amount of hydrogen is calculated by using the following equation:

UAH = V × (ρ[PNWP, T15 ] – ρ[PLL, T15 ]) / 1 000

where:

UAH	is the usable amount of hydrogen, expressed in kg;

V	is the volume of hydrogen tank(s), expressed in m3;

ρ[PNWP, T15] is the gas density at the given pressure and temperature for 100% of SOC, g/m3;

PNWP	is the gas pressure at the nominal working pressure, Mpa;

T15	is 283,15 K (15 °C) as reference temperature of the nominal working pressure;

PLL	is the lower limit pressure measured in the test, expressed in MPa.

Nominal working pressure and volume of hydrogen tank(s) are specified by the report of Model I, Annex 1 Part 1, UN R134. The total volume is applied if multiple tanks are loaded on the vehicle.

Gas density of hydrogen is determined by the following equation.

ρ(p, T)=pMRT(1Z(p, T))

where:

ρ[p, T]	is the gas density at the given pressure and temperature, g/m3;

p	is the pressure of the hydrogen tank at the end of the constant speed segment, MPa

T	is hydrogen gas tempareture of the hydrogen tank. 283,15 (15 °C) in K at the end of the constant speed segment.

R	is the gas constant, 8.314472x 10-6in m3 MPa K−1 mol−1;

M	is the molar mass of hydrogen, 2.01588, g/mol;

Z(p,T)	is the compressibility factor.

The Compressibility factors of hydrogen gas are listed in Table A7/2 to annex B7.

3.3.2.	Simplified approach to calculate UAH for 70MPa system

When PNWP is 70MPa, the density of hydrogen can be derived from the equations above.

ρ[70MPa, 15°C] = 40.22×103 (g/m3)

At the request of manufacturer, the density of hydrogen of the tank(s) at the end of the constant speed segment and UAH may be derived from approximate equations as follows,

Z(pLL, 15°C)=1

ρpLL, 15°C=PLL× M(gmol)RT= 8.4×102× PLL(MPa)

UAH = V × (40.17×103 – 8.4×102× PLL) / 1 000

II. Justification

- Addition of driving range test for NOVC-FCHV for user information due to the limited number of hydrogen stations (This proposal is limited to Level 1B.).

- On the other hand, Japan understands that the frequent amendments of this Regulation is not efficient from the viewpoint of UNECE amendment process and homologation process in each region. Therefore, Japan follows GRPE decision if this amendment would be voted at WP.29 as a consolidated document including the other amendments yet to come in the near future