|  | United Nations | ECE/TRANS/WP.29/2023/127 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 4 September 2023 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

191st session

Geneva, 14–16 November 2023

Item 4.9.2. of the provisional agenda

1958 Agreement:

Consideration of draft amendments to existing

UN Regulations submitted by GRPE

Proposal for Supplement 1 to the 02 series of amendments to UN Regulation No. 154 (WLTP)

Submitted by the Working Party on Pollution and Energy*

The text reproduced below was adopted by the Working Party on Pollution and Energy (GRPE) at its eighty-ninth session (ECE/TRANS/WP.29/GRPE/89, para. 25). It is based on ECE/TRANS/WP.29/GRPE/2023/16, GRPE-89-16-Rev.1 as amended by Addendum 1 of the report. It is a consolidated version. It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee (AC.1) for consideration at their November 2023 sessions.

5.		Approval

5.11.	This paragraph is only applicable for Level 1A

Requirements for type-approval regarding devices for monitoring the consumption of fuel and/or electric energy

5.11.1.	This paragraph is only applicable for Level 1A

The manufacturer shall ensure that the following vehicles of categories M1, N1 and N2 are equipped with a device for determining, storing and making available data on the quantity of fuel and/or electric energy used for the operation of the vehicle:

(a)	pure ICE and Not-Off-Vehicle Charging Hybrid Electric vehicles (NOVC-HEVs) powered exclusively by mineral diesel, biodiesel, petrol, ethanol or any combination of these fuels;

(b)	Off-Vehicle Charging Hybrid Electric Vehicles (OVC-HEVs) powered by electricity and any of the fuels mentioned in point (a).

5.11.2.	This paragraph is only applicable for Level 1B

The manufacturer shall ensure that the following vehicles are equipped with a device for determining, storing and making available data on the quantity of fuel and/or electric energy used for the operation of the vehicle:

(a)	pure ICE and Not-Off-Vehicle Charging Hybrid Electric vehicles (NOVC-HEVs) powered exclusively by mineral diesel, biodiesel, petrol, ethanol or any combination of these fuels;

(b)	Off-Vehicle Charging Hybrid Electric Vehicles (OVC-HEVs) powered by electricity and any of the fuels mentioned in point (a).

(c)	NOVC-FCHV, PEV, CNG, LNG,,,,,

5.11.3.	The device for monitoring the consumption of fuel and/or electric energy shall comply with the requirements laid down in Appendix 5.

6.		Specifications and tests

6.2.	Test procedure

Table A specifies the various test requirements for type approval of a vehicle.

| Table A |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Application of test requirements for type-approval and extensions |  |  |  |  |  |  |  |  |  |  |  |  |
| Vehicle category | Vehicles with positive ignition engines including hybrids1,2 |  |  |  |  |  |  |  | Vehicles with compression ignition engines including hybrids |  | Pure electric vehicles | Hydrogen fuel cell vehicles |
|  | Mono fuel |  |  |  | Bi-fuel3 |  |  | Flex-fuel3 | Mono fuel |  |  |  |
| Reference fuel | Petrol | LPG | NG/ Biomethane | Hydrogen (ICE) | Petrol | Petrol | Petrol | Petrol | Diesel | Petrol | — | Hydrogen (Fuel Cell) |
|  |  |  |  |  | LPG | NG/ Biomethane | Hydrogen (ICE) 4 | Ethanol (E85) |  |  |  |  |
| Type 1 test (for applicability of measured components to fuels and vehicle technology and therefore measurement procedures, see Table 1A and Table 1B) (limits) | Yes | Yes5 | Yes5 | Yes4 | Yes (both fuels) | Yes (both fuels) | Yes (both fuels) | Yes (both fuels) | Yes | Yes | — | — |
| ATCT (14°C test) | Yes | Yes | Yes | Yes4 | Yes (both fuels) | Yes (both fuels) | Yes (both fuels) | Yes (both fuels) | Yes | Yes | — | — |
| Evaporative emissions (Type 4 test) | Yes | Yes 6 | Yes 6 | — | Yes (petrol only) | Yes (petrol only) | Yes (petrol only) | Yes (petrol only) | — | Yes | — | — |
| Durability (Type 5 test) | Yes | Yes | Yes | Yes | Yes (petrol only) | Yes (petrol only) | Yes (petrol only) | Yes (petrol only) | Yes | Yes | — | — |
| OBD | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | — | — |
| OBFCM | Yes | YES7 | YES7 | — | — | — | — | Yes (both fuels) | Yes | Yes | YES7 | YES7 |

1 	Specific test procedures for hydrogen vehicles will be defined at a later stage.

2 	Particulate mass and particle number limits and respective measurement procedures shall apply only to vehicles with direct injection engines

3 	When a bi-fuel vehicle is combined with a flex fuel vehicle, both test requirements are applicable.

4 	Only NOx emissions shall be determined when the vehicle is running on hydrogen.

5 	For Level 1A only - Particulate mass and particle number limits and respective measurement procedures shall not apply. For Level 1B only - In the case that a mono-fuel gas vehicle has a petrol tank it shall also be tested using the applicable petrol reference fuel

6 	For Level 1B, if the mono-fuel gas vehicle does have a petrol tank “Yes”, if the mono-fuel gas vehicle does not have a petrol tank “—“, For Level 1A “—“

7	For Lebel 1B only

6.3.9.	OBFCM

The OBFCM device shall determine the parameters and store the lifetime values on board the vehicle in accordance to Appendix 5.

8.		Conformity of production (COP)

8.1.	Every vehicle produced under a type approval according to this Regulation shall conform with regard to the vehicle type approved. The conformity of production procedures shall comply with those set out in the 1958 Agreement, Schedule 1 (E/ECE/TRANS/505/Rev.3), with the following requirements:

8.1.1.	The manufacturer shall implement adequate arrangements and documented control plans and carry-out, at intervals specified in this Regulation, the necessary tests to verify continued conformity with the approved type. The manufacturer shall obtain agreement for these arrangements and control plans from the responsible authority. The responsible authority shall perform audits at specific intervals. This audit shall include production and test facilities as part of the product conformity and continued verification arrangements. Where necessary the responsible authority may require additional tests to be conducted.

8.1.2.	The manufacturer shall check the conformity of production by conducting the appropriate tests in accordance with Table 8/1 and Table 8/2 and with the OBD requirements, where applicable according to Table A in paragraph 6. Where applicable and if required according to Table A, the manufacturer shall determine and report the OBFCM device accuracy in accordance with Appendix 5.

The specific procedures for conformity of production are set out in paragraphs 8.2. to 8.4. and Appendices 1 to 4.

Table 8/1

Type 1 Applicable Type-1 CoP requirements for the different types of vehicle

| Type of vehicle | Criteria emissions | CO2 emissions | Fuel Efficiency | Electric energy consumption | OBFCM accuracy |
| --- | --- | --- | --- | --- | --- |
| Pure ICE | Level1A and Level 1B | Level 1A | Level 1B | Not Applicable | Level 1A |
| NOVC-HEV | Level 1A and Level 1B | Level 1A | Level 1B | Not Applicable | Level 1A |
| OVC-HEV | Level 1A and Level 1B: CD(1) and CS | Level 1A: CS only | Level 1B: CS only | Level1A and Level 1B: CD only | Level 1A : CS |
| PEV | Not Applicable | Not Applicable | Not Applicable | Level1A and Level 1B | Not Applicable |
| NOVC-FCHV | Not Applicable | Not Applicable | Exempted | Not Applicable | Not Applicable |
| OVC-FCHV | Not Applicable | Not Applicable | Exempted | Exempted | Not Applicable |

(1)  Only if there is combustion engine operation during a valid CD Type 1 test for CoP verification

Appendix 2

Verification of conformity of production for Type 1 test—statistical method

3.4.

For vehicles referred to in paragraph 5.11.1. and 5.11.2.excluding paragraph (c) of this Regulation the conformity of production of OBFCM devices as defined in paragraph 4.2. of Appendix 5 shall be evaluated as follows:

(1)	For each single test i performed for the purposes of paragraph 3. of this appendix the value xi shall be set equal to:

1 / (1 - Accuracy)

where the Accuracy of the OBFCM device shall be determined in accordance with paragraph 4.2. of Appendix 5.

(2)	The conformity of production of the OBFCM devices shall be evaluated according to the requirements of paragraph 3.3.1., but applying a factor A value of 1.0526.

(3)	If for the last test N performed for the purposes of paragraph 3. the decision (iii) of paragraph 3.3.1. with regard to the conformity of production of the OBFCM devices is reached, the sequence of tests shall be continued until a final decision (i) or (ii) of paragraph 3.3.1. is reached.

The Type Approval authority shall keep a record of the determined accuracies of OBFCM device at each test as well as the decision according to paragraph 3.3.1. after each test.

Appendix 5

Devices for monitoring on board the vehicle the consumption of fuel and/or electric energy

1.	Introduction

This appendix sets out the definitions and requirements applicable to the devices for monitoring on board the vehicle the consumption of fuel and/or electric energy.

2.	Definitions

2.1.	"On-board Fuel and/or Energy Consumption Monitoring Device" ("OBFCM device") means any element of design, either software and/or hardware, which senses and uses vehicle, engine, fuel and/or electric energy parameters to determine and make available at least the information laid down in paragraph 3 of this appendix, and store the lifetime values on board the vehicle.

2.2.	"Lifetime" value of a certain quantity determined and stored at a time t shall be the values of this quantity accumulated since the completion of production of the vehicle until time t.

2.3.	"Engine fuel rate" means the amount of fuel injected into the engine per unit of time. It does not include fuel injected directly into the pollution control device.

2.4.	"Vehicle fuel rate" means the amount of fuel injected into the engine and directly into the pollution control device per unit of time. It does not include the fuel used by a fuel operated heater.

2.5.	"Total Fuel Consumed (lifetime)" means the accumulation of the calculated amount of fuel injected into the engine and the calculated amount of fuel injected directly into the pollution control device. It does not include the fuel used by a fuel operated heater.

2.6.	"Total Distance Travelled (lifetime)" means the accumulation of the distance travelled using the same data source that the vehicle odometer uses.

2.7.	"Grid energy" means, for OVC-HEVs, the electric energy flowing into the battery when the vehicle is connected to an external power supply and the engine is turned off. It shall not include electrical losses between the external power source and the battery.

2.8.	"Charge-sustaining operation" means, for OVC-HEVs, the state of vehicle operation when the REESS state of charge (SOC) may fluctuate but the intent of the vehicle control system is to maintain, on average, the current state of charge.

2.9.	"Charge-depleting operation" means, for OVC-HEVs, the state of vehicle operation when the current REESS SOC is higher than the charge-sustaining target SOC value and, while it may fluctuate, the intent of the vehicle control system is to deplete the SOC from a higher level down to the charge-sustaining target SOC value.

2.10.	"Driver-selectable charge-increasing operation" means, for OVC-HEVs, the operating condition in which the driver has selected a mode of operation, with the intention to increase the REESS SOC.

2.11.	"Energy consumption rate" means the amount of energy consumed for vehicle propulsion per unit of time.

2.12.	“Battery SOCE” means the on-board usable battery energy (UBE)  performance at a specific point in its lifetime, expressed as a percentage of the UBE determined during type approval testing.

2.13.	“Battery SOCR” means the on-board electric range at a specific point in its lifetime, expressed as a percentage of the certified range.

3.	Information to be determined, stored and made available

The OBFCM device shall determine at least the following parameters and store the lifetime values on board the vehicle. The parameters shall be calculated and scaled according the standards referred to in paragraph 6.5.3.2. (a) of Appendix 1 to Annex C5.

The information listed in paragraph 3.1. and 3.2. shall be made available as signals through the serial port connector referred to in paragraph 6.5.3.2. (c). of Appendix 1 to Annex C5.

3.1.	For all vehicles referred to in paragraph 5.11. of this Regulation, with the exception of OVC-HEVs:

(a)	Total fuel consumed (lifetime) (litres);

(b)	Total distance travelled (lifetime) (kilometres);

(c)	Engine fuel rate (grams/second);

(d)	Engine fuel rate (litres/hour);

(e)	Vehicle fuel rate (grams/second);

(f)	Vehicle speed (kilometres/hour).

3.2.	For OVC-HEVs:

(a)	Total fuel consumed (lifetime) (litres);

(b)	Total fuel consumed in charge-depleting operation (lifetime) (litres);

(c)	Total fuel consumed in driver-selectable charge-increasing operation (lifetime) (litres);

(d)	Total distance travelled (lifetime) (kilometres);

(e)	Total distance travelled in charge-depleting operation with engine off (lifetime) (kilometres);

(f)	Total distance travelled in charge-depleting operation with engine running (lifetime) (kilometres);

(g)	Total distance travelled in driver-selectable charge-increasing operation (lifetime) (kilometres);

(h)	Engine fuel rate (grams/second);

(i)	Engine fuel rate (litres/hour);

(j)	Vehicle fuel rate (grams/second);

(k)	Vehicle speed (kilometres/hour);

(l)	Total grid energy into the battery (lifetime) (kWh);

The following information are only applicable Level 1B

(m) 	Energy consumption rate per second (Wh/s) ;

(n)	Battery SOCE (%) ;

(o)	Battery SOCR (%)

3.3.	For PEV, applicable Level 1B only

(a)	Total distance travelled (lifetime) (kilometres);

(b)	Vehicle speed (kilometres/hour);

(c)	Total grid energy into the battery (lifetime) (kWh);

(d)	Energy consumption rate per second (Wh/s);

(e)	Battery SOCE (%);

(f)	Battery SOCR (%).

3.4.	For NOVC-FCHV, applicable Level 1B only

(a)	Total fuel consumed (lifetime value) (kg);

(b)	Total distance traveled (lifetime value) (km);

(c)	Vehicle fuel consumption rate per second (g/s);

(d)	Vehicle speed (km/h).

3.5.	For CNG/LNG, applicable Level 1B only

(a)	Total fuel consumed (lifetime) (m3 or g);

(b)	Total distance travelled (lifetime) (kilometres);

(c)	Engine fuel rate (m3 or g/second);

(d)	Engine fuel rate (m3 or g/hour);

(e)	Vehicle fuel rate (m3 or g/second);

(f)	Vehicle speed (kilometres/hour).

3.6.	For all vehicles, applicable Level 1B only

(a)	Vehicle identification number (VIN) prescribed in ISO 3779, chassis number or those equivalent to these

4.	Accuracy

4.1.	With regard to the information specified in paragraph 3., the manufacturer shall ensure that the OBFCM device provides the most accurate values that can be achieved by the measurement and calculation system of the engine control unit.

4.2.	Notwithstanding paragraph 4.1., the manufacturer shall ensure that the accuracy is higher than - 0.05 and lower than 0.05 calculated with three decimals using the following formula:

Where:

Fuel_ConsumedWLTP (litres)		is the fuel consumption determined at the first test carried out in accordance with paragraph 1.2. of Annex B6, calculated in accordance with paragraph 6. of Annex B7, using emission results over the total cycle before applying corrections (output of step 2 in Table A7/1 of Annex B7), multiplied by the actual distance driven and divided by 100.

Fuel_ConsumedOBFCM (litres)	is the fuel consumption determined for the same test using the differentials of the parameter ‘Total fuel consumed (lifetime)’ as provided by the OBFCM device.

For OVC-HEVs the charge-sustaining Type 1 test shall be used.

4.2.1.	If the accuracy requirements set out in paragraph 4.2. are not met, the accuracy shall be recalculated for subsequent Type 1 tests performed in accordance with paragraph 1.2. of Annex B6, in accordance with the formulae in paragraph 4.2., using the fuel consumed determined and accumulated over all performed tests. The accuracy requirement shall be deemed to be fulfilled once the accuracy is higher than - 0.05 and lower than 0.05.

4.2.2.	If the accuracy requirements set out in paragraph 4.2.1. are not met following the subsequent tests pursuant to this point, additional tests may be performed for the purpose of determining the accuracy, however, the total number of tests shall not exceed three tests for a vehicle tested without using the interpolation method (vehicle H), and six tests for a vehicle tested using the interpolation method (three tests for vehicle H and three tests for vehicle L). The accuracy shall be recalculated for the additional subsequent Type 1 tests in accordance with the formulae in paragraph 4.2., using the fuel consumed determined and accumulated over all performed tests. The requirement shall be deemed to be fulfilled once the accuracy is higher than - 0.05 and lower than 0.05. Where the tests have been performed only for the purpose of determining the accuracy of the OBFCM device, the results of the additional tests shall not be taken into account for any other purposes.

5.	Access to the information provided by the OBFCM device

5.1.	The OBFCM device shall provide for standardised and unrestricted access of the information specified in paragraph 3. and shall conform to the standards referred to in paragraphs 6.5.3.1. (a) and 6.5.3.2. (a) of Appendix 1 to Annex C5.

5.2.	By way of exemption from the reset conditions specified in the standards referred to in paragraph 5.1. and notwithstanding paragraphs 5.3. and 5.4., once the vehicle has entered into service the values of the lifetime counters shall be preserved.

5.3.	The values of the lifetime counters may be reset only for those vehicles for which the memory type of the engine control unit is unable to preserve data when not powered by electricity. For those vehicles the values may be reset simultaneously only in the case the battery is disconnected from the vehicle. The obligation to preserve the values of the lifetime counters shall in this case apply for new type approvals at the latest from 1 January 2022 and for new vehicles from 1 January 2023.

5.4.	In the case of malfunctioning affecting the values of the lifetime counters, or replacement of the engine control unit, the counters may be reset simultaneously to ensure that the values remain fully synchronised.

5.5.	This paragraph is only applicable Level 1B

In cases where the lifetime values are no longer preserved, the fact that they are no longer preserved shall be recorded in the ECU and the record concerned shall not be deleted easily.

Annex A1 - Appendix 1

WLTP Test Report

Test Reports

Annex B6

Type 1 test procedures and test conditions

2.6.8.5.	OBFCM data recording and storing

During the Type 1 test, the following parameters referred in Appendix 5 of this Regulation shall be recorded and saved (1 Hz sampling frequency) by the testing lab and shall be made available by the approval authority if requested by a regional authority:

(a)	Engine fuel rate (grams/second);

(b)	Engine fuel rate (litres/hour);

(c)	Vehicle fuel rate (grams/second).

*	In accordance with the programme of work of the Inland Transport Committee for 2023 as outlined in proposed programme budget for 2023 (A/77/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.