Model Regulation for Thermal Propagation- Commenting sheet <Document: SIGTP-06_Draft_model_regulation(clean)>

Submitted by: _______FRANCE__________________

| ID | Line # | Para. | Comment | Proposal | Treatment |
| --- | --- | --- | --- | --- | --- |
|  |  | Definitions | Definition of adjacent cell is needed. Only cells in serie are monitored. Large cell could be defined by several cells in parallel. When a triggered cell is going to thermal runaway it means that you have effect on cells in parallel and you could have smoke more or less closer. | Adjacent cells mean cells in serial where only the thermal effect for thermal propagation is considered. Cells in parallel could not be considered adjacent because 3 cells in parallel could be replaced by one large cell. |  |
|  |  | 6.5 | Paragraph 6.5 on fire resistance : Editorial correction | “This test is not required when the REESS as installed in the vehicle, is mounted such that the lowest surface of the casing of the REESS is more than 1.5m above the ground. At the option request of the manufacturer, this test may be performed where when the of the REESS’s lower surface lowest surface of the casing of the REESS is higher than 1.5 m above the ground. The test shall be carried out on one test sample.” |  |
|  |  | 6.15.1 | => in ( c ), if the single cell thermal runaway cannot be triggered and we choose to repeat the same test procedure, it should be written this is possible to select a different cell than the cell selected for the 1st test. (It could be useful if it appears that the cell selected for the first time was not the cell with the highest risk of propagation stemming from a single cell failure.) | 6.15.1. The REESS or vehicle system shall provide a signal to activate the warning specified in paragraph 5.2.3. in the event of a thermal propagation which is triggered by an internal short circuit leading to a single cell thermal runaway in order to allow egress. This requirement is deemed to be met if one of the below conditions is satisfied: (a) the presence of a hazardous situation as defined by pass/fail criteria in paragraph 6.15.3.4. does not occur within 5 minutes following the warning signal; or (b) the single cell thermal runaway does not lead to thermal propagation in the REESS [during 2 hours after its triggering]; or (c) the single cell thermal runaway cannot be triggered and this is confirmed by repeating the same test procedure (i.e. the same trigger method and the same test level, same or different initiation cell selection) or by conducting a cell level test. |  |
|  |  | 6.15.3.4.1. and 6.15.3.4.2. | Those 2 paragraphs can be removed (redundancy with paragraph 6.15.1) | 6.15.3.4.1. If no thermal propagation is observed during the 2 hours after the thermal runaway was triggered in the initiation cell, the requirements of paragraph 6.15.3.4. are deemed to be satisfied. 6.15.3.4.2. If thermal runaway is not triggered during the test with the chosen trigger method, and confirmed by repetition of the same test procedure (i.e. the same trigger method and the same test level) or by conducting a cell level test with the same trigger method, the requirements of paragraph 6.15.3.4. are deemed to be satisfied. |  |
|  |  | 6.15.4.2 | The word “and” add confusion, would be in favor of removal of this word | The risk identification and mitigation analysis shall include: (a) Risk mitigation by design; (b) Risk mitigation by manufacturing control; and (c) Risk mitigation by other means; |  |
|  |  | 6.15.4.4 | The footnote already existing in the previous UNR100 series is not redundant with this paragraph and need to be kept in the regulation : “1 The manufacturer will be accountable for the verity and integrity of documentation submitted, and assume full responsibility for the safety of occupants against adverse effects arising from thermal propagation caused by internal short circuit” | 6.15.4.4. Conclusions (a) The concluding part of the report shall comprise a brief summary of the major results of the risk management analysis and a statement that the requirements in paragraphs 6.15.1. and 6.15.2. are satisfied, including: The methods used are scientifically and technically valid for the scope of the risk management analysis; (b) The data used are appropriate and reasonable in relation to the intention of the risk reduction analysis; (c) The interpretations are relevant and reflect the assumptions made and the limitations identified for the study; This part may be in the form of an internal or external critical review report, if the manufacturer has such a process in place."1 1 The manufacturer will be accountable for the verity and integrity of documentation submitted, and assume full responsibility for the safety of occupants against adverse effects arising from thermal propagation caused by internal short circuit |  |