|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | Introduction: |  |  |
|  | 1.1 This section provides background information concerning the deliberations to enable: 1.1.1(a) Monitoring and evaluation of ADS performance in use for Data Storage System for Autonomous Driving (DSSAD), and (b)1.1.2 To Complement the crash reconstruction data collected by the vehicle’s event data recorder, and (c)1.1.3 To identify if the ADS was operating the vehicle at a certain point of time. | [One of the functions of a DSSAD is] to support the in-service monitoring and reporting to ensure safe operation of ADS vehicles and validate their safety case. (UK) ] | Agreed below. DSSAD refers to a capability of a vehicle to monitor the performance of its Automated Driving System (ADS). This document provides recommendations to enable evaluations of ADS performance. This document has been prepared to support WP.29 deliberations under the 1958, 1997, and 1998 Agreements. |
|  | Purpose: |  | Deleted. |
|  | 2.1 This document provides recommendations concerning Data Storage Systems for Automated Driving (DSSAD) to support deliberations under the 1958, 1997, and/or 1998 Agreements as may be applicable. 2.2 Usage of the verbal forms “shall” (indicating an obligatory provision) and “may” (indicating a permissive provision) should be understood within the context of providing recommendations per the preceding paragraph. |  | Deleted. |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | Terms and Definitions: (Source: FRAV/VMAD integrated document) This section defines terms used in this document. Use of these terms and their definitions is recommended in the development of legal requirements related to ADS and ADS vehicles. |  | Comment from the co-chairs as of 20240828 Terms and Definitions: (Source: FRAV/VMAD integrated document) WP.29 GRVA Guidelines and recommendations for Automated Driving System safety requirements, assessments and test methods to inform regulatory development.) This section defines terms used in this document. Use of these terms and their definitions is recommended in the development of legal requirements related to ADS and ADS vehicles. Agreed to accept this. |
|  | Automated Driving System (ADS) means the vehicle hardware and software that are collectively capable of performing the entire Dynamic Driving Task (DDT) on a sustained basis. |  |  |
|  | (ADS) function means an ADS hardware and software capability designed to perform a specific portion of the DDT. |  |  |
|  | ADS feature means an ADS designed specifically for use within an Operational Design Domain (ODD). | Synchronized with the integrated documents. ADS feature means an application of an ADS ADS functionality designed specifically for use within an Operational Design Domain (ODD). | Agreed. |
|  | ADS vehicle means a vehicle equipped with an ADS. |  |  |
|  | Driver means a human user who performs in real time part or all of the DDT and/or DDT fallback for a particular vehicle. |  |  |
|  | Dynamic Driving Task (DDT) means the real-time operational and tactical functions required to operate the vehicle in on-road traffic. | Japan Proposal: Synchronized with the integrated documents. Dynamic Driving Task (DDT) means the real-time operational and tactical functions required to operate the vehicle in on-road traffic. | Agreed. |
|  | The DDT is always performed in its entirety by the ADS in operation (“the entire DDT” as stated in the definition of an “Automated Driving System” under para. 3.1.) which means the whole of the tactical and operational functions necessary to operate the vehicle. These functions can be grouped into three interdependent categories: sensing and perception, planning and decision, and control. | Japan Proposal: Synchronized with the integrated documents. The DDT is always performed in its entirety by the ADS which means the whole of the tactical and operational functions necessary to operate the vehicle in operation (i.e., the ADS performs “the entire DDT” as stated in the definition of an “Automated Driving System” under para. 3.1.) which means the whole of the tactical and operational functions necessary to operate the vehicle. These functions can be grouped into three interdependent categories: sensing and perception, planning and decision, and control. | Agreed. Comment from the co-chairs as of 20240828 Synchronized with the integrated documents. When the ADS is in operation, the DDT is always performed in its entirety by the ADS which means the whole of the tactical and operational functions necessary to operate the vehicle (i.e., the ADS performs “the entire DDT” as stated in the definition of an “Automated Driving System” under para. 3.2.). These functions can be grouped into three interdependent categories: sensing and perception, planning and decision, and control. |
|  | Sensing and perception include: - Monitoring the driving environment via object and event detection, recognition, and classification. - Perceiving other vehicles and road users, the roadway and its fixtures, objects in the vehicle’s driving environment and relevant environmental conditions. - Sensing the ODD boundaries, if any, of the ADS feature. |  | Agreed. Comment from the co-chairs as of 20240828 Synchronized with the integrated documents. Sensing and perception include: - Monitoring the driving environment via object and event detection, recognition, and classification. - Perceiving other vehicles and road users, the roadway and its fixtures, objects in the vehicle’s driving environment and relevant environmental conditions. - Sensing the ODD boundaries, if any, of the ADS feature. - Positional awareness. |
|  | Planning and decision include: - Predicting actions of other road users. - Response preparation. - Manoeuvre planning. |  |  |
|  | Control includes: · - Object and event response execution. - Lateral vehicle motion control. - Longitudinal vehicle motion control. - Enhancing conspicuity via lighting and signalling. |  |  |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | ADS fallback response means an ADS-initiated transition of control or an ADS-controlled procedure to place the vehicle in a minimal risk condition. | Japan Proposal: Synchronized with the integrated documents. ADS fallback response means an ADS system-initiated deactivation of the ADS transition of control or an ADS-controlled procedure to place the vehicle in a minimal risk condition. | Agreed. ADS fallback response means a system-initiated deactivation of the ADS or an ADS-controlled procedure to place the vehicle in a minimal risk condition. |
|  | DDT fallback means a response by the user to either perform the DDT or to achieve a minimal risk condition or a response by an ADS to achieve a minimal risk condition: (a) After the occurrence of one or more DDT performance-relevant system failures, or (b) Upon an ODD exit. | Japan Comment: Not defined in the integrated guideline. OICA: does not see a need, but would like to revisit after further review. Discussion: - Not sure items A and B are sufficient to capture the meaning of “DDT fallback” and not ready to delete this yet. It will be important to differentiate between variations of fallback plans. (AAPC) | Agree to delete. If this item comes up again, would refer to GRVA guidance document. |
|  | Fallback user means a user expected to perform the DDT pursuant to a transition of control. | Japan Proposal: Synchronized with the integrated documents. Fallback user means a user designated expected to perform the DDT pursuant to a transition of control. | Agreed. Comment from the co-chairs as of 20240828 Synchronized with the integrated documents. Fallback user means a user designated to perform the DDT pursuant to an ADS fallback response. |
|  | Minimal Risk Condition (MRC) means a stable and stopped state of the vehicle that reduces the risk of a crash. |  | Agreed (aligns with GRVA guidance document). |
|  | Occurrence means a safety-relevant event involving an ADS vehicle. |  | Agreed (aligns with GRVA guidance document). |
|  | Non-critical Occurrence means an operational interruption, defect, fault, or other circumstance that influenced or may have influenced ADS safety but did not result in a collision or serious incident. | TBD pending GRVA activity |  |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | Non-critical Occurrence means an operational interruption, defect, fault, or other circumstance that influenced or may have influenced ADS safety but did not result in a collision or serious incident. | TBD pending GRVA activity. OR New IWG ADS Document definition: “Non-critical Occurrence” means all occurrences which are not “Critical Occurrences.” |  |
|  | Critical Occurrence means an occurrence during which the ADS is performing the DDT and: (a) At least one person suffers an injury that requires medical attention as a result of being in the vehicle or being involved in the event. (b) The ADS vehicle, other vehicles or stationary objects sustain physical damage that exceeds a certain threshold. (c) Any vehicle involved in the event experiences an airbag deployment. | Japan Proposal: Synchronized with the integrated documents. Critical Occurrence means an occurrence during which at least one of the following criteria is fulfilled: the ADS is performing the DDT and: (a) At least one person suffers an injury that requires medical attention or dies as a result of being in the vehicle or being involved in the event. (b) The ADS vehicle, other vehicles or stationary objects sustain physical damage that exceeds a certain threshold. (c) Any vehicle involved in the event experiences an airbag deployment. NOTE: group will need to review where terms are used within the document and make sure the terms capture the system’s objective. OICA: DSSAD will not be able to capture the a) and we will need to be more specific with regards to what is meant by “critical occurrence.” | Agreed. |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | Operational Design Domain (ODD) means the operating conditions under which an ADS feature is specifically designed to function. |  | Agreed. Comment from the co-chairs as of 20240828 Synchronized with the integrated documents. Operational Design Domain (ODD) means the operating conditions under which an ADS feature is specifically designed to function.3 |
|  | Traffic scenario means a description of one or more real-world driving situations that may occur during a given trip. | Not defined in the integrated guideline/not relevant . | Agreed. |
|  | Nominal scenario means a traffic scenario representing usual and/or expected objects, object behaviors and/or road conditions. | Not defined in the integrated guideline/not relevant. | Agreed |
|  | Critical scenario means a traffic scenario representing unusual and/or unexpected objects, object behaviors, and/or road conditions. | Not defined in the integrated guideline./not relevant. | Agreed |
|  | Failure scenario means a traffic scenario representing a system failure that compromises the capability of the ADS to perform the entire DDT. | Not defined in the integrated guideline/not relevant | Agreed |
|  | Functional Scenario means a basic traffic scenario describing a situation and its corresponding elements at the highest level of abstraction in natural, non-technical language. | Not defined in the integrated guideline/not relevant | Agreed |
|  | Complex Scenario means a traffic scenario containing one or more situations that involve many other road users, unlikely road infrastructure, or abnormal geographic/environmental conditions. | Not defined in the integrated guideline/not relevant. | Agreed |
|  | Transition of Control (TOC) means a procedure by which the ADS transfers performance of the DDT to an ADS vehicle user. | Not defined in the integrated guideline/not relevant. UK: Want to maintain. AAPC/UK: Could refer to Guideline 3.1.38 and 3.1.39 System-initiated deactivation of the ADS means a procedure by which the ADS initiates the transfer of performance of the DDT from the ADS to a vehicle user. User initiated deactivation of the ADS means a procedure by which the user initiates the transfer of performance of the DDT from the ADS to a vehicle user. | Agreed to delete and use definitions for system-initiated deactivation of the ADS or user-initiated deactivation of the ADS. |
|  | TOC response means an ADS vehicle user intervention in performance of the DDT pursuant to a TOC request. | Not defined in the integrated guideline/not relevant. UK: Want to maintain. AAPC/UK: Could refer to Guideline 3.1.38 and 3.1.39 System-initiated deactivation of the ADS means a procedure by which the ADS initiates the transfer of performance of the DDT from the ADS to a vehicle user. User initiated deactivation of the ADS means a procedure by which the user initiates the transfer of performance of the DDT from the ADS to a vehicle user. | See above, agreed to delete. |
|  | TOC request means an alert issued by an ADS to an ADS vehicle user prompting the user to intervene in performance of the DDT. | Not defined in the integrated guideline. UK: Want to maintain AAPC/UK: Could refer to Guideline 3.1.38 and 3.1.39 | Agreed to delete. |
| 34 | System-initiated deactivation of the ADS means a procedure by which the ADS initiates the transfer of performance of the DDT from the ADS to a vehicle user. |  |  |
| 35 | User initiated deactivation of the ADS means a procedure by which the user initiates the transfer of performance of the DDT from the ADS to a vehicle user. |  |  |
| 36 | (ADS) User means a human user of an ADS vehicle. | Not defined in the integrated guideline. UK/US: Driver term might be more useful/relevant to refer to here. US: Want to make sure all terms in this document are complimentary to and agree with FRAV guidance document. | Revisit. |
| 37 | Unplanned event | Not defined in the integrated guideline. | Agreed to delete. |

|  | ADS failure | Not defined in the integrated guideline. US: Since this term isn’t defined in integrated guidelines, should we attempt to define this at all? It might not be appropriate for this group to set this definition. | Agreed to maintain as a placeholder, but a definition could be quite difficult. May need to have a feature specific failure definition. |
| --- | --- | --- | --- |
|  | Environmental data/conditions | Not defined in the integrated guideline. OICA Comment: These are not definitions; they are conditions detected by the ADS to determine ODD availability. Maybe too difficult to define now, need to determine later on in document what it is that needs to be recorded (related to ADS Performance of the DDT), and then add definition. | Agreed to delete |
|  | Safety authorities | Not defined in the integrated guideline. OICA Comment: This does not need to be defined in an agreement neutral document. | Agreed to delete. |
|  | DSSAD (to make it clear that this is not limited to data stored on the vehicle) | Not defined in the integrated guideline. OICA Comment: The place where to store the data should not be defined, for the freedom of design, just the idea of a storage system for the recorded data | Agreed to delete. |
|  | ADS function means an ADS hardware and software capability designed to perform a specific portion of the DDT. | AAPC: Need to include the ADS function definition 3.1.4 OICA: Not sure this is necessary. |  |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | - | UK proposal: Data Event means a data flag that corresponds to a specific event during the operation of an ADS vehicle. Japan: need to review time stamp and other terms within the format section of the document. | Revisit after review of Data format section |
|  | - | UK proposal: Triggering Event means a data event which requires the storing of data series. Japan: need to review time stamp and other terms within the format section of the document | Revisit after review of Data format section |
|  | - | UK proposal: Data Series means data of a time series which is required to be captured during a triggering event. Japan: need to review time stamp and other terms within the format section of the document | Revisit after review of Data format section |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | Data Storage and Security |  |  |
|  | The data storage system should be sufficient to record ADS performance elements listed in Paragraph 7. | OICA comment: This does not need to be specified, it needs to be able to store what it’s supposed to store. If it has to be stated, then replace the word record with 'store'. What is important is how long should the data be stored. Does not see the value in keeping this sentence. Wants to see a harmonized retention/storage time. Reg 157 may not be the best solution here. EC: disagree with the storage duration specific provision. Privacy could be a major factor. Not opposed to inclusion of the original draft text but opposed to the addition of provisions with specified times for specific data elements. Could industry think about whether it needs storage duration“harmonization” or is the issue about storage ‘Capacity?” What is the expectation from industry for data storage? Does the industry want to harmonization storage duration, or would they be satisfied with provisions on min/max periods for which the OEMs can store the data? AUS: need to be cautious and think about this further. UK: this is a good over-arching provision. Not harmful or confusing. Maybe we find a better term than “sufficient.” Focus more on the ability to capture the necessary elements. UK Proposal: The data storage system should be sufficient to record and store ADS performance elements listed in Paragraph 7. This shall consist of either data events which are data flags for particular events and or data elements which are time series data. OICA comment: UK’s proposal is technology neutral, and UK confirmed that intention. US comment: this is only technology neutral if data can be stored on- or off-board the vehicle. We don’t appear to have consensus yet since we have other issues to resolve (such as privacy concerns) and making sure that language is consistent throughout this document. UK: this isn’t directing any particular approach, simply stating that data needs to be retained (a high-level description of what the system should achieve). OICA Proposal: The data storage system should be sufficient to record and store ADS performance elements listed in Paragraph 7. This shall consist of data events (which are time stamps for particular events) and data elements (which are time series data). | Some of the group would like to keep some form of this provision and UK/US/JP will work in the interim to revise the language. Focus on the capacity of the system to record and store. Tentatively agreed to OICA proposal: The data storage system should be sufficient to record and store ADS performance elements listed in Paragraph 7. This shall consist of data events (which are time stamps for particular events) and data elements (which are time series data). |
|  | The DSSAD shall be protected against unauthorized access or manipulation/modification. The data may be stored on- or off-board the vehicle. | OICA comment: International Standards need to be referred to, e.g. ISO 21434. These are two different provisions, should be two paragraphs. UK Proposal: The DSSAD shall be protected against unauthorized access or manipulation/modification. The data may be stored on- or off-board the vehicle. | Agreed below. The DSSAD shall be protected against unauthorized access or manipulation/modification. |
|  | The DSSAD should be able to continuously store and time series data as determined by the regulatory authority/manufacturer to ensure the overall safety of the ADS system. Time series data is defined as data recorded over consistent intervals of time. [The DSSAD should be able to continuously store data. and time series data[ as defined by Annex XXX} by the regulatory authority/manufacturer to ensure the overall safety of the ADS system in compliance with national/regional requirements]. Time series data is defined as data recorded over consistent intervals of time.] | OICA comment: 1) Continuously should be deleted as it may be interpreted as continuous data recording. 2) Time stamp data must be applicable for normal conditions as amount of data to be stored will be too much to store if time series data will be required. 3) Overwrite rules need to be available and defined. The DSSAD should be able to continuously store and time series data as determined by the regulatory authority/manufacturer to ensure the overall safety of the ADS system. Time series data is defined as data recorded over consistent intervals of time. Or The DSSAD should be able to store time stamps for flags. Japan proposal: In case the system has a data upload function, the data should be stored at least in the vehicle until the data is uploaded to the off-board storage. The data should be stored on-board the vehicle unless the data is uploaded to the off-board storage. UK Proposal: The DSSAD should be able to continuously store and time series data as determined by the regulatory authority/manufacturer to ensure the overall safety of the ADS system. Time series data is defined as data recorded over consistent intervals of time. Korea Proposal: 1) Korea supports recording the time series data in DSSAD 2) Time series data is more effective than time stamp data to evaluate overall safety performance. 3) we also need to prepare for near-miss event or 'non-critical occurrence'. Time series data is helpful more than time stamp data. 4) EDR may not be able to record the data in non-critical occurrence. in this case, if all data elements in DSSAD are recorded by time stamp, is it possible to analyze the 'non-critical occurrence' case to fit the DSSADs definition? AUS: how long would the OEM be able to store this data? UK: Not sure we should revisit this language again. It was discussed at length at last meeting and we couldn’t come to an agreement. OICA: not sure we could discuss this issue at this point. This would be answered by individual CPs and OEMs. EC: may be able to remove this language if it is covered elsewhere. AAPC: The DSSAD should be able to continuously store and time series data as determined by the regulatory authority/manufacturer to ensure the overall safety of the ADS system. [move to definitions: Time series data is defined as data recorded over consistent intervals of time.] Are we talking about the availability of data? Whatever data you must store, the OEM must make sure it’s available. It could be on the system and or elsewhere. The availability of the data has to be assured. | Tentatively agreed below. Re-discussion required after agreement on Data Retrievability/Availability chapter. Agreed: In the case of the data intended to be stored off-board the vehicle cannot be transmitted, it should remain stored on the vehicle. |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | Data Format | OICA Comment Since the time stamp is recorded, there is no need to recommend recording interval times. Korea Proposal: 1) Need to distinguish between 'time stamp data' and 'time series data' 1-1) time stamp : there is no need recording interval times. 1-2) time series : To compare data between EDR and DSSAD, it needs not more than 0.5 seconds.. * if DSSAD doesn't record the time series data, we need to consider how to analyze the 'non-critical occurrence' using EDR. | Agreed below. Data Format |
|  | Each data element listed in Paragraph 6 should be stored in a standardized format by the regulatory authority or manufacturer/OEM/responsible party/regulatory authority. | OICA Proposal: Each data element listed in Paragraph 6 should be available in a standardized format. Each data element listed in Paragraph 6 should be stored in a standardized format by the regulatory authority or manufacturer/OEM/responsible party/regulatory authority. Or Each data element listed in Paragraph 6 should be available in a standardized format. OR Each data element listed in Paragraph 7 should be available in a standardized and readable format. Discussion: “Readable” was intended to mean not requiring specialized tools or external sources to interpret the data. May want to consider removing “not requiring specialized tools” since this may be required to protect data, although transparency and accessibility should be preserved. OICA/AAPC: Would like a written definition of what “readable” means to contracting parties. Need to consider the protection of CBI and/or personal data as well as legal concerns/sensitivities regarding these data. Japan Comment: Support. As much as the common data element can be retrieved, Authorities can analyze the data in the standardized way. Suggest reviewing paragraph 6.1 before returning to this item. UK Proposal: The performance elements in paragraph 7 should be stored in a standardised format. | Agreed to revisit at later date after discussion of item 6.1. Each data element listed in Paragraph 7 should be available in a standardized and readable format. |
|  | Each timestamp attached to the data should enable the party to determine when the intervention occurred. | Japan Comment: To be defined in the data element section Each timestamp attached to the data should enable the party to determine when the intervention occurred. | Deleted. |
|  | -- | Japan Proposal: To define time stamp data format. Time stamp data format 4.2.1. Time stamp data should be recorded in a clearly identifiable way with following data: (a) The event flag, as listed in paragraph 7.2.1. and 7.3.2. (b) Reason for the occurrence, as appropriate, and listed in paragraph 7.2.1. and 7.3.2. (c) Date (Resolution: yyyy/mm/dd); (d) Timestamp (i) Resolution: hh/mm/ss timezone e.g. 12:59:59 UTC; (ii) Accuracy: +/- 1.0 s. 4.2.2. A single timestamp may be allowed for multiple elements recorded simultaneously within the time resolution of the specific data elements. I f more than one element is recorded with the same timestamp, the information from the individual elements shall indicate the chronological order. UK Proposal: 6.2. Data Event 6.2.1 Each data event shall be recorded with the following data: (a)The data event, as listed in Table 1 of Paragraph 7.X.X. (b)The additional information noted in Table 1 for each data event, as appropriate. (c)Date (Resolution: yyyy/mm/dd); (d)Timestamp: (i)Resolution: hh/mm/ss timezone e.g. 12:59:59 UTC; (ii)Accuracy: +/- 1.0 s. (e)Location: (i)(GNSS) longitude + latitude + altitude; shall be recorded in decimal degrees and to at least [5] decimal places, but shall be unrounded. AAPC: would like further explanation of why “reason of occurrence” was included. Might consider revising this language. Accuracy: consider specifying higher resolution than +/- 1.0 s. Need to separate between time stamp data and any additional information/time series information. Location: implication in UK proposal that latitude/longitude/altitude must be calculated using GNSS, consider including an acceptable range of accuracy instead. OICA: Are geolocation data necessary? Are we tracking vehicle location at all times, or only if a data event is triggered? UK: Due to variability of location data, high resolution accuracy may be difficult to achieve. In the event of a crash, location information may be more easily recalled than time. GER: Currently requires geolocation data. JP: Need to carefully consider geolocation data and privacy protection. | Will revisit this at a later date. We first need to establish data elements, then we can discuss format. |
|  | --- | Japan Proposal: To define time series data format Time series data format 5.3.1. Time series data should be recorded over consistent recoding interval at certain sampling rate with sufficient resolution. 5.3.2. The sampling rate and resolution should be selected considering ADS design and feasible data size. (Remarks: further study is needed.) UK Proposal: 6.3. Data Elements 6.3.1. The data elements as listed in Table 2 of paragraph 7.X. shall be record when a data event occur that are identified as a triggering event in Table 1 of paragraph 7.X. 6.3.2. Each data element shall be recorded at least at the data sample rate note in Table 2 over the time period of [30s] before the triggering event to [30s] after." | Under discussion. Will revisit this at a later date. We first need to establish data elements, then we can discuss format. Time series data format 5.3.1. Time series data should be recorded over a specified period consistent recoding interval at certain sampling rate with sufficient an appropriate specified or relevant resolution. 5.3.2. The sampling rate and resolution should be selected considering ADS design and feasible data size. (Remarks: further study is needed.) |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | Data Retrievability/Accessibility | Japan proposal: Data Retrievability/ Accessibility Availability UK/US: Would like to keep original heading language. Need to establish elements of this section before we determine the final heading/title. OICA/US: Need to retain this chapter to establish how data is stored and retrieved so manufacturer responsibilities are clearly defined. OICA: Supports Japan’s proposal. | Will revisit. Need to establish elements of this section before we determine the final heading/title. |
|  | The data access interface in the vehicle should be standard and accessible for roadside control (France). | OICA Proposal: Data privacy issues - as DSSAD contains private data, data shouldn't be public. The data access interface in the vehicle should be standard and accessible for roadside control (France). Korea Proposal: 1) DSSADs must be recorded in the vehicle. 2) If the vehicle has crashed or the vehicle is burned fire, we suggest that data should be delivered to off-board at events. EC: Need to consider security and privacy issues. Agree with Korea that data must be preserved in case of fire, catastrophic crash, etc. AUS: Supports Korea’s proposal and EC comments. US: Need to spell out which parties have access to these data as well as data security provisions that must be considered. Data access is an issue of national law and is beyond the purview of this group. UK: By deleting, are we ruling out roadside access to data via vehicle interface? GER/UK: At least some data elements should be available by roadside access/vehicle interface. Would like to revisit this after we map out all data elements. US Proposal: The data access interface in the vehicle should be standardized*, and access should be defined by national law. *”Standardized” doesn’t necessarily mean there is a standard, just that there is consistency in access. OICA comment: are we asking about standardization of data port, or the kind of data? Need to understand/define this further. If we standardize DSSAD but not EDR, that would require more equipment. Should consider standardizing both at the same time. | US proposal: may be redundant and will return to this item. UK/GER/US proposal: revisit this after we map out all data elements. Focus should be on technical feasibility of access via the vehicle, not defining which authorities would have access to those data (that would be for national law to determine). EC/GER/UK proposal: revisit items 53, 54, 55, and 56 to potentially consolidate these items and conclude discussion. Should propose entirely new text rather than revisiting these items again. Agreed: All of the stored data defined in Section 6 of this guidance document should be accessible to entities as defined under national law. |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | The timestamped data should be retrievable by the OEM upon request, including after an impact or when the main onboard vehicle power supply is not available. | OICA Proposal: The timestamped data should be provided by the OEM upon request from a CP designated authority, including after an impact or when the main onboard vehicle power supply is not available. Japan Proposal: The stored data should be available by the request from safety Authority for their performance analysis The timestamped stored data listed in Paragraph 7 should be retrievable by the OEM vehicle or system manufacturer upon request from vehicle safety authority or relevant legal jurisdiction or the owner with permission including after an impact or when the main onboard vehicle power supply is not available. UK Proposal: The stored data shall be accessible on the vehicle and downloadable from the vehicle either wirelessly or via a standard connection [, if the condition of vehicle allows,] by either the ADS user or by an authority which has a legal requirement to access the data. EC Proposal: The stored data shall be accessible on the vehicle and downloadable from the vehicle either wirelessly or via a standard connection [, if the condition of vehicle allows,] by either the ADS user or an entity designated under the law of the contracting party. US Proposal: …Timestamp data must be accessible to the contracting party after impact. Germany Proposal: … Needs to be retrievable by authorized parties. Discussion: - Need to keep data privacy and security in mind, limit scope of access (US and OICA) - Keep language very simple (US and OICA) - “By request” might leave contracting parties dependent on OEMs and data would have to pass through OEM hands before authorities see it (UK) - Allowing access to data through requests to OEMs will allow for greater data protection (OICA) - Need to limit data access to contracting parties (not private entities) (US and OICA) - Keep language limited to how devices are designed and operated, stay within the scope of a vehicle regulation. Authorization of access typically handled through other authorities, and need to specify that the data should be protected against unauthorized access (AAPC) - Need to be very clear about who has access to these data and how the data will be used – would need to define the term “authorized parties” (US and OICA) - When designing the system, contracting party access to the information should be central to the design (US) - Limiting access to contracting parties would make it clear that how data are accessed are dependent on national law (OICA) - Specifying “system or vehicle manufacturer” could get into business-to-business contracts, use broader term “manufacturer” instead (Waymo) - UK could consider adding second paragraph: “For claim reasons, the driver of the vehicle should be able to access a log showing whether or not the vehicle was performing the DDT.” (OICA) - Potential use of “claim reasons” indicates insurance issues. Language should be limited to contracting party access, and contracting parties decide how the data are used. (US) - The issue of unauthorized access has already been agreed to in item 45. (UK) - Need to consider harmonization as we draft language (OICA) Proposed language for consideration as a result of discussion documented above: - Stored data shall be available for retrieval in accordance with applicable law of the contracting party. OR - The manufacturer should design the system to ensure the accessibility of data to authorized entities (in accordance with national laws). OR - The stored data listed in Paragraph 7 [now Paragraph 6] should be retrievable by the manufacturer for use by the contracting party’s vehicle safety authority or relevant legal jurisdiction, or the vehicle owner (with permission), including after an impact or when the main onboard vehicle power supply is not available. UK, US, EC, and Korea support options 1 or 2, OICA supports option 3. UK comment that waiting on manufacturer to provide information back to authorities would hinder the investigation of incidents. US/UK/EC Proposal, combining 1 and 2: The manufacturer should design the system to ensure the [immediate] accessibility of data [as defined in Paragraph 6] to authorized entities in accordance with the applicable law of the contracting party. OICA suggested adding: “If immediate access to data is required…” OICA also noted that the data belong to someone, so privacy and protection are still an issue, and defining “immediate” introduces additional difficulties. EC proposed keeping options 1 and 2 for now, and choose the most consistent option once other language is finalized. US proposed deleting this item and leaving it to national law. | Revisit proposed language once elements are established. EC/GER/UK proposal: revisit items 53, 54, 55, and 56 to potentially consolidate these items and conclude discussion. Should propose entirely new text rather than revisiting these items again. Agreed: The manufacturer should ensure the data is promptly available in a format that is standardized and readable as outlined in item 4.1. |

|  | It is the obligation of the manufacturer to ensure that the data listed above is provided upon the request of the regulatory authority in an readable format via the use of an electronic communication interface (R157) even when the main onboard vehicle power supply is not available. The method to retrieve the data will be determined by the manufacturer. | OICA proposal/comment: Delete, 6.3 is a repeat of 6.2. Also support edited text below. Japan proposal/comment/justification: Modify 6.3 and define the way of data retrieval either via data port or the method defined by manufacturer, by considering into the reality that various ADS design can be possible especially during emerging period. Edited text for discussion: It is the obligation of the manufacturer to ensure that the data listed above is provided upon the request of the regulatory authority in an readable format The stored data can be retrieved via the use of an electronic communication interface (R157) even when the main onboard vehicle power supply is not available. The, or the method to retrieve the data will be determined by the manufacturer. Discussion: - If manufacturers can decide on method of data retrieval, we may have too many tools that could be used. How manufacturers store the data is up to them but consider limiting methods of data retrieval to increase accessibility. (Germany) - Not comfortable with standardization across industry, would limit freedom of design (OICA) - Would help to clarify what exactly this section is asking; are there distinctions between data stored onboard the vehicle vs. data stored elsewhere? If there are connections on the vehicle, those should be standard; for data stored off-board, considering something potentially open-source or web-based to allow access to additional data without the need for [specialized tools]. (UK suggested and US supports) - Ongoing concern with the protection of data security and privacy. Not clear on what “standard” means and would like to keep this open since there are still unresolved issues. (OICA) - Suggest general text about accessibility (either using “electronic interface” or “as defined by manufacturer”, since we don’t know what the standard method of accessing onboard vehicle data might be. (Japan) - Need to balance need for data accessibility and protection of privacy by including end-to-end security considerations. (AAPC) GER comments: May need to discuss what exactly this provision is intended to be about. Should some data be available from the vehicle? How does that differ from data that may be stored in the cloud or elsewhere? Is this about tools, or is it about retrievability (which are very different things)? EC comment: 53, 54, and 55 are all centred around retrievability. Not sure what the added value of this is compared to the text in items 53 and 54. Japan/US proposal: Delete this item entirely as it duplicates previous items. GER comment: It appears that data accessibility even if power supply is not available is unique to this provision and should be addressed here or, if this is deleted, should be added to another item. | Need to work on revised language based on discussion 7/19, including considerations of where data might be stored (on or offboard the vehicle) and what exactly we are trying to address with this item. EC/GER/UK proposal: revisit items 53, 54, 55, and 56 to potentially consolidate these items and conclude discussion. Should propose entirely new text rather than revisiting these items again. Agreed: The DSSAD data (whether stored on or off-board the vehicle) should be available and retrievable through an electronic communication interface. |
| --- | --- | --- | --- |

|  | The manufacturers should provide an information package to regulatory authorities, any interested manufacturers or repair of components, diagnostic tools, or test equipment in machine readable way (e.g., ISO 22901) which includes the information about how the timestamped data can be retrievable and interpretable via the use of the electronic communication interface. | OICA Proposal: This requirement should be modified to "Instructions from the manufacturer shall be provided on how to access the data.", as same as R157 8.4.6. Info to repair and maintenance will come from reading out the ECU in the usual way and is not related to DSSAD. Instructions from the manufacturer shall be provided on how to access the data. Japan Proposal: Usually, safety Authority have a power to investigate in detail, if there is a reasonable suspicion on the provided data. Therefore, no need to define in detail, in this guideline. Instructions from the manufacturer shall be provided on how to access the data, if requested from safety Authority. UK Proposal: It shall not require any specialist tools or software to read the data. US Proposal (combining UK and Japan proposals above): Instructions from the manufacturer shall be provided on how to access the data [if requested by the safety authority]. It should not require any specialist tools or software to read the data. Discussion: - If specialized tools or software required, the manufacturer could provide those to the contracting party. (US) - Defining the output would help define data readability and interpretation needs. Would require OEM-specific software to translate into a readable format (OICA) - Easier for CP to go to manufacturer to request data in a readable format, rather than standardizing data formatting for all manufacturers (SAE) - From perspective of technology neutrality, it is not appropriate to require no special tool. it is enough to mandate giving tools or something to authority upon request. (Japan) At June session, tentatively agreed to language below: Instructions from the manufacturer shall be provided on how to access the data [if requested by the safety authority]. It should not require any specialist tools or software to read the data. Further discussion: - We shouldn’t limit data accessibility and retrievability just to safety authorities, individuals should have a right to access that data as well. Previous paragraph might dictate whether or not this additional item is necessary. (UK) - Agree that previous section should be defined first, and support Japan’s comment above regarding technology neutrality. (OICA) | Revisit at a later date. If comments regarding instruction, technology neutrality, and accessibility (no specialized tools required) are incorporated into block 55, this section could be deleted. EC/GER/UK proposal: revisit items 53, 54, 55, and 56 to potentially consolidate these items and conclude discussion. Should propose entirely new text rather than revisiting these items again. Agreed: The manufacturer should ensure there is a method to access the data and provide an information package about its usage to authorized entities. The method of accessing data via this interface shall be documented by the manufacturer and provided upon the request of the regulatory authority including any tools or software which are required for access. |
| --- | --- | --- | --- |
| 56 | The stored data should be retrievable even when the main onboard vehicle power supply is not available. |  | Agreed. |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | The DSSAD [onboard the vehicle] should be able to preserve the data even when exposed to shocks, extreme temperatures, water, or crashes. | OICA Proposal: Delete, this paragraph assumes the data is stored onboard. Japan Proposal: The DSSAD [onboard the vehicle] should be able to preserve the data even when exposed to shocks, extreme temperatures, water, or crashes. 5.5 The stored data elements should be retrievable even after an impact of a severity level set by relevant regulations (e.g. UN R94, R95 or R137). 5.5.1. Alternatively, only for heavy-duty vehicles, the stored data should be deemed retrievable if the DSSAD withstand mechanical shocks in the component test at a severity level specified in relevant regulations (e.g. Annex 9C of the 03 series of amendment to UN R100), and be mounted cab/passenger compartment or in a position of sufficient structural integrity to protect against physical damage that would prevent the retrieval of data at least in front and side impacts of a severity corresponding to the mechanical shock aforementioned. UK Proposal: 5.3. The stored data should be retrievable even after an impact of a severity level set by relevant regulations (e.g. UN R94, R95 or R137). 5.3.1 Alternatively, only for heavy-duty vehicles, the stored data should be deemed retrievable if the DSSAD withstand mechanical shocks in the component test at a severity level specified in relevant regulations (e.g. Annex 9C of the 03 series of amendment to UN R100), and be mounted cab/passenger compartment or in a position of sufficient structural integrity to protect against physical damage that would prevent the retrieval of data at least in front and side impacts of a severity corresponding to the mechanical shock aforementioned." US Proposal: 5.3. The stored data should be retrievable even after an impact. Korea Proposal: Data must be stored onboard to confirm data reliability even if data is stored off-board too. so, DSSAD should be safely preserved for analyzing the data in serious accidents. therefore, the protective performance requirements of DSSAD are needed. but, we continue to research about it. Discussion: - Shouldn’t cite specific regulation numbers or severity level, since those might change. Don’t need to include heavy-duty provisions here. Keep 5.3 but not 5.3.1. Regardless of where data system is stored or what it is exposed to, data must remain accessible. Manufacturer can choose where to store the data, and data storage needs could be costly to manufacturers. (US) - Need to be able to differentiate between whether human was driving or system was driving. Assessment of occupant protection needed, too. (Germany) - Could consider which data should be crash-safe to reduce burden and cost to manufacturers. Could be some redundancies with other EDRs. Also need to consider how long data are stored to reduce burden to manufacturers (so not keeping all data forever). (OICA) - Not possible to have data survive every incident, we should define terms such as “shock” and “crash” more specifically. Should address what to do with data stored offboard that might not have been transmitted due to connectivity issues/lapses (and how to preserve any not-transmitted data in the case of a crash, etc). (UK) - Point of this section is survivability of data, regardless of the severity of the incident or how/where data are stored (US, UK, and Japan) - If we keep broader US proposal, would need to define “impact” (UK) - Support Japan’s proposal since it is consistent with language outlined and established elsewhere, including citing specific regulations. (EC) Japan/US Revised Proposal: 5.5 The stored data elements should be retrievable even after an impact of a severity level set by relevant regulations. 5.5.1. Alternatively, only for heavy-duty vehicles, the stored data should be deemed retrievable if the DSSAD withstand mechanical shocks in the component test at a severity level specified in relevant regulations (e.g. Annex 9C of the 03 series of amendment to UN R100), and be mounted cab/passenger compartment or in a position of sufficient structural integrity to protect against physical damage that would prevent the retrieval of data at least in front and side impacts of a severity corresponding to the mechanical shock aforementioned. Further discussion: - Support Japan/US revised proposal, removing references to specific regulation, but still need to consider length of storage. (OICA) | US, UK, Japan, OICA, and EC support the following, and this language has been tentatively agreed to: The DSSAD [onboard the vehicle] should be able to preserve the data even when exposed to shocks, extreme temperatures, water, or crashes. 5.5 The stored data elements should be retrievable even after an impact of a severity level set by relevant regulations. 5.5.1. Alternatively, only for heavy-duty vehicles, the stored data should be deemed retrievable if the DSSAD withstand mechanical shocks in the component test at a severity level specified in relevant regulations (e.g. Annex 9C of the 03 series of amendment to UN R100), and be mounted cab/passenger compartment or in a position of sufficient structural integrity to protect against physical damage that would prevent the retrieval of data at least in front and side impacts of a severity corresponding to the mechanical shock aforementioned. |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | Performance Elements: | Korea Proposal: Why don't we use the matrix to discuss? * SG-DSSAD-14-02 DSSAD Trigger-Data Elements Proposal Comparison Matrix |  |
|  | The DSSAD should store the performance elements listed below [and provide upon request the relevant sensor data from cameras, radar and LIDAR from the vehicle (SAE) and tools to analyze these data:] | FRAV proposal The words should change to: The DSSAD should store the performance elements listed below if applicable. 1) because it allows to require sensor date other than DSSAD recorded. 2) And "tools to analyze data" is a propriety of manufacture or providing it may have a copyright issue of suppliers as well. 3) Contradicts with Tech-neutrality Required data should be listed, not repeated. 4) the words 'if applicable' are added to address ADS that permit a TOC whilst the vehicle is in motion and those which don’t (by design) The DSSAD should store the performance elements listed below if applicable. Japan Proposal: DSSAD data is the good starting point of the understanding of the certain point of time and the safety performance analysis / evaluation by vehicle safety authority. Suggest to delete latter half, for the simplification of this Guidance document. (Because usually vehicle safety Authority has a regal power to investigate and demand information, not limited to DSSAD data, if reasonable suspicion exists, regardless this guidance document.) The DSSAD should store the performance elements listed below for the purpose of understanding of ADS status and the further analysis of ADS safety performance. [and provide upon request the relevant sensor data from cameras, radar and LIDAR from the vehicle (SAE) and tools to analyze these data:] if applicable. |  |
|  | The DSSAD should record activation of a system fault warning. It should record activation of the ADS. The DSSAD shall record activation of a signal to indicate availability of an ADS feature for activation. The DSSAD shall record activation of an ADS feature. Any change in ADS mode should be recorded. | OICA Proposal: Paragraph 7.2 is a repeat of 7.3 and should be deleted. The DSSAD should record activation of a system fault warning. It should record activation of the ADS. The DSSAD shall record activation of a signal to indicate availability of an ADS feature for activation. The DSSAD shall record activation of an ADS feature. Any change in ADS mode should be recorded. Japan Proposal: Reorganize proposal on 7.2 and after | Agreed. |
|  | ADS Mode |  |  |
|  | ADS activated |  |  |
|  | ADS fault warning | OICA Proposal: delete, repeat of 7.12 |  |
|  | ADS feature available | OICA Proposal: delete. Please justify safety impact. More information could be taken from ADS feature activation. This will greatly increase the amount of data collected. |  |
|  | ADS performance of the DDT activated', |  |  |
|  | ODD exit notification issued | OICA Proposal: Transition of control requested. Also need MRC initiated and MRC complete. |  |
|  | Transition of control initiated | OICA Proposal: Change to: User control inputs detected |  |
|  | User control inputs satisfactory | OICA Proposal: Transition of control completed / MRC complete |  |
|  | ADS feature deactivated. | OICA Proposal: Repeat of 7.3.7, propose to delete. |  |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | Termination of ADS performance of the DDT | OICA Proposal: Repeat of 7.3.7, propose to delete |  |
|  | Speed/Velocity [Speed settings, frequency, intervals, timing] | OICA Proposal: Please clarify if this is crash related? Then EDR. |  |
|  | ADS initiated lane changes | OICA Proposal: Delete, this has no added value. |  |
|  | Turn signal activated (specify direction) | OICA Proposal: Delete, this has no added value. |  |
|  | Transition of control | OICA Proposal: Delete, repeat of 7.3.6 & 7.3.7 |  |
|  | Notifications, responses | OICA Proposal: Too broad, please specify |  |
|  | ADS monitoring the environment (road signs, construction, oncoming traffic, interactions with pedestrians and other vehicles) including sensor information. | OICA Proposal: too broad, needs more definition. Need to clarify how to use this information. These items should not be events but rather data fields for events, MRC initiated, transition of contol complete…. |  |
|  | Status of Electrical Power Supply | OICA proposal: delete, repeat of 7.12. |  |
|  | ADS failure | OICA proposal: The approval of an ADS includes a review of the faults detected by the diagnostic system. DSSAD might record the DTC of a fault when the fault warning is activated. |  |
|  | Other vehicle systems failures (maintenance and repair problems) | OICA Proposal: Too broad, if a failure is not ODD relevant, no need to record. |  |
|  | Environmental/weather conditions | OICA proposal: Need exact definition of what this means (rain sensor, temperature sensor, light sensor), and how this relates to the ODD, and the added safety value of recording it. |  |
|  | Emergency stop request/fallback to MRC initiated | OICA Proposal: 'Emergency stop request', other aspects of this are covered above. |  |
|  | Minimal Risk Condition: ADS falls back to MRC | OICA Proposal: delete, this is covered by 7.3.7 |  |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | --- | Japan Proposal: Restructuring the data elements into ADS status data and ADS safety performance analysis data, and corresponding sections in 7.2 and after. Data elements for understanding of ADS status (Remarks: need to be aligned with ADS design requirements, especially on driver / operator related status.) 6.2.1. The DSSAD should record an entry for each of the following events as time stamp data for recording ADS status: (a)Activation of the system (b)Deactivation of the system, due to: (i)Use of dedicated means for the driver/operator to deactivate the system; (ii)by fallback user (if applicable) (c)Transition of control to the fallback user, if applicable, due to: (i)Planned event; (ii)Unplanned event; (iii)fallback user unavailability (iv)fallback user not present or unbuckled (v)System failure; (vi)System override (d)Prevention of takeover under unsafe conditions, if applicable; (e)Start of Emergency Manoeuvre; (f)End of Emergency Manoeuvre; (g)Event Data Recorder (EDR) trigger input (h)[Detected collision] (i)Execution of a fallback to an MRC (j)Failure to achieve a MRC (k)Detected severe failure, on; (i) ADS (ii) vehicle [(l) Interaction with remote operator, if applicable] UK Proposal: 7.1. Data Event Details 7.1.1. The following table details the data events to be recorded, along with any additional information, and identifies which data event is a triggering event. Data Event; Additional Information; Triggering Event. [TBC]; [TBC]; [TBC]. |  |
|  | --- | Japan Proposal: Restructuring 6.3 Data elements for analysis of ADS safety performance 6.3.1.The DSSAD should record the data elements as specified in paragraph 6.3.2. and 6.3.3. if one of the following data element was recorded (a)EDR trigger input (b)Detected collision 6.3.2.The DSSAD should record an entry for each of the following occurrences as time stamp data if they happen within the [X seconds] before the event as per paragraph 6.3.1. : (a)ADS [feature/manoeuvre] activation (b)Abortion of ADS [feature/manoeuvre] (c)End of ADS [feature/manoeuvre] UK Proposal: 7.2. Data Elements Details 7.2.1. The following table details the data elements to be recorded during a triggering event, along with the data sample rate and resolution. Data Element; Resolution; Minimum data sample rate (samples per second). [TBC]; [TBC]; [TBC]. |  |
|  | --- | Japan Proposal: Restructuring 6.3.3. The DSSAD should record the following elements as time series data within the [X] seconds before the event as per paragraph 6.3.1. : (a) ADS detected objects within relevant area of the ADS vehicle at the event by (i) ADS detected distance to each detected object, and (ii) ADS detected relative velocity of each detected object [and/or (instead of (a), (b)) relevant sensor data (e.g. cameras, radar, LIDAR), if available] (b)[Detected location of the ADS vehicle (in GNSS coordinates)] (c)ADS control signal (e.g. steering wheel angle, accel/brake control) (d)Dynamics of the vehicle (e.g. velocity, acceleration, yaw rate) |  |

|  | Original draft text | Comments/Proposals | Discussion Outcome(s) |
| --- | --- | --- | --- |
|  | Misc. Specifications |  |  |
|  | Automated vehicles should be equipped with a device that records the operational status of the ADS. (Japan) | OICA Proposal: delete, repetition of chapter 6.3.1 Automated vehicles should be equipped with a device that records the operational status of the ADS. (Japan) Japan Proposal: Support OICA Proposal | Agree to delete this item. |
|  | Data uploaded [by whom] should be defined at a level of ensure privacy but enable timestamps so sequences can be understood, and “occurrences” (based on sequence patterns) can be identified and reported. | OICA Proposal: Privacy has to be protected regardless it is uploaded or not, therefore propose to delete "uploaded". This should be addressed by establishing DSSAD data records that are anonymous. Data uploaded [by whom] should be defined at a level to of ensure privacy but enable timestamps so sequences can be understood, and “occurrences” (based on sequence patterns) can be identified and reported. Japan Proposal: Proposes to describes in general way. DSSAD is allowed to be retrieved from vehicle safety authority or relevant regal jurisdiction, or the DSSAD data owner with permission. The retrieved data has to be treated to protect the privacy (e.g. restricted for the allowed usage, restriction to the data access, data retention period, anonymization) Discussion: - Believe this is an important consideration to keep in the document. (US) - Need to understand the purpose of this item and how privacy would be protected. As written, the original statement is confusing. (EC and US) - We should already have language to prevent unauthorized access to data (in item 45)– is this sufficient to address what this item covers? (EC) OICA Proposal Since this is addressed in item 45, propose deleting item 88. | Agreed that this item is addressed in item 45, and will delete. |
|  | The manufacturer should provide authorities or parties entrusted by national or regional legislation the information which allows access to the time-stamped data. | OICA Proposal: delete, repeat of 6.2 The manufacturer should provide authorities or parties entrusted by national or regional legislation the information which allows access to the time-stamped data. Japan Proposal: Support to avoid duplication | Agree to delete this item. |
|  | If the access to timestamped data is protected, the manufacturer should ensure authorities and parties authorized by national or regional legislation the authorization to access the timestamped data. | OICA Proposal: delete, repeat of 6.2 If the access to timestamped data is protected, the manufacturer should ensure authorities and parties authorized by national or regional legislation the authorization to access the timestamped data. Japan Proposal: Support to avoid duplication | Agree to delete this item. |
|  | Connectivity | Discussion: - Do we deal with this under the topic of data survivability, or does it need to be explicitly addressed here? (UK) - Would like to keep this item for future discussion/consideration. (US) |  |
|  | Confidentiality of Data | Discussion: - Would like to keep this item for future discussion/consideration. (US) - Consider adding stipulation that information/data provided to authorities should only be used in confidential ways. (OICA) |  |

This definition is based on SAE J3016 and ISO/PAS 22736 (Taxonomy and Definitions for Terms Related to Driving Automation Systems for On-Road Motor Vehicles). These standards define levels of driving automation based on the functionality of the driving automation system feature as determined by an allocation of roles in DDT and DDT fallback performance between that feature and the (human) user (if any). The term “Automated Driving System” is used specifically to describe a Level 3, 4, or 5 driving automation system.

Examples include minor incidents, safety degradation not preventing normal operation, emergency/complex maneuvers to prevent a collision, and more generally all occurrences relevant to the safety performance of the in-service ADS (like transfer of control, interaction with remote operator, etc.).

Examples include minor incidents, safety degradation not preventing normal operation, emergency/complex maneuvers to prevent a collision, and more generally all occurrences relevant to the safety performance of the in-service ADS (like transfer of control, interaction with remote operator, etc.).

Examples include but are not limited to environmental, geographical, and time-of-day restrictions, and/or the requisite presence or absence of certain traffic or roadway characteristics.

The TOC request, depending on the ADS design and reason for initiation of the transition of control, may aim to engage the user in performing the DDT (i.e., to the role of driver manually operating the vehicle) or to achieve an MRC.

[Contracting parties may further define technical specifications for data accessibility [under national law].]

If the system has more than one [feature/manoeuvre], the data element should be recorded in a distinguishable way whether which feature is activated/deactivated.