Proposal

2.		Definitions

[…]

2.xx.	“Advanced emergency braking system” means a system which can automatically detect an imminent forward collision and activate the vehicle braking system to decelerate the vehicle with the purpose of avoiding or mitigating a collision.

2.xx.	“Trigger activated” indicates the first trigger that was activated to cause the recording of the event.

2.yy.	“Vulnerable road user” means a person using no vehicle, such as a pedestrian, or using a vehicle without protective occupant compartment, such as a pedal cyclist, micro-vehicle user or motorcyclist.

5.	Requirements

[...]

5.3.1.	Conditions for triggering recording of data

An event shall be recorded by the EDR if one of the following threshold values is met or exceeded:.	If a vehicle is not fitted with a system referred to in Paragraphs 5.3.1.3. to 5.3.1.5., this document requires neither recording of data according to these paragraphs nor fitting of such systems.

5.3.1.1.	Change in longitudinal vehicle velocity more than 8 km/h within a 150 ms or less interval.

5.3.1.2.	Change in lateral vehicle velocity more than 8 km/h within a 150 ms or less interval

5.3.1.3.	Activation of Non-reversible occupant restraint system.

5.3.1.4.	Activation of Vulnerable road user secondary safety system

If a vehicle is not fitted with any Vulnerable Road User (VRU) secondary safety system, this document requires neither recording of data nor fitting of such systems.  However, if the vehicle is fitted with such a system, then it is mandatory to record the event data following activation of this system.

5.3.1.5.	Emergency braking demand by the advanced emergency braking system in response to detecting the possibility of an imminent collision.

[...]

5.3.3.	Conditions for establishment of time zero

Time zero is established at the time when any of the following first occurs:

5.3.3.1.	For systems with "wake-up" air bag control systems, the time at which the occupant restraint control algorithm is activated; or

5.3.3.2.	For continuously running algorithms,

5.3.3.2.1.	The first point in the interval where a longitudinal, cumulative delta-V of over 0.8 km/h is reached within a 20 ms time period; or

5.3.3.2.2.	For vehicles that record "delta-V, lateral," the first point in the interval where a lateral, cumulative delta-V of over 0.8 km/h is reached within a 5 ms time period; or

5.3.3.3.	Deployment of a non-reversible deployable restraint or activation of VRU secondary safety protection system.

5.3.3.4.	Occurrence of the emergency braking demand trigger of the advanced emergency braking system.

[…]

Annex 4 - Data elements and format

Add “AEB” to column ‘Event(s) recorded for’ against the following data elements:

- Speed, vehicle indicated

- Engine throttle, % full (or accelerator pedal, % full)

- Service brake, on/off

- Ignition cycle, crash

- Ignition cycle, download

- Safety belt status, driver

- Complete file recorded

- Engine rpm

- Anti-lock braking system activity

- Stability control

- Safety belt status, front passenger

- Safety belt status, rear passengers

- Tyre Pressure Monitoring System Warning Lamp Status

- Longitudinal acceleration (pre – crash)

- Lateral acceleration (pre – crash)

- Yaw Rate

- Advanced Emergency Braking System status

- Cruise Control System Status

- Adaptive  Cruise Control  Status (driving  automation  system level  1)

- Safety belt  status mid-  position front

- Lane departure  warning system  status

- Corrective  steering  function status

- Emergency  steering  function status

- Automatically  commanded  steering  function  category A  status

- Automatically  commanded  steering  function  category B1  status

- Automatically  commanded  steering  function  category B2  status

- Automatically  commanded  steering  function  category C  status

- Automatically  commanded  steering  function  category D  status

- Automatically  commanded  steering  function  category E  status

- Accident  emergency call  system status

Add the following new data element:

| Data element | Condition for requirement | Recording interval/time | Data sample rate | Minimum range | Accuracy | Resolution | Event(s) recorded for |
| --- | --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … | … | … |
| Trigger activated | Mandatory | Event | N/A | N/A | N/A | Change in longitudinal velocity, Change in lateral velocity, Activation of non-reversible occupant restraint system, Activation of vulnerable road user secondary safety system, [Advanced emergency braking demand for suspected vulnerable road user, Advanced emergency braking demand for suspected non-vulnerable road user (e.g. M- or N-category motor vehicle)] | All 5.3.1. triggers |

Justification

The aim of this proposal is to improve the capture rate of collisions which may be missed by the currently defined triggers, including but not limited to collisions with vulnerable road users, by using advanced emergency braking system interventions as a trigger criterion. This updated VERSION 2 of proposal SG-EDR-40-02 takes on-board feedback from the SG EDR.

Definitions:

The definition for ‘advanced emergency braking system’ is taken from UN Regulation No. 152, the regulation specifying requirements for these systems, with an additional sentence to clarify that the system is sometimes referred to by a different term. The definition is almost identical to that used in UN Regulation No. 169 except for the first sentence which reads “a system which can automatically detect an imminent forward collision” (UNR152, 2.1.) rather than “a system which can automatically detect a potential forward collision” (UNR169, 2.5.).

The definition of ‘trigger activated’ for the corresponding new data element is based on the definition used in UNR169, 2.38. and was expanded following group feedback to indicate clearly that only the first trigger causing the event record has to be recorded in case of multiple triggers covering the same event.

The definition for ‘vulnerable road user’ was developed in the informal working group EDR/DSSAD when discussing VRU secondary safety systems but was ultimately not used because it was not considered necessary in that context to define the term. It aims to capture all road users who are not protected by an external, crash-absorbing structure, and therefore face higher injury risk.

Requirements:

It is proposed to clarify in Paragraph 5.3.1. that any triggers except changes in longitudinal or lateral velocity are ‘if fitted’ requirements, i.e. do not demand fitment of the corresponding safety system (such as airbags, pop-up bonnets or AEBS) solely for the purpose of the EDR regulation.

The purpose Paragraph 5.3.1.5. is to improve the capture of collisions involving vulnerable road users, because these collisions with light collision opponents are inadequately addressed with current deceleration-based triggers (see document EDR-DSSAD-IWG-24-09). Furthermore, it is believed that the breadth of vehicle-to-vehicle collisions captured successfully can also be improved by including any emergency braking demand of the advanced emergency braking system as a trigger, irrespective of which type of target was detected. A UNR152-compliant advanced emergency braking system can detect motor vehicles, pedestrians and bicycles, provide a collision warning to the driver and, when a collision is imminent, demand an automatic emergency braking intervention of at least 5.0 m/s2. Some system implementations may have additional capabilities such as detecting motorcyclists.

Depending on the relative speeds and conditions, AEBS can mitigate or entirely prevent a collision. A prevented collision would result in a false positive EDR record. To minimise the number of false positive records, it is proposed to require a trigger only for an emergency braking demand rather than the preceding collision warning (note: this is equivalent to the requirements in UNR169, 5.3.1.3.). Remaining false positive records are considered unavoidable if the aim should be achieved to capture a larger share of vulnerable road user collisions based on current on-board sensor technology and established algorithms.

In Paragraph 5.3.3.4 is it is proposed to define ‘time zero’ for the new trigger analogous to UNR169, 5.3.3., i.e. as the time when the trigger occurs.

Note that an extended recording duration will be required to capture the collision which may occur with an unknown time delay after the trigger (compare with rollover event in UNR160). This should be discussed in a separate proposal.

In relation to data elements in Annex 4 it is proposed which data elements to record in case of an AEB trigger. From the complete list of ‘planar’ data elements, this omits data elements related to airbag deployment (because these would be captured in a separate event in case an airbag is actually deployed) and high frequency acceleration and delta-v data (because the AEB trigger does not coincide in time with the beginning of a crash but occurs some time before; therefore, this data may be misleading because it does not capture the actual time of the crash).

Furthermore, it is proposed to add a new element ‘Trigger activated’. This data element also exists in UNR169 and is relevant because with the addition of the proposed AEBS trigger it will become relevant for analysts to understand whether the event was a certain crash event (e.g. from airbag deployment trigger) or a suspected collision event based on pre-crash safety system intervention.

It is proposed to differentiate between AEBS emergency braking interventions in response to VRU targets and non-VRU target (typically motor vehicles of M or N category). This information will help researchers to identify records of vulnerable road user collisions in large collections of EDR datasets to allow targeted research in the increasingly relevant area of VRU safety. It is understood that the classification of targets is not always accurate and the data entry should be based on the best available information the AEBS system has available at the time of the brake demand. The word ‘suspected’ was included in the proposed title of the data entry (e.g. ‘Advanced emergency braking demand for suspected vulnerable road user’) to clarify that the classification is not always reliable.

Note that it should be clarified in the regulation how multiple triggers relating to the same event should be handled by the EDR. This should be discussed in a separate proposal.

The system may also be referred to as "Automatic emergency braking system”.