Proposal by the NL IWG Co-Chair amending Regulation No. 160. This editorial amendment aims clarifying the particular crash tests needed for evidencing the crash test performance and survivability. The proposed amendment can be included with a next update, potentially EDR Step 2. The modifications to the original text are marked in bold for new or strikethrough for deleted characters.

- Proposal

Paragraph 5.4., amend to read:

"5.4. Crash test performance and survivability

5.4.1. Each vehicle subject to the requirements of national or regional frontal crash
test regulations, shall conform with the specifications in paragraph 5.4.3.

5.4.2. Each vehicle subject to the requirements of national or regional side impact crash test regulations shall conform with the specifications of paragraph 5.4.3.

5.4.3. The data elements required by paragraph 5.1, shall be recorded in the format specified by paragraph 5.2, exist at the completion of the crash test and the complete data recorded element shall read "yes" after the test. Elements that are not operating normally in braking, etc.) are not required to meet the accuracy or resolution requirements in these crash tests.

The data shall be retrievable even after an impact of a severity level set by UN Regulations Nos. 94, 95 or 137 94 or 137, and 95.”

- Justification

Different views had been addressed during type approval activities on which collision tests are required to be conducted for evidencing the crash test performance and survivability:

- a side AND a full width frontal AND an offset front collision, or

- a side OR a full width frontal OR an offset front collision,

- a side AND a frontal collision, being either full width OR offset.

From 5.4.1. and 5.4.2. it follows that c) is the correct interpretation. The proposed amendment aims clarifying this.

_________________