Proposal

2.	Definitions

[...]

2.x	“Rollover protection system" means any protection system that protects occupants during a rollover, which is activated by a vehicle's crash sensing system.

5.	Requirements

[...]

5.3.1.	Conditions for triggering recording of data

An event shall be recorded by the EDR if one of the following threshold values is met or exceeded:

5.3.1.1.	Change in longitudinal vehicle velocity more than 8 km/h within a 150 ms or less interval.

5.3.1.2.	Change in lateral vehicle velocity more than 8 km/h within a 150 ms or less interval

5.3.1.3	Activation of Non-reversible occupant restraint system, including the rollover protection system.

5.3.1.4.	Activation of Vulnerable road user secondary safety system

If a vehicle is not fitted with any Vulnerable Road User (VRU) secondary safety system, this document requires neither recording of data nor fitting of such systems.  However, if the vehicle is fitted with such a system, then it is mandatory to record the event data following activation of this system.

[...]

5.3.3.	Conditions for establishment of time zero

Time zero is established at the time when any of the following first occurs:

5.3.3.1.	For systems with "wake-up" air bag control systems, the time at which the occupant restraint control algorithm is activated; or

5.3.3.2.	For continuously running algorithms,

5.3.3.2.1.	The first point in the interval where a longitudinal, cumulative delta-V of over 0.8 km/h is reached within a 20 ms time period; or

5.3.3.2.2.	For vehicles that record "delta-V, lateral," the first point in the interval where a lateral, cumulative delta-V of over 0.8 km/h is reached within a 5 ms time period; or

5.3.3.3.	Deployment of a non-reversible deployable restraint or activation of VRU secondary safety protection system.

Justification

There have been discussions on whether a rollover protection system (ROPS) should be considered a Non-reversible occupant restraint system. For this topic, a new definition is added for a ROPS. This definition is a combination of the purpose of a ROPS (occupant protection during a rollover) and the activation (taken from definition 2.27). To add the activation of a ROPS as a condition for triggering recording of data, it is simply included in 5.3.1.3.