Working document for the IWG on EDR/DSSAD / SG-EDR. 	This document serves as	a place holder for proposals under 	consideration as part of the ‘EDR Step 2’ workstream.

Proposal by the Informal Working Group on EDR/DSSAD amending the 01 Series of Amendments to UN Regulation No. 160 introducing new requirements in line with the ‘Step 2’ workstream (Programme of Work, GRSG, doc. WP.29-194-30).

The modifications to the text of document ECE/TRANS/WP.29/2020/123/Rev.1 as last amended by ECE/TRANS/WP.29/2021/58 are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Insert new paragraph 2.x.:

“[2.x.	“Rollover protection system” means any protection system that protects occupants during a rollover, which is activated by a vehicle's crash sensing system;]”

Paragraph 5.3.1.3., amend to read:

"5.3.1.3.	Activation of [rollover protection system or n]Non-reversible occupant restraint system.”

Paragraph 5.4., amend to read:

"5.4.	Crash test performance and survivability

5.4.1.	Each vehicle subject to the requirements of national or regional frontal crash test regulations, shall conform with the specifications in paragraph 5.4.3.

5.4.2.	Each vehicle subject to the requirements of national or regional side impact crash test regulations shall conform with the specifications of paragraph 5.4.3.

5.4.3.	The data elements required by paragraph 5.1, shall be recorded in the format specified by paragraph 5.2, exist at the completion of the crash test and the complete data recorded element shall read "yes" after the test. Elements that are not operating normally in braking, etc.) are not required to meet the accuracy or resolution requirements in these crash tests.
The data shall be retrievable even after an impact of a severity level set by UN Regulations Nos. [94, 95 or 137 95 and either 94 or 137].”

Paragraph x.x, amend to read:

“x.x.	…”

II.	Justification

- SG-EDR-42-08:

Paragraphs 2.x. and 5.3.1.3.: There have been discussions on whether a rollover protection system (ROPS) should be considered a Non-reversible occupant restraint system. For this topic, a new definition is added for a ROPS. This definition is a combination of the purpose of a ROPS (occupant protection during a rollover) and the activation (taken from definition 2.27). To add the activation of a ROPS as a condition for triggering recording of data, it is simply included in 5.3.1.3.

- SG-EDR-42-07:

Paragraph 5.4.: Different views had been addressed during type approval activities on which collision tests are required to be conducted for evidencing the crash test performance and survivability:

- a side AND a full width frontal AND an offset front collision, or

- a side OR a full width frontal OR an offset front collision, or

- a side AND a frontal collision, being either full width OR offset.

From 5.4.1. and 5.4.2. it follows that c) is the correct interpretation. The proposed amendment aims clarifying this.

- …

_________________