Proposal for a new series of amendments to UN Regulation No. 156 and revision to Consolidated Resolution on the Construction of Vehicles (R.E.3)

I.	Proposal for new series of amendments to UN Regulation No. 156

Paragraphs 7.2.1.2. and 7.2.1.2.3., amend to read:

“7.2.1.2.	Where a vehicle type uses RXSWIN Requirements for RXSWIN:

7.2.1.2.1.	The vehicle type shall have a dedicated RXSWIN for every Regulation to which the vehicle type has been approved, provided that the respective system has the capability to execute software updates and the vehicle manufacturer has declared the intention to  update it at the time of type approval to this Regulation.

7.2.1.2.2.	Where there is a subsequent software update that is relevant to a Regulation to which the vehicle has been approved without a dedicated RXSWIN, an RXSWIN shall be provided via an extension of the relevant type approval. The extension shall be obtained before the software update is executed.

7.2.1.2.3.	Each RXSWIN shall be uniquely identifiable. When type approval relevant software is modified by the vehicle manufacturer, the RXSWIN shall be updated if it leads to a type approval extension or to a new type approval.

7.2.1.2.2.4.	Each RXSWIN shall be easily readable in a standardized way via the use of an electronic communication interface, at least by the standard interface (OBD port).

If RXSWINs are not held on the vehicle, the manufacturer shall declare the software version(s) of the vehicle or single ECUs with the connection to the relevant type approvals to the Approval Authority. This declaration shall be updated each time the declared software version(s) is updated. In this case, the software version(s) shall be easily readable in a standardized way via the use of an electronic communication interface, at least by the standard interface (OBD port).

7.2.1.2.3.5.	The vehicle manufacturer shall protect the RXSWINs and/or software version(s) on a vehicle against unauthorised modification. At the time of Type Approval, the means implemented to protect against unauthorized modification of the RXSWIN and/or software version(s) chosen by the vehicle manufacturer shall be confidentially provided.

”

Insert new Paragraph 13., to read:

"13. 	Transitional provisions

13.1.	As from the official date of entry into force of the 01 series of amendments, no Contracting Party applying this Regulation shall refuse to grant or refuse to accept type approvals under this Regulation as amended by the 01 series of amendments.

13.2. 	As from [1 September 2028 ], Contracting Parties applying this Regulation shall not be obliged to accept type approvals issued to the original version of this Regulation, first issued after [1 September 2028].

13.3.	Until [1 September 2030], Contracting Parties applying this Regulation shall accept type approvals issued to the original version of this Regulation, first issued before [1 September 2028].

13.4.	As from [1 September 2030], Contracting Parties applying this Regulation shall not be obliged to accept type approvals issued to the original version of this Regulation.

13.5.	Notwithstanding paragraphs 13.2. and 13.4., Contracting Parties applying this Regulation shall continue to accept type approvals issued to the original version of this Regulation, for vehicles which are not affected by the changes introduced by the 01 series of amendments (i.e. vehicle types that already use RXSWIN).

13.6.	Contracting Parties applying this Regulation shall continue to grant extensions of existing approvals issued to the original version of this Regulation.

”

Annex 1., amend to read:

"…

9. 	Software Updates

9.1. 	General construction characteristics of the vehicle type:

9.2. 	The number of the Certificate of Compliance for Software Update Management System:

9.3.	Security measures.

9.3.1.	Documents for the vehicle type to be approved describing that the update process will be performed securely

9.3.2.	Documents for the vehicle type to be approved describing that the RXSWINs on a vehicle are protected against unauthorized manipulation

9.4.	Software updates over the air

9.4.1.	Documents for the vehicle type to be approved describing that the update process will be performed safely

9.4.2.	How a vehicle user is able to be informed about an update before and after its execution.

9.5.	Identification and accessibility of RXSWIN

9.5.1.	Information on how to read any RXSWIN, or the relevant software version(s) in the case where the RXSWIN is not held on the vehicle:

9.5.2.	Description on how to access the information from the auditable register of all software versions relevant to any RXSWIN:

"

Annex 2., amend to read:

"…

15. 	Signature:

16. 	The index to the information package lodged with the Approval Authority, which may be obtained on request is attached.

17.	Information on how to read any RXSWIN, or the relevant software version(s) in the case where the RXSWIN is not held on the vehicle:

18.	Description on how to access the information from the auditable register of all software versions relevant to any RXSWIN:

"

II.	Proposal for revision to Consolidated Resolution on the Construction of Vehicles (R.E.3)

Annex 7., amend to read:

"

Annex 7

Provisions on Software Identification Numbers

I1.		Introduction

UN Regulation No. 156 [15…] on uniform provisions concerning the approval of vehicles with regards to software update and software updates management system is defining "RX Software Identification Number (RXSWIN)" that means a dedicated identifier, defined by the vehicle manufacturer, representing information about the type approval relevant software of the Electronic Control System contributing to the UN Regulation No. X type approval relevant characteristics of the vehicle.

In order to make use of RXSWIN, relevant UN Regulations can shall refer, by incorporation, to and incorporate this annex, as necessary, to introduce relevant definitions and provisions. as follow:

II2.		Definitions

For the purpose of this Consolidated Resolution and the UN Regulations referring to this annex:

2.1.	"RxX Software Identification Number (RXSWIN)" means a dedicated identifier, defined by the vehicle manufacturer, representing information about the type approval relevant software of the Electronic Control System contributing to the UN Regulation No. X type approval relevant characteristics of the vehicle.

2.32.	"Software" means the part of an Electronic Control System that consists of digital data and instructions.

2.3 	"Software update" means a package used to upgrade software to a new version including a change of the configuration parameters.

2.24. 	"Electronic Control System" means a combination of units, designed to co-operate in the production of the stated vehicle control function by electronic data processing. Such systems, often controlled by software, are built from discrete functional components such as sensors, electronic control units and actuators and connected by transmission links. They may include mechanical, electro-pneumatic or electro-hydraulic elements. "The System", referred to herein, is the one for which type approval is being sought.

3.		Necessary amendments to the relevant UN Regulations

Note: The following paragraphs shall be integrated into or amended in the related Regulation referring to this annex.

In the paragraph titled “Definitions” the following sub-paragraph is inserted to read:

“x.x.		For the definitions with regard to Software Identification Number, refer to the Consolidated Resolution on the Construction of Vehicles (R.E.3), Annex 7, paragraph 2.”

In the paragraph titled "Product definitively discontinued” the following sub-paragraph is inserted to read:

“x.x.	The production is not considered definitively discontinued if the vehicle manufacturer intends to obtain further approvals for software updates for vehicles already registered in the market.”

III.		Requirements for software identification

For the purpose of this Consolidated Resolution and the UN Regulations referring to this annex:

3.1.	For the purpose of ensuring the software of the System can be identified, an RXSWIN may be implemented by the vehicle manufacturer.

3.2.	If the manufacturer implements an RXSWIN, the following shall apply:

3.2.1.	The vehicle manufacturer shall have a valid approval according to UN Regulation No. XXX [Software Update Process Regulation].

3.2.2.	The vehicle manufacturer shall provide the following information to the Approval Authority in the communication form of this Regulation (the Regulation referring to this annex):

(a)	The RXSWIN;

(b)	How to read the RXSWIN or software version(s) in case the RXSWIN is not held on the vehicle.

3.2.3.	The vehicle manufacturer may provide, in the communication form of the related Regulation a list of the relevant parameters that will allow the identification of those vehicles that can be updated with the software represented by the RXSWIN. The information provided shall be declared by the vehicle manufacturer and may not be verified by an Approval Authority.

3.2.4.	The vehicle manufacturer may obtain a new vehicle approval for the purpose of differentiating software versions intended to be used on vehicles already registered in the market from the software versions that are used on new vehicles. This may cover the situations where type approval regulations are updated or hardware changes are made to vehicles in series production. In agreement with the testing agency duplication of tests shall be avoided where possible.

IV.		Production definitely discontinued and RxSWIN

4.1.	If the holder of the approval completely ceases to manufacture a type of vehicle approved in accordance with this Regulation (the related Regulation referring to this annex), he shall so inform the authority which granted the approval. Upon receiving the relevant communication that authority shall inform thereof the other Contracting Parties to the 1958 Agreement applying this Regulation (the related Regulation referring to this annex) by means of a communication form conforming to the model in Annex [Communication form] to this the related Regulation.

4.2.	The production is not considered definitely discontinued if the vehicle manufacturer intends to obtain further approvals for software updates for vehicles already registered in the market.

V.	Necessary insertion in the Communication Form relevant to RxSWIN

Note: The communication form of the related Regulation referring to this annex shall include the mention Production definitively discontinued for such a case and shall include additional information regarding RXSWIN as follow (and marked in bold):

Note: In the annex titled “Communication” the communication form shall include the additional information regarding RXSWIN as follows:		
Communication

(Maximum format: A4 (210 x 297 mm))

issued by:		Name of administration:

......................................

......................................

......................................

1

Concerning:	 Approval granted

Approval extended

Approval withdrawn with effect from dd/mm/yyyy

Approval refused

Production definitively discontinued

of a vehicle type, pursuant to UN Regulation No. [this Regulation]

Approval No.:

Extension No.:

Reason for extension:

(…)

(…)

This vehicle type has the capability to execute software updates and is intended to be updated by the manufacturer: Yes/No

Software version(s) (if applicable and no RXSWIN is used):

Additional information regarding RXSWIN:

R[number of this Regulation]SWIN:

Is the R[number of this Regulation]SWIN held on the vehicle: Yes/No

Information on how to read the RX[number of this Regulation]SWIN or software version(s) in case the RX[number of this Regulation]SWIN is not held on the vehicle:

If applicable, list the relevant parameters that will allow the identification of those vehicles that can be updated with the software represented by the RX[number of this Regulation]SWIN under the item above:

"

III.	Justification

.

Distinguishing number of the country which has granted/extended/refused/withdrawn approval (see approval provisions in the Regulation).

Strike out what does not apply.