Proposal for a new series of amendments to UN Regulation No. 156 and revision to Consolidated Resolution on the Construction of Vehicles (R.E.3)

In this document, blue bold font indicates proposed amendments to R156 or to RE3 that have already been discussed during previous meetings of the IWG on CS/OTA. Red bold font indicates changes introduced by this new proposal.

Track changes identifies comments and changes made in session during the 33rd session of the CS/OTA IWG

I.	Proposal for new series of amendments to UN Regulation No. 156

Insert new Paragraph 7.1.1.3., to read:

“7.1.1.3.	A process whereby the vehicle manufacturer assigns a dedicated RXSWIN for every Regulation No. X type approval to which the vehicle manufacturer executes software updates.”

[Renumber former 7.1.1.3. and subsequent paragraphs]

Insert new Paragraph 7.1.4. OR 7.2.1.2.1., to read:

[7.1.4.	The vehicle manufacturer shall not execute software updates to vehicles already registered in the market, unless an RXSWIN has been assigned to every Regulation No. X type approval considered during the performing of the process in 7.1.1.8. .]

[If added as 7.1.4., Renumber former 7.1.5. and subsequent paragraphs]

[If added as 7.2.1.2.1., Renumber former 7.2.1.2.1. and subsequent paragraphs]

Paragraph 7.2.1.2., amend to read:

“7.2.1.2.	Where a vehicle type uses RXSWIN Requirements for RXSWIN:

Insert new Paragraph 13., to read:

"13. 	Transitional provisions

13.1.	As from the official date of entry into force of the 01 series of amendments, no Contracting Party applying this Regulation shall refuse to grant or refuse to accept type approvals under this Regulation as amended by the 01 series of amendments.

13.2. 	As from 1 September 2028, Contracting Parties applying this Regulation shall not be obliged to accept type approvals issued to the original version of this Regulation, first issued after 1 September 2028.

13.3.	Until 1 September 2030, Contracting Parties applying this Regulation shall accept type approvals issued to the original version of this Regulation, first issued before 1 September 2028.

13.4.	As from 1 September 2030, Contracting Parties applying this Regulation shall not be obliged to accept type approvals issued to the original version of this Regulation.

13.5.	Notwithstanding paragraphs 13.2. and 13.4., Contracting Parties applying this Regulation shall continue to accept type approvals issued to the original version of this Regulation, for vehicles which are not affected by the changes introduced by the 01 series of amendments (i.e., vehicle types that already use RXSWIN).

13.6.	Contracting Parties applying this Regulation shall continue to grant extensions of existing approvals issued to the original version of this Regulation.

”

Annex 1., amend to read:

"…

9. 	Software Updates

9.1. 	General construction characteristics of the vehicle type:

9.2. 	The number of the Certificate of Compliance for Software Update Management System:

9.3.	Security measures.

9.3.1.	Documents for the vehicle type to be approved describing that the update process will be performed securely

9.3.2.	Documents for the vehicle type to be approved describing that the RXSWINs on a vehicle are protected against unauthorized manipulation

9.4.	Software updates over the air

9.4.1.	Documents for the vehicle type to be approved describing that the update process will be performed safely

9.4.2.	How a vehicle user is able to be informed about an update before and after its execution.

9.5.	Identification and accessibility of RXSWIN

9.5.1.	Information on how to read any RXSWIN, or the relevant software version(s) in the case where the RXSWIN is not held on the vehicle:

9.5.2.	Description of how to access the information from the auditable register of all software versions relevant to any RXSWIN:

"

Annex 2., amend to read:

"…

9.	Information on how to read any RXSWIN, or the relevant software version(s) in the case where the RXSWIN is not held on the vehicle:

10.	Description on how to access the information from the auditable register of all software versions relevant to any RXSWIN:

[Renumber former 9. and subsequent paragraphs]

"

II.	Proposal for revision to Consolidated Resolution on the Construction of Vehicles (R.E.3)

Annex 7., amend to read:

"

Annex 7

Provisions on Software Identification Numbers

1.		Introduction

UN Regulation No. 156 [15…] on uniform provisions concerning the approval of vehicles with regards to software update and software updates management system is defining "RX Software Identification Number (RXSWIN)" that means a dedicated identifier, defined by the vehicle manufacturer, representing information about the type approval relevant software of the Electronic Control System contributing to the UN Regulation No. X type approval relevant characteristics of the vehicle.

In order to make use of RXSWIN, relevant UN Regulations can shall refer, by incorporation, to and incorporate this annex, as necessary, to introduce relevant definitions and provisions. as follow:

2.		Definitions

For the purpose of this Consolidated Resolution and the UN Regulations referring to this annex:

2.1.	"RxX Software Identification Number (RXSWIN)" means a dedicated identifier, defined by the vehicle manufacturer, representing information about the type approval relevant software of the Electronic Control System contributing to the UN Regulation No. X type approval relevant characteristics of the vehicle.

2.32.	"Software" means the part of an Electronic Control System that consists of digital data and instructions.

2.3 	"Software update" means a package used to upgrade software to a new version, including a change of the configuration parameters.

2.24. 	"Electronic Control System" means a combination of units, designed to co-operate in the production of the stated vehicle control function by electronic data processing. Such systems, often controlled by software, are built from discrete functional components such as sensors, electronic control units and actuators and connected by transmission links. They may include mechanical, electro-pneumatic or electro-hydraulic elements. "The System", referred to herein, is the one for which type approval is being sought.

3.		Necessary amendments to the relevant UN Regulations

Note: The following paragraphs shall be integrated into or amended in the related Regulation referring to this annex.

In the paragraph titled “Definitions” the following sub-paragraph is inserted to read:

“x.x.		For the definitions with regard to Software Identification Number, refer to the Consolidated Resolution on the Construction of Vehicles (R.E.3), Annex 7, paragraph 2.”

In the paragraph titled “Modification and extension of the vehicle type” the following sub-paragraphs are inserted to read:

“x.x.	The vehicle manufacturer may apply for a new vehicle approval for the purpose of differentiating software versions intended to be used on vehicles already registered in the market from the software versions intended to be used on new vehicles. This may cover the situations where type approval regulations are updated or hardware changes are made to vehicles in series production. In agreement with the Type Approval Authority or its Technical Service, duplication of tests for these approvals shall be avoided where possible.”

In the paragraph titled "Production definitively discontinued” the following sub-paragraph is inserted to read:

“x.x.	The production is not considered definitively discontinued if the vehicle manufacturer intends to obtain subsequent extensions of the approval for software updates of for vehicles already registered in the market.”

In the annex titled “Communication” the communication form shall be amended as follows:		
Communication

(Maximum format: A4 (210 x 297 mm))

issued by:		Name of administration:

......................................

......................................

......................................

1

Concerning:  Approval granted

Approval extended

Approval withdrawn with effect from dd/mm/yyyy

Approval refused

Production definitively discontinued

of a vehicle type, pursuant to UN Regulation No. [this Regulation]

Approval No.:

Extension No.:

Reason for extension:

(…)

(…)

Additional information regarding software identification and software updates (where applicable):

R[number of this Regulation]SWIN:

The R[number of this Regulation]SWIN is held on the vehicle2: yes/no

Information on how to read the RX[number of this Regulation]SWIN, or software version(s) in case the RX[number of this Regulation]SWIN is not held on the vehicle:

List of the relevant parameters that will allow the identification of those vehicles that can be updated with the software represented by the RX[number of this Regulation]SWIN under the item above:

"

III.		Justification

Distinguishing number of the country which has granted/extended/refused/withdrawn approval (see approval provisions in the Regulation).

Strike out what does not apply.