|  | E/ECE/324/Rev.2/Add.140/Amend.1−E/ECE/TRANS/505/Rev.2/Add.140/Amend.1 |  |
| --- | --- | --- |
|  |  | 17 December 2021 |

Agreement

Concerning the Adoption of Harmonized Technical United Nations Regulations for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these United Nations Regulations*

(Revision 3, including the amendments which entered into force on 14 September 2017)

_________

Addendum 140 – UN Regulation No. 141

Amendment 1

01 series of amendments – Date of entry into force: 30 September 2021

Revision 1 – Amendment 1

Supplement 1 to the 01 series of amendments – Date of entry into force: 8 October 2022

Revision 1 – Amendment 2

Supplement 2 to the 01 series of amendments – Date of entry into force: 4 January 2023; ECE/TRANS/WP.29/2022/87

Uniform provisions concerning the approval of vehicles with regard to their Tyre Pressure Monitoring Systems (TPMS)

This document is meant purely as documentation tool. The authentic and legal binding texts are: ECE/TRANS/WP.29/2021/10 and Rev.1.

_________

UNITED NATIONS

Uniform provisions concerning the approval of vehicles with regard to their Tyre Pressure Monitoring Systems (TPMS)

Contents

Page

1.	Scope		3

2.	Definitions		3

3.	Application for approval		4

4.	Approval			5

5.	Specifications and tests		5

6.	Supplementary information		9

7.	Modifications and extension of approval of the vehicle type		9

8.	Conformity of production		10

9.	Penalties for non-conformity of production		10

10.	Production definitively discontinued		10

11.	Names and addresses of Technical Services responsible for conducting 
		approval tests, and of Type Approval Authorities		10

12.	Transitional provisions		10

Annexes

1	Communication		12

2	Arrangements of approval marks		14

3	Test requirements for Tyre Pressure Monitoring Systems (TPMS)		15

4	Test requirements for Tyre Pressure Refilling Systems (TPRS) and Central Tyre Inflation systems

(CTIS)…………………….		20

5	Compatibility between towing vehicles and towed vehicles with respect to ISO 11992 data communication…………………….		22

6	Test procedure to assess the functional compatibility of vehicles equipped with ISO 11992 communication interfaces		28

R141r1am1E

7	Performance testing of Tyre Pressure Monitoring System/Tyre Pressure Refill System/

Central Tyre Inflation System

8	Alternative procedure for type approval of subject trailer(s)

9 	Provisions for vehicles equipped with an ADS ……………………………………………………

1.		Scope

This Regulation applies to the approval of vehicles of category: M1 up to a maximum mass of 3,500 kg, M2, M3, N1, N2, N3, O3 and O4, when equipped with a tyre pressure monitoring system.

2.		Definitions

For the purposes of this Regulation:

2.1.	"Approval of a vehicle" means the approval of a vehicle type with regard to its tyre pressure monitoring system.

2.2.		"Vehicle type" means vehicles which do not differ significantly in such essential aspects as:

(a)	The manufacturer’s trade name or mark;

(b)	Vehicle features which significantly influence the performances of the tyre pressure monitoring system;

(c)	The design of the tyre pressure monitoring system.

2.3.		"Wheel" means a complete wheel consisting of a rim and a wheel disc;

2.4	"Twin wheel" means the fitment of a pair of wheels on one side of an axle, on the same hub;

2.5.		"Tyre" means a pneumatic tyre, being a reinforced flexible envelope that is provided with, or forms in conjunction with the wheel on which it is mounted, a continuous, essentially toroidal, closed chamber containing a gas (usually air) or a gas and liquid, that is intended normally to be used at a pressure greater than atmospheric pressure;

Tyres shall be classified as follows:

(a)	Class C1 tyres — Tyres conforming to UN Regulation No. 30;

(b)	Class C2 tyres — Tyres conforming to UN Regulation No. 54 and identified by a load capacity index in single formation lower or equal to 121 and a speed category symbol higher or equal to ‘N’;

(c)	Class C3 tyres: Tyres conforming to UN Regulation No. 54 and identified by:

(i)	A load capacity index in single formation higher or equal to 122; or

(ii)	A load capacity index in single formation lower or equal to 121 and a speed category symbol lower or equal to ‘M’.

2.6.		"Maximum mass" means the maximum value of the vehicle stated by the manufacturer to be technically permissible (this mass may be higher than the "permissible maximum mass" laid down by the national administration);

2.7.	"Maximum axle load" means the maximum value, as indicated by the manufacturer, of the total vertical force between the contact surfaces of the tyres or tracks of one axle and the ground and resulting from the part of the vehicle mass supported by that axle; this load may be higher than the "authorized axle load" laid down by the national administration. The sum of the axle loads may be greater than the value corresponding to the total mass of the vehicle;

2.8.		"Tyre Pressure Monitoring System (TPMS)" means a system fitted on a vehicle which can evaluate the pressure of the tyres or the variation of pressure over time and transmit corresponding information to the user while the vehicle is running;

2.9.	"Cold tyre inflation pressure" means the tyre pressure at ambient temperature, in the absence of any pressure build-up due to tyre usage;

2.10.	"Recommended cold inflation pressure (Prec)" means the pressure recommended for each tyre position by the vehicle manufacturer, for the intended service conditions (e.g. speed and load) of the given vehicle, as defined on the vehicle placard and/or the vehicle owner's manual;

2.11.	"In service operating pressure (Pwarm)" means the inflation pressure for each tyre position elevated from the cold pressure (Prec) by temperature effects during vehicle usage;

2.12.	"Test Pressure (Ptest)" means the actual pressure of the tyre(s) selected for each tyre position after deflation during the test procedure.

2.13.	"Cumulative driving time" means the total time elapsed

where the vehicle of category M1 up to a maximum mass of 3,500 kg or N1 is driven at speeds equal to or higher than 40 km/h and further deducted by 120 seconds for each event where the vehicle speed drops below 40 km/h

or

where the vehicle of other categories than M1 up to a maximum mass of 3,500 kg and N1 is driven at speeds equal to or higher than 30 km/h and further deducted by 120 seconds for each event where the vehicle speed drops below 30 km/h.

2.14.	"Tyre Pressure Refill System (TPRS)" means a system fitted on a vehicle which refills underinflated tyres fitted to an axle of the vehicle with air pressure from a vehicle mounted reservoir (infrastructure) while the vehicle is running but not limited to.

2.15.	"Central Tyre Inflation System (CTIS)" means a system fitted on a vehicle which controls the air pressure in each tyre fitted to an axle of the vehicle with air pressure from a vehicle mounted reservoir (infrastructure) while the vehicle is running but not limited to.

### 2.16. 	“Automated Driving System (ADS)” means the vehicle hardware and software that are collectively capable of performing the entire Dynamic Driving Task (DDT) on a sustained basis.

### 2.17.	“Dynamic Driving Task (DDT)” means the real-time operational and tactical functions required to operate the vehicle.

3.		Application for approval

3.1.		The application for approval of a vehicle type with regard to its Tyre Pressure Monitoring System (TPMS) shall be submitted by the vehicle manufacturer or by his duly accredited representative;

3.2.		It shall be accompanied, in triplicate, by a description of the vehicle type with 	regard to the items specified in Annex 1 to this Regulation:

3.3.		A vehicle representative of the vehicle type to be approved shall be submitted 	to the Type Approval Authority or the Technical Service responsible for 	conducting the approval tests.

3.4.		The Type Approval Authority shall verify the existence of satisfactory 	arrangements for ensuring effective control of the conformity of production before type approval is granted.

4.		Approval

4.1.	If the vehicle submitted for approval pursuant to this Regulation meets all the requirements of paragraph 5. below, approval of that vehicle type shall be granted.

4.2.		An approval number shall be assigned to each type approved. Its first two digits (at present 01 for the Regulation as amended by the 01 series of amendments) shall indicate the series of amendments incorporating the most recent major technical amendments made to the regulation at the time of issue of the approval. The same Contracting Party may not assign the same number to another type of vehicle.

4.3.		Notice of approval or of extension or of refusal of approval of a vehicle type pursuant to this Regulation shall be communicated to the Contracting Parties to the Agreement which apply this Regulation by means of a form conforming to the model in Annex 1 to this Regulation.

4.4.	There shall be affixed, conspicuously and in a readily accessible place specified on the approval form, to every vehicle conforming to a vehicle type approved under this Regulation an international approval mark consisting of:

4.4.1.	A circle surrounding the letter "E" followed by the distinguishing number of the country which granted approval;

4.4.2.	The number of this Regulation, followed by the letter "R", a dash and the approval number to the right of the markings prescribed in paragraph 4.4.1.

4.5.	If the vehicle conforms to a vehicle type approved, under one or more regulations annexed to the Agreement, in the country which granted approval under this Regulation, the symbol prescribed in paragraph 4.4.1. need not be repeated; in such a case, the regulation and approval numbers and the additional symbols for all the regulations under which approval has been granted in the country which granted approval under this Regulation shall be placed in vertical columns to the right of the symbol prescribed in paragraph 4.4.1.

4.6.	The approval mark shall be clearly legible and be indelible.

4.7.	The approval mark shall be placed close to or on the vehicle data plate affixed by the manufacturer.

4.8.	Annex 2 to this Regulation gives examples of approval marks.

5.		Specifications and tests

5.1.		General

5.1.1.	Any vehicle of categories M1 up to a maximum mass of 3,500 kg, M2, M3, N1, N2, N3, O3 and O4, in all cases fitted with a Tyre Pressure Monitoring System (TPMS) complying with the definition of paragraph 2.8, shall meet the performance requirements contained in paragraphs 5.1.2. to 5.6. of this Regulation over a wide range of road and environmental conditions encountered within the territory of the Contracting Parties. Vehicles equipped with an ADS shall fulfill the specific provisions laid down in Annex 9.

5.1.1.1.	A Tyre Pressure Refill System (TPRS) or a Central Tyre Inflation System (CTIS) shall be deemed to be equivalent for Type Approval to a Tyre Pressure Monitoring System (TPMS) when it fulfils the requirements of paragraph 5.1.2., 5.1.3. and 5.4. to 5.6. and the test criteria of Annex 4 to this Regulation. In this case TPMS is not required to be installed.

5.1.1.2.	If more than one system as defined in paragraphs 2.8., 2.14. or 2.15. has been installed, the system(s) which communicate(s) warning messages to the driver shall be approved according to the requirements of this Regulation.

If more than one system is installed on the vehicle it has to be ensured that no contradictory information is displayed to the driver, e.g. by prioritization

5.1.2.	The effectiveness of the TPMS, TPRS or CTIS fitted on a vehicle shall not be adversely affected by magnetic or electrical fields. This shall be demonstrated by fulfilling the technical requirements and respecting the transitional provisions of UN Regulation No. 10 by applying:

(a)	The 03 series of amendments for vehicles without a coupling system for charging the Rechargeable Electric Energy Storage System (traction batteries);

(b)	The 06 series of amendments for vehicles with a coupling system for charging the Rechargeable Electric Energy Storage System (traction batteries).

5.1.3.	For vehicles of category M1 up to a maximum mass of 3,500 kg and N1, the system shall operate from a speed of 40 km/h or below, up to the vehicle's maximum design speed.

For vehicles of categories M2, M3, N2, N3, O3 and O4, the system shall operate from a speed of 30 km/h or below, up to the vehicle's maximum design speed.

5.1.4.	The vehicle shall fulfil the tests (puncture, diffusion and malfunction) as specified in Annex 3 to this Regulation.

5.1.5.	If a variant of any vehicle submitted for approval is fitted with twin wheels, that variant shall be used for the tests defined in Annex 3 to this Regulation and one of the tyres on a twin wheel (the ‘test tyre’) must be deflated for the puncture test in 2.5 of Annex 3 to this Regulation.

5.1.6.	For vehicles of categories M1 up to a maximum mass of 3500 kg and N1

In case of a given warning and if the TPMS is equipped with a reset function but does not detect a minimum pressure as defined in paragraphs 5.2. and 5.3. after executing the reset function, the reset control shall be designed and/or located inside the vehicle in such a way that the risk of an inadvertent reset by vehicle occupants or cargo is reduced.

For vehicles where the TPMS does not detect if the pressure is above a minimum pressure as defined in paragraphs 5.2. and 5.3. after executing the reset function, the TPMS shall include at least measures avoiding a reset if the vehicle did not become stationary after a pressure warning was issued, and either

(a)	Measures avoiding inadvertent reset control operation (e.g. shortly touching on the reset control or continuous blocking of the reset control by vehicle occupants or cargo), or

(b)	An activation by at least two deliberate actions (e.g. in a menu based system).

The manufacturer shall provide in the vehicle owner's handbook, or by any other communication means in the vehicle, the necessary information.

5.2.	Tyre pressure detection for incident-related pressure loss.

5.2.1.	For vehicles of category M1 up to a maximum mass of 3,500 kg and N1, fitted with tyres of the tyre class C1, the TPMS shall illuminate the warning signal described in paragraph 5.5. within not more than ten minutes of cumulative driving time after the in service operating pressure in one of the vehicle's tyres has been reduced by twenty per cent or it is at a minimum pressure of 150 kPa, whichever is higher.

5.2.2.	For vehicles of category M1 up to a maximum mass of 3,500 kg and N1, fitted with tyres of the tyre class C2, the TPMS shall illuminate the warning signal described in paragraph 5.5. within ten minutes of cumulative driving time after the in service operating pressure in one of the vehicle's tyres has been reduced by 20 per cent or it is at a minimum pressure of 220 kPa, whichever is higher.

5.2.3.	For vehicles of category M2, M3, N2 and N3, fitted with tyres of the tyre class C2 or C3, the TPMS shall illuminate the warning signal described in paragraph 5.5. within not more than ten minutes of cumulative driving time after the in-service operating pressure in one of the vehicle's rolling tyres in contact with the ground has been reduced by 20 per cent.

5.2.4.	For vehicles of category O3 and O4, fitted with tyres of the tyre class C2 or C3, the TPMS shall illuminate the warning signal described in paragraph 5.5. within not more than ten minutes of cumulative driving time after the in service operating pressure in one of the vehicle's rolling tyres in contact with the ground has been reduced by 20 per cent.

5.2.5.	The low tyre pressure warning signal described in paragraph 5.5. shall be illuminated whenever the towed vehicle TPMS provides low tyre pressure warning information via the communication interface described in paragraph 5.6.

5.3.	Detection for a tyre pressure level significantly below the recommended pressure for optimum performance including fuel consumption and safety.

5.3.1.	For vehicles of category M1 up to a maximum mass of 3,500kg and N1, fitted with tyres of the tyre class C1, the TPMS shall illuminate the warning signal described in paragraph 5.5. within not more than sixty minutes of cumulative driving time after the in service operating pressure in any of the vehicle's tyres, has been reduced by twenty per cent or it is at a minimum pressure of 150 kPa, whichever is higher.

5.3.2.	For vehicles of category M1 up to a maximum mass of 3,500kg and N1, fitted with tyres of the tyre class C2, the TPMS shall illuminate the warning signal described in 5.5. within not more than 60 minutes of cumulative driving time after the in-service operating pressure in any of the vehicle's tyres has been reduced by 20 per cent or it is at a minimum pressure of 220 kPa, whichever is higher.

5.3.3.	For vehicles of category M2, M3, N2 and N3, fitted with tyres of the tyre class C2 or C3, the TPMS shall illuminate the warning signal within not more than 60 minutes of cumulative driving time after the in-service operating pressure in any of the vehicle's rolling tyres in contact with the ground has been reduced by 20 per cent.

5.3.4. 	For vehicles of category O3 and O4, fitted with tyres of the tyre class C2 or C3, the TPMS shall transmit an appropriate warning signal described in 5.5 within not more than 60 minutes of cumulative driving time after the in-service operating pressure in any of the vehicle's rolling tyres in contact with the ground has been reduced by 20 per cent.

5.3.5.	The low tyre pressure warning signal described in paragraph 5.5. shall be illuminated whenever the towed vehicle TPMS provides low tyre pressure warning information via the communication interface described in paragraph 5.6.

5.4.	Malfunction detection.

5.4.1.	The TPMS/ TPRS/ CTIS shall illuminate the warning signal described in paragraph 5.5. not more than 10 minutes after the occurrence of a malfunction that affects the generation or transmission of control or response signals in the vehicle's TPMS/ TPRS/ CTIS.

5.4.2.	The malfunction indication warning signal described in paragraph 5.5. shall be illuminated whenever the towed vehicle TPMS/ TPRS/ CTIS provides a malfunction indication via the communication interface described in paragraph 5.6.

5.4.3.	The malfunction indication warning signal described in paragraph 5.5. shall be illuminated whenever no valid TPMS/ TPRS / CTIS information is available from a connected towed vehicle, that is required to have TPMS/ TPRS/ CTIS, via any communication interface described in paragraph 5.6.

5.5.	Warning indication.

5.5.1.	The warning indication shall be by means of an optical warning signal conforming to UN Regulation No. 121.

5.5.2	In the case of a vehicle of category N2 or N3 towing at least one vehicle of category O3 or O4, the optical warning signal referred to in 5.5.1. must indicate whether any warning relates to the individual towing or to the towed vehicle(s).

5.5.3.	The optical warning signal shall be activated when the ignition (start) switch is in the "on" (run) position (bulb check). This requirement does not apply to tell-tales shown in a common space.

5.5.4.	The optical warning signal must be visible even by daylight; the satisfactory condition of the signal must be easily verifiable by the driver from the driver's seat.

5.5.5	For vehicles of category O3 and O4, the optical warning signal referred to in 5.5.1 must be displayed to the driver of the towing vehicle of category N2 or N3, as specified in paragraph 5.5.4.

5.5.6.	The malfunction indication may be the same warning signal as the one used to indicate under-inflation. If the warning signal described in paragraph 5.5.1. is used to indicate both under-inflation and a malfunction of the TPMS, the following shall apply: with the ignition (start) switch in the "on" (run) position the warning signal shall flash to indicate a malfunction. After a short period of time the warning signal shall remain continuously illuminated as long as the malfunction exists and the ignition (start) switch is in the "on" (run) position. The flashing and illumination sequence shall be repeated each time the ignition (start) switch is in the "on" (run) position until the malfunction has been corrected.

5.5.7.	The tell-tale of the warning described in paragraph 5.5.1. may be used in a flashing mode in order to provide information about the reset status of the tyre pressure monitoring system in accordance with the owner's manual of the vehicle.

5.6.		Communications interface between towing and towed vehicles

5.6.1	Vehicles of category N2 or N3 towing at least one vehicle of category O3 or O4 and vehicles of category O3 and O4 shall be equipped with a communication interface to exchange TPMS/ TPRS / CTIS data information between towing and towed vehicles. This may be achieved as a wired or a wireless interface, provided that the TPMS/ TPRS/ CTIS equipment in the towing vehicle and in the towed vehicle(s) are compatible.

5.6.1.1.	The data communication with wired equipment shall be based on the braking electric control line conforming to ISO 11992-1:2019 and ISO 11992-2:2014 and be a point-to-point type using the seven pin connector according to ISO 7638-1:2018 or ISO 7638-2:2018 or an appropriate automated connector.

Other wired specifications may be used, provided that the TPMS/ TPRS/ CTIS equipment in the towing vehicle and in the towed vehicle(s) are compatible and fulfil the same functional requirements.

5.6.1.1.1.	The support of messages is specified within Part A of Annex 5 to this Regulation for the towing vehicle and the towed vehicle(s).

5.6.1.1.2.	The functional compatibility of towing and towed vehicles equipped with data communication lines as described in paragraph 5.6.1.1. above shall be assessed at the time of type approval by checking that the relevant provisions as specified in Part A of Annex 5 are fulfilled.

Annex 6 to this Regulation provides a procedure for tests that may be used to perform this assessment.

5.6.1.2.	In the case of a point-to-point link between a towing vehicle ECU and a towed vehicle ECU, the towed vehicle ECU, which constitutes part of the point-to-point link, shall provide an interface according to an open standard specification to allow the ECU(s) providing TPMS/ TPRS /CTIS functionality, which does (do) not constitute part of the point-to- point link, to connect, communicate and operate via the towed vehicle ECU which constitutes part of the point-to-point link with ECU of the towing vehicle, i.e. standardised gatewaying. This data communication interface is specified in Part B of Annex 5.

5.6.1.3.	In the case of data communication with wireless equipment, the communication link shall be an open standard specification. Provision shall be made to ensure that the wireless link is set up between the physically connected vehicles (as opposed to other vehicles in the vicinity), and that information shared over this link is secure against outside interference. The same functional requirements as required in paragraph 5.6.1.1. shall be fulfilled.

5.7. 		Alternative procedure for a subject trailer

5.7.1. 	In the case that a manufacturer chooses to approve a vehicle type of category O3 or O4 with regard to TPMS, TPRS or CTIS according to Annex 8 “Alternative procedure for type approval of subject trailer(s)” to this Regulation, then information is utilized from test reports issued in accordance with Annex 7, Appendix 3 and / or Appendix 4 to this Regulation.

6.		Supplementary information

6.1.	The owner’s manual, if any, of the vehicle shall contain at least the following information:

6.1.1.	A statement that the vehicle is equipped with such a system (and information how to reset the system, if the actual system includes such a feature).

6.1.2.	An image of the tell-tale symbol described in paragraph 5.5.1. (and an image of the malfunction tell-tale symbol, if a dedicated tell-tale is used for this function).

6.1.3.	Additional information about the significance of the low tyre pressure warning tell-tale illuminating and a description of the corrective action to be undertaken if this happens, including the reset procedure if the actual system includes such a feature.

6.2.	If no owner's manual is supplied with the vehicle, the information required in paragraph 6.1. above shall be displayed in a prominent place on the vehicle.

7.		Modifications and extension of approval of the				vehicle type

7.1.	Every modification of the vehicle type as defined in paragraph 2.2. of this Regulation shall be notified to the Type Approval Authority which approved the vehicle type. The Type Approval Authority may then either:

7.1.1.	Consider that the modifications made do not have an adverse effect on the conditions of the granting of the approval and grant an extension of approval;

7.1.2.	Consider that the modifications made affect the conditions of the granting of the approval and require further tests or additional checks before granting an extension of approval.

7.2.	Confirmation or refusal of approval, specifying the alterations, shall be communicated by the procedure specified in paragraph 4.3. above to the Contracting Parties to the Agreement applying this Regulation.

7.3.	The Type Approval Authority shall inform the other Contracting Parties of the extension by means of the communication form which appears in Annex 1 to this Regulation. It shall assign a serial number to each extension, to be known as the extension number.

8.		Conformity of production

8.1.	The conformity of production procedures shall comply with those set out in Schedule 1 to the Agreement (E/ECE/TRANS/505/Rev.3), with the following requirements:

8.2.	The Type Approval Authority which has granted type approval, may at any time verify the conformity of production in each production facility. The normal frequency of these verifications shall be at least once per year.

9.		Penalties for non-conformity of production

9.1.	The approval granted in respect of a vehicle type pursuant to this Regulation may be withdrawn if the requirements laid down in paragraph 8 are not complied with.

9.2.	If a Contracting Party to the Agreement, which applies this Regulation, withdraws an approval it has previously granted, it shall forthwith so notify the other Contracting Parties applying this Regulation, by means of a copy of the approval form bearing at the end, in large letters, the signed and dated annotation "APPROVAL WITHDRAWN".

10.		Production definitively discontinued

If the holder of the approval completely ceases to manufacture a type of vehicle approved in accordance with this Regulation, he shall so inform the authority which granted the approval. Upon receiving the relevant communication that authority shall inform thereof the other Contracting Parties to the Agreement applying this Regulation by means of a copy of the approval form bearing at the end, in large letters, the signed and dated annotation "PRODUCTION DISCONTINUED".

11.		Names and addresses of Technical Services 	responsible for conducting approval tests, and of 	Type Approval Authorities

The Contracting Parties to the Agreement which apply this Regulation shall communicate to the United Nations Secretariat the names and addresses of the Technical Services responsible for conducting approval tests and of the Type Approval Authorities which grant approval and to which forms certifying approval or extension or refusal or withdrawal of approval, issued in other countries, are to be sent.

12.		Transitional provisions

12.1.	As from the official date of entry into force of the 01 series of amendments, no Contracting Party applying this Regulation shall refuse to grant or refuse to accept type approvals under this Regulation as amended by the 01 series of amendments.

12.2.	As from 6 July 2022, for vehicle types of category N1 without twin wheels fitted on an axle, Contracting Parties applying this Regulation shall not be obliged to accept type approvals to the preceding series of amendments, first issued after 6 July 2022.

12.3.	Until 6 July 2022, for vehicle types of category M1 up to a maximum mass of 3,500 kg and without twin wheels fitted on an axle and until 6 July 2024 for vehicle types of category N1 without twin wheels fitted on an axle, Contracting Parties applying this Regulation shall accept type approvals to the preceding series of amendments, first issued before 6 July 2022.

12.4.	As from 6 July 2022, for vehicle types of category M1 up to a maximum mass of 3,500 kg and without twin wheels fitted on an axle and as from 6 July 2024 for vehicle types of category N1 without twin wheels fitted on an axle, Contracting Parties applying this Regulation shall not be obliged to accept type approvals issued to the preceding series of amendments to this Regulation.

12.5.	Notwithstanding the transitional provisions above, Contracting Parties who start to apply this Regulation after the date of entry into force of the most recent series of amendments are not obliged to accept type approvals which were granted in accordance with any of the preceding series of amendments to this Regulation/are only obliged to accept type approval granted in accordance with the 01 series of amendments.

12.6.	Notwithstanding paragraph 12.4., Contracting Parties applying this Regulation shall continue to accept type approvals issued according to the preceding series of amendments to this Regulation, for the vehicles/vehicle systems which are not affected by the changes introduced by the 01 series of amendments.

12.7.	Contracting Parties applying this Regulation may grant type approvals according to any preceding series of amendments to this Regulation.

12.8.	Contracting Parties applying this Regulation shall continue to grant extensions of existing approvals to any preceding series of amendments to this Regulation.

Annex 1

Communication

(maximum format: A4 (210 x 297 mm))

issued by:		Name of administration:

......................................

......................................

......................................

concerning:2	Approval

Approval extended

Approval refused

Approval withdrawn

Production definitively discontinued

of a vehicle type with regard to its tyre pressure monitoring system pursuant to UN Regulation No. 141.

Approval No.:  ....................						Extension No.:  .................

1.	Trade name or mark of the vehicle:

2	Vehicle type (if applicable, variants that are included):

3.	Manufacturer's name and address:

4.	If applicable, name and address of the manufacturer's representative:

5.	Vehicle submitted for approval on:

6.	Technical Service responsible for conducting approval tests:

7.	Date of test report:

8.	Number of test report:

9.	Brief description of the vehicle type:

9.1.	Mass of the vehicle when tested:

Front axle:

Second axle:

Third axle:

Fourth axle:

Fifth axle:

Sixth axle:

etc.

Total:

9.2.	Tyre Class, Marking and wheel size(s) of standard unit equipment:

9.3.	Brief description of the tyre pressure monitoring system2 / tyre pressure refill system2 / central tyre inflation system2 including implemented measures to avoid inadvertent reset control operation according to paragraph 5.1.6., if applicable

10.	Result of the tests:

10.1.	According to Annex 3 to this Regulation (TPMS), if applicable2:

|  | Measured Time to warning (mm:ss) |
| --- | --- |
| "Puncture test" |  |
| "Diffusion test" |  |
| "Malfunction test" |  |

10.2	According to Annex 4 to this Regulation (TPRS/ CTIS), if applicable2:

|  | Start of refilling Time [s] | Refill Process completed Time [s] | Malfunction warning ON Time [s] |
| --- | --- | --- | --- |
| "Refill functionality" |  |  |  |
| "Malfunction warning" |  |  |  |

11.	Position of approval mark:

12.	Reason(s) of extension (if applicable):

13.	Approval granted/refused/extended/withdrawn2

14.	Place:

15.	Date:

16.	Signature:

17.	The list of documents deposited with the Type Approval Authority which has granted approval is annexed to this communication and can be obtained upon request.

Annex 2

Arrangements of approval marks

(See paragraph 4.4. of this Regulation)

141R - 012439

a = 8 mm min.

The above approval mark affixed to a vehicle shows that the vehicle type concerned has, with regard to the equipment of a tyre pressure monitoring system, been approved in the Netherlands (E 4), pursuant to UN Regulation No. 141 under approval number 012439. The first two digits of the approval number indicates that the approval was granted in accordance with the requirements of UN Regulation No. 141 as amended by the 01 series of amendments.

Annex 3

Tests requirements for Tyre Pressure Monitoring Systems (TPMS)

1.	Test conditions

1.1.	General

In the case that both TPRS and TPMS are fitted to a vehicle, when TPMS is tested according to the tests outlined in this Annex, then TPRS shall be deactivated before commencing tests of TPMS. TPRS shall remain deactivated during tests of TPMS and can be reactivated after TPMS tests have been completed.

In the case that both CTIS and TPMS are fitted to a vehicle, when TPMS is tested according to the tests outlined in this Annex, then CTIS shall be deactivated before commencing tests of TPMS. CTIS shall remain deactivated during tests of TPMS and can be reactivated after TPMS tests have been completed.

1.2.		Ambient temperature.

The ambient temperature shall be between 0 °C and 40 °C.

1.3.		Road test surface.

The road shall have a surface affording good adhesion. The road surface 	shall be dry during testing.

1.4.		The tests shall be conducted in an environment free of interferences from 	radio wave.

1.5.	Vehicle conditions.

1.5.1. 	Test weight.

The vehicle may be tested at any condition of load, the distribution of the mass among the axles being that stated by the vehicle manufacturer without exceeding any of the maximum permissible mass for each axle.

However, in the case where there is no possibility to set or reset the system, the vehicle shall be unladen, but for systems which will automatically raise the lift axle when no load is detected the vehicle shall be laden enough to avoid lifting of those axles. For vehicles of category M1 up to a maximum mass of 3,500 kg, M2, M3, N1, N2, and N3 there may be, in addition to the driver, a second person on the front seat (if fitted) who is responsible for noting the results of the tests.

The load condition shall not be modified during the test.

1.5.2.	Vehicle speed.

The TPMS shall be calibrated and tested for vehicles of category M1 up to a maximum mass of 3,500 kg and N1:

(a)	In a speed range from forty km/h and 120 km/h or the vehicle's maximum design speed if it is less than 120 km/h for the puncture test to verify the requirements of paragraph 5.2. to this Regulation; and

(b)	In a speed range from forty km/h and 100 km/h or the vehicle's maximum design speed if it is less than 100 km/h for the diffusion test to verify the requirements of paragraph 5.3 to this Regulation and for the malfunction test to verify the requirements of paragraph 5.4. to this Regulation.

The TPMS shall be calibrated and tested for vehicles of categories M2, M3, N2, N3, O3 and O4:

(c) 	In a speed range from 30 km/h and 90 km/h (or the vehicle's maximum design speed if it is less than 90 km/h) for the puncture test to verify the requirements of paragraph 5.2. to this Regulation; and

(d)		In a speed range from 30 km/h and 90 km/h (or the vehicle's maximum design speed if it is less than 90 km/h) for the diffusion test to verify the requirements of paragraph 5.3 to this Regulation and for the malfunction test to verify the requirements of paragraph 5.4. to this Regulation.

The whole speed range shall be covered during the test.

For vehicles equipped with cruise control, the cruise control shall not be engaged during testing.

1.5.3.	Rim position.

The vehicle rims may be positioned at any wheel position, consistent with any related instructions or limitations from the vehicle's manufacturer.

1.5.4.	Stationary location.

When the vehicle is parked, the vehicle's tyres shall be shaded from direct sun. The location shall be shielded from any wind that may affect the results.

1.5.5.	Brake pedal application.

Driving time shall not accumulate during service brake application while the vehicle is moving.

1.5.6.	Tyres.

The vehicle shall be tested with the tyres installed on the vehicle according to the vehicle manufacturer's recommendation. However, the spare tyre may be utilised for testing TPMS malfunction.

1.5.7. 	Lift axle(s)

If the vehicle is equipped with lift axle(s), the lift axle(s) shall be fully lowered such that the tyres shall have contact with the ground during testing.

1.6.	Accuracy of pressure measurement equipment.

Pressure measurement equipment to be used for the tests contained in this annex shall be accurate to at least +/-3 kPa.

2.		Test procedure

The test shall be performed at a test speed within the range in accordance with paragraph 1.5.2. to this annex, at least once for the test case according to paragraph 2.6.1. to this annex ("puncture test"), and at least once for each test case according to paragraph 2.6.2. to this annex ("diffusion test").

2.1. 	Before inflating the vehicle's tyres, leave the vehicle stationary outside at ambient temperature with the engine off shaded from direct sunlight and not exposed to wind or other heating or chilling influences for at least one hour for vehicles of category M1 and N1 and at least 4 hours for vehicles of category M2, M3, N2, N3, O3 and O4. Inflate the vehicle's tyres to the vehicle manufacturer's recommended cold inflation pressure (Prec), in accordance with the vehicle manufacturer's recommendation for the speed and load conditions, and tyre positions. All pressure measurements shall be carried out using the same test equipment.

2.2.	With the vehicle stationary and the ignition locking system in the "Lock" or "Off" position, activate the ignition locking system to the "On" or "Run" position. The ECU controlling the tell-tale shall perform a check of lamp function for the low tyre pressure tell-tale as specified in paragraph 5.5.2. of this Regulation. This last requirement does not apply to tell-tales shown in a common space.

2.3.	If applicable, set or reset the tyre pressure monitoring system in accordance with the vehicle manufacturer's recommendations and verify in particular the measures avoiding inadvertent reset control operation according to paragraph 5.1.6.

2.4. 	Learning/ Tyre warming phase.

2.4.1.	For vehicles of category M1 up to a maximum mass of 3,500 kg and N1, drive the vehicle for a minimum of 20 minutes within the speed range in paragraph 1.5.2. to this annex, and with an average speed of 80 km/h (10 km/h). It is allowed to be outside the speed range for a maximum cumulative time of two minutes during this phase.

For vehicles of category M2, M3, N2, N3, O3 and O4, drive the vehicle for a minimum of 120 minutes within the speed range in paragraph 1.5.2. to this Annex, and with an average speed of 60 km/h (10 km/h). It is allowed to be outside the speed range for a maximum cumulative time of two minutes during this phase.

2.4.2.	At the discretion of the Technical Service, where the driving test is undertaken on a track (circle/oval) with only turns in a single direction, then the driving test in paragraph 2.4.1. above should be equally split 
(+/-2 minutes) in both directions.

2.4.3.	Within the five minutes of completing the learning phase, measure the warm pressure of the tyre(s) to be deflated. The warm pressure shall be taken as the value Pwarm. This value will be used for subsequent operations.

2.5.	Deflation phase.

2.5.1.	Procedure for the puncture test to verify the requirements of paragraph 5.2. to this Regulation.

Following the requirements specified in 5.1.5, deflate one of the vehicle's tyres within five minutes of measuring the warm pressure as described in paragraph 2.4.3. above, until it is at Pwarm - 20 per cent, or it is at a minimum pressure:

(a)	Of 150 kPa for vehicles of category M1 up to a maximum mass of 3,500 kg and N1, fitted with tyres of tyre class C1;

or

(b)	Of 220 kPa for vehicles of category M1 up to a maximum mass of 3,500 kg and N1, fitted with tyres of tyre class C2,

whichever is higher, namely Ptest. Following a stabilisation period of between two and five minutes the pressure Ptest shall be rechecked and adjusted if necessary.

2.5.2.	Procedure for the diffusion test to verify the requirements of paragraph 5.3. to this Regulation.

For vehicles of category M1 up to a maximum mass of 3,500 kg and N1, fitted with tyres of tyre class C1 deflate all tyres within five minutes of measuring the warm pressure as described in paragraph 2.4.3. above, until the deflated tyres are at Pwarm - 20 per cent plus a further deflation of 7 kPa, namely Ptest or it is at a minimum pressure of 150 kPa. Following a stabilisation period of between two and five minutes the pressure Ptest shall be rechecked and adjusted if necessary.

For vehicles of category M1 up to a maximum mass of 3,500 kg and N1, fitted with tyres of tyre class C2 deflate all tyres within five minutes of measuring the warm pressure as described in paragraph 2.4.3. above, until the deflated tyres are at Pwarm - 20 per cent plus a further deflation of 7 kPa, namely Ptest or it is at a minimum pressure of 220 kPa. Following a stabilisation period of between two and five minutes the pressure Ptest shall be rechecked and adjusted if necessary.

For vehicles of category M2, M3, N2, N3, O3 and O4 deflate all tyres within 15 minutes of measuring the warm pressure as described in paragraph 2.4.3. above, until the deflated tyres are at Pwarm 
- 20 per cent plus a further deflation of 7 kPa, namely Ptest. Following a stabilisation period of between 5 and 10 minutes the pressure Ptest shall be rechecked and adjusted if necessary.

2.6.	Low tyre pressure detection phase.

2.6.1.	Procedure for the puncture test to verify the requirements of paragraph 5.2. to this Regulation.

2.6.1.1.	Drive the vehicle along any portion of the test course (not necessarily continuously). The sum of the total cumulative drive time shall be the lesser of 10 minutes or the time at which the low tyre pressure tell-tale illuminates.

2.6.2.	Procedure for the diffusion test to verify the requirements of paragraph 5.3. to this Regulation.

2.6.2.1.	Drive the vehicle along any portion of the test course. After not less than twenty (20) minutes and not more than forty (40) minutes bring the vehicle to a complete standstill with the engine switched off and the ignition key removed for not less than one (1) minute or more than three (3) minutes. Resume the test. The sum of the total cumulative drive time shall be the lesser of sixty (60) minutes of cumulative driving under the conditions set out in paragraph 1.5.2. above or the time at which the low tyre pressure tell-tale illuminates.

2.6.3.		If the low tyre pressure signal did not illuminate, discontinue the test.

2.7.	Low pressure tell-tale illumination

2.7.1.	For vehicles of category M1 up to a maximum mass of 3,500 kg and N1

If the low tyre pressure tell-tale illuminated during the procedure in paragraph 2.6. above, deactivate the ignition locking system to the ''Off'' or ''Lock'' position. After a five minutes period, reactivate the vehicle's ignition locking system to the ''On'' (''Run'') position. The tell-tale must illuminate and remain illuminated as long as the ignition locking system is in the "On" ("Run") position.

2.7.2.	For vehicles of category M2, M3, N2, N3, O3 and O4

If the low tyre pressure tell-tale illuminated during the procedure in paragraph 2.6. above, deactivate the ignition locking system to the ''Off'' or ''Lock'' position. After a five minutes period, reactivate the vehicle's ignition locking system to the ''On'' (''Run'') position. The tell-tale must illuminate within ten minutes and remain illuminated as long as the ignition locking system is in the "On" ("Run") position.

2.8.		Inflate all of the vehicle's tyres to the vehicle manufacturer's recommended cold inflation pressure. Reset the system in accordance with the instructions of the vehicle manufacturer. Determine whether the tell-tale has extinguished. If necessary, drive the vehicle until the tell-tale has been extinguished. If the tell-tale does not extinguish, discontinue the test.

2.9.	Repetition of the deflation phase.

The test may be repeated, at the same or different loads, using the relevant test procedures in paragraphs 2.1. to 2.8. above, with the relevant tyre(s) on the vehicle under-inflated, in accordance with the provisions of paragraph 5.2. or 5.3. to this Regulation, whichever is relevant.

3.		TPMS malfunction detection

3.1.	Simulate a TPMS malfunction, for example, by disconnecting the power source to any TPMS component, disconnecting any electrical connection between TPMS components, or installing a tyre or wheel on the vehicle that is incompatible with the TPMS. When simulating a TPMS malfunction, the electrical connections for the tell-tale lamps shall not be disconnected.

3.2.	Drive the vehicle for up to ten minutes of cumulative time (not necessarily continuously) along any portion of the test course.

3.3.	The sum of the total cumulative drive time under paragraph 3.2. shall be the lesser of ten minutes or the time at which the TPMS malfunction tell-tale illuminates.

3.4.	If the TPMS malfunction indicator did not illuminate in accordance with paragraph 5.4. to this Regulation, as required, discontinue the test.

3.5.	For vehicles of category M1 up to a maximum mass of 3,500 kg and N1

If the TPMS malfunction indicator is illuminated or illuminates during the procedure in paragraphs 3.1 to 3.3 above, deactivate the ignition locking system to the ''Off'' or ''Lock'' position. After five minutes, reactivate the vehicle's ignition locking system to the ''On'' (''Run'') position. The TPMS malfunction indicator shall again signal a malfunction and remain illuminated as long as the ignition locking system is in the ''On'' (''Run'') position.

3.6.	For vehicles of category M2, M3, N2, N3, O3 and O4

If the TPMS malfunction indicator is illuminated or illuminates during the procedure in paragraphs 3.1. to 3.3. above, deactivate the ignition locking system to the ''Off'' or ''Lock'' position. After five minutes, reactivate the vehicle's ignition locking system to the ''On'' (''Run'') position. The TPMS malfunction indicator shall again signal a malfunction within ten minutes and remain illuminated as long as the ignition locking system is in the "On" ("Run") position.

3.7.	Restore the TPMS to normal operation. If necessary, drive the vehicle until the warning signal has extinguished. If the warning lamp has not extinguished, discontinue the test.

3.8.	The test may be repeated using the test procedures in paragraphs 3.1. to 3.6. above, with each such test limited to simulation of a single malfunction.

Annex 4

Test requirements for Tyre Pressure Refilling Systems (TPRS) and for Central Tyre Inflation System (CTIS)

1.	Test conditions

1.1.	Ambient temperature.

The ambient temperature shall be between 0 °C and 40 °C.

1.2.	Road test surface

The road shall have a surface affording good adhesion. Testing shall be performed on even ground.

1.3.	Vehicle condition

1.3.1.	Testweight

The vehicle may be tested at any condition of load, the distribution of the mass among the axles being that stated by the vehicle manufacturer without exceeding any of the maximum permissible mass for each axle.

However, in the case where there is no possibility to set or reset the system, the vehicle shall be unladen. For vehicles of category M1 up to a maximum mass of 3,500 kg, M2, M3, N1, N2, and N3 there may be, in addition to the driver, a second person on the front seat (if fitted) who is responsible for noting the results of the tests.

The load condition shall not be modified during the test..

1.3.2.	Driving situation

Tests are performed with the vehicle in standstill.

In case of vehicles of category O3 and O4, electric and pneumatic supply shall be provided.

1.3.3.	Rim position.

The vehicle rims may be positioned at any wheel position, consistent with any related instructions or limitations from the vehicle's manufacturer.

1.3.4	Stationary location

When the vehicle is parked, the vehicle's tyres shall be shaded from direct sun. The location shall be shielded from any wind that may affect the results.

1.4.	Tyres

The vehicle shall be tested with the tyres on the vehicle according to the vehicle manufacturer's recommendation.

1.5.	Accuracy of pressure measurement equipment

Pressure measurement equipment to be used for the tests contained in this Annex shall be accurate to at least +/-3 kPa.

2.	Test procedure

2.1.	If a variant of any vehicle submitted for approval is fitted with twin wheels, that variant shall be used for the test and one of the tyres on a twin wheel (the “test tyre”) must be deflated for the refilling test in paragraph 2.5.

2.2.	Before inflating the vehicle's tyres, leave the vehicle stationary outside at ambient temperature with the engine off shaded from direct sunlight and not exposed to wind or other heating or chilling influences for at least one hour for vehicles of category M1 and N1 and at least 4 hours for vehicles of category M2, M3, N2, N3, O3 and O4. Inflate the vehicle's tyres to the vehicle manufacturer's recommended cold inflation pressure (Prec), in accordance with the vehicle manufacturer's recommendation for the speed and load conditions, and tyre positions. All pressure measurements shall be carried out using the same test equipment.

2.3.		With the vehicle stationary and the ignition locking system in the "Lock" or "Off" position, activate the ignition locking system to the "On" or "Run" position. The TPRS / CTIS shall perform a check of lamp function for the low tyre pressure tell-tale as specified in paragraph 5.5.2. of this Regulation. This last requirement does not apply to tell-tales shown in a common space.

2.4. 	Vehicle conditioning

The pressure reservoir (mounted infrastructure) has to be filled according to UN Regulation No. 13, Series 11, Supplement 16, reservoir pressure limits. The compressed air supply must be granted during the tests according to paragraph 2.5. and to paragraph 2.6. to this Annex.

2.5.	Check the systems refill functionality

Check the systems refill functionality for incident-related pressure loss and for detection of a tyre pressure level significantly below the recommended pressure for optimum performance including fuel consumption and safety.

Inflate the vehicle's tyres to the vehicle manufacturer's recommended cold inflation pressure (Prec).

Deflate the tyre pressure of one tyre by 20% but not more than 50 kPa below the manufacturers recommended cold inflation pressure (Prec). During the deflation the tyre shall not be connected to the pneumatic circuit.

2.5.1. 	Check refilling according to Figure 1

Check that within 2 minutes, when the system is operational, the TPRS/ CTIS starts refilling and at least after 2 minutes the low tyre pressure tell-tale, as described in paragraph 5.5. of the Regulation, is “On”.

Refill process shall be completed within 8 min after the refill process has started and the low tyre pressure tell-tale, as described in paragraph 5.5. of the Regulation, is “Off” as soon as the refilling process is completed.

After the refilling process has been completed, check that the tyre pressure is in a range of +/- 5% of manufacturers recommended cold inflation pressure Prec.

2.5.2.		Check refilling according to Figure 2

Check that within 2 minutes when the system is operational the TPRS/ CTIS starts refilling and at least after 2 minutes the low tyre pressure tell-tale, as described in paragraph 5.5. of the regulation, is “On”.

Refill process shall not be completed within 8 min after the refill process has started and the low tyre pressure tell-tale, as described in paragraph 5.5. of the regulation, is “ON” after at least 2 minutes of refilling time.

The deflation rate during the test must be higher than the refilling rate.

Figure 1

Refilling check

Deflation 20% (max. 50 kPa)

Activation point

Refilling start point

max. 8min

max. 2min

Figure 2

Checking system malfunction warning functionality.

max. 2min

max. 8min

Deflation 20% (max. 50 kPa)

Activation point

Refilling start point

Low tyre pressure tell-tale

2.6.		TPRS/ CTIS malfunction detection

2.6.1.	Simulate a TPRS/ CTIS malfunction, for example, by disconnecting the energy source (electrical power and / or pneumatic pressure) to any TPRS/ CTIS component, disconnecting any electrical connection between TPRS/ CTIS components. When simulating a TPRS/ CTIS malfunction, the electrical connections for the tell-tale lamps shall not be disconnected.

2.6.2.	Restore the TPRS/ CTIS to normal operation. If the warning lamp has not extinguished, discontinue the test.

Annex 5

Compatibility between towing vehicles and towed vehicles with respect to ISO 11992 data communication

A. 	TPMS/ TPRS/ CTIS data communication between the towing vehicle and towed vehicle(s)

1.	General

1.1.	The requirements of Part A of this annex shall only apply to towing vehicles and towed vehicles equipped with a communication interface as described in paragraph 5.6.1.1. of this Regulation.

1.2.	This annex defines requirements applicable to the towing vehicle and towed vehicle with respect to the support of messages defined within 
ISO 11992-2:2014

1.3	Vehicles of subcategories X and Y are assumed not to tow a vehicle of categories O3 or O4.

2.	The parameters defined within ISO 11992-2:2014 that are transmitted by the communication interface shall be supported as follows:

2.1.	The following functions and associated messages are those that shall be supported by the towing vehicle or towed vehicle as appropriate:

2.1.1.	Messages transmitted from the towing vehicle to the towed vehicle, if supported:

| Function / Parameter | ISO 11992-2: 2014 reference |
| --- | --- |
| Reverse gear status | EBS12 Byte 2 Bit 5-6 |
| Braking system wheel-based vehicle speed | EBS12 Byte 7-8 |
| Time/Date – Seconds | SAE J1939 PGN 65254 TD Byte 1 |
| Time/Date – Minutes | SAE J1939 PGN 65254 TD Byte 2 |
| Time/Date – Hours | SAE J1939 PGN 65254 TD Byte 3 |
| Time/Date – Months | SAE J1939 PGN 65254 TD Byte 4 |
| Time/Date – Day | SAE J1939 PGN 65254 TD Byte 5 |
| Time/Date – Year | SAE J1939 PGN 65254 TD Byte 6 |
| Time/Date - Local minute offset | SAE J1939 PGN 65254 TD Byte 7 |
| Time/Date - Local hour offset | SAE J1939 PGN 65254 TD Byte 8 |
| Identification data index | RGE12 Byte 5 |
| Identification data content | RGE12 Byte 6 |

Note: 	Regarding the definition of the parameters of the Time/Date message, there is a known inconsistency between the SAE J1939 and ISO 11992-2:2014 standards. For the purposes of compliance to this Regulation, the Time/Date message definition (PGN 65254) provided in SAE J1939DA 202110 (publication date 21 October 2021) shall be used.”

2.1.2.	Mandatory messages transmitted from the towed vehicle to the towing vehicle:

| Function / Parameter | ISO 11992-2:2014 reference | Reference to paragraphs in this UN Regulation |
| --- | --- | --- |
| Tyre Pressure Status | EBS23 Byte 1 Bit 1-2 | Paragraph 5.2.4. Paragraph 5.3.5. Paragraph 5.4.3. |
| Tyre/wheel identification (pressure) | EBS23 Byte 2 | Paragraph 5.2.4. Paragraph 5.3.5. Paragraph 5.4.3. |

2.1.3.	Messages transmitted from the towed vehicle to the towing vehicle, if supported:

| Function / Parameter | ISO 11992-2:2014 reference |
| --- | --- |
| Tyre/wheel identification (for EBS23 pressure) | EBS23 Byte 2 |
| Tyre pressure | EBS23 Byte 5 |
| Tyre/wheel identification (for RGE23) | RGE23 Byte 1 |
| Tyre temperature | RGE23 Byte 2-3 |
| Air leakage detection | RGE23 Byte 4-5 |
| Tyre pressure threshold detection | RGE23 Byte 6 Bit 1-3 |
| Tyre module power supply status | RGE23 Byte 6 Bit 4-5 |
| Identification data index (1) | RGE23 Byte 7 |
| Identification data content (1) | RGE23 Byte 8 |

(1) Content of the Gateway ECU shall be prioritized

2.1.4.	The towed vehicle ECU transmitting the EBS23 and RGE23 messages shall assemble the EBS23 and RGE23 messages from TPMS/ TPRS/ CTIS content received from the ECU(s) providing TPMS/ TPRS/ CTIS functionality and data from other sources not defined in this Regulation.

Signals, other than Tyre Pressure Status (EBS23 Byte 1 Bit 1-2), within messages EBS23 and RGE23 shall be transmitted with the indication “not available” in case such data is not available.

2.2.	When the towed vehicle transmits the following messages, the towing vehicle shall provide a low tyre pressure warning to the driver:

| Function / Parameter | ISO 11992-2:2014 reference | Driver warning required |
| --- | --- | --- |
| Tyre Pressure Status (For Low Tyre Pressure Warning Indication) | EBS23 Byte 1 Bit 1-2 (002 — tyre pressure insufficient) (1) | References to paragraph 5.2.3., 5.2.4., 5.3.4., 5.3.5. and 5.5.2. in this UN Regulation |
| Tyre/wheel identification (corresponding to Tyre Pressure Status) | EBS23 Byte 2 (XXXXXXXX2 — actual Tyre/Wheel ID) OR (000000002 — Tyre/Wheel ID not defined or wheel not defined and axle > 1510) OR (111111112 — Tyre/Wheel ID not available or wheel = 1510 and axle = 1510) | References to paragraph 5.2.3., 5.2.4., 5.3.4., 5.3.5. and 5.5.2. in this UN Regulation |

(1)  To be noted that within the definition of EBS 23 “Tyre Pressure Status” ISO 11992-2 qualifies that "an insufficient tyre pressure shall be indicated, if the pressure is outside of a pressure range recommended by the tyre or vehicle manufacturer, to ensure an optimized operation with regard to the fuel consumption of the vehicle and life time of the tyre.". Therefore, it should be noted that a value of “002” could signify other tyre pressure conditions such as “over-pressure” which are not covered by this regulation.”

2.3.	When the towed vehicle transmits the following messages, the towing vehicle shall provide a TPMS/ TPRS/ CTIS malfunction indication to the driver:

| Function / Parameter | ISO 11992-2:2014 reference | Driver warning required |
| --- | --- | --- |
| Tyre Pressure Status (For TPMS/ TPRS/ CTIS Malfunction Indication) | EBS23 Byte 1 Bit 1-2 (102 — error indicator) | Reference to paragraph 5.4.1., 5.4.2. and 5.5.2. in this UN Regulation |
| Tyre/wheel identification (corresponding to Tyre Pressure Status) | EBS23 Byte 2 XXXXXXXX2 — actual Tyre/Wheel ID) OR (000000002 — Tyre/Wheel ID not defined or wheel not defined and axle > 1510) OR (111111112 — Tyre/Wheel ID not available or wheel = 1510 and axle = 1510) | Reference to paragraph 5.4.1., 5.4.2. and 5.5.2. in this UN Regulation |

2.3.1.	The towed vehicle shall transmit a Tyre Pressure Status value of "error indicator" within 10 minutes of cumulative driving (in accordance with paragraph 5.4.1. of this Regulation) for any scenario where a valid Tyre Pressure Status (i.e. tyre pressure sufficient or insufficient) cannot be transmitted.

Note that before towed vehicles needed to comply with this Regulation, some of them transmitted Tyre Pressure Status “not available” for some of these scenarios, including when the towed vehicle had no function to perform tyre pressure monitoring. Towed vehicles that are required to comply with this Regulation going forward shall instead transmit "error indicator" for these scenarios.

Note that the towing vehicle would not be required to display a towed vehicle TPMS/ TPRS/ CTIS malfunction indication in the case that valid towed vehicle TPMS information is available on an alternative communication interface.

2.4. 	When a permanent failure is detected in the communication line, the towing vehicle shall illuminate the towed vehicle TPMS/ TPRS/ CTIS malfunction indication signal.

Note that the towing vehicle would not be required to display a towed vehicle TPMS/ TPRS/ CTIS malfunction indication in the case that valid towed vehicle TPMS/ TPRS/ CTIS information is available on an alternative communication interface.

2.5. 	When a valid Tyre Pressure Status is temporarily not available (i.e. unavailable for less than 10 minutes of cumulative drive time), the towed vehicle shall transmit the following messages:

| Function / Parameter | ISO 11992-2:2014 reference | Driver warning required |
| --- | --- | --- |
| Tyre Pressure Status (TPMS/ TPRS/ CTIS data temporarily unavailable) | EBS23 Byte 1 Bit 1-2 (112 — not available) | Not applicable |
| Tyre/wheel identification (corresponding to Tyre Pressure Status) | EBS23 Byte 2 XXXXXXXX2 — actual Tyre/Wheel ID) OR (000000002 — Tyre/Wheel ID not defined or wheel not defined and axle > 1510) OR (111111112 — Tyre/Wheel ID not available or wheel = 1510 and axle = 1510) | Not applicable |

Note:	paragraph 2.3.1. of part A of this Annex specifies required transmitted values when valid Tyre Pressure Status is unavailable for any longer duration.

2.6. 	The support of all other messages defined within ISO 11992-2:2014 is optional for the towing vehicle and towed vehicle, unless required by other Regulations.

B.	Data communication between (i) a towed vehicle ECU constituting part of a point-to-point link with the towing vehicle (towed vehicle gateway ECU) and (ii) a towed vehicle ECU providing TPMS/ TPRS/ CTIS functionality

1.	General

1.1.	The requirements of Part B of this annex shall only apply to towed vehicles with a communication interface as described in paragraph 5.6.1.2. of this Regulation.

1.2.	This annex defines requirements applicable to the towed vehicle gateway ECU and the ECU providing TPMS/ TPRS/ CTIS functionality with respect to the provision of a standard ISO 11898:2015 interface and the support of messages defined within ISO 11992-2:2014.

2. 	The towed vehicle gateway ECU that is part of the point-to-point link shall provide an interface with the ECU providing TPMS/ TPRS/ CTIS functionality complying with data link layer and physical layer in accordance with ISO 11898-1:2015 and ISO 11898-2:2016.

2.1.	The CAN bit-rate for the ISO 11898-1:2015 interface shall be 250 kbit/s.

2.2.	The ISO 11898-2:2015 bus termination shall be configured on the vehicle in accordance with the guidelines of the vehicle manufacturer for the given installation.

2.3.	A power connection shall be made available to the towed vehicle ECU providing TPMS/ TPRS/ CTIS functionality in accordance with the vehicle manufacturer.

2.4.	The towed vehicle gateway ECU shall transmit, towards the towed vehicle ECU providing TPMS/ TPRS/ CTIS functionality, all messages and signals required to realise a reliable TPMS/ TPRS/ CTIS function.

3.	The parameters that are transmitted by the ISO 11898-1:2015 communication interface shall be as defined within ISO 11992-2:2014 and shall be supported as follows:

3.1.	The following functions and associated messages are those that shall be supported by the towed vehicle gateway ECU or towed vehicle ECU providing TPMS/ TPRS/ CTIS functionality as appropriate:

3.1.1.	Messages transmitted, if supported, from the towed vehicle gateway ECU to the towed vehicle ECU providing TPMS/ TPRS/ CTIS functionality:

| Function / Parameter | ISO 11992-2:2014 reference | Reference to paragraphs in this UN Regulation |
| --- | --- | --- |
| Reverse gear status (towing vehicle) | EBS12 Byte 2 Bit 5-6 | Paragraph 5.6.1.2. |
| Braking system wheel-based vehicle speed (towing vehicle) | EBS12 Byte 7-8 | Paragraph 5.6.1.2. |
| Identification data index (towing vehicle) | RGE12 Byte 5 | Paragraph 5.6.1.2. |
| Identification data content (towing vehicle) | RGE12 Byte 6 | Paragraph 5.6.1.2. |
| Time/Date – Seconds (towing vehicle) | TD11 Byte 1 | Paragraph 5.6.1.2. |
| Time/Date – Minutes (towing vehicle) | TD11 Byte 2 | Paragraph 5.6.1.2. |
| Time/Date – Hours (towing vehicle) | TD11 Byte 3 | Paragraph 5.6.1.2. |
| Time/Date – Months (towing vehicle) | TD11 Byte 4 | Paragraph 5.6.1.2. |
| Time/Date – Day (towing vehicle) | TD11 Byte 5 | Paragraph 5.6.1.2. |
| Time/Date – Year (towing vehicle) | TD11 Byte 6 | Paragraph 5.6.1.2. |
| Time/Date - Local minute offset (towing vehicle) | TD11 Byte 7 | Paragraph 5.6.1.2. |
| Time/Date - Local hour offset (towing vehicle) | TD11 Byte 8 | Paragraph 5.6.1.2. |
| Braking system wheel-based vehicle speed (towed vehicle) | EBS21 Byte 3-4 | Paragraph 5.6.1.2. |
| Lift axle 1 position (towed vehicle) | RGE21 Byte 2 Bit 1-2 | Paragraph 5.6.1.2. |
| Lift axle 2 position (towed vehicle) | RGE21 Byte 2 Bit 3-4 | Paragraph 5.6.1.2. |

Note: 	Regarding the definition of the parameters of the Time/Date message, there is a known inconsistency between the SAE J1939 and ISO 11992-2:2014 standards. For the purposes of compliance to this Regulation, the Time/Date message (PGN 65254) definition provided in SAE J1939DA 202110 (publication date 21 October 2021) shall be used.

3.1.2.	Mandatory messages transmitted from the towed vehicle ECU providing TPMS/ TPRS/ CTIS functionality to the towed vehicle gateway ECU:

| Function / Parameter | ISO 11992-2:2014 reference | Reference to paragraphs in this UN Regulation |
| --- | --- | --- |
| Tyre Pressure Status | EBS23 Byte 1 Bit 1-2 | Paragraph 5.6.1.2. |
| Tyre/wheel identification (pressure) | EBS23 Byte 2 | Paragraph 5.6.1.2. |

3.1.3.	Messages transmitted from the towed vehicle ECU providing TPMS/ TPRS/ CTIS functionality to the towed vehicle gateway ECU, if supported:

| Function / Parameter | ISO 11992-2:2014 reference | Reference to paragraphs in this UN Regulation |
| --- | --- | --- |
| Tyre/wheel identification (for EBS23 pressure) | EBS23 Byte 2 | Paragraph 5.6.1.2. |
| Tyre pressure | EBS23 Byte 5 | Paragraph 5.6.1.2. |
| Tyre/wheel identification | RGE23 Byte 1 | Paragraph 5.6.1.2. |
| Tyre temperature | RGE23 Byte 2-3 | Paragraph 5.6.1.2. |
| Air leakage detection | RGE23 Byte 4-5 | Paragraph 5.6.1.2. |
| Tyre pressure threshold detection | RGE23 Byte 6 Bit 1-3 | Paragraph 5.6.1.2. |
| Tyre module power supply status | RGE23 Byte 6 Bit 4-5 | Paragraph 5.6.1.2. |
| Identification data index | RGE23 Byte 7 | Paragraph 5.6.1.2. |
| Identification data content | RGE23 Byte 8 | Paragraph 5.6.1.2. |

3.1.4.	For messages defined in section 3.1. of Part B of this Annex, signals shall be transmitted with the indication "not available" in case the ECU does not provide such data.

3.2. 	The support of all other messages defined within ISO 11992-2:2014 is optional for the towed vehicle gateway ECU and the towed vehicle ECU providing TPMS/ TPRS/ CTIS functionality, unless required by other Regulations.

3.3.	The towed vehicle gateway ECU and the towed vehicle ECU providing TPMS/ TPRS/ CTIS functionality shall support diagnostics as per ISO 11992-4:2014.

4.	The towed vehicle ECU providing TPMS/ TPRS/ CTIS functionality shall use the source address 207 of "Other Trailer Devices" as per SAE J1939-71 standard for forwarding TPMS/TPRS/CTIS information with respect to trailer position in a road train like defined in ISO11992-2.

Annex 6

Test procedure to assess the functional compatibility of vehicles equipped with ISO 11992 data communication interface

1.	General

1.1.	This annex describes a procedure that may be used to check towing and towed vehicles equipped with a communication interface as described in paragraph 5.6.1.1. of this Regulation against the functional requirements referred to in paragraph 5.6.1.1.1. of this Regulation. Alternative procedures may be used at the discretion of the Technical Service if an equivalent level of checking integrity can be established.

1.2.	The references to ISO 7638 within this Annex apply to ISO 7638-1:2018 for 24V applications and ISO 7638-2:2018 for 12V applications.

1.3	Vehicles of subcategories X and Y are assumed not to tow a vehicle of categories O3 or O4.

2.	Towing vehicles

2.1.	ISO 11992 towed vehicle simulator

The simulator shall:

2.1.1.	Have a connector meeting ISO 7638 (7 pin) to connect to the vehicle under test. Pins 6 and 7 of the connector shall be used to transmit and receive messages complying with ISO 11992-2:2014;

2.1.2.	Be capable of receiving all of the messages transmitted by the motor vehicle to be type approved and be capable of transmitting all towed vehicle messages defined within ISO 11992-2:2014;

2.1.3.	Provide a direct or indirect readout of messages, with the parameters in the data field shown in the correct order relative to time

2.2.	Checking procedure

2.2.1.	Check the following, with the simulator connected to the motor vehicle via the ISO 7638 interface and whilst all towed vehicle messages relevant to the interface are being transmitted:

2.2.1.1.	Low Tyre Pressure Warning indication:

2.2.1.1.1.	Simulate a towed vehicle low tyre pressure warning and check that the low tyre pressure warning signal specified in paragraph 5.5 of this regulation is displayed.

The parameters defined in EBS 23 bytes 1 and 2 of ISO 11992-2:2014 shall be transmitted as follows:

| Control line signalling | EBS 23 Byte 1 Bits 1 - 2 | EBS 23 Byte 2 |
| --- | --- | --- |
| Low Tyre Pressure Warning for tyre/wheel identification number 1,7 (Axle 1, left inner) | 002 (tyre pressure insufficient) | 000101112 (Tyre/Wheel “1,7”) |

2.2.1.1.2.	Simulate a towed vehicle low tyre pressure warning (without known tyre/wheel ID) and check that the low tyre pressure warning signal specified in paragraphs 5.5 of this Regulation is displayed.

The parameters defined in EBS 23 bytes 1 and 2 of ISO 11992-2:2014 shall be transmitted as follows:

| Control line signalling | EBS 23 Byte 1 Bits 1 - 2 | EBS 23 Byte 2 |
| --- | --- | --- |
| Low Tyre Pressure Warning (without known tyre/wheel ID) | 002 (tyre pressure insufficient) | 000000002 (Tyre/Wheel ID not defined or wheel not defined and axle > 1510) OR 111111112 (Tyre/Wheel ID not available available or wheel = 1510 and axle = 1510) |

2.2.1.2.	TPMS/ TPRS/ CTIS Malfunction Warning:

2.2.1.2.1.	Simulate a towed vehicle TPMS/ TPRS/ CTIS malfunction, signalled by the towed vehicle TPMS/ TPRS/ CTIS, and check that the towed vehicle TPMS/ TPRS/ CTIS malfunction indication warning signal specified in paragraph 5.5.6. of this Regulation is displayed.

The parameters defined in EBS 23 bytes 1 and 2 of ISO 11992-2:2014 shall be transmitted as follows:

| Control line signalling | EBS 23 Byte 1 Bits 1 - 2 | EBS 23 Byte 2 |
| --- | --- | --- |
| TPMS/ TPRS/ CTIS Malfunction for tyre/wheel identification number 1,7 (Axle 1, left inner) | 102 (Error indicator) | 000101112 (Tyre/Wheel “1,7”) |

2.2.1.2.2.	Simulate a towed vehicle TPMS/ TPRS/ CTIS malfunction (without known tyre/wheel ID) and check that the towed vehicle TPMS malfunction indication warning signal specified in paragraph 5.5.6. of this Regulation is displayed.

The parameters defined in EBS 23 bytes 1 and 2 of ISO 11992-2:2014 shall be transmitted as follows:

| Control line signalling | EBS 23 Byte 1 Bits 1 - 2 | EBS 23 Byte 2 |
| --- | --- | --- |
| TPMS/ TPRS/ CTIS Malfunction (without known tyre/wheel ID) | 102 (Error indicator) | 000000002 (Tyre/Wheel ID not defined or wheel not defined and axle > 1510) OR 111111112 (Tyre/Wheel ID not available or wheel = 1510 and axle = 1510) |

2.2.1.2.3.	Simulate a permanent failure in the communication line and check that the towed vehicle TPMS/ TPRS/ CTIS malfunction indication warning signal specified in paragraph 5.5.6. of this Regulation is displayed.

2.2.1.2.4.	Note that the towed vehicle TPMS/ TPRS/ CTIS malfunction indication would not be displayed in the case that valid TPMS/ TPRS/ CTIS information is available on an alternative interface.

3.	Towed vehicles

3.1.	ISO 11992 towing vehicle simulator

Figure 1

Arrangement of device under test and vehicle simulator where TPMS/ TPRS/ CTIS functionality is provided by ECU connected via ISO 11898-1:2015 and 11898-2:2016 interface

Towed Vehicle

ISO 11992 Towing Vehicle Simulator

ISO 11992-2 Towed Vehicle ECU

ECU providing TPMS/ TPRS/ CTIS functionality

ISO 11898

ISO 11992-2

Figure 2

Arrangement of device under test and vehicle simulator where TPMS/ TPRS/ CTIS functionality is provided by ECU connected to towing vehicle

Towed Vehicle

ISO 11992 Towing Vehicle Simulator

ISO 11992-2 Towed Vehicle

ECU including TPMS/ TPRS/ CTIS functionality

ISO 11992-2

The simulator shall:

3.1.1.	Have a connector meeting ISO 7638 (7 pin) to connect to the vehicle under test. Pins 6 and 7 of the connector shall be used to transmit and receive messages complying with ISO 11992-2:2014;

3.1.2	Have a warning display and an electrical power supply for the towed vehicle;

3.1.3.	Be capable of receiving all of the messages transmitted by the towed vehicle to be type approved and be capable of transmitting all motor vehicle messages defined within ISO 11992-2:2014;

3.1.4.	Provide a direct or indirect readout of messages, with the parameters in the data field shown in the correct order relative to time.

3.2.	Checking procedure

3.2.1	Configure the ISO 11992-2:2014 towed vehicle ECU to use either VIN “AABBCCDDEE1234567” or the actual VIN of the towed vehicle.

3.2.2	Check the following, with the simulator connected to the towed vehicle and whilst all towing vehicle messages relevant to the interface are being transmitted:

3.2.2.1.	The transmitted VIN shall be the one configured in paragraph 3.2.1. of this Annex

3.2.2.2.	Follow the test procedure defined in Annex 3 of this Regulation for TPMS or Annex 4 of this Regulation for TPRS/ CTIS and check that the TPMS/ TPRS/ CTIS warning and malfunction signals are transmitted as defined in paragraphs 2.2. and 2.3. of Part A of Annex 5 to this Regulation.

Annex 7

Performance testing of Tyre Pressure Monitoring System / Tyre Pressure Refill System / Central Tyre Inflation System

1.	General

1.1.	This Annex defines the procedure to determine the performance of a Tyre Pressure Monitoring System (TPMS), Tyre Pressure Refill System (TPRS) or Central Tyre Inflation System (CTIS) intend to be fitted to vehicles of category O3 and O4.

1.2.	Tests carried out on trailers of category O4 will be deemed to cover the requirements for trailers of O3 category.

2.	Information document

2.1.	The manufacturer of the TPMS / TPRS / CTIS shall supply to the Technical Service an information document of the system(s) requiring performance verification. This document shall contain at least the information defined in Appendix 1 and if applicable Appendix 2 to this Annex.

2.2.	If applicable the manufacturer(s) of an ISO11992-2 Towed Vehicle ECU (Gateway ECU) shall supply to the manufacturer of the TPMS / TPRS / CTIS an information document of the system(s) requiring performance verification to be submitted to the Technical Service by the manufacturer of the TPMS / TPRS / CTIS. This document shall contain at least the information defined in Appendix 2 to this Annex.

3.	Definition of test vehicles

3.1.	Based on the information supplied in the information document, in particular the trailer applications defined in paragraph 2. of Appendix 1, the Technical Service shall carry out tests on representative trailer(s) having up to maximum number of axles defined in the information document and equipped with the respective to TPMS / TPRS / CTIS configuration. Additionally, when selecting trailer(s) for evaluation consideration shall also be given to the parameters defined in the following paragraphs.

3.1.1.	Number of axles.

3.1.2.	Number and type of rim and tyre sizes per axle

3.1.3.	Number and position of lift axles

3.1.4.	Distance between wheels and receiver/antenna

3.1.5.	Supported range of reference/nominal pressure

3.2.	For the purpose of the approval, semi-trailers, full trailers, dolly trailers and center axle trailers shall be deemed to be of the same vehicle type.

4.	Test schedule for reference trailer

4.1.	The following tests shall be conducted by the Technical Service on the vehicle(s) defined in paragraph 3. of this Annex for each TPMS / TPRS / CTIS configuration taking into consideration the application list defined in paragraph 1.4. of Appendix 1 and if applicable Appendix 2 to this Annex.

4.1.1.	In case of TPMS: Puncture Test: Conduct a test according to Annex 3 to this Regulation

4.1.2.	In case of TPMS: Diffusion Test: Conduct a test according to Annex 3 to this Regulation

4.1.3.	In case of TPRS / CTIS: Refill Test: Conduct a test according to Annex 4 to this Regulation

4.1.4.	In case of TPMS: Malfunction Test: Conduct a test according to Annex 3 to this Regulation

4.1.5.	In case of TPRS / CTIS: Malfunction Test: Conduct a test according to Annex 4 to this Regulation

4.1.6.	Data communication: In case of usage of ISO11992 Gateway: Communication Test: Conduct a test according to Annex 6 to this Regulation.

Annex 7 - Appendix 1

Tyre Pressure Monitoring System / Tyre Pressure Refill System/ Central Tyre Inflation System Information document for reference trailer

1.	General

1.1. 	Name and address of manufacturer

1.2. 	System name

1.3. 	System variations

1.4. 	System configurations (e.g. number of axles / number of tyres etc.)

1.5. 	Explanation of the basic function and/or philosophy of the system.

2. 	Applications

2.1. 	List of trailer types and TPMS / TPRS / CTIS configurations for which approval is required.

2.2. 	Schematic diagrams of the system configurations installed on the trailers defined in item 2.1. above with consideration given to the following parameters:

- 	Number of axles;

- 	Wheel locations;

- 	Position of lift axles;

- 	Position of receiver/antenna locations;

- 	Position of sensors.

2.3 	Installation / Integration limitations (e.g. distance between receiver/antenna and wheel or rim, input messages and signals required by TPMS so that the vehicle shall meet the performance requirements contained in paragraphs 5.1.2. to 5.6. of this Regulation and fulfil the tests (puncture, diffusion and malfunction) as specified in Annex 3 to this Regulation e.g. vehicle-speed)

2.4.	Additional information (if applicable) to the application of the TPMS / TPRS / CTIS

3. 	Component description

3.1. 	Sensor(s)

- 	Function

- 	Identification (e.g. part number(s))

3.2. 	Receiver(s) / Antenna (s)

- 	General description and function

- 	Identification (e.g. part number(s))

- 	Additional features (e.g. automatic configuration, variable

- 	parameters, diagnostics)

- 	Failure modes

3.3. 	Electrical equipment

- 	Circuit diagram(s)

- 	Powering methods

3.4. 	If applicable the Electro-magnetic compatibility according to UN Regulation No. 10 as last amended by

(a) 	the 03 series of amendments for vehicles without a coupling system for charging the Rechargeable Electric Energy Storage System (traction batteries);

(b) 	the 06 series of amendments for vehicles with a coupling system for charging the Rechargeable Electric Energy Storage System (traction batteries).

3.5.	Additional information (if applicable) to the component description of the TPMS / TPRS / CTIS

Annex 7 - Appendix 2

ISO11992-2 Gateway ECU Information document for reference trailer

1. 	General

1.1. 	Name and address of manufacturer of the Gateway

1.2. 	System name

1.3. 	System variations

1.4. 	System configurations (e.g. number of axles/ number of tyres etc.)

1.5. 	Explanation of the basic function and/or philosophy of the system.

2. 	Applications

2.1. 	List of trailer types and configurations for which approval is required.

2.2. 	Schematic diagrams of the system configurations installed on the trailers defined in item 2.1. above with consideration given to the following parameters:

- 	Enabling of TPMS gateway functionality.

2.3. 	Installation limitations

2.4. 	Additional information (if applicable) to the application of the TPMS / TPRS / CTIS.

3. 	Component description

3.1. 	Gateway for ISO11992 according Annex 5

- 	General description and function;

- 	Identification (e.g. part number(s));

- 	Limitations (e.g. compatibility with other CAN-bus attendees), output of messages and signals required by TPMS so that the vehicle shall meet the performance requirements contained in paragraphs 5.1.2. to 5.6. of this Regulation and fulfil the tests (puncture, diffusion and malfunction) as specified in Annex 3 to this Regulation e.g. vehicle-speed);

- 	The verification of this element might be provided by a separate test report.

3.2. 	Electrical equipment

- 	Circuit diagram(s);

- 	Powering methods.

3.3.	If applicable the Electro-magnetic compatibility according to UN Regulation No. 10 as last amended by

(a)	the 03 series of amendments for vehicles without a coupling system for charging the Rechargeable Electric Energy Storage System (traction batteries);

(b)	the 06 series of amendments for vehicles with a coupling system for charging the Rechargeable Electric Energy Storage System (traction batteries).

3.4.	Additional information (if applicable) to the component description of the TPMS / TPRS / CTIS.

Annex 7 – Appendix 3

Tyre Pressure Monitoring System/ Tyre Pressure Refill System / Central Tyre Inflation System Test Report for reference trailer

1.	Identification

1.1	Name and address of manufacturer of the Tyre Pressure Monitoring System (TPMS) / Tyre Pressure Refill System (TPRS) / Central Tire Inflation System (CTIS)

1.2	System name / model

1.3	Monitoring function for TPMS

1.4	Control Monitoring function for TPRS / CTIS

2.	System(s) and installations approved:

2.1	TPMS / TPRS / CTIS configurations (where appropriate): definition of parameters which have impact on the TPMS / TPRS / CTIS performance. All allowed sub-configurations shall be supported if marked accordingly, e.g. if only a subset of axles is implemented.

|  | Number of axles |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| Single Tyre |  |  |  |  |  |  |
| Twin Tyre |  |  |  |  |  |  |
| Nominal Pressure Range (kPa) |  |  |  |  |  |  |

	Type or specification of other factors, e.g. rim types and sizes,

	Number and position of Lift Axles,

	Position and configuration of receiver and /or antennas incl. the allowed installation area.

2.2	Other limitations defined by the Manufacturer (Range of application (trailer type(s) and number of axles, input messages and signals required by TPMS) so that the vehicle shall meet the performance requirements contained in paragraphs 5.1.2. to 5.6. of this Regulation and fulfil the tests (puncture, diffusion and malfunction) as specified in Annex 3 to this Regulation e.g. vehicle-speed)

2.3	Additional features (if applicable)

3.	Test results

3.1	Tyre Class, Marking and wheel size(s) of standard unit equipment

3.2	Brief description of the Tyre Pressure Monitoring System (TPMS) / Tyre Pressure Refill System (TPRS) / Central Tyre Inflation System (CTIS) including implemented measures to avoid inadvertent reset control operation according to paragraph 5.1.6. to this Regulation, if applicable

3.3	Result of the tests

3.4	According to Annex 3 (TPMS) to this Regulation, if applicable:

| Test | Measured Time to warning (mm:ss) |
| --- | --- |
| Puncture Test |  |
| Diffusion Test |  |
| Malfunction Test |  |

3.5	According to Annex 4 (TPRS / CTIS) to this Regulation, if applicable2:

| Test | Measured Time to warning (mm:ss) |
| --- | --- |
| Refill Test |  |
| Malfunction Test |  |

4.	Limits of installation

5.	Date of test:

6.	This test has been carried out and the results reported in accordance with Annex 3 /Annex 4 to UN Regulation No. 141 as last amended by the 01 series of amendments.

7.	Technical Service conducting the test

Signed: ................................... Date: ................................

Annex 7 – Appendix 4

ISO11992-2 Gateway ECU Test Report for reference trailer

1.	Identification

1.1.	Manufacturer of the ISO11992-2 Gateway ECU (name and address)

1.2.	System name / model

2.	System(s) and installations approved:

2.1.	Range of application (trailer type(s) and number of axles)

2.2.	System identification of TEBS system to provide ISO11992 Gateway function

3.	Test results

3.1.	Brief description of ISO11992-2 Gateway ECU

3.2.	Result of the tests

3.3.	According to Annex 6 (Gateway Function) to this Regulation:

| Test | Measured Time to warning (mm:ss) |
| --- | --- |
| Low Tyre Pressure Test |  |
| Malfunction Test |  |

4.	Limits of installation

4.1.	Other recommendations/limitations (Output of messages and signals required by TPMS so that the vehicle shall meet the performance requirements contained in paragraphs 5.1.2. to 5.6. of this Regulation and fulfil the tests (puncture, diffusion and malfunction) as specified in Annex 3 to this Regulation e.g. vehicle-speed)

4.2.	Other recommendations/limitations (Output of messages and signals required by TPRS / CTIS so that the vehicle shall meet the performance requirements contained in paragraphs 5.1.2. to 5.6. of this Regulation and fulfil the tests (puncture, diffusion and malfunction) as specified in Annex 4 to this Regulation e.g. vehicle-speed)

5.	Date of test:

6.	This test has been carried out and the results reported in accordance with Annex 6 to UN Regulation No. 141 as last amended by the 01 series of amendments.

7.	Technical Service conducting the test

Signed: ................................... Date: ................................

Annex 8

Alternative procedure for type approval of subject trailer(s)

1.	General

1.1.	This Annex defines an alternative procedure for type approval of trailers, regarding their Tyre Pressure Monitoring System (TPMS) / Tyre Pressure Refill System (TPRS) / Central Tyre Inflation System (CTIS) utilizing information from test reports issued in accordance with Annex 7, Appendix 3 and / or Appendix 4 to this Regulation.

1.2.	On completion of the verification procedures described in Annex 3, the Type Approval Authority shall issue a UN type approval certificate conforming to the model specified in Annex 1 to this Regulation.

2.	Application for type approval

2.1.	The application for UN type approval of a trailer type with regard to TPMS, TPRS or CTIS shall be submitted by the trailer manufacturer. The trailer manufacturer shall supply to the Type Approval at least the following:

2.2.	Test reports according Annex 7, Appendix 3 and / or Appendix 4 to this Regulation from the TPMS / TPRS / CTIS and/or Gateway supplier which confirms the function of the systems of a reference trailer, covering the application of subject trailer  to be approved.

2.2.1.	The information documents of a trailer type hereafter referred to as the "reference trailer" according to Annex 7 to this Regulation on which the TPMS / TPRS / CTIS is to be based. This trailer will have been subject to the actual tests defined in Annex 3 and / or Annex 4 and / or Annex 6 to this Regulation for the appropriate trailer. A trailer that has been approved to the alternative procedure defined in this Annex shall not be used as a reference trailer.

2.2.2.	A trailer, representative of the trailer type to be approved hereafter referred to as the "subject trailer".

3.	Verification

3.1.	The requirements of Annex 3 and / or Annex 4 and/or Annex 6 are considered to be met if the subject trailer configuration is equivalent with the following conditions:

3.1.1.	The subject trailer configuration complies with one of the marked configurations in the test report in terms of number of axles, number of tyres and position of lift axles

3.1.2	The allowed pressure range of the nominal pressure is within the range of the reference trailer.

3.1.3.	Installation/Integration and setup of TPMS / TPRS / CTIS and the ISO11992-2 Gateway ECU (if applicable) on the subject trailer is in accordance with the installation/integration and setup limitations specified for the reference trailer.

3.2.	Add information documents about subject trailer according to Appendix 1 and Appendix 2 of this Annex.

4.	General

4.1.	Testing of a trailer in accordance with Annex 3 and / or Annex 4 to this Regulation may be waived by the Type Approval Authority at the time of type approval of the trailer provided that

	the Tyre Pressure Monitoring System (TPMS) complies with the requirements of Annex 3 or

	the Tyre Pressure Refill System (TPRS) complies with the requirements of Annex 4 or

	the Central Tyre Inflation System (CTIS) complies with the requirements of Annex 4

and, if applicable, Gateway complies with the requirements of Annex 6 to this Regulation.

5.	Installation limitation checks shall be made according to Annex 7, Appendix 1 and Appendix 2, paragraph 2.3. of this Regulation, if applicable.

Annex 8 - Appendix 1

Tyre Pressure Monitoring System / Tyre Pressure Refill System / Central Tyre Inflation System Information document for subject trailer(s)

1. 	General

1.1. 	Name and address of manufacturer

1.2. 	System name

1.3. 	System configuration of the TPMS / TPRS / CTIS (e.g.: which components are included)

2. 	Application

2.1. 	System configuration of the subject trailer:

- 	Number of axles

- 	Wheel locations

- 	Position of lift axles

- 	Position of receiver/antenna locations

- 	Position of sensors

- 	Nominal pressure range.

2.2. 	Schematic diagrams of the system configuration installed on the trailer defined in item 2.1.

2.3.	Installation/ Integration limitations (e.g. input messages and signals required by TPMS so that the vehicle shall meet the performance requirements contained in paragraphs 5.1.2. to 5.6. of this Regulation and fulfil the tests (puncture, diffusion and malfunction) as specified in Annex 3 to this Regulation e.g. vehicle-speed)

2.4.	Additional information (if applicable) to the application of the TPMS / TPRS / CTIS

3. 	Component description

3.1. 	Sensor(s)

- 	Identification (e.g. part number(s)).

3.2. 	Receiver(s) / Antenna (s)

- 	Identification (e.g. part number(s)).

3.3. 	Electrical equipment

- 	Circuit diagram(s)

- 	Powering methods.

3.4. 	If applicable the electromagnetic compatibility according to UN Regulation No. 10 as last amended by

(a)	the 03 series of amendments for vehicles without a coupling system for charging the Rechargeable Electric Energy Storage System (traction batteries);

(b)	the 06 series of amendments for vehicles with a coupling system for charging the Rechargeable Electric Energy Storage System (traction batteries).

3.5.	Additional information (if applicable) to the component description of the TPMS / TPRS / CTIS.

Annex 8 - Appendix 2

ISO11992-2 Gateway ECU information document for subject trailer(s)

1. 	General

1.1. 	Name and address of manufacturer

1.2. 	System name

1.3. 	System configuration in respect to TPMS / TPRS / CTIS

2. 	Applications

2.1. 	System configurations applied; e.g. other ECUs connected to the same CAN bus as the TPMS / TPRS / CTIS ECU

2.2. 	Schematic diagrams of the system configurations installed on the subject trailers

2.3. 	Limitations (e.g. compatibility with other CAN-bus attendees, output of messages and signals required by TPMS so that the vehicle shall meet the performance requirements contained in paragraphs 5.1.2. to 5.6. of this Regulation and fulfil the tests (puncture, diffusion and malfunction) as specified in Annex 3 to this Regulation e.g. vehicle-speed)

2.4.	Additional information (if applicable) to the application of the TPMS / TPRS / CTIS

3. 	Component description

3.1. 	Gateway for ISO11992 according to Annex 5

- 	Identification (e.g. part number(s))

- 	Description of other connected components (e.g. on CAN-bus)

3.2. 	Electrical equipment

- 	Circuit diagram(s)

- 	Powering methods

3.3. 	If applicable the Electro-magnetic compatibility according to UN Regulation No. 10 as last amended by

(a)	the 03 series of amendments for vehicles without a coupling system for charging the Rechargeable Electric Energy Storage System (traction batteries);

(b)	the 06 series of amendments for vehicles with a coupling system for charging the Rechargeable Electric Energy Storage System (traction batteries).

3.4. 	Additional information (if applicable) to the component description of the TPMS / TPRS / CTIS."

ANNEX 9  Provisions for vehicles equipped with an ADS

This Annex is only applicable to vehicles equipped with an ADS (see definition 2.16).

- General

This Annex explains how this regulation is adapted to apply to vehicles equipped with an ADS. It does not add nor remove any requirement.

- Definitions

Definition of subcategories of vehicles equipped with an ADS:

Subcategory X: 		Intended to be used without a driver and designed to carry occupants.

Subcategory Y:	Intended to be used without a driver and not designed to carry occupants.

Subcategory Dual mode: 	Intended to be controlled either by a driver or by the ADS.

Note: any change of those definitions in RE3 will require an update of this chapter.

- General principles for the provisions for vehicles equipped with an ADS

The provisions in this Annex are based on the following principles:

- This regulation requires 2 mandatory signals to the driver. These signals are:

  - A warning signal for low tyre pressure (par. 5.2, 5.3)

  - A malfunction signal (par. 5.4)

- These signals shall be available to the ADS as a logical binary signal.

- In vehicles of subcategories X and Y, the optical warning light on the vehicle is not mandatory to fulfill this regulation.

- For dual mode vehicles both the requirements of this regulation and the provisions for the ADS in this Annex are mandatory.

Add b.:

The warning signal for low tyre pressure indicates a potential unsafe situation due to under inflation and its impacts on the environmental performances (f.ex. fuel consumption and CO2-emission). The manufacturer shall have procedures to take this into account.

The malfunction signal is activated when the vehicle is in a non-compliant state. Corrective actions shall be part of the procedures for an ADS.

The logical binary signals that are available for the ADS are not specified in this Annex. Based on the information provided by the vehicle manufacturer, the Type Approval Authority and its Technical Service will decide how these signals are to be used in the test methods. The test report shall contain instructions for third party entities to verify these signals for in use conformity.

Add c:

A description of the corrective action (f.ex. initiating inspection, repair or inflation of the tyres) is part of the instructions to the driver in conventional vehicles(paragraph 6 of the core of this regulation). The ADS safety [concept/strategy/design] shall have an equivalent procedure for corrective actions.

Add d.:

For dual mode vehicles the provisions of this Annex 9 are complementary to the existing requirements in the regulation. The provisions of Annex 9 provide a base to test the ADS mode.

Annex 9 – Appendix 1:

Special provisions for the approval of vehicles equipped with an ADS

The additional clarifications below shall be used for the approval of vehicles equipped with an ADS.

The numbers between brackets refer to the paragraphs of the core of this regulation that applies to vehicles not equipped with an ADS.

- (5.1.1.2.)

If more than one system as defined in paragraphs 2.8., 2.14. or 2.15. has been installed, the system(s) which communicate(s) to the ADS shall be approved according to the requirements of this Regulation.

If more than one system is installed on the vehicle it has to be ensured that no contradictory information is transmitted to the ADS, e.g. by prioritization

- (5.1.6)

Implication of reset on ADS (for discussion)

- (5.2.) Tyre pressure detection for incident-related pressure loss.

AND

(5.3.) Detection for a tyre pressure level significantly below the recommended pressure for optimum performance including fuel consumption and safety.

Where it reads: The TPMS/ TPRS/ CTIS shall illuminate the warning signal described in paragraph 5.5. of this regulation,

This shall be replaced for vehicles of subcategory X and Y by: shall transmit a logical binary signal to the ADS described in paragraph 5 of this Annex.

4	(5.4.) Malfunction detection.

4.1	(5.4.1.) The TPMS/ TPRS/ CTIS shall transmit a logical binary signal described in paragraph 5.5. no more than 10 minutes after the occurrence of a malfunction that affects the generation or transmission of control or response signals in the vehicle's TPMS/ TPRS/ CTIS.

4.2	(5.4.2.) The malfunction indication warning signal described in paragraph 5.5. shall be transmitted to the ADS whenever the towed vehicle TPMS/ TPRS/ CTIS provides a malfunction indication via the communication interface described in paragraph 5.6.

4.3	(5.4.3.) The malfunction indication warning signal described in paragraph 5.5. shall be transmitted to the ADS.

5	(5.5)Warning indication to the ADS

5.1	(5.5.1)The warning signal to the ADS is available as a logical binary output of the TPMS.

5.3	The paragraphs 5.5.4, 5.5.5 and 5.5.6 of this regulation describe the visibility of the TPMS tell-tale. These are not applicable for vehicles of subcategories X and Y.

5.4	(5.5.6)	The malfunction indication shall be available to the ADS controller as binary logical output of the TPMS/TPRS/CTIS.

Provisions for Annex 3 of this regulation (Tests requirements for TPMS)

6	(annex 3 - 1.5.1) Test weight

Vehicles of subcategories X and Y may be tested without a driver on board.

7.	(annex 3 - 2.) Test procedure

7.1	(annex 3 - 2.2.)	With the vehicle stationary and the ignition locking system in the "Lock" or "Off" position, activate the ignition locking system to the "On" or "Run" position. The ECU controlling the logical binary signal to the ADS shall perform a signal check for the low tyre pressure as specified in paragraph 3 (5.2) of this Annex.

7.2	(annex 3 - 2.6.) Low tyre pressure detection phase.

7.2.1	(annex 3 - 2.6.1.) Procedure for the puncture test to verify the requirements of paragraph 3 of this Annex (5.2. to this Regulation).

7.2.1.1	(annex 3 - 2.6.1.1.) Drive the vehicle along any portion of the test course (not necessarily continuously). The sum of the total cumulative drive time shall be the lesser of 10 minutes or the time at which the low tyre pressure is signaled by means of the logical binary output to the ADS.

7.3	(annex 3 - 2.6.2.) Procedure for the diffusion test to verify the requirements of paragraph 3 of this Annex (5.3. to this Regulation).

7.3.1	(annex 3 - 2.6.2.1.) Drive the vehicle along any portion of the test course. After no less than twenty (20) minutes and no more than forty (40) minutes bring the vehicle to a complete standstill with the engine switched off and the ignition key removed for no less than one (1) minute or more than three (3) minutes. Resume the test. The sum of the total cumulative drive time shall be the lesser of sixty (60) minutes of cumulative driving under the conditions set out in paragraph 1.5.2. of annex 3 or the time at which the low tyre pressure is signaled by means of the logical binary output to the ADS .

7.4	(annex 3 - 2.6.3.) If the low tyre pressure signal was not sent to the ADS, discontinue the test.

8.	(annex 3 - 2.7.)	Low pressure signal to the ADS

8.1	(annex 3 - 2.7.1.) For vehicles of category M1 up to a maximum mass of 3,500 kg and N1

If low tyre pressure is signaled to the ADS during the procedure in paragraph 2.6. of Annex 3, deactivate the ignition locking system to the ''Off'' or ''Lock'' position. After a five minutes period, reactivate the vehicle's ignition locking system to the ''On'' (''Run'') position. The low tyre pressure shall be signaled to the ADS and remain in that state as long as the ignition locking system is in the "On" ("Run") position.

8.2	(annex 3 - 2.7.2.) For vehicles of category M2, M3, N2, N3, O3 and O4

If low tyre pressure is signaled to the ADS during the procedure in paragraph 2.6. of Annex 3, deactivate the ignition locking system to the ''Off'' or ''Lock'' position. After a five minutes period, reactivate the vehicle's ignition locking system to the ''On'' (''Run'') position. The low tyre pressure shall be signaled to the ADS within ten minutes and remain in that state as long as the ignition locking system is in the "On" ("Run") position.

8.3	(annex 3 - 2.8.)	Inflate all of the vehicle's tyres to the vehicle manufacturer's recommended cold inflation pressure. Reset the system in accordance with the instructions of the vehicle manufacturer. Determine whether the low pressure signal to the ADS has extinguished. If necessary, drive the vehicle until the low pressure signal has been extinguished. If the low pressure signal does not extinguish, discontinue the test.

9.	(annex 3 - 3.) TPMS malfunction detection

9.1	(annex 3 - 3.3.)	The sum of the total cumulative drive time under paragraph 3.2. of Annex 3 shall be the lesser of ten minutes or the time at which the TPMS malfunction signal is sent to the ADS.

9.2	(annex 3 - 3.4.)	If the TPMS malfunction indicator did not send the signal to the ADS in accordance with paragraph 5.4. to this Regulation 4 of this annex, as required, discontinue the test and the requirement is not fulfilled.

9.3	(annex 3 - 3.5.)	For vehicles of category M1 up to a maximum mass of 3,500 kg and N1

If the TPMS malfunction signal is sent to the ADS ( f.ex. during the procedure in paragraphs 3.1 to 3.3 of annex 3) above, deactivate the ignition locking system to the ''Off'' or ''Lock'' position. After five minutes, reactivate the ignition locking system to the ''On'' (''Run'') position. The TPMS malfunction indicator shall again signal a malfunction and remain in this state as long as the ignition locking system is in the ''On'' (''Run'') position.

[Alternative text:

When the TPMS malfunction remains, the TPMS shall continue to signal this to the ADS at any new activation of the vehicle.]

9.4	(annex 3 - 3.6.)	For vehicles of category M2, M3, N2, N3, O3 and O4

If the TPMS malfunction signal is sent to the ADS (f ex. during the procedure in paragraphs 3.1. to 3.3. of annex 3)above, deactivate the ignition locking system to the ''Off'' or ''Lock'' position. After five minutes, reactivate the ignition locking system to the ''On'' (''Run'') position. The TPMS malfunction indicator shall again signal a malfunction within ten minutes and remain in this state as long as the ignition locking system is in the "On" ("Run") position.

9.5	(annex 3 - 3.7.)	Restore the TPMS to normal operation. If necessary, drive the vehicle until the warning signal to the ADS has extinguished. If the warning signal has not extinguished, discontinue the test and the requirement is not fulfilled.

Provisions for Annex 4 of this regulation (Test requirements for TPRS and for CTIS)

10	(annex 4 -1.3.)	Vehicle condition

10.1	(annex 4 - 1.3.1.) Test weight

The vehicle may be tested at any condition of load, the distribution of the mass among the axles being that stated by the vehicle manufacturer without exceeding any of the maximum permissible mass for each axle.

However, in the case where there is no possibility to set or reset the system, the vehicle shall be unladen. For vehicles of category M1 up to a maximum mass of 3,500 kg, M2, M3, N1, N2, and N3 there may be, in addition to the driver, a second person on the front seat (if fitted) to help performing the test. Vehicles of subcategories X and Y may be tested without a driver on board.

11	(annex 4 - 2.) Test procedure

12	(annex 4 -2.3.) With the vehicle stationary and the ignition locking system in the "Lock" or "Off" position, activate the ignition locking system to the "On" or "Run" position. The TPRS / CTIS shall perform a signal check of the low tyre pressure signal to the ADS as specified in paragraph 5.5.2. of this Regulation 7.2 of this annex.

13	(annex 4 - 2.5.1.) Check refilling according to Figure 1 (Annex 4)

Check that within 2 minutes, when the system is operational, the TPRS/ CTIS starts refilling and at least after 2 minutes the low tyre pressure signal is sent to the ADS, as described in paragraph 5.5. of the Regulation3 of this annex.

Refill process shall be completed within 8 min after the refill process has started. The low tyre pressure signal to the ADS, as described in paragraph 5.5. of the Regulation, shall be “Off” as soon as the refilling process is completed.

After the refilling process has been completed, check that the tyre pressure is in a range of +/- 5% of manufacturers recommended cold inflation pressure Prec.

14	(annex 4 - 2.5.2.) Check refilling according to Figure 2 (Annex 4)

Check that within 2 minutes when the system is operational the TPRS/ CTIS starts refilling and at least after 2 minutes the low tyre pressure signal is sent to the ADS, as described in paragraph 5.5 3 of this annex.

Refill process shall not be completed within 8 min after the refill process has started and the low tyre pressure signal to the ADS, as described in paragraph 3 of this annex (paragraph 5.5. of the regulation), is “ON” after at least 2 minutes of refilling time.

The deflation rate during the test must be higher than the refilling rate.

15	(annex 4 - 2.6.)	TPRS/ CTIS malfunction detection

15.1	(annex 4 - 2.6.1.) Simulate a TPRS/ CTIS malfunction, When simulating a TPRS/ CTIS malfunction, the electrical connections for the connection to the ADS shall not be disconnected.

15.2	(annex 4 - 2.6.2.) Restore the TPRS/ CTIS to normal operation. If the malfunction signal to the ADS has not extinguished, discontinue the test and the requirement is not fulfilled.

Annex 5 – Compatibility…

Vehicles equipped with an ADS are not expected to be towing O3 and 04 vehicles

Annex 7 – Performance

Vehicles equipped with an ADS are not expected to be towing O3 and 04 vehicles

*	Former titles of the Agreement:

Agreement concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958 (original version);

Agreement concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions, done at Geneva on 5 October 1995 (Revision 2).

As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.6, para. 2 - https://unece.org/transport/standards/transport/vehicle-regulations-wp29/resolutions.

This definition is based on SAE J3016 and ISO/PAS 22736 (Taxonomy and Definitions for Terms Related to Driving Automation Systems for On-Road Motor Vehicles). These standards define levels of driving automation based on the functionality of the driving automation system feature as determined by an allocation of roles in DDT and DDT fallback performance between that feature and the (human) user (if any). The term “Automated Driving System” is used specifically to describe a Level 3, 4, or 5 driving automation system.

The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.6, Annex 3 - https://unece.org/transport/standards/transport/vehicle-regulations-wp29/resolutions

1	Distinguishing number of the country which has granted/extended/refused/withdrawn approval (see approval provisions in the regulation).

2	Strike out what does not apply.

Strike out what does not apply

Strike out what does not apply

The reference trailer may be from a different manufacturer

Strike out what does not apply

Strike out what does not apply