Proposal for supplement 3 to the original series of amendments of UN Regulation No. 139 (Brake Assist Systems)

Submitted by expert from the European Commission *

The modifications to the current text are marked in “bold” for new or strikethrough for deleted characters.

I.	Proposal

Insert a new paragraph 1.2.3., to read:

"1.2.3.	Vehicles of categories X and Y1."

Paragraph 1 footnote 1, amend to read:

"1 M1 and N1 categories of vehicles areAs defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.48, para. 2 -www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htmlhttps://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 4.4.1 footnote 3, amend to read:

"3 The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.48 - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htmlhttps://unece.org/transport/vehicle-regulations/wp29/resolutions"

Insert new paragraph 5.7, to read:

"5.7.	Whilst an ADS feature is active, the BAS shall not influence the performance of the braking system."

Justification

This proposal is intended to ensure vehicles equipped with an Automated Driving System (ADS) are exempt or can comply with the requirements. The scope is amended to exempt fully autonomous vehicles as defined by the new vehicle categories X and Y in revision 8 of the Consolidated Resolution on the Construction of Vehicles (RE3).  In fully autonomous vehicles there will be no or limited provision for a driver and responsibility for effective braking will rest with the ADS.

For part time ADS vehicles (Dual Mode) amendments are made to section 5 of the regulation to ensure that when an ADS feature is active and the ADS is in control of the vehicle, the brake assist system does not affect the performance of the braking system.  Recognising that both categories of BAS defined in this regulation rely on driver input for their activation, such a change may not be necessary – however if an ADS causes the movement of a brake pedal this amendment will ensure that such movement does not influence the brake performance.

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.