Proposal for amendment to UN Regulation No. 140

(to be agreed and submitted to GRVA by TF-FADS)

Notes:

Modifications to the existing text of the Regulation indicated in bold for new characters and strikethrough for deleted characters.

I.	Proposal

Footnote 1, amend to read:

___________

"1 M1 and N1 categories of vehicles areAs defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev. 48, para. 2. - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.html
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraphs 2.7.1. and 2.7.2., amend to read:

"2.7.1.	That improves vehicle directional stability by at least having the ability to automatically control individually the braking torques of the left and right wheels on each axle1 to induce a correcting yaw moment based on the evaluation of actual vehicle behaviour in comparison with a determination of vehicle behaviour demanded by the driver or the ADS1 (as applicable);

2.7.2.	That is computer controlled with the computer using a closed-loop algorithm to limit vehicle oversteer and to limit vehicle understeer based on the evaluation of actual vehicle behaviour in comparison with a determination of vehicle behaviour demanded by the driver or the ADS (as applicable);"

Paragraphs 2.7.4. and 2.7.5., amend to read:

"2.7.4.	That has a means to monitor driver steering demand by the driver or the ADS (as applicable) and

2.7.5.	That has an algorithm to determine the need, and a means to modify propulsion torque, as necessary, to assist the driver in maintaining control of the vehicle."

Footnote 3, amend to read:

___________

"3 The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev. 48, Annex 3 - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htmlhttps://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 5.6.1., amend to read:

"5.6.1.	It shall be possible at a periodic technical inspection to confirm the correct operational status by visual observation of the warning signals following a power-on sequence.

Alternatively, in the case of categories X and Y1 suitable means shall be provided to enable the confirmation of the correct operational status of the system."

Paragraphs 6.2. and 6.2.1., amend to read:

"6.2.	Is operational over the full speed range of the vehicle, during all phases of driving including acceleration, coasting, and deceleration (including braking), and whilst an ADS feature is active, except:

6.2.1.	When the driver or the ADS (as applicable) has disabled ESC;"

Paragraph 7.4., amend to read:

"7.4.	ESC malfunction detection

The vehicle shall be equipped with a tell-tale that provides a warning to the driver of the occurrence of any malfunction that affects the generation or transmission of control or response signals in the vehicle's electronic stability control system.

The requirements of this section related to warning signals to the driver do not apply whilst an ADS feature is active. The following requirement shall apply instead:

Whilst an ADS feature is active, warning signals (e.g. failure status) shall be transmitted to the ADS according to the provisions specified for the tell-tales. The means by which it is ensured that existing detected faults are transmitted to the ADS before an ADS feature becomes active (e.g. previously detected faults which remain present) shall be documented by the manufacturer."

Paragraphs 7.5.1. to 7.5.1.2., amend to read:

"7.5.1.	The vehicle's ESC system shall always return to the manufacturer's original default mode that satisfies the requirements of paragraphs 6. and 7. at the initiation of each new ignition cycle, regardless of what mode the driver or the ADS had previously selected. However, the vehicle's ESC system need not return to a mode that satisfies the requirements of paragraphs 7. through 7.3. at the initiation of each new ignition cycle if:

7.5.1.1.	The vehicle is in a four-wheel drive configuration which has the effect of locking the drive gears at the front and rear axles together and providing an additional gear reduction between the engine speed and vehicle speed of at least 1.6, selected by the driver or the ADS for low-speed, off-road driving; or

7.5.1.2.	The vehicle is in a four-wheel drive configuration selected by the driver or the ADS that is designed for operation at higher speeds on snow-, sand-, or dirt-packed roads and that has the effect of locking the drive gears at the front and rear axles together, provided that in this mode the vehicle meets the stability performance requirements of paragraphs 7.1. and 7.2. under the test conditions specified in paragraph 8. However, if the system has more than one ESC mode that satisfies the requirements of paragraphs 7.1. and 7.2. within the drive configuration selected for the previous ignition cycle, the ESC shall return to the manufacturer's original default ESC mode for that drive configuration at the initiation of each new ignition cycle."

Paragraph 7.6., amend to read:

"7.6.	ESC Off tell-tale

If the manufacturer elects to install a control to turn off or reduce the performance of the ESC system under paragraph 7.5., the tell-tale requirements of paragraphs 7.6.1. to 7.6.4. shall be met in order to alert the driver or the ADS to the inhibited or reduced state of ESC system functionality. This requirement does not apply for the driver-selected mode referred to in paragraph 7.5.1.2."

Insert new paragraph 7.8. and subparagraphs, to read:

"7.8.	Special provisions for categories X and Y

7.8.1.	In the case of categories X and Y, all requirements in this regulation shall be fulfilled, and all respective tests shall be conducted.

7.8.2.	Absence of a steering control

7.8.2.1.	In the absence of a steering control (e.g. steering wheel), an equivalent parameter (e.g. steering rack translation) or the road wheel angle shall be used instead, and it shall be understood as a steering wheel angle applied throughout this regulation.

7.8.2.2.	The measurement of the equivalent parameter during the tests can be performed by two methods: either through a vehicle internal signal or by an external measurement device fitted. In case the vehicle internal signal is used, it shall be demonstrated by the manufacturer to the satisfaction of the technical service that there are no relevant differences between the measured values. Differences shall be less than or equal to 5 per cent."

Paragraph 8.3.2., amend to read:

"8.3.2.	Vehicle mass. The vehicle is loaded with the fuel tank filled to at least 90 per cent of capacity, and a total interior load of 168 kg comprised of the test driver, approximately 59 kg of test equipment (automated steering machine, data acquisition system and the power supply for the steering machine), and ballast as required to make up for any shortfall in the weight of test drivers and/or test equipment. Where required, ballast shall be placed in a location within the vehicle that reflects the intended mass distribution and is specified by the manufacturer. For vehicles with conventional seating, ballast may be placed on the floor behind the passenger front seat or if necessary in the front passenger foot well area. All ballast shall be secured in a way that prevents it from becoming dislodged during testing."

Insert new paragraph 8.3.6., to read:

"8.3.6.	Special provisions for vehicles of categories X and Y

Notwithstanding 8.3.5., in the case of categories X and Y, no external automated steering machine shall be used. To execute the required steering pattern in paragraphs 9.5.2., 9.5.3., 9.6. and 9.9., the steering equipment of the vehicle shall be used, with dedicated activation methods, which may include:

(a)	A test mode allowing to manually control or trigger the steering functions, or

(b)   Any other method subject to agreement between the vehicle manufacturer and the technical service, ensuring that the evaluation accurately reflects real-world ADS steering performance."

Insert new paragraph 9.12. and subparagraphs, to read:

"9.12.	Special provisions for vehicles of categories X and Y

9.12.1.	Test speed

In case the vehicle is not capable to reach the prescribed speeds for the test procedure specified in paragraphs 9.4., to 9.6. and 9.9.1., the maximum design speed of the vehicle shall be used instead.

9.12.2.	Steering performance

In accordance with 8.3.6., if the vehicle is not able to carry out the manoeuvres specified in 9.5.2. and 9.5.3. or the Sine with Dwell test as specified in 9.9., the maximum achievable steering rate performance allowed by the ADS shall be used instead.

9.12.3.	Lack of tyre saturation

In the case of vehicles which do not reach tyre saturation (as defined in paragraph 9.9.4.) at the maximum speed and steering rate attainable:

9.12.3.1.	A test shall be agreed between the vehicle manufacturer and the Technical Service and carried out on a test surface with a lower grip than that specified in paragraph 8.2.

9.12.3.2.	The test shall demonstrate that the ESC system has a positive effect on vehicle stability, such that without the intervention of the ESC, the manoeuvre would result in loss of stability. The dynamic manoeuvres, test conditions and results shall be included in the test report.

9.12.3.3.	Such vehicles are not required to undergo the Sine with Dwell test as specified in paragraph 9.9.

9.12.4.	Brake conditioning

When executing the stops in paragraph 9.4.2., sufficient brake demand shall be applied to bring the vehicle's antilock braking system (ABS) into operation for a majority of each braking event.

9.12.5.	Tyre conditioning

In case the vehicle is not able to reach the prescribed lateral acceleration in paragraph 9.5.1., the radius of the circle shall be decreased either until the lateral acceleration is reached or until full lock.

In case the vehicle is not able to reach the prescribed lateral acceleration in paragraph 9.5.2., the lateral acceleration resulting by the maximum achievable steering rate shall be used instead.

9.12.6.	Slowly increasing steer procedure

An equivalent steering pattern to that specified in 9.6. shall be defined by the vehicle manufacturer and shall be agreed with the Technical Service and appended to the type approval documentation.

9.12.7.	ESC malfunction detection

An equivalent method to that specified in 9.10. shall be defined by the vehicle manufacturer and shall be agreed with the Technical Service and appended to the type approval documentation.

9.12.8.	Post data processing – calculations for performance metrics

An equivalent method to that specified in paragraph 9.11. shall be defined by the vehicle manufacturer and shall be agreed with the Technical Service and appended to the type approval documentation.

In particular, it shall be established and explained in the type approval documentation, how

(a) The filtering method of the raw signals, as defined in paragraphs 9.11.1. to 9.11.3.,

(b) The calculation and filtering of the steering speed, as defined in paragraph 9.11.4.,

(c) Zeroing the signals, as defined in paragraph 9.11.5.,

(d) Definition of BOS and COS, as defined in paragraphs 9.11.6. and 9.11.7.,

(e) The time instances for yaw rate measurements, as defined in paragraph 9.11.8.,

(f) The lateral displacement measurement, as defined in paragraph 9.11.9.

were adjusted according to the actual sine wave pattern and measurement signal parameters used during the Test Procedure."