Proposal for a supplement to the 02 series of amendments of UN Regulation No. 152 on Advanced Emergency Braking Systems (AEBS)

Submitted by expert from the United Kingdom

The modifications to the current text are marked in “bold” for new or strikethrough for deleted characters.

I.		Proposal

Paragraph 1, amend to read:

"1.	Scope

This Regulation applies to the approval of vehicles of Category M1 and N11* with regard to an on-board system to:

(a) Avoid or mitigate the severity of a rear-end in lane collision with a passenger car;

(b) Avoid or mitigate the severity of an impact with a pedestrian;

(c) Avoid or mitigate the severity of an impact with a bicycle.

It does not apply to vehicles of categories X and Y1.”

Footnote 1, amend to read:

"1 As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.68, para. 2 - https://unece.org/transport/vehicle-regulations/wp29/resolutions https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 5.4.4, amend to read:

"5.4.4. While automated driving functions are in longitudinal control of the vehicle (e.g. ALKS is active) the AEBS function may be suspended or its control strategies (i.e. braking demand, warning timing) adapted without indication to the driver, as long as it remains ensured that the vehicle provides at least the same collision avoidance capabilities as the AEBS function during manual operation. Special provisions for vehicles equipped with an ADS1”

Insert new paragraphs 5.4.4.1 to 5.4.4.3:

“5.4.4.1. The AEBS shall be automatically deactivated whilst an ADS feature is active.

5.4.4.2. The AEBS shall be automatically reactivated upon deactivation of an ADS feature.

5.4.4.3. The provisions of paragraph 5.4.3. do not apply whilst an ADS feature is active.”

II.	Justification

- Existing 5.4.4 inappropriate.

- Explain 5.4.4.2. mean when there are no features active any more.

- ‘ADS Feature’ includes ALKS (or is this for RE3?).