Task Force on the Regulatory Fitness for Automated Driving Systems (FADS)

Twenty-seventh Meeting

_____________________________________________

| Date & Time | 11 September 2025 (09:30 – 12:30 CEST) 12 September 2025 (09:30 – 12:30 CEST) |
| --- | --- |
| Venue | Online (Microsoft Teams) Remote connection details below |

Provisional agenda (see details below)

- Approval of the agenda and minutes of the previous meeting

The agenda (FADS-27-01) was adopted.

The minutes of the 26th meeting (FADS-26-08) were approved.

- Amendment to UN Regulations and GTRs regarding their fitness for automated vehicles

  - Amendment of UN Regulation No. 79 for driverless vehicles

Document: 	FADS-26-07 (UK) Consolidated amendment proposal to R79

[For reference:  GRVA/2025/41 (FADS) New working document for the 23rd session of GRVA]

- No new document was received as there were no further needs for amendments.

- Conclusion: FADS-26-07 will be submitted to GRVA-23 as an informal document to amend GRVA/2025/41.

Link between R79 and R[ADS] - UK proposal

In FADS-25-11 industry explained the concept for the link between R79 and R[ADS]. This concept was reflected in the FADS R79 amendment GRVA-22-32.

An open item was to ensure R[ADS] has a corresponding requirement to ensure the compatibility is checked. UK has recently circulated a document to ADS Workshop experts proposing the following two provisions:

3.2.       The application for approval of a vehicle type with regard to the ADS shall be submitted by the manufacturer or by their duly accredited representative. It shall be accompanied by the documents mentioned below in triplicate:

[…]

3.2.4.                  In the case of ADS vehicles which are subject to the requirements of UN Regulation No. 79, an analysis showing that the safety concept of the ADS steering equipment is compatible with the safety concept of the ADS.

4.1.1.1.              In the case of ADS vehicles which are subject to the requirements of UN Regulation No. 79, the Approval Authority or the Technical Service shall verify the compatibility between the safety concept of the ADS steering equipment and the safety concept of the ADS.

- While OICA/CLEPA suggested having these provisions in the requirements sections, UK agreed that where this should sit will be a topic for discussion during the GRVA workshop on ADS scheduled in Geneva on 18 and 19 September 2025.

- Conclusion: FADS supports the proposal.

  - Amendment of UN Regulations No. 13 and 13-H for driverless vehicles

Documents: 	FADS-26-06 (FR) Removing ALKS exemption from R13 and R13-H amendments

[For reference: GRVA/2025/39 and GRVA/2025/40 (FADS) New working documents for the 23rd session of GRVA]

- The discussion was resumed on the French proposal to remove ALKS vehicle exemptions from R13 and R13-H.

- Industry expressed concerns about potential impacts on existing vehicle designs as the amendments introduce new requirements. It should therefore be dealt as a new series of amendments and not as supplement.

- EC understands there is no new requirements introduced but rather interpretation of existing requirements to new categories. EC supports to have them as supplements.

- UK confirmed that the application to earlier series should be avoided and limited to series where the CEL annex was updated and where ETBS provisions were introduced.

- Conclusion: Agreed to remove the exemption for ALKS vehicles and to reword proposals GRVA/2025/39 and GRVA/2025/40 respectively as supplements to 14 and 15 series to R13 and to 02 series to R13-H.

  - Amendment of UN Regulation No. 140 for driverless vehicles

Document: FADS-27-06 (EC) Proposal for amendments to R140

- The European Commission introduced the document which contains improvements of the text.

  - Provisions were added in 9.12.3. to cover the case when the vehicle is not able to achieve tyre saturation

  - Following a comment from Japan, a clarification on tyre conditioning when the vehicle is not able to reach the lateral acceleration was added.

  - As requested by UK, para. 9.12.8. about post data processing clarifies that the documentation shall demonstrate how the usual parameters are adjusted.

- Conclusion: The proposal will be submitted as an informal document to GRVA-23.

  - Amendment of GTR No. 8 for driverless vehicles

No discussion under this agenda item

  - Amendment of UN Regulation No. 89 for driverless vehicles

Document: FADS-27-05 (EC) Proposal for amendments to R89

- The European Commission introduced the document which contains improvements of the text.

  - 5.1.11. is a new paragraph dedicated to X and Y vehicles. UK suggested to improve the text as “ADS is capable to limit” is not strong enough.

  - 5.2.7.4. about deactivation of ASLD when ADS feature is active,

  - 21.2.7.5. “shall” was replaced by “may”. The reason is that the speed limit set before the activation of the ADS might not be appropriate anymore when ADS is deactivated. Same modification was done in 5.2.7.5.

- Conclusion: The proposal will be submitted as an informal document to GRVA-23 with squared brackets around the open items. The aim is to finalize the proposal for the 24th session of GRVA in January 2026.

  - Amendment of UN Regulation No. 90 for driverless vehicles

Document: FADS-27-02 (EC) Proposal for amendments to R90_clean

- The European Commission introduced the clean version of the proposal.

- Conclusion: The proposal will be submitted as an informal document to GRVA-23.

  - Amendment of UN Regulations No. 130, 131, 139 and 152 for driverless vehicles

Documents: 	FADS-27-07 (UK) Proposal for amendments to R131

FADS-27-08 (UK) Proposal for amendments to R152

- The United Kingdom presented the proposal FADS-27-07 with a straightforward concept to clarify the interactions between AEBS and ADS.

  - It was noted that a clarification might be needed in 5.4.5.2. in case of transition of one ADS feature to another.

- The same concept was applied to R152 in the proposal FADS-27-08.

- Conclusion: Both documents will be submitted as informal documents to GRVA-23.

Documents: 	FADS-27-03 (EC) Proposal for amendments to R130

FADS-27-04 (EC) Proposal for amendments to R139

- The European Commission introduced the documents.

- In FADS-27-04, the general title “Special provisions … “ in 5.7. was deleted.

- The Chair invited EC and UK to have a final check to ensure that the wording is the same in all proposals.

- EC informed they will work on other regulations like R158 (GRSG) as the approach is very similar.

- OICA/CLEPA mentioned that R171 and R175 will need similar amendments. The co-Chair requested volunteers to prepare documents for these regulations for the next meeting of FADS.

- Conclusion: Both documents will be submitted as informal documents to GRVA-23. Final version is reflected in the files with “_after_session” in the file name.

  - Document OICA/CLEPA for alternatives of ESC type-approval

Document: FADS-27-09 (OICA-CLEPA) ESC alternative v1

- OICA/CLEPA presented 2 alternatives for the type-approval of an ESC system and suggested to develop an appropriate wording in the UNR[ADS], R13 and R140.

- UK and EC agreed at high level as it looks workable even if refinements are needed.

- Australia expressed its concern about the cross-reference and would prefer to have it only in the ESC regulation and not ADS regulation.

- France said the proposal is not mature enough. R140 and R13 currently require performance checks including physical tests and replace them by the safety case seems to be anticipated. As the work on the R[ADS] will continue after June 2026, this could be taken on board later when more mature.

- OICA/CLEPA reminded that initially R140 was excluded and was surprised to hear that the safety case used to cover far more complex situation would not be suitable to cover the ESC test. The expert supported the UK approach to include the proposed text in square brackets.

- Conclusion: There is a general support from FADS for the concept, even if it was acknowledged that it will be difficult to add a new paragraph or requirements in the ADS regulation at this stage. It was recognized that more refinement of the proposal is needed to exempt all ADS vehicles from ESC and VSF requirements. Furthermore, as CPs had no time to review the text proposed, the co-Chair encouraged OICA/CLEPA to continue the discussion offline with the interested CPs.

- Coordination and harmonisation between GRs and IWG ADS

  - Review of amendment proposals from GRs other than GRVA

Concerns about GRBP definition harmonization

Document: GRBP/2025/21 amended by GRBP-82-11-r1 (R141)

- The European Commission requested to check the GRBP proposals being the first working documents endorsed with ADS amendments but following a different approach than FADS.

- Definitions from R.E.3 related to ADS are included in the proposals. This was a conscious decision of GRBP for clarity even if FADS in August questioned why definitions already present in R.E.3 were being referenced again.

- UK expressed concern that including category definitions in individual regulations creates risk if definition wording needs tweaking later.

- Special provisions for X and Y are grouped in a separate annex.

- Conclusion: The co-Chair proposed potentially raising this harmonization issue to WP.29, highlighting that GRBP has included these definitions directly in their regulations while other GRs reference to RE.3.

Proposal on the way forward in TF-AVRS

Document: [forthcoming, expected to be published as an informal document to GRVA-23]

- The Netherlands submitted a document expressing dissatisfaction with basic requirements in R79 stating that steering control shall be located so driver can operate vehicle safely.

- The idea is to bring category X and Y vehicles back into scope of R43, R46, R125, and R167 rather than exempting them, until the issue is solved a future amendment of R[ADS].

- It also questions the 6km/h manual operation which is not restricted to certain use cases and challenges behavioral aspects which should be covered in type-approval regulations.

- OICA/CLEPA expressed serious concern that agreements made in the AVC group on category X and Y definitions were being questioned. The limitation of 6km/h manual operation was intentionally included to provide exclusions from certain requirements. Industry is worried about having requirements that are inappropriate for these vehicles and questioned how behavioral aspects of manual modes could be regulated in type-approval.

- UK supported maintaining exemptions for paragraphs 5.7.1.18 and 5.7.1.19, arguing that constraining use of manual modes should be left to national regulations rather than type-approval.

- Conclusion: FADS acknowledges the intention of GRSG not to exclude X vehicles from the scope of some Regulations until dedicated requirements are designed. For the time being, FADS decided not to change the exemptions for remote manual controls of X/Y vehicles in R79.

Detailed review of:

Document: GRPE/2025/23 (R83)

- The European Commission presented the proposal addressing mostly details, but these have implications across 4-5 other emission regulations using similar definitions and provisions.

- Operating mode switch definitions for driver selectable modes remain open, with ongoing discussions about whether these should be limited to human users only.

- Physical device references like gear levers were suppressed along with controls that might be activated by humans.

- GRPE is still working on 7 other regulations.

Document: GRSG/2025/19 (R43)

- Vehicles of category X and Y are exempted from this regulation, which was seen by FADS as appropriate.

- Addition of footnotes clarifying that front exterior forward-facing glazing on category X and Y vehicles is not considered as a windscreen, similar to upper deck glazing on double-deck buses.

- Safety glazing material requisite for driver visibility definitions do not apply to X and Y.

- All windscreen-related annexes (4, 6, 8, and 10) do not apply to X and Y since they are not fitted with windscreens.

- Transmittance requirements are reduced below 70% while an ADS feature is active, raising concerns about first responder and law enforcement ability to see inside vehicles.

- Conclusion: FADS will suggest wording alignment for "while a driver performs the DDT" (to replace with "except whilst an ADS feature is active")

Document: GRSG/2025/20 (R46)

- Vehicles of category X and Y are exempted from this regulation.

- 15.1.5.: This requirement duplicates a similar requirement in R[ADS] regarding sufficient time for safe takeover, making the specific 500 ms requirement potentially misleading. It was also questioned whether this timing is appropriate for folding mirrors versus camera monitor systems, suggesting different technical requirements may need different timeframes.

- Conclusion: As FADS has some doubts about the requirement and sees potential conflict with R[ADS], FADS will raise the concern to GRSG task force AVRS. FADS will recommend to not put a specific timing as R[ADS] has a generic requirement.

Document: GRSG/2025/30 (R107)

- The co-Chair went through the proposal.

- Conclusion: FADS will provide feedback to GRSG task force AVRS regarding suggested wording changes for R107 requirements:

  - Replace "vehicles equipped with ADS feature type 2" with "whilst an ADS feature of type 2 is active" for consistency with other regulations.

  - Replace "shall be controlled by ADS" by "shall be capable of being controlled by ADS" to avoid creating direct ADS requirements in R107.

  - References to "satisfaction of technical service or type approval authority" to be replaced with references to ADS safety concept.

  - Wording "without human supervision" should be clarified as "without onboard human supervision" to distinguish from remote supervision capabilities.

  - Removing exemption for "vehicles of category X" in certain paragraphs where not necessary.

  - Other coordination activities

No discussion under this agenda item

- Any other business

Next meeting: 28th meeting from 20 to 23 October (hybrid in Paris). The co-chair from France shared practical information on the wiki page of the meeting.

__________________