Proposal for supplement [3] to 08 series of amendments to UN Regulation No.83

(Agreed and submitted to GRPE by TF-AVRS)

Notes:

Modifications to the existing text of the Regulation indicated in black bold for new characters and black strikethrough for deleted characters.

I.	Proposal

Annex 1, paragraph 2.6. amend to read:

"2.6.	Mass of the vehicle with bodywork and, in the case of a towing vehicle of category other than M1, with coupling device, if fitted by the manufacturer, in running order, or mass of the chassis or chassis with cab, without bodywork and/or coupling device if the manufacturer does not fit the bodywork and/or coupling device (including liquids, tools, spare wheel, if fitted, and driver, and, for buses and coaches, a crew member if there is a crew seat in the vehicle)7 (maximum and minimum for each variant). For vehicles of category X and category Y the driver’s mass shall not be included: ........ "

Annex 1, paragraph 3.4.3. amend to read:

“	[Selectable Ooperating] mode switch: with/without9”

Annex 2, paragraph 1.9.2. amend to read:

“	[Selectable Ooperating] mode switch: with/without1”

Annex 4, paragraph 5.7.2.,  amend to read:

“5.7.2.	The following checks shall be carried out: OBD checks (performed before or after the test), visual checks for lit indication of malfunction indicator from the OBD systemor lamps, checks on air filter, all drive belts, all fluid levels, radiator and fuel filler cap, all vacuum and fuel system hoses and electrical wiring related to the after-treatment system for integrity; checks on ignition, fuel metering and pollution control device components for maladjustments and/or tampering.”

Annex 4, Appendix 1, paragraph 1., amend to read:

"1.	Fuel tank level (full / empty). Is there an indication active of low fuel level fuel reserve light ON? If yes, refuel before test."

Annex 4, Appendix 1, paragraph 2., amend to read:

“2.	Are there any warnings lights on the instrument panel activated indicating a vehicle or exhaust after-treatment system malfunctioning that cannot be resolved by normal maintenance? (Mmalfunction Iindication Light, Eengine Sservice Iindication, etc?) If yes, the vehicle cannot be selected.”

Annex 4, Appendix 1, paragraph 1 3., amend to read:

"3.	Is there an indication active of low reagent level SCR light on after engine-on? If yes, the AdBlue reagent should be filled in, or the repair executed before the vehicle is used for testing."

Annex 4, Appendix 5, paragraphs 18. and 19., amend to read:

“18.	[Driver sSelectable] mode(s) used during the TA tests (pure ICE) or for charge sustaining test (NOVC-HEV, OVC-HEV, NOVC-FCHV).

19.	[Driver sSelectable] mode(s) used during the TA tests for charge depleting test (OVC-HEV).”

Annex 5, paragraphs 2.3. and 2.4. amend to read:

“2.3.	In the case of vehicles with manually-operated or semi-automatic-shift gearboxes, the test shall be carried out with the gear lever in the "neutral" position and with the clutch engaged.

2.4. 	In the case of vehicles with automatic-shift gearboxes, the test shall be carried out with the gear selector in either the "neutral" or the "parking" position.”