# Considerations on UN R154 ADS updates

The following considerations have been gathered by the European Commission through various discussions and exchanges as general ideas for initiating amendment work on UNR154. These considerations are intended solely for informational purposes and do not represent an official position of the European Commission.

# Proposal for a pragmatic approach to amend R154

- List the different tests that are performed under UNR 154. 
The following types of tests were found during the first review:

  - Road load testing in Annex B4, brake preconditioning (4.2.4.1.1.), warm-up (4.2.4.1.2.)
Notes: 
Annex B4 concerns road-load determination, which can be done in three ways:

    - by coasting down on a track,

    - by driving at constant speed on a track (torque-meter method), or

    - by testing in a wind tunnel.

The ‘golden method’ is by coast-down testing, where consistency is the most important, and this is the method where an ADS could replace a human driver. An ADS would be more consistent than a human.

  - Type 1 testing: Type 1 WLTP test, see Annex B6, 2.6.4.2. and onwards. This test is driven on a chassis dynamometer by a human driver. First reference to 'driver' is for decelerating in 2.6.4.5. Also see Annex B8 for PEVs.
Notes: 
The concept of “robot drivers” was already discussed many times and there was resistance in general.
Considering ISC testing: whichever option is allowed (or both), it would have to work for every production vehicle.

  - Type 4 testing: Type 4 test for evaporative emissions, see par. 6.6. and Annex C3, 6.6.2. to 6.6.4. (Note that there is only 6.6.2. in Annex C3). Main description of Type 4 test is in par. 6.5 to Annex C3.
Note: 
Type 4 test itself does not involve a human driver, but for the preconditioning a Type 1 test is driven.

  - Type 5 testing: Type 5 test for durability testing, can be done on track or road (see Annex C4, par. 6) using the Standard Road Cycle (SRC).
Note: It has been common practice for decades to use a “robot driver” for Type 5 tests, this task could be replaced by the ADS.

  - OBD testing (Annex C5, particularly Appendix 1 to Annex C5).
Note: OBD test essentially dies with Euro 7. We need a decision how far back we go in emissions standards to provide for ADS. 
OBD testing involves driving a Type 1 test to check emissions.

- Investigate whether the test operation by a human driver can be replaced by the ADS. 
It should not be difficult to prepare an ADS-equipped vehicle to perform this driving task, but it would require that for example the WLTC cycle is pre-programmed in the vehicle (alternatively, that a speed/time signal can be fed into the vehicle which is followed). High level discussion is needed on the general approach for allowing ADS to performing tests (requirements, conditions, tolerances, criteria, etc.).
Note: ADS may have “selectable modes”.

- Implement ADS testing in the text, which will need a broader discussion. 
We can add requirements / conditions for ADS vehicles in separate paragraphs, or in the given paragraph where the issue comes up.

- In parallel, where relevant, amend the current text to allow ADS operation. 
Generally, an approach is foreseen where preferably amendments are made at high level, to avoid that many similar amendments are needed. For example, if an amendment on ADS operation can be added to a definition, every time that the text refers to that definition it is implicitly covered.

# First screening analysis

A first analysis based on the AVRS screening document for UN R154 (document TF-AVRS-12-03 R154.02 (OICA).xls) shows that:

- There are 173 (potential) issues recognised in this document,

- There are many similar issues identified, which means that a given amendment can simply be copied to the others as well.

Ranking of the keywords with the largest share of hits (NOTE: some of these are double counted, e.g. ‘driver’ and ‘driver selectable mode’).

- Driver: 59% share (!)

- Driver selectable mode: 25% share

- Warning: 12% share

- Clutch: 10% share

- Operate: 9% share

- Accelerator: 8% share

- Seat: 5% share

- MI: 5% share

The proposal for amendment will have many duplications in it. This means we do not have to get agreement on 173 issues, but a much smaller amount.

Note: Amending definitions is much more effective than changing something in the text each time it appears.

# Considerations on testing ADS vehicles

- Analogy between manual driving and ADS in TA testing:
Conventional: The test engineer carrying out the emission tests knows that he/she is doing a Type 1 test, because he/she is driving the vehicle. 
ADS: the “test ADS” should know that it should do a Type 1 test, because it is driving the vehicle.

- There is a multitude of other tests which will require an ADS to be “pre-programed”. ESC tests in R140 (brake and tyre conditioning, sine with dwell test, etc.), brake tests in R13 and R13H (modified to fit ADS), steering equipment tests in R79 (modified to fit ADS) and basically everywhere, where a human should have driven the vehicle up to now for a TA test. In all these cases a human would prepare, learn the manoeuvre, carry it out and then repeat if not within the limits. The ADS has to have a similar capacity. It has to know the prescribed test manoeuvre and has to able to carry it out. It is not necessarily the same ADS that drives the vehicle on public roads.

- The ADS communicates with the vehicle systems through “transmission links” as called in R79. These are interfaces towards the steering, braking and other vehicle systems. These other vehicle systems know nothing about what happens in the outside world, they just carry out the demands sent by the ADS. The same is true for the emission control and after-treatment.

- TF FADS started considering a “modified” or “test” ADS only for the TA tests. Otherwise the mandated tests cannot be carried out on X and Y vehicles.

- Question for “dual mode”: are emissions the same in ADS mode and manual mode? Do we mandate only manual / only ADS? Do we mandate both? Do we leave it up to the TAA which one to choose? Do we leave the possibility to ask for both if deemed necessary?

# Other open issues

- Non-DDT tasks: all the information that is sent normally to the driver now has to be communicated to the ADS system. We need to discuss expected response / behaviour from the ADS, e.g. regarding the MI, fuel indicator, reagent indicator, etc.

- Which version of UNR 154 do we review? After a first discussion it was suggested to take UNR 154 Amendment 2 Supplement 2.

- Follow-up on other horizontal issues that need amendments to UNR 154, for example to redefine the key-on and key-off moments (considering that vehicles don’t have keys anymore).

# Solutions and examples in other regulations

The text below is taken from different amendment proposals of TF FADS, where the group was facing similar challenges in applying the existing testing provisions to the ADS. Sections of the amendment text are copied here to present examples. Bold text is used only to highlight important parts (NOT to highlight additions to the existing regulation text).

## R140 (Electronic Stability Control (ESC) Systems)

  - “For vehicles of category X and Y, an equivalent method of brake conditioning shall be specified by the vehicle manufacturer and shall be agreed with the Technical Service and appended to the type approval documentation.”

  - “For vehicles of category X and Y, the method how the slowly increasing steer procedure is performed shall be specified by the vehicle manufacturer and shall be agreed with the Technical Service and appended to the type approval documentation.”

  - “For vehicles of category X and Y, the method how the Sine with Dwell test is performed shall be specified by the vehicle manufacturer and shall be agreed with the Technical Service and appended to the type approval documentation. If the steering equipment of vehicle is not capable to execute the Sine with Dwell test with the sine wave frequency defined above, the highest achievable steering speed shall be used for the tests.”

## R79 (Steering equipment)

- “6.2. Provisions for motor vehicles

Paragraph 6.2.1. applies to all motor vehicles, both whilst being manually driven and whilst any ADS feature is active.

Paragraphs 6.2.2. to 6.2.5. do not apply to vehicles of categories X and Y, nor to vehicles of other categories whilst an ADS feature is active.

Paragraphs 6.2.6. and 6.2.7. apply only whilst an ADS feature is active.

…

6.2.6. The measurement of steering time of vehicles equipped with an ADS, with intact steering equipment, whilst the ADS is active.

…

6.2.7. The measurement of steering time of vehicles equipped with an ADS, with a failure in the steering equipment, whilst the ADS is active.

…

6.2.8. If the turning radii specified in paragraphs 6.2.6.1. or 6.2.7.1. are not attainable (because the steering angle available to the ADS is limited, or because full lock is reached first), the maximum attainable steering angle shall be used, and the maximum permitted steering time shall be adjusted proportionally according to the following formula…

…

6.2.9. The tests described in paragraphs 6.2.6. and 6.2.7. shall be conducted using a method subject to agreement between the manufacturer and the Technical Service, in order to demonstrate that the steering actuator(s) can achieve the specified steering times deliver the required steering rates.

The manufacturer shall demonstrate that the test activation method accurately replicates ADS steering performance, and a detailed description of the method used shall be included in the test report. The steering demand made shall be recorded in the test report alongside the results of each test.

In the case that the vehicle has full power steering in the manual driving mode, the tests described in paragraphs 6.2.6. and 6.2.7. are not required to be performed if the manufacturer can demonstrate that the steering performance available to the ADS is equivalent to that available in the manual mode.