Proposal for supplement [1] to UN Regulation No.143

(to be agreed and submitted to GRPE by TF-AVRS)

Notes:

Modifications to the existing text of the Regulation indicated in bold for new characters and strikethrough for deleted characters.

I.	Proposal

Paragraph 4.4.1., amend to read:

4.4.1.	Dual-fuel operating mode indicator:

(a) A dual-fuel engine retrofit system shall have a an visual indicationor indicating to the driver [user/operator] and/or the ADS the mode under which the engine operates (dual-fuel mode or diesel mode);

Paragraph 4.4.1.1., amend to read:

4.4.1.1.	The dual-fuel system shall warn the driver [user/operator] and/or the ADS by means of the dual-fuel mode indicationor or an audio signal (or both) when the engine operation is forced to diesel mode as required by paragraph 4.3.

Paragraph 4.4.1.2., amend to read:

4.4.1.2.	The dual-fuel mode indicationor shall be active for at least one minute on the selected mode as soon as the engine operating mode is changed from diesel to dual-fuel mode or vice-versa. This indication is also required for at least one minute at key-on, or at the request of the engine retrofit system manufacturer at engine cranking. The indication shall also be given upon the driver's request of the [user/operator] or the ADS.

Paragraph 7.1., amend to read:

7.1.	The dual-fuel engine retrofit system shall implement an OBD system that complies with the requirements for type 2B engines specified in paragraph 7. of Annex 11 to Regulation No. 49 and with the following restrictions:

(a) During diesel operation the diesel OBD system shall remain the only on board diagnostic system of the vehicle. The MI shall activate in case of a detected malfunction;

(b) During dual-fuel operation the diesel OBD system shall continue to monitor the original emission related components in use. The MI shall activate in case of a detected malfunction;

(c) During dual-fuel operation the dual-fuel ECU shall monitor the dual-fuel emission related components as well as their electrical connections. If the dual-fuel ECU detects a malfunction, the switch to diesel mode shall be performed as soon as possible. Operation in dual-fuel mode shall not be possible until the cause of the malfunction is removed. A clear visual or acoustic signal shall inform the driver [user/operator] and/or the ADS about the situation.