Proposal for supplement [12] to 03 series of amendments to UN Regulation No.24

(to be agreed and submitted to GRPE by TF-AVRS)

Notes:

Modifications to the existing text of the Regulation indicated in bold for new characters and strikethrough for deleted characters.

I.	Proposal

Paragraph 24.3.1., amend to read:

24.3.1.	The emission of visible pollutants by the vehicle type submitted for approval shall be measured by the two methods described in Annexes 4 and 5 to this Regulation, relating respectively to tests at steady speeds and to tests under free acceleration. If performing these tests on a hybrid electric vehicle or a vehicle equipped with an ADS requires a special procedure, this shall be detailed in the service manual (or equivalent media). This special procedure shall not require the use of special equipment other than that provided with the vehicle.

Annex 5, paragraph 2.4., amend to read:

2.4.	If the test is carried out on a vehicle, the gear-change control shall be set in the neutral position and the drive between engine and gear-box engaged.

Annex 5, paragraph 2.5., amend to read:

2.5.	With the engine idling, the accelerator controlengine speed demand shall be operatedincreased quickly, but not violently, so as to obtain maximum delivery from the injection pump. This position shall be maintained until maximum engine speed is reached and the governor comes into action. As soon as this speed is reached the acceleratorengine speed demand shall be releasedreduced until the engine resumes its idling speed and the opacimeter reverts to the corresponding conditions.