# Draft text proposal for including specific testing modes for ADS vehicles to UN R154

Modify par. 3.5.8. to read:

"Driveruser-selectable mode" means a distinct driveruser-selectable condition which could affect emissions, or fuel and/or energy consumption.

Add to section 3 (Definitions)

‘ADS coastdown mode’ means a mode of operation of an ADS that enables the accurate and repeatable determination of road load and dynamometer settings, including the execution of coastdown tests under ADS control.

Add to section 3 (Definitions)

‘ADS testing mode’ means a mode of operation of an ADS that enables the execution of dynamometer tests by allowing the ADS to follow prescribed driving speed profiles.

Add a new paragraph: 4.2.1.8.6 to Annex B4

Special provisions for vehicles of categories X and Y

In the case of vehicles of categories X and Y, the vehicle shall be equipped with an ADS coastdown mode capable of performing all tasks related to the road load determination procedure that would otherwise be performed by a driver.

The ADS coastdown mode shall fulfil at least the following functional requirements:

- It shall allow the vehicle to be driven to and from the test facility for the road load determination procedure either by the ADS or in manual driving mode;

- It shall allow the vehicle to be operated at the designated test facility and carry out the road load determination procedure, including at least the following tasks:

(i) Performing the vehicle warm-up procedure at a prescribed vehicle speed;

(ii) Executing coastdown runs by accelerating the vehicle to the prescribed speed and allowing it to coast down from the specified starting position on the test facility;

(iii) Ensureing, as far as technically possible, that drivetrain components are decoupled from the wheels and parasitic losses are minimised during coastdown runs;

(iv) Avoiding the use of brakes and minimising steering actions during the coastdown runs;

(v) Following operator instructions related to the conduct of the procedure (e.g. repeating runs on the basis of test evaluations, performing split runs, stopping the vehicle for safety reasons, etc.);

(vi) Performing any other task normally required from a human driver during the road load determination procedure on vehicles other than categories X and Y.

The ADS coastdown mode shall be subject to approval by the Type Approval Authority, and its use shall be recorded. At the request of the Type Approval Authority, the the correct functioning of the ADS coastdown mode shall be verified by means of checks performed on a test facility, including the monitoring of relevant on-board signals and/or the measurement of vehicle parameters.

The ADS coastdown mode shall not influence or falsify the coastdown results. If evidence of such influence is identified by the Type Approval Authority, the road load determination results shall be rejected.

If a vehicle is equipped with an ADS coastdown mode, it shall be engaged during the road load determination procedure and during testing on the chassis dynamometer.

Add a new paragraph: 2.4.4.1 to Annex B6

Special provisions for vehicles of categories X and Y

In the case of vehicles of categories X and Y the vehicle shall be equipped with an ADS testing mode capable of performing all tasks related to the test procedure on the chassis dynamometer that would otherwise be performed by a driver.

The ADS testing mode shall fulfil at least the following functional requirements:

- It shall allow the vehicle to be pushed and steered between the areas used for preconditioning, soaking and testing at the chassis dynamometer. Alternatively, manual controls may be used to drive the vehicle, but only for operations where this Regulation permits to drive the vehicle under its own power;

- It shall allow the vehicle to be operated at the designated test facility and carry out the specific driving speed profiles either by the ADS or in manual driving mode, including at least the following tasks:

(i) Initiating the powertrain start procedure;

(ii) Performing the applicable preconditioning and test cycles within the applicable speed trace tolerances;

(iii) Following operator instructions related to the conduct of the procedure (e.g. stopping the vehicle for safety reasons);

(iv) Shutting down the engine after the test;

(v) Performing any other task normally required from a human driver during the test procedure on vehicles other than categories X and Y.

The ADS testing mode shall be subject to approval by the Type Approval Authority and its use shall be recorded. At the request of the Type Approval Authority, the correct functioning of the ADS testing mode shall be verified by means of checks preformed on a chassis dynamometer including the monitoring of relevant on-board signals and/or the measurement of vehicle parameters.

The ADS testing mode shall not influence or falsify the test results. If evidence of such influence is identified by the Type Approval Authority, the test results shall be rejected.

If a vehicle is equipped with an ADS testing mode, it shall be engaged for all tests on the chassis dynamometer.

Other issues to solve:

- Can a dual mode ADS vehicle be part of the interpolation family? If so, should it be tested in ADS mode (likely with higher energy consumption) or by a human driver (no need for the manufacturer to add an ADS testing mode)

  - We can add a paragraph allowing for testing only in manual mode, if the manufacturer can demonstrate that in the ADS mode vehicle consumes less than the worst case vehicle. If not, the ADS vehicle should be tested as the worst case vehicle.

- Do we need to specify that the ADS testing mode shall follow the gear shift prescriptions, or is the combination of a manual gear shift transmission with an ADS vehicle not feasible?

  - I think we should add this, if it is not clear from the text about “following the speed profiles”. It should be technically feasible to include a manual gearshift mode in the “ADS testing mode” or to program the ADS to shift according to the provisions.

# Background information:

- ADS vehicle (cat. X and Y) should provide the possibility for coastdown testing
> provide for a specific test mode for coast-down testing (e.g. “ADS Coastdown mode”)

- Vehicle is coasted down on a test track 
> layout of the track needs to be uploaded to the vehicle

- Coastdown runs may be split at shorter tracks and need to be repeated depending on results
> allow possibility to give vehicle instructions for coastdown runs

- Prevent manipulation of coast-down test results
> need to approve the ADS coastdown mode by the TAA to ensure the validity of the test results (preferably also verify on-board signals, e.g. current from the batteries)

- There are two other road-load determination methods (torque-meter and windtunnel)
> preferably allow the ADS coastdown mode to also facilitate other methods, and at least the method used at type-approval

Suggestions for including the ADS coastdown mode

- Describe this in a dedicated paragraph applicable only to ADS vehicles of cat. X and Y

- Prepare a functional requirement for an ADS coastdown mode (what does it need to do) rather than specifying this in technical details (how it should work). 
> A functional requirement makes it easier to specify that the task of the driver in the current paragraphs on road-load determination is replaced by the ADS

- ADS coastdown mode shall also be engaged for the chassis dyno setting procedure.

Type 1 test

- ADS vehicle (cat. X and Y) should provide the possibility for Type 1 testing
> provide for a specific test mode for Type 1 testing (“ADS testing mode”)

- Vehicle can be fed with WLTC or any other driving cycle (enabling third-party testing)
> Vehicle speed cycle can be uploaded to the vehicle

- Prevent manipulation of Type-1 test results
> need to approve the ADS testing mode by the TAA to ensure the validity of the test results (avoid smoothing, possibly verify on-board signals)

- Check with SAE what they are developing for this 
(see remark Bill Coleman in meeting April 2025)

Additional comments

A few things that manufacturers and technical services may need to address in testing include:

- How to achieve the minimum number of run in kilometres

- How to achieve the warm up requirement before testing

- Ensuring the ADS does not activate the brakes

- Ensuring the ADS does not activate the steering during the coast down runs.

Other things for consideration include:

- How the ADS vehicle fits into an interpolation family – can a manufacturer test a non-ADS vehicle in the family?

- For the type 1 test, as an alternative to the ADS testing mode, a manual control mode could be used to follow the drive cycle.  This would require an operator to control the throttle and brake of the vehicle. This would remove the need for verification that the ADS testing mode is used to manipulate the tests.

  - Challenges for the type 1 test include pushing the vehicle onto the dynamometer – particularly if it doesn’t have a steering control.

- For the type 4 test, it should be noted that the vehicle must undergo part of the WLTP drive cycle on a chassis dynamometer and must be driven from the dynamometer to the shed – turning the engine off before entering the shed.  This could be difficult to program into an ADS test mode.

- For the type 5 test, the durability testing may also be undertaken on a chassis dynamometer.

- For the OBD test, we probably need to clarify what an ADS vehicle should do if an ADB warning is received. In most cases it should report to its operator that there is a defect that needs addressing, but if not addressed, it could be treated as an ODD boundary condition and prevent the vehicle from operating.

From UN R154

Par. 3.2.21 
"Vehicle coastdown mode" means a system of operation enabling an accurate and repeatable determination of road load and an accurate dynamometer setting.

Annex B4 – par. 4.2.1.8.5. Vehicle coastdown mode

If the determination of dynamometer settings cannot meet the criteria described in paragraphs 8.1.3. or 8.2.3. of this annex due to non-reproducible forces, the vehicle shall be equipped with a vehicle coastdown mode. The vehicle coastdown mode shall be approved and its use shall be recorded by the responsible authority.

If a vehicle is equipped with a vehicle coastdown mode, it shall be engaged both during road load determination and on the chassis dynamometer.