Proposal for Supplement [5] to the 07 series of amendments to UN Regulation No. 49 (Emissions of compression ignition and positive ignition (LPG and CNG) engines)

Submitted by the expert from the --- on behalf of the GRPE TF AVRS

The text reproduced below was prepared by the expert from --- on behalf of the GRPE TF AVRS, to enable the application of the regulation to vehicles equipped with an ADS. The modifications to the existing text of the UN Regulation are marked in “bold“ for new or strikethrough for deleted characters.

- Proposal

Footnote 1, amend to read:

"1 As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.3, para. 28 –

www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htmlhttps://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 2.35., amend to read:

"2.35.	"Malfunction indicationor" (MI) means an indicator which is part of the alert system and which clearly informs the driver of the vehicle and/or sends a signal to the ADS (as applicable), in the event of a malfunction;"

Paragraph 2.55. amend to read:

"2.55.	"Reference mass" means the mass of the vehicle in running order less the uniform mass of the driver of 75 kg , and increased by a uniform mass of 100 kg;

For vehicles of categories X and Y the mass of the driver shall not be subtracted;"

Paragraph 2.62., amend to read:

"2.62.	"Unladen mass" means the mass of the vehicle in running order without the uniform mass of the driver of 75 kg, passengers or load, but with the fuel tank 90 per cent full and the usual set of tools and spare wheel on board, where applicable;

For vehicles of category X and category Y the mass of the driver shall not be subtracted;"

Footnote 5, amend to read:

"3 The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.82/Amend.1 -www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htmlhttps://unece.org/transport/vehicle-regulations/wp29/resolutions”

Annex 2B, footnote 4, amend to read:

"4 Classified according to definitions listed in "Consolidated Resolution on the Construction of Vehicles (R.E.3)" - ECE/TRANS/WP.29/78/Rev.82, para.2. -

https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Annex 7, footnote 1, amend to read:

"1 As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3) - ECE/TRANS/WP.29/78/Rev.82, para.2. -

https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Annex 8, paragraph 1.2., insert new paragraph to read:

"1.2.	Special provisions for vehicles equipped with an ADS

For vehicles of categories X and Y, all tests in this annex shall be conducted, and all respective requirements shall be fulfilled. The in-service conformity test shall be conducted using dedicated methods, which may include assigning way points to define the test route, selection of different modes or any other method subject to agreement between the vehicle manufacturer and the technical service, ensuring that the test accurately reflects normal driving patterns.

For vehicles equipped with an ADS, other than those of categories X and Y, the in-service conformity test shall be performed at least in the manual driving mode. Tests do not have to be performed in ADS mode providing the manufacturer can demonstrate to the technical service that the same performance can be achieved whilst an ADS feature is active. However, testing to verify this shall be performed at the request of the technical service."

Annex 8, paragraph 2.1., amend to read:

"2.1.	The conformity of in-service vehicles or engines of an engine family shall be demonstrated by testing vehicles on the road operated over their normal driving patterns, conditions and payloads. The in-service conformity test shall be representative for vehicles operated on their real driving routes, with their normal payload and with the usual professional driver or the ADS (as applicable) of the vehicle. When the vehicle is operated by a driver other than the usual professional driver of the particular vehicle, this alternative driver shall be skilled and trained to operate vehicles of the category subject to be tested."

Annex 8 Paragraph 4.1., amend to read:

"4.1. 	Vehicle payload

Normal payload is a payload between 10 and 100 per cent of the maximum payload.

The maximum payload is the difference between technically permissible maximum laden mass of the vehicle and the mass of the vehicle in running order as specified in Annex 3 to Special Resolution No. 1 (ECE/TRANS/WP.29/1045, as amended by Amendments 1, and 2 and 3).

In case the legally permissible maximum vehicle weight is lower than the technically permissible laden mass of the vehicle, it is permitted to use the legally permissible maximum vehicle weight to determine the vehicle payload for the test run.

For the purpose of in- service conformity testing the payload may be reproduced and an artificial load may be used.

Approval authorities may request to test the vehicle with any payload between 10 to 100 per cent of the maximum vehicle payload. In case the mass of the PEMS equipment needed for operation exceeds 10 per cent of the maximum vehicle payload this mass may be considered as minimum payload."

Annex 9B, paragraph 1., amend to read:

"1.	Introduction

This annex sets out the technical requirements of on-board diagnostic (OBD) systems for the control of emissions from engine systems which are covered by this Regulation.

This annex is based on the world-wide harmonized OBD global technical regulation (gtr) No. 5.

For vehicles of categories X and Y, all [the general, performance, demonstration and documentation] requirements in this annex shall be fulfilled all corresponding tests shall be conducted. If necessary, dedicated methods may be used to conduct the tests. These may include assigning way points to define the test route, selection of different modes or any other method subject to agreement between the vehicle manufacturer and the technical service.

For vehicles equipped with an ADS, other than those of categories X and Y, the demonstration shall be performed at least in the manual driving mode. Tests do not have to be performed in ADS mode providing the manufacturer can demonstrate to the technical service that the same performance can be achieved whilst an ADS feature is active. However, testing to verify this shall be performed at the request of the technical service.

Annex 9B, paragraph 3.1., amend to read:

"3.1.	"Alert system" means a system on-board the vehicle which informs the driver of the vehicle and/or sends a signal to the ADS (as applicable) or any other interested party that the OBD system has detected a malfunction."

Annex 9B, paragraph 4., amend to read:

"4.	General requirements

In the context of this annex, the OBD system shall have the capability of detecting malfunctions, of indicating their occurrence by means of a malfunction indicator, of identifying the likely area of the malfunctions by means of information stored in computer memory, and communicating that information off-board.

The OBD system shall be designed and constructed so as to enable it to identify types of malfunctions over the complete life of the vehicle/engine. In achieving this objective, the Type Approval Authority will recognize that engines which have been used in excess of their regulatory useful life may show some deterioration in OBD system performance and sensitivity such that the OBD thresholds may be exceeded before the OBD system signals a malfunction to the driver of the vehicle or to the ADS (as applicable).

The above paragraph does not extend the engine manufacturer's compliance liability for an engine beyond its regulated useful life (i.e. the time or distance period during which emission standards or emission limits continue to apply)."

Annex 9B, paragraph 4.7.3.3., amend to read:

"4.7.3.3.	Connector location

The connector shall be located in the driver's side foot-well region of the vehicle interior in the area bound by the driver's side of the vehicle and the driver's side edge of the centre console (or the vehicle centreline if the vehicle does not have a centre console) and at a location no higher than the bottom of the steering wheel when in the lowest adjustable position. The connector may not be located on or in the centre console (i.e. neither on the horizontal faces near the floor-mounted gear selector, parking brake lever, or cup holders nor on the vertical faces near the stereo/radio, climate system, or navigation system controls). The location of the connector shall be capable of being easily identified and accessed (e.g. to connect an off-board tool). For vehicles equipped with a driver's side door, the connector shall be capable of being easily identified and accessed by someone standing (or "crouched") outside the driver's side of the vehicle with the driver's side door open.

The Type Approval Authority may approve upon request of the manufacturer an alternative location provided the installation position shall be easily accessible and protected from accidental damage during normal conditions of use, e.g. the location as described in ISO 15031 series of standards.

If the connector is covered or located in a specific equipment box, the cover or the compartment door must be removable by hand without the use of any tools and be clearly labelled "OBD" to identify the location of the connector.

The manufacturer may equip vehicles with additional diagnostic connectors and data-links for manufacturer-specific purposes other than the required OBD functions. If the additional connector conforms to one of the standard diagnostic connectors allowed in Appendix 6 to this annex, only the connector required by this annex shall be clearly labelled "OBD" to distinguish it from other similar connectors.

For vehicles of categories X and Y, all applicable provisions of this paragraph shall apply. For the purpose of this paragraph, “driver’s side” shall be understood as the side of the vehicle corresponding to the side of opposing traffic. In the case of vehicles not equipped with a steering wheel, the connector shall be positioned at a height such that it is outside the normal field of view of the occupant seated nearest to the connector."

Annex 9B, Appendix 5, Table 2, amend to read:

"Driver’s dDemanded engine torque (as a percentage of maximum engine torque)"

Annex 9C, paragraph 5.4., amend to read:

"5.4.	Requirements for incrementing the general denominator

…

(c) Continuous vehicle operation at idle (e.g., accelerator pedal released by driverno acceleration demand or engine speed demand beyond idle, and either vehicle speed less than or equal to 1.6 km/h or engine speed less than or equal to 200 min-1 above normal warmed-up idle) for greater than or equal to 30 seconds while under the conditions specified in the above subparagraph (a)."

Annex 11, paragraph 1., amend to read:

1.	Introduction

This annex sets out the requirements to ensure the correct operation of NOx control measures. It includes requirements for vehicles that rely on the use of a reagent in order to reduce emissions.

For vehicles of categories X and Y, all requirements of this annex shall be fulfilled and all respective demonstrations and tests shall be conducted. If necessary, dedicated methods may be used to conduct the demonstrations and tests. These may include assigning way points to define the test route, selection of different modes or any other method subject to agreement between the vehicle manufacturer and the technical service.

For vehicles equipped with an ADS, other than those of categories X and Y, the demonstrations and tests shall be performed at least in the manual driving mode. Tests do not have to be performed in ADS mode providing the manufacturer can demonstrate to the technical service that the same performance can be achieved whilst an ADS feature is active. However, testing to verify this shall be performed at the request of the technical service.

In the case of vehicles equipped with an ADS, whenever this annex mentions “driver” (e.g. driver inducement, driver warning, etc.), it shall be understood as an ADS feature being active. Whenever needed, special provisions regarding vehicles of categories X and Y or vehicles equipped with an ADS are detailed in the corresponding paragraphs.

Annex 11, footnote 1, amend to read:

"1 As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.83, para. 2. -

www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htmlhttps://unece.org/transport/vehicle-regulations/wp29/resolutions"

Annex 11, paragraph 2.1.2.2.3., amend to read:

"2.1.2.2.3.	The driverreagent warning system referred to in paragraph 4., 7. and 8. of this Annex shall be understood as the driver warning system in paragraph 3. of Appendix 6 to the 07 series of amendments to Regulation No. 83."

Annex 11, paragraph 2.2.3., amend to read:

"2.2.3	Detailed written information fully describing the functional operation characteristics of the driverreagent warning system as provided in accordance with paragraph 4. and of the driver inducement system as provided in accordance with paragraph 5. shall be submitted to the Type Approval Authority at the time of application for the type approval."

Annex 11, paragraph 2.4.3.1., amend to read:

"2.4.3.1.	The driverreagent warning system described in paragraph 4. shall be activated if no reagent dosing occurs at an ambient temperature ≤ 266 K (-7 °C)."

Annex 11, paragraph 3.1., amend to read:

"3.1.	The manufacturer shall furnish or cause to be furnished to all owners of new vehicles or new engines type-approved in accordance with this Regulation written instructions about the emission control system and its correct operation.

Those instructions shall state that if the vehicle emission control system is not functioning correctly, the driver or the ADS (as applicable) will be informed of a problem by the driverreagent warning system, and that operation of the driver inducement system as a consequence of ignoring this warning will result in the vehicle being unable to efficiently conduct its mission."

Annex 11, paragraph 3.7., amend to read:

"3.7.	The instructions shall explain how the warning system and driver inducement systems work. In addition, the consequences, in terms of vehicle performance and fault logging, of ignoring the warning system and not replenishing the reagent or rectifying a problem shall be explained."

Annex 11, paragraph 4., amend to read:

"4.	DriverReagent Warning System

The requirements of this section related to warning signals to the driver do not apply whilst an ADS feature is active. The following requirement shall apply instead:

Whilst an ADS feature is active, warning signals (e.g. low reagent level, incorrect reagent quality, too low a rate of reagent consumption, or a malfunction) shall be transmitted to the ADS according to the provisions specified for the visual alarms. The means by which it is ensured that existing detected faults are transmitted to the ADS before an ADS feature becomes active (e.g. previously detected warnings which remain present) shall be documented by the manufacturer."

Annex 11, paragraph 4.1., amend to read:

"4.1.	The vehicle shall include a driverreagent warning system using visual alarms that informs the driver or sends a signal to the ADS, when a low reagent level, incorrect reagent quality, too low a rate of reagent consumption, or a malfunction, has been detected that may be due to tampering and that will lead to operation of the driver inducement system if not rectified in a timely manner. The warning system shall also be active when the driver inducement system described in paragraph 5. has been activated."

Annex 11, paragraph 4.3., amend to read:

"4.3.	The driverreagent warning system may display short messages, including messages indicating clearly the following: (a) The remaining distance or time before activation of the low-level or severe inducements; (b) The level of torque reduction; (c) The conditions under which vehicle disablement can be cleared. The system used for displaying the messages referred to in this point may be the same as the one used for OBD or other maintenance purposes. In the case of vehicles equipped with an ADS, the signals may be sent to the ADS.

4.4.	At the choice of the manufacturer, the warning system may include an audible component to alert the driver (if applicable). The cancelling of audible warnings by the driver is permitted.

Annex 11, paragraphs 4.5. and 4.6., amend to read:

4.5.	The driver warning system shall be activated as specified in paragraphs 6.2., 7.2., 8.4., and 9.3. 4.6. The driverreagent warning system shall be deactivated when the conditions for its activation have ceased to exist. The driver warning system shall not be automatically deactivated without the reason for its activation having been remedied.

4.6.	The driver warning system shall be deactivated when the conditions for its activation have ceased to exist. The driverreagent warning system shall not be automatically deactivated without the reason for its activation having been remedied."

Annex 11, paragraphs 4.8. to 4.10., amend to read:

"4.8.	A facility to permit the driver (if applicable) to dim the visual alarms provided by the warning system may be provided on vehicles for use by the rescue services or on vehicles designed and constructed for use by the armed services, civil defence, fire services and forces responsible for maintaining public order.

4.9.	Details of the driverreagent warning system activation and deactivation procedures are specified in Appendix 2 to this annex.

4.10.	As part of the application for type approval under this Regulation, the manufacturer shall demonstrate the operation of the driverreagent warning system, as specified in Appendix 1 to this annex."

Annex 11, paragraphs 5. to 5.2., amend to read:

"5.	Driver Inducement system

5.1.	The vehicle shall incorporate a two-stage driver inducement system starting with a low-level inducement (a performance restriction) followed by a severe inducement (effective disablement of vehicle operation).

5.2.	The requirement for a driver inducement system shall not apply to engines or vehicles for use by the rescue services or to engines or on vehicles designed and constructed for use by the armed services, civil defence, fire services and forces responsible for maintaining public order. Permanent deactivation of the driver inducement system shall only be done by the engine or vehicle manufacturer."

Annex 11, paragraph 5.4.1., amend to read:

"5.4.1.	A "disable after restart" system shall limit the vehicle speed to 20 km/h ("creep mode") after the engine has been shut down at the request of the driver ("key-off") or the ADS, as applicable."

Annex 11, paragraph 5.5., amend to read:

"5.5.	The driver inducement system shall be enabled as specified in paragraphs 6.3., 7.3., 8.5., and 9.4."

Annex 11, paragraph 5.5.1., amend to read:

"5.5.1.	When the driver inducement system has determined that the severe inducement system shall be activated, the low-level inducement system shall remain activated until the vehicle speed has been limited to 20 km/h ("creep mode")."

Annex 11, paragraphs 5.6. to 5.8., amend to read:

"5.6.	The driver inducement system shall be deactivated when the conditions for its activation have ceased to exist. The driver inducement system shall not be automatically deactivated without the reason for its activation having been remedied.

5.7.	Details of the driver inducement system activation and deactivation procedures are described in Appendix 2 to this annex.

5.8.	As part of the application for type approval under this Regulation, the manufacturer shall demonstrate the operation of the driver inducement system, as specified in Appendix 1 to this annex."

Annex 11, paragraphs 6.1. to 6.2.1., amend to read:

"6.1.	The vehicle shall include a specific indicator on the dashboard that clearly informs the driver of the level of reagent in the reagent storage tank. Whilst an ADS feature is active, this information shall be sent to the ADS. The minimum acceptable performance level for the reagent indicator is that it shall continuously indicate the reagent level whilst the driver warning system referred to in paragraph 4. is activated to indicate problems with reagent availability. The reagent indicator may be in the form of an analogue or digital display, and may show the level as a proportion of the full tank capacity, the amount of remaining reagent, or the estimated driving distance remaining.

6.2.	Activation of the driverreagent warning system

6.2.1.	The driverreagent warning system specified in paragraph 4. shall be activated when the level of reagent is less than 10 per cent of the capacity of the reagent tank or a higher percentage at the choice of the manufacturer."

[6.2.2.	The warning provided shall be sufficiently clear for the driver to understand that the reagent level is low. When the warning system includes a message display system, the visual warning shall display a message indicating a low level of reagent. (for example, "urea level low", "AdBlue level low", or "reagent low").]

Annex 11, paragraph 6.2.3., amend to read:

"6.2.3.	The driverreagent warning system does not initially need to be continuously activated, however activation shall escalate in intensity so that it becomes continuous when the level of the reagent is approaching a very low proportion of the capacity of the reagent tank and the point where the driver inducement system will come into effect is approached. It shall culminate in a driver notification at a level that is at the choice of the manufacturer, but is sufficiently more noticeable than the point where the driver inducement system in paragraph 6.3. comes into effect."

Annex 11, paragraph 6.2.5., amend to read:

"6.2.5.	It shall not be possible to turn off the driverreagent warning system until the reagent has been replenished to a level not requiring its activation."

Annex 11, paragraph 7.2., amend to read:

"7.2.	Activation of the driverreagent warning system. When the monitoring systems detects or, as appropriate, confirms that the reagent quality is incorrect, the driver warning system described in paragraph 4. shall be activated. When the warning system includes a message display system, it shall display a message indicating the reason for the warning (for example: "incorrect urea detected", "incorrect AdBlue detected", or "incorrect reagent detected")."

Annex 11, paragraph 7.3., amend to read:

"7.3.	Activation of the driver inducement system

7.3.1.	The low-level inducement system described in paragraph 5.3. shall be enabled, and subsequently activated according to the requirements of that section, if the reagent quality is not rectified within 10 engine operating hours after the activation of the driver warning system described in paragraph 7.2.

7.3.2.	The severe inducement system described in paragraph 5.4. shall be enabled, and subsequently activated according to the requirements of that section, if the reagent quality is not rectified within 20 engine operating hours after the activation of the driver warning system described in paragraph 7.2."

Annex 11, paragraph 8.4., amend to read:

"8.4.	Activation of the driverreagent warning system

8.4.1.	The driverreagent warning system described in paragraph 4. shall be activated if a deviation of more than fifty per cent between the average reagent consumption and the average demanded reagent consumption by the engine system over a period to be defined by the manufacturer, which shall not be longer than the maximum period defined in paragraph 8.3.1., or, when applicable, paragraph 8.3.1.1., is detected. When the warning system includes a message display system, it shall display a message indicating the reason for the warning (for example: "urea dosing malfunction", "AdBlue dosing malfunction", or "reagent dosing malfunction.

8.4.2.	The driverreagent warning system described in paragraph 4. shall be activated in the case of interruption in reagent dosing. When the warning system includes a message display system, it shall display a message indicating an appropriate warning. This activation shall not be required where the interruption is demanded by the engine ECU because the vehicle operating conditions are such that the vehicle’s emission performance does not require reagent dosing."

Annex 11, paragraph 8.5., amend to read:

"8.5.	Activation of the driver inducement system

8.5.1.	The low-level inducement system described in paragraph 5.3. shall be enabled, and subsequently activated according to the requirements of that section, if an error in the reagent consumption or an interruption in reagent dosing is not rectified within 10 engine operating hours after the activation of the driver warning system specified in paragraphs 8.4.1. and 8.4.2.

8.5.2.	The severe inducement system described in paragraph 5.4. shall be enabled, and subsequently activated according to the requirements of that section, if an error in the reagent consumption or an interruption in reagent dosing is not rectified within 20 engine operating hours after the activation of the driver warning system in paragraphs 8.4.1. and 8.4.2."

Annex 11, paragraph 9.3., amend to read:

"9.3.	Activation of the driverreagent warning system

The driverreagent warning system described in paragraph 4. shall be activated in the case where any of the failures specified in paragraph 9.1. occurs, and shall indicate that an urgent repair is required. When the warning system includes a message display system, it shall display a message indicating either the reason for the warning (for example, "reagent dosing valve disconnected", or "critical emission failure")."

Annex 11, paragraph 9.4. and subparagraphs, amend to read:

"9.4.	Activation of the driver inducement system

9.4.1.	The low-level inducement system described in paragraph 5.3. shall be enabled, and subsequently activated according to the requirements of that section, if a failure specified in paragraph 9.1. is not rectified within 36 engine operating hours after the activation of the driverreagent warning system in paragraph 9.3.

9.4.2.	The severe inducement system described in paragraph 5.4. shall be enabled, and subsequently activated according to the requirements of that section, if a failure specified in paragraph 9.1. is not rectified within 100 engine operating hours after the activation of the driverreagent warning system in paragraph 9.3."

Annex 11, Appendix 1, paragraph A 1.1.1. amend to read:

"A 1.1.1.	The manufacturer shall submit to the Type Approval Authority a complete documentation package justifying the compliance of the SCR system with the requirements of this annex as regards its capabilities for monitoring and activation of the driverreagent warning and inducement system, which may include:"

Annex 11, Appendix 1, paragraph A 1.4.1.1. amend to read:

"A 1.4.1.1. 	Any additional vehicle components or sub-systems, such as ambient temperature sensors, level sensors, and driverreagent warning and information systems, that are required in order to perform the demonstrations shall be connected to the engine system for that purpose, or shall be simulated, to the satisfaction of the Type Approval Authority."

Annex 11, Appendix 2, paragraph A 2.1. amend to read:

"A 2.1.	Activation and deactivation mechanisms of the driverreagent warning system"

Annex 11, Appendix 2, paragraph A 2.2. amend to read:

"A 2.2.	Activation and deactivation mechanisms of the driverreagent warning system

A 2.2.1.	The driverreagent warning system shall be activated when the diagnostic trouble code (DTC) associated with a malfunction justifying its activation has the status defined in Table 1."

Table 1

Activation of the driverreagent warning system”

Annex 11, Appendix 2, paragraph A 2.2.1.1. amend to read:

"A 2.2.1.1. 	If the counter associated with the relevant failure is not at zero, and is consequently indicating that the monitor has detected a situation where the malfunction may have occurred for a second or subsequent time, the driverreagent warning system shall be activated when the DTC has the status "potential"."

Annex 11, Appendix 2, paragraph A 2.2.2. amend to read:

"A 2.2.2.	The driverreagent warning system shall be deactivated when the diagnostic system concludes that the malfunction relevant to that warning is no longer present or when the information, including DTCs relative to the failures, justifying its activation is erased by a scan tool."

Annex 11, Appendix 2, paragraph A 2.2.2.1.1. amend to read:

"A 2.2.2.1.1. Erasing of information, including DTCs relative to failures justifying the activation of a driverreagent warning signal and of their associated data, by means of a scan tool shall be performed in accordance with Annex 9B. "

Annex 11, Appendix 2, paragraph A 2.3.1. amend to read:

"A 2.3.1.	Activation and deactivation mechanism of the driver inducement system

The driver inducement system shall be activated when the warning system is active and the counter relevant to the type of malfunction justifying its activation has reached the value specified in Table 2."

Annex 11, Appendix 2, paragraph A 2.3.2. amend to read:

"A 2.3.2.	The driver inducement system shall be deactivated when the system no longer detects a malfunction justifying its activation, or if the information, including the DTCs, relative to the failures justifying its activation has been erased by a scan tool or maintenance tool."

Annex 11, Appendix 2, paragraph A 2.3.3. amend to read:

"A 2.3.3.	The driver warning and inducement systems shall be immediately activated or deactivated as appropriate in accordance with the provisions of paragraph 6. of this annex after assessment of the reagent quantity in the reagent tank. In that case, the activation or deactivation mechanisms shall not depend upon the status of any associated DTC. "

Annex 11, Appendix 2, paragraph A 2.5.2. amend to read:

"A 2.5.2.	Figure 2 illustrates the operation of the activation and deactivation mechanisms when monitoring the reagent availability for five cases:

(a) Use case 1: the driver continues operating the vehicle is operated in spite of the warning until vehicle operation is disabled;

(b) Repair case 1 ("adequate" refilling): the driver refills the reagent tank is refilled so that a level above the 10 per cent threshold is reached. Warning and inducement are de-activated;

(c) …"

Annex 11, Appendix 2, paragraph A 2.5.3. amend to read:

“A 2.5.3.	Figure 3 illustrates three cases of wrong urea quality:

(a) Use case 1: the driver continues operating the vehicle is operated in spite of the warning until vehicle operation is disabled;

(b) Repair case 1 ("bad" or "dishonest" repair): after disablement of the vehicle, the driver changes the quality of the reagent is changed, but, soon after, changes it is changed again for a poor quality one. The inducement system is immediately reactivated and vehicle operation is disabled after 2 engine operating hours;

(c) Repair case 2 ("good" repair): after disablement of the vehicle, the driver rectifies the quality of the reagent is rectified. However, some time afterwards, he refills it is refilled again with a poor quality reagent. The warning, inducement, and counting processes restart from zero."

Annex 14, paragraph 4.3.1. amend to read:

"4.3.1.	Dual-fuel operating mode indicator

Dual-fuel engines and vehicles shall provide to the driver a visual indication to the driver or a signal to the ADS (as applicable) of the mode under which the engine operates (dual-fuel mode, diesel mode, or service mode)."

Annex 14, paragraph 4.3.1.2. amend to read:

"4.3.1.2. 	The dual-fuel mode indicator shall be set for at least one minute on dual-fuel mode or diesel mode as soon as the engine operating mode is changed from diesel to dual-fuel mode or vice-versa. This indication is also required for at least one minute at key-on, or at the request of the manufacturer at engine cranking. The indication shall also be given upon the driver’sa request by the driver or the ADS."

Annex 14, paragraph 4.3.2. amend to read:

"4.3.2.	A dual-fuel vehicle shall be equipped with a dual-fuel warning system that alerts the driver or the ADS (as applicable) that the gaseous fuel tank will soon become empty."

Annex 14, paragraph 4.3.2.1. amend to read:

"4.3.2.1.	Characteristics of the dual-fuel warning system

The dual-fuel warning system  shall consist of a visual alert system to the driver (icon, pictogram, etc.) left to the choice of the manufacturer, or a signal to the ADS (if applicable). It may include, at the choice of the manufacturer, an audible component. In that case, the cancelling of that component by the driver is permitted.

...

A facility to permit the driver to dim the visual alarms provided by the warning system may be provided on vehicles for use by the rescue services or on vehicles designed and constructed for use by the armed services, civil defence, fire services and forces responsible for maintaining public order."

II.		Justification

This Regulation is amended to enable the type approval of vehicles of category X and Y and other vehicles equipped with an ADS.

The Regulation was originally drafted for vehicles with driver and manual driving controls, and it is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and manual driving controls. In the absence of driver/manual driving controls, provisions related to them shall not be taken into account if not already covered by this amendment.

In case of vehicles equipped with an Automated Driving System (ADS) other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply. In case an ADS feature is active the relevant ADS requirements have to be fulfilled.