|  | United Nations | ECE/TRANS/WP.29/GRPE/2026/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General XX December 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Ninety fourth session

Geneva, 17-19 March 2026

Item 4(a) of the provisional agenda

UN Regulation No. OBM

Proposal for supplement 1 to the original version of UN Regulation No. OBM (Uniform provisions concerning the approval of light duty passenger and commercial vehicles with regards to On-board monitoring (OBM) systems, environmental vehicle passport (EVP) and in-vehicle display of environmental data)

Submitted by the experts from the Task Force AVRS

This document proposes a new Supplement to the original version of UN Regulation No. OBM. The new supplement intends to bring this Regulation up to date in the sense of provisions for automated vehicles and automated driving systems. The modifications to the current text of the Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Paragraph 2., amend to read:

- "2.	Abbreviations

General abbreviations

ADS	Automated Driving System

HEV	Hybrid-Electric Vehicle

ICE	Internal Combustion Engine

…

### Paragraph 3.2., amend to read:

"3.2.	"OBM trip" means each period of vehicle operation that starts with the activation of the propulsion system and ends with the deactivation of the propulsion system. For the purpose of determining the end of an OBM trip, sequences of engine shut-off events followed by an engine re-start commanded by the vehicle control system (as a result of the operation of stop-start systems, hybrid vehicle control or automatic stall recovery) shall not be considered a deactivation of the propulsion system. For vehicles of categories X and Y, an OBM trip means the period of operation between activation of the ADS and deactivation of the ADS;"

Add new paragraph 3.22., to read:

"3.22.	"Driver Warning System" means an Excess Exhaust Emissions Driver Warning System (EEEDWS) as described in Annex 5."

Annex 4, paragraph 2.5., amend to read:

#### "2.5.	OBM trip odometer (parameters 2.1 and 2.2)

#### The OBM system shall provide a ‘Vehicle odometer value’ and an ‘OBM trip distance’ value to indicate both the total driven distance in the vehicle lifetime, as indicated to the driver (if applicable) and the total driven distance during the OBM trip at the end of the OBM trip. These odometer values shall use the same data source as the odometer value displayed to the driver (if applicable). "

Annex 4, paragraph 2.5., amend to read:

#### "2.6.2.	For the purpose of calculating this parameter, idle shall mean that there accelerator pedal is released by the driver is no acceleration demand and either the vehicle speed is less than or equal to one km per hour or engine speed is less than or equal to 200 rpm above normal warmed- up idle (as determined in the drive position for vehicles equipped with an automatic transmission)."

Annex 4, Appendix 1., table lines 1.2. and 1.43., amend to read:

# "OBM parameters to support vehicle testing

| Reference | Name | Description | Required for Instantaneous data stream |
| --- | --- | --- | --- |
| General Parameters |  |  |  |  |
| … |  |  |  |
| 1.2 | MI status | Malfunction Indicationor status | Yes |
| … |  |  |  |
| 1.43 | Accelerationor pedal position demand | Relative or "learned" pedal position or acceleration demand shall be displayed as a normalised value | Yes |
| …" |  |  |  |

Annex 5, insert new paragraph 1.2., to read:

"1.2	For vehicles of categories X and Y, requirements for driver warnings to be displayed shall be read as a requirement for a signal to be sent to the ADS. Requirements for visual or audible alarms are not applicable.

#### For vehicles equipped with an ADS other than categories X and Y, and whilst an ADS feature is active, a requirement for a driver warning to be displayed shall be read as a requirement for a signal sent to the ADS."

Annex 5, paragraph 2.4.1.3., amend to read:

"2.4.1.3.	The requirement for the inducement method shall not apply to vehicles designed and constructed for use by the rescue services, armed services, civil defence, fire services and forces responsible for maintaining public order. Permanent deactivation of the driverreagent inducement system for these vehicles shall only be done by the vehicle manufacturer."

- Justification

- At its 190th session in June 2023, WP.29 endorsed the report (ECE/TRANS/WP.29/2023/86) transmitted by the expert groups on regulatory fitness for automated vehicles and invited the GRs to start the work on amending the regulations identified by the expert groups in the report.

- …