| # CLEPA SEG EMC & OICA EMC TF # Change Proposal Form (One major comment per form.) (Shaded blocks for the use by the IWG Secretary only.) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IWG document Number: |  | IWG-EMC-47-09-Rev.1 |  |  | Date: 23rd June 2025 |  |  |
| Proposer’s Name, Affiliation, and E-mail: OICA (VDA) Andreas Gierstorfer, BMW, andreas.gierstorfer@bmw.de CLEPA Andrea Scholl, Bosch, Andreamarie.scholl@de.bosch.com |  |  |  |  | Paragraph: Appendix 1 |  | Page: |
| Summary of Change (25 words or less): Update of references to ISO 11451-1, 11452-1 and 11451-2 for vehicle and ESA radiated immunity tests. The new editions of all three standards have been published on June 13th, 2025. |  |  |  |  |  |  |  |
| Reason for Change (Justification): Regarding ISO 11451-1 and 11452-1, the modulations and frequency ranges to be tested with these modulations are already fully in line with R10.07. Additionally, the update of requirements for different types of artificial networks (AN, HV-AN, DC-charging-AN, AMN and AAN) is also already in line with R10.07. Regarding ISO 11451-2, the calibration and verification methods have been improved to achieve better field uniformity: - 4 field probes (instead of single field probe) in the frequency range 2 to 6 GHz - Verification of frontal and lateral field uniformity |  |  |  |  |  |  |  |
| Original text: 6. ISO 11451 “Road vehicles – Vehicle test methods for electrical disturbances from narrowband radiated electromagnetic energy”: Part 1: General principles and terminology (ISO 11451-1, Fourth edition 2015); Part 2: Off-vehicle radiation sources (ISO 11451-2, Fourth edition 2015); Part 4: Harness excitation methods (ISO 11451-4, Fourth edition 2022); 7. ISO 11452 “Road vehicles – Component test methods for electrical disturbances from narrowband radiated electromagnetic energy”: Part 1: General principles and terminology (ISO 11452-1, Fourth edition 2015); |  |  |  |  |  |  |  |
| Revise To: 6. ISO 11451 “Road vehicles – Vehicle test methods for electrical disturbances from narrowband radiated electromagnetic energy”: Part 1: General principles and terminology (ISO 11451-1, Fifth edition 2025); Part 2: Off-vehicle radiation sources (ISO 11451-2, Fifth edition 2025); Part 4: Harness excitation methods (ISO 11451-4, Fourth edition 2022); 7. ISO 11452 “Road vehicles – Component test methods for electrical disturbances from narrowband radiated electromagnetic energy”: Part 1: General principles and terminology (ISO 11452-1, Fifth edition 2025); |  |  |  |  |  |  |  |
| As Modified Text: |  |  |  |  |  |  |  |
|  | Accepted As Written |  |  | Withdrawn |  |  |  |
| X | Accepted As Modified |  |  | Rejected |  |  |  |
|  | Deferred |  |  | Other |  |  |  |
| Rejection Reason / Comments: |  |  |  |  |  |  |  |
| Proposal Deferred To: |  |  |  |  |  |  |  |
| Proposal Disposition By: CLEPA |  |  |  |  |  | Date: 24/06/2025 |  |