Draft proposal to no longer exclude ESD from R.10 (08)

Insert a new paragraph 6.5., to read:

# “6.5.	Specifications concerning immunity of vehicles to Electrostatic discharge.”

6.5.1. 	Method of testing

The method described in ISO 10605: 2023 shall be used as the method of measurement of the vehicle in areas where an ESD in standard use is possible (e.g. by touching by the driver or occupants during driving, entry into or exit from the vehicle). The vehicle shall be tested in brake mode.

6.5.2.	Vehicle immunity type approval limits

The test level and function performance status classification (FPSC) according to the table below applies for test points accessible from inside and outside of the vehicle.

TABLE

| Discharge type | Immunity test level | Function performance status classification (FPSC) |  |
| --- | --- | --- | --- |
|  |  | Related to immunity related functions | Not related to immunity related functions |
| Contact | +/- 4 kV | I | II |
| Air | +/- 8 kV | I | II |

”

Renumber following paragraphs accordingly

Paragraph 6.10.4., delete.

6.10.4.	Electrostatic discharge

For vehicles fitted with tyres, the vehicle body/chassis can be considered to be an electrically isolated structure. Significant electrostatic forces in relation to the vehicle's external environment only occur at the moment of occupant entry into or exit from the vehicle. As the vehicle is stationary at these moments, no type approval test for electrostatic discharge is deemed necessary.

Renumber following paragraphs accordingly

JUSTIFICATION

The exemption in current par.6.10.4., which states that ESD testing is deemed not necessary, dates back from older times (was par. 6.8.4 in R.10-02). For the convential vehicles at that time this assumption was probably justified.

However, technology have evolved over the past decades, leading to the application of more and more electronics, controllers, etc. in modern vehicles. Also electrical vehicles are strongly emerging. We believe that the exemption for ESD is no longer justified.

As an historic analogy: initially specifications for testing of radiation (par. 6.2.) were carried out on vehicles with spark ignition. Due to technological development (more modern diesel engines), the limitation to vehicles with spark ignition was deleted.

There are many use cases in which ESD could occur. Not only entering/exiting vehicle, but also e.g. during service and maintenance work on an electric vehicle. Risk reduction must always be resolved technically first, before moving on to warnings and personal protective equipment.

In our view ESD testing should be included in R.10 for safety reasons. Not only comfort related systems, but also safety related systems that are electronically controlled, fitted to vehicles are increasing in number. For today’s hardware ESD may not directly be a “chip killer”, however, it can certainly cause a disturbance or malfunction of electronically controlled systems. It must be ensured that modern vehicles are sufficiently robust in this respect and therefore ESD testing of the vehicle should be included in R.10.

As many vehicle manufacturers probably already apply the ISO 10605 standard, therefore the additional burden is deemed minimal.