|  | United Nations | ECE/TRANS/WP.29/GRSP/2025/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 17 September 2024 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Seventy-seventh session

Geneva, 5–9 May 2025

Item Y of the provisional agenda

UN Regulation No. 145 (ISOFIX anchorage systems, ISOFIX top tether anchorages and i-Size seating positions)

Proposal for supplement 2 to the 01 Series of Amendments to UN Regulation No. 145 (ISOFIX anchorage systems, ISOFIX top tether anchorages and i-Size seating positions) *

Submitted by the GRSP Task Force AVRS (Automated Vehicle Regulation Screening)

The text reproduced below was prepared by the expert from Germany on behalf of the TF GRSP AVRS, to enable the application of the regulation to vehicles equipped with an ADS [feature type 2]. The modifications to the existing text of the UN Regulation are marked in “bold“ for new or strikethrough for deleted characters.

- Proposal

Insert a new paragraph 0, to read:

"0.	Introduction

  - For supplement 2 to the 01 Series of Amendments:

0.1.1.	The Regulation is amended to account for vehicles of category X.

0.1.2. 	The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and without manual driving controls inside the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 	In case of vehicles with an ADS where those vehicles are also equipped with a manual driving mode operating at a speed of more than 6 km/h it is expected that in the manual driving mode the technical requirements can be applied as they would for a conventional vehicle. In a mode where an ADS feature is active the relevant ADS requirements have to be fulfilled.

Paragraph 1, amend to read:

1.1.	Scope

This Regulation applies to:

(a)		Vehicles of category M11 with regard to their ISOFIX anchorage systems and their ISOFIX top tether anchorages intended for child restraint systems. Other categories of vehicles fitted with ISOFIX anchorages have also to comply with the provisions of this Regulation;

(b)		Vehicles of any category with regard to their i-Size seating positions, if any are defined by the vehicle manufacturer.

This regulation does not apply to vehicles of category Y

Insert new footnote 1, to read:

"1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.8, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 2.5. amend to read:

2.5.	"A front passenger seat" means any seat where the "foremost H-point" of the seat in question is in or in front of the vertical transverse plane through the driver's R-point. For vehicles of category X1, the "R" point of the most forward passenger seat shall be taken instead of the R-point of the driver's seat.

Paragraph 2.15. amend to read:

2.15. 	"Anti-rotation device":

(a)	An anti-rotation device for an ISOFIX universal child restraint system consists of the ISOFIX top-tether;

(b)	An anti-rotation device for an ISOFIX semi-universal child restraint system consists of either a top tether, the vehicle dashboard or, a support leg or any other means intended to limit the rotation of the restraint during a frontal impact;

Paragraph 4.4.1., former footnote 1, renumber as footnote 2.

Paragraph 5.3.1. amend to read:

5.3.1.	Any vehicle of category M1 shall be equipped at least with two ISOFIX positions which satisfy the requirements of this Regulation.

At least two of the ISOFIX positions shall be equipped both with an ISOFIX anchorages system and an ISOFIX top tether anchorage.

The type and number of ISOFIX fixtures, defined in UN Regulations No. 16 and/or No. 173, which can be installed on each ISOFIX position are defined in UN Regulations No. 16. and/or No. 173.

Paragraph 5.3.3 amend to read:

5.3.3.	Notwithstanding paragraph 5.3.1. at least one of the two ISOFIX positions systems shall be installed at the second seat row.

II.		Justification

See paragraph 0

*	In accordance with the programme of work of the Inland Transport Committee for 2023 as outlined in proposed programme budget for 2023 (A/77/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.