|  | United Nations | ECE/TRANS/WP.29/GRSP/2025/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 17 September 2024 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Seventy-seventh session

Geneva, 5–9 May 2025

Item Y of the provisional agenda

UN Regulation No. 16 (Safety-belts, ISOFIX and i-Size)

Proposal for supplement 2 to the 9 Series of Amendments to UN Regulation No. 16 (Safety-belts, safety-belt reminders, restraint	systems, child restraint systems) *

Submitted by the expert from Germany on behalf of the GRSP TF AVRS

The text reproduced below was prepared by the expert from Germany on behalf of the TF GRSP AVRS, to enable the application of the regulation to vehicles equipped with an ADS[F-2]. The modifications to the existing text of the UN Regulation are marked in “bold“ for new or strikethrough for deleted characters.

- Proposal

Insert a new paragraph 0, to read:

"0.	Introduction

  - For supplement 2 to the 09 Series of Amendments:

    - The Regulation is amended to account for vehicles of category X. The scope is updated to reflect that this Regulation does not apply to vehicles of category Y, since no occupants are inside the vehicle under any circumstances.

0.1.2.	The Regulation was originally drafted for vehicles with driver and manual driving controls and it is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and manual driving controls. In the absence of driver/manual driving controls, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 	In case of vehicles with an ADS where those vehicles are also equipped with a manual driving mode operating at a speed of more than 6 km/h it is expected that in the manual driving mode the technical requirements can be applied as they would for a conventional vehicle. In a mode where an ADS feature is active the relevant ADS requirements have to be fulfilled.

Paragraph 1., amend to read:

1.		Scope

This Regulation applies to:

1.1.	…

1.7.	At the request of the manufacturer, it also applies to installing i-Size child restraint systems, in case i-Size seating positions are defined by the vehicle manufacturer.

This Regulation does not apply to vehicles of category Y.

Paragraph 1, footnote 1, amend to read:

"1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.78, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 2.18.1., amend to read:

2.18.1.	“A front passenger seat" means any seat where the "foremost H-point" of the seat in question is in or in front of the vertical transverse plane through the driver's R-point. For vehicles of category X, the "R" point of the most forward passenger seat shall be taken instead of the R-point of the driver's seat.

Paragraph 2.41., amend to read:

2.41.		"Safety-belt reminder", means a system dedicated to alert the driver when any of the occupants do not use the safety-belt. The system is constituted by a detection of an unfastened safety-belt and by two levels of driver's alert: a first level warning and a second level warning.

[Insert new paragraphs 2.55. to 2.56., to read:

"2.55.	"ADS feature of type 1 (ADSF-1)" means an ADS1 feature which includes an ADS fallback response requiring a fallback user

2.56.	“ADS feature of type 2 (ADSF-2)” means an ADS feature which does not include an ADS fallback response requiring a fallback user."]

[Insert a new paragraph 6.1.5., to read:

“6.1.5.	An ADS may control the safety belt equipment [safety belt reminder equipment] provided the ADS complies with the technical requirements of UN Regulation No. [XXX]”]

Paragraph  6.2.5.3.1.5., amend to read:

6.2.5.3.1.5.	In cases where the operation of a retractor depends on an external signal or power source, the design shall ensure that the retractor locks automatically upon failure or interruption of that signal or power source. However, this requirement need not be met in the case of a retractor with multiple sensitivities, provided only one sensitivity is dependent on an external signal or power source and the failure of the signal or power source is indicated to the driver by optical and/or acoustical means or whilst an ADS[F-2] [feature] is active, transmitted as a logic signal to the ADS.

Paragraph 6.4.1.4.1.1., amend to read:

6.4.1.4.1.1.	In the case of an occupant having a steering control in front the driver, contact of the chest with the steering control assembly would be allowed, if the latter meets the requirements of UN Regulation No. 12 and provided contact does not occur at a speed higher than 24 km/h. For this assessment the seat shall be considered to be in the positions specified in paragraph 7.7.1.5. below.

Paragraph 7.10.1., amend to read:

7.10.1.	The test report shall record the results of all the tests in paragraph 7. above and in particular:

(a)	The type of device used for the test (acceleration or deceleration device);

(b)	The total velocity change;

(c)	The trolley speed immediately before impact only for deceleration trolleys;

(d)	The acceleration or deceleration curve during all the velocity change of the trolley;

(e)	The maximum forward displacement of the manikin;

(f)	The place – if it can be varied – occupied by the buckle during the test;

(g)	The buckle-opening force;

(h)	Any failure or breakage;

(i)	For any restraint system other than the one for the driver the contour of the contact should be evaluated versus the movement of the manikin in the sled by the technical service in this report.

Insert new paragraph 8.1.10., to read:

8.1.11.	In case of vehicles where an ADS feature is active, a seating position with the steering control placed in front, may be designated as suitable for the installation of a Child Restraint System, provided all requirements for a passenger seating position as defined in paragraphs 8.1.9.2. to 8.1.9.4. are fulfilled.

Paragraph 8.4.1.1., amend to read:

8.4.	Safety-belt reminder equipment

8.4.1.	Requirements per specific seating position and exemptions

The seating position of the driver (if applicable) of M and N categories of vehicles3 as well as and the seating positions of the front passengers occupants of seats in the same row as the driver seat of M and N categories of vehicles shall be equipped with a safety-belt reminder satisfying the requirements of paragraph 8.4.3.

Paragraph 8.4.2.1.1., amend to read:

8.4.2.1.1.	The visual warning shall be so located as to be readily visible and recognisable in the daylight and at night time by the driver and distinguishable from other alerts. Whilst an ADS[F-2] [feature] is active, the warning shall be transmitted as a logic signal to the ADS.

Paragraph 8.4.2.2.2., amend to read:

8.4.2.2.2.	The aAudible warning shall be easily recognized by the driver (if applicable). Whilst an ADS[F-2] [feature] is active, the audible warning shall be easily recognisable by the passengers.

Paragraph 8.4.3. amend to read:

8.4.3.		Safety-belt reminder for the seating position of the driver (if applicable) and the seating positions of the front passengers occupants of seats in the same row as the driver.

Paragraph 8.4.3.1. amend to read:

8.4.3.1.	Safety-belt reminders for the seating position of the driver (if applicable) and the seating positions of the front passengers occupants of seats in the same row as the driver shall fulfil the requirements set out in paragraph 8.4.2.

Paragraph 8.4.4.2. amend to read:

8.4.4.2.	The visual warning shall indicate at least all rear seating positions to allow the driver to identify, while facing forward as seated on the driver seat, any seating position in which the safety-belt is unfastened. Whilst an ADS[F-2] [feature] is active, the warning shall be transmitted as a logic signal to the ADS.  For vehicles that have information on the occupancy status of the rear seats, the visual warning does not need to indicate unfastened safety-belts for unoccupied seating positions. For seats, which can be fixed to different designated seating positions within the vehicle (e.g. floor rail mounted), the visual warning shall at least indicate when any rear safety belt is unfastened.

Annex 9, paragraph 3., amend to read:

3.		In the case of safety-belts fitted with a type 4N retractor, it shall be indicated in the installation instructions and on any packaging that this belt is not suitable for installation in motor vehicles used for the carriage of passengers having not more than nine seats, including that of the driver.

Annex 16, amend to read:

| Minimum requirements for safety-belts and Retractors |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle category | Forward facing seating positions |  |  |  |  |  | Rearward-facing seating positions | Side-facing seating position |
|  | Outboard seating positions |  |  | Centre seating position |  |  |  |  |
|  | Front |  | Other than front | Front |  | Other than front |  |  |
| M1 | Ar4m |  | Ar4m | Ar4m |  | Ar4m | B, Br3, Br4m | - |
| M2 < 3.5 t | Ar4m, Ar4Nm |  | Ar4m, Ar4Nm | Ar4m, Ar4Nm |  | Ar4m, Ar4Nm | Br3, Br4m, Br4Nm | - |
| M2 > 3.5 t | Br3, Br4m, Br4Nm, or Ar4m or Ar4Nm ● |  | Br3, Br4m, Br4Nm, or Ar4m or Ar4Nm ● | Br3, Br4m, Br4Nm or Ar4m or Ar4Nm ● |  | Br3, Br4m, Br4Nm or Ar4m or Ar4Nm ● | Br3, Br4m, Br4Nm | - |
| M3 | Br3, Br4m, Br4Nm, or Ar4m or Ar4Nm ● See para. 5.1.7. for conditions when a lap belt is permitted |  | Br3, Br4m, Br4Nm, or Ar4m or Ar4Nm ● See para. 5.1.7. for conditions when a lap belt is permitted | Br3, Br4m, Br4Nm or Ar4m or Ar4Nm ● See para. 5.1.7. for conditions when a lap belt is permitted |  | Br3, Br4m, Br4Nm or Ar4m or Ar4Nm ● See para. 5.1.7. for conditions when a lap belt is permitted | Br3, Br4m, Br4Nm | B, Br3, Br4m, Br4Nm |
| N1 | Ar4m, Ar4Nm |  | Ar4m, Ar4Nm, Br4m, Br4Nm Ø | B, Br3, Br4m, Br4Nm or A, Ar4m, Ar4Nm* |  | B, Br3, Br4m, Br4Nm | B, Br3, Br4m, Br4Nm | - |
|  |  |  | Para. 5.1.2.1. lap belt permitted if seat is inboard of a passageway | Para. 5.1.6. lap belt permitted if the windscreenZ is not in the reference zone |  |  |  |  |
| N2 | Br3, Br4m, Br4Nm or Ar4m, Ar4Nm* Para. 5.1.6. lap belt permitted if the windscreenZ is outside the reference zone and/or for the driver's seat for a seat having a steering control in front |  | B, Br3, Br4m, Br4Nm | B, Br3, Br4m, Br4Nm, or A, Ar4m, Ar4Nm* Para. 5.1.6. lap belt permitted if the windscreenZ is not in the reference zone |  | B, Br3, Br4m, Br4Nm | B, Br3, Br4m, Br4Nm | - |
| N3 | Br3, Br4m, Br4Nm or Ar4m, Ar4Nm* Para. 5.1.6. lap belt permitted if the windscreenZ is outside the reference zone and/or for the driver's seat for a seat having a steering control in front |  | B, Br3, Br4m, Br4Nm | B, Br3, Br4m, Br4Nm, or A, Ar4m, Ar4Nm* Para. 5.1.6. lap belt permitted if the windscreenZ is not in the reference zone |  | B, Br3, Br4m, Br4Nm | B, Br3, Br4m, Br4Nm | - |
| A: three-point (lap and diagonal) belt 3: automatically locking retractor |  | B: 2-point (lap) belt 4: emergency locking retractor |  |  | r: retractor N: higher response threshold |  | m: emergency locking retractor with multiple sensitivity |  |
| *: Refers to para. 5.1.6. of this Regulation |  | Ø: Refers to para. 5.1.2.1. of this Regulation |  |  | ●: refers to para. 5.1.7. of this Regulation Z: For the purpose of this regulation: In case of vehicles of category X, the front exterior forward facing glazing or surface is deemed to be a windscreen |  | (see paras. 2.12.3. and 2.12.5. of this Regulation) . |  |

Note a:	In all cases all S-type belts may be fitted in place of all possible A or B type belts, provided their anchorages comply with UN Regulation No. 14.

Where a harness belt has been approved as a S-type belt according to this Regulation, using the lap belt strap, the shoulder belt straps and possibly one or more retractors, one or two additional crotch straps including their attachments for their anchorages may be provided by the manufacturer/applicant. These additional anchorages need not meet the requirements of UN Regulation No. 14

Note b: For M2 and M3 vehicles of all classes, forward facing Seats facing Built in Child Restraint Systems shall be equipped with at least Ar seat belts. "

Annex 18 1.(e) amend to read:

1.(e)		A load of 40 kg is placed on each seat cushion for the seating position of the driver (if applicable) and the seating positions of  front passengers occupants of seats in the same row as the driver's seat, or the state in which occupants are on board the vehicle is simulated by an alternative method specified by the vehicle manufacturer, provided an occupant’s load does not exceed 40 kg. This may also be done for the rear seats at the request of the vehicle manufacturer; Or alternatively (at the choice of the manufacturer):

An object or human representing a 5th percentile adult female is placed on each seat cushion as specified by the manufacturer for the seating position of the driver (if applicable) and the seating positions of front passengers occupants of seats in the same row as the driver seat, or the state in which occupants are on board the vehicle is simulated by an alternative method specified by the vehicle manufacturer as agreed by the Technical Service and the Type Approval Authority. This may also be done for the rear seats at the request of the vehicle manufacturer.

Annex 18, 2.2. amend to read:

2.2.		Testing the seating position(s) in the same row as the driver's seat of front passengers.

2.2.1.		Testing the seat(s) in the same row as the driver's seat of front passengers when the safety-belt is unfastened before the journey:

(a) The safety-belt(s) of the seat(s) in the same row as the driver's seat of front passengers is/are not fastened;

(b) The safety-belts of the seats other than the seat(s) in the same row as the driver's seat of front passengers are fastened;

(c) A load of 40 kg is applied to the seats of front passengers (s) in the same row as the driver's seat, or the state in which occupants are on board the vehicle is simulated by a method specified by the manufacturer;

Or alternatively (at the choice of the manufacturer):

An object or human representing a 5th percentile adult female is placed on each seat cushion as specified by the manufacturer for front passengers in the same row as the driver seat, or the state in which occupants are on board the vehicle is simulated by an alternative method specified by the vehicle manufacturer as agreed by the Technical Service and the Type Approval Authority. This may also be done for the rear seats at the request of the vehicle manufacturer.

(d) The test vehicle is driven, at the choice of the manufacturer, under one of the conditions in paragraphs 2.1.1.1. to 2.1.1.3. of this annex or combination thereof;

(e) The state of the safety-belt reminder is checked for all of the seats of front passengers (s) in the same row as the driver's seat, in conditions (a) to (d).

2.2.2.		Testing the seating position of front passengers in the same row as the driver’s seat when the safety-belt becomes unbuckled during the journey:

(a) 	The safety-belts of the driver’s seat and seats for front passengers other than the driver’s seat are fastened;

(b) 	A load of 40 kg is applied to the seats of front passengers (s) in the same row as the driver's seat, or the state in which occupants are on board the vehicle is simulated by a method specified by the manufacturer;

Or alternatively (at the choice of the manufacturer):

An object or human representing a 5th percentile adult female is placed on each seat cushion as specified by the manufacturer of front passengers in the same row as the driver seat, or the state in which occupants are on board the vehicle is simulated by an alternative method specified by the vehicle manufacturer as agreed by the Technical Service and the Type Approval Authority. This may also be done for the rear seats at the request of the vehicle manufacturer.

(c) 	The test vehicle is driven, at the choice of the manufacturer, under one of the conditions in paragraphs 2.1.1.1. to 2.1.1.3. of this annex or combination thereof;

(d)	 The safety-belt(s) of the seats of front passengers in the same row as the driver's seat is (are) unbuckled;

(e) 	The state of the safety-belt reminder is checked for all of the seats of front passengers (s) in the same row as the driver's seat, for each condition (a) to (d).

Annex 18, 2.3. amend to read:

2.3.	Testing the rear seats:

(a) 	With the test vehicle stationary, the safety-belts of all seats are fastened;

(b) 	The test vehicle is put in normal operation and kept running;

(c) 	The safety-belt of one of the rear seats is unfastened;

(d) 	The functioning of the safety-belt reminder is checked for all of the seating positions in all seating rows;

(e) 	Alternatively, at the request of the vehicle manufacturer, the test procedures specified in paragraphs 2.2. to 2.2.2. for the seating position of front passengers (s) in the same row as the driver's seat may be used for any rear seating position instead.

II.		Justification

See chapter 0

*	In accordance with the programme of work of the Inland Transport Committee for 2023 as outlined in proposed programme budget for 2023 (A/77/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.