Proposal

2.		Definitions

…

2.xx.	“Acceleration control for pedal error (ACPE)” means a system to detect misapplication of the accelerator control by the driver and to control unintended acceleration.

2.xx.	“Advanced driver distraction warning (ADDW)” means a system that helps the driver to continue to pay attention to the traffic situation and that warns the driver when he or she is distracted.

2.xx.	“Blind spot monitoring system” means a system that detects and communicates to the driver the presence of another vehicle in a zone to the side of the subject vehicle or the approach of another vehicle into that zone.

2.xx	“Current trip” means the period of time extending from the event back to either the initiation of the powertrain or opening of the driver’s door, whichever period is shorter.

2.xx.	“Direction indicator” means a device mounted on a motor vehicle or trailer which, when operated by the driver, signals the latter's intention to change the direction in which the vehicle is proceeding.

2.xx.	“Driver control assistance system (DCAS)” means the hardware and software collectively capable of assisting a driver in controlling the longitudinal and lateral motion of the vehicle on a sustained basis.

2.xx.	“Driver drowsiness and attention warning (DDAW)” means a system that assesses the driver’s alertness through vehicle systems analysis and warns the driver if needed

2.xx. 	"Emergency stop signal" means a signal to indicate to other road users to the rear of the vehicle that a high retardation force has been applied to the vehicle relative to the prevailing road conditions.

2.xx.	“Rear automatic braking system” means a system which can automatically detect an imminent reversing collision and activate the vehicle braking system to decelerate the vehicle with the purpose of avoiding or mitigating a collision.

…

Annex 4 - Data elements and format

| Data element | Condition for requirement | Recording interval/time | Data sample rate | Minimum range | Accuracy | Resolution | Event(s) recorded for |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acceleration control for pedal error (ACPE) status | Mandatory | -5.0 to 0 seconds relative to time zero | 10 Hz | N/A | N/A | Faulted, Off, On but not intervening, Intervening | All 5.3.1. triggers |
| Advanced driver distraction warning (ADDW), number of warnings issued on current trip | Mandatory | Event | N/A | [0 to 100] | N/A | 1 | All 5.3.1. triggers |
| Advanced driver distraction warning (ADDW), time elapsed since its last warning issued on current trip | Mandatory | Event | N/A | [0 to 240 sec] | [+/– 1 sec] | 1 sec | All 5.3.1. triggers |
| Blind spot monitoring system status | Mandatory | [-5.0 to 0 second relative to time zero; subject to discussion on Step 2 triggers] | 2 Hz | N/A | N/A | Off, Faulted, On not warning, On warning left- side, On warning right-side | All 5.3.1. triggers |
| Direction indicator switch status | Mandatory | [-5.0 to 0 second relative to time zero; subject to discussion on Step 2 triggers] | 2 Hz | N/A | N/A | Left, Right | All 5.3.1. triggers |
| Driver control assistance system (DCAS) status1 | Mandatory | [-5.0 to 0 second relative to time zero; subject to discussion on Step 2 triggers] | 2 Hz | N/A | N/A | Faulted, Active, Passive, Inactive, Off | All 5.3.1. triggers |
| Driver control assistance system (DCAS), driver monitoring system, number of hands-on requests issued on current trip due to driver disengagement | Mandatory | Event | N/A | [0 to 100] | N/A | 1 | All 5.3.1. triggers |
| Driver control assistance system (DCAS), driver monitoring system, number of eyes-on requests issued on current trip due to driver disengagement | Mandatory | Event | N/A | [0 to 100] | N/A | 1 | All 5.3.1. triggers |
| Driver control assistance system (DCAS), driver monitoring system, number of direct control alerts issued on current trip due to driver disengagement | Mandatory | Event | N/A | [0 to 100] | N/A | 1 | All 5.3.1. triggers |
| Driver control assistance system (DCAS), driver monitoring system, time elapsed since its last warning issued on current trip | Mandatory | Event | N/A | [0 to 60 sec] | [N/A] | 0.1 sec | All 5.3.1. triggers |
| Driver control assistance system (DCAS), driver monitoring system, last warning type issued on current trip | Mandatory | Event | N/A | N/A | N/A | Hands-on request, Eyes-on request, Direct Control Alert (due to driver disengagement), Direct Control Alert (due to exiting system boundaries) No request | All 5.3.1. triggers |
| Driver drowsiness and attention warning (DDAW), number of warnings issued on current trip | Mandatory | Event | N/A | [0 to 10] | N/A | 1 | All 5.3.1. triggers |
| Driver drowsiness and attention warning (DDAW), time elapsed since its last warning issued on current trip | Mandatory | Event | N/A | [0 to 60 min] | +/– 1 min | 1 min | All 5.3.1. triggers |
| Emergency stop signal status | Mandatory | [-5.0 to 0 second relative to time zero; subject to discussion on Step 2 triggers] | 2 Hz | N/A | N/A | Off, On | Planar VRU AEB |
| [Rear automatic braking system status] | Mandatory | [-1.0 to 0 second relative to time zero; subject to discussion on Step 2 triggers] | 10 Hz | N/A | N/A | Engaged, Faulted, Off, Not Engaged | Planar VRU |

1 Status of the DCAS feature in the highest operational state

Justification

This VERSION 4 of the proposal builds on VERSION 3 (document EDR-DSSAD-IWG-27-16) and consultations with OICA on revised data formats with the aim to reduce memory requirements. Changes compared to VERSION 3 are tracked and highlighted yellow in the text above.

As defined in Mutual Resolution Mutual Resolution No. 2 (M.R.2) of the 1958 and the 1998 Agreements containing vehicle propulsion system definitions, document ECE/TRANS/WP.29/1121.