Draft Meeting Minutes of the 29th Meeting of the Informal Working Group on

Electrical Vehicle Safety - Global Technical Regulation

(EVS-GTR)

Location:	Torrance, CA (American Honda) + Web participation (underlined)

Date: 　　　　 February 17-19, 2025

Chair: 		Mr. Martin KOUBEK (the United States)

Vice-Chair:	Mr. Aleksander LAZAREVIC (European Union)

Vice-Chair:    	Dr. Hao Weijian (China) on behalf of Ms. CHEN Chunmei

Secretary: 	Dr. Kenichiroh KOSHIKA (Japan)

Participants:	Canada, China, Germany, Japan, Korea, the Netherlands, the United Kingdom, the United States, the European Union, OICA, test houses and laboratories, related industries -- total about 50 participants (35 in-person).

- Welcome

- The chair, Mr. Martin KOUBEK welcomed the participants and opened the meeting. He thanked American Honda Motor Company to provide the meeting facility.

- Ms. Alice LEE, Vice President of the Product Regulatory Office of American Honda welcomed the participants and noted that the work of EVS will provide the harmonized way for transition to zero emission vehicles. Honda will organize a tour in their facility on Thursday morning. Mr. David LIU, Director of Governmental Affairs of American Honda gave housekeeping instructions.

- Participants attending in-person made self-introduction.

2.  Approvals

- The secretary noted that there is no comment received on the draft minutes from the 28th meeting, while one question was raised from CLEPA about the pulsating DC. This issue was planned to be addressed at the co-sponsors meeting in November, but the meeting itself was cancelled. It will be considered at this session.

- The secretary introduced the draft agenda (EVS29-A03 [0130]).

- The minutes of the 28th meeting (EVS29-A07) and agenda were adopted.

3.  Reports of UN Activities

- At 194th WP29 November 2024, the mandate of this IWG was confirmed for March 2025.

- At 76th GRSP December 2024, the chair briefed that he delivered a report on the topics from the 28th IWG.

4.  Update on ongoing and planned research and rulemaking activities

4.1. Transposition of GTR 20 to National Regulation

  - China

    - The updates of GB38031 (traction battery safety) and GB18384 (electrical safety) are ongoing.

    - GB38031 is expected to be published in the first half of 2025 and will become mandatory from July 2026.

    - GB18384 will reflect the update of ISO6469-3 and will include bottom impact of the battery.

  - European Union

    - The 05 series of amendments to UN Regulation No.100 that contains the test requirements of thermal propagation will be adopted by WP29/AC1 in March 2025.

    - The scope of UN R100 is slightly expanded from that of GTR20 to include category O vehicles. Thermal propagation requirements will also apply to batteries containing alkali-metal anode.

    - For the European Union, UN Regulations need to be included in the list of the general safety regulation (Regulation (EU) 2019/2144) for the implementation as mandatory requirements and UNR100-05 will be implemented as from September 2027 for new types and from September 2029 for all vehicles at the earliest.

    - Zero CO2 policy from 2035 is still in place, but the details are still being addressed under strategic dialogue.

    - Vehicle electrification should also be robust enough in view of the development and transition towards vehicle automation.

  - Japan

    - Japan has been involved in the amendments of UN R100 and there is no other specific topic to report.

  - The United States

    - The Final Rule based on GTR No. 20 was published on 20 December 2024. In the US, only documentation requirement applies for thermal propagation, and it applies to parking, external charging, and active driving possible modes.

    - In light of the regulatory developments, including the FMVSS No. 305a and UN R 100, the GTR No. 20 should allow the approaches provided in the US rules and the UN R 100 at this stage

    - It will be necessary to request WP.29 to grant another extension of the GTR No. 20 to finalize the current work and to indicate future issues and priorities.

  - Canada

    - Canada will evaluate the US rules and begin the process to introduce into Canadian regulation in regard to future work, micro-mobility was identified as one of the possible topics.

    - The proposal (EVS29-K03) submitted for this meeting was intended as the starting point of discussion.

  - Korea

    - Korea informed participants of temporary measures being considered to address thermal incidents that would be implemented on voluntary basis.

  - The Netherlands

    - Two documents were provided for this session: one on the field data, and another on the technologies on batteries.

4.2. Related standardization committees

- OICA briefed the activity of ISO TC where a revision of ISO 6469-1 is on-going. Definitions of operating mode and thermal runaway criteria are being discussed. Addition of new trigger methods and ways to reduce the variabilities in the different procedures are also considered.

- Technical information from Contracting Parties and Industry (OICA) about the ten (10) items for Phase 2

5.1 Thermal propagation and methods of initiation in battery system

<Risk management analysis>

- CA introduced the document EVS29-K03 which tries to merge the sets of requirements being implemented in the US and the UN R 100 together into the GTR format.

- EC noted that the language should be neutral, and more time is required for EC and EU Member States to review the text.

- The reference to the other regulations should be static ones if it is included.

- China asked whether the protection scope of GTR No. 20 is limited to vehicle occupants or not, because the revision of GB38031 will include protection of surroundings. If GTR scope is limited to vehicle occupants, the expansion of the protection scope should be considered for the future amendments.

- OICA commented that the US and China should register FMVSS305a/Part 561 and GB38031 as the Compendium of Candidate under the 98 Agreement. Then, the documents are made available through UN website.

- Several comments were made on the document:

  - 5.4.12.	Who will determine how “5.4.12.2. or 5.4.12.3.” should be described.

  - 5.4.12.2.	What is the safety risks to be addressed in the risk management analysis should be clarified.

  - 5.4.12.2.5. Part IV of the risk management analysis should be added.

  - 5.4.12.3	The text should be aligned to para. 6.15.1 of UNR100-05.

- CA (EVS29-K05) and OICA (EVS29-K06) provided the comparison of risk management analysis methods in Part 561 and UNR100-05. The common technical elements should be included in GTR document.

- China noted that, in their update of GB38031, the documentation requirements remain those based on GTR20 phase 1.

- US commented that GTR should reflect the common safety elements of Part 561 and UNR100-05 so that neither Regulation would have to be changed.

- Participants are invited to analyse the table provide by CA to identify the technical differences for further discussion.

- CA introduced the updated document (EVS29-K08) prepared jointly by US and CA, where the prescription of the risk analysis method reflects the common safety elements between UN R 100 and the FMVSS. CA cautioned that the reference to a specific certification process should be eliminated.

- EC wondered whether the optional character of the provisions in the GTR may lead to variety of interpretations and difference in the implementation when the third countries transpose the GTR into their national regulations.

- NL commented that it is important to describe the examples implemented in respective countries/regions in the Part I of the GTR.

- OICA commented that there might be a potential risk of having less technical provisions. OICA also enhanced that preventive technologies should also be considered as the meaningful option to be accepted since the testing approach does not allow such a measure.

- CA agreed that it is necessary to have the preamble, and the risk may be mitigated by referencing the recently published regulations.

- CA commented that since current GTR only contains RiMA and therefore they proposed the improvement of RiMA in this document.

- OICA noted that the provisions proposed by US and CA are less descriptive than GTR20 phase 1 which was deemed by some to be too vague for implementation.

- Some CPs replied that the goal is to reflect the common safety elements across the recent regulatory actions (e.g. FMVSS, UN R 100-05, GB 38031, etc.) without impacting the recently published regulations, and prevent the introduction of entirely new approaches not yet evaluated by the IWG.

- CN provided EVS29-K07 adding the provisions of GB38031 into the comparison table of EVS29-K06. CN noted their requirements with request to the risk management analysis are based on GTR20 Phase 1.

- JP, KR and UK agreed to have GTR texts supplemented with the preambles to provide additional information.

- The chair explained his plan for the future steps, he will report to WP29 and ask for additional time (6-12 months?) solely for drafting the proposal for GTR No. 20 amendment 1. If it is supported by WP29, a virtual meeting (will be held by the end of May 2025. US and CA will take the lead in coordinating and maintaining the draft document.)

- NL suggested to request the extension of the mandate until December so that the final document will be submitted to December GRSP.

- OICA commented that multiple points raised in the phase 2 TOR have been withdrawn and should not be reopened.

- CN noted that harmonization of test procedures is their concern.

- EC asked what will be accomplished in this week. There are several limitations on testing requirements those still to be worked on further. For category L vehicles, UNR136 has already been in place.

- OICA reminded that when the UNR for category L was developed, it was too complicated to merge these vehicles into UNR100 and therefore UNR136 was developed and from that experience, it will be better to develop separate GTR for micromobility.

- OICA commented that common languages for testing are necessary in GTR document.

- EC recalled that CN was participating SIG Thermal Propagation, but CN did not fully agree with the model regulation in the end. EC considers that if testing provisions are included in GTR, those should be those based on UNR100, since the later represent a compromise solution of several contracting parties.

- US suggested to identify and reflect the common safety elements between the UN R 100 and GB 38031 - related to the test procedures for inclusion in the GTR No. 20 Amendment 1, as was done for the Phase 2 documentation method between FMVSS and UN R 100.

- CN want to propose the update of test method improvements.

- OICA commented that if the testing procedure is included in the GTR, there must be common criteria, while there are critical differences between UNR100-05 and GB38031 as summarized in EVS29-K09.

- EC noted that only reflecting RiMA updates into the GTR20 will notfully reflect the work performed by SIG TP.

- CN considered the differences between UNR100 and GB38031 are not too many and should be able to find a consensus.

- KR noted that they changed their mind from those described in EVS29-K04.

- CN confirmed its support on the RiMA part of the requirement, while, for test methods, CN has concerns on the initiation methods. The initiation methods prescribed in the Part I of GTR20 (phase 1) have been implemented in CN for some years without any issues and therefore these initiation methods will be appropriate for GTR20.

- EC noted that, in principle, the contents of GTR20 should not cause any conflict with UNR100-05. The differences between UNR100 and GB38031 are not only for on the initiation methods, but the application method, operating modes and criteria are also different. UNR100 is a harmonized regulation among contracting parties to the 58 Agreement and already including several compromises as indicated in the comparison table (EVS29-K09).

- CN commented that the differences of the vehicle operation mode and the criteria are associated with the difference of the scope of protection. In CN, the protection scope has been extended to properties in addition to vehicle occupants. The extension of the protection scope was discussed at former IWGs, but no consensus achieved and has been listed as one of the future topics.

- CN introduced their work and briefed on the choice of initiation methods and the difference of internal heater methods between UNR100-05 and GB38031.

- OICA commented that the differences of initiation methods are under discussion at ISO6469-1, where the differences of the internal heater methods will be treated as two different methods.

- NL recalled that during the discussion at SIG-TP, the efforts were made to allow the choice of appropriate initiation method and to be technology neutral in order to accommodate future initiation methods.

- KR considers that the initiation methods in UNR100-05 are sufficient.

- OICA voiced concerns about the sentence proposed by CN to allow CP to choose one initiation method as problematic since, the manufacturers may be required to test with each initiation method and defeat harmonization. OICA also claimed that leaving it open to other methods may increase uncontrolled introduction of new initiation methods.

- UK commented that there is no need to state that a CP will choose a specific approach (i.e., RiMA or test).

- The group amended the text of selection of trigger methods. 5 initiation methods, i.e. 4 methods from UNR100-05 and CN version of internal heater method (method 2), will be included.

- The second paragraph of selection of test level was removed.

- OICA pointed out there are significant differences between UNR and GB in the vehicle operation modes and the pass/fail criteria.

- CN generally agreed to keep criteria in the current draft. because the protection scope of GTR remains as vehicle occupants in phase 2.

- CN explained the background of the operation mode. According to their incident data, more than 70% of the incidents when the vehicle is parked occurs within 1 hour after the switching off the vehicle.

- NL commented that since no one may be inside the parked vehicle, the owner/user outside the vehicle and/or rescue services should be made aware of a critical thermal issue. CN noted that parking mode is the more stringent scenario and one test combining all requirements is more feasible. In addition, there may be occupant in the parking mode.

- OICA shared their understanding that, for this phase of GTR, the mode of operation should stay as it is because the condition in GB is due to the extended scope of protection.

- CN will confirm the contents of the updated document and will be shared through UN Wiki site as EVS29-K10.

5.2 Other issues

  - CLEPA – clarification of specific voltage conditions (GRSP-75-15)

- Proposed to add an additional note (in line with UNR 100) for the definition of specific voltage conditions to clarify that pulsating DC is not considered as AC.

- EC suggested to support this proposal unless any objection raised from the group.

- US requested more time to review this proposal.

- Others are fine with inclusion of this additional note.

  - The Netherlands – Incidents data for electric vehicles (EVS29-K01)

- NL introduced the material.

- CA commented that according to their literature study, the actual rate of fire occurrence is not quite different between EVs and ICE vehicles.

- KR asked about submerging container. NL responded it is a solution to use for de-energizing the EVs after the incidents.

- OICA recalled that as previously commented several times, the rate of fire occurrence is similar between EVs and ICE vehicles. Larger picture should be looked at before making the requirements for EVs more and more complex.

- CA admitted that single cell internal short circuit is a typical case of EV, but the field statistics does not show any increase of the risk.

- OICA commented that if a vehicle fire occurs in a covered garage, the fire spreads regardless of the powertrain.

  - Korea – EV fire automatic reporting system (EVS29-K04)

- KR introduced their proposal (slide #3) where KR have changed their position on point 3 of the slide. In KR, the fire incidents in KR only occurred while parking.

- OICA commented that if the report is made without having thermal propagation, it might be unnecessary use of the public service resources.

- JRC commented that vehicle with REESS that may cause thermal propagation, thermal runaway may not always cause thermal propagation.

- CA supported that this topic should be one of the issues to be addressed in the future.

- UK asked about the duration of vehicle parking during for which the automatic reporting is required.

- The chair asked about the toxicity and flammability of gases emitted from batteries should be further addressed. JRC commented their research is still ongoing. OICA commented that there are no significant differences in the toxic gases from EV fire and ICE fire.

- The chair was made aware of other potential topics to be considered for future phases .

  - CN: Scope of protection

  - CN: Bottom protection of REESS

  - KR: Automatic REESS failure reporting system

  - KR: Overcurrent test for heavy duty vehicles

6 Wrap up of the meeting, Action items

- The chair will prepare the report for March WP29 and request extension of the mandate until March 2026 (or December 2025). The report contains (a) activities made since March last year, (b) possible directions of the thermal propagation requirements, (c) future topics.

- The chair indicated that the draft amendments to GTR No. 20 that were developed in Phase 2 would be submitted as informal document to May GRSP and then the formal document would be submitted for December GRSP.  The formal document for December GRSP needs to be submitted by early September.

- Drafting group led by US and CA will start preparing the draft amendment of the GTR and to share the participants who may contribute for the drafting. NL, DE, EC, UK, CN, KR, JP as well as OICA may contribute to the drafting.

- NL noted that after concluding phase 2, any new topics should be first discussed in GRSP..

7. Future meetings

- TBD.

8. Closings

- The chair thanked all participants for active participation for this meeting and appreciated the hospitality given by American Honda Motor Company.