Proposal for supplement 2 to the 06 series of amendments to UN Regulation No. 110 (CNG and LNG vehicles)

Submitted by expert from the Netherlands *

The text reproduced below was prepared by the expert from OICA on behalf of the GRSG-TF-AVRS to amend UN Regulation No. 110, in order to enable the application of this Regulation to vehicles of categories X and Y. The modifications to the current text are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Insert new paragraphs 0. and 0.1 to 0.1.2., to read:

“0.		Introduction

0.1.		Supplement 2 to the 06 series of amendments is introduced to take into account vehicles of category X and Y, as well as conventional vehicles (i.e. vehicles with a manual mode operating at speeds above 6 km/h) which are equipped with an ADSF-2.

0.1.1. 	The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and without manual driving controls in the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.2. 	In case of vehicles with an ADS where those vehicles are also equipped with a manual driving mode operating at a speed of more than 6 km/h it is expected that in the manual driving mode the technical requirements can be applied as they would for a conventional vehicle. In a mode where an ADS feature is active the relevant ADS requirements have to be fulfilled.”

Paragraph 1 and Annex4F, footnote 1, amend to read:

"1		As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.38, para. 2 -
https://unece.org/transport/standards/transport/vehicle-regulations-wp29/resolutions

https://unece.org/transport/vehicle-regulations/wp29/resolutions

Insert new paragraphs 4.79. to 4.81. to read:

4.79.	"Occupant compartment" means the space for occupant (driver and/or passengers) accommodation bounded by the roof, floor, side walls, doors, outside glazing, front bulkhead, and the plane of the rear compartment bulkhead or the plane of the rear seat back support;

4.80.	"ADS feature of type 1 (ADSF-1)" means an ADS feature which includes an ADS fallback response requiring a fallback user

4.81.	“ADS feature of type 2 (ADSF-2)” means an ADS feature which does not include an ADS fallback response requiring a fallback user."

Paragraph 7, footnote 11, amend to read:

" The distinguishing numbers of the Contracting Parties to the 1958 Agreement are

reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles

(R.E.3), document ECE/TRANS/WP.29/78/Rev.68, Annex 3

www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.html

https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 12, footnote 12, amend to read:

"2 The distinguishing numbers of the Contracting Parties to the 1958 Agreement are

reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles

(R.E.3), document ECE/TRANS/WP.29/78/Rev.38, Annex 3

www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.html

https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 18.1.7. amended to read:

18.1.7.	No appliances shall be connected to the CNG and/or LNG system other than those strictly required for the proper operation of the engine of the motor vehicle. Notwithstanding the provisions of paragraph 18.1.7., vehicles may be fitted with a heating system to heat the passengers occupant compartment and/or the load area which is connected to the CNG and/or LNG system.

Paragraph 18.3.1.15. amended to read:

18.3.1.15.	Gas-tight housing for those components installed inside the luggage and passengers occupant compartment. If the gas-tight housing will be destroyed in case of fire, the pressure relief device may be covered by the gas-tight housing.

Paragraph 18.5.5.1. amended to read:

18.5.5.1.	A gas-tight housing over the CNG container(s)/CNG accumulator fittings, which fulfils the requirements of paragraphs 18.5.5.2. to 18.5.5.5. shall be fitted to the CNG fuel container/CNG accumulator, unless the CNG container(s)/CNG accumulator is installed outside the vehicle, outside enclosed spaces such as passengers occupant compartment or cargo area or engine compartment.

Paragraph 18.5.5.7. amended to read:

18.5.5.7.	The gas-tight housing shall contain all the components installed into the luggage or passengers occupant compartment.

Paragraph 18.8.7. amended to read:

18.8.7.	In a passengers occupant compartment or enclosed luggage compartment the fuel lines shall be no longer than reasonably required, and in any case shall be protected by a gas-tight housing.

Paragraph 18.10.2. amended to read:

18.10.2.	When the CNG/LNG container or tank is installed in the passengers occupant compartment or an enclosed (luggage) compartment the filling unit shall be located at the outside of the vehicle or in engine compartment.

II.		Justification

- See paragraph 0.

- New paragraph 4.79. aims to clarify the provisions for vehicles in case of separate driver´s compartment.

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.