Proposal for supplement 3 to the original version of UN Regulation No. 159 (Moving Off Information Systems)

Submitted by expert from the European Commission *

The text reproduced below was prepared by the expert from the European Commission JRC on behalf of the GRSG-TF-AVRS to amend UN Regulation No. 159, in order to clarify the requirements for vehicles equipped with Automated Driving Systems. The modifications to the current text are marked in “bold” for new or strikethrough for deleted characters.

I.	Proposal

Insert a new paragraph and subparagraphs 0.8, to read:

"0.8.	For supplement 3 to the original version of the Regulation:

0.8.1.	The Regulation is amended to account for vehicles of category X and Y.

0.8.2.	Recognising the need for different requirements to deal with vehicles fitted with Automated Driving Systems (ADS), this regulation has been amended to clarify that MOIS are only required to work when a driver is in control, and that vehicles without the ability to be controlled by a driver (vehicles of categories X and Y) are not required to comply with this Regulation. These changes are made based on the assumption that ADS vehicles must have good perception of all other road users in proximity of the vehicle, and the ADS must take all reasonable steps to avoid collision.

0.8.3.	In the case of vehicles equipped with an ADS other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply."

Paragraph 1.1., insert a new footnote 1, to read:

"1.1. 	This Regulation applies to the approval of vehicles of categories M2, M3, N2 and N31 with regard to an onboard system to detect and inform the driver of the presence of pedestrians and cyclists in the close-proximity forward blind-spot of the vehicle and, if deemed necessary based on manufacturer strategy, warn the driver of a potential collision."

"1 As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.8, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 1.3., amend to read:

"1.3.	The following vehicles of category M and N1 shall be exempted from this Regulation:

(a) Vehicles where installation of any device for moving off information system is incompatible with their on-road use may be partly or fully exempted from this Regulation, subject to the decision of the Type Approval Authority.

(b) Vehicles of categories X and Y1. "

Paragraph 2.11. amend to read:

"2.11. 	"Ocular reference point" means the middle point between two points 65 mm apart and 635 mm vertically above the reference point which is specified in Annex 1 of ECE/TRANS/WP.29/78/Rev.682 on the driver's seat. The straight line joining the two points runs perpendicular to the vertical longitudinal median plane of the vehicle. The centre of the segment joining the two points is in a vertical longitudinal plane which shall pass through the centre of the driver's designated seating position, as specified by the vehicle manufacturer."

Paragraph 2.11., footnote 2, amend to read:

"2 See Annex 1 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.68 - www.unece.org/trans/main/wp29/wp29wgs/wp29gen/wp29resolutions.htmlhttps://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 5.1.1. amend to read:

"5.1.1.	Any vehicle fitted with a MOIS complying with the definition of paragraph 2.1. above shall meet the requirements contained in paragraphs 5.2. to 5.8. of this Regulation.

When the vehicle is equipped with a means to automatically deactivate the MOIS in situations such as, having street cleaning equipment, snowploughs, or front loader garbage collectors attached, or the activation of an ADS feature, the following provisions shall apply as appropriate:

The vehicle manufacturer shall provide a list of situations and corresponding criteria where the MOIS is automatically deactivated to the technical service at the time of type approval and it shall be annexed to the test report.

The MOIS shall be automatically reactivated as soon as the conditions that led to the automatic deactivation are not present anymore.

Except in the case of an ADS feature being active, Aa constant optical warning signal shall inform the driver that the MOIS has been deactivated. The failure warning signal specified in paragraph 5.8. below may be used for this purpose. "

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.