Proposal for supplement 5 to the original version of UN Regulation No. 151 (Blind Spot Information Systems)

Submitted by expert from the European Commission *

The text reproduced below was prepared by the expert from the European Commission on behalf of the GRSG-TF-AVRS to amend UN Regulation No. 151, in order to clarify the requirements for vehicles equipped with Automated Driving Systems. The modifications to the current text are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Insert a new paragraph 0.8 and subparagraphs, to read:

"0.8.	For supplement 5 to the original version of the Regulation:

0.8.1.	The Regulation is amended to account for vehicles of categories X and Y.

0.8.2.	Recognising the need for different requirements to deal with vehicles fitted with an Automated Driving System (ADS), this regulation has been amended to clarify that BSIS are required to work when a driver is in control, and that vehicles of categories X and Y are not required to comply with this Regulation.  These changes are made based on the assumption that ADS has good perception of all other road users in proximity of the vehicle, and the ADS takes all reasonable steps to avoid collision.

0.8.3.	In the case of vehicles equipped with an ADS other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply."

Paragraph 1.1., insert footnote 1, to read:

"1.1.	This Regulation applies to the blind spot information system of vehicles of categories N2, N3, M2 and M31.

"1 See Annex 1 toAs defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.68,  - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Insert a new Paragraph 1.3., to read:

"1.3.	This regulation does not apply to vehicles of categories X and Y1."

Paragraph 2.5., amend to read:

"2.5. 	"Ocular reference point" means the middle point between two points 65 mm apart and 635 mm vertically above the reference point which is specified in Annex 1 of ECE/TRANS/WP.29/78/Rev.681 on the driver's seat. The straight line joining the two points runs perpendicular to the vertical longitudinal median plane of the vehicle. The centre of the segment joining the two points is in a vertical longitudinal plane which shall pass through the centre of the driver's designated seating position, as specified by the vehicle manufacturer."

Insert a new Paragraph 5.2.3., to read:

"5.2.3.	For vehicles equipped with an ADS, the BSIS shall be deactivated whilst an ADS feature is active. The BSIS shall be reactivated at the latest [500 ms] after the ADS initiates or receives the request to start the process of transferring the DDT back to a driver."

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.