|  | United Nations | ECE/TRANS/WP.29/GRSP/2025/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 17 September 2024 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on General Safety Provisions

Seventy-seventh session

Geneva, 5–9 May 2025

Item Y of the provisional agenda

UN Regulation No. 55 (Mechanical coupling)

Proposal for supplement 1 to the 03 Series of Amendments to UN Regulation No. 55 (Mechanical coupling) *

Submitted by the GRSG Task Force AVRS (Automated Vehicle Regulation Screening)

The text reproduced below was prepared by the expert from European Commission on behalf of the TF GRSG AVRS, to enable the application of the regulation to vehicles equipped with an ADS [feature type 2]. The modifications to the existing text of the UN Regulation are marked in “bold“ for new or strikethrough for deleted characters.

- Proposal

Insert a new paragraph 0, to read:

"0.	Introduction

  - For supplement 1 to the 03 Series of Amendments:

0.1.1.	The Regulation is amended to account for vehicles of category X and Y.

0.1.2. 	The Regulation was originally drafted for vehicles with driver and manual driving controls. It is the intention of this new amendment to keep the spirit of the regulation and to extend its application to vehicles without driver and without manual driving controls inside the vehicle. In the absence of driver/manual driving controls in the vehicle, provisions related to them shall not be taken into account if not already covered by this amendment.

0.1.3. 	In the case of vehicles equipped with an ADS other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active the relevant ADS requirements have to be fulfilled."

Annex 5, Paragraph 7.4., amend to read:

"7.4.	Locking devices to prevent uncoupling of fifth wheel couplings

The fifth wheel coupling shall be locked in the coupled position by two positive mechanical locking devices each of which shall remain effective should the other fail.

The primary locking device shall operate automatically but the secondary locking device may either be automatic or be engaged manually. The secondary locking device may be designed to work in conjunction with the primary device and provide an additional positive mechanical lock for the primary device. It shall only be possible to engage the secondary locking device if the primary device is properly engaged.

It shall not be possible for the locking devices to be released inadvertently.  Release shall require intentional action by the driver or operator of the vehicle, or by the ADS.

The closed and locked position of the coupling shall be indicated visually by a mechanical device and it shall be possible to verify the position of the indicator by feel, for example, to allow the position to be checked during darkness. The indication device shall indicate the engagement of both primary and secondary locking devices, however, it is sufficient for the engagement of only one device to be indicated if, in this case, the engagement of the other device is a simultaneous and inherent feature of the design.

For vehicles equipped with an ADS, the engagement status of the coupling shall be transmitted as a logic signal to the ADS."

Annex 5, Paragraph 10.4, amend to read:

"10.4. 	Locking to prevent inadvertent uncoupling:

In the closed position the coupling shall be locked by two positive mechanical engagement locking devices each of which shall remain effective should the other fail.

The closed and locked position of the coupling shall be clearly indicated externally by a mechanical device.	It shall be possible to verify the position of the indicator by feel, for example, in the dark.

The mechanical indication device shall indicate the engagement of both locking devices (an AND condition).

However, it is sufficient for the engagement of only one locking device to be indicated if, in this situation, engagement of the second locking device is an inherent feature of the design.

For vehicles equipped with an ADS, the engagement status of the coupling shall be transmitted as a logic signal to the ADS."

Annex 5, Paragraph 13.2., amend to read:

"13.2.	Remote indication (for the driver or operator of the vehicle)"

Annex 5, Paragraph 13.2.8., amend to read:

"13.2.8.	In order to avoid distracting the driver during normal driving, there shall be a provision for switching off the remote indication device but this shall be automatically reactivated when the coupling is next opened and closed - see paragraph 123.2.1."

Annex 5, Paragraph 13.3.3, amend to read:

"13.3.3.	If remote control involves the coupling being opened by external force, the condition under which the external force acts on the coupling shall be indicated appropriately to the driver or the operator of the vehicle (as applicable). This is not necessary if the external force is only operative while the remote control is operating."

Annex 5, insert new paragraph 13.4., to read:

"13.4.	Engagement status in case of vehicles equipped with an ADS

If the vehicle is equipped with an ADS, a logical signal shall be transmitted to the ADS indicating at least the closed and doubly locked position of the coupling."

II.		Justification

See paragraph 0

*	In accordance with the programme of work of the Inland Transport Committee for 2023 as outlined in proposed programme budget for 2023 (A/77/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.