|  | United Nations | ECE/TRANS/WP.29/GRSG/2025/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 11 July 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on General Safety Provisions

130th session

Geneva, 6–9 October 2025

Item 14(b) of the Provisional Agenda

Awareness of the Proximity of Vulnerable Road Users:

UN Regulation No. 167 (Vulnerable Road Users Direct Vision)

Proposal for supplement 1 to the 01 Series of Amendments to UN Regulation No. 167 (Vulnerable Road Users Direct Vision)

Submitted by the expert from the Kingdom of the Netherlands on behalf of the Task Force on Automated Vehicles Regulatory Screening*

The text reproduced below was prepared by the expert from the Kingdom of the Netherlands on behalf of the Task Force on Automated Vehicles Regulatory Screening 
(TF-AVRS). The modifications to the current text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Insert new paragraphs 0.10. to 0.10.2., to read:

“0.10.		Supplement 1 to the 01 series of amendments is introduced to take into account vehicles of category X and Y, as well as vehicles with a manual mode operating at speeds above 6 km/h which are equipped with an ADSF-2 (i.e. an ADS feature which does not include an ADS fallback response requiring a fallback user).

0.10.1. 	The Regulation was originally drafted for vehicles with a driver and manual driving controls. In the absence of a driver in the vehicle, there is no need to fulfil any of the requirements of this Regulation and therefore vehicles of categories X and Y are exempted from the scope of this Regulation.

0.10.2. 	In case of vehicles equipped with an ADS other than vehicles of categories X and Y, in the manual driving mode no special provisions or exemptions apply. In a mode where an ADS feature is active basically no requirements apply, which are related to a driver.”

Paragraph 1.1., amend to read (addition of a reference to a new Footnote 1):

“1.1.	This Regulation applies to the approval of vehicles of categories M2, M3, N2 and N31 with regard to their Direct Vision to reduce blind spots to the greatest extent possible, considering the needs of the specific type of vehicle and the operation it is intended for.”

Insert new footnote 1, to read:

"1	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3.), document ECE/TRANS/WP.29/78/Rev.8, para. 2 - 
https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Insert new paragraph 1.3., to read:

"1.3.	This Regulation does not apply to vehicles of categories X and Y1”

Paragraphs 2.13. to 2.15., amend the references to former Footnote 1 to read:

"2.13. 	"Three-dimensional reference system" means a coordinate system as defined by Addendum 6 of Mutual Resolution No. 1 (M.R.1)12. In this framework the longitudinal axis of the vehicle is designated the X axis, the lateral axis is the Y axis and the vertical axis is the Z axis.

2.14. 	"H-point Manikin" means a three-dimensional H-Point Machine as defined in Addendum 6 of Mutual Resolution No. 1 (M.R.1) 12.

2.15. 	"R-Point" means the seating reference point as defined in Addendum 6 of Mutual Resolution No. 1 (M.R.1). 12

Footnote 1, renumber as footnote 2

Paragraph 4.5.1.(a), amend the references to former Footnote 2, to read:

“4.5.1. 	A circle surrounding the letter "E" followed by:

(a) 	The distinguishing number of the country which has granted approval23; and

(b) The number (…)”

Footnote 2, renumber as footnote 3.

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.