|  | United Nations | ECE/TRANS/WP.29/GRSG/2025/XX |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General XX XX 2025 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on General Safety Provisions

130th session

Geneva, 6-9 October 2025

Item 12(a) of the Provisional Agenda

UN Regulation No. 160 (Event Data Recorder)

Proposal for Supplement 1 to the 02 Series of Amendments to Regulation No. 160 (Event Data Recorder)

Submitted by the experts of the Informal Working Group on EDR/DSSAD*

The text reproduced below was prepared by the experts of the Informal Working Group on EDR/DSSAD. It follows the work from the GRSG-TF-AVRS amending the UN Regulations under purview of GRSG in order to enable the application of those Regulations to vehicles equipped with an ADS. The modifications to the current text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

- Introduction

Insert new paragraph 0.4., to read:

“0.4.	Supplement 1 to the 02 series of amendments is introduced to take vehicles of categories X and Y into account, and vehicles which can be operated either by a driver or an ADS.”

1.		Scope

Paragraph 1.1., footnote 1, amend to read:

"1 	As defined in Section 2 of the Consolidated Resolution on the Construction of Vehicles (R.E.3),  	(document ECE/TRANS/WP.29/78/Rev.78, para 2) – 			https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 1.3., amend to read:

“1.3.	The following data elements are excluded from the scope: VIN, associated vehicle details, location/positioning data, information of the driver about any user, date and time of an event.”

2.	Definitions

Paragraphs 2.16., 2.23., 2.29., 2.30., 2.37., 2.38., 2.55., 2.56., 2.58., 2.66., 2.67., amend to read:

"2.16. 	"Deployment time, frontal air bag" means (for both driver and front passenger front seated occupants) the elapsed time from crash time zero to the deployment command or for multi- staged air bag systems, the deployment command for the first stage.”

"2.23. 	“Far-side impact centre air bag deployment, time to deploy" means the deployment time of an air bag between driver and front seat passenger the front seated occupants, relative to Time 0.”

"2.29. 	"Lateral acceleration" means the component of the vector acceleration of a point in the vehicle in the y-direction. The lateral acceleration is positive from left to right, from the perspective of the driver when seated in the vehicle facing the direction of forward vehicle travel.”

"2.30.	"Longitudinal acceleration" means the component of the vector acceleration of a point in the vehicle in the x-direction. The longitudinal acceleration is positive in the direction of forward vehicle travel."

"2.37. 	"Occupant size classification" means, for front passenger seated occupants, the classification of an occupant as an adult and not a child, and for the driver (if applicable), the classification of the driver as not being of small stature as indicated in the data format.”

"2.38. 	"Operational" means that the system or sensor, at the time of the event, is active or can be activated/deactivated by the driver.”

"2.55. 	"Time to deploy, pretensioner" means the elapsed time from crash time zero to the deployment command for the safety belt pretensioner (for both driver and front passenger front seated occupants).”

“2.56.	"Time to deploy, side air bag/curtain" means the elapsed time from crash time zero to the deployment command for a side air bag or a side curtain/tube air bag (for both driver and front passenger front seated occupants).”

"2.58. 	"Time to nth stage" means the elapsed time from crash time zero to the deployment command for the nth stage of a frontal air bag (for both driver and front passenger front seated occupants).”

"2.66.	"X-direction" means in the direction of the vehicle’s X-axis, which is parallel to the vehicle's longitudinal centreline. The X-direction is positive in the direction of forward direction2 vehicle travel."

"2.67. 	"Y-direction" means in the direction of the vehicle’s Y-axis, which is perpendicular to its X-axis and in the same horizontal plane as that axis. The Y-direction is positive from left to right, from the perspective of the driver when seated in the vehicle facing the direction of forward vehicle travel when facing the forward direction2.”

Insert new footnote 2, to read:

"2 	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3),  	document ECE/TRANS/WP.29/78/Rev.8, para 1 –

https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 4.4.1., footnote 2, renumber as footnote 3, and amend to read:

"23 The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in 	Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3),

document ECE/TRANS/WP.29/78/Rev.78, Annex 3 –

https://unece.org/transport/vehicle-regulations/wp29/resolutions"

Paragraph 11., footnote 3, renumber as footnote 4.

Annex 4, amend to read:

Annex 4

Data elements and format1

Table 1

| Data element | Condition for requirement1 | Recording interval/time2 (relative to time zero) | Data sample rate (samples per second) | Minimum range | Accuracy3 | Resolution3 | Event(s) recorded for4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Safety belt status, driver(20) | Mandatory | -1.0 sec | N/A | Fastened, not fastened | N/A | Fastened, not fastened | Planar Rollover |
| Safety belt status, front passenger(9)(20) | Mandatory | -1.0 sec | N/A | Fastened, not fastened | N/A | Fastened, not fastened | Planar Rollover |
| Safety belt status, rear passengers(17)(20) | Mandatory | -1.0 sec | N/A | Fastened, not fastened | N/A | Fastened, not fastened | Planar Rollover |
| Safety belt status, mid-position front(20) | Mandatory | -1.0 sec | N/A | Fastened, not fastened | N/A | Fastened, not fastened | Planar Rollover |
| Frontal air bag deployment, time to deploy, in the case of a single stage air bag, or time to first stage deployment, in the case of a multi-stage air bag, driver.(20) | Mandatory | Event | N/A | 0 to 250 ms | ±2 ms | 1 ms | Planar |
| Frontal air bag deployment, time to deploy, in the case of a single stage air bag, or time to first stage deployment, in the case of a multi-stage air bag, front passenger.(8)(20) | Mandatory | Event | N/A | 0 to 250 ms | ±2 ms | 1 ms | Planar |
| Frontal air bag deployment, time to nth stage, driver. (17)(16)(20) | Mandatory if fitted with a driver’s frontal air bag with a multi- stage inflator. | Event | N/A | 0 to 250 ms | ±2 ms | 1 ms | Planar |
| Frontal air bag deployment, time to nth stage, front passenger. (16) (9)(20) | Mandatory if fitted with a front passenger’s frontal air bag with a multi-stage inflator. | Event | N/A | 0 to 250 ms | ±2 ms | 1 ms | Planar |
| Side air bag deployment, time to deploy, driver(20) | Mandatory | Event | N/A | 0 to 250 ms | ±2 ms | 1 ms | Planar |
| Side air bag deployment, time to deploy, front passenger(20) | Mandatory | Event | N/A | 0 to 250 ms | ±2 ms | 1 ms | Planar |
| Side curtain/tube air bag deployment, time to deploy, driver side(20) | Mandatory | Event | N/A | 0 to 250 ms | ±2 ms | 1 ms | Planar Rollover |
| Side curtain/tube air bag deployment, time to deploy, passenger side(20) | Mandatory | Event | N/A | 0 to 250 ms | ±2 ms | 1 ms | Planar Rollover |
| Pretensioner deployment, time to fire, driver(20) | Mandatory | Event | N/A | 0 to 250 ms | ±2 ms | 1 ms | Planar Rollover |
| Pretensioner deployment, time to fire, front passenger(9)(20) | Mandatory | Event | N/A | 0 to 250 ms | ±2 ms | 1 ms | Planar Rollover |
| Seat track position switch, foremost, status, driver(20) | Mandatory if fitted and used for deployment decision | -1.0 sec | N/A | Yes or No | N/A | Yes or No | Planar Rollover |
| Seat track position switch, foremost, status, front passenger(9)(20) | Mandatory if fitted and used for deployment decision | -1.0 sec | N/A | Yes or No | N/A | Yes or No | Planar Rollover |
| Occupant size classification, driver (if applicable)(20) | If recorded | -1.0 sec | N/A | 5th percentile female or larger | N/A | Yes or No | Planar Rollover |
| Occupant size classification, front passenger(9)(20) | If recorded | -1.0 sec | N/A | 6yr old HIII US ATD or Q6 ATD or smaller | N/A | Yes or No | Planar Rollover |
| (20) The seating position shall be recorded as “position x-y” only for vehicles of category X. A separate row shall be added to the report for each seating position, and the positions shall be denoted as: x = seat row number, starting with 1 at the vehicle front; y = seat number, starting with 1 at the vehicle’s left side. For example, ‘position 1-1’ denotes the front leftmost seat and ‘position 1-2’ denotes the front second seat from the left. |  |  |  |  |  |  |  |

II.	Justification

- The Regulation is amended to account for vehicles equipped with an ADS.
References to “driver” and “passenger” are amended to “occupants”, where relevant.

- This supplement 1 to the 02 series of amendments does not impact existing non-ADS vehicles. For only ADS vehicles, the seating position will be recorded as “position x-y” by new footnote 20 (following the existing principle within UN Regulation No. 169).

- Text from paragraphs 2.29. and 2.30. removed as this is superfluous and covered by paragraphs 2.66. respectively 2.67., and new footnote 2.

*	In accordance with the programme of work of the Inland Transport Committee for 2025 as outlined in proposed programme budget for 2025 (A/79/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.