UN Regulation No. XXX

Uniform provisions concerning the approval of vehicles with regard to Automated Driving Systems

# 0.		Introduction

1.	This Regulation establishes uniform safety provisions and a harmonized methodology for validating Automated Driving System (ADS) safety to enable the safe introduction of this technology in the market. As a general concept, this Regulation requires the ADS to deliver a level of safety in mixed traffic at least equivalent to that of a competent and careful human driver.

2.	Due to the diversity of ADS vehicle configurations and the characteristics and constraints of their Operational Design Domains (ODD), this Regulation does not include detailed prescriptive requirements and specific test procedures. Instead, this Regulation covers Manufacturer requirements and processes to ensure safety management capability over the lifecycle of the ADS. This is alongside high-level outcome focused requirements for the ADS and a Safety Case approach where manufacturers are required to demonstrate, through a structured body of claims, arguments and evidence, that the ADS meets the requirements of the Regulation and is free from unreasonable risk. While Safety Management System (SMS) and Safety Cases are well established in the aerospace and nuclear industries, this is a first application for regulations administered by the World Forum for Vehicle Regulations (WP.29) with both the UN Regulation and UN Global Technical Regulation for ADS in a combined approach.

3.	This Regulation includes requirements for the ADS’s performance of the Dynamic Driving Task (DDT), the interaction between the ADS and its users, and provisions relevant to safe use such as data storage systems, cyber security and software update management. Requirements for in-service monitoring and reporting (ISMR) cover the manufacturer monitoring the performance of the ADS and reporting to the respective authorities on significant incidents that occur.

4.	The Regulation further establishes obligations on manufacturers to implement and maintain a SMS addressing the entire lifecycle of the ADS, to substantiate a comprehensive safety case and to demonstrate the credibility of the testing environments and methods used.

5.	This Regulation also details procedures to assess the documentation and evidence provided by the manufacturer. This covers the audit of the SMS, the assessment of the Safety Case and test environment, and assessor confirmatory testing to confirm the accuracy of the evidence provided by the manufacturer in the Safety Case.

6.	The requirements in this Regulation are written with the expectation that ADS software does not include the use of online in-vehicle learning Artificial Intelligence that self modifies system behaviour. Artificial Intelligence can be used to analyse and improve ADS software in an engineering environment. By means of a software update (over the air or connected) this update can be installed in the vehicle, again without in-vehicle learning features during operation of this version.

7.	The content of the Regulation is based on current evidence and research available to WP.29. As both the technology and regulatory approach develops, updates may be required to maintain a contemporary regulatory framework for ADS.

8.	To support consistent interpretation, application and assessment of these provisions, an accompanying interpretation/guidance document has been prepared, which manufacturers, technical services and relevant authorities should pay due regard to.

# 1.		Scope

1.1			This Regulation applies to the type-approval of vehicles of Categories M, 		N, L6 and L7, with regard to their Automated Driving Systems.

1.2.	This Regulation does not apply to vehicles with regard to any ADS feature that has been approved pursuant to UN Regulation No. 157 as an Automated Lane Keeping System (ALKS) except with regard to the integration of the ALKS with the ADS and any interaction of the ALKS with other ADS features.

# 2.		Definitions

(Content from the IWG on ADS).

# 3.		Application for Approval

3.1.	Prior to application for approval, and as early as reasonably practicable, the manufacturer or their duly accredited representative shall provide the following information to the Approval Authorities0F of all of the Contracting Parties in whose territory features of the ADS can be active (‘receiving Approval Authorities’).

(a)	A single point of contact for the receiving Approval Authorities to request information from the manufacturer,

(b)	The expected granting Approval Authority and the Technical Service being used by the manufacturer, if already selected, and

(c)	Brief details of the ADS, its Features and their ODDs; this information shall be treated as confidential by the receiving Approval Authority.

3.1.1.	In the case that the territory of an additional Contracting Party is added as part of an application for extension of a type-approval, the requirements of paragraph 3.1 shall apply mutatis mutandis with respect to that Contracting Party and its Approval Authority.

3.2.	The application for approval of a vehicle type with regard to its ADS shall be submitted by the manufacturer or by their duly accredited representative. It shall be accompanied by the documents mentioned below in triplicate:

3.2.1.	A certificate of compliance for the SMS in accordance with this regulation

3.2.2.	A description of the vehicle type with regard to the items mentioned in paragraph [2.39], together with a documentation package as required in Annex [1 Appendix 1] which gives access to the basic design of the ADS and the means by which it is linked to other vehicle systems or by which it directly controls output variables. The numbers and/or symbols identifying the vehicle type shall be specified.

3.2.3.		In the case of ADS with features which can be active in the territory of Contracting Parties other than the Contracting Party issuing the approval, the manufacturer shall provide to the granting approval authority the following information for each territory:

(a)	Summary of how freedom from unreasonable risk has been defined, including details of specificities for the respective territory (if any);

(b)	Summary of how the safety level of a competent and careful human driver has been determined, including details of specificities for the respective territory (if any);

(c)	Summary of how applicable traffic rules have been identified, interpreted and assessed;

(d)	Summary of specific testing carried out regarding the territory;

(e)	A single point of contact for the receiving Approval Authorities to request information from the manufacturer.

(f)	Details of the authorities identified for fulfilling the obligation to provide post-deployment notifications and reports to the ‘relevant authority’.

3.2.4.		In the case of ADS vehicles which are subject to the requirements of UN Regulation No. 79, an analysis showing that the safety concept of the ADS steering equipment is compatible with the safety concept of the ADS.

3.3.	In cases where information is shown to be covered by intellectual property rights or to constitute specific know-how of the manufacturer or of their suppliers, the manufacturer or their suppliers shall make available sufficient information to enable the checks referred to in this Regulation to be made properly. Such information shall be treated on a confidential basis.

# 4.		Approval

4.1.		Approval Authorities shall grant, as appropriate, type approval with regard to Automated Driving Systems, only to such vehicle types that satisfy the requirements of this Regulation.

4.1.1.		The Approval Authority or the Technical Service shall verify in accordance with paragraph [8], that the vehicle manufacturer has taken the necessary measures relevant for the vehicle type in respect of:

(a)	The test environments according to paragraph [7.2]

(b)	The safety case according to paragraph [7.3]

(c)	Post-deployment safety according to paragraph [7.4]

(d)	Other manufacturer requirements according to paragraph [7.5]

4.1.1.1.	In the case of ADS vehicles which are subject to the requirements of UN Regulation No. 79, the Approval Authority or the Technical Service shall verify the compatibility between the safety concept of the ADS steering equipment and the safety concept of the ADS.

4.1.2.		The Approval Authority or the Technical Service shall verify by confirmatory testing of a vehicle of the vehicle type, according to paragraph [8.3.3], that the vehicle manufacturer has implemented the measures they have documented.

4.1.2.1.	Confirmatory Testing shall at least include track and real-world testing

Note:	If the IWG on ADS includes this text in para. 8, then the WS accepts that it is deleted from this paragraph and moved to para. 8

4.1.2.2.	Track testing may be omitted from confirmatory testing if the Approval Authority or the Technical Service deems the evidence collected by real world testing sufficient to verify that the manufacturer has implemented the measures they have documented.

Note:	If the IWG on ADS includes this text in para. 8, then the WS accepts that it is deleted from this paragraph and moved to para. 8

4.1.3.		The Approval Authority or the Technical Service shall verify that the Certificate of Compliance for SMS submitted by the manufacturer is valid and is applicable to the vehicle type.

4.1.4.		The Approval Authority shall refuse to grant the type approval where the vehicle manufacturer has not fulfilled one or more of the requirements of this regulation.

4.1.5.		The Approval Authority shall also refuse to grant the type approval where the Approval Authority or Technical Service has not received sufficient information from the vehicle manufacturer to assess the Automated Driving System of the vehicle type.

4.2.		Notice of approval or of extension or refusal of approval of a vehicle type pursuant to this Regulation shall be communicated to the Parties to the 1958 Agreement which apply this Regulation, by means of a form conforming to the model in Annex [X] to this Regulation.

4.3.		Approval Authorities shall not grant any type approval without verifying that the manufacturer has put in place satisfactory arrangements and procedures to properly manage all aspects required by this Regulation.

4.4.		The Approval Authority and its Technical Services shall ensure, in addition to the criteria laid down in Schedule 2 of the 1958 Agreement that they have:

(a)	Competent personnel with appropriate skills and specific knowledge of functional safety, safety of the intended functionality, modelling & simulation, and human factors.

(b)	Implemented procedures for the uniform evaluation according to this Regulation.

4.5.	Approvals covering ADS features which can be active in the territory of a Contracting Party other than the Contracting Party issuing the approval

4.5.1.	Before granting an approval according to this UN Regulation, the granting Approval Authority shall inform the Approval Authorities2F of the respective Contracting Parties in whose territory any feature of the Automated Driving System can be active. The following information shall be provided by the granting Approval Authority to each receiving Approval Authority at least 30 days prior to granting the approval:

(a)	The information described in paragraphs 3.2.3.(a) to (f) above.

(b)	Details of requirements where a significant interpretation has been made (if any)

This information shall be considered as confidential by the receiving Approval Authority.

[This notification is only for the purpose of information and does not provide any rights or obligations to the notified Type-Approval Authorities within the Type-Approval procedure. The notifying/granting Type-Approval Authority will remain in the full responsibility at any time of the Type-Approval procedure until its completion.]

4.5.2.	Following a review of the documentation described in paragraph 4.5.1, the receiving Approval Authority may provide comments to the granting Approval Authority on the interpretation or application of this UN Regulation with respect to their territory. Comments shall be provided within 30 days of receipt of the documentation described in paragraph 4.5.1. In case of dispute, a detailed justification shall be provided.

4.5.2.1.	Following receipt of any comments, the granting Approval Authority shall either:

(a)	Ensure that the comments have been taken into account and grant the approval with the respective Contracting Party included in the list in Appendix [x] to Annex 1, or;

(b)	Ensure that the manufacturer has excluded the territory of the respective Contracting Party from the ODD of the ADS feature(s) concerned and exclude that Contracting Party from the list in Appendix [x] to Annex 1.

4.5.2.2.	In case of any dispute between Contracting Parties this shall be settled in accordance with Article 10 and Schedule 6 of the 1958 Agreement.

4.5.4.	In the case that the territory of an additional Contracting Party is added as part of the extension of a type approval, the requirements of paragraphs 4.5.1. and 4.5.2. shall apply mutatis mutandis with respect to that Contracting Party and its Approval Authority.

4.5.5.	In the case of modifications to a vehicle type resulting in extension of an approval which covers territory of other Contracting Parties, the granting Approval Authority shall consider whether these changes affect the validity of the information previously reviewed by receiving Approval Authorities. If so, the Approval Authorities of the relevant Contracting Parties shall be consulted in accordance with paragraph 4.5.1 and 4.5.2.

4.5.6.	Each Approval Authority shall, within 14 days after granting or extending a type approval pursuant to this Regulation, upload the type approval together with the supplementing documentation (including all related test reports) in English language to the secure internet database "DETA", established by the United Nations Economic Commission for Europe.

4.6.	There shall be affixed, conspicuously and in a readily accessible place specified on the approval form, to every vehicle conforming to a vehicle type approved under this Regulation, an international approval mark consisting of:

4.6.1.	A circle surrounding the letter "E" followed by the distinguishing number of the country which has granted approval

4.6.2.	The number of this Regulation, followed by the letter "R", a dash and the approval number, to the right of the circle prescribed in paragraph 4.6.1. above; and

4.7.	If the vehicle conforms to a vehicle type approved under one or more other Regulations annexed to the Agreement, in the country which has granted approval under this Regulation, the symbol prescribed in paragraph 4.6.1. above need not be repeated; in such a case, the Regulation and approval numbers and the additional symbols shall be placed in vertical columns to the right of the symbol prescribed in paragraph 4.6.1. above.

4.8.	The approval mark shall be clearly legible and be indelible.

4.9.		The approval mark shall be placed close to or on the vehicle or bodywork data plate affixed by the manufacturer.

4.10.	Annex [2] to this Regulation gives examples of arrangements of approval marks.

# 5.		General requirements

(Content from the IWG on ADS).

# 6.	ADS Safety requirements

(Content from the IWG on ADS).

# 7.		Manufacturer requirements

(Content from the IWG on ADS).

# 8.	Compliance Assessment

(Content from the IWG on ADS).

# 9.	Modifications and extension of approval of the vehicle type

9.1.	Every modification of the vehicle type with regard to this Regulation shall be notified to the Type Approval Authority which approved that vehicle type. The Type Approval Authority may then either:

(a)	Decide, in consultation with the manufacturer, that a new type approval is to be granted; or

(b)	Apply the procedure contained in paragraph 9.1.1. (Revision) and, if applicable, the procedure contained in paragraph 9.1.2. (Extension)

9.1.1. 	Revision

When particulars recorded in the information documents of Annex 1 - Appendix 1 have changed and the Type Approval Authority considers that the modifications made are unlikely to have appreciable adverse effect, and that in any case the vehicle still meets the requirements, the modification shall be designated a "revision".

In such a case, the Type Approval Authority shall issue the revised pages of the information documents of Annex 1 - Appendix 1 as necessary, marking each revised page to show clearly the nature of the modification and the date of reissue. A consolidated, updated version of the information documents of Annex 1 - Appendix 1, accompanied by a detailed description of the modification, shall be deemed to meet this requirement.

9.1.2.	Extension

The modification shall be designated an "extension" if, in addition to the change of the particulars recorded in the information folder:

(a)	Further inspections or tests are required; or

(b)	Any information on the communication document (with the exception of its attachments) has changed; or

(c)	Approval to a later series of amendments is requested after its entry into force.

9.2.	Notice of confirmation, extension, or refusal of approval shall be communicated by the procedure specified in paragraph 4.2. above, to the Contracting Parties to the Agreement applying this Regulation. In addition, the index to the information documents and to the test reports, attached to the communication document of Annex 1, shall be amended accordingly to show the date of the most recent revision or extension.

9.3.	The Type Approval Authority issuing the extension of approval shall assign a series number to each communication form drawn up for such an extension.

# 10.	Conformity of Production

Procedures for the conformity of production shall conform to the general provisions defined in Article 2 and Schedule 1 to the Agreement (E/ECE/TRANS/505/Rev.3) and meet the following requirements:

10.1.	Every vehicle approved pursuant to this UN Regulation shall be so manufactured as to conform to the type approved;

10.2.	The Type Approval Authority which has granted the approval may at any time verify the conformity of control methods applicable to each production unit. The normal frequency of such inspections shall be once every two years.

# 11. 	Penalties

11.1.	The approval granted in respect of a vehicle type pursuant to this Regulation may be withdrawn if the requirements laid down in paragraph 10. on conformity of production are not complied with.

11.2. 	The approval granted in respect of a vehicle type pursuant to this UN Regulation may be withdrawn if the post-deployment safety requirements laid down in paragraph [7.4] are not complied with.

11.3	If a Contracting Party withdraws an approval it had previously granted, it shall forthwith so notify the other Contracting Parties applying this UN Regulation by sending them a communication form conforming to the model in Annex 1 to this UN Regulation.

# 12.	Production definitively discontinued

12.1. 	If the holder of the approval completely ceases to manufacture a type of vehicle approved in accordance with this UN Regulation, the holder shall so inform the Approval Authority which granted the approval, which in turn shall forthwith inform the other Contracting Parties to the Agreement applying this Regulation by means of a communication form conforming to the model in Annex 1 to this UN Regulation.

12.2. 	The production is not considered definitely discontinued if the manufacturer intends to obtain further approvals for software updates for vehicles already registered in the market.

# 13.	Names and addresses of Technical Services responsible for conducting approval tests and of Type Approval Authorities

13.1	The Contracting Parties to the Agreement applying this Regulation shall communicate to the Secretariat of the United Nations the names and addresses of the Technical Services responsible for conducting approval tests and of the Type Approval Authorities which grant approval and to which forms certifying approval or refusal, or extension or withdrawal of approval, issued in the other countries, are to be sent.

# 14.	Certificate of compliance for a Safety Management System (SMS)

14.1.		Each Contracting Party issuing type approvals pursuant to this Regulation shall appoint an Approval Authority to carry out the assessment of the manufacturer and to issue a Certificate of Compliance for the SMS.

14.2.		An application for a Certificate of Compliance for SMS shall be submitted by the manufacturer or by their duly accredited representative.

14.3.		It shall be accompanied by the undermentioned documents in triplicate, and by the following in particular:

14.3.1.	Documents describing the Safety Management System.

14.3.2.		A signed declaration using the model as defined in Appendix [X] to Annex [XXX].

14.3.3.		In the context of the assessment, the manufacturer shall declare using the model as defined in Appendix [X] to Annex [X] and demonstrate to the satisfaction of the Approval Authority or its designated technical service that they have the necessary processes to comply with all the requirements for the SMS according to this Regulation.

14.4.		When this assessment has been satisfactorily completed and in receipt of a signed declaration from the manufacturer according to the model as defined in Appendix [X] to Annex [X], a certificate named “Certificate of Compliance for a Safety Management System as described in Annex [X] to UN Regulation No. [XXX]” shall be granted to the manufacturer.

14.5.		The Approval Authority shall use the model set out in Annex [X] to this Regulation for the Certificate of Compliance for SMS.

14.6.	The initial Certificate of Compliance for SMS issued by the Approval Authority will have a validity of maximum three years. The Approval authority shall perform a re-assessment within one year after granting the first ADS approval  under this the certificate of compliance.

14.7.		The Approval Authority which has granted the Certificate of Compliance for SMS may at any time verify that the requirements for it continue to be met. The Approval Authority shall withdraw the Certificate of Compliance for SMS if the requirements laid down in this Regulation are no longer met.

14.8.		The manufacturer shall inform the Approval Authority or its designated technical service of any change that will affect the relevance or validity of the Certificate of Compliance for SMS. After consultation with the manufacturer, the Approval Authority or its designated technical service shall decide whether a new assessment is necessary.

14.9.	In due time, permitting the Approval Authority to complete its assessment before the end of the period of validity of the Certificate of Compliance for SMS, the manufacturer shall apply for a new (or for the extension of the existing) Certificate of Compliance for SMS.	 
The Approval Authority shall, subject to a positive outcome of the assessment, issue a new Certificate of Compliance or an extension of the existing Certificate of Compliance with a validity for a further period of maximum three years. The Approval Authority shall verify that the SMS continues to comply with the requirements of this Regulation. The Approval Authority shall issue a new certificate (or extend the existing certificate) in cases where changes have been brought to the attention of the Approval Authority or its designated technical service and assessment of the changes result  in a positive judgement.

14.10.		The expiry or withdrawal of the manufacturer’s Certificate of Compliance for SMS shall be considered, with regard to the vehicle types to which the SMS concerned was relevant, as modification of approval, as referred to in this regulation, which may include the withdrawal of the approval if the conditions for granting the approval are no longer met.

Annex 1

Communication

(Maximum format: A4 (210 x 297 mm))

issued by:		Name of administration:

......................................

......................................

......................................

1

Concerning:	 	Approval granted

Approval extended

Approval withdrawn with effect from dd/mm/yyyy

Approval refused

Production definitively discontinued

of a vehicle type with regard to ADS pursuant to UN Regulation No XXX

Approval No.:

Extension No.:

Revision No.:

Reason for extension or revision:

1.	Trade name or mark of vehicle):

2.	Vehicle type

3.	Manufacturer's name and address

4.	If applicable, name and address of manufacturer’s representative

5.	General construction characteristics of the vehicle:

5.1	Photographs and/or drawings of a representative vehicle

6.	Description and/or drawing of the ADS including:

6.1.	ADS intended uses:

6.2 	List of all ADS Features:

6.3.	Overview of the ADS subsystems (incl. components):

6.4.	Installation of the ADS sensing system:

7.	ADS Feature(s) and overview (for each feature):

| Item | Description | Feature 1 | … | Feature n |
| --- | --- | --- | --- | --- |
| 7.1. | ADS Feature type (i.e. ADSF-1 or ADSF-2): |  |  |  |
| 7.2. | ADS feature(s) performance and limitations:: |  |  |  |
| 7.3. | Specified maximum speed of the ADS features declared by the manufacturer including the associated internal and external conditions (e.g. ODD, mass, etc.) |  |  |  |

7.4	Overview of the interactions between the ADS and its user(s) and/or drawing of the ADS Human-Machine Interface (HMI) including:

| Item | Description | Feature 1 | … | Feature n |
| --- | --- | --- | --- | --- |
| 7.4.1. | Means to activate, deactivate and to take over the performance of the DDT from the system (if applicable) |  |  |  |
| 7.4.2. | Methods to detect user availability (if applicable) |  |  |  |
| 7.4.3. | Methods to determine user engagement (if applicable) |  |  |  |

7.5.	Written description and/or drawing of the information given to the user, including:

| Item | Description | Feature 1 | … | Feature n |
| --- | --- | --- | --- | --- |
| 7.5.1. | System status |  |  |  |
| 7.5.2. | The role of the fallback user (if applicable) |  |  |  |
| 7.5.3 | Adapted performance in case of failure |  |  |  |
| 7.5.4 | ADS feature deactivation to manual driving (if applicable) |  |  |  |
| 7.5.5 | Fallback to an Minimal Risk Condition |  |  |  |

8.	Data Storage System for Automated Driving (DSSAD):

8.1.	DSSAD performance verified after the tests performed according to Annex [X]: yes/no

8.2.	DSSAD documentation concerning data retrievability, data integrity self-check and protection against manipulation of stored data verified: yes/no

9.	Cyber Security and Software updates

9.1.	Cyber Security Type Approval Number (if applicable):

9.2.	Software Update Type approval number (if applicable):

10.	Safety case:

10.1.	Manufacturers document reference for the Safety Case (including version number):

11.	Technical Service responsible for conducting approval tests

11.1. 	Date of report issued by that service

11.2. 	(Reference) Number of the report issued by that service

12.	Approval granted/extended/revised/refused/withdrawn2

13.	Position of approval mark on vehicle

14.	Place

15.	Date

16.	Signature

17.	Annexed to this communication is a list of documents in the approval file deposited at the administration services having delivered the approval and which can be obtained upon request.

Additional information regarding software identification and software updates (where applicable):

18.	R[number of this Regulation]SWIN:

18.1.	Is the R[number of this Regulation]SWIN held on the vehicle2: Yes/No

18.2.	Information on how to read the R[number of this Regulation]SWIN or software version(s) in case the R[number of this Regulation]SWIN is not held on the vehicle:

18.3	If applicable, list the relevant parameters that will allow the identification of those vehicles that can be updated with the software represented by the R[number of this Regulation]SWIN under the item above:

Annex 1 - Appendix 1

Information document

The following information shall be supplied in triplicate and include a list of contents. Any drawings or pictures shall be supplied in appropriate scale and in sufficient detail on size A4 or on a folder of A4 format. Photographs, if any, shall show sufficient detail.

Information document

Information document No … relating to the type-approval of a type of a Vehicle with regard to the Automated Driving System (ADS) pursuant to UN Regulation No. X.

[0.	General information

0.1.	Make (trade name of manufacturer):

0.2. 	Type:

0.2.1. 	Commercial name(s) (if available):

0.3. 	Means of identification of type, if marked on the vehicle:1

0.3.1. 	Location of that marking:

0.4 	Category of vehicle:2

0.5.	Company name and address of manufacturer:

0.8. 	Name(s) and Address(es) of assembly plant(s):

0.9. 	Name and address of the manufacturer's representative (if any):

1.	Automated Driving System (ADS)

1.1.	ADS description (as per para. 6.3 of the regulation)

1.1.1.	ADS intended uses (as per para. 7.3.1.2 of the regulation)

1.2.	Description of the ADS feature(s)

1.2.1.	Operational Design Domain/Boundary conditions and expected operating conditions (ref. .3.1.3 of the ADS regulation)

1.2.2.	System Outline (ref. 7.3.1.4 and 6.3.1.4 of the ADS regulation)

1.2.2.1.	Main ADS Functions (functional architecture)

1.2.2.1.1.	Vehicle-internal functions (e.g. Sensing and perception, Decision-making and planning, Information display/user interface)

1.2.2.1.2.	Vehicle-external functions (e.g. backend, off-board infrastructure needed)

1.2.2.2.	Overview of the major components of the ADS

1.2.2.2.1.	Control units

1.2.2.2.2.	Sensors and installation of the sensors on the vehicle

1.2.2.2.3.	Actuators

1.2.2.2.4.	Maps and positioning

1.2.2.2.5.	Other hardware

1.2.2.3.	ADS diagrams and schematics

1.2.2.3.1.	Schematic system layout (e.g. block diagram)

1.2.2.3.2.	List and schematic overview of interconnections

1.3.	ADS feature Operational information

(ref. 6.3.1.5/6/7/8/9/11/12/14/15/16 and 5.1 and 5.2 of the ADS regulation)

1.3.1.	Input and output relevant to ADS and their ranges

1.3.2.	Description of the ADS Performance of DDT (nominal, critical and failure scenarios, at ODD Boundaries, fallback strategies)

1.3.2.1.	General explanation of the main design provisions built into the ADS so as to generate safe operation under fault conditions, under operational disturbances and the occurrence of conditions that would exceed the ODD

1.3.2.3.1.	General description of failure handling main principles and fall-back level strategy including fallbacks to a Mitigated Risk Condition

1.3.3.	Description of the interactions between the ADS and its User(s)

1.3.3.2.	Human machine interaction concept with occupants, other roads users and remote interactions including information provided to the users

1.3.3.3.	Description of means to activate, deactivate the ADS feature and to take over the performance of the DDT system (if applicable)

1.3.3.4.	Description of the ADS feature deactivation to manual driving (if applicable)

1.3.3.5.	Methods to detect driver availability (if applicable)

1.3.3.6.	Methods to determine driver engagement (if applicable)

1.3.3.7.	Conditions for triggering a request to the occupants or for remote intervention

1.4.	Safety case (ref. 6.3.2 and 6.2, 6.1.4-8 and 6.4 of the ADS regulation)

1.4.1.	Manufacturer Statement that the vehicle is free from unreasonable risks

1.4.1.1.	Manufacturer’s declaration:

The manufacturer(s) ............................................................. affirm(s) that the ADS is free of unreasonable safety risks to the occupants and other road users.

1.4.2.	Description of ADS’s detection, identification, and response to hazards including the list of all identified hazards and their mitigations.

1.4.3.	Description of the strategy to determine that the ADS feature is within its ODD, to remain within its ODD in case of operational disturbances (e.g. expected conditions exceed ODD ranges), to limit sudden ODD exits and frequent activation/deactivation situations.

1.4.4.	Description of the strategy to prevent or mitigate abuse/misuse and errors by occupants, and to prevent, mitigate or deter harm to occupants caused by external sources and abuse/misuse of the vehicle or its systems from external sources.

1.4.5.	Description of the strategy to manage the risk for passengers and to avoid operating the vehicle when the general working condition are not satisfactory

1.4.6.	Description of the strategy to determine collision with safety-relevant object and to assess the damage (i.e. trivial or non-trivial damage).

1.4.7.	Verification and validation by the manufacturer for the compliance to the DDT performance requirements, the interaction interactions between the ADS and its User(s) and the other ADS requirements and the conclusion that the system is designed in such a way that it is free from unreasonable risks for the occupants and other road users and reasonable coverage of the ODD and its boundaries is achieved .

1.4.8.	Description of the adopted approach to derive behavioural competencies and scenarios that are ODD-relevant.

1.4.8.1.	Selection of nominal, critical and failure scenarios.

1.4.9.	Description of the used methods and tools (software, laboratory, others) and summary of the credibility assessment.

1.4.9.1.	Description of the results.

1.4.9.2.	Uncertainty of the results.

1.4.9.3.	Interpretation of the results.

1.4.10.	DSSAD and ISMR.

1.4.10.1	ADS data elements.

1.4.10.2.	Type of data stored.

1.4.10.3.	Storage location.

1.4.10.5.	Recorded occurrences and data elements.

1.4.10.6.	Means to ensure data security and data protection.

1.4.10.7.	Means to access the data.

1.4.11.	Cyber security and software update.

1.4.11.1.	Cyber Security type-approval number (If applicable).

1.4.11.2.	Number of the certificate of compliance for cyber-security management system (If applicable).

1.4.11.3.	Software update type-approval number (If applicable).

1.4.11.4.	Number of the certificate of compliance for software-update management system (If applicable).

1.4.12.5.	Software Identification of the ADS.

1.4.12.5.1.	Information on how to read the RxSWIN or software version(s) in case the RxSWIN is not held on the vehicle.

1.4.12.5.2.	If applicable, list the relevant parameters that will allow the identification of those vehicles that can be updated with the software represented by the RxSWIN.

1.5.	Operational Information (ref. 5.2.4, 5.3, 6.5 regulation): Information to user, Maintenance information, Instructions in case of failures and ADS request, Operational measures (e.g. when intervention is needed) (to be annexed to the information document)

1.6.	Means to enable periodic road worthiness tests (ref. 5.3 of the ADS regulation) ]

Note: this appendix is in square bracket as the experts will review it for consistency with annex1

Annex 1- Appendix 2

Addendum [X] to type approval Communication No … 
concerning the type approval of a vehicle type with regard to ADS pursuant to UN Regulation No. [ADS]

Additional information

Contracting Parties where the vehicle manufacturer has declared that the ADS has been assessed to comply with this regulation, including local traffic rules:

| Country | Assessed | Comments on any restrictions |  |
| --- | --- | --- | --- |
| E 1 Germany | Yes/No(as apply) |  |  |
| E 2 France |  |  |  |
| E 3 Italy |  |  |  |
| E 4 Netherlands |  |  |  |
| E 5 Sweden |  |  |  |
| E 6 Belgium |  |  |  |
| E 7 Hungary |  |  |  |
| E 8 Czech Republic |  |  |  |
| E 9 Spain |  |  |  |
| E 10 Serbia |  |  |  |
| E 11 United Kingdom |  |  |  |
| E 12 Austria |  |  |  |
| E 13 Luxembourg |  |  |  |
| E 14 Switzerland |  |  |  |
| E 16 Norway |  |  |  |
| E 17 Finland |  |  |  |
| E 18 Denmark |  |  |  |
| E 19 Romania |  |  |  |
| E 20 Poland |  |  |  |
| E 21 Portugal |  |  |  |
| E 22 Russian Federation |  |  |  |
| E 23 Greece |  |  |  |
| E 24 Ireland |  |  |  |
| E 25 Croatia |  |  |  |
| E 26 Slovenia |  |  |  |
| E 27 Slovakia |  |  |  |
| E 28 Belarus |  |  |  |
| E 29 Estonia |  |  |  |
| E 30 Republic of Moldova |  |  |  |
| E 31 Bosnia and Herzegovina |  |  |  |
| E 32 Latvia |  |  |  |
| E 34 Bulgaria |  |  |  |
| E 35 Kazakhstan |  |  |  |
| E 36 Lithuania |  |  |  |
| E 37 Turkey |  |  |  |
| E 39 Azerbaijan |  |  |  |
| E 40 North Macedonia |  |  |  |
| E 41 Andorra |  |  |  |
| E 43 Japan |  |  |  |
| E 45 Australia |  |  |  |
| E 46 Ukraine |  |  |  |
| E 47 South Africa |  |  |  |
| E 48 New Zealand |  |  |  |
| E 49 Cyprus |  |  |  |
| E 50 Malta |  |  |  |
| E 51 Republic of Korea |  |  |  |
| E 52 Malaysia |  |  |  |
| E 53 Thailand |  |  |  |
| E 54 Albania E 55 Armenia |  |  |  |
| E 56 Montenegro |  |  |  |
| E 57 San Marino |  |  |  |
| E 58 Tunisia |  |  |  |
| E 60 Georgia |  |  |  |
| E 62 Egypt |  |  |  |
| E 63 Nigeria |  |  |  |
| E 64 Pakistan |  |  |  |
| E 65 Uganda |  |  |  |
| E 66 Philippines |  |  |  |
| E 67 Viet Nam |  |  |  |
| E 68 Kyrgyzstan* |  |  |  |
| * The list of Contracting Parties applying UN Regulation No. [ADS] is available online: https://treaties.un.org/Pages/ViewDetails.aspx?src=TREATY&mtdsg_no=XI-B-16&chapter=11 |  |  |

Annex 2

Arrangements of approval marks

Model A

(See paragraph [4.6.] of this Regulation)

E11

a

3

a

a

2

XXXR – 002439

a

3

a = 8 mm (minimum)

The above approval mark affixed to a vehicle shows that the vehicle type concerned has, with regard to its ADS, been approved in the United Kingdom (E11) pursuant to UN Regulation No. [XXX] under approval No. 002439. The approval number indicates that the approval was granted in accordance with the requirements of UN Regulation No. [XXX] in its original version.

Model B

(See paragraph [4.7.] of this Regulation)

a

3

a

3

E11

a

3

a

2

a

| XXX | a 2 a 2 002439 |
| --- | --- |
| 31 | 021628 |

The above approval mark affixed to a vehicle shows that the vehicle type concerned has been approved in the United Kingdom (E11) pursuant to UN Regulation Nos. [XXX] and 31. The first two digits of the approval numbers indicate that these approvals were granted in accordance with the requirements of UN Regulation No. [XXX] in its original version and UN Regulation No. 31 as amended by the 02 series of amendments.

Annex [X]- Appendix[X]

Model of declaration of compliance for Safety Management System

Manufacturer’s declaration of compliance with the requirements for

safety management System.

Manufacturer Name:

Manufacturer Address:

(Manufacturer Name) attests that the necessary processes to comply with the requirements for the Safety Management System laid down in paragraph [X] of UN Regulation No.[ XXX] are installed and will be maintained.

Done at: 	(place)

Date:

Name of the signatory:

Function of the signatory:

..........................................................

(Stamp and signature of the manufacturer’s representative)

Annex [X]

Model of Certificate of Compliance for SMS

Certificate of compliance for

safety management system

With UN Regulation No. [This Regulation]

Certificate Number [Reference number]

[……. Approval Authority]

Certifies that

Manufacturer: ..................................................................................................................................

Address of the manufacturer: ..........................................................................................................

complies with the provisions of paragraph [X.X] of Regulation No. [XXX]

Checks have been performed on:......................................................................................................

by (name and address of the Approval Authority or Technical Service): ........................................

Number of report:........................

The certificate is valid until […..Date]

Done at [……Place]

On […….Date]

[………….Signature]

Attachments: description of the Safety Management System by the manufacture

-------------------------------------------

(Stamp and signature of the manufacturer’s representative)

Using the email address(es) provided on the online platform (“/343 Application”) provided to UNECE and dedicated to the exchange of such information: https://www.unece.org/trans/main/wp29/datasharing.html

Contracting Parties without an Approval Authority shall publish the relevant contact details on the online platform (“/343 Application”) in lieu of the details of an Approval Authority for this UN Regulation [along with a note stating that this is not an Approval Authority].

These provisions shall be kept under periodic review by GRVA for amendment as necessary to support effective implementation of this UN Regulation.

Using the email address(es) provided on the online platform (“/343 Application”) provided by UNECE and dedicated to the exchange of such information: https://www.unece.org/trans/main/wp29/datasharing.html

Contracting Parties without an Approval Authority shall publish the relevant contact details on the online platform (“/343 Application”) in lieu of the details of an Approval Authority for this UN Regulation [along with a note stating that this is not an Approval Authority].

The distinguishing numbers of the Contracting Parties to the 1958 Agreement are reproduced in Annex 3 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), document ECE/TRANS/WP.29/78/Rev.7

Distinguishing number of the country which has granted/extended/refused/withdrawn approval (see approval provisions in the Regulation).

Strike out what does not apply.