| ADS IWG Working Document Change Proposal Form One major comment per form (Shaded blocks for use by the IWG Secretariat) |  |  |  |
| --- | --- | --- | --- |
| Document Reference | ADS-17-18 | Date | 3 December 2025 |
| Agenda item | 6.7. | Outcome |  |
| Proposed by (affiliation only—no personal information) |  |  |  |
| SAE |  |  |  |
| Summary of Change (25 words or less) |  |  |  |
| The change would clarify that remote termination is intended to ensure the ability to deactivate the system in the case of emergency (e.g., cyberattack) and not to bypass the ADS reasoning on the need to trigger a Fallback. |  |  |  |
| Reason for Change (Justification) |  |  |  |
| The notion of triggering a fallback to MRC remotely can go against the system design philosophy of Level 4 architectures. A L4 ADS feature must be able to determine when reaching MRC is necessary, and can execute that maneuver even under reasonably foreseeable failures. The idea that a human could "bypass" that reasoning and request an MRC (in situations in which safety is compromised, as the system is no longer capable of safely continuing operations) would imply that a human makes that determination of safety, and can be considered to break some of the logic used to design a L4 ADS system. It can also add security concerns and vulnerabilities into the system from external operators and overall generate a communication connection to a heavily critical function (the MRC trigger) that in general an ADS developer may not want. |  |  |  |

| Location | 6.1.4.4.1 |  |  |
| --- | --- | --- | --- |
| Original text |  |  |  |
| 6.1.4.4. The ADS shall be capable of remote termination. 6.1.4.4.1. Remote termination for an ADS performing the DDT shall be capable of triggering an ADS fallback response. 6.1.4.4.2. Remote termination of an ADS or ADS feature(s) shall render it unable to be activated by a user until such time as the remote termination is rescinded. |  |  |  |
| Revised text |  |  |  |
| 6.1.4.4.1. Remote termination for an ADS performing the DDT shall be capable of triggering an ADS fallback response. result in a stable stopped condition or, if appropriate, resumption of manual driving in accordance with paragraph 6.2.2.3. |  |  |  |